Maryland Department of Transportation State Highway Administration

Appendix D - Frequency Guide, Construction Phases

Maryland Department of Transportation

State Highway Administration

# Material Quality Assurance Processes

## Aggregate Materials, Controlled Mixtures

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify load ticket for correct JMF.	1  per	1	Placement Day

Project Site Testing	Material Sampling and Testing

Compaction and Moisture analysis performed by Project Engineer. Submit compaction report to MMD for clearance.

1  per	1	Placement Day

1  per	1	Lane Mile per Lift

## Aggregate Materials, Crushed Gradations

### Material Production

| SATD Material Review |  |  |  | Material Certification for Project Usage |
| --- | --- | --- | --- | --- |
| Material to be sampled as Common Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 1 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Capping Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Modified Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Select Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Construction |  |  |  |  |
| Material Verification for SATD Materials |  |  |  | Material Verification |
| Project Engineer to verify material product, submit Form 14. | 1 | per | 1 | Placement Day |
| Project Engineer to submit verification samples of Common Borrow material. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of Modified Borrow material. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of material of Select Borrow. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Site Testing |  |  |  | Material Sampling and Testing |
| Compaction and Moisture analysis of Common Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Compaction and Moisture analysis of Capping Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Compaction and Moisture analysis of Modified Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| 35 LBS. in a Soils Bag |  |  |  |  |
| Compaction and Moisture analysis of Select Borrow to be | 4 | per | 1 | Lane Mile per Lift |
| performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |

Compaction and Moisture analysis of Select Borrow for Trench Backfill to be performed by Project Engineer.

Submit Compaction report to MMD for clearance.

## Aggregate Materials, High Friction Surf. Treatment

1  per	150	Linear Feet per 3 Foot Lift

### Material Production

| SATD Certification Review |  |  |  | Material Certification for Project Usage |
| --- | --- | --- | --- | --- |
| Material Certification/Documentation to be provided by material supplier during Source Submission. | 1 | per | 1 | Prior to Usage on Project |
| SATD Material Review |  |  |  | Material Certification for Project Usage |
| Supplier to provide SATD samples of Calcined Bauxite Aggregate 45 days prior to construction. Supplier to contact SATD for delivery instructions. | 2 | per | 1 | Prior to Usage on Project |
| Supplier to provide SATD samples of Polymer Resin Binder. Supplier to contact SATD for delivery instructions | 1 | per | 1 | Prior to Usage on Project |

Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify the coverage rate of the retained aggregate meets the required quantity of the special provision

Project Engineer to contact the Office of Material Technology to coordinate field acceptance testing (Table 3 of Special Provision) within 60 days after Construction

## Aggregate Materials, Other Stone and Sand

1  per	1	Test Section

1  per	0.1	Lane Mile

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify material, Submit form 14 for clearance

## Aggregate Materials, Recycled PCC Gradations

1  per	1	Item Completion

### Material Production

| SATD Certification Review |  |  |  | Material Certification for Project Usage |  |
| --- | --- | --- | --- | --- | --- |
| Material Certification to be provided by material supplier during facility visit. | 1 | per | 1 | Prior to Usage on Project |  |
| . |  |  |  |  |  |
| SATD Material Review |  |  |  | Material Certification for Project Usage |  |
| Material to be sampled as Common Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |  |
| 35 LBS in a Soils Bag |  |  |  |  |  |
| Material to be sampled as Modified Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |  |
| 35 LBS in a Soils Bag |  |  |  |  |  |
| Material to be sampled as Select Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |  |
| 35 LBS in a Soils Bag |  |  |  |  |  |
| Project Construction |  |  |  |  |  |
| Project Sampling and Lab Testing of SATD Material |  |  |  |  | Material Verification |
| Project Engineer to submit verification samples of 1 Common Borrow material. |  | per | 15000 | Cubic Yard |  |
| 35 LBS in a Soils Bag Project Engineer to submit verification samples of material 1 of Modified Borrow. |  | per | 15000 | Cubic Yard |  |
| 35 LBS in a Soils Bag Project Engineer to submit verification samples of Select 1 |  | per | 15000 | Cubic Yard |  |
| Borrow material. |  |  |  |  |  |

35 LBS in a Soils Bag

Project Site Testing	Material Sampling and Testing

Project Engineer to verify pH of placed recycled materials.	1  per	1	Project Completion

Project Engineer to verify pH of placed material, and perform Compaction and Moisture analysis of Common Borrow. Submit Compaction report to MMD for clearance.

4  per	1	Lane Mile per Lift

1  per	2000	Cubic Yard

Project Engineer to verify pH of placed recycled material, and perform Compaction and Moisture analysis of Modified Borrow.

4  per	1	Lane Mile per Lift

1  per	2000	Cubic Yard

Project Engineer to verify pH of placed material, and perform Compaction and Moisture analysis of Select Borrow. Submit Compaction report to MMD for clearance.

4  per	1	Lane Mile per Lift

1  per	2000	Cubic Yard

## Asphalt Mixtures

### Material Production

Notification of Production for ATD materials	Production Notification

Producer to notify the Asphalt Technology Division 1 day prior to production.

.

1  per	1	Placement Day

### Project Construction

| Core Sampling for Asphalt Mix Density Analysis |  |  |  | Material Sampling and Testing |
| --- | --- | --- | --- | --- |
| Project Engineer shall verify that the contractor takes QC and QA Density Cores at random locations in the pavement. (1 Sample = 5 Cores Min. See Material QA Manual for more information) | 2 | per | 1 | Production Day |
| 1 Sample = 5 - 6" Dia. Cores (min) |  |  |  |  |
| Density Gauge testing for Patches or Small Quantities |  |  |  | Material Sampling and Testing |
| Patch density measured at random locations with an asphalt density gauge, recorded on a Form 73-1P and submitted to OMT. See specification 505.03.14 for additional details. | 1 | per | 1 | Lift of each Patch |
| 2 Readings per test |  |  |  |  |
| Box Sampling for Asphalt Mixture |  |  |  | Material Sampling and Testing |
| Project Engineer shall verify that the contractor takes QC | 2 | per | 1000 | Ton |
| and QA Mix Box at random intervals during placement of asphalt mixture. | 2 | per | 1 | Production Day |
| 1 Sample = 1-50 LB Box (2-50 LB for 25.0mm Mix, 2-50 LB for 37.5mm Mix) |  |  |  |  |

Box Sampling for Patches or Small Daily Quantities of Asphalt Mixture	Material Sampling and Testing

Project Engineer shall verify that the contractor takes QC and QA Box Samples at random intervals at least once per mix for every 1000 tons or once per mix weekly, whichever yields the greater frequency. See 505.03.12 for additional details.

1 Sample = 1-50 LB Box (2-50 LB for 25.0mm Mix, 2-50 LB for 37.5mm Mix)

2  per	5	Placement Day

2  per	1000	Ton

Asphalt Mix Density Pay Factor Analysis	Sample Analysis

Density QA and QC results are to be processed by the Asphalt Technology Division to develop a recommended "Density Pay Factor".

1  per	1	Sample Set

Asphalt Mix Pay Factor Analysis	Sample Analysis

Mixture QA and QC results are to be processed by the Asphalt Technology Division to develop a recommended "Mixture Pay Factor".

1  per	1	Project Completion

1  per	6000	Ton

Evaluation of Pavement Surface Profile	Project Wide Testing

Surface profiling will be completed as indicated in Section 535 of the Special Provisions.

## Cement Materials and Bagged Mixtures, Group 1

1  per	1	Item Completion

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and submit to OMT for clearance.

1  per	1	Delivery to Project

## Cement Materials and Bagged Mixtures, Group 2

### Material Production

CTD Material Review and Acceptance	Material Certification for Project Usage

Submit sample of material accompanied by a Form 88 and Material certification (1.03 w/o test data) to OMT for job specific approval.

1 Factory Sealed Container in a Factory Sealed Container

1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and a material sample. Submit to OMT for verification and clearance

## Cement Materials and Bagged Mixtures, Group 3

1  per	1	Delivery to Project

### Material Production

CTD Material Review and Acceptance	Material Certification for Project Usage

Submit Form 88 and Material Certification (1.03 w/o test data) to OMT for job specific approval.

1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to verify approval stamp.	1  per	1	Delivery to Project

## Cement Treated Base/Cement Treated Subgrade

### Material Production

SATD Material Review	Material Certification for Project Usage

Materials to be sampled at its place of origin. Project Engineer to contact SATD to schedule sampling.

1  per	1	Prior to Usage on Project

1  per	1	Prior to Usage on Project

Project Construction

| Project Site Testing |  |  |  | Material Sampling and Testing |
| --- | --- | --- | --- | --- |
| Construct a 500 foot long control strip onsite according to section 502.03.05 | 1 | per | 1 | Project |
| Acceptance of the control strip will be based on | 1 | per | 1 | Cement Modified Subgrade |
| performing Falling Weight Deflectometer (FWD) testing and analysis per Section 3.10 of the Pavement Performance Specifications and the Pavement and Geotechnical Design Guide | 1 | per | 1 | Cement Treated Base |
| Application Rate Verification by weight of cement | 1 | per | 1 | Truck Load |
| Project Engineer to submit FM 14 for Depth and Pulverization check | 3 | per | 1 | Placement Day |
| Compaction and Moisture analysis performed by Project | 1 | per | 1 | Placement Day |
| Engineer. Submit compaction report to MMD for clearance. | 2 | per | 2 | Lane Mile per Lift |

## Compost Materials

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to inspect compost certification provided by the Contractor with the delivery to verify that compost is of the correct type per 920.02.05, and that compost is sourced from a MDA or MDE approved producer. Project Engineer to visually inspect the placed material to ensure the compost is fit for usage and applied as specified.

Project Engineer to submit Form 14 with copy of product label or delivery ticket for clearance.

## Concrete Deck Protection

1  per	1	Delivery to Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and submit to OMT for clearance.

## Contract Specified Material Systems

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to collect manufacturers certification (TC 1.03 w/o test results) and fill out Form 14 indicating material was placed in accordance with documents.

Submit certification and Form 14 to OMT for clearance.

## Curing Compounds (Liquid Membrane)

1  per	1	Item Completion

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/ test data) and submit to OMT for clearance.

## Decorative Concrete Finish, Horizontal Surface

1  per	1	Delivery to Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to inspect placed materials and submit Form 14 to MMD for clearance.

## Decorative Concrete Stains, Vertical Surface

1  per	1	Item Completion

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect manufacturer’s material certifications (TC 1.03 w/ test data) and submit to MMD for clearance.

## Decorative Reinforced Thermoplastic

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Material/Product Verification at Project Site. Project Engineer to verify material product, and submit Form 14 for material clearance, which validates the following: Verifying that the imprinting tools and materials are compatible; Monitoring the Contractor/subcontractor conformance with the Quality Control Plan; Verifying the Contractor/subcontractor qualifications to produce this type of markings and comply with these and the

manufacturer’s specifications; Verifying the actual results of all Quality Control tests for the actual materials in the batch submitted for the project.

1  per	1	Item Completion

1  per	1	Prior to Usage on Project

## Electrical Accessories

### Material Production

Approval of Material Prior to Shipment	Material Certification for Project Usage

Producer to submit Catalog Cuts to the Office of Traffic and Safety.

1  per	1	Prior to Usage on Project

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to verify material product, submit Form 14.

## Emulsified Asphalts, Other

1  per	1	Delivery to Project

### Project Construction

ATD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/ test data), FORM 88, and material samples. Submit items to OMT for testing and clearance.

1 Gallon in a Widemouth Plastic Jar

## Emulsified Asphalts, Tack Coat

1  per	1	Item Completion

### Project Construction

ATD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/ test data), FORM 88, and material samples. Submit items to OMT for testing and clearance.

1 Gallon in a Widemouth Plastic Jar

## Fertilizers

1  per	1	Item Completion

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to verify that fertilizer is from the correct source. Project Engineer to inspect fertilizer labels, visually inspect fertilizer, and verify fertilizer composition and proposed application rates. Project Engineer to verify fertilizer usage and submit Form 14 with fertilizer labels for material clearance.

## Form Release

1  per	1	Delivery to Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to inspect placed material and submit Form 14.

## Geosynthetic Materials, Geotextiles

1  per	1	Delivery to Project

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to receive certified roll data (TC1.03 w/ test data) and submit to MMD for clearance.

## Geosynthetic Materials, Other

1  per	1	Delivery to Project

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify material.	1  per	1	Delivery to Project

## Liquid Pavement Marking

### Project Construction

Pavement Marking Material Verification at Project Site	Material Verification

Project Engineer to collect manufacture’s certifications (TC 1.03 w/ test data) and OMT stamped documentation approving the material batch, submit to MMD for clearance.

1  per	1	Delivery to Project

Post Placement Project Wide Testing	Project Wide Testing

Field Testing at Project Site by the Pavement Marking Team as per MSMT 729.

## Miscellaneous Chemical Materials

1  per	1	Item Completion

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/ test data) and submit certs and Form 14 to OMT for clearance.

## Miscellaneous Landscape Materials

1  per	1	Delivery to Project

### Project Construction

Visual Inspection of Placed Material	Material Sampling and Testing

Project Engineer to inspect material. Project Engineer to submit Form 14 for material clearance.

## Miscellaneous Materials

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to inspect placed material and submit Form 14.

Project Engineer to collect certifications (TC 1.03 w/ test data) and submit certs to OMT for clearance.

## Miscellaneous Materials with Certifications

1  per	1	Delivery to Project

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to collect material certifications (TC 1.03 w/o test data) and submit to OMT for clearance.

## Mulch

1  per	1	Delivery to Project

### Project Construction

Visual Inspection of Placed Material	Material Sampling and Testing

Project Engineer to inspect placed material to verify that it from the approved source and fit for use. Project Engineer to verify that mulch is placed uniformly and correctly during and after placement. Project Engineer to submit Form 14 for material clearance.

## Pavement Markers

1  per	1	Delivery to Project

### Project Construction

Pavement Marking Material Verification at Project Site	Material Verification

Project Engineer to collect manufacturer's Certification (TC 1.03 w/o test data), and submit certifications to MMD for clearance.

1  per	1	Delivery to Project

Post Placement Project Wide Testing	Project Wide Testing

Field Testing at Project Site by the Pavement Marking Team as per MSMT 729.

## Pavement Marking Mat'l: Other

1  per	1	Item Completion

### Project Construction

Pavement Marking Material Verification at Project Site	Material Verification

Project Engineer to collect OMT stamped manufacturer's certification (TC 1.03 w/ test data) and submit them to MMD for clearance.

## Pavement Marking Preformed Thermoplastic

1  per	1	Delivery to Project

### Project Construction

Pavement Marking Material Verification at Project Site	Material Verification

Project Engineer to collect manufacturer's Certification (TC 1.03 w/o test data), and submit certifications to MMD for clearance.

1  per	1	Delivery to Project

Post Placement Project Wide Testing	Project Wide Testing

Field Testing at Project Site by the Pavement Marking Team as per MSMT 729.

## Pavement Marking Tape

1  per	1	Item Completion

### Project Construction

Pavement Marking Material Verification at Project Site	Material Verification

Project Engineer to collect manufacturer's Certification (TC 1.03 w/o test data), and submit certifications to MMD for clearance.

1  per	1	Delivery to Project

Post Placement Project Wide Testing	Project Wide Testing

Field Testing at Project Site by the Pavement Marking Team as per MSMT 729.

## Pavement Repair Materials

1  per	1	Item Completion

### Project Construction

ATD Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and submit certs to OMT for clearance.

## Plant Materials

1  per	1	Delivery to Project

### Material Production

LPD Material Review	Material Certification for Project Usage

LPD to inspect plant materials before installation. LPD to inspect plant materials after installation. LPD to submit Form 14 for materials clearance.

## Portland Cement Concrete Mixtures

1  per	1	Prior to Usage on Project

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to notify OMT one working day prior to production at concrete@sha.state.md.us

1  per	1	Production Day

Project Construction

| CTD Material Verification |  |  |  | Material Verification |
| --- | --- | --- | --- | --- |
| Project Engineer to conduct field tests. (Air Entrainment, Slump, etc.). | 1 | per | 1 | Delivery to Project |
| Project Engineer to collect certifications (TC 1.03 w/ test data) and submit to OMT for clearance. | 1 | per | 1 | Delivery to Project |
| Concrete Sampling and Testing |  |  |  | Material Sampling and Testing |
| Project Engineer to mold acceptance cylinders, Prime | 1 | per | 50 | Cubic Yard |
| Contractor to cure and transport cylinders to OMT for testing. | 1 | per | 1 | Production Day |
| 2 Cylinders = 1 Sample |  |  |  |  |
| Mix 7 -Project Engineer to mold acceptance cylinders, Prime Contractor to cure and transport cylinders to OMT for testing. | 3 | per | 1 | Placement Day |
| 2 Cylinders = 1 Sample |  |  |  |  |
| Pay Factor Calculation |  |  |  | Sample Analysis |
| OMT to perform statistical analysis on all acceptance concrete cylinder tests for project. | 1 | per | 1 | Item Completion |

## Portland Cement Concrete Mixtures, Latex Modified

### Project Construction

| CTD Material Verification |  |  |  | Material Verification |
| --- | --- | --- | --- | --- |
| Project Engineer to perform concrete plastic tests. | 1 | per | 1 | Delivery to Project |
| Typically one slump and one air entrainment. |  |  |  |  |
| Chemical Tests |  |  |  | Material Sampling and Testing |
| Submit a sample or certification (TC 1.03 w/ test data) of each component to OMT for verification. | 1 | per | 1 | Lot / Production Day |
| 2 Quarts - Latex, 1 Quart Each Admixture in a Plastic Jug |  |  |  |  |
| Concrete Sampling and Testing |  |  |  | Material Sampling and Testing |
| Submit sample to OMT for testing. | 2 | per | 1 | Truck |

Pay Factor Calculation	Sample Analysis

OMT to perform statistical analysis on all acceptance concrete cylinder tests for project.

1  per	1	Item Completion

## Precast Concrete, Drainage Structures

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to notify the Concrete Technology Division prior to production at concrete@sha.state.md.us. Please provide the design standard with the notification.

1  per	1	Prior to Usage on Project

CTD Material Review and Acceptance	Material Certification for Project Usage

Submit copies of any shop drawings, specifications, or plan sheets to the designer as per the Invitation for Bids document.

1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to visually inspect precast elements, record all QC stamps on a Form 14, and verify stamp matches the certification. Collect manufacture's certification and submit it along with the Form 14 to MMD for clearance.

## Precast Concrete, Miscellaneous

1  per	1	Delivery to Project

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to notify the Concrete Technology Division between 48 hours prior to shipment and end of shipping day at concrete@sha.state.md.us.

1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to visually inspect precast elements, record all QC stamps on a Form 14, and verify stamp matches the certification. Collect manufacture's Certification and submit it along with the Form 14 to MMD for clearance.

## Precast Concrete, Pipe

1  per	1	Delivery to Project

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to notify the Concrete Technology Division between 48 hours prior to shipment and end of shipping day at concrete@sha.state.md.us.

1  per	1	Prior to Usage on Project

CTD Material Review and Acceptance	Material Certification for Project Usage

Plant QC Personnel to perform lot inspection	1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to visually inspect pipe, record all QC Stamps from pipe on a Form 14, and verify a pipe stamp matches the certification. Collect manufacture’s Certification and submit it along with the Form 14 to MMD for Clearance.

## Precast Concrete, Prestressed and Structural

1  per	1	Delivery to Project

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to submit Shop Drawings and provide Production Notice to the Concrete Technology Division (concrete@sha.state.md.us) 30 business days before commencing production of any prestressed or other structural concrete items.

1  per	1	Prior to Usage on Project

CTD Material Review and Acceptance	Material Certification for Project Usage

Direct Inspection at Facility during Casting, Approval of Material for Project Use.

1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to visually inspect precast elements, record all stamps on a Form 14, and verify stamps match the certification. Collect manufacture's certification and submit it along with the Form 14 to MMD for clearance.

## Precast Concrete, Walls

1  per	1	Delivery to Project

### Material Production

Notification of Production for CTD materials	Production Notification

Producer to submit Shop Drawings and provide Production Notice to the Concrete Technology Division (concrete@sha.state.md.us) 30 business days before commencing production of any precast MSE or Noise Barrier components.

1  per	1	Prior to Usage on Project

CTD Material Review and Acceptance	Material Certification for Project Usage

Direct Inspection at Facility during Casting, Approval of Material for Project Use.

1  per	1	Prior to Usage on Project

Test Panel Verification	Material Certification for Project Usage

Test panel to be evaluated by OMT, LOD and OOS.	1  per	1	Prior to Usage on Project

### Project Construction

CTD Material Verification	Material Verification

Project Engineer to visually inspect precast elements, record all stamps on a Form 14, and verify stamps match the certification. Collect manufacture's certification and submit it along with the Form 14 to MMD for clearance.

## Recycled Materials for HMA Mixtures

1  per	1	Delivery to Project

### Project Construction

ATD Material Verification	Material Verification

Project Engineer to visually inspect placed material and submit Form 14 to OMT for clearance.

## Reflective Sheeting

1  per	1	Item Completion

### Project Construction

OOTS Material Verification	Material Verification

See source response for "Aluminum Signs" for this item for acceptance

## Sealers: Qualified Liquid Materials

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to collect certifications (TC1.03 w/ test data) and submit certifications to OMT for clearance.

## Seed for Mixes

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to inspect labels, seed material, seed mixtures, and seed application methods. Project Engineer to submit copies of labels with Form 14 for material clearance.

## SM Group 1 - General Approval

1  per	1	Item Completion

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to submit Form 14 to the Materials Management Division confirming the material and its placement are acceptable and conform to specifications.

1  per	1	Delivery to Project

## SM Group 2 - General Approval with Certification

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect manufacturer's certification (TC 1.03 w/o test data) indicating the specification to which the material conforms. Submit certification with Form 14 Stamp to the Materials Management Division for clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

Project Engineer to collect manufacturer's certification (TC 1.03 with test data) indicating the specification to which the material conforms. Submit certification with Form 14 Stamp to the Materials Management Division for clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

## SM Group 3 - Evaluation Before Shipment

1  per	1	Delivery to Project

1  per	1	Delivery to Project

### Material Production

OMT approval of material for shipment, Structural Materials	Material Certification for Project Usage

Submit Material Certification (TC 1.03 w/ Batch Specific Test Data) to metals@mdot.maryland.gov for review prior to delivery. If acceptable the document will be stamped by OMT and returned to the supplier to accompany the material to the project site.

The Structural Materials team or an Authorized Representative will inspect the material, all relevant certifications, and may take material samples at the production facility. Please contact metals@mdot.maryland.gov to schedule an inspection.

Submit Material Certification (TC 1.03 w/o test data) for timber preservatives to metals@mdot.maryland.gov. If acceptable the document will be stamped by OMT and returned to the source of supply to accompany the material to the project site.

Submit Sample, Form 88 and Material Certification (TC 1.03 w/o test data) for clay brick materials to OMT for evaluation. Form 88 will be returned to PE. Alternate sampling may be requested by contacting metals@mdot.maryland.gov 3 weeks prior to delivery.

10 Bricks

Submit Sample, Form 88 and Material Certification (TC 1.03 w/o test data) for concrete block to OMT for evaluation. Form 88 will be returned to PE. Alternate sampling may be requested by contacting metals@mdot.maryland.gov 3 weeks prior to delivery.

6 Blocks

1  per	1	Prior to Usage on Project

1  per	1	Prior to Usage on Project

1  per	1	Prior to Usage on Project

1  per	50000	Each

1  per	6000	Each

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect OMT stamped documentation indicating review and approval of material. Submit document to the Materials Management Division for materials clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

## SM Group 4 - Detectable Warning Surfaces

1  per	1	Delivery to Project

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and submit certs to OMT for clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

1  per	1	Delivery to Project

## SM Group 5 - Producer and Batch Data Tracked

### Material Production

OMT approval of material for shipment, Structural Materials	Material Certification for Project Usage

Batch/Heat of Material must be approved by OMT. The producer to submit Material Certification (TC 1.03 w/ test data) and Sample to Structural Materials for evaluation. Contact metals@mdot.maryland.gov for further information.

1  per	1	Batch, Lot or Heat

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect OMT stamped Material Certification (TC 1.03 w/ test data) and submit to the Materials Management Division for materials clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

## SM Group 6 - Qualified Facilities, Limited Review

1  per	1	Delivery to Project

### Material Production

Quality Assurance of Structural Material production facilities.	Material Certification for Project Usage

Facility re-qualification required, Structural Materials or an Authorized Representative to conduct facility evaluation and reissue facility certification. Please contact metals@mdot.maryland.gov to schedule an evaluation.

1  per	12	Months

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect the stamped supplier's Certification of Compliance listing the materials shipped and submit to the Materials Management Division for materials clearance.

## SM Group 7 - Qualified Facilities, Cert Review

1  per	1	Delivery to Project

### Material Production

Quality Assurance of Structural Material production facilities.	Material Certification for Project Usage

Facility re-qualification required. Structural Materials or an Authorized Representative to conduct facility evaluation prior to material approval. Contact metals@mdot.maryland.gov to determine inspection schedule.

1  per	12	Months

OMT approval of material for shipment, Structural Materials	Material Certification for Project Usage

Submit Material Certification (TC 1.03 w/batch test data). All Material Test Result (MTR’s) for steel and iron products must include a statement that material was produced (Melted and Manufactured in the United States) and sent to metals@mdot.maryland.gov for review and approval. If acceptable, the documents will be stamped and returned to the vendor to accompany the delivery.

Submit Material Certification (TC 1.03 w/ test data) and a sample to Structural Materials, contact metals@mdot.maryland.gov. If acceptable the document will be stamped by OMT and returned to the source of supply to accompany the delivery.

1  per	1	Prior to Usage on Project

1  per	1	Prior to Usage on Project

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect OMT stamped documentation indicating review and approval of material. Submit document to the Materials Management Division for materials clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

## SM Group 8 - Qualified Facilities, Production Insp

1  per	1	Delivery to Project

### Material Production

Quality Assurance of Structural Material production facilities.	Material Certification for Project Usage

Facility re-qualification required. Structural Materials to conduct facility evaluation and reissue facility certification.

1  per	12	Months

OMT approval of material for shipment, Structural Materials	Material Certification for Project Usage

The Metals team or an Authorized Representative will inspect the fabrication, all relevant certifications, and may take material samples at the production facility. Please contact metals@mdot.maryland.gov to determine inspection schedule.

1  per	1	Prior to Usage on Project

### Project Construction

Material Verification at Project Site, Structural Materials	Material Verification

Project Engineer to collect OMT stamped documentation indicating review and approval of material. Submit document to the Materials Management Division for materials clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance. Submit the Domestic Materials Self-Certification Form to Materials Management Division for materials clearance.

## Sod & Seed Mixes with MDA Labels

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to inspect and verify labels or tickets. Project Engineer to submit copy of label or ticket with Form 14 for material clearance.

## Soil Amendments

1  per	1	Delivery to Project

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to inspect labels and verify composition of material product. Project Engineer to determine that material is fit for use and applied as specified. Project Engineer to submit copy of product label with Form 14 for material clearance.

## Soil Materials, Fill Soil

1  per	1	Item Completion

### Material Production

| SATD Material Review |  |  |  | Material Certification for Project Usage |
| --- | --- | --- | --- | --- |
| Material to be sampled as Common Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Capping Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Modified Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Select Borrow at its place of origin. Project Engineer to contact SATD to schedule a sampling. | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |
| Material to be sampled as Clay Core material at its place of origin. Project Engineer to contact SATD to schedule a sampling | 2 | per | 1 | Prior to Usage on Project |
| 35 LBS in a Soils Bag |  |  |  |  |

Project Construction

Project Sampling and Lab Testing of SATD Material	Material Verification

| Project Engineer to submit verification samples of Common Borrow. | 1 | per | 15000 | Cubic Yard |
| --- | --- | --- | --- | --- |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of material of Capping Borrow. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of material of Modified Borrow. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of material of Select Borrow. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Engineer to submit verification samples of Clay Core material. | 1 | per | 15000 | Cubic Yard |
| 35 LBS in a Soils Bag |  |  |  |  |
| Project Site Testing |  |  |  | Material Sampling and Testing |
| Compaction and Moisture analysis of Common Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report, Form OMT 161 (both pages) or OMT 031 to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| Compaction and Moisture analysis of Capping Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| Compaction and Moisture analysis of Modified Borrow to | 4 | per | 1 | Lane Mile per Lift |
| be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| Compaction and Moisture analysis of Select Borrow to be | 4 | per | 1 | Lane Mile per Lift |
| performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |
| Compaction and Moisture analysis of Select Borrow for Trench Backfill to be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 150 | Linear Feet per 3 Foot Lift |
| Compaction and Moisture analysis of Clay Core material to be performed by Project Engineer. Submit Compaction report to MMD for clearance. | 1 | per | 2000 | Cubic Yard |

## Soil Materials, Furnished Landscaping Soils

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify material and submit the approved Form 27B to the Materials Management Division for clearance. Form 27B may be found on the Documents Tab on the material source submission.

## Soil Materials, Salvaged & Other Landscaping Soils

1  per	1	Delivery to Project

### Material Production

SATD Material Review	Material Certification for Project Usage

Project Engineer to contact SATD at least 30 days prior to material usage or placement. SATD will sample material at source, ensure material testing, and provide a Soil Test Report (Form 27B) and the Nutrient Management Plan.

20 LB in a Soils Bag

Project Engineer to contact SATD at least 30 days prior to material usage or placement. SATD will sample material at source, ensure material testing, and provide a Soil Test Report (Form 27B).

20 LB in a Soils Bag

1  per	1	Prior to Usage on Project

1  per	1	Prior to Usage on Project

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify material and submit the approved Form 27B to the Materials Management Division for clearance.

1  per	1	Item Completion

## Soil Stabilization Matting

### Project Construction

LPD Material Verification	Material Verification

Project Engineer to visually inspect soil stabilization matting and fasteners during installation. Project Engineer to visually inspect placed material after installation.

## Surface Treatments

1  per	1	Delivery to Project

### Material Production

Notification of Production for ATD materials	Production Notification

Producer to notify the Asphalt Technology Division one week prior to material placement.

.

1  per	1	Placement Day

Project Construction

| ATD Material Verification |  |  |  | Material Verification |
| --- | --- | --- | --- | --- |
| Project Engineer to collect material certifications (TC 1.03 w/o test results) and submit to materials management for clearance. | 1 | per | 1 | Delivery to Project |
| . |  |  |  |  |
| Sampling and Testing of ATD materials |  |  |  | Material Sampling and Testing |
| Project Engineer to sample emulsion and mixture. Sample to be delivered to OMT for verification. | 1 | per | 1 | Production Day |
| 1 Gallon of Mixture and 1 Gallon of Emulsion in a Widemouth Plastic Jar |  |  |  |  |

## Traffic and Related Materials, Miscellaneous

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to submit Form 14 to the Materials Management Division confirming the product is undamaged, acceptably placed, and conforms to specification.

## Traffic Barrier End Treatments

1  per	1	Delivery to Project

### Material Production

Approval of Material Prior to Shipment	Material Certification for Project Usage

Contractor to submit "Request to use NCHRP-350 Product" to the Engineer of Record. The Engineer of Record refers to a Project's design Engineer or plan sheet approver who is a Licensed Engineer in the field of design and MASH implementation.

1  per	1	Prior to Usage on Project

### Project Construction

OOTS Material Verification	Material Verification

See source response for "Metal Traffic Barrier, Certified Installer" for this item for acceptance procedures.

Project Engineer to collect manufacturer's certification (TC 1.03 w/o test data) and submit certifications to the Materials Management Division for clearance.

## Traffic Control Materials, Qualified

1  per	1	Delivery to Project

1  per	1	Delivery to Project

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to collect certifications (TC 1.03 w/o test data) and submit certs to the Materials Management Division for clearance.

## Traffic Control Materials, Qualified & Temporary

1  per	1	Delivery to Project

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to submit Form 14 to the Materials Management Division confirming the product is undamaged, acceptably placed, and conforms to specification. Indicate the product manufacturer and model number (when available).

## Traffic Control Materials, Specialty Items

1  per	1	Delivery to Project

### Material Production

Approval of Material Prior to Shipment	Material Certification for Project Usage

Producer to submit Shop Drawings or Catalog Cuts to the Office of Traffic and Safety for evaluation.

1  per	1	Prior to Usage on Project

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to submit Form 14 to the Materials Management Division confirming receipt of approved documentation from OOTS and conformance to specifications.

## Traffic, Misc. Services and Products

1  per	1	Delivery to Project

### Project Construction

OOTS Material Verification	Material Verification

Project Engineer to submit Form 14 to the Materials Management Division confirming the product is undamaged, acceptably placed, and conforms to specification. Indicate the product manufacturer and model number (when available).

## Water, Pesticides, and Adjuvants

1  per	1	Delivery to Project

### Material Production

LPD Material Review	Material Certification for Project Usage

Project Engineer to contact LPD to ensure that pesticides and pesticide application methods meet specifications. An Integrated Pest Management Plan (IPM Plan) or Integrated Pest Management Form (IPMF) may be required for approval by LPD.

1  per	1	Prior to Usage on Project

### Project Construction

Visual Inspection of Placed Material	Material Sampling and Testing

Project Engineer to inspect placed material. Project Engineer to submit Form 14 for material clearance.

1  per	1	Item Completion