Maryland Department of Transportation State Highway Administration

Appendix E - Frequency Guide, Small Quantity Provisions

Maryland Department of Transportation

State Highway Administration

# Materials With Small Quantity Provisions

## Aggregate Materials, Controlled Mixtures

### Project Construction

Project Site Testing	Material Sampling and Testing

Compaction and Moisture analysis performed by Project Engineer. Submit compaction report to MMD for clearance.

Project Engineer will conduct compaction and moisture analysis.

1  per	1	Placement Day

1  per	1	Lane Mile per Lift

Requirement waived if the quantity is less than 200 TON per day or 1000 TON per project.

## Aggregate Materials, Crushed Gradations

### Project Construction

Material Verification for SATD Materials	Material Verification

Project Engineer to verify material product, submit Form 14.

1  per	1	Placement Day

To be used when the aggregate is required in drainage or general conditions.

Requirement waived if the quantity is less than 100 TON per day or 500 TON per project.

## SM Group 3 - Evaluation Before Shipment

### Material Production

OMT approval of material for shipment, Structural Materials	Material Certification for Project Usage

| Submit Sample, Form 88 and Material Certification (TC | 1 per 50000 | Each |
| --- | --- | --- |
| 1.03 w/o test data) for clay brick materials to OMT for |  |  |
| evaluation. Form 88 will be returned to PE. Alternate |  |  |
| sampling may be requested by contacting |  |  |
| metals@mdot.maryland.gov 3 weeks prior to delivery. |  |  |
| Only applicable for Clay Brick 10 Bricks |  |  |

Requirement waived if the quantity is less than 200 EA per day or 1000 EA per project.

| Submit Sample, Form 88 and Material Certification (TC | 1 per 6000 | Each |
| --- | --- | --- |
| 1.03 w/o test data) for concrete block to OMT for |  |  |
| evaluation. Form 88 will be returned to PE. Alternate |  |  |
| sampling may be requested by contacting |  |  |
| metals@mdot.maryland.gov 3 weeks prior to delivery. |  |  |
| Only applicable for Masonry Concrete Block 6 Blocks |  |  |

Requirement waived if the quantity is less than 200 EA per day or 1000 EA per project.

## Soil Materials, Salvaged & Other Landscaping Soils

### Material Production

SATD Material Review	Material Certification for Project Usage

# Materials With Small Quantity Provisions

Project Engineer to contact SATD at least 30 days prior to material usage or placement. SATD will sample material at source, ensure material testing, and provide a Soil Test Report (Form 27B) and the Nutrient Management Plan.

Required for Salvaged Topsoils.

20 LB in a Soils Bag

1  per	1	Prior to Usage on Project

### Requirement waived if the quantity is less than 30 SY per project.

Project Engineer to contact SATD at least 30 days prior to material usage or placement. SATD will sample material at source, ensure material testing, and provide a Soil Test Report (Form 27B).

Required for Subsoils, Furnished or Salvaged.

20 LB in a Soils Bag

1  per	1	Prior to Usage on Project

Requirement waived if the quantity is less than 30 SY per project.