# Office of Materials Technology Graded Aggregate Base

# Plant Certification Procedure

# Soils and Aggregate Technology Division 7450 Traffic Drive

Hanover, Maryland 21076

# GAB Plant Contacts

Dr. Intikhab Haider, Division Chief

443-572-5162 or IHaider2@mdot.maryland.gov

Vladimir Tanchev, Field Assistant Division Chief 443-572-5024 or VTanchev@mdot.maryland.gov

Amanuel Welderufael, Lab Assistant Division Chief 443-572-5276 or Awelderufael@mdot.maryland.gov

Stuart Sommers, Field Team Leader

443-695-0927 or SSommers@mdot.maryland.gov

Table of Contents

GAB Plant Contacts	2

Introduction	4

GAB Plant Requirements	5

GAB Plant Certification Process	6

GAB Plant Certification Process Flow Chart	7

GAB: Test Frequency and SampleQuantity Requirements	8

“Appendix A”: Job Mix Formula (JMF) Letter - Request Form	9

“Appendix B”: GAB Plant - Quality ControlPlan (Template)	11

“Appendix C”: GAB -1 First Load Ticket	14

“Appendix D”: Form – 43 GAB Plant Report	16

# Introduction

This document details the certification procedure of Graded Aggregate Base Plants and serves as a guideline for Aggregate Producers who request for the certification of their GAB plants. The Certified GAB Plants List will be updated and published online on a quarterly basis on the last day of January, April, July and October of each year. Plant Certification will be valid for two years. GAB Plants are certified when they are issued with an Approved JMF Letter and their QC Plan has been approved by MDOT, SHA. All plants are required to satisfy the MDOT SHA’s Standard Specification for Construction and Materials, section 915.04 Base Course Plants ( https://roads.maryland.gov/ohd2/2021_Standard_Specifications.pdf ) Plants to maintain their certification.

GAB producers are responsible for keeping track of their records, including due dates for submitting a Job Mix Formula (JMF) determination request and Quality Control Plans review. Producers are also responsible for informing the Soils and Aggregate Technology Division ofany changes to producer information, such as producer name, quarry name, subsidiary producer’s name, contact information, qualified use of aggregate.

The following acronyms will be used throughout the document:

GAB: Graded Aggregate Base

RC-GAB: Recycled Concrete -Graded Aggregate Base SATD: Soils and Aggregate Technology Division MDOT: Maryland Department of Transportation

SHA: State Highway Administration

JMF: Job Mix Formula QC: Quality Control QA: Quality Assurance

# GAB Plant Requirements

GAB Plants producing graded aggregate base material shall meet MDOT SHA’s Standard Specification for Construction and Materials Section 915.04 .

Once a written request has been received, a facility visit will be scheduled to inspect all Quality Control testing equipment and to witness the GAB production process and verify the required documentation is posted and/or on file at the GAB Plant:

The facility visit will witness and/or verify the following:

- Lab

  - Lab facility condition & functionality

  - Lab & Truck scale certifications

  - QC Testing equipment

  - Production sample storage

  - Approved JMF / Approved QC Plan / Approved Contract List / Plant Certificate (Posted on a bulletin board)

  - Daily Production Log

  - Records

- Technician Certification(s)

  - MARTCP Certificate https://www.roads.maryland.gov/mdotsha/pages/Index.aspx?PageId=53

  - Reciprocal Certificate (State)

- GAB Plant Production

  - Pug Mill mixer box

  - Stockpile

  - Belt

- QC Testing/Reporting Procedures

  - Frequency of test for Gradation and moisture determination

  - Sampling (AASHTO R 90)

  - Reporting requirements (Form 43)

  - First load ticket (Form GAB-1)

- Reporting/Material Failure Resolution Procedures

  - Re-sample for QC re-test

  - Stop shipping

  - Notify all stakeholders (Internal and External)

# GAB Plant Certification Process

### Written Request:

The GAB producer shall submit a written request to: Dr. Intikhab Haider, Division Chief

Soils and Aggregate Technology Division Maryland State Highway Administration 7450 Traffic Drive

Hanover, MD 21076 OR

### IHaider2@mdot.maryland.gov

- SATD requires GAB producers to submit their written requests, for the renewal of JMF Letters and approval of QC Plans, 60 days before the expiration of an existing JMF Letter and/or a QC Plan. In case of a new plant or a change to the JMF, producers can send their written requests at any time of the year. Test samples shall be delivered to MDOT SHA laboratory, at least 45 days prior to the next publication date of the GAB Certified Plant List.

- Please refer to Appendix A for “Job Mix Formula (JMF) Letter - Request Form”. Producers shall complete and submit this form for certifying a new or existing GAB Plant.

- Please refer to Appendix B for “GAB Quality Control Plant Template”. Producers will complete and submit their Quality Control Plan using the format provided in this template.

- Safety Data Sheet (SDS): RC-GAB requests must be accompanied by a Safety Data Sheet with specific material identification and source(s) of material.. No RC-GAB sample will be accepted prior to receipt of the SDS.

### Confidentiality:

All information provided by a Producer, Fabricator or Supplier will be kept confidential by SHA and OMT. An exception could occur when the Producer, Fabricator or Supplier makes it publicly available or when agreed upon with SHA and OMT. If the need arises for the release of confidential information by either law or contractual requirements, the Producer, Fabricator or Supplier would be informed of the release of information unless prohibited by law. This confidentiality includes information obtained from sources other than the Producer, Fabricator or Supplier (e.g., complainant, regulators).

# GAB Plant Certification Process Flow Chart

Producer submits JMF Determination Request and QC Plan to SATD

SATD determines inspection, sampling and testing charges

New Plant: Cost Letter is sent

for plant inspection, sampling and testing charges.

Existing Plant with Active

Contract Number: SATD issues a “Form 12” and producers sample material per AASHTO R - 90 and deliver test samples.

Existing Plant without active

Contract Number, receives a Cost Letter for testing charges.

Once payment has been received, SATD inspects plant, samples material and tests it for JMF determination.

SATD tests material for JMF determination.

Once payment has been received SATD issues a “Form 12”,producers sample material per AASHTO R -90 and deliver samples. SATD tests material for JMF determination.

SATD approves QC Plan (requires adjustments if need be) within 10 days.

SATD issues an approved JMF Letter, (if plant passes inspection in case of new Plant).

SATD issues a GAB Plant Certificate, with the same expiry date as its QC Plan and JMF Letter.

# GAB: Test Frequency and Sample Quantity Requirements

| GAB Category | Required Tests | Specifications (AASHTO) | % Max | Sampled / Delivered by | Test Frequency | Material Quantity |
| --- | --- | --- | --- | --- | --- | --- |
| Virgin Aggregate GAB | Sieve Analysis | T-27 | N/A | SATD | 10 production days / 1000 Tons of production | 1 bag of GAB (35 lbs.) |
|  | Specific Gravity & Absorption | T-85 | N/A | SATD | 6 months | 1 bag of #57 (35 lbs.) |
|  | For JMF Determination |  |  |  |  |  |
|  | Density Moisture Relationship | T-180 | N/A | Producer | 2 year | 3 bags of GAB (35 lbs.) |
|  | Plasticity Index | T-90 | 6 | Producer |  |  |
| Recycled Concrete GAB | Sieve Analysis | T-27 | N/A | SATD | 10 production days/ 1000 Tons of production | 1 bag of GAB (35 lbs.) |
|  | pH Test | T-289 | 12.4 |  |  |  |
|  | Specific Gravity & Absorption | T-85 | N/A | SATD | 6 months | 1 bag of #57 (35 lbs.) |
|  | For JMF Determination |  |  |  |  |  |
|  | Density Moisture Relationship | T-180 | N/A | Producer | 2 year | 3 bags of GAB (35 lbs.) |
|  | Plasticity Index | T-90 | 6 |  |  |  |
|  | pH Test | T-289 | 12.4 |  |  |  |
|  | Los Angeles Abrasion | T-96 | 50 |  |  |  |
|  | Magnesium Sulfate Soundness | T-104 | 18 |  |  |  |

“Appendix A”: Job Mix Formula (JMF) Letter - RequestForm

“Appendix B”: GAB Plant - Quality Control Plan (Template)

(The purpose of the GAB QC Plan Template is to have all GAB producers submit their Quality Control Plans in a consistent manner. Producers may cut and paste their current plans and submit in a similar format as the template. Additional pages may be added, for technician certifications.)

Quality Control Plan for GAB Plants

Reference: MD Standard Specifications for Construction and Materials Sections 915 / AASHTO T-27, AASHTO T-11, MSMT 251, R 90

  - Plant Description

    - Plant Type

    - Plant Address

  - Plant Certified Technicians

MARTCP Qualified Personnel / Contact Information/Card#/Expiry Date (Submit copies of all MARTCP cards)

  - Product Description

  - Sampling and testing frequency procedures

  - Corrective Action Procedure

  - Documentation (all documentation will be reported to GAB@MDOT.Maryland.gov )

    - 24 Hour Notice (form 43) to be sent to the GAB email address and retained on file.

    - Form GAB-1 (First Load Ticket) will be sent to project and retained on file.

    - Report production Form 43 (or cancellation) with QC testing the following workday and send to the GAB email address and retained on file.

  - Truck Scales (915.01.04)

Calibration Frequency (submit copies of Scale Certifications)

  - Lab Scales Weighing / Measuring Devices (915.01.04) Calibration Frequency (submit copies of Scale Certifications)

  - Construction and Maintenance of GAB and Aggregate Stockpiles (915.01.03)

“Appendix C”: GAB -1 First Load Ticket

# Maryland Department of Transportation State Highway Administration

“Appendix D”: Form – 43 GAB Plant Report