Prepared by the Pavement and Geotechnical Division of the Office of Materials Technology

DISCLAIMER

The Maryland Department of Transportation State Highway Administration assumes no responsibility for the conclusions and statements made by other entities using the information presented in this report.

TABLE OF CONTENTS

1	Overview	1-1

1.01	Introduction	1-1

1.02	MDOT Organization	1-2

1.03	SHA PM Guide Organization	1-5

1.04	Section Version History	1-1

2	Planning	2-1

2.01	Introduction	2-1

2.02	Update of HMIS Tables	2-3

2.03	Production of Annual Section Table	2-6

2.04	Production of ARAN Collection List	2-8

2.05	ARAN Collection List QA	2-10

2.06	Production of Skid Collection List	2-12

2.07	Section Version History	2-14

3	Field Data Collection	3-1

3.01	Introduction	3-1

3.02	ARAN Pre-Data Collection Actions	3-3

3.03	ARAN Data Collection	3-17

3.04	ARAN Post-Data Collection Actions	3-23

3.05	ARAN DMI Calibration	3-35

3.06	Preventive Maintenance of ARAN Van	3-49

3.07	Test Loop ARAN Data Collection	3-51

3.08	Test Loop Data Analysis	3-56

3.09	Skid Pre-Data Collection Actions	3-71

3.10	Skid Data Collection	3-75

3.11	Skid Post-Data Collection Actions	3-79

3.12	Skid DMI Calibration	3-82

3.13	Skid Water Calibration	3-90

3.14	Skid Force Calibration	3-94

3.15	Skid Test Loop Data Collection	3-102

3.16	Section Version History	3-105

4	Construction Data	4-1

4.01	Introduction	4-1

4.02	Processing Paving & Maintenance History Data	4-3

4.03	Processing Access Permit Data	4-20

4.04	Processing As Bid Unit Cost Data	4-22

4.05	Processing As Built Unit Cost Data	4-44

4.06	Reviewing Projects in MCMS	4-67

4.07	Running Transport Application	4-71

4.08	Looking Up Fund Type	4-74

4.09	Making Lane Configuration Changes	4-76

4.10	Adding to Project Engineer List	4-86

4.11	Viewing Paving & Maintenance History by Route	4-88

4.12	Promoting As Bid to Construction History	4-91

4.13	Generating As Built Report	4-93

4.14	HMA Tonnage Application	4-95

4.15	KPA Reports	4-109

4.16	Section Version History	4-114

5	Data Processing	5-1

5.01	Introduction	5-1

5.02	Loading of ARAN Data into Vision	5-3

5.03	ARAN Submittal QA	5-25

5.04	Post-Processing of ARAN GPS Data	5-37

5.05	Running Global LCMS Processor	5-59

5.06	Running Roughness Processor	5-72

5.07	Route Matching	5-81

5.08	Performing 100% Drive Through	5-132

5.09	Running Rut Processor	5-153

5.10	Running Classification and Rating Processors	5-162

5.11	Running Vibing Processor	5-171

5.12	Running Thumbnail Creator	5-179

5.13	Running Curve Fit Batch Processor	5-187

5.14	Reporting and Uploading Data to Oracle	5-196

5.15	Reformatting ARAN Hard Drives	5-210

5.16	Creating PAGDQC Report	5-214

5.17	Running Routing Importer	5-228

5.18	Changing Latitude/Longitude	5-235

5.19	Accessing Electronic ARAN Logs	5-240

5.20	Using eGIS	5-244

5.21	Changing Image Banner or Logo	5-250

5.22	Year-End ARAN Data Closeout	5-253

5.23	Route Matching - Transfers	5-266

5.24	Loading Pavement Surface Friction Data	5-278

5.25	Skid Submittal QC	5-284

5.26	Section Version History	5-287

6	Data Migration	6-1

6.01	Introduction	6-1

6.02	Outlier Review	6-3

6.03	Update Final Condition Detail Table	6-15

6.04	Update Rut Detailed Table	6-17

6.05	Update Cracking Detailed Table	6-19

6.06	Update Concrete Detailed Table	6-22

6.07	Migrating to Business Plan Tables	6-24

6.08	Section Version History	6-27

7	Friction Management	7-1

7.01	Introduction	7-1

7.02	Development/Review of Friction Site and Demand Categories	7-2

7.03	Network Definition and Segmentation	7-11

7.04	Section Site and Demand Category Analysis	7-15

7.05	Data Aggregation	7-17

7.06	Section Prioritization	7-21

7.07	Generating Reports	7-25

7.08	Future Improvements to the PFM Program	7-28

7.09	Section Version History	7-30

8	HPMS Data Reporting	8-1

8.01	Introduction	8-1

8.02	Production of HPMS Full-Extent Table	8-2

8.03	Production of HPMS Sample Table	8-6

8.04	Production of HPMS Pavement Metadata file	8-17

8.05	Production of HPMS Submission Comments	8-21

8.06	QA of HPMS Tables	8-25

8.07	Process PM2 Dataset	8-28

8.08	NHS Pavement Condition Dataset	8-46

8.09	Section Version History	8-66

9	Trend Analysis	9-1

9.01	Introduction	9-1

9.02	Updating IRI Performance Model	9-2

9.03	Updating Cracking Performance Models	9-5

9.04	Updating Rutting Performance Model	9-9

9.05	Updating Faulting Performance Model	9-13

9.06	Updating Friction Performance Model	9-16

9.07	Section Version History	9-21

10	Optimization	10-1

10.01	Introduction	10-1

10.02	Defining Network and Importing Data into the RoadCare Optimization Process	10-2

10.03	Network Segmentation and Data Rollup	10-9

10.04	Needs Analysis and Running Simulation Engine	10-20

10.05	Generating Reports	10-37

10.06	Section Version History	10-51

11	Appendices	11-2

11.01	Skid Collection List SQL Statement	11-2

11.02	Test Loop Data Analysis Details	11-5

11.03	Fund Type	11-10

11.04	POSPac List Files Macro	11-12

11.05	Base Report	11-14

11.06	IRI Speed Adjustment	11-68

11.07	Examples for Changing Latitude/Longitude	11-90

11.08	Outlier Review - Outlier Codes	11-93

11.09	IRI Performance Model	11-97

11.10	Cracking Performance Models	11-101

11.11	Rutting Performance Model	11-142

11.12	Friction Performance Model	11-155

11.13	Optimization Data Rollup Methods	11-159

11.14	Pavement Friction Management Program SQL Packages	11-168

11.15	Section Version History	11-205

11.16	Document Change Log	11-206

LIST OF FIGURES

Figure 1: MDOT Organization Chart	1-2

Figure 2: Pavement Management Section Organization Chart	1-3

Figure 3: SHA PM Information Flowchart	1-6

Figure 4: SHA PM Guide SOPs Flowchart	1-7

Figure 5: Planning Flowchart	2-2

Figure 6: Field Data Collection Flowchart	3-2

Figure 7: Construction SOPs Flowchart	4-2

Figure 8: Data Processing SOPs Flowchart	5-2

Figure 9: Data Migration SOPs Flowchart	6-2

Figure 10: SHA Pavement Friction Management (PFM) program Flowchart	7-1

Figure 11: HPMS Data Reporting SOPs Flowchart	8-1

Figure 12: Trend Analysis and Modeling SOPs Flowchart	9-1

Figure 13: Fund 77 Optimization Flowchart	10-1

LIST OF ABBREVIATIONS

| Abbreviation | Definition |
| --- | --- |
| ADA | Americans with Disabilities Act |
| ADC | Assistant Division Chief |
| API | Application Programming Interface |
| ARAN | Automatic Road Analyzer |
| BMP | Beginning Mile Point |
| CALs | Collectors/Arterials and Locals |
| CHART | Coordinated Highways Action Response Team |
| CI | Condition Index |
| CRCP | Continuously Reinforced Concrete Pavement |
| DAT | Data Analysis Team |
| DC | Division Chief |
| DCS | Data Control Software |
| DMI | Distance Measuring Instrument |
| DPT | Data Processing Team |
| EMP | Ending Mile Point |
| FCD | Functional Cracking Density |
| FED | Field Explorations Division |
| FHWA | Federal Highway Administration |
| FMIS | Financial Management Information System |
| GIS | Geographic Information System |
| GPS | Global Positioning System |
| GT | Ground Truth |
| HLR | Highway Location Reference |
| HMA | Hot Mix Asphalt |
| HPMS | Highway Performance Monitoring System |
| ICC | Inter County Connector |
| IFB | Invitation for Bid |
| IRI | International Roughness Index |
| JCP | Jointed Concrete Pavement |
| KPA | Key Performance Area |
| KPI | Key Performance Indicator |
| LCMS | Laser Crack Measurement System |
| LM | Lane Miles |
| LMY | Lane-Mile-Years |
| MCMS | Maryland Construction Management System |
| MDOT | Maryland Department of Transportation |
| MPD | Mean Profile Depth |
| MPO | Metropolitan Planning Organization |
| NHS | National Highway System |
| OMT | Office of Materials Technology |
| OPA | Other Principal Arterial |
| PAGD | Pavement and Geotechnical Division |
| PCC | Portland Cement Concrete |
| PDW | Pavement Data Warehouse |
| PM | Pavement Management |
| PMT | Project Management Tool |
| PSR | Present Serviceability Rating |
| PTT | Pavement Testing Team |
| PVT | Pavement |
| QA | Quality Assurance |
| QC | Quality Control |
| ROW | Right-of-Way |
| RSL | Remaining Service Life |
| SCD | Structural Cracking Density |
| SHA | State Highway Administration |
| SOP | Standard Operating Procedure |
| SPR | System Preservation Reports |
| SQL | Structured Query Language |
| SVR | Server |
| TL | Team Leader |
| TMDL | Total Maximum Daily Load |
| UTM | Universal Transverse Mercator |
| VMT | Vehicle Miles Traveled |

FOREWORD

The Pavement Management Guide was written to document the procedures followed by the pavement management engineers and technicians of the Maryland Department of Transportation (MDOT) State Highway Administration (SHA) Pavement Management (PM) Section for pavement data processing and analysis to support SHA’s System Preservation efforts.

All the position titles described throughout this guide are referenced to SHA specific positions and roles. As such, a current organizational chart for the PM Section was also included in the MDOT Organization in this guide to assist the readers with understanding the organizational structure within the PM Section.

A vast majority of the SHA’s pavement management functions are performed by engineers and technician staff in the PM Section of the Pavement and Geotechnical Division (PAGD) of the Office of Materials Technology (OMT). The Team Leaders and Assistant Division Chief are responsible for the quality and timeliness of all the pavement management functions for the Division. Other data collection divisions within OMT also support the pavement management functions of the Pavement Management Section.

SHA has adopted a policy of continuous improvement when managing data quality. Infrastructure condition measurements are used to support data-driven decision making which maximizes the return on public investment. As technological advancements are implemented, both current data and historic data are managed to provide both accuracy and consistency by storing and re-processing historic raw data within current protocols, to the extent feasible considering prior technology limitations. Data analysis procedures are continuously updated as and when more data becomes available, thereby facilitating improved decision making. This document is actively managed to reflect the latest pavement data collection, pavement data quality management and pavement data analysis procedures that are followed by SHA.

Any questions or comments concerning this guide should be directed to:

Robert Steudler
Assistant Division Chief for Pavement Management Section
Pavement and Geotechnical Division
Maryland Department of Transportation State Highway Administration
Office of Materials Technology
7450 Traffic Drive
Hanover, MD 21076
Phone: 443-572-5065
Fax: 410-787-0486
E-mail: rsteudler@mdot.maryland.gov

# Overview

## Introduction

This Maryland Department of Transportation (MDOT) State Highway Administration (SHA) Pavement Management (PM) Guide provides a comprehensive set of standard operating procedures (SOPs) designed to assist in conducting different processes involved in the collection, processing, analysis and reporting of PM data. These data directly support SHA’s pavement asset management program, including all data development, analysis, and any state or federal reporting efforts, including the Highway Performance Monitoring System (HPMS) deliverables, among other applications.

The set of SOPs contained in this guide includes procedures related to the production and submittal of data to the FHWA HPMS as well as other procedures related to the production of pavement management data to support the strategies and goals defined by SHA. This guide is a living document. The contents will be updated and new procedures will be added as they become available.

Each SOP document includes the following five elements: (i) general aspects of the SOP; (ii) frequency of the SOP; (iii) purpose of the SOP; (iv) resources required to perform the SOP; and (v) procedural steps to perform the SOP. The latter SOP element includes the Quality Control (QC) and Quality Assurance (QA) processes performed to check that the data collected and processed at each step have acceptable quality. The definition of QC and QA used in this document follow the definitions used internally by SHA staff: QC refers to the quality checks performed by the personnel involved in conducting the process whereas QA refers to the independent quality checks performed by personnel not involved in conducting the process.

The materials contained in the Guide are meant to be useful for both the training of new employees and as a reference to be utilized as needed throughout the course of work. This is a resource document. It is not intended to override or replace the necessary use of good judgment, common sense, and research of current best practices. The audience for whom the Guide was written should have a basic background and general understanding of pavement management, as well as knowledge of the technical skills required for the position and function of the specific SOP.

The following two sections of this Overview chapter provide general information on the MDOT divisions related to the SOPs included in this Guide, as well as the organization of the document and a description of the relationship among SOPs. The remaining chapters of the document contain the set of PM SOPs grouped by the PM process stage they belong to.

## MDOT Organization

The MDOT divisions relevant to the SOPs contained in this Guide are shown in Figure 1. SHA and the Maryland Transportation Authority (MDTA) are sub-divisions of the MDOT Secretary. The MDTA is responsible for the administration of toll facilities, among other responsibilities. SHA is responsible for the administration of the majority of high volume public highways in Maryland outside Baltimore City. SHA is also responsible for the collection, processing, analysis and reporting of pavement management data on sections administered by other offices, such as the MDTA’s toll roads.

The SOPs in this guide relate to both planning and operations work done by respective offices and divisions. The planning efforts which relate to highway inventory management and HPMS reporting are conducted by the Innovative Performance Planning Division (IPP), the Data Services Division (DSD), and the Data Governance Division (DGD). The IPP group is responsible for the submittal of the Data Quality Management Plan (DQMP) to meet requirements from item §490.319(c) of the FHWA’s third final rule 06/12/2019 for national performance management measure regulations. The DSD is responsible for the highway inventory management and the annual submittal of the HPMS data to the FHWA. The DGD is responsible for hosting and managing several of the databases referenced in the SOPs of this document. The Highway Development group is responsible for the development of major projects. Within the Operations branch of SHA, the Office of Materials Technology (OMT) is responsible for the field data collection efforts, pavement management, and pavement design. The other offices within the Operations branch provide more tangible components of asset management, including project development, construction and maintenance.

See Figure 1 for an overview of MDOT organization.

Figure 1: MDOT Organization Chart

As noted in the organizational breakdown in the previous page, the PM section of SHA is a sub-group of the Pavement and Geotechnical Division (PAGD), located at the OMT. The PM section of the PAGD, along with the Field Explorations Division (FED), handle many pavement data collection activities, all pavement data processing and analysis responsibilities, and several reporting functions which support SHA business plan goals and performance measures.

The Field Explorations Division (FED) is responsible for all pavement field data collection activities. Along with daily data collection, the FED handles equipment calibration, validation, verification, maintenance, and QC/QA of the collected data. The procedures contained in this version of the Guide pertain to data collection activities relating to the Automatic Road Analyzer (ARAN) and skid trailers.

The PM section of the PAGD consists of several teams with defined roles in the pavement management process. Figure 2, shown below, summarizes the various teams and their place in the PM section of the PAGD.

Figure 2: Pavement Management Section Organization Chart

The following list provides a brief description of the roles and responsibilities of the different PM teams, focusing on the processes included in the SOPs of this version of the Guide:

- The Data Processing Team (DPT) is responsible for the processing, updating, managing, developing, and QC/QA of construction data and of the condition data collected in the field by the FED.

- The Data Warehouse Team (DWT) is mainly responsible for the management of PM databases and the integration of data with databases administered by others. It provides support to other PM teams to facilitate the production and processing, quality control, analysis, and reporting of data.

- The Data Analysis Team (DAT) is mainly responsible for the analysis of pavement management data. This includes the projection of the pavement network condition, the optimization of maintenance and rehabilitation strategies, as well as the reporting of pavement management data, including State-wide public reports and state reporting and subsequent federal reporting by planning divisions. The DAT also provides protocol development, outlier review, and any re-processing of historic data as new protocols emerge.

## SHA PM Guide Organization

The Guide has several sections of interest, representing the many different stages of PM. These include the following:

- The section on Planning provides guidance on the annual planning and updating of inventory information for the list of sections to collect pavement surface condition data with the ARAN survey vans (ARAN Collection List) and skid trailers (Skid Collection List).

- The section on Field Data Collection provides guidance on field testing requirements, equipment calibrations, validation, verification, and maintenance, related to the collection of pavement surface condition data with the ARAN survey vans and skid trailers.

- The section on Construction Data provides guidance on the collection, importing and processing of construction data from various data sources necessary to support PM activities.

- The section on Data Processing provides guidance on the processing and management of the different data elements collected in the field by the ARAN survey vans and skid trailers.

- The section on Data Migration provides guidance on the updating of performance data tables and migration to various data formats.

- The section on Friction Management provides guidance on addressing pavement friction requirements across the SHA pavement network.

- The section on HPMS Data Reporting provides guidance on the updating, formatting, and reporting of data to the FHWA’s HPMS.

- The section on Trend Analysis provides guidance on the annual updating of pavement condition metric performance models to be used in the optimization of targeted projects, among other applications.

- The section on Optimization provides guidance for the entry of required inputs and generation of the yearly targeted projects list.

- Several Appendices are provided for reference.

Figure 3 and Figure 4 provide a summary of the processes, databases, elements, and procedures which make up the pavement management SOPs described in this document. Figure 3 is an overall summary of database production, beginning with the inventory list received from the DSD and ending with the generation of the optimized targeted projects list. The white trapezoids denote databases; the arrows between the databases indicate the flow of data throughout the PM process; and the colored boxes group the databases together by the stage in which they are produced or utilized (e.g., Planning stage).

Figure 4 is provided to visualize the order and stage of each of the SOPs included in this version of the Guide, as well as the relationship between the different SOPs. Each white circle indicates a stage of pavement management. The rectangles which branch off from the stages are individual SOPs developed for the completion of the particular stage. The dotted lines which surround various stages denote the frequency with which they are performed.

Figure 3: SHA PM Information Flowchart

Figure 4: SHA PM Guide SOPs Flowchart

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated MDOT organization chart and flowcharts. Added one-liner for Friction Management section |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Planning

Click to go to Update of HMIS Tables

Click to go to Production of Annual Section Table

Click to go to Production of ARAN Collection List

Click to go to ARAN Collection List QA

Click to go to Production of Skid Collection List

## Introduction

This section describes the set of standard operating procedures (SOP) used by staff from the Data Analysis Team (DAT) of the Pavement and Geotechnical Division (PAGD) at the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to produce the annual lists of highway segments in the network to be collected by the Field Explorations Division team. The SOPs included in this version of the guide focus on the production of the Automatic Road Analyzer (ARAN) Collection List and the Skid Collection List. The ARAN Collection List consists of the routes on which data are to be collected using the ARAN survey vans for the upcoming year. The Skid Collection List consists of the routes on which data are to be collected using the skid trailers for the upcoming year.

The first step in the planning process is to update the Highway Management Information System (HMIS) tables using the annual inventory data provided by the Data Services Division (DSD). The next stage in the process consists of processing the DSD inventory data to create the ARAN Collection List. This stage includes three SOPs: the production of the Section Table, the production of the Collection List, and, lastly, the QA checks of the annual ARAN Collection List. Following the finalizing of the ARAN Collection List, a procedure is followed to create the Skid Collection List. These tasks are to be completed before the commencement of the data collection season.

See Figure 5 for an overview of the planning process.

Figure 5: Planning Flowchart

## Update of HMIS Tables

### General

This section describes the SOP used by DAT staff to update the HMIS_UNIVERSE and HMIS_MASTER tables based on the Data Services Division’s (DSD) latest inventory data. This section also includes the quality control (QC) processes to check that the updated Highway Management Information System (HMIS) tables are correctly processed and stored. The procedure documented in this section is the first step of the Pavement Management (PM) planning stage.

### Frequency

The two HMIS tables in question are updated once a year, after the DSD releases its annual inventory list tables, which generally happens around June.

### Purpose

The purpose of this SOP is to populate the HMIS_UNIVERSE and HMIS_MASTER tables with data from the DSD’s annual inventory list. The two tables serve as the basis for the Pavement and Geotechnical Division (PAGD) annual inventory update and creation of the collection lists for the upcoming data collection season.

### Resource Requirements

The updating of HMIS tables involves two people: (1) a DAT member knowledgeable in database management, the two HMIS tables, and the DSD’s ARCHIVE tables and, (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the updating of the tables and perform the QC checks. These time estimates assume that no issues are detected during the QC checks. The actual level of effort required will increase if the number of issues to address during QC increases.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management | 1 | 2.0 |
| DAT Team Leader (TL) | Supervisor | 1 | AR 06/12/2019 |

### Procedure

The procedure to update the two HMIS tables using the Pavement Data Warehouse (PDW) program and perform QC is comprised of the following two tasks:

- updating of HMIS Tables, and

- performing QC of the updated tables.

1- Updating of HMIS Tables – performed by DAT member

Summary: This task uses the PDW program to update the HMIS_UNIVERSE and HMIS_MASTER tables.

- Start PDW program and in the main menu, navigate to Analysis > HMIS_Annual_Update. The HMIS Annual Update input box will appear on the screen (see below image). The Year field in the input box is defaulted to the current year minus one (each year the DSD releases inventory data corresponding to the previous year).

- From the drop-down menu list in the Please Select a Update field, select HMIS_UNIVERSE/MASTER. The detailed information required for updating the HMIS tables will then appear on the input box. This information includes the procedure name, the appropriate schema, and the tables to be updated (see below image).

- Using Oracle SQL Developer, check whether ARCHIVE.UNIV[YEAR] and ARCHIVE.MASTER[YEAR]06/12/2019 exist or not.

  - If they do exist, proceed to Step 4.

  - If they do not exist, stop and wait until the DSD is done creating its latest inventory data before updating the HMIS tables.

- Click the Perform Annual Update button of the input box. The procedure POPULATE_MAST_UNIV will be run automatically in the background.

  - If no errors occur during the importing of the DSD data, a message will appear on the screen indicating the completion of the procedure once it is done. If so, proceed to Step 6.

  - If errors occur during the importing of the DSD data, a message will appear on the screen indicating that the procedure cannot be completed. If so, proceed to Step 5.

- If errors occur during the importing of the DSD data in Step 4, then debug the POPULATE_MAST_UNIV procedure using the Oracle SQL Developer to find the location(s) in the code producing the error. Once issue(s) producing errors have been identified, modify the POPULATE_MAST_UNIV code accordingly, until the procedure can be run without errors.

2- Performing QC of Updated Tables – performed by DAT member

Summary: This task performs QC checks of the updated tables to correct any errors that may have occurred during the update.

- Once the HMIS tables are created, perform the following QC checks to confirm that the DSD inventory data for the latest year have been correctly populated into the HMIS_UNIVERSE_ALL_YEARS and HMIS_MASTER_ALL_YEARS tables.

  - Completeness check – Using Oracle SQL Developer, check that the number of records for the updated HMIS_UNIVERSE_ALL_YEARS and HMIS_MASTER_ALL_YEARS tables are similar to the number of records in the same tables for the previous year.

  - Validity check – Using Oracle SQL Developer, check that:

    - Values inputted in the updated HMIS tables are within the expected range for the field. For example, lane width values should consist of positive values not greater than 20 feet.

    - Amount of missing values is not excessive.

- If the updated HMIS tables pass the QC checks, then the tables are acceptable. Otherwise, perform a detailed inspection of the POPULATE_MAST_UNIV procedure using Oracle SQL Developer to identify and to resolve the issue(s) preventing successful completion of the QC checks in the previous step. Issues to look for include changes in the field names of the inventory list (with respect to the previous year’s list) and missing data elements. Once the cause(s) of the issue(s) has/have been identified, modify POPULATE_MAST_UNIV procedure accordingly and regenerate the HMIS tables. Repeat Step 6 and Step 7 until the HMIS tables generated pass all QC checks.

- Notify other members of the team that the HMIS_UNIVERSE_ALL_YEARS and the HMIS_MASTER_ALL_YEARS tables have been correctly updated.

## Production of Annual Section Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to produce and to quality control (QC) the annual Section Table. This table contains roadway inventory data for the entire Maryland Department of Transportation State Highway Administration (SHA) highway network, which are extracted from the Data Services Division (DSD) inventory data tables in the format and structure required to meet the Pavement and Geotechnical Division (PAGD) needs.

### Frequency

The Section Table is generated once per year, after the HMIS_UNIVERSE and HMIS_MASTER tables are updated with the annual DSD inventory data (see Update of HMIS Tables).

### Purpose

The purpose of this SOP is to generate the Section Table using the annual DSD inventory data, as contained in the HMIS UNIVERSE and MASTER tables. The Section Table serves as the primary information source for producing the Automatic Road Analyzer (ARAN) Collection List, which contains the complete set of sections to be surveyed in the current year.

### Resource Requirements

The production and QC of the Section Table involves two people: (1) a DAT member knowledgeable in database management and, (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the production of the Section Table and perform the QC checks. These time estimates assume that no issues are detected during the QC checks. The actual level of effort required will increase if the number of issues to address during QC increases.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management | 1 | 40.0 |
| DAT Team Leader (TL) | Supervisor | 1 | AR 06/12/2019 |

### Procedure

The procedure to produce and QC the Section Table is comprised of the following two tasks:

- producing the Section Table, and

- performing QC of the Section Table.

1- Producing the Section Table – performed by DAT member

Summary: This task uses Oracle SQL Developer to produce the Section Table from the updated HMIS Tables.

- Using the Oracle SQL Developer, verify that the HMIS_UNIVERSE AND HMIS_MASTER tables have been correctly updated with the annual DSD inventory data for the current year. If not, update the tables following the procedure described in Update of HMIS Tables.

- Run SECTION_TABLE_PROP_PKG package to produce the initial version of the Section Table.

2- Performing QC of the Section Table – performed by DAT member

Summary: This task performs QC checks of the created Section Table to correct any errors that may have occurred during the production by comparing mileage of the network and mileage of specific sections to the previous year.

- Total mileage check: using the Oracle SQL Developer, compare the total mileage of the initial Section Table version to the total mileage of the same table for the previous year.

  - If the difference in total mileage is not greater than 100 miles, then the Section Table generated for the current year is acceptable and no further checks are required – proceed to Step 5. The 100 miles allowance is intended to account for the addition of highway sections from one year to the next, minor processing errors, and other factors. This value represents less than 1% of the total highway network managed by the SHA.

  - Otherwise, proceed to Step 4.

- Individual route mileage check – using the Oracle SQL Developer, compare the mileage for the individual routes in the initial version of the Section Table to the mileage for the same individual routes in the Section Table for the previous year. The difference in mileage for each individual route should not exceed 10%. Otherwise, inspect the SECTION_TABLE_PROP_PKG package to identify issue(s) that may explain the large differences in mileage for the individual routes. Possible issues to look for when inspecting the code include the presence of duplicate records and incorrect mainline codes. The mainline code is used to classify the road mileage of the inventoried road into mainline, interchange ramps, service roads, couplet, and others. The two possible outcomes from the inspection are:

  - If no issues are detected in the code that explain the large difference in mileage for a specific route, look for additional information that may explain the difference. A possible source of information for this is the “Change Log” published by the DSD as part of their inventory lists. This log contains the differences in DSD’s inventory data with respect to the previous year’s list.

  - If issues are detected in the code, modify the code as appropriate until the difference in total mileage between the Section Table produced for the current year and the one for the previous year is less than 100 miles.

- Once the QC checks have been passed, name the final version of the resulting table as SECTION_TABLE and store it in the PAV_CONHIST user within the ASSET_NEW database.

## Production of ARAN Collection List

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to produce and to quality control (QC) the Automatic Road Analyzer (ARAN) Collection List. This list contains the routes on which data are to be collected using ARAN survey vans, as part of the annual field data collection season. The ARAN Collection List is produced from the current year’s Section Table (see Production of Annual Section Table).

### Frequency

The ARAN Collection List is produced annually, after the final version of the Section Table for the current year has been generated.

### Purpose

The purpose of this standard operating procedure (SOP) is to produce the ARAN Collection List from the current year’s Section Table generated using the Production of Annual Section Table. The list is used by the Field Explorations Division (FED) to plan their annual ARAN data collection.

### Resource Requirements

The production and QC of the ARAN Collection List involves two people: (1) a DAT member knowledgeable in database management and, (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the production of the ARAN Collection List and perform the QC checks. These time estimates assume that no issues are detected during the QC checks. The actual level of effort required will increase if the number of issues to address during QC increases.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management | 1 | 40.0 |
| DAT Team Leader (TL) | Supervisor | 1 | AR 06/12/2019 |

### Procedure

The procedure to produce and QC the ARAN Collection List is comprised of the following three tasks:

- producing the Initial ARAN Collection List,

- performing QC of the Initial ARAN Collection List, and

- replacing Initial Section Table GPS Data with ARAN GPS Data.

1- Producing the Initial ARAN Collection List – performed by DAT member

Summary: This task uses Oracle SQL Developer to produce the Initial ARAN Collection List from the Section Table.

- Using the Oracle SQL Developer, check that the Section Table for the current year exists in the PAV_CONHIST user in the ASSET_NEW database. If it does not, follow the procedure described in Production of Annual Section Table to produce the Section Table before starting the generation of the ARAN Collection List.

- Run the ARAN_COLLECTION_LIST_PKG package to produce an initial version of the ARAN Collection List. The procedures contained in this package define the location of the starting and ending mile-points for each network route by aggregating the sections within each route in the Section Table.

2- Performing QC of the Initial ARAN Collection List – performed by DAT member

Summary: This task performs QC checks of the created Initial ARAN Collection List to correct any errors that may have occurred during the production by comparing mileage of the network to the previous year.

- Once the initial version of the ARAN Collection List has been created, use the Oracle SQL Developer to compare the resulting total mileage to the mileage from the previous year’s ARAN Collection List.

  - If the difference in mileage is less than 100 miles, then the initial version of the ARAN Collection List is acceptable – proceed to Step 5.

  - Otherwise, proceed to Step 4.

- If the difference in total mileage between the current and last year’s ARAN Collection List is greater than 100 miles, inspect the ARAN_COLLECTION_LIST_PKG package to identify the issues that are causing the difference in mileage. Once the issues have been identified, modify the package as appropriate and generate another version of the ARAN Collection List. Apply the QC check described in the previous step to the new version of the list. Repeat this process until the resulting ARAN Collection List passes the QC check in Step 3.

3- Replacing Initial Section Table GPS Data with ARAN GPS Data – performed by DAT member

Summary: This task replaces initial Section Table GPS data with GPS data from the ARAN survey vehicles and results in the submittal of the final version of the ARAN Collection List.

- The inventory data for each highway section in the initial version of the ARAN Collection List are extracted from the Section Table, which, in turn, contains the inventory data released by the Data Services Division (DSD). The GPS data contained in the initial version of the list are replaced with GPS coordinates measured by the Maryland Department of Transportation State Highway Administration (SHA) ARAN survey vehicles and reviewed by the Data Processing Team (DPT) staff during the previous year’s data collection season06/12/2019. This step is performed by running the UGS_VISION_MATCHED_EXPORT_PKG package from the Oracle SQL Developer. Manual entry of GPS coordinates may also be required in this step for locations where the GPS data have been obtained through ad-hoc communications with the DPT staff based on the results of their ARAN data processing procedures.

- Submit the final version of the ARAN Collection List with updated GPS data to the personnel involved in the quality assurance (QA) of the ARAN Collection List.

## ARAN Collection List QA

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Processing Team (DPT) and the Data Analysis Team (DAT) to perform quality assurance (QA) of the Automatic Road Analyzer (ARAN) Collection List before the list is submitted to the Field Explorations Division (FED). The QA is applied to a version of the ARAN Collection List that incorporates Global Positioning System (GPS) data from the previous year’s ARAN data collection.

### Frequency

The QA of the ARAN Collection List is performed annually, once a version of the ARAN Collection List with revised GPS data has been produced.

### Purpose

The purpose of this SOP is to perform QA of the ARAN Collection List to confirm, prior to actual data collection, that the (1) routes on the list are feasible for data collection and (2) location and inventory information on the list is accurate and complete.

### Resource Requirements

The QA of the ARAN Collection List involves three people: (1) a DPT member knowledgeable in inventory data collection for the Maryland Department of Transportation State Highway Administration (SHA) highway network, (2) a DAT member knowledgeable in database management and, (3) a supervisor who, as required, provides guidance and decision-making. The inventory data expert’s (IDE) primary role is to identify possible errors in inventory data and to ensure that routes on the list are feasible for data collection. The database management expert’s (DBME) primary role is to produce updated versions of the ARAN Collection List based on the IDE’s review. The estimated effort levels in the table below represent the total time, in man-hours, to complete the QA of the ARAN Collection List. These time estimates assume that no issues are detected during the QA checks. The actual level of effort required will increase if the number of issues to address during QA increases.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Inventory Data Expert | 1 | 4.0 |
| DAT Staff | Database Management | 1 | 8.0 |
| DAT Team Leader (TL) | Supervisor | 1 | AR 06/12/2019 |

### Procedure

The procedure to perform QA of the ARAN Collection List is comprised of two tasks:

- checking feasibility and accuracy of data collection routes, and

- checking inclusion of revised GPS data.

1- Checking Feasibility and Accuracy of Data Collection Routes – performed by IDE from the DPT and DBME from the DAT

Summary: This task is performed to confirm that the routes defined by the ARAN Collection List are feasible for data collection and only homogeneous inventory data features are contained within the section limits. Another important element of this first check is confirming that the routes on the list can accommodate multiple collection runs when required06/12/2019.

- IDE: open initial version of ARAN Collection List. The list is received by IDE via email as a spreadsheet in MS Excel file format, and it is named “ARAN_Collection_[FISCAL_YEAR]_[DATE]”06/12/2019.

- IDE: using initial version of ARAN Collection List spreadsheet, search for systematic errors concerning the feasibility and accuracy of the data collection routes, especially at locations where systematic errors are often identified. If systematic errors are identified, report these errors to DBME for remedial action.

- DBME: modify ARAN_COLLECTION_LIST_PKG package to account for the systematic errors detected by the IDE (if any). Re-run the package and inspect the output list until the systematic errors detected by the IDE are no longer present. Re-submit updated version of the ARAN Collection List to the IDE.

- Repeat Step 1 to Step 3 until the IDE does not identify further inventory errors on the ARAN Collection List produced by the DBME.

2- Checking Inclusion of Revised GPS Data – performed by IDE from the DPT and DBME from the DAT

Summary: This task is performed to confirm that the updates made to the GPS data during last year’s processing of ARAN collected data (conducted following Production of ARAN Collection List) are incorporated into the present year’s ARAN Collection List.

- IDE: locate lists of revised GPS data that resulted from corrections performed to the ARAN Collection List coordinates during last year’s data processing.

- IDE: randomly select a location with revised GPS data and confirm that GPS coordinates in the initial version of the ARAN Collection List for the selected location match the coordinates on the revised GPS list. Typically, the IDE checks approximately 10 locations with revised GPS data. If the coordinates do not match, report discrepancy to DBME for remedial action.

- DBME: inspect UGS_VISION_MATCHED_EXPORT_PKG package and modify accordingly to incorporate revised GPS coordinates, which were not included in the initial version of the ARAN Collection List, as reported by the IDE. Re-run the package and inspect the GPS coordinates in the output list until the reported discrepancies identified by the IDE are no longer present. Re-submit updated version of the ARAN Collection List to the IDE.

- Repeat Step 5 to Step 7 until the IDE is able to confirm that the revised GPS data have been incorporated into the ARAN Collection List.

- Once the checks have been completed and the modifications required by the IDE have been addressed, submit final version of ARAN Collection List to the FED.

## Production of Skid Collection List

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to produce and to quality control (QC) the “Skid Collection List.” This list contains the routes on which skid data are to be collected using the two SHA owned skid trailers06/12/2019, as part of the field data collection program. Skid collection is performed on a two-year cycle, and therefore the list of routes is similar on alternating data collection years. The “Skid Collection List” is produced from the current year’s “ARAN Collection List” (see Production of ARAN Collection List).

### Frequency

The “Skid Collection List” is produced annually by the SHA DAT staff, after the final version of the ARAN Collection List for the current year has been generated.

### Purpose

The purpose of this SOP is to produce the “Skid Collection List” from the current year’s “ARAN Collection List” generated using Production of ARAN Collection List. The list is used by the Field Explorations Division (FED) of the SHA to plan their annual skid data collection program.

### Resource Requirements

The production and QC of the “Skid Collection List” involves the following two SHA staff members: (1) a DAT staff member knowledgeable in database management and, (2) a DAT supervisor – typically the DAT Team Leader (TL) – who, as required, provides guidance and decision-making. The estimated effort levels shown in the table below represent the total time, in man-hours, to complete the production of the “Skid Collection List” and to perform the QC checks. These time estimates assume that no issues are detected during the QC checks. The actual level of effort required will increase if issues are identified during the QC checks.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff Member | Database Management | 1 | 4.0 |
| DAT TL | Supervisor | 1 | As Required 06/12/2019 |

### Procedure

The procedure used to produce and to QC the “Skid Collection List” is comprised of the following two tasks:

- production of the “Skid Collection List,” and

- QC of the “Skid Collection List.”

1- Production of the “Skid Collection List” – performed by DAT staff member

Summary: This task uses Oracle SQL Developer to produce the “Skid Collection List” from the “ARAN Collection List.”

- Using Oracle SQL Developer, check that the “ARAN Collection List” for the current year exists. If it does not, follow the procedure described in Production of ARAN Collection List to produce the “ARAN Collection List” before starting the generation of the “Skid Collection List.”

- Using Oracle SQL Developer, run the statement provided in Skid Collection List SQL Statement to produce the “Skid Collection List.”

2- QC of the “Skid Collection List” – performed by DAT member

Summary: Under this task, QC checks of the “Skid Collection List” are performed to correct errors that may have occurred during the production. These checks are done by comparing the network mileage included in the current year’s list to the network mileage from two years ago. Since skid data collection is performed on a two-year cycle, the list of routes and hence network mileage is similar on alternating data collection years; i.e., every other year. For example, the mileage included in the “Skid Collection List” created in 2018 should be compared to the mileage included in the “Skid Collection List” created in 2016.

- Once the “Skid Collection List” has been created, use Oracle SQL Developer to compare the resulting total network mileage to the total network mileage from the “Skid Collection List” from two years ago.

  - If the difference in total mileage is less than 100 miles, then the “Skid Collection List” is acceptable, and the process is complete.

  - Otherwise, proceed to Step 4.

- If the difference in total mileage is greater than 100 miles,

  - Inspect the SQL statement to identify the issues that are causing the difference in mileage.

  - Once the issues have been identified, modify the statement as appropriate and generate another version of the Skid Collection List.

  - Apply the QC check described in the previous step to the new version of the list.

  - Repeat this process until the resulting Skid Collection List passes the QC check in Step 3.

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | Aditya R. | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and DWT with DAT |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated planning flowchart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Field Data Collection

Click to go to ARAN Pre-Data Collection Actions

Click to go to ARAN Data Collection

Click to go to ARAN Post-Data Collection Actions

Click to go to ARAN DMI Calibration

Click to go to ARAN Preventive Maintenance

Click to go to ARAN Test Loop Data Collection

Click to go to Test Loop Data Analysis

Click to go to Skid Pre-Data Collection Actions

Click to go to Skid Data Collection

Click to go to Skid Post-Data Collection Actions

Click to go to Skid DMI Calibration

Click to go to Skid Water Calibration

Click to go to Skid Force Calibration

Click to go to Skid Test Loop Data Collection

## Introduction

This section describes the set of standard operating procedures (SOP) conducted by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to perform field data collection activities and quality control (QC) and quality assurance (QA) of pavement data. The SOPs contained in this version of the guide focus on the calibration, validation and daily data collection activities for the Automatic Road Analyzer (ARAN) survey vans and skid trailers.

The daily data collection activities are divided into six SOPs: ARAN Pre-Data Collection Activities, ARAN Data Collection, ARAN Post-Data Collection Activities, Skid Pre-Collection Activities, Skid Data Collection, and Skid Post-Data Collection Activities. These SOPs describe the tasks required from the beginning of the data collection day to the final shut down of the survey van, and include all quality checks conducted throughout the different daily activities.

This section contains four SOPs related to the calibration and the validation of measurements of the ARAN survey vans. The first is a procedure for calibrating the ARAN’s Distance Measuring Instrument (DMI) once a month and involves the adjustment of the DMI measurements to bring their accuracy and precision to the target standard. The second SOP is a procedure for the activities related to the annual preventive maintenance activities on the ARAN vans. This is typically performed before the start of the data collection season in which all sub-components of the vehicles are checked and corrective actions are taken, if necessary. The third SOP involves the collection of test data with the ARAN survey van along a predetermined “test loop.” This is typically performed once every three weeks during the collection season to check that the quality of measurements is within acceptable standards. The fourth SOP outlines the checks performed on data collected on the test loop to validate the measuring equipment before and throughout the data collection season.

In addition, this section contains four SOPs related to the calibration and the validation of measurements of the skid trailers. The first is a procedure for calibrating the skid trailer’s Distance Measuring Instrument (DMI) once a month and involves the adjustment of the DMI measurements to bring their accuracy and precision to the target standard. The second and third SOPs are procedures for the calibration of water output and force measurements. The fourth SOP involves the collection of test data with the skid trailer along a predetermined “test loop.” This is typically performed once every three weeks during the collection season to check that the quality of measurements is within acceptable standards.

Figure 6 depicts a flowchart of the field data collection process.

Figure 6: Field Data Collection Flowchart

## ARAN Pre-Data Collection Actions

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members of the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Division (FED) to perform preliminary routing, safety, and diagnostic checks prior to daily data collection using an ARAN 9000 survey van with ARAN Collection System (ACS) software Version 2.306/12/2019 (hereafter referred to in this SOP as ARAN). This SOP is one of three to be utilized for daily data collection with the referenced ARAN. It is succeeded by ARAN Data Collection and then ARAN Post-Data Collection Actions. The pre-data collection actions covered in this SOP start from the daily route planning before data collection to the commencement of the first data collection run of the day. This procedure utilizes the ARAN Collection List (see Production of ARAN Collection List) for the current year.

### Frequency

This SOP is to be performed each day of ARAN data collection, prior to the first data collection run of the day. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to develop an efficient routing plan for the data collection and to check the ARAN van is safe to operate and functioning properly for data collection by:

- reducing the effect of environmental factors on the quality of collected data,

- conducting vehicle-related safety checks, and

- validating the functionality of the ARAN sub-components.

### Resource Requirements

The actions detailed in this SOP are to be performed by the crew of the ARAN van, which is composed of a driver and an operator. The estimated effort levels in the table below represent the total time, in man-hours, required for completion of the daily planning, safety checks, and diagnostics of sub-components. These time estimates assume adequate weather conditions and that the diagnostic checks do not produce results that require additional troubleshooting.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 1.5 |
| ARAN Testing Crew | Operator | 1 | 1.5 |
| FED TL or Coordinator | Supervisor | 1 | AR 06/12/2019 |

### Procedure

The procedure to be followed by the ARAN crew members before starting their data collection runs for the day is comprised of three tasks:

- daily planning of data collection routes,

- vehicle and safety checks, and

- daily diagnostic checks of the ARAN sub-components.

1- Route Planning – performed by driver and operator, with input from Supervisor

Summary: This task creates a routing outline plan for the daily data collection and assesses conditions to determine whether data collection should proceed or be suspended.

- Develop an initial routing plan outline for the day using the selected sections from the ARAN Collection List (see Production of ARAN Collection List) to be collected within the county and a map of the county06/12/2019.

  - The route is selected based on several logistic and practical factors such as proximity to office, proximity to ARAN van storage location06/12/2019, proximity to gas stations, and others, with the objective of maximizing the collected mileage for the day.

  - The starting location for the day (first test run) is selected to minimize the impact of sunlight interference with the ARAN’s forward facing camera06/12/2019.

- Assess weather conditions for the day in order to decide whether to perform or to cancel data collection. Typical weather factors affecting this decision are:

  - Fog, rain, snow, or ice – ARAN data collection must be cancelled if there is fog, rain, snow, or ice regardless of intensity06/12/2019.

  - Surface wetness or ice – If it is not raining or snowing, but the pavement surface is determined to be wet or icy after a visual inspection, data collection is delayed until the surface is dry06/12/2019.

  - Excessive winds – If winds are determined to be strong enough to affect measurements06/12/2019, data collection is delayed until conditions subside.

If weather considerations do not result in the cancelation or delay of data collection, proceed to Step 3.

2- Vehicle and Safety Checks – performed by driver and operator

Summary: This task performs vehicle-related safety assessments to check that the ARAN is in proper working order and safe to operate.

- Driver: Turn on ARAN van ignition.

- Driver: Ensure mirrors (left, right, and rear view) and seat position are properly set for safe driving.

- Operator: Clean windshield. Remove dirt or visual obstructions using windshield cleaner so that right-of-way (ROW) images are clear.

- Operator: Check tire pressure of all tires, including inner tires from rear axis. If pressure does not read 65 psi, adjust air (release or fill) until 65 psi is achieved on all tires06/12/2019.

- Operator: Clean inertial profilers, Laser Crack Measurement Systems’ (LCMS), and grade sensors’ lenses using a dry, soft rag06/12/2019.

- Driver: Check that all ARAN van lights are functioning properly. This includes beacons, hazards, turning signals, and brake lights. If any lights or beacons are not functioning, communicate issue to Supervisor for further remedial action and suspend data collection until the issue has been resolved.

- Driver: Test that back-up alarm is functioning properly by placing vehicle in reverse and moving slightly. If the backup alarm is not functioning, communicate issue to Supervisor for further remedial action and suspend data collection until the issue has been resolved.

3- Diagnostic Sub-Component Checks – performed by driver and operator

Summary: This task runs diagnostics on the various ARAN sub-components to check that they are functioning properly for data collection.

- Operator: Turn on power inverters using control panel on the center console and check that the inverter reading in the back of the van reads 120 +/- 3 V. If inverter reading is within tolerance, proceed to Step 11. Otherwise, turn inverter off and back on again and check new inverter’s reading. If reading is still incorrect, battery level may not be adequate for data collection and further troubleshooting is required. If this occurs, communicate issue to Supervisor and suspend data collection until the issue has been solved. Please refer to the ARAN 9000 Operation Guide for further information regarding this step.

- Operator: Power up main computer by turning the system start switch (either front control or on the AC power box) to the third position clockwise and release. Start switch will return to the second position on its own. Wait until all computers have started Windows operating system and login using proper sign-in credentials.

- Operator: Open the ACS software by double clicking on the icon “ARAN Collection System” (see image below with icon circled in red) on the ROW desktop06/12/2019.

- Operator: Once the ACS software is open, check that the different ARAN sub-systems are correctly connected by observing the icon lights on the right column of the window (see image below). If the sub-systems are correctly connected, all of the icon lights should turn green after a few seconds.

  - If all icons turn green, proceed to Step 14.

  - If one or more icon lights remain red (as shown in the image below), identify which sub-system is not properly connected and attempt to troubleshoot issue. For example, operator should reconnect the cables connecting the sub-system equipment to the computer, as a first step. If Operator is not able to solve the issue, he/she should contact Roadware’s Technical Service for assistance. If still not able to correct issue, communicate issue to the Supervisor and suspend daily data collection until all sub-systems are properly connected.

- Operator: Click on the large “Power” button in the upper-left corner of the window (see above image). ACS main screen appears (see below image).

- Operator: Login using proper sign-in credentials.

- Operator: Perform Bounce Test. Make sure the vehicle is on a level, smooth surface (garage floor at SHA OMT is acceptable). Click on the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “Roughness”, then “Bounce Test.” The test screen will appear (see below image)06/12/2019.

- Operator: When ready, hit start. The progress bar on the “Bounce Test” window will begin to fill up from left to right.

- Operator: When progress bar reaches the beginning of the “Bounce” section, instruct the driver to “start bouncing.”

- Driver: Apply pressure to the front of the vehicle, causing it to bounce06/12/2019.

- Operator: When the progress bar reaches the end of the “Bounce” section, instruct the driver to “stop bouncing.”

- Driver: Step back from the vehicle and allow the test to complete with the vehicle stationary.

- Operator: Wait until the screen indicates that the test is complete. Click “Next.” Results screen will appear (see below image). There are four component results (left-static, right-static, left-bounce, right-bounce) that will either show as PASS or FAIL, then an overall PASS or FAIL rating will also be displayed.

  - If the overall result is PASS, click Finish and proceed to Step 23.

  - If the overall result is FAIL, click “Cancel” and return to Step 16. Repeat test up to 5 times until a passing result is achieved. If a passing result is still not achieved, communicate issue to the Supervisor and suspend daily data collection until issue is resolved.

- Driver: Drive the van from the bounce test location to an appropriate and safe location – such as the parking lot of the OMT building – to perform the dynamic diagnostic checks.

- Operator: Perform Grade Diagnostic Check. Click on the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “Grade.” Grade diagnostic window will appear.

- Driver: Begin driving around the test area.

- Operator: Click “Start.” Check diagnostic window.

  - All 4 grade sensors should be showing readings. Readings should change as ARAN van moves and should be within reasonable values according to the pavement surface being driven.

  - Pitch and roll sensors should also be showing readings. Readings should change as ARAN van moves and should be within reasonable values according to the pavement surface being driven.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 26 are acceptable, proceed to Step 27.

  - If checks in Step 26 were not acceptable, stop vehicle. Reconnect the cables connecting the sub-system equipment to the computer and restart test by returning to Step 24. If Operator is not able to resolve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve the issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform DMI Diagnostic Check. Return to the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “DMI.” DMI diagnostic window will appear (see below image).

- Driver: Continue driving around the same test area.

- Operator: Click “Start.” Check diagnostic window.

  - DMI Count should increase as vehicle moves.

  - Graph should update in real time.

  - Graph should have a positive slope when increasing speed and return to x-axis when vehicle stops.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 29 are acceptable, proceed to Step 30.

  - If checks in Step 29 are not acceptable, stop vehicle. Check DMI connection on wheel. Reconnect cables connecting the sub-system equipment to the computer and restart test by returning to Step 27. If Operator is not able to solve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform LCMS Diagnostic Check. Return to the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “LCMS.” LCMS diagnostic window will appear.

- Driver: Continue driving around the same test area.

- Operator: Click “Start.” Check diagnostic window.

  - All icon lights should be green.

  - Two images in the center of the window should be updating as the ARAN van moves.

  - Images should be representative of the pavement that is being driven over.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 32 are not acceptable, stop vehicle. Reconnect the cables connecting sub-system equipment to the computer and restart test by returning to Step 30. If Operator is not able to solve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform PS LV Diagnostic Check. Return to the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “POS LV.” POS LV diagnostic window will appear.

- Driver: Continue driving around the same test area. Ensure no buildings or trees are obstructing the GPS receiver.

- Operator: Click “Start.” Check diagnostic window.

  - Two GPS readings should be reasonable and updating as the vehicle moves. It may take a few moments for the GPS readings to display.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 35 are acceptable, proceed to Step 36.

  - If checks in Step 35 are not acceptable, stop vehicle. Reconnect the cables connecting sub-system equipment to the computer and restart test by returning to Step 33. If Operator is not able to solve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform Roughness Diagnostic Check. Return to the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “Roughness.” Roughness diagnostic window will appear.

- Driver: Continue driving around the same test area.

- Operator: Click “Start.” Check diagnostic window.

  - Four plots in total (2 laser, 2 accelerometer) should be updating as the ARAN van moves.

  - Laser readings should appear reasonable based on smoothness of test pavement.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 38 are acceptable, proceed to Step 39.

  - If checks in Step 38 are not acceptable, stop vehicle. Reconnect the cables connecting sub-system equipment to the computer and restart test by returning to Step 36. If Operator is not able to solve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform Video Diagnostic Check. Return to the “Diagnostics” drop down menu from the main ACS ribbon at the top of the screen. Select “Video.” Video diagnostic window will appear.

- Driver: Continue driving around the same test area.

- Operator: Click “Start.” Check diagnostic window. There is a tab for each video stream.

  - Each image should be clear.

  - Sunlight “wash-out” should be minimal.

  - Image should change as ARAN van moves.

- Operator: Select “Stop” at bottom of screen after checks are confirmed (should take approximately 1 minute).

  - If checks in Step 41 are acceptable, proceed to Step 42.

  - If checks in Step 41 are not acceptable, stop vehicle. Reconnect the cables connecting sub-system equipment to the computer and restart test by returning to Step 39. If Operator is not able to solve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve issue, communicate issue to the Supervisor and suspend daily data collection until sub-system is functioning properly.

- Operator: Perform dummy test run. Select the “Collect” drop down menu from the main ACS ribbon at the top of the screen. Click on “Summary.” Data collection window will appear (see below image).

- Operator: In the search bar above the map window, type in “DUMMY.” Click the magnifying glass to begin the search.

- Operator: A list of search results will appear. The section for the dummy test run will read “9999921 DUMMY, AA DUMMY E.” Click “Add to List.”

- Operator: In the queue list of selected sections, click on “DUMMY”. Click “Make Current” at the bottom of the map window.

- Operator: Confirm the beginning and end of the selected section by the flag icons in the map window.

- Operator: Follow ARAN Data Collection, beginning with Step 14, for completing the dummy data collection run.

- Operator: If the dummy data collection run does not result in issues, diagnostics are completed06/12/2019. If issues are discovered during dummy test run, troubleshooting of specific sub-component is necessary before beginning data collection.

## ARAN Data Collection

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members of the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Division (FED) to perform data collection using an ARAN 9000 survey van with ARAN Collection System (ACS) software Version 2.306/12/2019 (hereafter referred to in this SOP as ARAN). This SOP is one of three to be utilized for daily data collection with the referenced ARAN. It is preceded by ARAN Pre-Data Collection Actions and succeeded by ARAN Post-Data Collection Actions. The data collection activities covered in this SOP start from the end of the daily diagnostic checks and dummy data collection run (see ARAN Pre-Data Collection Actions) to the commencement of the daily data export (see ARAN Post-Data Collection Actions). This SOP describes the process of completing a full day of data collection. This procedure utilizes the ARAN Collection List (see Production of ARAN Collection List) for the current year. The ACS software should already be initialized from ARAN Pre-Data Collection Actions.

### Frequency

This SOP is to be performed daily for each day of ARAN data collection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to successfully daily data collection using the ARAN van and to perform quality control (QC) checks of the data while is being collected.

### Resource Requirements

The actions detailed in this SOP are to be performed by the crew of the ARAN van, which is composed of a driver and an operator. The estimated effort levels in the table below represent the total time, in man-hours, required for completion data collection. These time estimates assume adequate weather conditions and a typical data collection schedule. Actual time to complete individual runs in the field will vary between routes.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 7.0 |
| ARAN Testing Crew | Operator | 1 | 7.0 |

### Procedure

The procedure to be followed by the ARAN crew members for each unique data collection run is comprised of two tasks:

- route selection and

- data collection.

1- Route Selection – performed by operator

Summary: This task uses the ACS software to add sections to the data collection queue.

- Operator: There are two options for selecting daily routes for data collection based on the routing plan completed in ARAN Pre-Data Collection Actions in the ACS:

  - Browsing the populated list under “Mission Management.” If this is the preferred method, select the “Mission Management” drop down menu from the main ACS ribbon at the top of the screen. Click on “Routed Segments.” The list of sections that have already been imported into the system will appear. Proceed to Step 2.

  - Searching for a specific route in the “Collect” window. If this is the preferred method, skip to Step 3.

- Operator: Find the section for which data are to be collected and select it so that the row is highlighted (see below image). Click “Add to List.” Multiple sections may be added to queue list, if desired06/12/2019.

- Operator: Select the “Collect” drop down menu from the main ACS ribbon at the top of the screen. Click on “Summary.” Data collection window appears (see below image). If section(s) was (were) selected using the “Mission Management” window, skip to Step 5.

- Operator: In the search bar above the map window (see above image), type in the section identifier of the route to be collected. A list of search results will appear. Select the appropriate section and click “Add to List.” Multiple sections may be added to queue list, if desired06/12/2019.

- Operator: In the queue list of selected sections, click on the section that is to be collected next. Click “Make Current” at the bottom of the map window (see above image). Confirm the beginning and end of the selected section by the flag icons in the map window. Otherwise, the incorrect section may be selected and the route selection will have to be repeated by returning to Step 1.

2- Data Collection – performed by driver and operator

Summary: This task uses the ACS software to complete a data collection run.

- Driver: Navigate vehicle to selected first route and approach beginning of test section. Throughout collection run, adhere to local traffic laws. Attempt to maintain a steady path in the driving lane06/12/2019.

- Operator: Click “Start” at the bottom of the screen prior to reaching the beginning of the test section. System will begin to initialize06/12/2019. A green progress bar will begin filling up at the bottom of the screen (see below image). System is initialized and lead-in is complete when bar is completely green.

- Operator: When van reaches beginning of section, click “Begin Segment.” The image below depicts a flowchart for the data collection process.

- Operator: Perform Manual/Visual QC of data being collected through information displayed in monitor windows during data collection. Visual checks include the following06/12/2019:

  - Forward cameras – Right-of-way (ROW) camera window and Left camera window. Confirm that the images are crisp, clear and void of obstructions, that there is minimal sunlight interference, and that the images are updating as the vehicle moves down the road.

  - LCMS pavement images – Confirm that both sets of pavement images are updating as the vehicle moves down the road, and that identifiers on the pavement (such as paint stripes or large cracks) are visible in the image window.

  - International Roughness Index (IRI) values – Confirm that left and right IRI plots show reasonable numbers based on operator’s experience and condition of the pavement being collected.

  - Grade – Confirm % grade change window shows a reasonable value and it is updating as the vehicle moves down the road.

  - GPS – Confirm Position and Orientation System for Land Vehicles (POS LV) status is green.

  - Fault Count – Confirm fault count windows shows reasonable values and are updating as the vehicle moves down the road.

  - Additional Notes – Notes regarding special events during data collection should be added where necessary by typing the appropriate event abbreviation into the dialogue box on the data collection screen. Common events and their abbreviations for data entry are shown in below table.

| Description | Event Abbreviation |
| --- | --- |
| Lane Exception | L |
| Milled Surface | M |
| Construction Zone | C |
| Rumble Strips in Lane | R |
| Wet Surface | W |
| Road Closed Ahead | X |
| Roundabout | O |
| Traffic Signal Red | T |
| New Pavement | N |
| Debris | D |

If no issues arise from the checks in this step, proceed to Step 10. If issues do arise, stop the data collection run and attempt to fix problem. For example, operator should reconnect the cables connecting malfunctioning sub-system equipment to the computer, as a first step. If Operator is not able to resolve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve the issue, communicate issue to the Supervisor and suspend daily data collection until all sub-systems are functioning properly. When issue has been corrected, return to Step 5.

- Operator: Observe Automated QC Checks. Error messages will appear in a window towards the bottom of the data collection screen. Messages highlighted in ORANGE are warnings but do not require the data collection run to be stopped (such as one GPS receiver going out for a moment). Messages highlighted in RED are alerts that require the data collection run to be stopped and the specific error corrected.

  - If no issues arise from the checks in this step, proceed to Step 11.

  - If issues do arise, stop the data collection run and attempt to fix problem. For example, operator should reconnect the cables connecting malfunctioning sub-system equipment to the computer, as a first step. If Operator is not able to resolve the issue, contact Roadware’s Technical Service for assistance. If still not able to resolve the issue, communicate issue to the Supervisor and suspend daily data collection until all sub-systems are functioning properly. When issue has been corrected, return to Step 1.

- Operator: If multiple segments exist along the section, click “Accept” to mark the end of a segment. This will end that segment and begin the next segment automatically. When van reaches the end of the entire section, click “End Segment.”

- Operator: Click “Stop” at the bottom of the screen once the data collection route is complete. Green status bar will appear and lead-out process will begin. Lead-out is complete when bar is completely green.

- Operator: In the queue list of selected sections, click on the section that is to be collected next. Click “Make Current” at the bottom of the map window (see above image). Confirm the beginning and end of the selected section by the flag icons in the map window. Otherwise, an incorrect section may be selected and the route selection will have to be repeated by returning to Step 1.

- Driver: Navigate vehicle to next selected route and approach beginning of test section. Return to Step 7 and repeat for all data collection runs for the day06/12/2019.

## ARAN Post-Data Collection Actions

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members of the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Division (FED) to perform data management and validation activities after daily data collection using an ARAN 9000 survey van with ARAN Collection System (ACS) software Version 2.306/12/2019 (hereafter referred to in this SOP as ARAN). This SOP is one of three to be utilized for daily data collection with the referenced ARAN. It is preceded by ARAN Pre-Data Collection Actions and then ARAN Data Collection. The post-data collection actions covered in this SOP start from the completion of the last data collection run of the day (see ARAN Data Collection) to system shut down.

### Frequency

This SOP is to be performed each day of ARAN data collection06/12/2019, following the last data collection run of the day.

### Purpose

The purpose of this SOP is to perform post-data collection quality control (QC) checks of the ARAN data collected each day (see ARAN Data Collection) and to safely store those data by:

- properly exporting the data,

- checking quality of imaging and roughness data through the review of different runs, and

- documenting the events of the day through a daily report, referred to as the end of day daily report.

### Resource Requirements

The actions detailed in this SOP are to be performed by the crew of the ARAN van, which is composed of a driver and an operator. The estimated effort levels in the table below represent the total time, in man-hours, required for completion of all post-data collection activities. These time estimates assume no issues are encountered with exporting and QC of data.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 0.5-1.0 |
| ARAN Testing Crew | Operator | 1 | 0.5-1.0 |
| FED TL or Coordinator | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to be followed by the ARAN crew members following completion of data collection runs for the day is comprised of five components:

- data export,

- run review,

- end of day daily report,

- pavement image QC, and

- computer shut down and hard drive removal06/12/2019.

1- Data Export – performed by Operator

Summary: This task uses the ACS software to export the collected data at the end of the day.

- Select the “Data Management” drop down menu from the main ACS ribbon at the top of the screen. Select “Export.”

- Select today’s date from window. Ensure “Entire Day” bubble is filled in at top of window. All runs for the day should appear in the dialogue box (see below image).

- Select “Start” (see above image). Progress bar appears as data exports (see below image).

- Small notification window will appear that says “Export Completed.” Click “OK.”

2- Run Review and ROW Imaging QC – performed by Operator

Summary: This task uses the ACS software to review a sample of the collected data and QC the collected images.

- Select the “Data Validation” drop down menu from the main ACS ribbon at the top of the screen. Select “Review.” Review window appears (see below image).

- Select “Change Collection” at top left of window (see above image). Window appears with a list of runs completed for the date selected (see below image). Randomly select a specific run to be reviewed by highlighting the row of the corresponding section06/12/2019.

- Click “OK” (see above image).

- Click “Play.” Software will simulate the data collection of the selected section (see below image). Operator should observe the following06/12/2019:

  - Route map – Confirm by visual check that map in the upper left corner is showing the correct “trace” of the data collection run, which indicates the GPS was working properly.

  - Attribute Table – Confirm by visual check that the table in the bottom right corner is populated without gaps06/12/2019 and that the values for the corresponding attributes are reasonable06/12/2019.

  - Cameras – Confirm via visual check that the camera images in the upper right corner are updating and that they are of acceptable quality. Toggle between the two cameras to perform visual check on both.

Exit the review if the data collection run is complete and the checks have been found to be satisfactory. If issues are noted during the visual checks in Step 8, return to Step 5 and repeat the steps again, but select a different run. If the same issues are identified over multiple runs, communicate issue with Supervisor who will help decide if further action is required, such as recollection of data.

3- End of Day Daily Report – performed by Operator

Summary: This task uses the ACS software to produce a daily report.

- Select the “Data Validation” drop down menu from the main ACS ribbon at the top of the screen. Select “End of Day Daily Report.” “Daily Report” window appears (see below image).

- Check appropriate bubbles next to section identifiers (see above image). Designations are as follows:

  - C = Regular collection

  - X = Eliminated data or dummy file

  - R = Recollected data

  - V = Validation data (test loop)

- Add road names under the “Comments” column for each section.

- Select “Output Directory” (F Drive).

- Confirm all report types have been checked. Click “Generate.” Progress bar will appear as report is generated (see below image).

4- Pavement Image QC – performed by Operator

Summary: This task uses the LCMS Road Inspect software to review and QC collected LCMS pavement images.

- On the ROW computer desktop, double click on the “PAVE” computer icon. Monitor will switch to the PAVE computer desktop.

- Double-click on the “LCMS Road Inspect” icon (see below image).

- Click “Open” in upper right hand corner (see below image).

- Select F Drive.

- Select “FIS Pavement” icon.

- Scroll down to folder with today’s data and double click.

- Select random image in folder.

- Click “Clear All” on right of window.

- Click “Process.” A new window will appear when image processing is complete (see below image).

- Check image for quality. Right and left Laser Crack Measurement System (LCMS) images should line up well, image should be clear and pavement defects/markings should be visible. Otherwise, note the name of the section and communicate issue to Supervisor for further review of collected data and troubleshooting.

- Click “Save Result” and a new dialogue box will appear (see below image).

- Check all boxes in new dialogue box (see below image). Click “Save.”

- Click “Select Folder” (see above image). Navigate to “ROW PC”, “Removable.”

- Create new folder at this location and name it as “pavqc[MMDD]” using today’s date at the end (e.g. “pavqc1122”).

- Click “Open.”

- Click “Save.” Window disappears.

- Click “Save” on the “Process Result” window. This window will disappear.

- Return to Step 16 and repeat procedure for 2 more images in the same data folder06/12/2019. If one of the images reviewed identifies issues or raises concerns, it may be necessary to inspect additional images.

- Close Program.

- Click the “ROW” computer icon on desktop to return to ROW computer.

5- Computer Shut Down and Hard Drive Removal – performed by Operator

Summary: This task completes computer shut down in the ARAN and removes the hard drives from the vehicle for data processing.

- While back on the ROW computer, close the ACS.

- Click “OK” (see below image).

- Double-click on the shutdown icon for each computer on the ROW desktop (ROW, PAVE, Server).

- Turn off inverter using controls on the center console.

- Remove six hard drives from back of van and turn them in to supervisor06/12/2019.

- Confirm all computers and inverter have powered down successfully.

Turn off van engine and lock doors.

## ARAN DMI Calibration

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Divisions (FED) to calibrate the Distance Measuring Instrument (DMI) mounted on the ARAN vans. The steps described in this SOP are based on the ARAN Collection System (ACS) software Version 2.306/12/2019 developed for operating ARAN 9000 vans (hereafter referred to as ARAN). The reference measurements used for the calibrating procedure described in this SOP consist of two one-mile long pavement sections marked on the shoulder lanes along both directions of a road segment on MD Route 100, which have been measured by a state surveyor using thermally compensated steel tape.

### Frequency

This SOP should be followed for the calibration of the DMI mounted on ARAN vans once per month during the data collection period06/12/2019.

### Purpose

The purpose of this SOP is to check the accuracy of the distance measurements taken by the DMI mounted on ARAN vans and, if necessary, perform corrective actions for recalibration.

### Resource Requirements

Calibration of the ARAN’s DMI is performed by the crew of the ARAN van, composed of a driver and an operator. The estimated effort levels in the table below represent the total time, in man-hours, required for driving from the SHA’s Office of Materials Technology (OMT) location to the current calibration sites on MD 100, performing three repeat runs of the test, and driving back to the OMT location.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 1.5 |
| ARAN Testing Crew | Operator | 1 | 1.5 |

### Procedure

The procedure to calibrate the DMI mounted on ARAN 9000 vans using ACS software Version 2.3 is comprised of a single task:

- calibrating DMI.

1- Calibrating DMI – performed by driver and operator

Summary: This task performs a DMI calibration for the ARAN 9000 van using ACS software at the SHA OMT DMI calibration site.

The two calibration sites consist of one-mile long marked sections located near the OMT on the shoulder lanes of MD 100: one in the east-bound direction and the other in the west-bound direction (see map screenshot below).

- Driver: Drive the ARAN van to the starting point of the calibration site in the east-bound direction. The starting point can be identified by multiple markings (see the following photos):

  - an orange paint mark on a tree to the right of the road,

  - a metal pole just beyond the pavement shoulder,

  - paint markings on the pavement shoulder (line and “ST”), and

  - a survey nail in the pavement shoulder.

- Operator: Select the “Calibration” drop down menu from the main ACS ribbon at the top of the screen. Select “DMI.” This will open the ACS’s “DMI Calibration Wizard” application (see below image).

- Driver: Drive the ARAN van until the front bar of van is aligned to the “start” position marking on pavement surface and survey nail as shown in below image06/12/2019.

- Operator: Enter calibration site length on “DMI Calibration Wizard” (see below image). The length of the current calibration site is 1000 mmi.

- Operator: Click “Next” on the DMI Calibration Wizard to start calibration.

- Driver: Drive forward on shoulder lane from the start mark until a few feet before the end mark. The “DMI Calibration Wizard” should register travelled distance on screen (see below image). Attempt to stay as straight as possible in the shoulder lane using the white stripe, edge of pavement, and guardrail (where present) as references.

- Driver: Drive ARAN van until front bar of van is aligned to “end” position marking on pavement surface and survey nail as shown in the below image06/12/2019.

- Operator: Click “Next” to end the DMI calibration (see below image).

- Operator: Click “Finish” on ACS’s “DMI Calibration Wizard” and save results (see below image). The ACS’s DMI Calibration application will perform the necessary adjustments to the DMI readings to match the measured distance during the calibration procedure to the reference measurement (scale factor).

- Driver: Drive the ARAN van to starting point of the calibration site in the west-bound direction and repeat Step 2 to Step 9 to perform second run of DMI calibration. See photos below for similar markers in the west-bound direction.

- Driver: Drive the ARAN van to starting point of the calibration site in the east-bound direction and repeat Step 2 to Step 9 to perform third run of DMI calibration.

The DMI calibration procedure is completed once the three runs (two at the calibration site in the east-bound direction and one at the calibration site in the west-bound direction) are performed. The calibration factors for the 3 runs must agree within 0.1 percent and preferably within 0.05 percent. If they do not agree, consult Supervisor and discuss next steps for remedial actions.

## Preventive Maintenance of ARAN Van

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Divisions (FED) to support and to document the preventive maintenance activities performed annually by Fugro Roadware (Roadware) on the ARAN van and its sub-components.

### Frequency

This SOP should be followed for the support and documentation of the annual ARAN preventive maintenance activities. Typically, these preventive maintenance activities are performed in early March, before commencement of the annual data collection season, and takes approximately one week.

### Purpose

The purpose of this procedure is to support and to document the preventive maintenance activities performed by Roadware on the ARAN vans, and, if necessary, to perform follow-up actions. Preventive maintenance is done annually to ensure the ARAN vans are functioning properly prior to the start of the data collection season.

### Resource Requirements

Preventive maintenance work on the ARAN and its documentation on a check-list are performed by a representative from Roadware. Support with the preventive maintenance work and follow-up activities are performed by the crew of the ARAN van, composed of a driver and an operator. The FED team leader is required for oversight and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, required for completion and documentation of the annual preventive maintenance activities on the ARAN van and its sub-components. Time estimates assume only minor issues, if any, are encountered during the conduct of the required work.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 40 |
| ARAN Testing Crew | Operator | 1 | 40 |
| FED Team Leader (TL) | Supervisor | 1 | 5 |

### Procedure

The procedure to support and document preventive maintenance activities of the ARAN van and its sub-components is comprised of a single task:

- supporting and documenting ARAN preventive maintenance.

1- Supporting and Documenting ARAN Preventive Maintenance – performed by driver and operator with support from Supervisor

Summary: This task provides support for ARAN Preventive Maintenance operations, documents results of the maintenance, and makes decisions regarding follow-up actions.

- ARAN Testing Crew: Provide technical support as requested by the Roadware technician during the conduct of the preventive maintenance activities.

- Supervisor: Obtain preventive maintenance report06/12/2019 from the Roadware technician at completion of the work. Refer to the beginning pages of the report for a checklist of preventive maintenance activities. Check the following items on the list to confirm their completion:

  - Server computer updates and maintenance.

  - Right-of-way (ROW) computer updates and maintenance.

  - Pavement computer updates and maintenance.

  - Alternating Current (AC) distribution box checks.

  - LSDP junction box enclosure checks.

  - Cables, racks, dog house, and miscellaneous checks.

  - Distance Measurement Instrument (DMI) checks.

  - Chassis, generator, inverter, and charger checks.

- Supervisor: Refer to the “Recommendations” section of the report. Note suggestions or outstanding issues noted by Roadware and decide if further follow-up actions are required.

- Supervisor: Refer to the “System Calibration” section of the report. Review the following calibrations and their results:

  - DMI calibration.

  - Roughness Calibration.

  - Static Roughness Test Results.

  - Static Block Test Results.

  - Roughness Bounce Test Results.

  - Reverse Runs to Verify Roll, Pitch, and Heading.

  - Frame Angle.

  - Static Grade Test.

  - Static Rock Test.

  - Grade Verification.

  - Test Loop 10 Repeat Runs06/12/2019 – Review metrics including IRI, cross-fall, rutting, etc.

Decide if follow-up remedial actions are required based on the results from the individual checks.

## Test Loop ARAN Data Collection

### General

This section describes the standard operating procedure (SOP) followed by the Automatic Road Analyzer (ARAN) crew members from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Division (FED) to perform data collection on a carefully selected designated test loop using an ARAN 9000 survey van with ARAN Collection System (ACS) software Version 2.306/12/2019 (hereafter referred to as ARAN). The data collection route is called the “test loop.” The test loop begins and ends near the Office of Materials Technology (OMT) location. It is approximately 13.1 miles in length and is divided into segments of varying length based on surface type, intersection locations, ramp locations, and other common road features. The route was selected specifically because it includes a variety of surface types and road textures. The map06/12/2019 below depicts the route for the test loop. The data collected on this test loop are used to validate all ARAN sub-component data.

The required preliminary actions and diagnostic checks outlined in ARAN Pre-Data Collection Actions should be completed prior to the beginning of ARAN data collection runs on the referenced test loop.

### Frequency

This SOP requires that three ARAN data collection runs be performed on the test loop once a month throughout the data collection period06/12/2019.

### Purpose

The purpose of this SOP is to perform ARAN data collection runs on the test loop06/12/2019.

### Test Loop Map

### Resource Requirements

The test loop data collection is performed by the crew of the ARAN van, composed of a driver and an operator. The estimated effort levels in the table below represent the total time, in man-hours, required for completion of all test loop activities, including preliminary checks and diagnostics, three data collection runs, and data export and management. These time estimates assume no issues are encountered during the required work.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ARAN Testing Crew | Driver | 1 | 2.0 |
| ARAN Testing Crew | Operator | 1 | 2.0 |
| FED Team Leader (TL) | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to complete data validation on the test loop is comprised of a single task:

- completing data validation on ARAN Test Loop.

1- Completing Data Validation on ARAN Test Loop – performed by driver and operator

Summary: This task is performed to complete three runs on the ARAN test loop.

- Operator: Follow ARAN Pre-Data Collection Actions, beginning with Step 3, for vehicle and diagnostic checks.

  - If checks do not identify equipment issues, proceed to Step 2.

  - If checks identify equipment issues, consult ARAN Pre-Data Collection Actions for troubleshooting methods. Communicate issue(s) to the Supervisor and suspend test loop data collection until all sub-systems are functioning properly.

- Operator: Select the “Collect” drop down menu from the main ACS ribbon at the top of the screen. Click on “Summary.” Data collection window appears (see below image).

- Operator: In the search bar above the map window, type in “TL 1”. Click the magnifying glass to begin the search.

- Operator: A list of search results will appear. The section for the first test loop will read “999911 Test Loop 1, AA TL 1 E”. Click “Add to List.” Repeat search for “TL 2” and “TL 3”. All three test loop runs should now be listed in the section queue to the left of the map (see above image).

- Operator: In the queue list of selected sections, click on “Test Loop 1.” Click “Make Current” at the bottom of the map window (see above image). Confirm the beginning and end of the selected section by the flag icons in the map window.

- Driver and Operator: Follow ARAN Data Collection, beginning with Step 14, for completing the first test loop data collection run.

  - If run is completed successfully, proceed to Step 7.

  - If run is not completed successfully or if one or more sub-systems malfunctioned during data collection, attempt to resolve the issue(s). If Operator is not able to resolve the issue(s), contact Roadware’s Technical Service for assistance. If still not able to resolve the issue(s), communicate issue(s) to the Supervisor and suspend test loop data collection activities until all sub-systems are again functioning properly. When resolved, return to Step 5 and recollect first test loop run.

- Operator: In the queue list of selected sections, click on “Test Loop 2”. Click “Make Current” at the bottom of the map window. Confirm the beginning and end of the selected section by the flag icons in the map window.

- Driver and Operator: Follow ARAN Data Collection, beginning with Step 14, for completing the second test loop data collection run.

  - If is completed successfully, proceed to Step 9.

  - If run is not completed successfully or if one or more sub-systems malfunctioned during data collection, attempt to resolve the issue(s). If Operator is not able to resolve the issue(s), contact Roadware’s Technical Service for assistance. If still not able to resolve the issue(s), communicate issue(s) to the Supervisor and suspend test loop data collection activities until all sub-systems are again functioning properly. When resolved, return to Step 7 and recollect second test loop run.

- Operator: In the queue list of selected sections, click on “Test Loop 3”. Click Make Current at the bottom of the map window. Confirm the beginning and end of the selected section by the flag icons in the map window.

- Driver and Operator: Follow ARAN Data Collection, beginning with Step 14, for completing the third test loop data collection run.

  - If run is completed successfully, proceed to Step 11.

  - If run is not completed successfully or if one or more sub-systems malfunctioned during data collection, attempt to resolve the issue(s). If Operator is not able to resolve the issue(s), contact Roadware’s Technical Service for assistance. If still not able to resolve the issue(s), communicate issue(s) to the Supervisor and suspend test loop data collection activities until all sub-systems are again functioning properly. When resolved, return to Step 9 and recollect third test loop run.

- Operator: Follow ARAN Post-Data Collection Actions, beginning with Step 1, for data export and management following the completion of the third test loop run06/12/2019.

## Test Loop Data Analysis

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED), Data Processing Team (DPT), and Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to help assure the Automatic Road Analyzer (ARAN) quality controlled (QC’d) and quality assured (QA’d) deliverables (pavement data and imagery) are precise, accurate, and consistent throughout the collection season.

The SHA test loop is a collection of 45 sections creating an approximately 13.1 miles long run near the Office of Materials Technology (OMT) location. The test loop is divided into 45 segments of varying length according to surface type, construction history, intersection locations, ramp locations, number of lanes, other common road features, and historic ground truth study locations. The route was selected specifically because it includes a variety of pavement structures, surface types, and road textures. The data collected on the test loop are used to validate ARAN sub-component data. Details regarding the test loop ARAN data collection process is described in Test Loop ARAN Data Collection. Test Loop Analysis Details provides further information on the data analysis procedure.

### Frequency

This procedure detailed in this SOP is performed by the SHA staff once per year as part of the initial data collection program (pre-collection), where the SHA operated ARAN vehicles complete 10 runs of the SHA test loop prior to the start of the data collection season, and thereafter every 3 weeks throughout the FED collection and delivery season, as part of the 3 week test loop data collection program, where the ARAN vehicle completes 3 runs of the test loop.

### Purpose

The purpose of this SOP is to perform analysis on the SHA test loop data and imagery collected by the ARAN vehicles order to:

- confirm collected and processed data are precise, accurate, and consistent;

- determine the acceptable range of variance or standard deviation for each performance measure to be used for outlier review; and

- conduct research on topics such pavement data collection and data processing techniques, and pavement performance and deterioration modeling.

### Resource Requirements

The SHA test loop data analysis involves the following three SHA staff members: (1) a DPT staff member who is knowledgeable in the Vision software to perform pavement imagery quality control (QC) and lane adjustment, (2) a DAT staff member to perform condition data QC and to generate the test loop QC report, and (3) the DPT Team Leader (TL) to perform quality assurance (QA) functions and who, as required, provides guidance and decision-making. The FED TL and Assistant Division Chief (ADC) may also be required for error resolution and decision-making.

The estimated effort levels shown in the table below represent the total time, in man-hours, required for completion of SHA test loop data analysis. These time estimates assume no issues are encountered while performing the required work.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff Member | Imagery QC | 1 | 4.0 |
| DAT Staff Member | Condition Data QC | 1 | 4.0 |
| DPT TL | QA | 1 | As Required06/12/2019 |
| FED TL | Error Resolution | 1 | As Required |
| ADC | Error Resolution | 1 | As Required |

### Procedure

The procedure to perform the SHA test loop data analysis is comprised of the following four tasks:

- perform pavement imagery QC,

- perform drive-through for lane adjustment and pavement event flagging and generate test loop QC report,

- perform condition data QC and update test loop QC report, and

- submit test loop QC report.

1- Perform Pavement Imagery QC – performed by DPT Staff Member

Summary: Under this task, the pavement images collected by the ARAN on the SHA test loop are checked for correct aspect ratios, correct imaging intervals, and proper quality.

- After data have been loaded into Vision (see Loading of ARAN Data Vision) and LCMS Global Processor ran (see Running Global LCMS Processor), take measurements using the measurement tools in Vision (see below images).

- Compare measurements taken in Step 1 to the corresponding ground truth measurements to confirm that the images are not distorted (see markings in image below).

  - If images are within tolerance of +/- 3% of the actual measured length, proceed to Step 3.

  - Otherwise, refer to Test Loop Data Analysis Details for error resolution guidelines.

- Once the LCMS Global Processor has been run and the images created, check to confirm that the chainages have an interval of 0.004 miles (see chainage values in image below).

  - If the chainage values are at the correct interval, proceed to Step 4.

  - Otherwise, refer to Test Loop Data Analysis Details for error resolution guidelines.

- Visually check the pavement images to make sure there is no image distortion or gap/overlap in the middle and along the length of the images (see below image).

  - If the images do not have issues, proceed to Step 5.

  - Otherwise, refer to Test Loop Data Analysis Details for error resolution guidelines.

2- Perform Drive-through for Lane Adjustment and Pavement Event Flagging and Generate Test Loop QC Report – performed by DPT Staff Member

Summary: Under this task, the Vision software is used to analyze the LCMS output, correct errors in crack or joint detection, and mark events. Additionally, a QC report in MS Excel is created and results of the DPT QC checks are populated prior to submittal to the DAT for the subsequent tasks.

- Follow Performing 100% Drive Through for performing drive-through of pavement images and LCMS output. Confirm the accuracy of GPS readings.

- Open the “Test Loop Quality Control Summary Report” MS Excel template and create a new file with the following naming scheme: “ARAN#_COLLECTIONDATETL_QC_REPORT_ANALYSISDATE”06/12/2019. Complete the required fields and submit the MS Excel template to the DAT staff member that will perform the subsequent tasks (see below image).

  - Under “List of Files,” enter the names of the files being reviewed.

  - Next to “Upload/Import,” place a check mark under the “Satisfactory” cell heading and enter reviewer initials under the “Checked By” cell heading. If the upload/import was not successfully completed, resolve the issue before continuing.

  - Next to “Image Aspect Ratio,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 2 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” provide an explanation under the “Comment” cell heading and list the required action under the “Action” cell heading.

  - Next to “Imaging Interval,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 3 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” provide an explanation under the “Comment” cell heading and list the required action under the “Action” cell heading.

  - Next to “Overlap/Gap,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 4 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” provide an explanation under the “Comment” cell heading and list the required action under the “Action” cell heading.

  - Next to “GPS Accuracy,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 5 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” provide an explanation under the “Comment” cell heading and list the required action under the “Action” cell heading.

3- Perform Condition Data QC and Update Test Loop QC Report – performed by DAT Staff Member

Summary: Under this task, the various condition data metrics from the SHA test loop are reviewed against ground truth measurements. Additionally, the QC report created during the DPT QC checks is updated with the results of the DAT review.

- Open the “TL Condition QC” MS Excel template and create a new file with the date of the test loop data collection being reviewed06/12/2019.

- Paste raw data from the test loop collection effort as directed in the “Paste” worksheet” (see below image).

- Click over to the “Main” worksheet. Values for the current test loop data collection will populate in the spreadsheet for the following metrics under the “2018 Measured”06/12/2019 column: International Roughness Index (IRI), Rut, Structural Cracking (SC) Density, Functional Cracking (FC) Density, CrossFall, and Distance Measuring Instrument (DMI).

- Review DMI data in Columns “AM” and “AN.” There is only one section for evaluating DMI. The cell in Column “AN” with a bold box around it should be populated with “Pass.” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If the bold box reads “Pass,” check is complete.

  - If the bold box reads “Fail,” investigate the magnitude of the range error before continuing.

- Review the IRI data in Columns “I” and “J” (see below image). Any cell in Column “J” with a bold box around should be populated with “Pass.” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If all or most bold boxes read “Pass,” check is complete.

  - If there are several bold boxes that read “Fail”, investigate the magnitude of the range errors before continuing.

- Review the rutting data in Columns “P” and “Q” (see below image). Any cell in Column “Q” with a bold box around it should be populated with “Pass.” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If all or most bold boxes read “Pass,” check is complete.

  - If there are several bold boxes that read “Fail,” investigate the magnitude of the range errors before continuing.

- Review the SC Density data in Columns “W” and “X” (see below image). Any cell in Column “X” with a bold box around it should be populated with “Pass.” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If all or most bold boxes read “Pass,” check is complete.

  - If there are several bold boxes that read “Fail,” investigate the magnitude of the range errors before continuing.

- Review the FC Density data in Columns “AD” and “AE” (see below image). Any cell in Column “AE” with a bold box around it should be populated with “Pass.” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If all or most bold boxes read “Pass,” check is complete.

  - If there are several bold boxes that read “Fail,” investigate the magnitude of the range errors before continuing.

- TL Condition QC template showing Functional Cracking Density data review columns with Pass and Fail indicators

- Review the crossfall data in Columns “AK” and “AL” (see below image). Any cell in Column “AL” with a bold box around it should be populated with “Pass,” Further details regarding the comparison criteria can be found in Test Loop Data Analysis Details.

  - If all or most bold boxes read “Pass”, check is complete.

  - If there are several bold boxes that read “Fail”, investigate the magnitude of the range errors before continuing.

- Open the “Test Loop Quality Control Summary Report” MS Excel file created by the DPT. Complete the fields listed and submit to the DPT, DAT, and FED team leaders for review (see below image).

- Place the correct date in the “Date” cell (see above image).

- Next to “DMI,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 10 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the section value failed and the magnitude of the range error was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

- Next to “IRI,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 11 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the number of failed sections was substantial and the magnitude of range errors was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

- Next to “RUT,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 12 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the number of failed sections was substantial and the magnitude of range errors was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

- Next to “SCD (Crack),” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 13 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the number of failed sections was substantial and the magnitude of range errors was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

- Next to “FCD (Crack),” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 14 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the number of failed sections was substantial and the magnitude of range errors was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

- Next to “Crossfall,” place a check mark under the “Satisfactory” or “Unsatisfactory” cell heading based on the review completed in Step 15 and enter reviewer initials under the “Checked By” cell heading. If the review was “Unsatisfactory,” meaning that the number of failed sections was substantial and the magnitude of range errors was high, provide an explanation under the “Comment” cell heading and the required action under the “Action” cell heading (see above image).

4- Submit Test Loop QC Report – performed by DAT Staff Member and Team Leaders

Summary: Under this task, the results of the SHA test loop data analysis are documented and reported.

Ensure that SHA “Test Loop Quality Control Summary Report” is fully populated (see below image). This is done by a DAT staff member. - If report is complete, send report to DAT, DPT, and FED TLs. - If report is incomplete, fill in missing sections or ask DPT Staff Member to fill in those missing sections, depending on which portion of the report is incomplete.

- The report resulting from Step 17 is checked by the DAT, DPT, and FED TLs. - If no issues are identified, communicate results to other team leaders and save report to proper location. - If issues are identified, discuss with relevant parties and determine proper course of action. Once issues have been resolved, save report to proper location.

## Skid Pre-Data Collection Actions

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform route planning, safety checks, and diagnostic checks prior to the start of skid data collection each day. The steps described in this SOP are based on the ICC MDRPro 2.506/12/2019 software developed for operating Skid Trailers 7 and 806/12/2019. This SOP is one of three to be utilized for daily data collection with the referenced skid trailers – it is succeeded by Skid Data Collection and then Skid Post-Data Collection Actions. This SOP utilizes the Skid Collection List (see Production of Skid Collection List) for the current year.

Note: due to the frequent changes in skid collection software and the differences between the skid trailers, the main body of this SOP contains general steps to perform the required pre-data collection tasks. The skid user manual for the specific skid trailer should be used as supporting documentation to this SOP. The steps for using the current version of the skid data collection software, ICC MDRPro 2.5, in support of this SOP are provided at the end of this SOP.

### Frequency

This SOP is to be performed daily by SHA FED staff throughout the skid data collection period06/12/2019, prior to the first data collection run each day.

### Purpose

The purposes of this SOP are to develop an efficient route plan for skid data collection and to check the skid trailer to confirm it is safe to operate and functioning properly prior to data collection each day. This is accomplished by:

- Planning of data collection route such that the possible effect of environmental factors on data quality is reduced,

- conducting vehicle-related safety checks, and

- validating the functionality of the skid trailer components.

### Resource Requirements

The actions detailed in this SOP are performed by the SHA FED skid trailer crew, which is comprised of a driver and an operator. The estimated effort levels shown in the table below represent the total time, in man-hours, required for completion of the daily planning, safety checks, and diagnostics checks. These time estimates assume adequate weather conditions and that the diagnostic checks do not produce results that require additional troubleshooting.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| Skid Testing Crew | Driver | 1 | 1.5 |
| Skid Testing Crew | Operator | 1 | 1.5 |
| FED Team Leader (TL) or Coordinator | Supervisor | 1 | As Required 06/12/2019 |

### Procedure

The procedure required to perform pre-data collection actions on the skid trailer is comprised of the following three tasks:

- planning of data collection routes,

- performing vehicle safety checks, and

- performing daily equipment component diagnostic checks.

1- Planning of Data Collection Route – performed by driver and operator, with input from Supervisor

Summary: Under this task, the planned skid data collection route for the day is created and environmental conditions are assessed to determine whether data collection should proceed or be postponed.

- Develop an initial skid data collection route for the day using the selected sections from the Skid Collection List (see Production of Skid Collection List). The route is to be selected based on several logistic and practical factors – e.g., proximity to office and to gas stations – with the objective of maximizing the collected mileage for the day.

- Assess weather conditions for the day in order to decide whether to perform or to cancel skid data collection. Typical weather factors affecting this decision include:

  - Fog, snow, or ice – skid data collection must be cancelled if there is fog, rain, snow, or ice regardless of intensity 06/12/2019.

  - Surface wetness or ice – If it is not raining or snowing, but the pavement surface is wet (standing water) or icy after a visual inspection, data collection is to be delayed until the surface is dry 06/12/2019.

  - Below freezing – if the temperature is below freezing, data collection is delayed until the temperature rises above freezing.

If weather considerations do not result in the cancelation or delay of data collection, proceed Step 3.

2. Performing Vehicle Safety Checks – performed by driver and operator

Summary: This task performs vehicle-related safety assessments to check that the skid trailer is in proper working order and safe to operate.

- Driver: Turn on skid vehicle ignition.

- Driver: Ensure mirrors (left, right, and rear view) and seat position are properly set for safe driving.

- Operator: Check pressure of all tires, including skid trailer test tires. Vehicle tires should have pressure as specified by the vehicle manufacturer for given conditions and skid trailer tires should read 24 psi.

- Driver: Make sure water tank is filled.

- Driver: Check that skid trailer lights are functioning properly. This includes beacons, hazards, turning signals, and brake lights. If any lights or beacons are not functioning, communicate issue to Supervisor for further remedial action and suspend data collection until issue has been resolved.

3- Performing Daily Equipment Component Diagnostic Checks – performed by driver and operator

Summary: This task initiates the skid data collection computer system and runs diagnostic tests on the skid trailer sub-components to check that they are functioning properly for data collection.

- Driver: Position the skid trailer on level ground and out of the way of traffic.

- Operator: After confirming that engine is running, turn on the “Air” and “Trailer Power.” The specific location of these switches may vary between vehicles; consult the user manual of the specific skid trailer for details.

- Operator: Turn on computers. The specific location of these switches may vary between vehicles; consult the user manual of the specific skid trailer for details.

- Operator: Login to the proper computer user profile with the appropriate user name and password. The specific details of the user profiles may vary between vehicles; consult the user manual of the specific skid trailer for details.

- Operator: Login to the current skid data collection software by double-clicking on the MDRPro desktop icon and entering the appropriate sign-in credentials; consult the user manual of the specific skid trailer for details and the end of this SOP for instructions on using the ICC MDRPro 2.5 software.

- Operator: Perform pre-data collection diagnostic tests to ensure the skid trailer sub-components are functioning properly; consult the user manual of the specific skid trailer for details and the end of this SOP for instructions on using the ICC MDRPro 2.5 software.

  - If the calibration values are within the required tolerance, proceed to Skid Data Collection for commencement of daily skid data collection.

  - If the calibration values are out of range with respect to the required tolerance, contact Supervisor for remedial action and suspend data collection until the issue has been resolved.

ICC MDRPro 2.5 Skid Pre-Data Collection Software Instructions

The following instructions can be followed to supplement Task 3 of this SOP when using the ICC MDRPro 2.5 skid data collection software to perform the required pre-test diagnostic checks.

The steps in this section appear in multiple places for different calibration procedures. Refer to the specific procedure sections below for ICC MDRPro 2.5 software instructions.

Observe the values on the screen for “Left Force” and “Left Load” (see below image). Record both numbers in the record book kept in the skid vehicle. If values are out of range, move to more level ground and repeat test.

## Skid Data Collection

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform daily skid data collection. The steps described in this SOP are based on the ICC MDRPro 2.506/12/2019 software developed for operating Skid Trailers 7 and 806/12/2019. This SOP is one of three to be utilized for daily data collection with the referenced skid trailers – it is preceded by Skid Pre-Data Collection Actions and succeeded by Skid Post-Data Collection Actions. The data collection activities covered in this SOP start after the end of the daily diagnostic checks (see Skid Pre-Data Collection Actions) and they end prior to commencement of the daily data export (see Skid Post-Data Collection Actions) – i.e., it describes the procedure for completing a full day of skid data collection. This SOP utilizes the Skid Collection List (see Production of Skid Collection List) for the current year.

Note: due to the frequent changes in skid collection software and the differences between the skid trailers, the main body of this SOP contains general steps to perform the required data collection tasks. The skid user manual for the specific skid trailer should be used as a supporting documentation to this SOP. The steps for using the current version of the skid data collection software, ICC MDRPro 2.5, in support of this SOP are provided at the end of this SOP. This software should already be initialized from Skid Pre-Data Collection Actions.

### Frequency

This SOP is to be performed daily by SHA staff throughout the data collection period 06/12/2019.

### Purpose

The purposes of this SOP are to perform daily skid data collection and to perform visual quality control (QC) checks on those data while they are being collected.

### Resource Requirements

The actions detailed in this SOP are performed by the SHA FED skid trailer crew, which is comprised of a driver and an operator. The estimated effort levels shown in the table below represent the total time, in man-hours, required for completion of a full day of skid data collection. These time estimates assume adequate weather conditions and a typical data collection schedule. Actual time to complete individual runs in the field will vary between routes.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| Skid Testing Crew | Driver | 1 | 6.0 |
| Skid Testing Crew | Operator | 1 | 6.0 |
| FED Team Leader (TL) or Coordinator | Supervisor | 1 | As Required06/12/2019 |

### Procedure

The procedure required to perform daily skid data collection is comprised of the following single task:

- performing daily data collection.

1- Performing Daily Data Collection – performed by driver and operator

Summary: Under this task, daily skid data collection runs are completed using the skid trailer. The vehicle should be properly functioning and the computer system appropriately initiated after completion of the pre-data collection actions detailed in Skid Pre-Data Collection Actions.

- Operator: Navigate to the data collection screen on the current skid collection software; consult the skid user manual of the specific skid trailer for details.

- Operator: select the directory where data are to be stored by navigating to the appropriate folder; consult the skid user manual of the specific skid trailer for details.

- Operator: Enter the appropriate information for the specific run to be collected; consult the skid user manual of the specific skid trailer for details.

- Operator: Copy information from Step 3 into log book kept inside the skid vehicle.

- Driver: Once entry of information from Step 3 into the data collection software has been confirmed, drive towards beginning of test segment.

- Operator: Initiate the Distance Measuring Instrument (DMI) and data collection at the start of the test segment. Friction tests will occur approximately every 0.1 miles for mainline roads and 0.05 miles for shorter segments and ramps. The wheel will lock at these increments and a skid number will be recorded. If the speed limit is greater than 40 mph, operator should attempt to maintain a speed of 40 mph throughout the test. Otherwise, operator should maintain the posted speed limit. Consult the skid user manual of the specific skid trailer for details.

- Operator: Observe data collection screen during test, including: skid numbers, water output, and speed. If data does not seem reasonable, abandon test and restart. If data continues to appear unreasonable, contact Supervisor for further remedial action and suspend data collection until the issue has been resolved.

- Operator: At the end of the test segment, end the data collection and save the data; consult the skid user manual of the specific skid trailer for details.

- Repeat Step 1 through Step 8 for each data collection run during the day. After completion of the last data collection run, proceed to Skid Post-Data Collection Actions for end-of-day shut-down activities.

ICC MDRPro 2.5 Skid Data Collection Software Instructions

The following instructions can be followed to supplement Task 1 of this SOP when using the ICC MDRPro 2.5 skid data collection software to perform the required data collection.

Click on the “Main” tab. All collection information for a specific run will be entered in this window (see below image).

Select the proper directory under “MDR Data Path”06/12/2019 by selecting the “Browse” button next to the dialogue box and navigating to the appropriate folder. Type in the file name in the dialogue box that reads “File Name” following the appropriate format06/12/2019. Fill in all required fields under “Run Settings” (see above image). This includes: County, District, Route Number, Beg Ref Post, Lane, Direction, Ascending/Descending, Operator, and Driver.

Press the “F9” key on the keyboard to open the data collection screen. Ensure the run setting information at the top of the screen is correct, including: Reference Post, Direction, District, County, Route Number, Beginning Mile Post, and Ending Mile Post (see below image). If information is incorrect, return to “Main” tab and update information before continuing with data collection.

Press the “F4” key to initiate the Distance Measuring Instrument (DMI) and data collection.

At the end of the data collection run, press the “F3” key on the keyboard to end the test.

Press the “F10” key to exit the data collection screen. Click “Yes” to save data collection run. Click “Yes” again to return to main software screen

## Skid Post-Data Collection Actions

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform end-of-day data management and shut-down actions following completion of the daily skid data collection. The steps described in this SOP are based on the ICC MDRPro 2.506/12/2019 software developed for operating Skid Trailers 7 and 806/12/2019. This SOP is one of three to be utilized for daily data collection with the referenced skid trailers – it is preceded by Skid Pre-Data Collection Actions and Skid Data Collection. The post-data collection actions covered in this SOP start from the completion of the last data collection run of the day (see Skid Data Collection) to system shut down.

Note: due to the frequent changes in skid collection software and the differences between the skid trailers, the main body of this SOP contains general steps to perform the required end-of-day data management and shut down tasks. The skid user manual for the specific skid trailer should be used as a supporting documentation to this SOP. The steps for using the current version of the skid collection software, ICC MDRPro 2.5, in support of this SOP are provided at the end of this SOP. The software should already be ready for the post data collection tasks.

### Frequency

This SOP is to be performed daily by SHA staff throughout the data collection period06/12/2019, following the last data collection run of the day.

### Purpose

The purpose of this SOP is perform end-of-day data management and shut-down actions following completion of the daily skid data collection.

### Resource Requirements

The actions detailed in this SOP are performed by the SHA FED skid trailer crew, which is comprised of a driver and an operator. The estimated effort levels shown in the table below represent the total time, in man-hours, required for completion of the post-data collection actions. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| Skid Testing Crew | Driver | 1 | 0.5 |
| Skid Testing Crew | Operator | 1 | 0.5 |
| FED Team Leader (TL) or Coordinator | Supervisor | 1 | As Required06/12/2019 |

### Procedure

The procedure required to perform the skid post-data collection actions is comprised of the following tasks:

- transferring collected skid data, and

- shutting down skid trailer system and scanning log book records.

1- Transferring Collected Skid Data – performed by operator

Summary: Under this task, the collected skid data is transferred from the on-board computer to a flashdrive for transmittal to the FED TL. This task is only performed at the completion of a county or during the collection of the Test Loop (see Skid Test Loop Data Collection).

- If all segments for the county have been collected, or if the collection effort is on the Skid Test Loop (see Skid Test Loop Data Collection), proceed to Step 2. Otherwise, skip to the system shut-down task that starts in Step 3.

- Plug in flashdrive to USB port of on-board computer and navigate to the data storage folder. Copy data and safely eject flashdrive. Consult the skid user manual of the specific skid trailer for details.

2- Shutting Down Skid Trailer System and Scanning Log Book Records – performed by driver and operator

Summary: Under this task, the on-board computer system and vehicle are shut down.

- Operator: Safely shut down all on-board computers; consult the skid user manual of the specific skid trailer for details.

- Operator: Turn off “Trailer Power”, “Air Compressor”, and “Beacon;” consult the skid user manual of the specific skid trailer for details.

- Driver: Record vehicle mileage in log book and turn off ignition. Lock all doors.

- Once back in the office, scan log book records for the county and deliver flashdrive and scans to FED TL if Task 1 was required.

ICC MDRPro 2.5 Skid Post-Data Collection Software Instructions

The following instructions can be followed to supplement Tasks 1 and 2 of this SOP using ICC MDRPro 2.5 skid data collection software to perform the required post-data collection actions.

To copy data to flashdrive:

- Double-click on the “My Computer” icon on the computer desktop screen.

- Double-click on “C Drive” and then “Data.”

- Scroll down to the folder of the county that was just completed. Right-click and select “Copy.”

- Navigate to the flashdrive (“Removable Disk (F:)”). Right-click and select “Paste.”

- On the bottom right-hand corner of the screen, right-click on the USB icon and select “Eject Flash Disk.” A message will appear when it is safe to remove the flashdrive. Unplug flashdrive from computer.

To shut down computers:

- Click on the Windows icon at the bottom left-hand corner of the screen and select “Shut Down.”

- Toggle to second computer screen by pressing the black button on the back of the computer monitor.

- On the second computer desktop screen, click on the red power button and select “Shut Down.”

- Wait for a message on the screen to read “It is now safe to turn off computer” and then press the red button below the keyboard (hold until it beeps). Confirm that both computers are now off.

- On the middle console, turn off “Trailer Power”, “Air Compressor”, and “Beacon.”

## Skid DMI Calibration

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to calibrate the Distance Measuring Instrument (DMI) mounted on the skid trailers. The steps described in this SOP are based on the ICC MDRPro 2.506/12/2019 software developed for operating Skid Trailers 7 and 806/12/2019. The reference measurements used in the DMI calibration procedure described in this SOP are obtained by a state surveyor using thermally compensated steel tape. These reference measurements are obtained along two one-mile long pavement sections, marked on the shoulder lanes in both directions, on a segment of MD Route 100.

Note: due to the frequent changes in skid collection software and the differences between the skid trailers, the main body of this SOP contains general steps to perform the required DMI calibration tasks. The skid user manual for the specific skid trailer should be used as a supporting documentation to this SOP. The steps for using the current version of the skid collection software, ICC MDRPro 2.5, are provided at the end of this SOP.

### Frequency

This SOP should be performed monthly by SHA FED staff throughout the data collection period06/12/2019.

### Purpose

The purpose of this SOP is to check the accuracy of the distance measurements taken by the DMI mounted on Skid Trailers 7 and 8 using the ICC MDRPro 2.5 software and, if deemed necessary, to recalibrate the DMI.

### Resource Requirements

Calibration of the DMI is performed by the SHA FED skid trailer crew, which is comprised of a driver and an operator. The estimated effort levels shown in the table below represent the total time, in man-hours, required for driving from the SHA’s Office of Materials Technology (OMT) location to the calibration pavement sections on Route MD 100, performing three data collection runs on these two calibration pavement sections, and driving back to the OMT location.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| Skid Testing Crew | Driver | 1 | 1.5 |
| Skid Testing Crew | Operator | 1 | 1.5 |
| FED Team Leader (TL) or Coordinator | Supervisor | 1 | As Required06/12/2019 |

### Procedure

The procedure required to calibrate the DMI mounted on Skid Trailers 7 and 8 using the ICC MDRPro 2.5 software consists of the following two tasks:

- performing repeat calibration runs, and

- saving data and performing data QC.

1. Performing Repeat Calibration Runs – performed by driver and operator

Summary: Under this task, data from three runs are collected along the two calibration pavement sections on Route MD 100 for use in checking the accuracy of the distance measurements taken by the DMI mounted on Skid Trailers 7 and 8 using the ICC MDRPro 2.5 software and, if deemed necessary, for recalibrating the DMI.

The calibration sites consist of two one-mile long pavement sections on Route MD 100, located near the OMT and marked along the shoulder lanes – one in the east-bound direction and the other in the west-bound direction (see map screenshot below). Two runs are performed on the east-bound direction and one run in the west-bound direction, for a total of 3 data collection runs.

- Operator: Follow start-up procedures for initiating the computer system and performing standard vehicle checks as described in Skid Pre-Data Collection Actions.

- Driver: Drive the skid trailer to the starting point of the calibration site in the east-bound direction. The starting point can be identified by the following markings:

  - an orange paint mark on a tree to the right of the road (see first photo that follows this list),

  - a metal pole just beyond the pavement shoulder (see second photo that follows this list),

  - paint markings on the pavement shoulder (line and “ST”; see second photo that follows this list),

  - a survey nail in the pavement shoulder, and

  - a yellow sign that reads “DMI Calibration Site.”

- Operator: Initiate the skid data collection software and navigate to the “DMI Calibration” window; consult the skid user manual of the specific skid trailer for details

- Driver: Drive the skid trailer until the front bumper of the vehicle is aligned with the starting position, as identified by one or more of the earlier referenced markings06/12/2019.

- Operator: Begin the first test run in the software; consult the skid user manual of the specific skid trailer for details

- Driver: Drive forward on shoulder lane at a safe speed, from the starting position until a few feet before the end mark as identified by metal pole just beyond the pavement shoulder and survey nail shown in the photo below. The calibration interface should display travelled distance on the computer screen. Attempt to stay as straight as possible in the shoulder lane using the white stripe, edge of pavement, and guardrail (where present) as references.

- Driver: Drive skid trailer until front bumper of vehicle is aligned with “ending” position, as identified by metal pole just beyond the pavement shoulder and survey nail shown in the photo above06/12/2019.

- Operator: End the first test run; consult the skid user manual of the specific skid trailer for details

- Driver: Drive the skid trailer to starting point of the calibration site in the west-bound direction and repeat Step 2 to Step 8 to perform second DMI calibration run.

- Driver: Drive the skid trailer to starting point of the calibration site in the east-bound direction and repeat Step 2 to Step 8 to perform the third DMI calibration run.

2- Saving Data and Performing Data QC – performed by operator

Summary: Under this task, the DMI calibration results are saved and the resulting calibration factors are evaluated for acceptable accuracy.

- Once the third DMI calibration run has been completed, save the data set.

- Examine the calibration factors from the results reported in the software; consult the skid user manual of the specific skid trailer for details. The calibration factors for the 3 runs must agree within 0.1 percent and preferably within 0.05 percent.

  - If they do not agree, consult TL and discuss next steps for remedial actions.

  - If they do agree, exit the calibration program. Consult the skid user manual of the specific skid trailer for details.

ICC MDRPro 2.5 DMI Calibration Software Instructions

The following instructions can be followed to supplement Task 1 of this SOP when using the ICC MDRPro 2.5 skid data collection software to perform the required DMI calibration.

Click on the “MDRPro 2” icon on the desktop of the computer (see below image).

Login by typing “user” in the “Password” text box (see below image).

Click on the “Calibration” tab and then the “Distance Cal” button (see below image).

Press the “F6” key to start a new calibration set after the “Distance Calibration” window opens (see below image).

Press the “F2” key to start the DMI calibration run.

Press the “F3” key to stop the DMI calibration run.

The following details can be followed to supplement Task 2 of this SOP using ICC MDRPro 2.5 skid data collection software to save the DMI Calibration.

Once third run is complete, press the “F9” key to save the calibration set.

Press the “F10” key to exit the calibration program.

## Skid Water Calibration

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform the static water calibration of Skid Trailer 7, which is the smaller of the SHA friction data collection vehicles. This skid trailer has a 450 gallon water tank and it is typically used in congested areas – the counties of X1, X2 and X3. Skid Trailer 8, the larger of the SHA friction data collection vehicles, uses a different water calibration procedure.

The static water calibration is performed at the Office of Materials Technology (OMT) calibration area.

Note: The user manual for Skid Trailer 7 should be used as supporting documentation to this SOP. The steps for using the current version of the Skid Trailer 7 data collection software, ICC MDRPro 2.5, are provided at the end of this SOP.

### Frequency

The static water calibration of Skid Trailer 7 is performed by the SHA FED staff once per year, prior to the start of the data collection season06/12/2019.

### Purpose

The purpose of this SOP is to check that the quantity of water being released by Skid Trailer 7 is consistent with the on-board equipment readings.

### Resource Requirements

The static water calibration of Skid Trailer 7 involves the following SHA FED staff members:

- an operator knowledgeable in the data collection software,

- a driver responsible for maintaining a steady speed of 40 mph,

- an assistant to oversee water released into the bucket and,

- a timer to indicate the start and end of the calibration test.

The estimated effort levels shown in the table below represent the total time, in man-hours, to complete the skid water calibration. These time estimates assume no issues are encountered during the calibration.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| FED Staff | Operator | 1 | 0.5 |
| FED Staff | Assistant | 1 | 0.5 |
| FED Staff | Driver | 1 | 0.5 |
| FED Staff | Timer | 1 | 0.5 |
| FED Team Leader | Supervisor | 1 | As required 06/12/2019 |

### Procedure

The procedure required to perform the static calibration of the water being released by Skid 7 is comprised of a single task:

- perform static water calibration.

1- Perform Static Water Calibration – performed by driver, operator, assistant, and timer

Summary: Under this task, a static water calibration of Skid Trailer 7 is performed at the SHA OMT calibration area to check that the quantity of water being released by the referenced skid trailer is consistent with the on-board equipment readings.

- Driver: Position the vehicle so that the skid trailer is in the “test pit” in the garage and place wood blocks under the skid trailer tires.

- Assistant: Plug power cord from skid trailer in to a power source and initialize equipment.

- Assistant: Connect the air supply to the trailer “tongue” and turn on (for water pump).

- Assistant: Jack up the vehicle rear tires.

- Assistant: Place the splash guard on the water nozzle and place the test bucket underneath the water nozzle.

- Operator: Initialize the ICC MDRPro2 skid data collection software and login; see the end of this SOP. Navigate to the water calibration screen (see the end of this SOP); consult the user manual of Skid Trailer 7 for details.

- Driver: Turn on the vehicle and begin to “drive” the vehicle to spin the wheels elevated by the jacks. Attempt to maintain a speed of 40 mph.

- Operator: When the driver is ready (at a speed of 40 mph), signal “Go” to the timer.

- Timer: Start stopwatch.

- Operator: Turn on the water pump in the ICC MDRPo 2.5 software interface at the same time as the timer starting the stopwatch. Consult the Skid Trailer 7 user manual for details. Water will begin to fill the bucket.

- Timer: Monitor the stopwatch and signal “Stop” to the operator and assistant at the 15 second mark.

- Operator: On the timer’s “Stop” signal, turn off the water pump in the ICC MDRPro 2.5 software interface. Consult the Skid Trailer 7 user manual for details.

- Assistant: On the timer’s “Stop” signal, place the pan between the nozzle and bucket to prevent excess drip after the test has ended.

- Assistant: Remove the pan once the drip has stopped and dispose of the water in the drip pan.

- Assistant: With the help of the operator, remove the filled bucket and place on the scale. Measure the weight of the filled bucket.

- Operator: Remove tare (10.2 lbs) from the measured weight and divide the weight of the water (lbs) by 8.33 (lbs/gal) to convert the weight to gallons.

- Operator: Multiply the weight in gallons by four (15 second test) to compute a measurement of gallons per minute. The desired range is 27 to 28 gal/min.

- Operator: Normally, the first test is thrown out and a second test is run by repeating Step 6 to Step 17. Examine the results of the second test.

  - If the flow rate after the second test is in the desired range of 27 to 28 gal/min, the calibration is complete. Save the results of the calibration, power down the software, remove the vehicle from the blocks, remove the bucket and splash guard from the skid trailer, and remove all connections used during the calibration.

  - If the flow rate is still not in the desired range of 27 to 28 gal/min after the second test, troubleshooting is required. Possible solutions include water pump replacement and filter cleaning. Discuss troubleshooting and problem resolution with TL.

ICC MDRPro 2.5 Post-Data Collection Software Instructions

The following instructions can be followed to supplement Task 1 of this SOP when using the ICC MDRPro 2.5 skid data collection software to perform the calibration of the water being released by Skid Trailer 7.

Click on the “MDRPro 2” icon on the desktop of the computer (see below image).

Login by typing “user” in the “Password” text box (see below image).

Click on the “Calibration” tab and then “Water Cal” then “Water Arm On” so that the water pump arm is angled downwards.

Click “Water Pump On” at the same time as the timer starting the stopwatch. Water will begin to fill the bucket.

On the timer’s “Stop” signal, click “Water Pump Off”.

## Skid Force Calibration

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration’s (SHA) to perform skid force calibrations on the SHA friction data collection vehicles. The procedure described in this document is specific to the SHA Skid Trailers 7” and 8 data collection vehicles and to the use of the ICC MDRPro v2.5 skid data collection software. This procedure may not be applicable to other friction data collection vehicles or data collection software.

The skid force calibration is performed at the SHA Office of Materials Technology (OMT) calibration area.

Note: The user manuals for Skid Trailers 7 and 8 should be used as supporting documentation to this SOP. The steps for using the current version of the skid collection software, ICC MDRPro 2.5, are provided at the end of this SOP.

### Frequency

The skid force calibration of Skid Trailers 7 and 8 is performed by the SHA FED staff once per year, prior to the start of the data collection season06/12/2019, or when required.

### Purpose

The purpose of this SOP is to check that the load applied to left tire of Skid Trailers 7 and 8 and the traction readings recorded by the force meter are consistent with the on-board equipment readings as well as within the calibration standards; and to make the necessary adjustments when required.

### Resource Requirements

The skid force calibration involves the following SHA FED staff members: (1) an operator knowledgeable in the data collection software, and (2) a load meter assistant responsible for increasing and decreasing load levels and monitoring traction readings. The estimated effort levels shown in the table below represent the total time, in man-hours, to complete the skid force calibration. These time estimates assume no issues encountered during the calibration.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| FED Staff | Operator | 1 | 1.0 |
| FED Staff | Assistant | 1 | 1.0 |
| FED Team Leader | Supervisor | 1 | As required 06/12/2019 |

### Procedure

The procedure required to perform the skid force calibration for Skid Trailers 7 and 8 is comprised of the following two tasks:

- perform Skid Force calibration, and

- evaluate Skid Force calibration results.

1- Perform Skid Force Calibration – performed by operator and assistant

Summary: Under this task, a skid force calibration is performed at the SHA OMT calibration area on Skid Trailers 7 and 8. This is accomplished by measuring force readings underneath the left tire of the referenced skid trailers at varying pressure levels.

- Operator: Position vehicle so that the skid trailer is in the “test pit” in the garage and place wood calibration blocks under the skid trailer right tire. Place load plate under the skid trailer left tire (see below photo06/12/2019).

- Operator: Plug skid trailer power cord into power source and initialize equipment.

- Operator: Initialize the skid data collection software and login. Navigate to the water calibration screen (see details at the end of this SOP); consult user manual of specific skid trailer for details.

- Assistant: Connect air cylinder and chains to load plate (see below photo).

- Assistant: Connect load meter to load plate and to the ports in the vehicle.

- Assistant: Connect air supply to trailer “tongue” (see below photo) and turn on water pump.

- Allow 15 minutes for the equipment to warm up.

- Assistant: Raise left wheel using air jack behind trailer and push the “Zero” button on the force meter. The traction reading should be less than 5 and the load reading should be around 1,085 (see below photo). If not, make sure all components are connected and the software is initialized. Do not proceed to Step 9 until readings are acceptable.

- Operator: In software interface, turn on the break and check that the readings on the load meter have not changed. If they have changed, do not continue until issue is resolved. Observe that load and traction readings through the software’s auto-test function are updating. If they are not, make sure the cable from the load meter is plugged into the correct vehicle serial ports before continuing; consult user manual of specific skid trailer for details

- Operator: Begin test; consult user manual of specific skid trailer for details.

- Operator: Instruct the load meter assistant to begin increasing the load by turning the pressure switch clockwise.

- Assistant: Visually monitor the traction reading and continue to increase the load until the traction reading is near 100. The traction reading can be within +/- 10 of 100, but it is crucial that the load does not decrease. If the traction reading goes slightly above 100, DO NOT decrease the load to reach the desired traction reading.

- Assistant: When the traction reading is near 100, signal to the operator.

- Operator: Click the test option in the skid data collection software. Repeat Step 11 to Step 13 for traction levels 200, 300, 400, 500, 600, 700, and 750.

- Assistant: After completion of testing at traction level 750, begin to decrease the load and repeat the process for traction levels 700, 600, 500, 400, 300, 200, 100, and 0.

2- Evaluate Skid Force Calibration Results – performed by operator

Summary: Under this task, the results of the skid force calibration are evaluated by comparing the force readings as measured by the force meter with the applied load as measured under the skid tire by the data collection software. The steps for using the current version of the skid collection software, ICC MDRPro 2.5, are provided at the end of this SOP.

- Operator: Once testing is complete, check the percent error for the force and load. Out of tolerance values will be flagged by the software.

- Operator: Check for non-linearity issues.

- Operator: Examine a best fit curve of force meter readings to truck readings. Skid Number (SN) values should be within 0.3 points (standard acceptance criteria is within 3 points).

- Operator: Exit software, power down the computer, remove connections, and break down the calibration setup.

ICC MDRPro 2.5 Skid Force Calibration Software Instructions

The following instructions can be followed to supplement Task 1 of this SOP when using ICC MDRPro 2.5 skid data collection software to perform the required force calibration on Skid Trailers 7 and 8.

Click on the “MDRPro 2” icon on the desktop of the computer (see below image).

Login by typing “user” in the “Password” text box (see below image).

Click on the “Calibration” tab and then on “Force Cal” button (see below image).

In the software interface, make sure the first “Calibration Team” field is populated with the name of the operator and assistant.

Click on the “Break On” button.

Click on the “Autotest” button and confirm that load and traction readings are updating.

Click on the “Test” button. The data will populate for the “0” load row under the “M-Force” column in the interface table (see below image).

Select the next cell down in the “M-Force” column that corresponds with the “100” “Cal F” row. The cell will turn yellow when selected.

Click the “Test” button when prompted by the Assistant. Data will be populated in the selected yellow cell.

Confirm that the correct cell is selected (colored yellow) in the “M-Force” column before clicking on the “Test” button at each traction level.

The following details can be followed to supplement Task 2 of this SOP using ICC MDRPro 2.5 skid data collection software to save and exit the required force calibration.

Click on the “Calc” button and the software will produce a best fit curve of force meter readings to truck readings.

When finished, click on the “Report”, “Break Off”, “Exit”, “Yes”, and “Exit” buttons, in that order.

## Skid Test Loop Data Collection

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform skid data collection on a route specifically selected for this purpose. The steps described in this SOP are based on the ICC MDRPro 2.506/12/2019 software developed for operating Skid Trailers 7 and 806/12/2019. The data collection route is called the “test loop.” As depicted in the map06/12/2019 below, the test loop begins and ends near the Office of Materials Technology (OMT) location. It is approximately 13.1 miles in length and it is divided into segments of varying length based on surface type, intersection locations, ramp locations, and other common road features. This route was selected because it includes a variety of surface types and textures, which are used to validate the SHA skid data.

The preliminary actions and diagnostic checks outlined in Skid Pre-Data Collection Actions must be completed prior to the beginning of skid data collection runs on the referenced test loop.

### Frequency

This SOP requires that SHA FED staff perform three skid data collection runs on the test loop once per month during the data collection season 06/12/2019.

### Purpose

The purpose of this SOP is to perform skid data collection runs on the test loop to validate the SHA skid data.

### Resource Requirements

The actions detailed in this SOP are performed by the SHA FED skid trailer crew, which is comprised of a driver and an operator. The estimated effort levels shown in the table below represent the total time, in man-hours, required for completion of all test loop activities, including preliminary checks and diagnostics, three data collection runs, and data export and management. These time estimates assume no issues are encountered during the required work.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| Skid Testing Crew | Driver | 1 | 2.0 |
| Skid Testing Crew | Operator | 1 | 2.0 |
| FED Team Leader (TL) or Coordinator | Supervisor | 1 | As Required06/12/2019 |

### Procedure

The procedure required to complete skid data validation on the SHA test loop is comprised of the following task:

- completing skid data collection on SHA test loop.

1- Completing Skid Data Collection on SHA Test Loop – performed by driver and operator

Summary: Under this task, three runs on the SHA skid test loop are completed.

- Operator: Follow Skid Pre-Data Collection Actions, Step 3 through Step 16, for vehicle and diagnostic checks and system initialization.

  - If checks do not identify equipment issues, proceed to Step 2.

  - If checks identify equipment issues, consult Skid Pre-Data Collection Actions for troubleshooting methods. Communicate issue(s) to the TL or Coordinator and suspend test loop data collection until all systems are functioning properly.

- Driver and Operator: Follow Skid Data Collection, Step 1 through Step 10, for completing each test loop data collection run. The file name should be “TESTLOOP[RUN#][DATE]”. Three runs are to be completed.

  - If runs are successfully completed, proceed to Step 3.

  - If a run is not successfully completed or if one or more systems malfunctioned during data collection, attempt to resolve the issue(s). If Operator is not able to resolve the issue(s), communicate issue(s) to the TL or Coordinator and suspend test loop data collection activities until all systems are again functioning properly. When resolved, return to Step 2 and recollect the test loop runs.

- Driver and Operator: Follow Skid Post-Data Collection Actions, Step 1 through Step 6, for data management and system shut-down following the completion of the third and final test loop run06/12/2019.

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Construction Data

Click to go to Processing Paving & Maintenance History Data

Click to go to Processing Access Permit Data

Click to go to Processing As Bid Unit Cost Data

Click to go to Processing As Built Unit Cost Data

Click to go to Reviewing Projects in MCMS

Click to go to Running Transport Application

Click to go to Looking Up Fund Type

Click to go to Making Lane Configuration Changes

Click to go to Adding to Project Engineer List

Click to go to Viewing Paving & Maintenance History by Route

Click to go to Promoting As Bid to Construction History

Click to go to Generating As Built Report

Click to go to HMA Tonnage Application

Click to go to KPA Reports

## Introduction

This section describes the set of standard operating procedures (SOP) conducted by staff from the Data Processing Team (DPT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to collect, import, and process construction data from various sources necessary to support PM activities. The information collected, imported, and processed in these SOPs are incorporated into the Business Plan Tables.

The first two construction SOPs involve collecting and importing construction data from various historical sources. The first SOP describes the process required to import construction data from paving and maintenance history data, while the second SOP describes the process required to import construction data from access permit information.

The next two construction SOPs involve collecting and importing construction data from cost data sources. The first SOP describes the process required to import construction data from As Bid unit cost information, while the second SOP describes the process required to import construction data from As Built unit cost information.

The final set of construction SOPs describe supplemental procedures designed to support the SOPs described above. This set includes nine SOPs: reviewing projects in the Maryland Construction Management System, running the Transport application, looking up fund type, making lane configuration changes, adding to the Project Engineer list, viewing paving and maintenance history by route, promoting As Bid to construction history, generating an As Built report, and running the Hot Mix Asphalt Tonnage application.

Figure 7 shows a flowchart of the construction SOPs.

Figure 7: Construction SOPs Flowchart

## Processing Paving & Maintenance History Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to collect and update construction history data in the Pavement Data Warehouse (PDW). Projects are classified as either “Paving” or “Maintenance.” Paving projects consist of a continuous treatment applied to the roadway (i.e. mill and overlay, reconstruction, etc.). Maintenance projects consist of spot improvements applied to the roadway (i.e. patching, crack sealing, slurry/micro sealing, etc.). Construction history is used within the Pavement Management System (PMS) to fuel pavement inventory information, assist in the development of performance models, assist the Pavement Design Team, and assist external customers with knowledge concerning the structural history, composition, and traffic information for SHA controlled roadways. There are various external and internal data sources for the collection of construction history information.

### Frequency

The updating of construction history is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to collect and update paving and maintenance construction history data, verify and update inventory information associated with the project, perform quality control (QC) of gathered data, and update the data in the PDW.

### Resource Requirements

Data collection and upload of paving and maintenance information involves two people: (1) a user knowledgeable in collection of construction history data from various sources and the PDW software to perform data collection, QC, and uploading, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user and QC | 1 | 1.0-16.006/12/2019 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to collect and update paving and maintenance history data is comprised of the following three tasks:

- collection of paving and maintenance data,

- input of collected paving data into the PDW, and

- input of collected maintenance data into the PDW.

1- Paving and Maintenance Data Collection – performed by PDW user

Summary: This task collects and reviews paving and maintenance construction data from a variety of sources in order to retrieve the necessary inputs for data entry into the PDW.

| Data Item | Source |
| --- | --- |
| Project Contract (and FMIS) # | Project Plans/IFB |
| Project Fund Type | IFB/FMIS Lookup (Internal function) |
| Project State Construction P.E. | Assignment List Database |
| Project Functional Class | HLR/P & G Report |
| Project Project Description | IFB |
| Location Code | Contract Number (BA,AA,etc) |
| Location Route | Bid Tab (single Ad)/IFB/HLR/Project Plans/P & G Report |
| Location BMP & EMP | IFB/HLR/Project Plans/P&G Report/Visidata |
| Location Inventory Direction | HLR/Visidata |
| Project Rehab Thickness | IFB/Project Plans/P&G Report |
| Project Rehab Design Life | IFB/P&G Report |
| Project Rehab Construction Type | Project Plans/IFB |
| Project Rehab Construction Layers | Project Plans/IFB/P&G Report |
| Project Rehab Mix Design | HMA Tonnage Database/IFB/P&G Report |
| Project Rehab Paving, Patching, and Milling Dates | MCMS |

- For each project where available, review the Right-of-Way (ROW) video collected by the Automated Road Analyzer (ARAN) vehicle, “Visidata.exe.” Review the video to determine the limits of work for the project and verify, where possible, the type of construction action.

- Check the Invitation for Bid (IFB) for relevant pavement history data. IFBs can be received directly from the Office of Maintenance (OOM) and the Office of Highway Design (OHD). They may also come from either the Pavement Design (PD) Team or General Test team.

- Obtain pertinent pages of the IFBs that relate to construction history and dispose of remaining pages. The pertinent pages are (but not limited to): Title Page, Project Description, Category 500 Paving, Asphalt Cement Price Index, Paving Typical Sections, Plans Sheet, Soil Boring, Pavement Coring Log, and 2000, 5000 & 6000 series items in Schedule of Prices section (located near end of document).

- Retrieve Project Plan Set from the source. Review pertinent pages that relate to construction history. Ensure that all the pertinent data is enclosed in the plan set – paving typical sections, plan sheets, etc.

- Identify the project limits using the Highway Location Reference (HLR) Guide. The HLR Guide is categorized by county and route. Verify the mile points. The HLR also gives the Functional Class and AADT (Refer to Highway Location Referencing Guide).

- Access the MCMS database through the PDW application (see Reviewing Projects in MCMS). Export the relevant spreadsheets to Excel. Relevant pages are: Items Report and IDR Detail. For area-wide projects, ensure the appropriate Financial Management Information System (FMIS) number is chosen when exporting the spreadsheet to Excel. In the Items Report, all 2000, 5000 and 6000 series items should be evaluated to determine which items are relevant.

- Check Hot Mix Asphalt (HMA) Tonnage production database. Sort the data by “Contract Number” and hide all the unnecessary fields in the database. Keep Paving Location, Mix Number Production Date, and Actual Tonnage. Compare the total project estimated tonnage to the accumulated “actual” tonnage (see HMA Tonnage Application).

  - If the values are close, copy both tables (Estimated & Actual) into an Excel spreadsheet. Proceed to Step 8.

  - If the values are not close, further investigation is required before proceeding. Cross reference MCMS report and contact Project Engineer to reconcile any differences.

- Access the Assignment List database and print a summary with the State Construction P.E. listed.

- Check PM records for the PM – 003 form. This form contains the mile points (or station numbers), location information, mix design, project engineer, and type of construction.

2- Paving Data Input into PDW and QC – performed by PDW user

Summary: This task inputs collected paving construction data into the PDW.

- If the construction history data was paving, proceed to Step 11. If the construction history data was maintenance, skip to Step 26.

- Open the PDW application.

- On the Main Menu, click on “Data.” From the “Data” drop-down menu, choose “Construction Data” (see below image).

- In the “Select an Action” section choose “Add New” and then “Paving History” from the “Source” drop-down menu (see below image).

- Enter the required “Contract No” (and FMIS number if applicable). Click “Save” and then “OK” in the dialogue box that appears (see below image).

- Click the “Locations” tab. Click “Add New” at bottom of screen (see below image).

- Enter route information: County, Municipal (if needed), Prefix, Route, and Direction. Click “Confirm.” The “BMP” & “EMP” will automatically populate under the “Limits” section. Adjust the mile points to fit the specific project. When entering a ramp, always chose the Main Inventory direction in the “Location” section denoted by the asterisk (*). Click “Confirm” and then proceed by inputting the “Exit #” and “Ramp.”

- Enter construction information: Location Description, Paving, Patching and Milling Dates, and State Construction P.E. Click “Save” at the bottom of the screen. The locations that were entered will now appear in the location table (see below image). If the State Construction P.E. is not listed in the drop-down menu, then he/she will need to be added to the list before continuing (see Adding to Project Engineer List).

- Double-click on a location (it will be highlighted in blue when selected), then click on “Additional Data.” Select the “Fund Type.” To verify the “Fund Type,” refer to Looking Up Fund Type. In the “Critical Dates” section, the definitions for each critical date are as follows:

  - Received – any correspondence received from the Districts or the P.E.

  - Identified – project was identified by the PM team.

  - Processed – the data entry began for the project.

- Fill in the fields and click “Save” (see below image). Repeat this step for each location in the project.

- Double-click on a location (it will be highlighted in blue when selected). Click on the “Construction” tab.

- Click “Add New.” Select a “Construction Type” from the drop-down menu. Click the “Definition” button for details regarding each type.

- Fill in all the appropriate fields: Layer, Thickness, Material Type, Mix Method, Mix Type, Band, and ESAL. Then, select the correct “Material Description.”

- Click on the boxes above each lane at the bottom of the screen or select the appropriate option from the “Quick Lane Coverage” drop-down menu. Click on “Apply Treatment” to save the treatment for the selected section (see below image).

- Repeat Step 19 to Step 22 for all locations on the project.

- Click on “QC/QA Checklist” tab. Click on the relevant boxes on “QC/QA Check List” (left side of screen) and then click the “Save” button on the bottom of screen. If all data for the project is entered completely, click the “Check” button (see below image). Otherwise, return to incomplete data and input before proceeding.

- Click on “Supporting Documents” tab and upload any supporting documents using the “Upload” button (see below image). After this step, the procedure is complete (next section is for promotion through maintenance history only).

3- Maintenance Data Input into PDW and QC – performed by PDW user

Summary: This task inputs collected maintenance construction data into the PDW.

- Open the PDW application.

- From the Main Menu, navigate to Data > Construction Data (see below image).

- In the “Select an Action” section choose “Add New” and then “Maintenance” from the “Source” drop-down menu (see below image).

- A dialogue box will appear: “Do you want to import the data from Maintenance spreadsheet?” Click “No” except for the reports from Office of Maintenance (see below image).

- Enter the required “Contract No” (and FMIS number if applicable). Click “Save” and then “OK” in the dialogue box that appears (see below image).

- Click the “Locations” tab. Click “Add New” at bottom of screen (see below image).

- Enter route information: County, Prefix, Route, and Direction. Click “Confirm.” The “BMP” & “EMP” will automatically populate under the “Limits” section. Adjust the mile points to fit the specific project. When entering a ramp, always chose the Main Inventory direction in the “Location” section denoted by the asterisk (*). Click “Confirm” and then proceed by inputting the “Exit #” and “Ramp.”

- Enter construction information: Location Description, Paving, Patching and Milling Dates, and State Construction P.E. Click “Save” at the bottom of the screen. The locations that were entered will now appear in the location table (see below image). If the State Construction P.E. is not listed in the drop-down menu, then he/she will need to be added to the list before continuing (Adding to Project Engineer List).

- Double-click on a location (it will be highlighted in blue when selected), then click on “Additional Data.” Select the “Fund Type.” To verify the “Fund Type,” refer to Looking Up Fund Type. In the “Critical Dates” section, the definitions for each critical date are as follows:

  - Received – any correspondence received from the Districts or the P.E.

  - Identified – project was identified by the PM team.

  - Processed – the data entry began for the project.

- Fill in the fields and click “Save” (see below image). Repeat this step for each location in the project.

- Double-click on a location (it will be highlighted in blue when selected). Click on the “Construction” tab.

- Click “Add New.” Select a “Construction Type” from the drop-down menu. Click the “Definition” button for details regarding each type (see below image).

- Fill in all the appropriate fields: Layer, Thickness, Material Type, Mix Method, Mix Type, Band, and ESAL. Then, select the correct “Material Description.”

- Click on the boxes above each lane at the bottom of the screen or select the appropriate option from the “Quick Lane Coverage” drop-down menu. Click on “Apply Treatment” to save the treatment for the selected section (see below image).

- Repeat Step 19 to Step 22 for all locations on the project.

- Click on “QC/QA Checklist” tab. Click on the relevant boxes on “QC/QA Check List” (left side of screen) and then click the “Save” button on the bottom of screen. If all data for the project is entered completely, click the “Check” button (see below image). Otherwise, return to incomplete data and input before proceeding.

- Click on “Supporting Documents” tab and upload any supporting documents using the “Upload” button (see below image). After this step, the procedure is complete.

## Processing Access Permit Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to collect and update access permit data in the Pavement Data Warehouse (PDW). Access Permits have limited information available. Relevant inventory information is gathered from a variety of sources including but not limited to: Highway Location Reference (HLR), Plan Sets, Access Permit Contracts, and Material Managements reports.

### Frequency

The updating of access permit data is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to collect access permit data, verify and update inventory information associated with the project, perform quality control (QC) of gathered data, and update the data in the Unit Cost Database of the PDW.

### Resource Requirements

Data collection and upload of access permit information involves two people: (1) a user knowledgeable in collection of access permit data from various sources and PDW software to perform data collection, QC, and uploading, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user and QC | 1 | 1.0-16.006/12/2019 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to collect and upload access permit data is comprised of the following two tasks:

- collection of access permit data

- input of collected data into PDW.

1- Access Permit Data Collection – performed by PDW user

Summary: This task collects and reviews access permit data from a variety of sources in order to retrieve the necessary inputs for data entry into the PDW.

- Retrieve Project Plan Set from the source. Extract Permit #, Project Description, Location, and Rehab Information.

- Identify the project limits using the Highway Location Reference (HLR)Guide. The HLR guide is categorized by county and route. Verify the mile points. The HLR also gives the Functional Class and AADT (Refer to Highway Location Referencing Guide).

- Locate the Project File in the Materials Management file room. The files are not in numerical order but in sequential order. Therefore, ensure that the appropriate file folder is acquired.

- Scan all Contracts in the folder.

2- Access Permit Data Input into PDW and QC – performed by PDW user

Summary: This task inputs collected access permit construction data into the PDW.

- Refer to Processing Paving & Maintenance History Data, Section 2 for step by step directions. The steps for “Access Permit” projects are the same as “Paving” History.” Be sure that the Contract # is entered in the Access Permit format.

## Processing As Bid Unit Cost Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to collect and update As Bid unit cost data in the Pavement Data Warehouse (PDW). As Bid projects are single advertised projects. As Bid data is processed and analyzed prior to the commencement of construction. The primary data source for this information is bid tabulations published by the Office of Highway Design, extracted from the SHA website. Relevant inventory information to supplement the cost data is gathered from a variety of sources, including, but not limited to: Pavement & Geotechnical Reports (P&G Report), Pavement Management (PM) Base, project plans, Visidata, Vision and the Highway Location Referencing (HLR) System.

### Frequency

The updating of As Bid unit costs is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to collect As Bid unit cost data, verify and update inventory information associated with the project, perform quality control (QC) of gathered data, and update the data in the Unit Cost Database of the PDW.

### Resource Requirements

Data collection and upload of As Bid unit cost information involves two people: (1) a user knowledgeable in collection of As Bid unit cost data from various sources and the PDW software to perform data collection, QC, and uploading, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff (Fund 77) | PDW user and QC | 1 | 1.5 |
| DPT Staff (Non-Fund 77) | PDW user and QC | 1 | 2.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to collect and upload As Bid unit cost data is comprised of the following two tasks:

- collection of As Bid unit cost data

- input of collected data into PDW and QC.

1- As Bid Data Collection – performed by PDW user

Summary: This task collects and reviews As Bid unit cost data from a variety of sources in order to retrieve the necessary inputs for data entry into the PDW.

- For each project where available, review the Right-of-Way (ROW) video collected by the Automated Road Analyzer (ARAN) vehicle, “Vision.” Review the video to determine the limits of work for the project and verify, where possible, the type of construction action (see image below).

- Identify the project limits using the Highway Location Reference (HLR) Guide in conjunction with the Project Description from the Invitation for Bids (IFB). The HLR Guide is categorized by county and route. Verify the mile points. The HLR also gives the Functional Class and AADT (Refer to Highway Location Referencing Guide and Project Description).

- Check the IFB for relevant pavement history data. IFBs can be received directly from the Office of Maintenance (OOM) and the Office of Highway Design (OHD). They may also come from either the Pavement Design (PD) Team or General Test team. Maryland eMarketplace is also a location where the IFBs can be found (see below images).

- Remove pertinent pages of the IFBs that relate to construction history and dispose of remaining pages. The pertinent pages are (but not limited to): Title Page, Project Description, Category 500 Paving –Asphalt Cement Price Index, Paving Typical Sections, Plans Sheet, Soil Boring, Pavement Coring Log, and 2000, 5000 & 6000 series items in Schedule of Prices section (located near end of document).

- Retrieve Project Plan Set from the source. Review pertinent pages that relate to construction history. Ensure that all the pertinent data is enclosed in the plan set – paving typical sections, plan sheets, etc. The plan set can be found on the Maryland eMarketplace site (see below image).

- Access Transport database through the PDW application (see Running Transport Application). Export the spreadsheet to Excel. Identify the paving related items (typically 2000, 5000 & 6000 series items).

- Access the Bid Tabulation on the SHA homepage. Select the contract number for the project and print. The Bid Tabulation contains the contract number, letting date (date of bid opening) and the completion date (see below image).

- Access the PMbase from the Office of Materials Technology (OMT) homepage. Enter the route information required and retrieve the Construction History for the route. The construction history data includes the last treatment applied to the roadway and the pavement type.

- Locate the PAGD Geotechnical Report on the N:\OMT\Design Projects drive which is categorized by district. The folders are categorized by the contract number, by route, or by project description (or a combination thereof). Extract from the contract folder the Pavement Recommendation Memorandum and an excel spreadsheet entitled Design Help. The report will indicate the design life (if not, ask the pavement engineer who worked on the project). Use the Design Help to compute the ESALs as follows:

ESALs = ADT×18×% Truck ADT100×% Directional Distribution100

Where:
ADT = Average Daily Traffic
% Directional Distribution = Percentage of traffic in the direction of interest (assume 50% if unknown)
% Truck ADT = Percentage of trucks in the traffic stream

2- As Bid Data Input into PDW and QC – performed by PDW user

Summary: This task inputs collected As Bid data into the PDW.

- Open the PDW application.

- On the Main Menu click on “Reporting.” From the “Reporting” drop-down menu, choose “Transport” (see below image).

- In the “Select a Transport Report” section choose “Report – Contracts Not in Unit Cost” from the drop-down menu (see below image).

- Highlight and remove all the projects that are not good candidates such as projects not having pavement related items in the 5000 series (tree teaming, Snow removal, guardrail repairs).

- Double-click on a contract for the detailed item report.

- Examine and identify all the mainline and shoulder related items – include Excavation, GAB, common borrow, HMA, concrete, patching, milling/grinding, rumble strips and topsoil. These items are located in the following codes: 2000 – Borrow and Excavation, 5000 – Paving, and 6000 – Shoulder items.

- Check the pavement-related Items and click “Promote to As Bid” button (see below image).

- A dialogue box will appear: “You will need to enter the location associated with the items selected. Do you want to continue?” Click “Yes” (see below image).

- Click “Add New” at the bottom left hand corner of the screen to add a new location. Fill in the route information: County, Prefix, Route, Direction, From MP, To MP, and Description. Click “Save.” Repeat this step until all the locations for the project are entered. When all the locations have been entered and appear in the table, click “OK” (see below image).

- A dialogue box will appear: “A Unit Cost As Bid record has been successfully created for the selected items. Do you want to review the created As Bid record?” Click “Yes” (see below image).

- Enter data for: Fund Type, Letting Date, Cement Price, and Binder (should automatically appear). Click “Save” (see below image).

- Click the “Location” tab to enter the location information. The locations entered for the project during Step 19 will appear in the location table. Click “Save.” Double-click on a location in the location table. When it is highlighted in blue, click “View/Add UC Measures” (see below image).

- Enter the required information: Functional Class, Pavement type, Est. Completion Date, Thickness, Design Life, Action, Action Year, and ESALs. The Total Bid should automatically populate if the contract was promoted through the Transport feature. See Step 24 to automatically populate Lane Miles, Pavement Area, and Shoulder Area (see below image).

- To attain the completion date, go to the SHA Intranet page and click on the Business Tab (see below image).

- Click on the “Contracts, Bids & Proposals” link on the left side and then click on the “Bid Tabulations” link. Search for the contract and click on the “Contract Number” (see below images).

- In the “Edit Location Associated with Current Unit ID” section of the screen click on the “Add” pull-down menu and make sure all the locations are added to this unit cost measures. If the As Bid was promoted from the “Transport” feature, the locations will already appear in the table. If the project wasn’t promoted from the “Transport” feature, the locations must be enter manually using the aforementioned steps.

- Click on the drop-down menu in the “Quick Lane Coverage” section of the screen. Select “All” then press “Calc Lane Miles/Area” and the PDW will automatically generate the quantities for lane miles, pavement area and shoulder area (see image below).

- Verify the accuracy of the quantities that the PDW generated. Click on “Open Worksheet.” Fill in the appropriate fields (extracted from the typical section within the Plans Set): Lanes, Lane Mi, L (length), m/l W (width), Shoulder W (width), and Shoulder SY. The total at the bottom of the worksheet should be within 5% of the PDW auto generated quantity (see below image).

  - If the total is within 5% of the PDW auto generated quantity, proceed to Step 26.

  - If the total is not within 5% of the PDW, further investigation is required to reconcile any differences. Click on the SY Pavement tab of the spreadsheet and plug in the surface tonnage and the thickness. The SY surface will automatically populate. Compare that result to the worksheet and the PDW result. In addition, if there is any grinding, compare the grinded SY to the result.

- Click “Save” and then “Close.” The PDW will automatically return to the “Location” tab.

- Double-click on a location in the table while on the “Location” Tab. Make sure the location is highlighted in blue. Click on the “Construction” Tab (see below image).

- Click on “Add New.” Select a “Construction Type” from the drop-down menu. The definition of each “Construction Type” can be viewed by clicking the “Definition” button (see below image).

- Fill in all the appropriate fields: Layers, Thickness, Material Type, Mix Method, Mix Type, Band, and ESAL. Select the correct “Material Description.”

- Click on the boxes above each lane on the bottom of the screen or select the appropriate option from the “Quick Lane Coverage” drop-down menu. For all Unit Cost - As Bid, the treatment date is the “Completion Date.” Click on “Apply Treatment” to save the treatment for the selected section (see below image).

- Repeat Step 27 to Step 30 for all locations of the project. Then, proceed to Step 24.

- Click on the “Item Cost” tab. Double-click on each item (highlighting the item in blue) in the table and assign the relevant activity under the “Activity” drop-down menu. Click “Save.” Repeat for each item in the table. For Bid Adjustments, the unit must be assigned as “Each” (see below image).

- Click on “QC/QA Checklist” tab. Click on the relevant boxes on “QC/QA Check List” (left side of screen) and then click the “Save” button on the bottom of screen. If all data for the project is entered completely, click the “Check” button (see below image). Otherwise, return to incomplete data and input before proceeding.

- Scan, download and save all the supporting documents in a folder (local driver, PRG).

- Click on “Supporting Documents” tab and click on the “Browse” button to open the files location, select the file and click “OK” and then click on the “Upload” button (see below image).

- The supporting documents should include, but is not limited to:

  - IFB

  - Plans

  - Notes and other draft used by the user

  - The spreadsheet used to verify the accuracy of the quantities

  - A screenshot of the first page of the bid tabulation showing the completion date

  - The “Design Help” spreadsheet showing the calculation the ESALs

  - PAGD Geotechnical Report

  - PMBase Report

## Processing As Built Unit Cost Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to collect and update As Built unit cost data in the Pavement Data Warehouse (PDW). As Built projects are area-wide projects. As Built data is processed and analyzed approximately three months after the completion of construction. The primary data source for this information is bid tabulations published by the Office of Highway Design (OHD), extracted from the SHA website. Relevant inventory information to supplement the cost data is gathered from a variety of sources, including, but not limited to: Pavement & Geotechnical Reports (P&G Report), Pavement Management (PM) Base, project plans, Visidata, and the Highway Location Referencing (HLR) System. As Built projects can be promoted either from the Maryland Construction Management System (MCMS) or from paving or maintenance history.

### Frequency

The updating of As Built unit costs is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to collect As Built unit cost data, verify and update inventory information associated with the project, perform quality control (QC) of gathered data, and update the data in the Unit Cost Database of the PDW.

### Resource Requirements

Data collection and upload of As Built unit cost information involves two people: (1) a user knowledgeable in collection of As Built unit cost data from various sources and the PDW software to perform data collection, QC, and uploading, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user and QC | 1 | 4.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to collect and upload As Built unit cost data is comprised of the following three tasks:

- collection of As Built unit cost data

- input of collected data into the PDW by promotion from MCMS, and

- input of collected data into the PDW by promotion from paving or maintenance history.

1- As Built Data Collection – performed by PDW user

Summary: This task collects and reviews As Built construction data from a variety of sources in order to retrieve the necessary inputs for data entry into the PDW.

- For each project where available, review the Right-of-Way (ROW) video collected by the Automated Road Analyzer (ARAN) vehicle, “Visidata.exe.” Review the video to determine the limits of work for the project and verify, where possible, the type of construction action.

- Identify the project limits using the Highway Location Reference (HLR)Guide. The HLR Guide is categorized by county and route. Verify the mile points. The HLR also gives the Functional Class and AADT (Refer to Highway Location Referencing Guide).

- Check the Invitation for Bid (IFB) for relevant pavement history data. IFBs can be received directly from the Office of Maintenance (OOM) and the OHD. They may also come from either the Pavement Design (PD) Team or General Test team.

- Remove pertinent pages of the IFBs that relate to construction history and dispose of remaining pages. The pertinent pages are (but not limited to): Title Page, Project Description, Category 500 Paving – Asphalt Cement Price Index, Paving Typical Sections, Plans Sheet, Soil Boring, Pavement Coring Log, and 2000, 5000 & 6000 series items in Schedule of Prices section (located near end of document).

- Retrieve Project Plan Set from the source. Review pertinent pages that relate to construction history. Ensure that all the pertinent data is enclosed in the plan set – paving typical sections, plan sheets, etc.

- Access the MCMS database through the PDW application (see Reviewing Projects in MCMS). Export the relevant spreadsheets to Excel. Relevant pages are: Items Report and IDR Detail. For area-wide projects, ensure the appropriate Financial Management Information System (FMIS) number is chosen when exporting the spreadsheet to excel. In the Items Report, all 2000, 5000 and 6000 series items should be evaluated to determine which items are relevant.

- Check Hot Mix Asphalt (HMA) Tonnage production database. Sort the data by “Contract Number” and hide all the unnecessary fields in the database. Keep Paving Location, Mix Number Production Date, and Actual Tonnage. Compare the total project estimated tonnage to the accumulated “actual” tonnage (see HMA Tonnage Application).

  - If the values are close, copy both tables (Estimated & Actual) into an Excel spreadsheet. Proceed to Step 8.

  - If the values are not close, further investigation is required before proceeding. Cross reference MCMS report and contact Project Engineer to reconcile any differences.

- Access the Bid Tabulation on the SHA homepage. Select the contract number for the project and print. The Bid Tabulation contains the contract number and letting date (date of bid opening).

- Access the PMbase from the Office of Materials Technology (OMT) homepage. Enter the route information required and retrieve the Construction History for the route, IRI, and Friction. The construction history data includes the last treatment applied to the roadway and the pavement type.

- Locate the PAGD Geotechnical Report on the “OMTOOCShared:\ OMT\Design Projects” drive which is categorized by district. The folders are categorized by the contract number, by route, or by project description (or a combination thereof). Extract from the contract folder the pavement recommendation memorandum and an excel spreadsheet entitled “Design Help.”

2- As Built Data Input into PDW and QC (Promotion from MCMS) – performed by PDW user

Summary: This task inputs collected As Built data that was promoted from MCMS into the PDW.

- If the As Built data was promoted from the MCMS, proceed to Step 11. If the As Built data was promoted from paving or maintenance history, skip to Step 33.

- Open the PDW application.

- On the Main Menu click on “Reporting.” From the “Reporting” drop-down menu, choose “MCMS” (see below image).

- In the “Select a MCMS Report” section choose “Specify a Contract” from the drop-down menu (see below image).

- Enter the required “Contract No.” Click on the “Select a Report” drop-down menu and select “Report – IDR by Job No. Detail” (see below image).

- Select the specific “Project Description” from the drop-down menu. Make sure the FMIS matches the specific project needed. Click “Export to Excel” and when promoted, save the document in the desired location.

- Repeat Step 14 to Step 15, except in the “Select a Report” drop-down menu, select “Report – IDR by Job No. Item Cost.”

- With the “Report – IDR by Job No. Item Cost” open in the PDW window, check the boxes of the items that apply to pavement and then press the “Promote to As Built” button (see below image).

- A dialogue box will appear: “The Contract/FMIS has been promoted from Paving Hist to As Built.” Click “OK.” Another dialogue box will appear: “You will need to enter the location associated with the items selected. Do you want to continue?” Click “Yes” (see below images).

- The PDW automatically retrieves the paving or maintenance associated with this contract and FMIS number. Therefore, the location should automatically appear in the location tab. Double-click a location in the location table (it will be highlighted in blue when selected). Edit the description and mile points if needed. Click “Save” (see below image). If all the locations are not listed, click “Add New” and enter required information before clicking “Save.”

- A dialogue box will appear: “A Unit Cost As Built record has been successfully created for the selected items. Do you want to review the created As Bid record?” Click “Yes” (see below image).

- Enter data for: Fund Type, Letting Date, Cement Price, and Binder (should automatically appear). Check the appropriate boxes in the “Project Feature” section and click “Save” (see below image).

- Click the “Location” tab to enter the location information. The locations entered for the project during Step 19 will appear in the location table. Click “Save.” Double-click on a location in the location table. When it is highlighted in blue, click “View/Add UC Measures” (see below image).

- Enter the required information: Functional Class, Pavement type, Est. Completion Date, Thickness, Design Life, Action, Action year, Design Life, PCI, IRI, AADT, ESALs, Friction. The Total Bid should automatically appear if the contract was promoted through the MCMS feature. The Lane Miles, Pavement Area, and Shoulder can be automatically generated by the PDW application (see below image).

- In the “Edit Location Associated with Current Unit ID” section of the screen, click on the “Add” pull-down menu and make sure all the locations are added to this unit cost measures. If the As Built was promoted from the MCMS feature, the locations will already appear in the table.

- Click on the drop-down menu in the “Quick Lane Coverage” section of the screen. Select “All Thru” then press “Calc Lane Miles/Area” and the PDW will automatically generate the quantities for lane miles, pavement area and shoulder area (see below image).

- Verify the quantities that the PDW generated. Click on “Open Worksheet.” Fill in the appropriate fields (extracted from the typical section within the Plans Set): Lanes, Lane Mi, L (length), m/l W (width), Shoulder W (width), and Shoulder SY. The total at the bottom of the worksheet should be within 5% of the PDW auto generated quantity (see below image).

  - If the total is within 5% of the PDW auto generated quantity, proceed to Step 28.

  - If the total is not within 5% of the PDW, further investigation is required to reconcile any differences.

- Click “Save” and then “Close.” The PDW will automatically return to the “Location” tab.

- Double-click on a location in the table while on the “Location” tab. Make sure the location is highlighted in blue. Click on the “Construction” tab. Since the project has been promoted from MCMS and the PDW retrieved the construction history from the paving or maintenance history, the construction layers should already be populated (see below image). Make sure the construction data is correct using the information gathered in the “Data Collection” section. If the data is not accurate, make the necessary adjustments to correct the construction data. However, the “Construction History” must be edited as well. Repeat this step for all locations on the project.

- Click on the “Item Cost” tab. Double-click on each item (highlighting the item in blue) in the table and assign the relevant activity under the “Activity” drop-down menu. Click “Save.” Repeat for each item in the table. For As Built Adjustments, the unit must be assigned as “Each.” Make sure the “As Built” box is checked for every item.

- Click on “QC/QA Checklist” tab. Click on the relevant boxes on “QC/QA Check List” (left side of screen) and then click the “Save” button on the bottom of screen. If all data for the project is entered completely, click the “Check” button (see below image). Otherwise, return to incomplete data and input before proceeding.

- Click on “Supporting Documents” tab and upload any supporting documents using the “Upload” button (see below image). After this step, the procedure is complete (next section is for promotion through paving or maintenance history only).

3- As Built Data Input into PDW and QC (Promotion from Paving or Maintenance History) – performed by PDW user

Summary: This task inputs collected As Built data that was promoted from Paving or Maintenance History into the PDW.

- Open the PDW application.

- On the Main Menu click on “Data.” From the “Data” drop-down menu, choose “Construction Data” (see below image).

- Click on the “Action” drop-down menu and select “View/Edit/Delete” (see below image).

- Select either “Paving” or “Maintenance” from the “Source” drop-down menu.

- Enter the “Contract No.” of the projected to be promoted to Unit Cost – As Built (see below image).

- Double-click the Contract & FMIS number of the specific project that will be promoted to Unit Cost – As Built. The PDW will open the “Construction” main tab for that specific contract.

- Click the “MCMS” button. A MCMS window with the contract number already entered will open. Click “Export to Excel” for the relevant spreadsheets and press Close” (see below image).

- Click the “Promote” button. A dialogue box will appear: “WARNING. You are about to promote Paving History/Maintenance records to As Built records for Contract: _____ and FMIS _____. Are you sure you want to continue?” Click “OK” (see below image).

- Once the promotion is complete a dialogue box will appear: “Promotion done!” Click “OK.” Then close the “Construction History” project.

- Click on “Data.” Select the “Construction Data” option from the drop-down menu. Click on the “Action” drop-down menu and select “View/Edit/Delete.” Select “UC- As Built” from the “Source” drop-down menu

- Enter data for: Fund Type, Letting Date, Cement Price, and Binder (should automatically appear). Check the appropriate boxes in the “Project Feature” section. Click “Save” (see below image).

- Click the “Location” tab and make changes if necessary. Click “Save.” Double-click on a location in the location table. When it is highlighted in blue, click “View/Add UC Measures” (see below image).

- Enter the required information: Total Bid, Functional Class, Pavement type, Est. Completion Date, Thickness, Design Life, Action, Action year, Design Life, PCI, IRI, AADT, ESALs, Friction. The Total Bid will not appear automatically because the contract was promoted from Construction History. The Lane Miles, Pavement Area, and Shoulder can be automatically generated by the PDW application.

- In the “Edit Location Associated with Current Unit ID” section of the screen, click on the “Add” pull-down menu and make sure all the locations are added to this unit cost measures. The locations will not automatically populate in the location table because the project was promoted from Construction History.

- Click on the drop-down menu in the “Quick Lane Coverage” section of the screen. Select “All Thru” then press “Calc Lane Miles/Area” and PDW will automatically generate the quantities for lane miles, pavement area and shoulder area (see below image).

- Verify the quantities that the PDW generated. Click on “Open Worksheet.” Fill in the appropriate fields (extracted from the typical section within the Plans Set): Lanes, Lane Mi, L (length), m/l W (width), Shoulder W (width), and Shoulder SY. The total at the bottom of the worksheet should be within 5% of the PDW auto generated quantity (see below image).

  - If the total is within 5% of the PDW auto generated quantity, proceed to Step 49.
If the total is not within 5% of the PDW, further investigation is required to reconcile any differences.

- Click “Save” and then “Close.” The PDW will automatically return to the “Location” tab.

- Double-click on a location in the table while on the “Location” tab. Make sure the location is highlighted in blue. Click on the “Construction” tab. Since the project has been promoted from Construction History, the PDW should have populated the construction layer (see below image). Make sure the construction data is correct using the information gathered in the “Data Collection” section. If the data is not accurate, make the necessary adjustments to correct the construction data. However, the “Construction History” must be edited as well. Repeat this step for all locations on the project.

- Click on the “Item Cost” tab. Double-click on each item (highlighting the item in blue) in the table and assign the relevant activity under the “Activity” drop-down menu. Click “Save.” Repeat for each item in the table. For As Built Adjustments, the unit must be assigned as “Each.” Make sure the “As Built” box is checked for every item.

- Click on “QC/QA Checklist” tab. Click on the relevant boxes on “QC/QA Check List” (left side of screen) and then click the “Save” button on the bottom of screen. If all data for the project is entered completely, click the “Check” button (see below image). Otherwise, return to incomplete data and input before proceeding.

- Click on “Supporting Documents” tab and upload any supporting documents using the “Upload” button (see below image).

## Reviewing Projects in MCMS

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to review construction information for single-ad and area-wide projects using the Maryland Construction Management System (MCMS) within the Pavement Data Warehouse (PDW). MCMS is a standalone application run by the inspection division of SHA. The database contains detailed construction information that is significant to data collection and processing of SHA construction projects. MCMS is the primary source for construction information for single-ad and area-wide projects. Three spreadsheets created by MCMS are utilized: an itemized report with quantities and unit cost, a detailed construction report categorized by item, and the job order (applies only to area-wide projects). The PDW has an internal MCMS feature to streamline the data collection process.

### Frequency

The use of the MCMS application within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to review construction information using the MCMS application and produce reports of the desired information for use in construction data collection. See Processing Paving & Maintenance History Data, Processing Access Permit Data, and Processing As Built Unit Cost Data.

### Resource Requirements

The running of the MCMS application involves two people: (1) a user knowledgeable in PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 4.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to run the MCMS application for construction information reporting is comprised of a single task:

- generating an MCMS report in the PDW.

1- Generating an MCMS Report in the PDW – performed by PDW user

Summary: This task generates a report from the MCMS application within the PDW. The output of this task is an Excel spreadsheet with a list of desired contracts and associated construction information.

- Open the PDW application.

- Select the “Reporting” button from the main menu and click MCMS from the drop-down list (see below image).

- In the Select a MCMS Report drop-down list, click Specify a Contract (see below image).

- Another window will automatically open. Enter the Contract No (see below image).

- On the left side of the window, change the Select a Filter for MCMS Items to User's Pick and check the boxes for all the desired item series (see below image).

- On the Select a Report drop-down menu, choose the desired option. The first four reports apply to single-ad projects, while the last three reports apply to area-wide projects (see below image).

  - If a single-ad project option is chosen, proceed to Step 8.

  - If an area-wide project option is chosen, proceed to Step 7.

- Choose the correct description from the Select a Job Description drop-down menu and confirm the FMIS matches the specific project needed. If it does not match, confirm the correct job description was selected (see below image).

- Once the desired contracts are displayed, click the Export to Excel button. Name the document in the save prompt window, choose the proper save location, and click Save. The spreadsheet will automatically open in Excel.

- Repeat Step 2 to Step 8 for each desired MCMS report.

## Running Transport Application

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to review bid tabulation information for single-ad projects in the Pavement Data Warehouse (PDW) prior to Unit Cost – As Bid data processing (see Processing As Bid Unit Cost Data). The Transport application within the PDW is used to accomplish this task. It is operated by the Office of Construction within the SHA as a means to prompt the DPT when contracts are ready.

### Frequency

The use of the Transport application within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to review bid information and determine if the contracts are good candidates for promotion to the Unit Cost - As Bid processing.

### Resource Requirements

The running of the Transport application involves two people: (1) a user knowledgeable in PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 1.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Transport application is comprised of a single task:

- Running Transport application in the PDW.

1- Running Transport Application in the PDW – performed by PDW user

Summary: This task generates a list from the Transport application within the PDW for contracts that are good candidates for Unit Cost – As Bid. This task is completed prior to data processing of Unit Cost – As Bid contracts as outlined in Processing As Bid Unit Cost Data.

- Open the PDW application.

- Select the “Reporting” button from the main menu and click “Transport” from the drop-down list (see below image).

- In the Select a Transport Report drop-down list, select Report – Contracts Not in Unit Cost (see below image).

- Double-click on a contract to see the detailed item report (see below image). The PDW will automatically open the detailed item report. Not every project that is displayed in the Transport application qualifies as a good candidate for “Unit Cost – As Bid”, the project must contain 5000s series paving items.

  - If the project is a good candidate for Unit Cost – As Bid, continue to Processing As Bid Unit Cost Data.

  - If the project is not a good candidate for Unit Cost – As Bid, click the Remove button. A dialogue box will appear (see below image). Reviewed contract not a good candidate will already be populated in the text area. Click OK.

## Looking Up Fund Type

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to look up the fund type for a specific project using an application within the Pavement Data Warehouse (PDW). SHA has a numbering convention for projects and the last two digits in the contract number indicate the type of construction project. However, this is not always the case for area-wide projects. The Mother Contract could indicate a rehab project while the Financial Management Information System (FMIS) number could represent an intersection improvement. The FMIS lookup application gives fund type directly from the Office of Finance. Area-wide projects have a contract and FMIS number, whereas single-ad projects are assigned a single contract number. However, using the county and the next five digits, an FMIS can be created and used as the FMIS for lookup purposes (e.g., “AL3805177” is a single-ad contract number, while “AL380(A/B/C)51” is the FMIS number). The list of fund types has been included as an appendix (see Fund Type).

### Frequency

The use of the fund type lookup application within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to look up the fund type for a specific construction project.

### Resource Requirements

Looking up a construction project fund type involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to look up a construction project fund type is comprised of a single task:

- running fund type lookup application in the PDW.

1- Running Fund Type Lookup Application in the PDW – performed by PDW user

Summary: This task utilizes the fund type lookup application in the PDW to identify the fund type for a specific contract.

- Open the PDW application.

- Select the “Administration” button from the main menu and click “Lookups” from the drop-down list, then “Fund/FMIS” (see below image).

- A new window will automatically appear. In the “Select a FMIS” drop-down list, select the appropriate FMIS number (see below image).

- The FMIS information will populate in the window. The first column in the table of the appendix (“Fund”) indicates the fund type. Reference Fund Type.

## Making Lane Configuration Changes

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to make changes to lane configurations using an application within the Pavement Data Warehouse (PDW). SHA construction projects can modify, extend, or create new lanes and roadways. These changes need to be reflected in the PDW. Lane modification changes (modified lane, new lane or extended lane) in the PDW are only allowed for completed projects. Proposed projects, As-Bid, or incomplete projects are prohibited from making lane modifications.

### Frequency

Making changes to lane configurations within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to obtain pertinent data relating to changes in lane configuration and then to modify or extend existing routes or create new routes in the PDW.

### Resource Requirements

Making changes to lane configurations within the PDW involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 4.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to make changes to lane configurations within the PDW is comprised of the following four tasks:

- identifying lane configuration changes,

- viewing or modifying an existing route,

- creating a new route, and

- extending an existing route.

1- Identifying Lane Configuration Changes – performed by PDW user

Summary: This task obtains and verifies several key pieces of construction data in order to determine the extents and type of lane configuration change to be made. Plan sets are required for all lane configuration changes.

- Retrieve Project Plan Set from the source. Review pertinent pages that relate to construction history. Check that all the pertinent data is enclosed in the plan set (including paving typical sections, plan sheets, etc.). Using the plan set, determine where the lane modification should occur (Beginning Milepost, Ending Milepost, and Lane Width).

- Review the Right-of-Way (ROW) video collected by the Automated Road Analyzer (ARAN) vehicle (Visidata.exe). Use the video to determine the project limits and where the lane configuration changes occurred.

- Identify the project limits using the Highway Location Reference (HLR) Guide. The HLR Guide is categorized by county and route. Verify the mile points (Refer to Highway Location Referencing Guide).

- Determine the appropriate type of lane configuration change.

  - If an existing route needs to be modified, proceed to Step 5.

  - If a new route needs to be created, proceed to Step 14.

  - If an existing route needs to be extended, proceed to Step 22.

2- Viewing or Modifying Existing Route – performed by PDW user

Summary: This task uses the PDW software to modify an existing route based on determined lane configuration changes. The “View/Modify Existing Route” feature within the PDW allows for modifications of the route, such as creation or removal of lanes, construction of turning lanes, etc. When modifying a lane within the PDW system, all of the history associated with that lane is modified as well. Check the accuracy of the changes that will be made to the route prior to completion.

- Open the PDW application.

- Select the “Data” button from the main menu and click “Inventory” from the drop-down list, then “View/Modify an Existing Route” (see below image).

- A new window will automatically appear. Enter the required information for the route to be modified: County, Municipal (if needed), Prefix, Route, and Direction. Press “Confirm.” In the “Limits” section, enter the Beginning Milepost and the Ending Milepost for the area where the lane modification will occur. Press “Confirm.” The existing lane configuration will be displayed at the bottom of the screen (see below image).

- In the PDW lane configuration system, roadways have “breaks” which are sections of the roadway. These breaks can occur where turn lanes begin or end, shoulders exist, acceleration or deceleration lanes begin or end, etc. These breaks are displayed in the drop-down menu in the “Configuration Breaks” section. Select the range where the lane configuration will be modified from the drop-down menu. Click the “Modify Lane Config” button (see below image).

- A new window will automatically appear. In the “Action” drop-down menu, choose “Change Lane Feature.” Enter required information for: From, To Location, Type, Sub Type, Asc. Type, Width, and Lane No. Click “Preview Change” (see below image).

- A dialogue box will appear with a warning regarding the proposed changes. Double-check that the changes are correct and click “OK” (see below image).

- Click “Apply.” A dialogue box will appear. Click “OK” (see below image).

- Another dialogue box will appear indicating that the lane configuration changes have been applied and to refresh the inventory diagram to display the lane configuration changes. Click “OK” (see below image).

- Confirm the lane configuration changes were made successfully by running the Route History application as outlined in Viewing Paving & Maintenance History by Route.

3- Adding a New Route – performed by PDW user

Summary: This task uses the PDW software to create a route that does not currently exist in the PDW. Currently, the “Add a New Route” function is in the draft phases. This feature has not been used in a practical application but only for testing purposes.

- Open the PDW application.

- Select the “Data” button from the main menu and click “Inventory” from the drop-down list, then “Add a New Route” (see below image).

- A new window will automatically appear. Enter the required information for the route to be added: County, Municipal (if needed), and Prefix. Press “Continue” (see below image).

- A new window will automatically appear. Enter the required information for the route to be added: County, Municipal (if needed), Prefix, Route, Suffix, and Mainline. Click “Create Route” (see below image).

- A dialogue box will appear. Verify that all entered route information is correct and click “Yes” (see below image).

- Proceed to the “Define Segments” section of the screen. Enter the Beginning Milepost and Ending Milepost and click “Create Segments.” In the “Define a Section” section, enter the break points to split the route into different subsections, each with a different lane configuration. Click “Create Sections.”

- For each lane configuration break, define the lanes. Click the “View Lane Configuration” button to view the lanes graphically and click “Save Lane Configuration” to save the defined the lane configuration.

- Once all subsections have been defined (all items in the drop-down list have “defined” next to them), click the “Create Lanes and Lane Configuration” button to finish the entire process of creating a new route.

4- Extending a Route – performed by PDW user

Summary: This task uses the PDW software to extend an existing route beyond the current inventory. Currently, the “Extend a Route” function is in the draft phases. This feature has not been used in a practical application but only for testing purposes.

- Open the PDW application.

- Select the “Data” button from the main menu and click “Inventory” from the drop-down list, then “Extend an Existing Route” (see below image).

- A new window will automatically appear. Enter the required information for the route to be extended: County, Municipal (if needed), Prefix, Route, Direction, Beginning Milepost of Extension, and Ending Milepost of Extension. Press “Continue” (see below image).

- A dialogue box will appear confirming the beginning and ending mileposts for the extending portion of the existing route. Check that the mileposts are correct and click “Yes” (see below image).

- A new window will automatically appear. The “Define a Route” section will be grayed out because the extension is for an existing route. Enter the Beginning Milepost and Ending Milepost in the “Define Segments” section and click “Create Segments” (see below image).

- In the “Define a Section” section, enter the break points to split the route into different subsections, each with a different lane configuration. Click “Create Sections.”

- For each lane configuration break, define the lanes. Click the “View Lane Configuration” button to view the lanes graphically and click “Save Lane Configuration” to save the defined the lane configuration.

- Once all subsections have been defined (all items in the drop-down list have “defined” next to them), click the “Create Lanes and Lane Configuration” button to finish the entire process of extending an existing route.

## Adding to Project Engineer List

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to add the name of a Project Engineer (PE) to the SHA PE list within the Pavement Data Warehouse (PDW). The PDW tracks the Project Engineer for construction projects. In the event of a discrepancy in provided information or a question on the project the, PE is listed. During the data entry process, the name of the PE may not appear in the drop-down menu on the “Location” tab and requires an addition to the PE list. The list is an internal feature in the PDW system that can be modified to add additional names to the drop-down menu.

### Frequency

Adding the name of a PE to the SHA PE list within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to add the name of a PE to the SHA PE list.

### Resource Requirements

Adding the name of a PE to the SHA PE list within the PDW involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to add the name of a PE to the SHA PE list within the PDW is comprised of a single task:

- adding PE name to the SHA PE list in the PDW.

1- Adding PE name to the SHA PE list in the PDW – performed by PDW user

Summary: This task adds the name of a PE to the SHA PE list within the PDW so that it will appear in the drop-down menu of the “Location” tab.

- Open the PDW application.

- Navigate to Administration > PickList > Users > SHA PE (see below image).

- A new window will automatically appear. Scroll to the bottom of the list in the window and in the blank spot add the next number in sequential order, P.E. First & Last Name, and a 1. The 1 indicates that the PE is still active while a 0 indicates that the PE has retired (see below image).

- Click Save. Close any open tabs and close the PDW. Reopen the PDW application and confirm that the entered name appears in the drop-down menu on the Location tab.

## Viewing Paving & Maintenance History by Route

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to view paving and maintenance history by route within the Pavement Data Warehouse (PDW). The PDW application imported thousands of records from the old Access Database and new records are being created on a daily basis. Most projects fit together like puzzle pieces along the roadway; where project “A” stops, project “B” begins. The PDW has a feature that enables you to see all the construction history for a segment of roadway.

### Frequency

Viewing paving and maintenance history by route within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to view paving and maintenance history by route within the PDW. It should be utilized when lane configuration changes are made to a route in the PDW as outlined in Making Lane Configuration Changes.

### Resource Requirements

Viewing paving and maintenance history by route within the PDW involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to view paving and maintenance history by route within the PDW is comprised of a single task:

- viewing construction data by route in the PDW.

1- Viewing Construction Data by Route in the PDW – performed by PDW user

Summary: This task generates a report that displays all construction history (paving and maintenance) for a specified route.

- Open the PDW application.

- From the main menu, Navigate to Reporting > Construction Data > By Route (see below image).

- A new window will automatically appear. Enter the required location information: County, Municipal, Prefix, Route, and Direction. Click “Confirm” (see below image).

- Choose the Beginning Milepost and Ending Milepost for the section for which the construction history is needed. This information can be manually entered, or the closest reference point can be selected from the drop-down menu. Click “Confirm” (see below image).

- A lane configuration diagram will appear on the left-hand side of the screen. Select the Source drop-down menu and choose “Paving Only” (or “Paving & Maintenance” if desired). Click “New Report by Route” and a dialogue box will appear. Click “OK” (see below image).

- Click Larger View to display the entire report. Click Add Format to make the window easier to read (see below image).

## Promoting As Bid to Construction History

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to promote As Bid projects to Construction History within the Pavement Data Warehouse (PDW). Unit Cost – As Bid information is processed prior to construction; several pieces of data used for Unit Cost – As Bid are also needed in Construction History. The PDW application has a feature to easily promote these items.

### Frequency

Promoting As Bid projects to Construction History within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to promote As Bid projects to Construction History within the PDW.

### Resource Requirements

Promoting As Bid projects to Construction History within the PDW involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to promote As Bid projects to Construction History within the PDW is comprised of a single task:

- promoting As Bid project to construction history in the PDW.

1- Promoting As Bid Project to Construction History in the PDW – performed by PDW user

Summary: This task promotes As Bid contracts to Construction History in the PDW.

- Open the PDW application.

- Find the specific contract under “Unit Cost – As Bid.”

- Open the contract and click on the “As Bid_Main” tab (see below image).

- Click the green “Promote” button at the top of the screen. A dialogue box will appear with a warning about the promotion to construction history. Click “OK” (see below image).

- Another dialogue box will appear that reads “Promotion Done!” Click “OK.”

- Close the As-Bid screen and proceed to the specific contract in Paving History.

## Generating As Built Report

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to generate an As Built report within the Pavement Data Warehouse (PDW). The Maryland Construction Management System (MCMS) internal application has several different reports that it can generate. For Unit Cost – As Built, a report by month in which paving was completed is essential to the data processing of Unit Cost – As Built information. This feature is helpful with the three-month rule used by the Pavement Management (PM) team with Unit Cost – As Built.

### Frequency

Generating an As Built report within the PDW is a continuous process that occurs year-round.

### Purpose

The purpose of this SOP is to generate an As Built report within the PDW.

### Resource Requirements

Generating an As Built report within the PDW involves two people: (1) a user knowledgeable in the PDW software to run the application, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to generate an As Built report within the PDW is comprised of a single task:

- generating As Built report in the PDW

1- Generating As Built Report in the PDW – performed by PDW user

Summary: This task generates a report for As Built contracts in the PDW. The output of this task is an Excel spreadsheet with the desired As Built data.

- Open the PDW application.

- From the main menu, navigate to Reporting > MCMS.

- In the Select a MCMS Report box, select UC Report – Contract Not in UC Month (see below image).

- In the Select Year and Month box, indicate the Year and Month desired (see below image).

- The contracts that are not currently in the Unit Cost database for that month will automatically appear. Review the contracts and remove any that are not good candidates for Unit Cost by highlighting the contract row and clicking Remove (see below image).

- When only the desired contracts remain in the window, click Export to Excel. Name the document in the save prompt window, chose the proper save location, and click Save.

## HMA Tonnage Application

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to validate asphalt tonnage values in construction records. This procedure uses the HMA_Tonnage dataset provided by the Office of Materials Technology’s (OMT) Asphalt Technology Division and the submitted As-Built PM-003 forms to compare asphalt properties and tonnage values for all construction records entered into the Pavement Data Warehouse (PDW). The HMA_Tonnage dataset includes two tables that are reviewed by the DPT: “Actual” and “Estimated.” The “Actual” table is used for the data validation, while the “Estimated” table provides supporting data (such as routing number). Important comparisons taken from this procedure include the mix design and tonnage values.

### Frequency

The validation of asphalt tonnage values occurs quarterly as a minimum but can also occur as-requested.

### Purpose

The purpose of this SOP is to validate asphalt tonnage values in construction records by comparing the dataset provided by the OMT Asphalt Technology Division with the As-Built PM-003 forms.

### Resource Requirements

The validation of asphalt tonnage values involves one person: (1) a user, typically the DPT Team Leader (TL), knowledgeable in Microsoft Access and Excel, to validate the construction records and perform quality control (QC) checks. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT TL | Access user and QC | 1 | 1.0 |

### Procedure

The procedure to validate asphalt tonnage values in construction records is comprised of two components:

- validating asphalt tonnage in Microsoft Access, and

- performing QC of validated tables.

1- Validating Asphalt Tonnage in Microsoft Access – performed by DPT TL

Summary: This task validates asphalt tonnage values in construction records by comparing the dataset provided by the OMT Asphalt Technology Division with the As-Built PM-003 forms.

- Open Microsoft Access and select “Get External Data.” Browse and select the file at the following path: S:\SHA\OMT\PLT_PROD\2009\HMA-2009.accdb. Make sure “Import tables…” is selected and click “OK” (see below image)06/12/2019.

- Select the “Tables” tab and highlight “Actual” and “Estimated”, then click “OK” (see below image).

- Select the working directory: PMtestData\Construction History\HMA Tonnage\ 2009\2009 4Q (year & quarter).

- Add new fields in “Actual” table for “FMIS”, “Fund”, and “District” (see below image).

- Select Data Type for new fields by clicking on the “Design” tab along the top ribbon and changing the “Data Type” column to the appropriate type for each new field (see below image).

- Return to Access table and hide all fields except: Mix Number, Contract Number, FMIS, Production Date, Fund, Tonnage/Actual, District (see below image).

- Sort table by Contract and Production Date (see below image).

- Populate Fund Type (currently manual – see below image). In the future, could be automated with OOF supplied FMIS Data # spreadsheet.

- Clean up Contract and FMIS fields (see below image).

- Populate District field (currently manual – see below image). In the future, could be automated with OOF supplied FMIS Data # spreadsheet.

2- Performing QC of Validated Tables – performed by DPT TL

Summary: This task performs QC checks on the validated tonnage tables.

- Locate, analyze, and eliminate duplicates (look at Contract, FMIS, and Tonnage/Actual fields – see below image).

- Complete Validation spreadsheet (see below image).

- Post completed files to the following location: Pmdata\01 Report Data\KPA Report (see below image).

- Notify Assistant Division Chief (ADC) and Data Analysis Team (DAT) via email with Validation spreadsheet attached (see below image).

2- Performing QC of Validated Tables – performed by DPT TL

Summary: This task preforms QC checks on the HMA tonnage validated tables.

- Check the final HMA tonnage validated tables for accuracy and completeness.

## KPA Reports

### General

This section describes the standard operating procedures (SOPs) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to manage and maintain the KPA (Key Performance Area) series of tables and views within the Pavement Data Warehouse (PDW). The KPA database objects are a sophisticated set of Oracle database structures in the PAV_CONHIST schema designed to manage, aggregate, and report on construction and preservation data for pavement management activities. These tables and views serve as the data backbone for System Preservation Reports and related analytics, providing essential information for performance monitoring, trend analysis, and strategic planning. The KPA system functions as an Extract, Transform, Load (ETL) layer that extracts raw data from operational construction history tables, transforms it by adding administrative attributes, calculates ratios and aggregations, assigns construction type classifications, and loads processed data into reporting-ready tables and views while maintaining historical snapshots for comprehensive analysis.

The KPA tables are integrated with multiple source systems including construction history data (CONHIST_MAIN), unit cost project data (UNIT_COST_MAIN), location and lane configuration data, and various lookup tables containing administrative attributes such as functional classification, government control, and median type information. The system maintains both current and historical data to support year-over-year trend analysis and long-term performance evaluation.

### Frequency

The KPA tables and views are updated on a monthly basis to ensure current and accurate data is available for System Preservation Reports and other pavement management activities. This monthly update cycle ensures that recent construction activities, cost data, and maintenance actions are reflected in the reporting system in a timely manner.

### Purpose

The purpose of this SOP is to maintain and update the KPA series of tables and views to support System Preservation Reports, construction data analysis, unit cost reporting, maintenance tracking, and pavement management decision-making. The KPA system provides essential data aggregation and reporting capabilities that enable SHA to monitor construction accomplishments, track maintenance activities, analyze unit costs, evaluate performance trends, and support strategic planning for pavement preservation and improvement programs.

### Resource Requirements

The maintenance and update of KPA tables involves staff with specialized knowledge of Oracle database systems, PL/SQL programming, and pavement management data structures. The process requires understanding of construction history data relationships, unit cost analysis, maintenance reporting requirements, and the complex data flow between source systems and reporting tables. The estimated effort levels represent the total time required to complete the monthly update process and ensure data quality and accuracy.

| Position | Function | Resources | Effort Level | Additional Details |
| --- | --- | --- | --- | --- |
| DPT Staff | Oracle database administrator and PL/SQL developer | 1 | Monthly execution and monitoring | Executes monthly update procedures and performs quality control |
| DPT TL | Technical supervisor and data validation specialist | 1 | As required (AR) | Provides guidance, reviews results, and resolves technical issues |

### Procedure

The KPA Reports system utilizes a comprehensive set of Oracle database tables, views, and PL/SQL packages to process and aggregate construction and maintenance data for reporting purposes. The system is organized into core data tables, reporting views, and specialized database objects that work together to provide comprehensive construction and preservation reporting capabilities. The update process follows a specific sequence to ensure data consistency and referential integrity across all related tables and views.

#### Main KPA Database Objects

The KPA system consists of several categories of database objects, each serving specific functions in the overall data processing and reporting workflow:

Core Data Tables: 1. KPA_UNITS_TABLE - Stores unit-level project data with one record per FMIS number, including contract information, county designation, lane miles, bid amounts, project type classification, construction categories, and calculated ratios for analysis and reporting 2. KPA_ITEMS_TABLE - Stores detailed item-level cost data for each project, providing comprehensive breakdown of construction costs by individual line items, quantities, and unit prices for financial analysis and cost tracking 3. UNIT_CONHIST - Stores construction history data at the unit level, including detailed lane information, treatment specifications, contract details, and construction type assignments for historical tracking and performance analysis 4. KPA_MAINT_LOC, KPA_MAINT_FUNC, KPA_MAINT_TABLE - Three-tier hierarchical structure for maintenance data aggregation and reporting, incorporating functional classification and government control attributes for comprehensive maintenance tracking

Reporting Views: 1. KPA_SECTION - A comprehensive view providing detailed section-level information including route identification, direction, lane number, and administrative fields such as functional class, NHS code, government control, and median type for regulatory and planning purposes 2. KPA_HISTORY - Stores historical construction data across multiple years, used for reporting and trend analysis to support long-term planning and performance evaluation 3. KPA_CONHIST - A simplified view of historical construction data from KPA_HISTORY table, providing core fields including fund, contract, FMIS, section ID, layer information, thickness, material details, and construction type for streamlined reporting 4. KPA_CONHIST_DETAILED - A comprehensive view that joins KPA_SECTION, KPA_CONHIST, and MATERIAL_CATEGORY_VIEW to provide detailed construction history with full section information, administrative attributes, and complete material specifications including mix methods, binder types, and traffic levels 5. KPA_MAINT - A maintenance reporting view that provides calculated lane miles, square yards, and quantities based on ratio calculations from KPA_MAINT_TABLE, including functional class, government control, and material information for maintenance activities

Specialized Views: 1. ANNUAL_UNITCOST_UPDATE_INPUT - Specialized view designed for annual unit cost updates that combines data from MCMS (Maryland Construction Management System) and unit cost databases to support cost analysis and budget planning

#### PL/SQL Package Operations

The KPA system utilizes three main PL/SQL packages that orchestrate the data processing workflow:

1. KPA_UNIT_COST_PKG

This package is responsible for populating the core KPA tables from source construction and unit cost data. The package includes two main procedures:

- MAIN Procedure: Performs comprehensive data processing by truncating and repopulating KPA_UNITS_TABLE, KPA_ITEMS_TABLE, and UNIT_CONHIST from source tables including UNIT_COST_MAIN, LOCATION_VIEW, UNITS, and ITEMS. The procedure processes construction data from multiple sources, calculates lane miles and cost aggregations, and ensures data consistency across all related tables.

- ASSIGN_CONSTRUCTION_CATEGORY Procedure: Performs sophisticated analysis to assign construction type classifications and calculate ratios for each unit by analyzing lane miles, treatments, and construction activities from CONHIST_MAIN and related configuration tables. This procedure determines the primary construction type for each project based on the distribution of construction activities and calculates percentage ratios for reporting purposes.

2. KPA_PROFILE (Master Orchestration Package)

This package serves as the master controller for the entire KPA update process, coordinating the execution of all related procedures and maintaining performance monitoring. The package includes several key procedures:

- RUN Procedure: Executes the complete KPA update workflow by calling UNIT_COST, MAINT, and PAVHIST procedures in the correct sequence while monitoring execution time and performance metrics for each component.

- UNIT_COST Procedure: Initiates the unit cost data processing by calling KPA_UNIT_COST_PKG.MAIN to update all unit cost related tables and ensure current construction cost data is available for analysis.

- MAINT Procedure: Initiates the maintenance data processing by calling KPA_MAINT_PKG.MAIN to update all maintenance related tables and ensure current maintenance activity data is captured for reporting.

- PAVHIST Procedure: Executes comprehensive pavement history processing including data quality checks through PDW_CHECKS.MAIN, validates lane mile point consistency, truncates and repopulates various construction and section tables, and calls SECTION.MAIN to update section-level data for reporting.

- Reporting Procedures (REPORTS_CONHIST, REPORTS_UNITS, REPORTS_ITEMS, REPORTS_SECTION, REPORTS_TONNAGE): Generate specialized reporting cursors for different data categories including construction history, unit costs, item costs, section data, and tonnage information to support various reporting requirements and analytical needs.

3. KPA_MAINT_PKG

This package processes maintenance data through a three-step hierarchical approach that progressively adds administrative attributes and calculates reporting ratios:

- MAIN Procedure: Orchestrates the complete maintenance data processing workflow by executing the three-step hierarchy in the correct sequence and ensuring data quality throughout the process.

- Step 1 - Base Maintenance Data Processing: Populates KPA_MAINT_LOC from CONHIST_MAIN where APP_SOURCE_ID equals 5 (maintenance projects), extracting basic maintenance location data including coordinates, lane information, material descriptions, and completion dates.

- FUNC Procedure (Step 2): Enhances the maintenance data by adding functional classification information from NEWHMIS2 administrative tables, creating KPA_MAINT_FUNC with detailed functional class attributes for regulatory and planning purposes.

- GOVT Procedure (Step 3): Further enhances the data by adding government control information, creating the final KPA_MAINT_TABLE with comprehensive administrative attributes including government control codes and descriptions.

- PER_RATIO Procedure: Calculates lane mile ratios for maintenance reporting by analyzing the distribution of maintenance activities across lanes and projects, ensuring accurate proportional reporting for budget allocation and performance measurement.

#### Execution Sequence and Data Flow

The KPA process follows a strictly defined execution sequence to ensure data consistency and referential integrity. This order is enforced by running KPA_PROFILE.RUN, which calls each package in sequence to ensure data consistency:

- UNIT_COST Processing: Calls KPA_UNIT_COST_PKG.MAIN to process all unit cost projects, construction history, and related cost data from source systems into standardized reporting tables.

- MAINT Processing: Calls KPA_MAINT_PKG.MAIN to process maintenance data through the three-tier hierarchy, adding administrative attributes and calculating reporting ratios.

- PAVHIST Processing: Executes comprehensive data quality checks, validates lane mile point consistency, and calls SECTION.MAIN to process pavement history and section data for reporting purposes.

This sequence ensures that unit cost data is processed first, followed by maintenance data processing, and finally comprehensive pavement history processing that depends on the previously processed data.

#### Source Table Relationships

The KPA system integrates data from multiple source tables within the PDW:

Primary Source Tables: - CONHIST_MAIN: Main construction history table containing contract data, construction dates, and project information (APP_SOURCE_ID = 1 for unit cost projects, APP_SOURCE_ID = 5 for maintenance projects) - UNIT_COST_MAIN: Comprehensive unit cost project data including contract numbers, FMIS information, letting dates, and project classifications - LOCATION_MAIN: Location data with precise coordinates, Linear Referencing System (LRS) information, and spatial relationships - LAYER_CONFIGURATION, LANE_CONFIGURATION: Detailed treatment specifications, layer information, and lane configuration data for construction activities - UNITS, ITEMS, MEASUREMENTS: Cost data, quantity information, and measurement specifications for financial analysis and reporting - CONS_FNGM: Section-level administrative data including functional class, government control, and other regulatory attributes (referenced by KPA_SECTION view)

Administrative Source Tables: - NEWHMIS2 Administrative Tables: Functional classification, government control, and other regulatory data from the Highway Information Management System - LU_FEATURE_SUB_TYPE, LU_MATERIAL_CATEGORY, LU_LAYER_TYPE: Lookup tables containing standardized codes and descriptions for materials, treatments, and construction categories

### Troubleshooting

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| KPA tables not updating properly | PL/SQL package execution failure | Check Oracle error logs and verify source table availability; restart KPA_PROFILE.RUN procedure |
| Data inconsistencies between KPA tables | Execution sequence not followed correctly | Ensure UNIT_COST runs before MAINT and PAVHIST; verify all procedures complete successfully |
| Missing construction history data | Source tables (CONHIST_MAIN) not current | Verify construction history data entry is complete and QC_ID_CHECK fields are populated |
| Performance issues during updates | Large data volumes or system constraints | Monitor execution times using KPA_PROFILE performance timing; consider breaking updates into smaller batches |
| Ratio calculations incorrect | Lane mile or measurement data issues | Verify MEASUREMENTS table data quality and lane configuration accuracy in source systems |

### References

- Oracle Database documentation for PL/SQL package development and maintenance

- PDW Data Processing Team procedures for construction history data entry

- SHA System Preservation Report requirements and specifications

- Highway Information Management System (HMIS) administrative data standards

- Maryland Construction Management System (MCMS) integration procedures

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Jul-23 | Aditya R. | Document format converted to plain text. Updated KPA Reports section with comprehensive procedures for managing KPA tables and views. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and EDW with PDW |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart. Minor editorial changes |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Data Processing

Click to go to Loading of ARAN Data into Vision

Click to go to ARAN Submittal QA

Click to go to Post-Processing of ARAN GPS Data

Click to go to Running Global LCMS Processor

Click to go to Running Roughness Processor

Click to go to Route Matching

Click to go to Performing 100% Drive Through

Click to go to Running Rut Processor

Click to go to Running Classification and Rating Processors

Click to go to Running Vibing Processor

Click to go to Running Thumbnail Creator

Click to go to Running Curve Fit Batch Processor

Click to go to Reporting and Uploading to Oracle

Click to go to Reformatting ARAN Hard Drives

Click to go to Creating PAGDQC Report

Click to go to Running Routing Importer

Click to go to Changing Latitude/Longitude

Click to go to Accessing Electronic ARAN Logs

Click to go to Using eGIS

Click to go to Changing Image Banner or Logo

Click to go to Year-End ARAN Data Closeout

Click to go to Route Matching - Transfers

Click to go to Loading Pavement Surface Friction Data

Click to go to Skid Submittal QC

## Introduction

This section describes the set of standard operating procedures (SOP) conducted mainly by staff from the Data Processing Team (DPT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to process and manage data elements collected in the field with the ARAN survey vans and skid trailers by the Field Explorations Division (FED). The SOPs contained in this section begin with the loading of data into the Vision database. The subsequent processes are nearly all completed using the Vision program and involve running various “processors” and performing QC/QA checks on the data. The processing is completed with the transfer of the Vision database into Oracle.

The various processors run in the Vision program include: GPS (in conjunction with the POSPac program), Laser Crack Measurement System (LCMS), Roughness, Rut, Classification and Rating, Vibing, and Curve Fit. These processors are run for batches of data as they are delivered by the FED throughout the data collection season. The data processing is also supplemented by other manual procedures, including the matching of routes and performing a drive through of LCMS data by a user in Vision.

This section also describes supplemental procedures designed to support the SOPs described above. This set includes eight SOPs: creating a Pavement and Geotechnical Division Quality Control (PAGDQC) report, running the routing importer, accessing electronic ARAN logs, changing latitude/longitude, using eGIS, modifying the image banner or logo on the collected images, performing the year-end ARAN data closeout, and performing transfers during route matching.

The final two SOPs in this chapter describe the loading and QC of collected skid data. Once the skid data is loaded into the PDW, it is ready to be used for trend analysis. No further processing is performed on skid data.

Figure 8 shows a flowchart of the Data Processing SOPs.

Figure 8: Data Processing SOPs Flowchart

## Loading of ARAN Data into Vision

### General

This section describes the standard operating procedure (SOP) followed by the Maryland State Highway Administration’s (MD SHA’s) Data Processing Team (DPT) to load final deliverable submittals from Office of Materials Technology’s (OMT) Field Explorations Division (FED) as acquired from their Automatic Road Analyzer (ARAN) pavement survey data and imagery collection program. Using vendor supplied DCS loading software, routines are performed to load the deliveries into a network imagery (Videolog) server and SQL database(s). Quality control (QC) and quality assurance (QA) routines are then performed on the loading process . The loading of the deliverables is performed in batches usually every 4 – 5 workdays throughout the annual data collection season and anytime year-round. This SOP applies to the data loading and QC/QA of each individual deliverable batch.

Each submittal consists of a survey data SQL database, Right-of-Way and Pavement imagery files, daily field QC files, and orientation (GPS) files. They are delivered by FED members to DPT members in six (or more) 1-terabyte (TB) external hard-disks. These disks are delivered to the DPT as-extracted from the ARAN van computers without any further processing. Each submittal consists of an original and a duplicate disk for the three different types of data and imagery sets. The three data elements are named (1) “ROW”, which contains Right-of-Way (ROW) images and FED quality control information, (2) “PVT”, which contains Laser Crack Measurement System (LCMS) images, and (3) “SVR”, which contains left ROW camera images and all other measurements taken by other sensors (i.e., data measured by grade sensors, longitudinal profilers, inertial measurement unit, and global positioning system). More disks may be introduced into a batch when the amount of collected data of a particular dataset for the batch is larger than 1-TB.

Once the field survey data database is uploaded into the Vision software database(s) and imagery links are established with DCS loading software, Vision post-processing and analysis routines are performed. The results are then imported into the MD SHA Pavement Management System (PMS).

### Frequency

The loading of FED ARAN submittals into the Videolog server and Vision database is completed every time a delivery is received by the DPT, which typically occurs every 4 to 5 days. Assuming an average of 60 lane miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data and imagery. The actual frequency varies according to the proximity of the collected routes to OMT’s Hanover, MD office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring potential recollection. FED’s annual ARAN collection season typically runs from April to October but deliveries sometimes occur year-round.

### Purpose

The purpose of this SOP is to :

- record received disks with ARAN collected data and imagery and upload daily log sheets into the Pavement Data Warehouse (PDW),

- upload the measurements and imagery data collected by the ARAN equipment in the field to the Vision database using the Roadware Data Control Software (DCS),

- load the “Pavement and Geotechnical Division Quality Control” (PAGDQC) report06/12/2019 to the PDW, and

- confirm that the ARAN collected data is successfully loaded into the Vision database and, if necessary, perform corrective actions.

### Resource Requirements

The loading of ARAN data involves two people: (1) a user knowledgeable in Roadware DCS software to load the ARAN data batch and perform QC, and (2) a supervisor to perform QA of the loading process. The estimated effort levels in the table below represent the total time, in man-hours, to complete the data loading and QC/QA processes. These time estimates assume average batch quantities and that no issues need to be addressed during the QC and QA.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | DCS user and QC | 1 | 4.0 |
| DPT Team Leader (TL) | Supervisor - QA checks | 1 | 0.5 |

### Procedure

The procedure followed by the DPT staff for loading the ARAN data and performing QC/QA is comprised of the following tasks:

- scanning of daily log sheets,

- recording of received ARAN drives,

- loading of ARAN Data,

- final data loading QC,

- creating PAGDQC report,

- DPT TL notification, and

- data loading QA.

1- Scanning of Daily Log Sheets – to be completed by DPT Staff

Summary: This task creates digital copies of the daily log sheets completed by the FED during data collection.

- Scan the paper copies of the field daily log sheets delivered by the FED along with the 1-TB hard disks.

- Name each of the scanned files as “Rec[arrival date][ARAN Number]” (e.g., “Rec050517A3.pdf”).

- Save each of the scanned sheets as a PDF in the server folder for the corresponding data collection year: \\shahanartico2\shaomtvideolog\ [YEAR]\Data\Field Logs (e.g., “\\shahanartico2\shaomtvideolog\2017\Data\ Field Logs”).

2- Recording of Received ARAN Drives – to be completed by DPT Staff

Summary: This task records the disks with ARAN collected data received from the FED into the PDW.

- Open the “Data Warehouse” program and click on “Data”, “Aran”, “Record Drive and First Image” (see below image).

“ARAN Drive Info” window will appear. The only two boxes in the window to be completed are “Record Drive Info” and “Daily Log File” (see below image).

- Obtain labels of the received disks from the daily log sheets. Each disk with the ARAN data collected in the field is labeled using two numbers separated with a dash mark (e.g., “173-17”). The first number refers to the order at which the disk was used while the second number refers to the year the disk was purchased (some years the year of purchase is on the left while in other years it’s on the right). The labels of the disks (original(s) and duplicate(s)) for each of the three sets of data elements, referred to by the FED as “server” (SVR), “pavement” (PVT), and “right-of-way” (ROW), are reported in columns 4 to 9, as shown in the image below. In some cases, the ARAN operators omit to write down the year of purchase part of the label (as is the case in the daily log sheet shown as example). The labels to be recorded into the Data Warehouse program must contain both the disk number and the purchase year (e.g., the labels of the hard disk from the daily log sheet in the image are 33-17 and 35-17 for SVR, 126-17 and 127-17 for PVT, and 48-17 and 46-17 for ROW).

- Insert one of the “original” SVR disks into the external drive docking station.

- Identify what drive the disk was connected to. This information can be found in the “Windows Explorer” (see below image). If no new disk appears in the computer (“New Volume” does not appear), try to re-insert the disk into docking station until it is detected. If the disk is still not detected, contact SHA Information Technology (IT) staff for assistance.

- In the Data Warehouse program, click the “Browse” button under the “Record Drive Info” box (see below image).

- Select the computer drive to which the disk with the ARAN collected data was connected to (computer drive “F:/” in the image shown as example).

- Type the complete label of the connected disk (i.e., including both the disk number and the purchase year) into the “Please Input Drive #” field of the “Record Drive Info” box (see below image).

- Click on the “Record” button. The “ARAN Drive Info” application will automatically populate the “Daily Log File”. Once the following message window appears, click on the “OK” button (see below image).

- Repeat Step 8 to Step 11 for recording the label of the corresponding “duplicate” disk of the dataset being recorded (i.e., SVR data in first iteration). The label of the duplicate disk is recorded with the original disk of the corresponding dataset connected so the same information will be inputted into the “Please locate the driver” - only the label inputted into the “Please input Drive #” will be different.

- Unplug disk from external docking station.

- If more than one “original” disk with the analyzed dataset was received (e.g., SVR), repeat Step 6 to Step 13 until the labels of all the original and duplicates disks are recorded.

- Repeat Step 6 to Step 13 twice; one time for recording the labels of the original and duplicate disks with the PVT data and the second time for the disks with the ROW data.

3- Loading of ARAN data – to be completed by DPT Staff

Summary: This task uploads the measurements and imagery data collected by the ARAN equipment in the field to the Vision database using the Roadware DCS and identifies any errors which may occur during the data upload.

- Connect the original (or duplicate, these will produce the same result) 1-TB hard disks with each of the three sets of data elements produced by the ARAN survey vans to the docking station of the computer with the Roadware DCS software. All three drives must be connected.

- Navigate to: \\shahanartico2\shaomtvideolog\2017\Data. Open the appropriate ARAN folder (named as “[YY]_A[#ARAN]”; e.g., “17_A3” for ARAN 3 and “17_A4” for ARAN 4. Right click and make a new folder. Name the folder “Rec [datereceived]” (see below image).

- Copy the folder named “Posdata” from the “SVR” hard disk drive to the “Rec” folder that was just created (see below image).

- Copy the log and Pavement QC folders from the “ROW” hard disk drive to the “Rec” folder that was just created (see below image).

- Open the Roadware DCS.exe program. The default location of the Roadware DCS software is “C:\Program Files (x86)\Fugro Roadware\Data Control Software\Roadware.DCS.exe”. A desktop shortcut of the software may also have been created, however, confirm that the shortcut directs to the Roadware.DCS.exe file as it is sometimes linked to an incorrect file. Click the “Cancel” button when the dialogue box appears (see below image).

- The program will open. The “Select a Project” window should have project “SHA_MD_VISION_PROJECT1” listed under the “Project Name” column, as shown in the image below. If project “SHA_MD_VISION_PROJECT1” is not listed, contact the SHA IT staff for assistance. Otherwise, click on the “OK” button to proceed (see below image).

- Confirm that all connected 1-TB disks with ARAN data are detected by the DCS software. For this, check if all the disks are listed after the “Drives to scan” message (see below image). The name of the computer drives to which the disks are connected (“F:/”, “G:/” and “I:/” in the image) are variable. They will be randomly assigned to a letter from “E:/” to “Z:/”. Also, the plug to which they are connected is not relevant.

  - If one or more of the disks are not listed in the “Drives to scan” line, click on the “Scanning Options” link (appears in bold, blue text after the list of connected disks). Select all computer drives to which the disks with ARAN data are connected to and click on “OK” (see below image). If one, or more, of the connected disks are not detected by the computer, reconnect the disk to a different slot of the docking station and confirm that it was detected by the computer. If the disk is still not detected by the computer connect the duplicate disk to the docking station. If neither the original nor the duplicate disks are detectable by the computer after repeated trials, suspend the procedure and communicate issue with Supervisor and FED TL for remedial actions (e.g., recollection of data).

- Once all the drives with connected ARAN disks are listed on the “Drives to Scan” line, click on the “Scan for Drives” button. Next, the DCS software will perform QC checks to confirm that all the data elements collected by the ARAN survey vans are present in the connected disks (see below image).

- Scan for Drives Button

- The DCS software will finish scanning the connected drives (see below image).

- DCS Scanning Results

  - First, read the information provided in the “Drive QC Results” box. If all data elements collected by the ARAN survey vans were contained in the connected disks, then the “Drive QC Results” box would be empty. Otherwise, a list of error messages listing the data elements missing in the connected disks will appear (as shown in the image above. For example, the error messages listed from the “Drive QC Results” box shown in the image indicate that the data elements in the SVR and Pavement drives are missing). If one or more data elements were listed as missing, reconnect the corresponding disks to the docking station and repeat Steps 19 and 20. If data elements are still reported as missing in the “Drive QC Results” box, connect the duplicate of the corresponding disks. If it is not possible to detect the missing data elements from either the original or the duplicate disks, report the need for recollecting the routes contained in the affected disks.

  - Once no missing elements are listed in the “Drive QC Results” box, check that the default upload path in the “Upload Data Files and Images” box is \\shahanartico2\shaomtvideolog\[YEAR]\Video. Otherwise, type in that upload path.

  - Next compare the scanned copy of the daily log data to the filter in the imported data. For this, click on the “Filter” icon of the first row (see below image).

The following window will appear.

- With the previous window open, access the electronic daily logs (see below image). To access the scanned daily logs, navigate to (\\shahanartico2\shaomtvideolog\[YEAR]\Data\[YY]_A[ARAN Number]\Rec[MMDDYYYY]). Compare the information in both windows. The “Session” column in Vision and the “FILENAME” column in Excel are the same (YYYMMDD.24hrTime). Deselect any files with an “X” in the “Status” column in Excel. “DUMMY” files will always have an “X” in the “Status” column and “999921” in the “HEADER” column.

- Once you have the correct files selected, click the Start button in Vision. The “Upload and Import Wizard Progress” window will open (see below image). Click Details to see what step the program has completed.

- After the program is complete, click the View Results button (see below image).

Click the `Details` tab on the left-hand window to view the results of the import. Check window for errors (see below images for examples).
-  If there are no errors, proceed to Step 27.
-  If there are some errors, but not errors with every run, reimport the runs with errors by returning to Step 25.
-  If every run received an error (such as all runs saying, "no images found in database," suspend data import and inform FED that recollection may be necessary.

Import Result – Summary with no errors

Import Result – Detail with no errors

Import Result – Detail “no images found” errors (recollection may be necessary)

4- Final Data Loading QC – to be completed by DPT Staff

Summary: This task ensures that the collected “ROW” and “Left” images collected with the ARAN equipment in the field are acceptable and flags image data sets which may be incomplete or of unacceptable quality.

- Open Vision. Select the “Images” drop-down menu and select “Left”. “Left” viewer tab will appear (see below image). Images will only be displayed if a file is selected. If the window opens and there is no image, make sure a file is selected in the Section Explorer window.

- Select the “Images” drop-down menu and select “ROW”. “ROW” viewer tab will appear (see below image). Images will only be displayed if a file is selected. If the window opens and there is no image, make sure a file is selected in the Section Explorer window

- Arrange “Section Explorer”, “ROW”, and “Left” windows so that they can be viewed simultaneously on the screen. Right-click on the “Section Explorer” tab and select “New Horizontal Tab Group” (see below image). The “Section Explorer” window will move to the bottom of the Vision display.

- Right-click on the “ROW” tab and select “New Vertical Tab Group.” The “ROW” window will move to the right of the Vision display, next to the “Left” window (see below image).

- Select one of the imported files in the “Section Explorer” window. The first image for both cameras will appear in the “Left” and “ROW” windows. Check the image for quality. Click the “skip to last” button to view the last image (see below image). Check the image for quality. Check at least one file for each day of data collection.

  - If images are of acceptable, proceed to Step 32.

  - If images are missing (more than half) or otherwise not of acceptable quality, return to Step 25 and reimport the files.

5- Creating and Importing PAGDQC Report – to be completed by DPT Staff

Summary: This task creates and populates a PAGD_QC_CSV file for all collection runs imported into the Vision database for each FED submission. The file is then imported into the PDW to update the Edit/View Unique Run Status program to allow tracking, status, and notes of each collection run (FED QC/QA, Route Matching QC/QA, etc.).

- Refer to Creating PAGDQC Report for creation and uploading of the PAGD_QC Report. This step is essential for FED QC/QA staff can provide their feedback for each collection run as part of their QC/QA review. The FED review will determine if a collection run requires recollection and if it’s ready for office post-processing.

6- DPT TL Notification – to be completed by DPT Staff

Summary: This task notifies the DPT TL of completion of data loading and results from QC.

- Notify DPT TL of completion of data loading and results from QC.

  - Compose an email to the DPT TL (cc the Assistant Division Chief for Pavement Management in the email).

  - Indicate that the received data has been loaded into the Vision database.

  - Attach the PAGD_QC Report and the PDF with the scanned ARAN Collection Daily Logs.

  - Use the following email subject: New Aran [ARAN NUMBER] Submission (e.g., “New ARAN 3 Submission”).

  - Include all problems or concerns with the received data found while performing the previous steps.

7- Data Loading QA and FED Notification – to be completed by DPT TL

Summary: This task confirms the completeness of the data loading and ensures the correct attachments were included in the notification email from DPT staff. This task also uploads the data via ProjectWise and alerts the FED TL when complete.

- Open the notification email sent from DPT Staff. Make sure the following files are provided as an attachment to the email:

  - FED QC Spreadsheet (PAGDQC Report renamed)

  - ARAN Field Log

If one or both of these files were not attached to the email, contact the DPT Staff that submitted the notification and request to resubmit email with correct files.

- Navigate to folder location of the FED daily QC folders on the network: \\shahanartico2\shaomtvideolog\[YEAR]\Data\[YY]_A[ARANNumber]\Rec[MMDDYYYY]. Confirm that one folder per data collection date in the batch (use the ARAN Field Log to get the list of data collection days in the batch) has been created and that they have the necessary files. See image below for the list of files expected to be populated in each daily collection folder.

- Use ProjectWise Explorer V8i to create folder in the corresponding data collection year within the FED’s ARAN ProjectWise directory with the following name “[ARANNUMBER][YYYYMMDD (for last day of collection in batch)]” (e.g., “A3_20170725”). Import all items into the created folder including the FED QC Spreadsheet, the ARAN Field Log, and the folders for each data collection date with the corresponding files.

- Send email to FED TL to communicate completion of data loading into the Vision Database as well as completion of uploading FED daily field QC files and sample imagery. Include the ProjectWise link of the uploaded folder in the email. This allows the FED TL to start their QA review (ARAN Submittal QA) to determine if any collection is invalid and/or requires recollection prior to the post-processing procedures.

## ARAN Submittal QA

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Field Explorations Division (FED) to perform quality assurance (QA) of the data collected using the Automatic Road Analyzer (ARAN) survey vans before those data are submitted to the Data Processing Team (DPT). The ARAN collected data are subject to additional quality checks by other teams of the SHA Office of Materials Technology (OMT) in subsequent stages of their Pavement Management System (PMS)06/12/2019.

### Frequency

The QA of an ARAN data submittal starts every time a batch of ARAN collected data is uploaded to a Vision Database by the DPT, which typically occurs every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the OMT office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is for the FED staff to assure that the data collected using the ARAN survey vans are of acceptable quality before they are delivered to other OMT teams and SHA offices for their use in various PMS applications. For example, Right-of-Way (ROW) images are submitted by the FED to SHA offices other than the OMT.

### Resource Requirements

The QA of the ARAN data submittal involves two people: (1) a reviewer of ARAN data who performs the QA checks and (2) a supervisor who, as required, makes decisions based on the QA results on whether or not to recollect data on one or more routes where data have already been collected. The estimated effort levels in the table below represent the total time, in man-hours, to perform the QA of the ARAN data submittal.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| FED Staff | Reviewer | 1 | 1.0-24.006/12/2019 |
| FED TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to perform QA of the ARAN data submittal is comprised of the following six tasks:

- preliminary actions,

- QA of International Roughness Index (IRI) data,

- QA of ARAN daily settings,

- QA of images,

- reporting of QA results, and

- decision to recollect.

The ARAN data elements inspected during the quality checks in this SOP are IRI measurements, settings used during data collection, and the different images produced by the system: Right-of-Way (ROW), Left Camera (Left), and laser crack measurement system range (LCMSRange) and intensity (LCMS3D) images. The QA of IRI data checks for completeness and validity of testing conditions. The QA of ARAN daily settings is performed to verify that the setting used during each data collection day were the ones intended. The QA of images checks for their completeness and fidelity.

The following steps describe the process conducted by members of the FED to QA the ARAN data submittal:

1- Preliminary Actions - performed by Reviewer

Summary: This task identifies the ARAN submittal to be checked, removes previously reviewed runs from QA list and initializes a new submission spreadsheet.

- Open the server folder containing the ARAN submittal data. A link to the ARAN submittal’s folder is provided in the email sent by the DPT to notify completion of the data loading into the Vision database (see Loading of ARAN Data into Vision). Clicking the link will automatically navigate to the submittal folder. If the link is not included in the completion notification, then contact the DPT TL to request information on the location of the server folder with the ARAN submittal data.

- Identify previously collected routes – Print a copy of the scanned field daily log and compare it with the log from the submittal immediately prior to the one in question for the corresponding ARAN survey van to detect if any of reported runs have already been reviewed. Duplicate runs are sometimes collected by the ARAN crew due to routing strategies and other reasons. Cross off the duplicate runs, if any.

- Initialize sheet for new submission in yearly spreadsheet (see image below). Run MS Excel and open the “QA yearly spreadsheet” corresponding to the specific ARAN survey van associated with the submittal under review. The QA yearly spreadsheet is named as “[YYYY]_Aran[VanNumber] [ReviewerInitials]” (e.g., “2017_Aran4_BE”) and it is stored in the reviewer’s computer. Create a new sheet and name it as “[ARAN_Number] [YYYYMMDD]” (e.g., “A3_20170807”). Once the new sheet has been named, copy the columns and sections for “FED QC Report,” “Settings”, and “Images” from the sheet with the last ARAN submittal information, paste them into the new sheet, and clear out each of the sections.

2- QA of IRI Data - performed by Reviewer

Summary: This task creates and populates a spreadsheet which evaluates the fidelity of the collected IRI data. The spreadsheet checks both completeness of IRI data and the speed at which the measurements were taken.

- Open the following two files using MS Excel:

  - “FED_QC_100000.csv,” located in the submittal’s folder (see Step 1), and

  - “IRI Review” spreadsheet, located on the Reviewer’s computer. This spreadsheet has the IRI QA checks pre-loaded.

- Copy all the data from the “FED_QC_1000000” spreadsheet (do not copy the column headers) and paste it into the “IRI Review” spreadsheet at cell “J3” (see image below). The IRI QA results for each run are reported on columns “Right IRI Check,” “Left IRI Check” and “Avg Speed.” The values in these three columns will refresh automatically once the new values are pasted. Flagged values are marked with a red cell background, as illustrated in the image below.

- Report every flagged run from the IRI completeness check or the IRI measuring speed check into the “FED QC Report” section of the “QA submittal review” spreadsheet by copying their “Unique Run” number and commenting on with the “Unique Run” number and the text “Bad IRI” (see below image). The IRI completeness check consists of reviewing the percentage of IRI values (either from the left or right sensor) missing for the run. Flagged values in the “Right IRI Check” and “Left IRI Check” columns indicate that 25% or more of the IRI measured data for the run are missing. The IRI measuring speed check is performed to detect if the speed at which the ARAN van collected the IRI values was under the acceptable threshold of 35 miles per hour. Flagged values in the “Avg Speed” column indicate the testing speed for the run was lower than acceptable. Once reporting of all flagged runs has been completed, save the “IRI Review” spreadsheet under the name “Aran_[ARAN_Number]_ [MMDD]_IRI_Fidelity” using the MS Excel file extension.

3- QA of ARAN Daily Settings - performed by Reviewer

Summary: This task creates and populates a spreadsheet which evaluates the accuracy of ARAN settings. The spreadsheet performs a side by side comparison of setting values for the data submittal and the pre-loaded functions. Conflicts with setting inputs are reported as errors.

- Open the ARAN daily settings file corresponding to the first data collection day of the submittal using a text editor software. The ARAN settings file for each data collection day is located in the sub-folder for the corresponding day within the server submittal’s folder (see Step 1). The ARAN settings files are named as “MD[Number]_Settings_[YYYYMMDD].xml” (e.g., “MD854682_Settings_ 20170713.xml).

- Open the ARAN settings review spreadsheet in MS Excel and select the sheet corresponding to the ARAN associated with the submittal data (e.g., ARAN 3). This spreadsheet contains the ideal setting values for each of the ARAN survey vans and it is used as the reference to check if the ARAN daily settings used in the field were the correct ones. The ARAN settings may differ between ARAN vans.

- Copy everything from the ARAN daily settings file, and paste it into the settings review spreadsheet in cell “A2” (see below image).

- Once the ARAN daily settings values have been pasted into the settings review spreadsheet, the spreadsheet will automatically perform a line-by-line comparison between setting values through its pre-loaded functions. The output from this comparison consists of the count of lines with differences in the settings, referred to as “Error Count” (as presented in the top right corner of the above image where 15 differences were found). Each line of the ARAN daily settings in conflict with the reference setting values is flagged in red color.

- Report the “Error Count” for the ARAN daily settings, as well as each of the line numbers in conflict with the reference settings, into the “Settings” section of the yearly spreadsheet separating by data collection date (as shown in the image below).

- Repeat Step 7 to Step 11 for each data collection day in the ARAN submittal data.

4- QA of Images - performed by Reviewer

Summary: This task uses Vision software to check for abnormalities in the collected images from the ARAN data submittal and reports any issues found in the “Submittal Review Spreadsheet.”

- Open Roadware Vision software06/12/2019. A screen with a list of projects will appear (see below image). Select project “OMT_ARAN_PROD_2017_1” and click on the “OK” button. If the project is not listed, Reviewer should contact SHA Information Technology (IT) staff for assistance with connecting the correct project folder to Reviewer’s account.

- Load or set up Vision workspace. If no workspace in Vision is saved, or if the workspace is to be changed, set up the workspace as preferred containing, as a minimum, the Section Explorer, the ROW, Left, LCMS3D, and LCMSRange images, and the map window. The various image windows are opened using the “Images” button on the menu ribbon. With the map window open, click on the “Main Menu”, “Base Maps” button within the “Map” window and add the basemap of choice (e.g., “Google Map”). Once workspace set up is finalized, save it using “File”, “Save Workspace” to make it available for future QA of Images. If a workspace in Vision is available, load it from “File”, “Select Workspace”.

- The “Section Explorer” will show the list of all runs collected for the project. Double click on the first (see above image).

- Find runs to review from the list of runs marked in the printed daily logs reported by the ARAN crew members. Ignore duplicate runs previously checked as a part of the previous submittal (crossed out runs in the daily log, as described in Step 2). For this:

  - Right click anywhere at the top bar in the “Section Explorer” window in Roadware Vision. Double click on time to reveal the data collection date for each unique run. Scroll up or down to the point at which the ARAN submittal begins, as indicated in the field daily log.

  - Type in code “1723”—if submittal data is from ARAN 3—or code “1724” —if submittal data is from ARAN 4—under the “Vehicle” column to filter out runs collected by ARAN vans other than the one corresponding to the submittal under review.

- Select the run to review and click on the “Play” button at the bottom of Roadware Vision to begin reviewing the images in the submittal. Set the image rate to 8 (every 1/8 frame of the submittal) and speed to ~80% (moderate percent). This should be done based on personal preference.

- Look for abnormalities during the visual inspection, such as:

  - Missing images,

  - Improper lighting (./images too bright or too dark),

  - Presence of spots blocking the vision—such as water drops, and

  - Other abnormalities in the images.

- Hit the “Stop” button every time an abnormality is detected and report the issue in the “Image QC” section of the Submittal Review Spreadsheet annotating the collection name and the frame at which the issue occurred (as shown in the image below).

- Repeat Step 17 and Step 18 for each run in the submittal until all issues for every run have been reported in the Submittal Review Spreadsheet.

5- Reporting of QA Results - performed by Reviewer

Summary: This task uses the Pavement Data Warehouse software to input the results of the previous checks for each run collected in the ARAN data submittal. This task also results in an email submission indicating the reviewer’s portion of the data submittal QA is complete.

- Open the Pavement Data Warehouse (PDW) software and navigate to “Data”, “ARAN”, “View/Edit Unique Run Status.”

- The Pavement Data Warehouse will populate with list of runs completed (see below image). Complete the QA information for each data collection run of the submittal. (generally each run has one route)

  - Select “Year”, “County”, and “Test Date” to find the corresponding runs for each submittal (see above image).

  - Double click on the reviewed runs, type in relevant notes (add a “Standard Note” using drop-down list or a custom note in the “Add Note” field), press Save, and check off the “QC reviewed” button in the next window that opens (see below image).

Repeat Step 21 until the QA information for each run in the ARAN data submittal is inputted into the Pavement Data Warehouse06/12/2019.

- Reply to the original notification email from the DPT (see Step 1) communicating that the QA of the submittal has been completed (see below image).

  - Post a link to the submittal’s server folder in reviewer’s email by copying the link that was provided in the original email.

  - Attach Reviewer’s Submittal Review Spreadsheet.

  - Attach Reviewer’s IRI Fidelity spreadsheet.

  - Include list of major issues detected during conduct of QA review checks, if any, performed on the ARAN data submittal that Reviewer considers important to highlight.

  - Submit email to Supervisor and copy to DPT Team Leader and DAT Team Leader.

6- Decision To Recollect - performed by Supervisor

Summary: This task performs necessary checks of the reviewer’s tasks for the review of an ARAN data submittal.

- Upon receipt of QA submittal email, Supervisor should review errors reported and decide if data should be recollected. The decision to recollect is based on several factors:

  - Data type and completeness.

    - Settings – some setting errors are expected, such as DMI calibration factor (would be different if vehicle was calibrated recently), others are unexpected and could have a more critical influence on data.

    - IRI – check the percentage of incomplete data. If it is determined that a large quantity of data is missing, recollection may be necessary.

    - Video – completeness is an essential component of the deliverable. Decision to recollect is based on percentage of data that is missing or of poor quality. Some cloudiness or minor visual obstructions may be allowable whereas a large amount of light (washed out) images may be a candidate for recollection.

  - Extent to which the data quality is affected by errors.

  - Length and type of road where errors occur.

    - Interstate or HPMS sections always get recollected (important to State).

    - Smaller local roads – recollection is typically based on the earlier stated factors, but sometimes decision is made that recollection is not necessary.

  - Extent of error – Determine if errors are of a localized nature or if they exist across a large amount of data.

When decisions are made to recollect, communicate list of sections where data should be recollected to the ARAN crew.

## Post-Processing of ARAN GPS Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to process the Global Positioning System (GPS) data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The post-processing of ARAN GPS data is performed in batches throughout the annual data collection season. This SOP applies to the GPS post-processing of each individual ARAN data batch. The post-processing of ARAN GPS data is performed after completion of loading the ARAN data into Vision as outlined in Loading of ARAN Data into Vision. This process results in the generation of ARAN GPS location data for import into the Vision database.

### Frequency

The post-processing of GPS data is completed every time a batch of ARAN collected data is received by the DPT, which typically occurs every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to process the ARAN GPS data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The processing of GPS data involves two people: (1) a user knowledgeable in Roadware Vision and POSPac MMS 6.206/12/2019 software to process the GPS data and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the GPS processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | POSPac/DCS user and QC | 1 | 4.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The POSPac computer on the ARAN van starts collecting data as soon as it is powered on. The data files are around 8 MB in size – whenever a file reaches that general file size, a new file is automatically created. If the POSPac computer is running for the entire data collection day, there would be approximately 160 to 180 daily files. During a typical data collection day, the computer is shut down at least once, resulting in two sets of files (or more depending on the number of shut downs). The image below shows an example set of daily files.

Each set needs to be processed and imported separately. If the wrong POSPac data is imported into the wrong set of files, the GPS mapping for those days will be incorrect and will need reprocessing. The MS Excel Macro code for sorting the list of POSPac file names has been included as an appendix (see POSPac List Files Macro).

The procedure to process ARAN GPS data and perform QC/QA is comprised of the following three tasks:

- processing GPS data,

- performing QC of the processed GPS data, and

- performing QA of the processed GPS data.

1- Processing GPS Data – performed by POSPac and DCS user

Summary: This task uses the POSPac software to process GPS files from the ARAN data submittal and Roadware DCS to import the results into Vision.

- Open a blank document in MS Excel. Click on the “Developer” tab. If the tab is not shown, consult the help tool within Excel for directions on how to enable it. Click the “Visual Basic” button (see below image).

- Copy the MS Excel Macro code provided in POSPac List Files Macro and paste it into the dialogue window in Excel. Click “Run” (see below image).

Paste code in this window.

- A “Browse” window will pop-up requesting the location of the POSPac files (see below image). The POSPac files should be located in the “Videolog” server folder for the corresponding ARAN data batch, named as “[YYYY]\Data\[YY]_A[ARAN#]\Rec[MMDDYY]\Posdata” (e.g., “2017\Data\ 17_A3\Rec062217\Posdata”).

If the POSPac files are not on the “Videolog” server, they need to be copied from the hard drive. Roadware DCS copies the files to the video folder, but they are not separated by ARAN number.

Once the POSPac files are located, select the “Posdata” folder at the above location and click “OK.”

- Sort the Excel results into different sections.

  - Select the column or highlight the cells that have files listed.

  - Hit the “Alt” key, followed by the “D” key, followed by the “S” key (do not hold all keys down at once).

  - The “Sort” window will appear (see below image). Click “OK.”

- Organize the entries by cutting and pasting file sets into separate columns. A file set is defined as a collection of files with the same characters prior to the “.XXX” extension. For example, an entry in Excel that reads “LV201706220717.001” belongs to the file set “LV201706220717” and should be included in the same column as all entries with that pre-extension name. Each file name should have its own column (see below image). The number of file sets is dependent upon the number of computer restarts performed throughout the day. Typically, there will be at least 2 different file sets, but it is not unusual to have more.

- Open the POSPac MMS 6.2 program by clicking on the “POSPac” icon (see below image) located on the Desktop. Start-up screen once program is opened may appear different depending on user’s computer settings and previous experience with POSPac. Search for the POSPac MMS 6.2 file using the windows explorer if the desktop shortcut is not available.

- Click on the “Tools” drop-down menu and select Options (see below image).

- Confirm that the “Project management folder” location, under the “File Locations” menu (see below image), is pointed to the correct “Videolog” server folder location: “\\shahanartico2\shaomtvideolog\[YEAR]\Data\POS Export\”. Otherwise, click the “…” button and browse to that location. Click “OK.” If this is the beginning of the data collection season, the folders for “Data” and “POS Export” will need to be created before this step.

- If desired, the starting state may also be changed under the “Startup and Display” menu (see below image). In most cases, the most convenient setting is “Open new default project.” Click “OK” to exit the “Options” window and save setting changes.

- A new project should now be created in POSPac MMS (see below image). If not, click the “New Default Project” button. Then, click the “Import” button.

An import window will appear on the right-hand side of the program. If the program has been used before, the window will display the last files that were imported. Otherwise, it will be blank.

- Using “Windows Explorer”, navigate to the file location of the POSPac files. Copy the address displayed at the top of the window. Paste the address in the “Import Folder” dialogue box in the POSPac Import window (see images below).

After pasting the address, do not hit enter. The program will automatically show the POSPac files after pasting the address.

- Import the first file set. Click the “File Name” column. This will sort the file names (see below image). Compare the sorted list in the POSPac Import window to the Excel sheet with the sorted columns created in Step 1 to Step 5. All file sets will be displayed in the POSPac Import window. However, each set must be processed individually. Using the Excel sheet as a guide, select only the first set of files in the POSPac Import window. Make sure all files begin with a capital “LV.” The case of the “L” and “V” at the beginning of the file names must match for a given set. If some of the files in the set begin with “lv”, change them to “LV” in the data folder before importing into POSPac.

- Click “Import” once all the files in a set are selected (see above image). Several boxes will open and close as the program takes a few minutes to process the files. Do nothing until the “Raw Data Check In” window appears (see below image). Click “OK.”

- “Project Definition” window will appear (see below image). Click “OK.”

- If the following error message appears, it’s due to an error in the file name (some files in the set have “LV” while others have “lv”). Click “OK” and return to Step 10 once all the files in a set have matching cases (see below image).

- Review map display of processed data in POSPac. The program will show a plot of the ARAN path for the processed file set (see typical image below). The map size will depend upon the size of the file set. If the map shows too many straight lines or strange overlaps are visible (indicating the road path may not have been followed and the GPS readings are incorrect), the data may need to be examined for validity (see abnormal image below).

- Typical Mapping

- Abnormal Mapping

- Save the file. Select “Save As” from the main ribbon. The format for saving files is [MMDDYY][ARAN#]_pt[#FILEFORTHEDAY] (see below image). The “pt” in the filename is used to distinguish different sets of files in the same day. Repeat Step 11 to Step 13 for all file sets in the batch.

- Rename POSPac files so they may be loaded into Vision. Open two “Windows Explorer” windows.

  - In the first “Explorer” window, navigate to the “POS Export” folder (see below image).

- Select the first file set folder. Navigate to “Mission 1”, then “Extract.” Find the files named vnav_Mission1.out and vrms_Mission1.out (they will be listed last). Copy these files (see below image).

  - In the second “Explorer” window, navigate to the ARAN Submission file (see below image).

- Paste the copied files into this folder. The “Mission 1” part of the copied files must be renamed. To do this, open the “PAGD_QC_1000000” spreadsheet located in the same folder. Find a “UniqueRun” from the corresponding date of the POSPac file set. Replace “Mission 1” in both copied files with that “UniqueRun” value. Repeat this step for all file sets in the batch (see images below).

- Open the Data Control Software. Click “OK.”

- Click the “Import SBET” tab. The “Import SBET” window will appear (see below image). SBET is a type of GPS file, which does not exist in the ARAN data output. However, this import also works for vnav and vrms files.

- Browse for the file location. The file location is the ARAN submission folder in which the vnav and vrms files were just copied and renamed in Step 14. Once the location is set, click the “Select Group” button (see above image). The “Group Selector” window will open (see below image).

- Select the correct data batch in the window (corresponds to the proper ARAN number and collection date). Click “OK.” The DCS will display all files in the selected batch. This may take a few minutes to process (see below image).

- Click the “Start” button. An import progress window will appear. You can click the drop-down arrow next to “Details” to see the import status. When the program is done, the import progress window will show 100% (see below image).

2- Performing QC of Processed GPS Data – performed by DCS user

Summary: This task reviews errors encountered when importing and performs a visual check of the results in Vision’s mapping window prior to completion.

- Click the “View Results” button.

  - If there are no errors, open Vision’s mapping window to visually inspect the routes. Ensure the vehicle’s routing (red line) is in close proximity to the green pins (see below image). When this is finished, the process is complete.

  - If there are errors, they will appear in the bottom window (see below image). Proceed to Step 21.

- Check to make sure the files receiving errors are valid. Go to the Vision “Section Explorer” and filter for the “Collection ID” associated with the error. The Collection ID can be found in the “Message” column of the error window.

  - If the files which received errors are marked as “DUMMY”, they are not valid, and the errors do not need to be corrected (process is complete).

  - If the files are valid, their GPS files were not imported properly, and they will need to be found and imported. Proceed to Step 22.

- In the Vision window, first note the collection date and time. Then, navigate to the GPS files and find the corresponding files of that date and time. Run POSPac for those files by completing Step 6 to Step 14.

- Import revised files into Vision. Complete Step 15 to Step 19. When importing, make sure to deselect files that were already correctly imported.

  - If the files import this time with no error messages, the process is complete.

  - If the files still do not import, contact Supervisor to discuss next steps.

3- Performing QA of Processed GPS Data – performed by Supervisor

Summary: This task performs QA checks on the processed GPS data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

- Verify ARAN collection path matches the route overlay in the Vision map.

## Running Global LCMS Processor

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to process the Laser Crack Measurement System (LCMS) data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The processing of LCMS data is performed in batches throughout the annual data collection season. This SOP applies to the LCMS processing, quality control (QC), and quality assurance (QA) of each individual ARAN data batch. The processing of LCMS data is performed after completion of loading the ARAN data into Vision as outlined in Loading of ARAN Data into Vision and post-processing of GPS data as outlined in Post-Processing of ARAN GPS Data. This process results in the generation of ARAN pavement imagery, transverse profiles, and population of cracking and lane width data on images.

### Frequency

The LCMS Global Processor is run every time a batch of ARAN collected data is received by the DPT, which typically occurs every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from April through October but deliveries sometimes can occur year-round.

### Purpose

The purpose of this SOP is to process the LCMS data and perform QC/QA of the results.

### Resource Requirements

The running of the LCMS Global Processor involves two people: (1) a user knowledgeable in Roadware Vision software to run the LCMS Global Processor and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the LCMS processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 4.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

Additional recommendations (from current Wisecrax practice): - Use Vision v3.1.1 with the appropriate year database for the assigned file. - A high-resolution monitor and a mouse with a scroll wheel are recommended for efficient image analysis.

### Procedure

The procedure to run the LCMS Global Processor and perform QC/QA is comprised of the following three tasks:

- running LCMS Global Processor in Vision,

- performing QC of the processed LCMS data, and

- performing QA of the processed LCMS data.

1- Running LCMS Global Processor in Vision – performed by Vision user

Summary: This task uses Vision software to process LCMS image files from the ARAN data submittal.

- Open Vision.

  - If the LCSM Global Processor has not been run on any of the files in the database, proceed to Step 2.

  - If the LCSM Global Processor has been run on any of the files in the database, proceed to Step 17.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Select “Crack Processing” from the window on the left. Click the box next to “LCMS Global Processor” in the window on the right. Then click the “Options” tab (see above image).

- Instead of changing settings manually, search for preset schemas. Click the “Load” button (see below image).

- Navigate to the location: “\\shahanpmdata1\pmdata\05 Documentation\PM Manuals\Vision\Schemas\[YEAR].” In this folder, look for an XML file named “LCMS_GLOBAL_PROCESSOR_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8.

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see following image). The LCMS Global Processor will take anywhere from 1-3 hours to run. It will depend on the number of lane miles, the number of files, the number of cores in the computer, and database traffic.

- A dialogue box will appear when processing is complete.

  - If the box says, “Batch processing has completed”, proceed to Step 11.

  - If the box says, “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select “View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 11.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

2- Performing QC of Processed LCMS Data – performed by Vision user

Summary: This task checks the quality of processed LCMS data through visual inspection of images and reasonableness of crack length values, as well as runs the “Process Monitor” application.

- Close the processor and restart Vision. In the “Section Explorer”, select a file that was just run in the processor. Click the “Rate” drop-down menu and then click “Pavement Distress” (see below image).

- A “Pavement” window will appear (could take a few minutes). At the bottom right of the screen, select the “Cracks” tab. In the window on the right, a list of all the files, their chainage, number of cracks, and other information will be displayed (see following image).

- Check the “Crack” and “LaneWidth” columns for zeroes. There should be a minimal amount of zeroes in the “Crack” column and no zeroes in the “LaneWidth” column. If all zeroes are showing for both columns, the LCMS Global Processor was not processed successfully on the selected file.

  - If the file was not processed correctly, return to Step 1.

  - If the file was processed correctly, but the wrong file was selected for the QC, return to Step 11 and select the correct file.

  - If this check did not result in any issues, proceed to Step 14.

- Check the LCMS3D Images. Select a file that was processed and click on the “Images” drop-down menu. Click on “LCMS3D.” The image will appear in the window (see below image). At a minimum, check the first and last images.

  - If the images are displayed correctly, proceed to Step 15.

  - If the images are not displayed correctly, the data will need to be reprocessed. Return to Step 2.

- Check the LCMSRange Images. Select a file that was processed and click on the “Images” drop-down menu. Click on “LCMSRange.” The image will appear in the window (see below image). At a minimum, check the first and last images.

  - If the images are displayed correctly, proceed to Step 16.

  - If the images are not displayed correctly, the data will need to be reprocessed. Return to Step 2.

- Repeat Step 14 and Step 15 for at least 3 files from the batch.

- Perform the Process Monitor Check. Click the “Process” drop-down menu and select “Process Monitor.” The “Process Monitor” window will appear (see below image).

- Click on the “Checkbox” drop-down menu and select Clear. This will uncheck all the checked processes. Running the status check for all processes will take several hours to complete.

- Recheck the box next to “Crack Detection”. Click the “Get Status” button. The process will take a few minutes. A status bar at the top of the screen will estimate the remaining time to complete. When it is finished, the status bar will show all green and the “Crack Detection” field in the right window will show crack lengths in milimiles (see below image). Lane Detection is also done during the LCMS Global Processor, but it will be run in a later step.

- On the toolbar above the right window, deselect the “Matched Only” button. Find the first file that was part of the batch that was processed. Scroll through the other files in the batch. Make sure the values for “Crack Detection” are similar to the values for “Length.” They will never be exact. The “Crack Detection” value should never be less than half of the “Length” value.

  - If all the files in the batch show reasonable values for “Crack Detection,” proceed to Step 21.

  - If one or more of the files in the batch show unreasonable values for “Crack Detection,” the data will need to be reprocessed. Return to Step 2.

- Repeat Step 2 to Step 10. This time, however, in Step 4, select the schema file titled “LCMS_GLOBAL_PROCESSOR_LANE_DETECTION_ONLY-[MOST RECENTDATE].” Never run this schema without first running the “LCMS_GLOBAL_PROCESSOR_[MOST RECENTDATE]” schema. Once these steps are repeated, process is complete (./images do not need to be checked twice).

3- Performing QA of Processed LCMS Data – performed by Supervisor

Summary: This task performs QA checks on the processed LCMS data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

## Running Roughness Processor

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Roughness Processor in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The Roughness Processor is run in batches throughout the annual data collection season. This SOP applies to the processing of an ARAN data batch (or batches). The Roughness Processor is run after running the LCMS Global Processor (see Running Global LCMS Processor). This process results in the generation of ARAN pavement roughness profile data in the form of the International Roughness Index (IRI).

### Frequency

The Roughness Processor is run on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Roughness Processor for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Roughness Processor involves two people: (1) a user knowledgeable in Roadware Vision software to run the processors and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 3.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Roughness Processor and perform QC/QA is comprised of the following three tasks:

- running Roughness Processor in Vision,

- performing QC of the processed data, and

- performing QA of the processed data

1- Running Roughness Processor in Vision – performed by Vision user

Summary: This task uses Vision software to process IRI data from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Roughness Processor” in the window on the right.

- Click the “Options” tab. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “Roughness_Processor_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

Step 8.		Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image).

Note: if a red "X" appears next to the name of the run during processing, the Roughness Processor has already been ran for that file (see below image).

2- Performing QC of Processed IRI Data – performed by Vision user

Summary: This task checks the quality of processed IRI data through an inspection of errors reported by Vision during processing.

- A dialogue box will appear when processing is complete.

  - If the box says “Batch processing has completed”, proceed to Step 11.

  - If the box says “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select”View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 11.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

3- Performing QA of Processed IRI Data – performed by Supervisor

Summary: This task performs QA checks on the processed IRI data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

## Route Matching

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to match the inventory data with the data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. This SOP is performed after the running of the Roughness Processor (see Running Roughness Processor). Performing route matching may require the user to reference several additional SOPs developed to supplement the route matching procedure, including Accessing the Electronic ARAN Logs (Accessing Electronic ARAN Logs), Changing Lat/Long (Changing Latitude/Longitude), Using eGIS (Using eGIS), and Importing Inventory Changes (Running Routing Importer). For further information regarding route matching, including troubleshooting and common examples, refer to the “Vision Matching 2015” guide.

### Frequency

Route matching is completed on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to compare and match the expected set of routing inventory data with the ARAN data collected in the field and to perform quality control (QC) and quality assurance (QA) checks. It is also referred to as “segmenting.

### Resource Requirements

Route matching involves two people: (1) a user knowledgeable in Vision software and the Pavement Data Warehouse (PDW) application to perform the route matching and QC, (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete route matching and QA. These time estimates assume average batch quantities and no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision/PDW user and QC | 1 | 40.0 |
| DPT Team Leader (TL) | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to compare and match the inventory data with the ARAN data collected in the field is comprised of the following five components:

- changing workspaces in Vision,

- route matching,

- updating table in the PDW,

- performing QC of matched routes, and

- performing overall QA of the process.

1- Changing Workspaces in Vision – performed by Vision user

Summary: This task opens the appropriate windows in Vision and arranges them so that route matching can be performed efficiently.

- Open Vision.

- The “Section Explorer” window will appear when the program opens. All routes that were imported via the Routing Importer (Running Routing Importer) are listed. If data has been collected for a specific route, the “Collection” and “Vehicle” fields will be populated (see below image).

- Filter out routes that have not been collected. Select the “Filter” icon under the “Collection” column and select “Is not null” from the drop-down menu (see below image).

- Add Date/Time to the columns. Right-click on the header bar and select “Column Chooser” (see below image). Drag the “Time” column into the “Column Chooser” window, or double-click it in the list.

- Open “Process Monitor” window. Click the “Process” drop-down menu and then select “Process Monitor.” The “Process Monitor” window will appear as a new tab with “Section Explorer”, which was already open (see below image).

- Open “Section Composition” window. Click the “Segment” drop-down menu and then select “Section Composition.” The “Section Composition” window will appear as a new tab (see below image).

- Open image windows. Click the “Images” drop-down menu and then select “Left.”. Repeat the sequence for the “ROW” image window. Both image windows will appear as new tabs (see below image).

- Open the “Map” window. Click the “Map” button. The “Map” window will appear as a new tab (see below image).

- Undock the map. Right-click on the “Map” tab and select “Undock” from the drop-down menu (see below image). The map will undock. Drag the new map window to the second monitor (the one without the Vision program). Maximize the map screen on the second monitor.

- Click the button with the horizontal lines on the top left of the map window. Click the “Layers” option. The default layer settings will appear. Uncheck the following: “Collected Sessions” and “Station Data” from the list (see below image).

- Click on the “Base Maps” option. The typical preferred map is selected by clicking the “Google” tab and then checking “GoogleMap” in the map list. When the map selection is complete, click the two left-facing arrows at the top of the options window to close the tab (see below image).

- At the bottom left corner of the window, select the “Measure Distance” icon. This will prevent the program from jumping to different routes during route matching (see below image). While this setting prevents the map from jumping to a different route, the “Measure Distance” setting will also draw a line on the map whenever it is clicked. To remove measurements from the screen when it gets cluttered, select the icon with the horizontal lines at the top left of the screen, select the “Measurements” option and click the large “X.”

- Return to the remaining windows still docked in the Vision program. Right-click on the “Section Composition” tab header and select “New Horizontal Tab Group” (see below image).

- Repeat Step 31 for the “Left” and “ROW” image windows (see below image).

- In the “Section Composition” window, select the “Custom” drop-down menu and choose “Section Composition” (see below image). This will move the vehicle location on the map and the sensor bar in the “Section Composition” window twice for every single movement in the image windows.

- Resize the images. Click the “Maximize” button in the upper right hand corner of the Vision program. Hover the mouse over the top of each image window. A toolbar will appear. Click the “Fit” button (see below image).

- In the “Process Monitor” tab, click the “Check” drop-down menu and select “Clear” (see below image). This will remove the checks beside the items listed in the process table.

- In the “Process Monitor” window, click the “Get Status” button. The status of the task will be shown in a progress bar beside the “Get Status” button (see below image).

- Move the vertical bar which separates the two windows in the “Process Monitor” tab to the left so that the table with “Unique Run” as the first column can be seen on the right (see below image). It may be necessary to resize the image windows to accomplish the optimal view.

- Continue arranging windows until the “Matched” column can be seen in the “Process Monitor” window. If a file has already been matched, a distance value will appear in the “Matched” column. Otherwise, the column will be blank (if it has not been matched or if it was transferred to another route). Check that the Vision program windows and the map window look similar to the images below before continuing with route matching.

- Vision window (Process Monitor, ROW, Left, Section Composition)

- Map window

2- Route Matching – performed by Vision user

Summary: This task uses Vision software to compare and match a collected ARAN file to the appropriate inventory section. This task utilizes many Vision tools (see below image).

- In the “Process Monitor” window, select a file by clicking on the appropriate row in the table. The candidates for route matching will have blanks in the “Matched” column of the table (see below image).

- Using the “Section Composition” window, click the “Auto-Match” button (see below image).

- A “Check Out” window will open (see below image). In order to perform route matching on a file, it must first be checked out.

  - If the “OK” button on the “Check Out” window is grayed out and a user is listed under the “User” column, that user must check the file back in and save before continuing.

  - Otherwise, click “OK.”

- The “Automatic Section Matching” window will appear. Click the “Run” button. Results will appear in the “Matching Results” dialogue box.

  - If there are no errors, a route will appear with a green arrow and a globe icon next to the session name. Click “OK” (see below image).

  - If there are errors for a single route, a route will appear with an exclamation point icon next to the session name and the “OK” button will be grayed out. Investigate the possible causes and resolve issue with the specific file before continuing. The most common solution is using the snapping procedure to bypass auto-matching (see Step 71 and Step 72).

  - If there are errors for multiple routes, the routes will appear with exclamation point icons next to the session name and the “OK” button will be grayed out. This typically indicates that there are multiple files for the same route. Investigate the possible causes and resolve issue with the specific files before continuing. Possible solutions include removing one of the files by: transferring one file to another route or rejecting an older route if the routes are duplicates.

- A dialogue box will appear.

  - If no errors or irregularities are present, click “Yes” (see below image). Proceed to Step 48.

- Accept Match Window

  - If errors or irregularities are present, manual segmentation is required. Proceed to Step 44.

- In the “Section Composition” window, click the “Landmark Editing” drop-down menu and select “Insert Landmark” (see below image).

- Locate the appropriate reference location on the sensor bar and align the pink line with that location.

- Snap the appropriate marker to the location reference. Right-click on the marker, click “Snap” and then “Snap right” or “Snap left” depending on which way the marker needs to move (see below image).

- Apply the “Add Match” function and select “All.” If any segments of the match require removal, tag the individual segment and select “Delete Match” to remove.

- Repeat Step 44 to Step 47 for all landmarks and segments.

- Check the starting point of the file. Click the sensor bar in the “Section Composition” window somewhere close to the starting landmark (green star before blue bar – see below image).

  - If the starting marker is greater than 4.2 mmi (approximately 22.176 feet) from the vehicle position, proceed to Step 51 to place a “gap” in the Section Composition.

  - Otherwise, proceed to Step 61.

- In the “Map” window, use the measuring tool to draw a line from the base of the starting marker to the beginning vehicle position (see below image). Do not draw the line at an angle, which would add distance to the measurement.

- View the distance of the line in the “Measurement” window of the map (see below image).

- Using the distance, calculate the gap needed (for the above distance, a gap would need to be placed from 0.0 to 0.05).

- Click the “Landmark Editing” drop-down menu and select “Insert Landmark” (see below image).

- The “Insert Landmark” window will appear. Enter the correct chainage into the “Chainage” box (see below image).

- Click “OK” and the sensor bar should move to the specified location.

- Right-click inside the dotted box which contains the sensor bar for the route in the “Section Composition” window (see below image).

- Select “Align”, then “Shift Begin.” This will move the start of the file to the location of the sensor bar.

- Right-click inside the dotted box which contains the sensor bar for the route in the “Section Composition” window. Select “Add Match”, then “From current” (see below image). This will match from the location of the sensor bar to the end.

- The “Match Editor” window will open. Change “Rubber-band” to “Re-chain” and click “OK” (see below image).

- Navigate to the end of the run (there should be a yellow triangle near the end). Place the sensor bar close to the yellow triangle.

- Look at the “Map” window. Move the sensor bar in the “Section Composition” window and the yellow vehicle position in the “Map” window to a location close to the green balloon using the navigation controls (see below image).

- Check the distance between the yellow vehicle position marker and the ending marker (green balloon) using the measurement method as described in Step 51.

  - If the ending marker is greater than 4.2 mmi (approximately 22.176 feet) from the vehicle position, place a gap at the end of the run.

  - Otherwise, right-click inside the dotted box which contains the sensor bar for the route in the “Section Composition” window. Select “Snap”, then “Snap Left” (see below image). This will move the ending point to the correct spot and the triangle on the sensor bar should turn green.

- With the starting location still selected, check the “ROW Image” and “Left Image” to confirm that there are images present.

  - If there is no image present for one or both of the image windows, investigate missing image issue and resolve before continuing (may require reimporting of image files).

  - Otherwise, proceed to Step 65.

- Check the stopping point of the file. Click the sensor bar in the “Section Composition” window somewhere close to the ending landmark (green star before blue bar).

- In the “Map” window, zoom in to at least 16 feet. Make sure the vehicle position (yellow triangle) is close enough to the ending marker (green balloon). Since the sensor bar is only placing the vehicle position close to the end of the route, it may be necessary to use the arrow keys to move the vehicle to the exact end of the route. Use the measuring tool in the “Map” window if the distance cannot be determined visually.

  - If the ending marker is greater than 4.2 mmi (approximately 22.176 feet) from the vehicle position, repeat Step 51 to Step 63 for placing a “gap” in the Section Composition.

  - Otherwise, proceed to Step 67.

- With the ending location still selected, check the “ROW Image” and “Left Image” to confirm that there are images present.

  - If there is no image present for one or both of the image windows, investigate missing image issue and resolve before continuing (may require reimporting of image files).

  - Otherwise, proceed to Step 68.

- Using the “Section Composition” window, click the “Rechain” button (see below image).

- Click “OK” in the “Rechain Section” window (see below image).

The blue bar for the route will now show a grid of white lines.

- Check that the starting and ending markers are green.

  - If markers remain yellow, proceed to Step 71.

  - Otherwise, skip to Step 74.

- If one or both of the markers remain yellow, snapping must occur to move the markers to the proper location. Move the cursor close to the correct location in the “Section Composition” window. Right-click and select “Snap”, then “Snap left” or “Snap right”, depending on which direction the marker needs to move (see below image).

- In the “Match Snap” window, select “Re-chain” and click “OK” (see below image).

- Repeat Step 71 and Step 72 for other markers if necessary.

  - If markers are now both green, proceed to Step 74.

  - Otherwise, investigate other possible solutions to resolve the issue before continuing.

- Check that the beginning milepost (BMP) is correct.

  - If the BMP is less than zero or between 0.0 and 0.1, it is incorrect and needs to be updated. Proceed to Step 75.

  - If the BMP is 0.0 or greater than 0.1, no changes are required. Proceed to Step 76.

- Click on the starting marker in the “Section Composition” window, click “Landmark Editing”, and then “Edit Landmark.” Enter the correct BMP in the “Chainage” box and click “OK” (see below image). Correct BMP can be looked up in the routing inventory in the “Section Explorer” window.

- Check if multiple routes exist in same file.

  - If multiple routes exist, refer to Accessing Electronic ARAN Logs for accessing electronic ARAN logs.

  - If multiple routes do not exist, proceed to Step 77.

- Return to the “Section Composition” window and click the “Complete” button (see below image).

- Click the “OK” button on the dialogue box that appears (see below image). Place any pertinent notes in the “Comment” field. Comments may include: adjusted GPS landmarks, stopped 100’ early, started 100’ late, adjusted mile points, or gap due to road closure.

- In the “Section Composition” window, click the “Check In” button (see below image).

- Click the “OK” button on the dialogue box that appears (see below image).

- In the “Section Composition” window, click the “Save” button (see below image).

- Click the “OK” button on the dialogue box that appears (see below image).

3- Updating Table in the PDW – performed by PDW user

Summary: This task updates the “View/Edit Unique Run Status” table in the PDW with the matched route and includes any notes for the run.

- Open the PDW application.

- Select “Data” and then click “ARAN” and “View/Edit Unique Run Status” from the drop-down menu (see below image).

- Data Dropdown Menu ARAN

- In the table that opens, select the current year and “All” in the “County” field. The table will populate based on the above query (see below image).

- Double click one of the records in the table. It does not matter which record is selected. In the window that opens, click the arrow next to the “File Name” drop-down list and select the file name that was just matched (see below image).

- File Name Dropdown Menu PDW

- Check the box next to “Match QC” once the correct file is selected. The program will add the user name and date. If any notes need to be added, select a common note from the “Standard Note” drop-down menu or type a manual note in the “Add. Note” field (see below image).

- Add text in the “Match Note” field, which should include user initials, date, and any notes from Step 78.

- Click the “Save” button.

- If closing out a District, complete the status table and submit to the DPT TL via email (see below image).

4- Performing QC of Matched Routes – performed by Vision and PDW user

Summary: This task performs route and systemic level reviews of matched routes to check for accuracy and completeness. Vision and PDW application tools are utilized to determine if data is ready for production to further processing routines can occur.

- Perform route level QC inspection by opening a matched route in Vision. Check for the following key items:

  - Check that all sections in the run are matched, including ramps. Match any remaining sections, if necessary. Use Vision Map, Layers, Routed Nodes feature to assist in process (see image below).

- Matched as collected (1), but now needs the ramp match (2)

  - Check landmark accuracy. If landmark needs adjustment, update the year’s inventory spreadsheet with new GPS and load into Vision. Maintain spreadsheet for reference by others.

- Example of a bad landmark that requires adjustment

  - Check the section beginning and ending landmark mileage. Correct if necessary.

- Example of false landmark mile point (158 should read 0)

  - Accommodate gaps in collection (missing collection, late start, early end, road closures, etc.). Determine if a gap (or gaps) exists. Accommodate in the matching process where necessary and place a note in the Vision run upon check out as well as in the PDW for others’ reference.

- Example of collection missing (and match accommodated) at the start of a route

  - Verify collection run joins. Revisit start and end of each run to verify the joins are correct. Use keyboard left or right buttons to navigate to beginning or end of a run after it has been tagged.

  - Determine if transfers are required and complete transfers where necessary. Use Vision Map, Layers, Routed Nodes feature to help identify routes needing transfer.

- Example: Section 1 was the original collection and Sections 2 and 3 require transfer to their respective ramps.

  - Check that all collection runs are either matched, transferred, or rejected. Determine if all sections within a collection session are identified and match, transfer, or reject accordingly. Rejecting routes should only be utilized when a collection run has been determined to be unsalvageable due to data or imagery issues (recollection is needed) or a substantial portion of a run is not useful (except where it can be matched to another route).

  - Enter and apply pertinent notes (QC and review notes) for each collection run in Vision. Notes should be entered after selecting the “Complete” button (see below images).

- Note examples include:

  - Enter and apply pertinent notes (QC and review notes) for each collection run in the PDW application (see below images).

- Perform systemic level QC inspection by opening Vision once per week. Click “Segment” and select “Match Discrepancy Finder” from the drop-down menu (see below image).

-  In the "Group Partition Selector", select "All", enter "4" in the "Discrepancy Threshold" box, and click "OK" (see below image).

Review results. A yellow triangle in the "CO" column indicates a failed QC (requires review), while a blue check mark indicates a passed QC (see below image). Review each failed QC and determine if discrepancy is reasonable and that QC notes exist within Vision and the PDW. Address the issue by populating notes where needed and making appropriate correct actions.

Click "View" and select "SQL Queries", then "Segmenting QC" to determine what routes require further review of accuracy and completeness (see below image). Take corrective actions where necessary.

For each of the above items, run the SQL procedure to attain collection run(s) with issues that require review to determine accuracy and completeness. Evaluate and address any issues and make sure QC notes are placed in the PDW Unique Run status program for each run. Before selecting "Execute," Short Matches requires "4" value entry, GPS Matching Discrepancies require "4" value entry, and Continuous Match Gaps requires "0.1" value entry where Values are null.

Example of results

- Communicate results of QC review to DPT TL and discuss any lingering issues that may need to be resolved.

5- Performing QA of Overall Process – performed by Supervisor

Summary: This task performs QA checks on route matching process.

- See existing QA guide.

## Performing 100% Drive Through

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to perform quality control (QC) and quality assurance (QA) checks of the Laser Crack Measurement System (LCMS) pavement image output using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. This SOP is performed after completion of route matching (see Route Matching).

### Frequency

The 100% drive through of pavement images is performed every time a batch of ARAN collected data is received by the DPT. Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to perform a thorough check of the LCMS pavement image output, correct any errors in crack or joint detection, and perform QC and QA checks.

### Resource Requirements

Performing the 100% drive through involves three people: (1) a user knowledgeable in Vision software to perform QC on the pavement images, (2) a user knowledgeable in Vision software to perform QA on the analyzed images, and (3) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the drive through QC/QA. These time estimates assume average batch quantities and no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 336.0 |
| DPT Staff | QA | 1 | 25.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to perform the 100% drive through and perform QC/QA is comprised of the following three tasks:

- performing 100% drive through QC,

- performing QA of the analyzed images, and

- performing overall QA of the process.

1- Performing 100% Drive Through QC – performed by Vision user

Summary: This task uses Vision software to analyze the LCMS output, correct errors in crack or joint detection, and mark events.

- Open Vision software. Select the target database and click “OK” (see below image)

- Click on “Rate” and select “Pavement Distress” from the drop-down menu (see below image).

Also select “Segment” → “Section Composition” to keep route matching and milepost context visible during review.

- From the menu on the right side of the window, deselect: Potholes, Raveling, Bleeding, Pickouts, and Curbs & Drop-offs (see below image).

Quick pre-check: Click on “Cracks” (bottom panel) and scroll from begin to end. If all values in the Cracks column are 0 for a file, pause and notify the Team Leader (likely LCMS issue); otherwise proceed.

- Click on “Images” and select “ROW” from the drop-down menu (see below image).

- Adjust the “ROW” window by clicking on the “ROW” tab and selecting “New Vertical Tab Group” (see below image).

Set the Distance increment to “4.0” (control near Speed) for consistent 4 milli-mile navigation. Establish file access control: use Access Control → Lock (Pavement tab). If a username already appears in the Status Control [Distress] user column, do not lock and consult the Team Leader.

- Start driving through using the “Play button (see below image).

Critical: Save your work regularly (about every mile). There is no undo; frequent saves reduce risk of data loss.

- Identify pavement type.

  - If the section is Jointed Concrete Pavement (JCP), proceed to Step 8.

  - If the section is Continuously Reinforced Concrete Pavement (CRCP), proceed to Step 10.

  - If the section is Flexible (Asphalt) Pavement, proceed to Step 12.

- Navigate to beginning of section. Draw a “JCP Distress Event” line from the beginning to the end of the JCP concrete section (see below images).

- Analyze first slab. Examine slab for cracking. If a crack (or cracks) exists, draw a point distress labeled “JCP_Cracked_Slab” in the middle of the slab. Otherwise, draw a point distress labeled “Intact_JCP_Slab” in the middle of the slab. Ensure the slab joint was detected (as indicated by a horizontal blue line at the joint). If it was not detected, draw the joint line in manually. To identify a slab as cracked or intact, select the “Distress” button, double-click on the slab, and select the event. Repeat slab analysis for all slabs in section (see below images).

- Navigate to beginning of section. Draw a “CRCP Distress Event” line from the beginning to the end of the CRCP section (see below image).

- Analyze the pavement, moving from beginning to end of the section. Adjust blue line with the “Lane Mark” tool if pavement lane is not lined up properly. Draw area distress markers over top of any observed distresses in the images: “CRCP_Punchout” for punchouts, “CRCP_Long” for longitudinal cracks, “CRCP_Patch_Asphalt” for asphalt patches, or “CRCP_Patch_Concrete” for concrete patches. Follow the HPMS Field Manual (FHWA, December 2016) for drawing distress areas. To add a distress, select the “Distress” button, double-click on the pavement, and select the event. Continue to end of pavement section (see below image).

Note: For CRCP, exclude transverse cracking from analysis; focus on longitudinal cracks, patching, and punchouts. If needed, draw full lane-width joints at the begin and end of each CRCP area to clearly bound the section.

- Navigate to beginning of section. Virtually drive the pavement section. Make the following adjustments where necessary:

  - If the blue line is not aligned with the marked line (either inside or outside), then adjust it manually (see below image).

  - Verify lane width and edges: blue edge lines should lie immediately inside the left/right pavement markings. Where markings are absent, place blue lines at the image edges. The minimum acceptable lane width is 95 units; adjust with the Lane Mark tool and use Erase as needed to improve cracking/rutting accuracy.

Special considerations:
  - Gutter pans inside lane markings: exclude the gutter pan interface from lane width and cracking calculations; cracks along the asphalt–gutter joint are false positives and should be removed.
  - Inlets/storm grates within the lane: adjust blue lines so grates fall outside detection boundaries and delete any associated cracks on/around the inlet perimeter.

- Delete false positive crack detections (can be due to manhole covers, inlets, gutter pans, rumble strips, etc.).
    Additionally watch for false detections from painted letters/arrows and other markings, raised or recessed pavement markers (RPMs), ATR saw cuts and sealed areas, gouges/surface irregularities, and isolated concrete areas within predominantly asphalt sections.
- Draw distress event lines for noted items, including bridges, railroads, crosswalks, stop bars, bad images, disregarded cracks, wet pavement, lane deviations, debris, brick pavement, crack seals, etc. Draw a point event for any pavement change. To draw a distress event, click on the "Distress" button and right-click at the start of the event, right-click where the event should end, and choose the event. Continue to end of pavement section (see below images).

Bridge/Tunnel tagging protocol:
 - Draw full lane-width joints at the begin and end of each bridge/tunnel section.
  - Draw a longitudinal line from begin to end and tag as bridge/tunnel (e.g., Event_Bridge). Remove false cracks near boundaries; the event bounds where cracks will be removed by downstream processing.

Crack sealed areas:
  - Draw a longitudinal line from begin to end of sealed segments and tag as Event_Sealed_Cracking_Exists so downstream programs can correctly include/exclude sealed cracks. For sealed patch edges, tag the entire edge length.

2- Performing QA of Analyzed Images – performed by Vision user

Summary: This task checks the QC’ed pavement images through a visual inspection.

- Open Vision.

- Complete Step 2 through Step 12 for two random files for each drive through operator. Ensure that protocol has been followed. Correct any errors and resolve any issues before continuing.

- Report findings to supervisor.

3- Performing QA of Overall Process – performed by Supervisor

Summary: This task performs QA checks on the drive through process.

- Perform weekly data QA routines.

- TL review of QC staff notification of recollection and QA staff weekly data review.

- Review of production status – receive weekly update from QA staff (last ARAN collection date and lane miles fully processed).

## Running Rut Processor

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Rut Processor in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The Rut Processor is run in batches throughout the annual data collection season. This SOP applies to the processing of an ARAN data batch (or batches). The Rut Processor is run after completion of the ARAN data drive-through (see Performing 100% Drive Through). This process results in the generation of ARAN pavement transverse profile (rutting) data.

### Frequency

The Rut Processor is run on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Rut Processor for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Rut Processor involves two people: (1) a user knowledgeable in Roadware Vision software to run the processors and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 3.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Rut Processor and perform QC/QA is comprised of the following three tasks:

- running Rut Processor in Vision,

- performing QC of the processed rut data, and

- performing QA of the processed rut data

1- Running Rut Processor in Vision – performed by Vision user

Summary: This task uses Vision software to process rut data from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Rut Processor” in the window on the right.

- Click the “Options” tab. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for the one of three rut XML files for the ARAN data to be processed (see below image).

  - “RUT_Processor_ARAN3_[MOST RECENTDATE]” for all ARAN 3 data.

  - “RUT_Processor_ARAN4_AfterJune21_[MOST_RECENTDATE]” for all ARAN 4 data collected after June 21, 2017.

  - “RUT_Processor_ARAN4_BeforeJune21_[MOST_RECENTDATE]” for all ARAN 4 data collected before June 21, 2017.

- Select the XML file for the ARAN 3 and click “Open.”

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch and appropriate ARAN vehicle. Under the “Vehicle” column, “1723” is for the ARAN 3 and “1724” is for the ARAN 4. Filtering the list can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image).

2- Performing QC of Processed Rut Data – performed by Vision user

Summary: This task checks the quality of processed rut data through an inspection of errors reported by Vision during processing and a visual inspection of the rut depth graph.

- A dialogue box will appear when processing is complete.

  - If the box says “Batch processing has completed”, proceed to Step 11.

  - If the box says “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select “View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 11.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

- Check the “Batch Processor” window for runs that did not process. The software occasionally skips runs. In the window showing processed results, check the “Run Rate” column. Any runs that have blanks in this column will need to be reprocessed. Select those runs, removing runs already processed, and change the output location to a new temporary folder. Click “Reset.”

- Click on “Plug-Ins” tab and select “Transverse Profile Explorer” tab (see below image)

- Select a session and click on the “Go” button to open the profile of the run (see below image).

- The transverse profile and the Rut Depth should be showing a graph similar to the graph below (see below image).

  - If either the “Transverse Profile” or “Rut Depth” graph is blank, the processor will need to be run again by returning to Step 2.

  - Otherwise, proceed to Step 13.

- Return to Step 2 and repeat the process for both types of ARAN 4 data, selecting the appropriate XML file each time06/12/2019.

3- Performing QA of Processed Rut Data – performed by Supervisor

Summary: This task performs QA checks on the processed rut data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

## Running Classification and Rating Processors

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Classification and Rating Processors in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The Classification and Rating Processors are run in batches throughout the annual data collection season. This SOP applies to the processing of an ARAN data batch (or batches). The Classification and Rating Processors are run after completion of the ARAN data drive-through (see Performing 100% Drive Through). This process results in the generation of ARAN cracking data.

### Frequency

The Classification and Rating Processors are run on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Classification and Rating Processors for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Classification and Rating Processors involves two people: (1) a user knowledgeable in Roadware Vision software to run the processors and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 3.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Classification and Rating Processors and perform QC/QA is comprised of the following three tasks:

- running Classification and Rating Processors in Vision,

- performing QC of the processed data, and

- performing QA of the processed data.

1- Running Classification & Rating Processors in Vision – performed by Vision User

Summary: This task uses Vision software to process cracking data from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Classification Processor” and “Rating Processor” in the window on the right.

- Click the “Options” tab with the “Classification Processor” highlighted. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “CLASSIFICATION_PROFILE_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Click the “Options” tab with the “Rating Processor” highlighted. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “RATING_PROCESSOR_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image).

2- Performing QC of Processed Data – performed by Vision user

Summary: This task checks the quality of processed cracking data through an inspection of errors reported by Vision during processing.

- A dialogue box will appear when processing is complete.

  - If the box says “Batch processing has completed”, proceed to Step 12.

  - If the box says “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select”View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 12.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

- Check the “Batch Processor” window for runs that did not process. The software occasionally skips runs. In the window showing processed results, check the “Run Rate” column. Any runs that have blanks in this column will need to be reprocessed. Select those runs, removing runs already processed, and change the output location to a new temporary folder. Click “Reset.”

- Click on the “Rate” tab and select “Pavement Distress” from the drop-down menu (see below image).

- The downward image list will open in the “Pavement” tab. Make sure that all the frames have been classified and rated by observing that the rating sign is in front of all frames. There should also be boxes around all the cracks.

  - If all rows have the rating sign and the cracks have boxes around them, proceed to Step 11.

  - If there are red flags or blank spaces where the rating sign should be, or if there are cracks not surrounded by boxes, those files will need to be re-classified and rated (see below image). Return to Step 2.

3- Performing QA of Processed Rut Data – performed by Supervisor

Summary: This task performs QA checks on the processed data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

## Running Vibing Processor

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to perform vibing routines in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. Vibing is the process of adding a status bar and logo to the “Left” and “ROW” images collected with the ARAN van.

The Vibing Processor is run in batches throughout the annual data collection season. The actual name of the processor in Vision is “Exif Geo-Tagging and Vibing Processor,” but is commonly referred to as “vibing.” This SOP applies to the processing of an ARAN data batch (or batches). The Vibing Processor is run after running the Classification and Rating Processors (see Running Classification and Rating Processors). This process results in the generation of a status bar at the top of the “Left” and “ROW” images and adds latitude/longitude data to the image properties.

### Frequency

The Vibing Processor is run on batches of ARAN collected data as they are received by the DPT. Vibing can be performed on the same data multiple times. If changes in route mileage occur, the vibing processor will need to be rerun. Performing the rerun on the entire data set will not harm the data output. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Vibing Processor for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Vibing Processor involves two people: (1) a user knowledgeable in Roadware Vision software to run the processor and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 40.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Vibing Processor and perform QC/QA is comprised of the following five tasks:

- running the Vibing Processor in Vision for “Left” images,

- performing QC of processed “Left” images,

- running the Vibing Processor in Vision for “ROW” images,

- performing QC of processed “ROW” images, and

- performing QA of the processed data.

1- Running Vibing Processor for “Left” Images – performed by Vision user

Summary: This task uses Vision software to generate a banner and logo on the “Left” image files from the ARAN data submittal. This task also adds latitude/longitude data to the “Left” image properties.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Exif Geo-Tagging and Vibing Processor” in the window on the right.

- Click the “Options” tab06/12/2019. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “Left.” Select the file and click “Open” (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image). The Vibing Processor will take anywhere from 1-3 hours to run. It will depend on the number of lane miles, the number of files, the number of cores in the computer, and database traffic. Other batch processors can process multiple files at one time, however, this processor only processes one file at a time

- Vibing Processor Running

- A dialogue box will appear when processing is complete.

  - If the box says “Batch processing has completed”, proceed to Step 11.

  - If the box says “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select”View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 11.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

2- Performing QC of Processed “Left” Images – performed by Vision user

Summary: This task checks that the banner and logo were added to the “Left” processed images through visual inspection.

- Close the processor and restart Vision. In the “Section Explorer” window, select a file that was run in the processor (see below image).

- Click the “Images” drop-down menu and select “Left.” The first image that appears may not display the banner and logo generated during processing. This is normal. Only the area that is “matched” will display the banner and logo. There are two methods of locating the matched area (choose one and proceed):

  - Click the “Play” button at the bottom of the window and wait until the matched area appears (see below image).

  - Click the “Segment” drop-down menu and select “Section Comparison.” The area in the checkered blue is matched. Click somewhere in the checkered section and Vision will move to that location (see below image).

- The “Left” image now should display the blue banner and logo.

  - If you do see the banner and logo after checking the first and last image, proceed to Step 14.

  - If you do not see the banner and logo, first check that the image is fit on the screen. Move the mouse to the top of the image so that a control bar appears. Click the “Fit” button. If the banner and logo are now displayed, proceed to Step 14.

  - If the banner and logo are still not displayed, the affected files will need to be reprocessed. Return to Step 2 and perform processing for all affected files. Occasionally, the batch processor will skip files. If a file is found in the QC process which does not have the banner and logo, it will be necessary to check all files for correctness and rerun the processor on all affected files.

3- Running Vibing Processor for “ROW” Images – performed by Vision user

Summary: This task uses Vision software to generate a banner and logo on the “ROW” image files from the ARAN data submittal. This task also adds latitude/longitude data to the “ROW” image properties.

- Run the same processor for the “ROW” images by repeating Step 2 to Step 10. However, this time in Step 4, select the XML file named “ROW.”

4- Performing QC of Processed “ROW” Images – performed by Vision user

Summary: This task checks that the banner and logo were added to the “ROW” processed images through visual inspection.

- Perform the same QC steps for the “ROW” images by repeating Step 11 to Step 13. However, this time in Step 12 select “ROW.”

5- Performing QA of Processed Data – performed by Supervisor

Summary: This task performs QA checks on the processed vibing data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection date).

- Review Vision app’s ROW imagery display and check for banner.

## Running Thumbnail Creator

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Thumbnail Creator in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The Thumbnail Creator is run in batches throughout the annual data collection season. This SOP applies to the processing of an ARAN data batch (or batches). The Thumbnail Creator is run after running the Vibing Processor (see Running Vibing Processor). This process results in the generation of reduced file sizes for the “Left”, “ROW”, and “Pavement” collected images.

### Frequency

The Thumbnail Creator is run on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Thumbnail Creator for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Thumbnail Creator involves two people: (1) a user knowledgeable in Roadware Vision software to run the processor and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 40.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Thumbnail Creator and perform QC/QA is comprised of the following three tasks:

- running the Thumbnail Creator in Vision,

- performing QC of the processed data, and

- performing QA of the processed data.

1- Running Thumbnail Creator in Vision and QC – performed by Vision user

Summary: This task uses Vision software to reduce the file sizes of the “Left”, “ROW”, and “Pavement” image files from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Thumbnail Creator” in the window on the right.

- Click the “Options” tab06/12/2019. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “Thumbnail [MOST RECENT DATE] All images and both Arans.” Select the file and click “Open” (see below image).

- Make sure the output folder is pointing to the correct location in the window to the right (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image). The Thumbnail Creator will take anywhere from 1-3 hours to run. It will depend on the number of lane miles, the number of files, the number of cores in the computer, and database traffic. Other batch processors can process multiple files at one time, however, this processor only processes one file at a time

- A dialogue box will appear when processing is complete.

  - If the box says “Batch processing has completed”, proceed to Step 11.

  - If the box says “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select”View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 11.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

2- Performing QC of Processed Data – performed by Vision user

Summary: This task checks that the folders contain the correct number of processed images and performs visual inspection of a sample of images to look for abnormalities.

- Navigate to the output folder (see below image).

- Check the number of files in each folder. There should be the same number of files in each folder type (“Left”, “ROW”, “LCMSRange”, and “LCMS3D”).

  - If the number of files match in each folder, proceed to Step 13.

  - If there are discrepancies, isolate the files that are missing from certain folders and reprocess those files by returning to Step 2 and repeating the procedure only for the files in question. Proceed to Step 13.

- Spot check a few of the images in “Windows Explorer.”

  - If the images look acceptable, process is complete. However, if there are changes in mileage that cause the Vibing Processor to be rerun, those files will also need to be reprocessed in the Thumbnail Creator.

  - If there are issues with the processed images, isolate the files that are missing or incorrect and reprocess those files by returning to Step 2 and repeating the procedure only for the files in question.

3- Performing QA of Processed Data – performed by Supervisor

Summary: This task performs QA checks on the processed images.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection date).

- Review Videlog server to make sure imagery is posted and available.

## Running Curve Fit Batch Processor

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Curve Fit Batch Processor in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. The Curve Fit Batch Processor is run in batches throughout the annual data collection season. This SOP applies to the processing of an ARAN data batch (or batches). The Curve Fit Batch Processor is run after completion of the ARAN data route matching (see Route Matching). This process results in the generation of ARAN vertical and horizontal roadway (geometric) data.

### Frequency

The Curve Fit Batch Processor is run on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season.

Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the Curve Fit Batch Processor for collected ARAN data and perform quality control (QC) and quality assurance (QA) checks.

### Resource Requirements

The running of the Curve Fit Batch Processor involves two people: (1) a user knowledgeable in Roadware Vision software to run the Curve Fit Batch Processor and perform QC, and (2) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the processing and QC/QA checks. These time estimates assume average batch quantities and no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 3.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Curve Fit Batch Processor and perform QC/QA is comprised of the following three tasks:

- running Curve Fit Batch Processor in Vision,

- performing QC of the processed curve fit data, and

- performing QA of the processed curve fit data.

1- Running Curve Fit Batch Processor in Vision – performed by Vision User

Summary: This task uses Vision software to process curve fit data from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Curve Fit Batch Processor” in the window on the right. Then click the “Options” tab (see above image).

- Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schemas are stored06/12/2019. In this folder, look for an XML file named “CurveFit2017_Protocol_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Set the output location to the following: \\shahanpmdata1\PMtestData\Archive Condition Data\Data17\Curve_Fit_Files (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8.

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image).

2- Performing QC of Processed Data – performed by Vision user

Summary: This task checks the quality of processed curve fit data through an inspection of errors reported by Vision during processing.

- A dialogue box will appear when processing is complete.

  - If the box says, “Batch processing has completed”, proceed to Step 12.

  - If the box says, “Batch processing has completed with errors”, proceed to Step 10.

- View the errors. Click the “Exceptions” drop-down menu and select “View Exceptions.”

  - If the error is “Missing Images”, this is acceptable. Proceed to Step 12.

  - If any other errors are present, reprocessing is required. Click the “Exceptions” drop-down menu and select “Reprocess Exceptions.” This will remove all the files from the data screen except for the files that had exceptions. Next, click the “Start” button to reprocess.

- Check the “Batch Processor” window for runs that did not process. The software occasionally skips runs. In the window showing processed results, check the “Run Rate” column. Any runs that have blanks in this column will need to be reprocessed. Select those runs, removing runs already processed, and change the output location to a new temporary folder. Click “Reset” (see below image).

- Once reprocessing is complete, navigate to the temporary folder that was created for the reprocessed results. Move all files and move to the main folder that contains the original processed files. Select “Move and Replace” if there are any duplicate files (see below image).

3- Performing QA of Processed Rut Data – performed by Supervisor

Summary: This task performs QA checks on the processed curve fit data.

- TL review of QC staff notification of recollection.

- Review of production status – receive weekly update from QC staff (last ARAN collection data fully processed).

## Reporting and Uploading Data to Oracle

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to upload data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff and processed by DPT staff in Roadware Vision software to Oracle using the Pavement Data Warehouse (PDW). This SOP is performed once all other data processing is completed for a specific batch of data. A list of columns required for running the report generator in Vision is included as an appendix (see Base Report. In addition, the methodology behind the International Roughness Index (IRI) speed adjustment is also included as an appendix (see IRI Speed Adjustment).

### Frequency

The report generation and Oracle upload is performed on batches of ARAN collected data as they are received by the DPT. The frequency and timing varies according to schedule, available resources, and quantity of data received. The batches are processed in a continuous flow throughout the data collection season. Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to run the report generator in Vision and then upload the processed data from Vision to Oracle using the PDW. The operations performed in the PDW utilize SQL Developer packages to populate the output tables and perform a speed adjustment for the IRI data. The UPD_VISION_TABLE package populates the Base tables, while the CONDITION_PKG creates new fields in the output tables (LEFTIRI_ADJ and RIGHTIRI_ADJ) and populates them with IRI data adjusted for speed. The output tables will serve as the basis for developing the Business Plan Tables.

### Resource Requirements

Running the report generator and uploading data to Oracle involves four people: (1) a user knowledgeable in Vision and the PDW to run the report generator and upload the data, (2) a database management expert to perform QA in Oracle, (3) a database management expert who, as required, performs code repair and troubleshooting, and (4) a supervisor who, as required, provides guidance and decision-making and performs QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume average batch quantities and no issues encountered during uploading.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision/PDW user | 1 | 120.0 |
| DAT Staff | QA | 2 | 0.5 |
| DAT Staff | Code Repair | 1 | AR06/12/2019 |
| DPT TL | Supervisor and QA | 1 |  |

### Procedure

The procedure to generate the reports and upload data to Oracle is comprised of the following four tasks:

- defining a new report,

- running the report generator,

- uploading data from Vision to Oracle, and

- performing QA of uploaded data.

1- Defining a New Report – performed by Vision user

Summary: This task uses Vision software to define a new report to be used in the generation of ARAN data reports. If the report has been previously defined and created in Vision, the user may proceed to the next component (Step 8).

- Determine if the desired report has already been defined and created in Vision.

  - If the report has been previously defined and created, proceed to the next component by skipping to Step 8.

  - If the report has not been previously defined and created, proceed to Step 2.

- Open Vision and connect to the proper database.

- Click the “Report” drop-down menu and select “Report Generator.” The “Report Generator” window will open with all the reports that have been previously defined (see below image).

- Click “Report Editor” and the “Report Template Editor” will appear. Click the “New” button, as highlighted in red, to create a new report, click the “Modify” button, as highlighted in blue, to modify an existing report, or click the “Delete” button, as highlighted in black, to delete an existing report.

- Click the “New” button and enter the information for: “Name”, “Description”, “Grouping”, “Interval length mode”, “Interval reset mode”, “Break by”, and “Intervals” as shown in the image below. Click “OK.” A new report named “Base” will be created in Vision (see below image).

- After the new “Base” report is created, click the “Columns” tab in the “Report Templates Editor” window to define all columns needed in the new report, one column at a time. Again, click the “New” button to add a new column, click the “Modify” button to modify an existing button, or click the “Delete” button to delete an existing column (see below image).

- Click the “New” button and enter the information for: “Name”, “Description”, “Type” (data type such as TEXT or NUMBER, decimals, aggregation type such as MAX, MIN, FIRST…), “Conditional Split” and “Transformation” for each column that is needed. The below image gives an example for defining a column named “EVENT_BRICK_OFFICE.” The details of each column of the “Base” report can be found in the appendix Base Report. Once all columns are defined, the new report will be ready to be ran by the Report Generator in Vision (see below image).

2- Running Report Generator – performed by Vision user

Summary: This task uses Vision software to process report data from the ARAN data submittal.

- Open Vision.

- Click the “Process” drop-down menu. Select “New Batch Processor.” The “Batch Processor” window will open (see below image).

- Click the box next to “Report Generator Processor” in the window on the right.

- Click the “Options” tab. Instead of changing settings manually, search for preset schemas. Click the “Load” button. Navigate to the folder where schema files are stored06/12/2019. In this folder, look for an XML file named “Base_Report_[MOST RECENTDATE].” Select the file and click “Open” (see below image).

- Click “Data” from the top menu bar. Every file in the database will appear in the window. The list must be filtered to only include the sections from the current batch. This can be accomplished through two different methods:

  - Removing unwanted files, or

  - Removing all files and selecting a batch to re-add.

- Choose removal method. If removing all files and selecting a batch to re-add is preferred, skip to Step 7. Otherwise, use a combination of the Shift and Ctrl keys to highlight all unwanted files in the window. Click “Remove.” Proceed to Step 8 (see below image).

- To remove all files, click the “Check” drop-down menu and select “All.” Click “Remove.” Then, click the “Reload” (first blue arrow) button. The “Group Partitions” window will open (see below image).

Select the current batch by clicking the checkbox next to the appropriate batch name. Click the “OK.”

- Once only the desired files appear in the “Batch Processor” window, they may be processed. Click “Process”, then “Start.” The files will begin to appear under the session window (see below image).

A dialogue box will appear when processing is complete.

3- Uploading Data from Vision to Oracle – performed by PDW user

Summary: This task uses the PDW to migrate processed data from Vision to Oracle in the form of “Base_#” tables.

- Open the Pavement Data Warehouse application.

- Click the “Data” drop-down menu and select “Aran”, then “Data Transfer from SQL to Oracle” (see below image).

- The “Data Transfer from Vision to PDW” window will appear. Ensure the following items are selected then click “Transfer” (see below image):

  - Reporting Year – “[Current Year]”

  - Database – “OMT_ARAN_PROD_[CURRENTYEAR]_1”

  - Schema – “AGGREGATION”

  - Table – “BASE_1_609344”

- Click “OK” on the following prompt (see below image).

- Click “OK” after the DOS prompt disappears (see below image).

- A window will appear indicating that the records have been transferred. Click “OK” (see below image).

- Repeat Step 18 to Step 21 three times, changing the “Table” drop-down option in Step 18 to “BASE_4_437376”, then “BASE_100_934”, and finally “BASE_1000000” in that order. Please note that the PDW program first calls the “TALENT” tool to transfer a base table such as “BASE_1_609344” in Vision to a stage table named “VISION_ BASE_1_609344” in Oracle, then a stored procedure named “UPD_VISION_TABLE” in Oracle production database will be called by PDW to move the data from this stage table into the final base table “PDW17_BASE_1” in Oracle using 2017 data collection as example.

4- Performing QA of Uploaded Data – performed by Supervisor

Summary: This task performs QA checks on the uploaded data tables.

The completeness check is needed to make sure a) the base tables in Oracle do not contain any invalid runs or “not to use” runs marked in EDIT_CUR_YR table, and b) the base tables in Oracle contain all the valid runs in EDIT_CUR_YR table. This completeness check is currently done by running a SQL query. The base tables after the clean-up will serve as the basis for populating the business plan tables. Then, the speed adjustment fields are checked to ensure they were created and populated correctly. Finally, the data in the tables are checked for reasonableness.

## Reformatting ARAN Hard Drives

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to consolidate and reformat the hard drives which contain data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. Consolidation and re-formatting may occur any time after the ARAN data has been loaded into the Vision database, as outlined in Loading of ARAN Data into Vision.

### Frequency

Consolidation and reformatting of ARAN hard drives occurs sporadically, whenever external hard drive space is low (available hard drives are all full or in use).

### Purpose

The purpose of this SOP is to free up space on the ARAN hard drives so that more data can be collected. The drives are consolidated to save space and then reformatted so that they may be used again.

### Resource Requirements

Consolidation and reformatting of ARAN hard drives involves two people: (1) a user familiar with computer data storage to consolidate and reformat the drives, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the consolidation and re-formatting. These time estimates assume no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Operator | 1 | 4.0 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to consolidate and reformat ARAN hard drives is comprised of a single task:

- consolidating and reformatting ARAN hard drive.

1- Consolidating and Reformatting ARAN Hard Drive – performed by operator

Summary: This task consolidates and reformats an ARAN hard drive.

- Prior to consolidation, copy all data from the hard drive to another drive and re-record the drive data.

- Place the drive that has been backed up as part of Step 1 into the offline computer. “Admin” rights are needed to reformat drives.

- Turn the computer on and log-in using proper credentials.

- Once the computer has initialized and user is logged in, press the “Windows Key” and “E” on the keyboard.

- The “My Computer” window will open. Right click on “Computers” on the left side of the menu. Select “Manage” (see below image).

- The “Computer Management” window will open. Find “Storage” on the left side of the menu and select the sub-item “Disk Management” (see below image). The computer may take a few moments to load the disk configuration information.

- Right-click on the drive to be reformatted. Select “Format” (see below image).

- On the next window, click “OK.” Reformatting will begin.

- Once the reformatting has completed, click “OK.”

- Place the reformatted drive in a separate location and alert the FED of its location.

## Creating PAGDQC Report

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to create the “Pavement and Geotechnical Division Quality Control” (PAGDQC) report and upload it to the Pavement Data Warehouse (PDW). The report details the Automatic Road Analyzer (ARAN) data collected in the field by Field Explorations Division (FED) staff. The report is utilized within several other post-processing activities by the DPT.

### Frequency

Creating and importing the PAGDQC Report is conducted in-part with other post-processing SOPs utilized by the DPT. The DPT processes ARAN collected data in batches upon receipt from the FED. Each ARAN data batch is typically delivered to the DPT every 4 to 5 days. Assuming an average of 60 miles collected per day, each batch contains, on average, between 240 and 300 miles of ARAN data. The actual frequency varies according to the proximity of the routes on which data were collected for the data batch to the Office of Materials Technology (OMT) office and it ranges from 1 day (when collected near the OMT office) to 2 weeks (when collected far from the OMT office). Shorter turn-around periods are preferred to minimize the amount of mileage requiring data recollection. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to create the PAGD QC Report and upload it to the PDW.

### Resource Requirements

The creation and import of the PAGDQC Report involves two people: (1) a user knowledgeable in Roadware Vision software to create the PAGDQC Report and upload it to the PDW and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the report generation and upload. These time estimates assume average batch quantities and that no issues are to be addressed.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision User | 1 | 1.0 |
| DPT Team Leader (TL) | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure followed by the DPT staff for creation and import of the PAGDQC Report is comprised of a single task:

- creating and importing PAGDQC Report.

1- Creating and Importing PAGDQC Report – to be completed by DPT Staff

Summary: This task creates and populates a PAGD_QC_CSV file for all collection runs imported into the Vision database for each FED submission. The file is then imported into the PDW to update the Edit/View Unique Run Status program to allow tracking, status, and notes of each collection run (FED QC/QA, Route Matching QC/QA, etc.).

- Open Roadware Vision program. Click the “Report” drop-down menu and select “Report Generator” and the “Report Generator” window will appear (see below image).

- Select “PAGD_QC” from the list of reports. Click “Check.” The “Report Structure Explorer” window will open (see below image).

- Click the “Synchronize” button (see above image). When the button un-highlights itself, the process is complete. Now, click “Update” and wait for the button to un-highlight itself as well. Performing these two actions clears the PAGD_QC table in Vision06/12/2019. Close the “Report Structure Explorer” window.

- Click the “Select Source” button. The “Group Partitions Explorer” will open (see below image). Choose the batch of data to be reported (or create a batch to select the correct data). Click “OK.”

- The selected files will be shown in the bottom of the “Report Generator” window. Confirm they are correct. If they are incorrect, return to Step 4. Otherwise, click “Generate.” The “Report Job” window will open (see below image).

- Update settings. Click the “Filter”button and select “Unmatched.” This will run the report on both Matched and Unmatched files (see below image).

- Click the “Regenerate” button (see below image, button is highlighted in blue).

- Click the “Start” button (see below image, button is highlighted in blue).

- Files will begin processing. The “Eventlog” column will indicate the status of the processing. A progress bar will also estimate the overall status. When the program is complete, the progress bar will disappear and the “Eventlog” column will say “Processed” for each file (see below image).

- Report Job Window Processed

- If the PAGD_QC is being run on a submission, it must be exported. Otherwise, proceed to Step 9.

- To export, click the drop-down arrow next to the “Save” button and select “Export all” (see below image). Browse for the appropriate folder location and click “OK.” Click “Yes” to overwrite existing files. As the program writes the export, a red circle will appear beside each file (see below image). A dialogue box will appear when the export is finished. Click “OK.”

To confirm the export, navigate to the folder where the export was saved. The file should be called “PAGD_QC_1000000.csv”. Open the file and ensure there are no gaps in the data. Below is a table of column headers and a note on whether they should contain data.

| Field | Data? | Field | Data? |
| --- | --- | --- | --- |
| IDLocator | Data | RightRut_501_10 | Blank |
| L_County | Data | RightRut_1001_99 | Blank |
| L_RouteID | Data | Grade_N_99_1001 | Blank |
| L_Dir | Data | Grade_N_10_501 | Blank |
| UniqueRun | Data | Grade_N_5_001 | Blank |
| CollectionVehicle | Data | Grade_0 | Blank |
| DCSTimeStamp | Data | Grade_001_5 | Blank |
| IDSession | Data | Grade_501_10 | Blank |
| BeginDistanceStamp | Data | Grade_10_99 | Blank |
| EndDistanceStamp | Data | LeftIRI_TotalRecords | Data |
| BeginChainage | Data | RightIRI_TotalRecords | Data |
| EndChainage | Data | LeftRut_TotalRecords | Blank |
| Status | Data | RightRut_TotalRecords | Blank |
| Uniquerun1 | Data | Grade_TotalRecords | Blank |
| SPEED_AVG | Data | GPSTime_MAX | Data |
| LeftIRI_Avg | Data | GPSTime_MIN | Data |
| RightIRI_Avg | Data | GPSTime_COUNT | Data |
| LeftRut_Avg | Blank | Hertz_Actual | Blank |
| RightRut_Avg | Blank | SPEED_STDEV | Data |
| Grade_Avg | Blank | SPEED_CV | Blank |
| LeftIRI_0 | Blank | L_RUT_STDEV | Blank |
| LeftIRI_1_9 | Blank | L_RUT_CV | Blank |
| LeftIRI_10_999 | Data | R_RUT_STDEV | Blank |
| LeftIRI_1000_9998 | Varies | R_RUT_CV | Blank |
| LeftIRI_9999 | Blank | L_IRI_STDEV | Data |
| RightIRI_0 | Blank | L_IRI_CV | Blank |
| RightIRI_1_9 | Blank | R_IRI_STDEV | Data |
| RightIRI_10_999 | Data | R_IRI_CV | Blank |
| RightIRI_1000_9998 | Varies | GRADE_ABS_AVG | Blank |
| RightIRI_9999 | Blank | GRADE_ABS_STDEV | Blank |
| LeftRut_0 | Blank | GRADE_ABS_CV | Blank |
| LeftRut_001_2 | Blank | XFALL_ABS_AVG | Blank |
| LeftRut_201_5 | Blank | XFALL_ABS_STDEV | Blank |
| LeftRut_501_10 | Blank | XFALL_ABS_CV | Blank |
| LeftRut_1001_99 | Blank | COLL_CATEGORY | Data |
| RightRut_0 | Blank | PRIORITY | Data |
| RightRut_001_2 | Blank | RSECTION_COPY | Data |
| RightRut_201_5 | Blank |  |  |

- Import PAGD_QC into the PDW. Open the program icon to the left below (“GetLatestVe…”), not the one to the right (see below images).

- When the program opens, navigate to “Data\ARAN\Data Transfer from SQL to Oracle” (see below image).

- The window below will open. Make the following changes to the window (see below image).

  - Change the “Reporting Year” to the correct year using the drop-down menu.

  - Select database “OMT_ARAN_PROD_2017_1” using the drop-down menu.

  - Select DB Schema “DBO” using the drop-down menu.

  - Select Table “DCSESSIONS” using the drop-down menu.

Click “Transfer” (see above image). Click “OK” when the next dialogue box opens. The next dialogue box to appear will say “Please wait until DOS Prompt disappear”. Click “OK” after the DOS window closes. Click “OK” to return to the “Data Transfer” window.

- Change the Table to “LOCATORS” and click the “Transfer” button again (see below image). Click “OK” when the next dialogue box opens. The next dialogue box to appear will say “Please wait until DOS Prompt disappear”. Click “OK” after the DOS window closes. Click “OK” to return to the “Data Transfer” window.

- Change the Schema to “AGGREGATION” and the Table to “PAGD_QC_1000000” and click the “Transfer” button again (see below image). Click OK when the next dialogue box opens. The next dialogue box to appear will say “Please wait until DOS Prompt disappear”. Click “OK” after the DOS window closes. Click “OK” to return to the “Data Transfer” window.

- Change the “Schema” to “DBO”, then select the “MATCHES” table and click the “Transfer” button (see below image).

- Check the PDW’s “View/Modify Unique Run Status” table to make sure the data was imported. The PDW’s “View/Modify Unique Run Status” table needs to be updated once a week to show which files have been matched.

## Running Routing Importer

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to run the Routing Importer in the Roadware Data Control Software (DCS) using the Automatic Road Analyzer (ARAN) Collection List produced in the planning stage of the ARAN data collection process (see Production of ARAN Collection List). This SOP is performed once the ARAN Collection List for the year has been created and quality assurance (QA) has been performed.

### Frequency

The Routing Importer is run once per year, prior to the start of the annual data collection season, and then rerun as-needed, in the event any routes change throughout the year.

### Purpose

The purpose of this SOP is to import all routing data into the Vision Database using the Roadware DCS Routing Importer. It is also used to update existing routing data as-needed.

### Resource Requirements

The running of the Routing Importer involves two people: (1) a user knowledgeable in Roadware DCS software to run the importer, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the import. These time estimates assume no issues encountered during importing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | DCS user and QC | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to run the Routing Importer is comprised of a single task:

- running Routing Importer.

1- Running Routing Importer – performed by Roadware DCS user

Summary: This task uses Roadware DCS to import or update routing files into the Vision Database for ARAN data collection.

- Open Roadware DCS.

- Click the “Tools” drop-down menu and select “Routing Importer”, then select the current database06/12/2019.

- Make sure the correct database is displayed in the “Roadware Routing Importer” window. If not, return to Step 2 and select correct database. Once the database is correct, click “Browse” next to “Input Source File” (see below image).

- Select Inventory file06/12/2019 (see below image). Click “Open.”

- Select preferred template. There are two options:

  - Open an existing template. Click “Browse” next to “Template”. Select the proper template (XML file) from the same folder as the inventory file. Click “Open” and then “Next” and proceed to Step 8.

  - Create a new template. Instead of browsing for a file, click “Next” with the template blank and proceed to Step 6.

- “Mappings” window will open. Change settings to the preferred configuration. Click “Next” (see below image).

- “Units and Options” window will open. Ensure the “Chainage Units” are in “Miles” and the “GPS Units” are in “Degrees.” In addition, make sure the box next to “Skip importing” is checked for new inventory or “Overwrite” for updated inventory. Click “Next” (see below image).

- “Verification” window will appear.

  - If an existing template was selected in Step 5 and the inventory is new, click “Next.”

  - If an existing template was selected in Step 5 and the inventory is updated, click “Back” to return to the “Units and Options” window and make sure “Overwrite” is selected. Then, click “Next” and “Next” again.

  - If a new template was created in Step 5 click “Next.”

- “Summary” window will appear. Click “Next” (see below image).

- Import progress will be displayed. When the progress bar reaches 100%, import is complete. A summary below the progress bar will list the number of sections imported, overwritten, or skipped. When finished, click “Finish” and the program will close (see below image).

## Changing Latitude/Longitude

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to resolve incorrect route GPS information by changing the latitude and longitude of the beginning or ending point along a route. This SOP is used during route matching (Route Matching) to correct discrepancies between the inventory beginning/ending points and the collected route. Examples of situations where the latitude and longitude should be changed have been included as an appendix (see Examples for Changing Latitude/Longitude).

### Frequency

Changing the latitude and longitude of the beginning or ending point of a route occurs on an as-needed basis whenever incorrect locations are displayed on the map during route matching.

### Purpose

The purpose of this SOP is to resolve incorrect route GPS information by changing the latitude and longitude of the beginning or ending points along the route.

### Resource Requirements

Changing the latitude and longitude of the beginning or ending point of a route is performed by a member of the DPT during route matching to resolve incorrect route GPS information. The estimated effort levels in the table below represent the total time, in man-hours, to complete the attribute changes. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user | 1 | 0.5 |

### Procedure

The procedure to change the latitude and longitude of the beginning or ending point of a route is comprised a single task:

- changing the latitude and longitude of route beginning or ending point.

1- Changing the Latitude and Longitude of Route Beginning or Ending Point – performed by Vision user

Summary: This task identifies incorrect GPS information in the Vision database and resolves it by changing the latitude and longitude of the beginning or ending point of a route. This is a supplemental, corrective task that occurs during route matching (see Route Matching), so the Vision program will already be open, and the user will have decided that a beginning or ending location on the map is incorrect and needs to be updated.

- In the “Map” window, click on the “Options” icon (three horizontal lines in the upper left-hand corner of the screen).

- Select “Base Maps”, then “Google”, then “GoogleSatelliteMap” to turn on the satellite view so that the lane configuration of the route in question can be seen.

- Before continuing, check the same route in the other direction and confirm the location of the corresponding beginning or ending point (green balloon icon on map).

- Open the inventory Excel file for the current year. It will be stored at: \\shahanartico2\shaomtvideolog\(YEAR)\Data\Notes). The name of the file will have the format “ARAN_Collection_Routes_(MostCurrentDate).”

- Use the filter button (under “Data,” then “Filter”) to find the route in question. Filter by Column E, “ID_RTE_N.” Make sure both directions of the route are displayed (see below image).

- Copy the header row (Row 1) and the route rows and paste them into a blank Excel file.

- Copy the beginning latitude (Column X, “BEG_LAT”) and longitude (Column Y, “BEG_LONG”) for the first route direction into a Google search engine web page (see below image).

- Hit the Enter key. The results of the Google search will appear (see below image).

- Click on the map in the search results window to open Google Maps. Zoom in to the red balloon marker. Check that the red balloon is in the identical location as the green balloon in the Vision “Map” window.

  - If the two balloons are in the same location, proceed to Step 10.

  - Otherwise, the wrong latitude and longitude have been copied. Check that the route number is correct and that the correct coordinates have been copied into the browser. Resolve before continuing.

- In Google Maps, click the location on the map of the corrected beginning location. A dialogue box will open at the bottom of the screen. Click on the coordinates listed (see below image).

- The search box will change and the red balloon will move to the new location. Repeat if the new location is not satisfactory. Otherwise, copy the coordinates that are now displayed in the search box (see below image).

- Paste the coordinates back into the new Excel file in the appropriate fields. Highlight the updated cells in yellow.

- If multiple beginning or ending points require an update, repeat Step 7 to Step 12 for the other locations. When all points have been updated, proceed to Step 14.

- Save the Excel file. The name of the file can either be (1) the route name or (2) InventoryChanges(MMDDYY) if more than one route is being updated at once.

- Email the Excel file with the updated data to the DPT TL and the database developer.

- Import the updated routing information (see Running Routing Importer) before continuing with the matching of that route.

## Accessing Electronic ARAN Logs

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to access the electronic Automatic Road Analyzer (ARAN) logs in order to review data collection information. The electronic logs are created on the ARAN computer by field crew members of the Field Explorations Division (FED). The operator manually adds comments to the electronic log. The log is then added to the Pavement Data Warehouse (PDW) when the drives are recorded. This SOP is used during route matching (Route Matching) to check if multiple routes were collected in a single file.

### Frequency

Accessing electronic ARAN logs occurs on an as-needed basis whenever it is suspected that multiple routes may have been collected in a single file during route matching (see Route Matching).

### Purpose

The purpose of this SOP is to access information in the electronic ARAN logs to determine if multiple routes were collected in a single file.

### Resource Requirements

Accessing electronic ARAN logs is performed by a member of the DPT during route matching (see Route Matching) to determine if multiple routes were collected in a single file using the PDW application. The estimated effort levels in the table below represent the total time, in man-hours, to complete electronic log export and investigation. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 0.5 |

### Procedure

The procedure to access electronic ARAN logs is comprised a single task:

- accessing electronic ARAN logs.

1- Accessing Electronic ARAN Logs – performed by PDW user

Summary: This task accesses information in the electronic ARAN logs to determine if multiple routes were collected in a single file using the PDW.

- Open the PDW application.

- Click “Data” and select “ARAN”, then “Record Drive and First Image” from the drop-down menus (see below image).

- The “ARAN Drive Info” window will appear. Select the correct “Collection Year” from the drop-down menu under “Daily Log File” (see below image).

- In the box next to “Collection Year”, select the correct log or choose “All.” It is easier to select “All”, but the export will take longer (see below image).

- Click “Report.” The table that populates can be used to view routes and look at the ARAN operator’s notes, but they are not searchable (see below image).

- Click “Export to Excel” and choose a name and location to save the file. Click “Save” and wait for export to complete (if “All” was selected it may take a few minutes). Do not interrupt export by clicking the Excel icon on the taskbar.

- When the export is complete, open the Excel file and compare the number of rows to the “Record Count” in the PDW window (see below image).

- Examine the Excel spreadsheet. Find the file name in question using the “FILENAME” field, which displays the date and time of the collected route. The file names in Vision will not exactly match the file names from the electronic ARAN log (the time in Vision is in 12-hour format while the time in the log is in 24-hour format). The “HEADER” column will display the name of the route that was collected. Find the route in question and look at the “COMMENTS” column (see below image). If a file contains more than one route, it will be specified in this column (look for a “+” or “and” in the text). Please note that the “HEADER” and “COMMENTS” fields are entered manually in the field and the chance for human error exists.

## Using eGIS

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to search for route identifier information using the Maryland Department of Transportation’s (MDOT) eGIS system. This SOP is used during route matching (Route Matching) when discrepancies exist which involve checking route information. For further information regarding eGIS, refer to the Vision Matching 2015 guide.

### Frequency

Using the MDOT eGIS system occurs on an as-needed basis whenever route information must be checked during route matching.

### Purpose

The purpose of this SOP is to reference routing information using the MDOT eGIS system.

### Resource Requirements

Using the MDOT eGIS system is performed by a member of the DPT during route matching (see Route Matching) to resolve discrepancies in routing information. The estimated effort levels in the table below represent the total time, in man-hours, to complete the identification of a single route. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user | 1 | 0.5 |

### Procedure

The procedure to identify a route using the MDOT eGIS system is comprised a single task:

- identifying a route using eGIS.

1- Identifying a Route Using eGIS – performed by Vision user

Summary: This task uses the MDOT eGIS system to identify a route.

- Open a web browser and navigate to: https://onemdot/Pages/default.aspx. The MDOT intranet page will open (see below image).

- Click on “SHA APPS” and then select “eGIS.” The eGIS page will open (see below image).

- Uncheck the box beside “Centerline HLR Report” on the right side of the screen, then click the “Route Search” button (see below image).

- Use the drop arrows to select the correct District for the route in question (if the route is in Baltimore City, do not select a District).

- Select the correct County (Baltimore City will be an option at the bottom of the list if no District was selected).

- Select the correct Municipality if the route is in a city.

- Check the circle next to “Route Number.”

- In the drop-down menus, select the “Route Prefix” and “Route Number” (see below image). If the Route Number is not known, select a known Route Number that is close to the desired route.

- The map will populate with the route starting and ending points, as well as intermediate points along the route. The map can be moved by clicking and dragging or zoomed by using the mouse wheel. If the desired route number was not known, zoom in to its location. Click the “Identify” button at the bottom of the map screen and then click on the route on the map. The “Identify Features” window will populate with the correct route number and other information (see below image).

  - If the primary direction is required, change the “Select ID layer” to “Government Control” and view the “ROUTEID” field at the bottom of the window (see below image).

## Changing Image Banner or Logo

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Processing Team (DPT) to change the banner or logo on “Left” and “Row” images in the Roadware Vision software using data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. This SOP is used during the running of the Vibing Processor (see Running Vibing Processor) in the event that the banner or logo associated with the schema needs to be updated06/12/2019.

### Frequency

The changing of the banner or logo for “Left” and “ROW” images is performed as-needed when an alteration to the current format is requested. This is preferably performed prior to the beginning of the ARAN data collection season to ensure continuity in the output image format. The annual ARAN data collection season typically lasts from March to Mid-December.

### Purpose

The purpose of this SOP is to change the image banner or logo applied to ARAN collected “Left” and “ROW” images.

### Resource Requirements

Changing the image banner or logo involves two people: (1) a user knowledgeable in Roadware Vision software to update the image format prior to running the Vibing Processor, and (2) a supervisor who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the format changes. These time estimates assume no issues were encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision user and QC | 1 | 0.5 |
| DPT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to change the image banner or logo for “Left” and “ROW” ARAN images is comprised of a single task:

- changing the image banner or logo in Vision

1- Changing the Image Banner or Logo in Vision – performed by Vision user

Summary: This task uses Vision software to change the image banner or logo that is applied to ARAN collected “Left” and “ROW” images.

- While on the “Options” tab select “Vibe Parameters” and then click the “…” button on the right side of the screen (see below image).

- The “Banner Editor” window will appear. Change the image to either “Left” or “ROW” depending on which banner is to be edited by click the “Camera” drop-down menu and selecting “Left” or “ROW” (see below image).

- Click the “Fields” tab and the “Expression” window will populate. To make a change, click on one of the expressions. Edit the necessary information in the table to the right (see below image). Make sure the “FieldFont” and “FontColor” fields are the same for each field name.

  - To move the banner to a different location on the image, click and hold on the desired part of the banner and move to new location (see below image).

- To change the logo, click on the expression “MD_SHA_Logo_RGB_SM.p…” Click the “FilePath” table to the right and select the “…” button. Browse to the location of the new logo and select it (see below image).

- When all changes have been made, click the “Save” button on the “Banner Editor” toolbar. Click the “X” to close the “Banner Editor.”

- Click the “Save” button on the options screen. Change the filename to match its previous name but with the current date at the end.

## Year-End ARAN Data Closeout

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Processing Team (DPT) of the Maryland Department of Transportation State Highway Administration (SHA) to perform the year-end data closeout. The closeout entails copying data from the Vision program and from the Pavement Data Warehouse (PDW) into a Microsoft Excel spreadsheet, then running queries on the spreadsheet to identify missing or incomplete data, and where possible addressing those data issues. The year-end data closeout is performed at the district level – each district will have its own year-end data closeout spreadsheet.

### Frequency

The year-end data closeout is performed once per year by SHA DPT staff, after all routes have been matched (see Route Matching) and the required drive-through checks have been completed (see Performing 100% Drive Through).

### Purpose

The purpose of this SOP is to complete the data processing activities for the year by identifying missing routes, performing quality control (QC) checks of matched data, and filling in gaps identified in the data set.

### Resource Requirements

Performing the year-end data closeout involves the following two SHA staff members: (1) a DPT staff member knowledgeable in Roadware Vision software and the Pavement Data Warehouse (PDW) to identify missing routes, fill gaps in the data, and perform QC checks, and (2) a supervisor – typically a DPT Team Leader (TL) – who, as required, provides guidance and decision-making and performs quality assurance (QA). The estimated effort levels shown in the table below represent the total time, in man-hours, to complete the closeout and QC/QA checks. These time estimates assume no issues encountered during processing.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff Member | Vision/PDW user and QC | 1 | 20.0 |
| DPT TL | Supervisor and QA | 1 | As Required06/12/2019 |

### Procedure

The procedure to perform the SHA year-end data closeout is comprised of the following two tasks:

- completion of year-end data closeout spreadsheet, and

- QA of year-end data closeout spreadsheet.

1- Completion of Year-End Data Closeout Spreadsheet – performed by DPT staff member knowledgeable in Vision software and PDW

Summary: Under this task, missing routes are identified, data gaps are filled, and QC checks are performed by first building a district-level Microsoft Excel spreadsheet using data from Vision and the PDW.

- Follow the steps in Creating PAGDQC Report for the creation of the PAGDQC Report in Vision and then updating the PAGDQC Report in the PDW.

- In the PDW, navigate to and select “View/Modify Unique Run Status” (see below image).

- Using the drop-down menu, select the appropriate “Year” and set the “County” field to “All” (see below image).

- After the table populates with the corresponding records, select the “Export to Excel” button beneath the table (see below image) and save the Excel file as “AllEditTable.xlsx.”

- In the PDW, navigate to and select “FED” under the “Reporting” tab (see below image).

- Using the drop-down tabs, select the appropriate “Year,” then select “Pre-Route Matched - Priority 1” under “Report,” and then select “All” for “County” (see below image).

- After the table populates with the corresponding records, select the “Export to Excel” button beneath the table and save the Excel file as “Route Matched Priority 1.xlsx.”

- Repeat Step 5 through Step 7, except select “Route Matched - Priority 2” under “Reports” and save the Excel file as “Route Matched Priority 2.xlsx.”

- In the PDW, navigate to and select “Route Matching QC” (see below image).

- Using the drop-down tabs, select the appropriate “Year” and select “Matched Milepoint Other Than Expected with All Unique Run Status” under “Option” (see below image).

- After the table populates with the corresponding records, select the “Export to Excel” beneath the table and save the Excel file as “Matched Milepoint Other Than Expected.xlsx.”

- Save all exported Microsoft Excel files to a local drive.

- Rename the “View/Modify Unique Run Status” Microsoft Excel workbook to the name of the district for which the year-end data closeout is being performed.

- Open “View/Modify Unique Run Status” Microsoft Excel sheet. Use a filter on the “County” column of the sheet to select and delete all records that are not associated with the district for which the year-end data close-out is being performed.

- Rename the sheet tab of the open Microsoft Excel file to the name of the district for which the year-end data close-out is being performed (see below image).

- Create seven new sheets in the Microsoft Excel file by clicking on the “+” icon next to the existing sheet names. Name the new sheets as follows:

  - Matched priority 1 Diff >-5

  - Matched priority 1&2 >.10mi

  - Milepoints other than expected

  - Match discrepancies

  - Rechain

  - Step 6

  - Step 7

- Copy the data from the saved (in Step 12) Microsoft Excel files from the PDW into the corresponding new sheets as follows:

  - Copy “Route Matched - Priority 1” to “Matched priority 1 Diff >-5” sheet

  - Copy “Route Matched - Priority 1” and “Route Matched – Priority 2” to “Matched priority 1&2 >.10mi” sheet (Note: do not copy routes listed in the “Route Matched – Priority 2” file that have a “DIFF_PERC” and “COLLECTED LENGTH” of zero; they are routes that do not need to be collected).

  - Copy “Route Matching QC” to “Milepoints other than expected”

- In the three new sheets where data have been copied to, use a filter on the “County” columns to select and delete all records that are not associated with the district for which the year-end data close-out is being performed.

- In the main “District” sheet, move the “Route” column and all columns to the right of the “Route” column to Column N. This will leave Column I through M blank (see below image).

- Rename the blank columns as follows:

| Column | Name |
| --- | --- |
| I | Dir+CO+Route |
| J | Milesage Diff >0.10 |
| K | ’Milesage Diff >0.10 Notes |
| L | differences >-5 |
| M | ’differences >-5 notes |

- In the “Matched priority 1 Diff >-5” sheet, move the “ROUTE” column and all columns to the right of the “ROUTE” column one column to the right. The resulting columns will be as follows (Column B, J, and K will need to be added manually):

| Column | Name |
| --- | --- |
| A | COUNTY |
| B | Dir+CO+Route |
| C | ROUTE |
| D | DIRECTION |
| E | PRIORITY |
| F | SECTION_LENGTH |
| G | COLLECTED_LENGTH |
| H | DIFF_LENGTH |
| I | DIFF_PERC |
| J | Collected? |
| K | Checked |

- In the “Matched priority 1&2 >.10mi” sheet, move the “ROUTE” column and all columns to the right of the “ROUTE” column one column to the right. The resulting columns will be as follows (Column B, J, and K will need to be added manually):

| Column | Name |
| --- | --- |
| A | COUNTY |
| B | Dir+CO+Route |
| C | ROUTE |
| D | DIRECTION |
| E | PRIORITY |
| F | SECTION_LENGTH |
| G | COLLECTED_LENGTH |
| H | DIFF_LENGTH |
| I | DIFF_PERC |
| J | Collected? |
| K | Checked |

- Enter appropriate formulas into the newly created columns for the three sheets addressed in Step 19 through Step 22. The table below summarizes the appropriate formula to be entered for each new column in the three sheets (see image after table).

| Sheet | Column | Formula | Notes |
| --- | --- | --- | --- |
| District | Dir+CO+Route | =Q2&\" \"&G2&\" \"&N2&\" \"&O2&\"\" | This is for routes where the “RSUFF” column is blank. |
| District | Dir+CO+Route | =Q3&\" \"&G3&\" \"&N3&\" \"&O3&\" \"&P3&\"\" | Filter the “RSUFF” column hide blank cells, then paste the formula to the remaining records. |
| District | Milesage Diff >0.10 | =IF(COUNTIF('matched priority 1&2 >.10mi'!B:B, \$I2)=0, "Distance Okay","> 0.10") | If there are any matching Dir+CO+Route records in the Milesage Diff >0.10 sheet, then the formula will return >0.10. Until you start removing records all the routes listed will show >0.10. |
| District | differences >-5 | =IF(COUNTIF('matched priority 1 Diff >-5'!B:B, \$I2)=0, "Distance Okay","> -5") | If there are any matching Dir+CO+Route records in the Milesage Diff >-5 sheet, then the formula will return >-5. Until you start removing records all the routes listed will show >-5. |
| Matched priority 1 Diff >-5 | Dir+CO+Route | =D2&\" \"&A2&\" \"&C2 |  |
| Matched priority 1&2 >.10mi | Dir+CO+Route | =D2&\" \"&A2&\" \"&C2 |  |

- Using the Filter tool in Excel, isolate the following data and delete the records.

| Sheet | Data | Notes |
| --- | --- | --- |
| Matched priority 1 Diff >-5 | DIFF_PERC | Remove any record that has a DIFF_PERC of -4 to 100. |
| Matched priority 1 Diff >-5 | DIFF_LENGTH | Remove any record that has a DIFF_LENGTH -0.01 to inf. |
| Matched priority 1&2 >.10mi | DIFF_LENGTH | Remove any record that has a DIFF_LENGTH -0.1 to inf. |

- Open Vision. Click the “Segment” drop-down menu and select “Match Discrepancy Finder.” The “Find Match Discrepancies” window will appear (see below image).

- Click on the heading of the “Discrepancies” column so that the records (i.e., rows) on the table are sorted such that those records with data discrepancies are listed at the top of the table.

- Double-click on each file with a discrepancy and Vision will navigate to that file. Open the “Section Composition” window to edit the file. Follow the steps in Route Matching Route Matching to correct match discrepancies.

- Click the “Refresh” button at the top of the “Find Match Discrepancies” window so that changes made in the “Section Composition” window are reflected in the “Find Match Discrepancies” table.

- Repeat Step 27 and Step 28 until no more discrepancies exist.

- Right click on the “Find Match Discrepancies” window and select “Copy All with Headers” (see below image).

- Paste the records that were copied into the “Match Discrepancies” sheet created in the Microsoft Excel file.

- Run the rechain query in Vision (see query at the end of this SOP).

  - If all files have been rechained, paste a screenshot of the query results in the “Rechain” sheet created in the Microsoft Excel file.

  - If there are files that have not been rechained, follow the steps in Route Matching Route Matching to rechain. Then, rerun the query and paste a screenshot of the query results in the “Rechain” sheet created in the Microsoft Excel file.

- In the “District” sheet, check every route that has a >0.1 in Column J or a >-5 in Column L.

  - If the matching is correct, enter “Acceptable” in Column K (for notes pertaining to Column J) or Column M (for notes pertaining to Column L).

  - If the matching is incorrect, correct it in Vision and enter “Fixed” in Column K (for notes pertaining to Column J) or Column M (for notes pertaining to Column L). If a file is new or re-matching uncovers a previously non-drive through section, add the filename to the “Step 6” sheet in the Microsoft Excel workbook. If a file only requires re-importing, add file name to the “Step 7” sheet in the Microsoft Excel workbook.

- In the “Matched priority 1 Diff >-5” sheet, check all routes that have a “-100” in the “DIFF_PERC” column. Enter “Collected” or “No Collected” in Column J. All other routes listed in this sheet should be accounted for on the “District” sheet.

- In the “Matched priority 1&2 >.10mi” sheet, check all routes that have a “-100” in the “DIFF_PERC” column. Enter “Collected” or “No Collected” in Column J. Compare the filtered list to the one from the “Matched priority 1 Diff >-5” sheet. All other routes listed in this sheet should be accounted for on the “District” sheet.

- Email the completed Microsoft Excel file to the DPT TL for review.

2- QA of Year-End Data Closeout Spreadsheet – performed by TL

Summary: This task performs QA checks on the year-end district-level closeout spreadsheet.

- Review the transmitted Excel file and discuss any issues with DPT Staff for resolution.

Rechain Query

select
    c.uniquerun,
    c.idlocator,
    c.l_county,
    c.l_routeid,
    c.l_dir,
    c.routedlength,
    c.beginchainage,
    c.endchainage,
    c.dmi_distance,
    c.matched_distance,
    c.distance_diff
from
    (
        select
            b.uniquerun,
            b.idlocator,
            b.l_county,
            b.l_routeid,
            b.l_dir,
            b.routedlength,
            b.beginchainage,
            b.endchainage,
            b.dmi_distance,
            b.matched_distance,
            (abs(b.dmi_distance) - abs(b.matched_distance)) distance_diff
        from
            (
                select a.uniquerun,
                    a.idsession,
                    a.idlocator,
                    a.l_county,
                    a.l_routeid,
                    a.l_dir,
                    a.routedlength,
                    a.beginchainage,
                    a.endchainage,
                    (a.EndDistanceStamp_MMI - a.BeginDistanceStamp_MMI) DMI_Distance,
                    (a.EndChainage - a.BeginChainage) Matched_Distance,
                    a.matchcomment
                from
                    (
                        SELECT DCSessions.Uniquerun,
                            Matches.IDSession,
                            Matches.BeginChainage,
                            Matches.EndChainage,
                            Matches.BeginDistanceStamp * 0.621371192237334 BeginDistanceStamp_MMI,
                            Matches.EndDistanceStamp * 0.621371192237334 EndDistanceStamp_MMI,
                            Locators.IDLocator,
                            Locators.L_County,
                            Locators.L_RouteID,
                            Locators.L_Dir,
                            Matches.MatchComment,
                            abs(Matches.EndChainage - Matches.BeginChainage) RoutedLength
                        FROM dbo.Matches,
                            dbo.DCSessions,
                            dbo.Locators
                        where DCSessions.IDSession = Matches.IDSession
                            and DCSessions.IDLocator = Locators.IDLocator
                            and Locators.L_Routeid not like 'TL%'
                            and UPPER(Locators.L_Routeid) not like 'DUMMY'
                    ) a
            ) b
    ) c
where distance_diff > 1
    or distance_diff < -1

## Route Matching - Transfers

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Processing Team (DPT) of the Maryland Department of Transportation State Highway Administration (SHA) to transfer data collected by the Automatic Road Analyzer (ARAN) survey vans to the appropriate route or section using the vendor supplied Vision post-processing software. Data collection may occur on one or more routes under the same run/file route header to maximize efficiency (e.g., collection on interchange ramps). It is also possible that errors may occur (e.g., route keyed in incorrectly or extra collection on adjacent routes). To accommodate this issue, the ARAN Vision software contains features that enable the user to transfer and re-assign a portion(s) or entire collection run to the appropriate route(s) or sections(s).

### Frequency

Route matching transfers are a routine part of the SHA Route Matching process (Route Matching), which occur upon arrival of the Field Explorations Division’s (FED’s) ARAN QC/QA’d deliverables. These FED deliverables typically arrive during the April through early October timeframe, but they can also arrive at other times of the year.

### Purpose

The purpose of this SOP is to document the SHA routines and procedures associated with the route matching transfer process. The latest version of the ARAN “Vision Training Guide” should be referenced when addressing issues not covered by this SOP.

### Resource Requirements

The route matching transfer routine involves the following two SHA staff members: (1) a DPT staff member who is knowledgeable in the Vision software and the Pavement Data Warehouse (PDW) application to perform the route matching transfers, and (2) the DPT Team Leader (TL) to perform quality assurance (QA) functions and who, as required, provides guidance and decision-making. The Quality Control (QC) and Quality Assurance (QA) routines are incorporated into the Route Matching process (Route Matching). Any issues (and associated recommendations) are discussed with the DPT TL for guidance and decision-making. The estimated effort levels shown in the table below represent the total time, in man-hours, to complete route matching transfers for each FED deliverable. These time estimates assume average batch quantities and that no major issues are encountered during the transfer process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | Vision/PDW user and QC | 1 | 1.5 |
| DPT Team Leader (TL) | Supervisor and QA | 1 | 0.15 |

### Procedure

The procedure for transferring a portion(s) or entire ARAN data collection run to the appropriate route(s) or sections(s) is performed, as needed, during the Route Matching process (Route Matching). Once a route or section is identified for transfer, the following three tasks are performed:

- determine the target route or section,

- perform the transfer routine and QC, and

- perform QA.

There are 15 steps associated with these three tasks and the table below summarizes the specific steps associated with each task.

| Task | Steps |
| --- | --- |
| 1 | 1 - 2 |
| 2 | 3 - 14 |
| 3 | 15 |

1- Determine the Target Route or Section – performed by Vision and PDW user

Summary: This task identifies the collection run’s destination route or section.

- Using the PDW’s FED ARAN Electronic Daily logs feature (see Accessing Electronic ARAN Logs), review the collection run(s) to determine if the field crew identified additional routes to be assigned to the collection run. Use the “COMMENTS” field to identify comments, as illustrated in the image below.

- If a matched collection run contains an abnormally high (>0.10 mile) lead-in and lead-out, investigate the collection run to determine if the lead-in and lead-out could potentially be on another route that can also be matched. To help identify such cases, use the Vision Map’s “Routed Nodes” feature, HLR Guide, and eGIS’s SHA Highway System content feature tools illustrated and described in the following images.

Vision Routes Nodes feature:

HLR Guide:

SHA Highway Location Reference Webpage

eGIS’s SHA Highway System content feature:

http://shagbegis1/egis/

Using eGIS’s Street Map or Imagery Basemap feature, zoom in to the required area.

- Select SHA Highway System

- Left click “Point”

- Hover pointer over required portion of route and then left click.

- Route number will now be displayed.

- If additional route details are required, see below example display.

2- Perform the Transfer Routine and QC – performed by Vision user

Summary: This task allows the user to transfer and re-assign a portion or entire collection run to the appropriate route(s) or sections(s). QC is then performed.

- Open the “Section Composition” window, then right click on the collection run/file. The below window will now appear.

- Select “Transfer” from the displayed menu. The window below will appear.

- Left click on the “Select” button. The window below will appear.

- Locate the route to be assigned by using the route search (AL, BA, etc.), then route prefix (CO, IS, MD, etc.), and then select the appropriate route/direction. Select “OK”. The search window will close, and the “Transfer” window illustrated in the image below will be displayed.

- Confirm the destination of the transfer. If not correct, left click on the “Select” button again and search for the correct route.

- If an entire collection run requires transfer to a single route, select “Entire fragment” (see red arrow in above image). If transferring only a portion of the collection run/file, use the “Start from current position” or “End at current position” (see black arrow in above image). Note: If file contains two or more routes, user will need to start the transfer from the end – working backwards.

- Left click on the “OK” button. The collection run’s background will appear as shown in the below image.

- While hovering over the collection run, right click and select the “Go to transfer” button, which is shown in the below image. If there is an inadvertent transfer to the incorrect route, select the “Recall” button, which is also shown in the below image.

- The “Section Composition” window will open for the transfer’s destination(s) as in the images below. Using Route Matching - Transfers Task 2 (Developing a Route Match Candidate and QC), review and verify the transfer’s route destination and that the transferred run is located between the destination’s route expected begin and end anchor landmarks.

- Complete the transferred run’s route match by completing Route Matching - Transfers Tasks 2 and 3.

- Once the match is complete, select “Go to source” as shown in the below image to go back to the original route.

- Communicate processing and/or QC issues (including recommendations) to DPT TL so they can be resolved.

3- Perform QA – performed by DPT Team Leader

Summary: This task performs QA checks on route transfer process.

- The procedure for completing this step is incorporated in Route Matching - Transfers Task 5 (Performing Quality Assurance (QA), which is performed on a weekly basis.

## Loading Pavement Surface Friction Data

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA) Data Processing Team (DPT) to load pavement surface friction data collected by the Field Explorations Division (FED) into the Pavement Data Warehouse (PDW). During this process, the pavement surface friction data are converted to a format consistent with the other SHA pavement condition measures.

### Frequency

The loading of pavement surface friction data into the PDW is completed every time a delivery of such data is received by the DPT. The actual frequency varies according to the proximity of the routes on which data were collected for the delivery. The data collection season typically lasts from April to October, but deliveries may occur year-round.

### Purpose

The purpose of this SOP is to document the procedure for:

- Loading raw pavement surface friction data into the PDW and

- Converting the format of those data to one consistent with that for other pavement condition measures, in preparation for the 4-Phase Study (see Outlier Review), further analysis, and reporting.

### Resource Requirements

The loading of pavement surface friction data involves two people: (1) a DPT staff member that is knowledgeable of the PDW application for loading the raw friction data and for reassigning route identifiers for invalid records, and (2) the DPT Team Leader (TL) to perform quality assurance (QA) functions and who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered during loading.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DPT Staff | PDW user | 1 | 20.0 |
| DPT TL | Supervisor and QA | 1 | AR06/12/2019 |

### Procedure

The procedure required to load the pavement surface friction data into the PDW and to make the needed format changes to those data is comprised of the following two tasks:

- load and validate pavement surface friction data into the PDW, and

- convert validated pavement surface friction data from point values to ranges.

1- Load and Validate Pavement Surface Friction Data into the PDW – performed by PDW user

Summary: Under this task, the pavement surface friction data files, as delivered by the FED, are first loaded into the PDW and then route identifiers for invalid records are reassigned.

- Launch PDW and open the “Friction Data Loader” by clicking “Data,” “Friction,” and then selecting “Route Data Upload” from the main PDW interface.

- In the “Friction Data Loader” window, select a data upload method:

  - “By Folder” – if loading multiple data files at one time, or

  - “By File Name” – if loading a single data file.

- Click “Browse” to the right of the selected data upload method and navigate to the location of the friction data file/folder to upload. Friction data files from the FED will be in either text (.txt) or CSV (.csv) format. Use the drop-down menu at the top of the window to select the “Reporting Year“ (see image below).

- Click “Upload Data to Temp” to populate the friction data into the”FRICTION_TMP” table in Oracle. Once the upload is complete, the message in the following image will appear:

- In the “Friction Data Loader” window, click on the “Route Valid Check” button to check that all routes are valid in the friction data file(s). A route would be invalid if the county code, route number, or direction have been assigned incorrectly. Routes that successfully pass the check will have the data fields”global_route_id” and “sub_route_id” assigned to the associated friction data. The message shown in the following image will appear when the route validation process is complete:

- Click on the “Review Results” button and the window shown in the image below will appear. Use the”Selection**“** field at the upper left side of the window to view”Report - Valid Friction Route Passes” or “Report - Invalid Route Passes.” The focus of the data review will be on the invalid route records.

- View the “Report – Invalid Route Passes” and highlight rows in the table by clicking in the left-most column. After rows are highlighted, click on the “Modify Route Information“ button at the bottom of the window. The screen shown in the image below will appear:

- Use the fields to the right of the “New Route” row to re-assign correct route identifiers to the friction data. Click the”Apply Route Change**“** button to overwrite the original route identifiers with the newly selected ones. The options under”New Route” directly come from the PDW “Global Route Pav” table and therefore are the only valid identifiers. Updated records will also be assigned a “global_route_id” and “sub_route_id.”

- Repeat Step 7 and Step 8 until there are no more records in the invalid route report table.

- In the “Friction Data Loader” window (Step 1) click the “Accept Temp Data” button. The friction data in the”FRICTION_TMP” table will then be uploaded to the “FRICTION” table. The “FRICTION” table is the final table designated for storing friction data.

2- Convert Validated Pavement Surface Friction Data from Point Values to Ranges – performed by PDW user

Summary: Pavement surface friction is measured at a single point and therefore does not span the full length of a segment or range of pavement segments. To be consistent with other pavement condition measures, this task converts a single mile point to the beginning and ending mile points of every pavement surface friction measurement record.

- Navigate to the folder located at the following file path: N:\\OMT\\Pavement and Geotechnical\\PaveMang\\Quality Management Plan\\Pavement Management Documentation\\Friction Data Adjustments/.

- The below image shows the contents of the above referenced folder:

- Click on the SQL script in the folder named “Friction Update Script.sql.” The script will call and execute 10 individual SQL files (Steps 1 through Step 10 in the above image) in sequence to process the friction data. After the script finishes processing, the friction data will be ready for the 4-Phase Study (see Outlier Review) to identify potential outliers in the data. Following the outlier study, the data will be populated into the “BUSINESS_PLAN” tables for reporting and for further analysis).

## Skid Submittal QC

### General

This section describes the standard operating procedure (SOP) used by staff from the Field Explorations Division (FED) of the Maryland Department of Transportation State Highway Administration (SHA) to perform quality control (QC) checks on skid data collected by the agency.

### Frequency

This SOP requires that QC checks on skid data collected by the SHA be performed in batches as they are received from the FED crews throughout the data collection season 06/12/2019. The frequency and timing of the QC checks varies according to data collection schedule, available resources, and quantity of data received. Typically, the QC checks are performed once the skid data for an entire county has been completed.

### Purpose

The purpose of this SOP is to perform QC checks on the skid data collected by the SHA FED in order to identify and eliminate unreasonable data points.

### Resource Requirements

The actions detailed in this SOP are performed by the SHA FED Team Leader (TL). The estimated effort level shown in the table below represents the total time, in man-hours, required for completion of the QC checks for a given batch of skid data. These time estimates assume no issues are encountered during the checks.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| FED TL or Coordinator | QC | 1 | 1.0 |

### Procedure

The procedure required to complete the QC checks for a given batch of skid data is comprised of the following two tasks:

- running skid report, and

- running skid data analyzer.

5- Running Skid Report

Summary: Under this task, the Skid Report 1.3.2.0 software is used to analyze the skid data contained within a given batch by checking for data completeness and for appropriate use of file naming schemes.

- On the desktop, double-click on the “Skid Report 1.3.2.0” icon.

- Select “Skid Summary” from the drop-down menu06/12/2019.

- Select the desired data set to report.

- Select “Run Report.” This will create a .csvs file for the data.

- Perform required completeness checks on .csvs file as described below. Highlight any issues and alert the appropriate FED personnel if changes to the data must be made.

  - The file name is displayed correctly.

  - The collected data is complete when compared to the Skid Collection List.

  - The section numbers are logical for the specific road being collected (R section).

- Save .csvs file with QC notes in appropriate location.

6- Running Skid Data Analyzer

Summary: Under this task, the Skid Data Analyzer 1.1 software is used to analyze the skid data contained within a given batch by checking for unreasonable data points in relation to speed, water output, and skid number (SN).

- On the desktop, double-click on the “Skid Data Analyzer 1.1” icon.

- Select “Settings” and then “Location Settings.”

- Under “Save Route Data,” navigate to the appropriate collection list file.

- Under “Skid Results Data,” navigate to the .csvs file for the data collection.

- Click “Save.”

- Click on “Test Criteria Settings.”

- Accept or adjust thresholds for the various test criteria, including: MPH, Water Average, Lock Friction Value, Slip Friction Value, and Standard Deviation. The current value for each threshold is shown in the table below.

| Criteria | Minimum Value | Maximum Value |
| --- | --- | --- |
| Speed | 22 mph | 42 mph |
| Water Average | 60% of speed | 80% of speed |
| Lock Friction Value | 10 | 75 |
| Slip Friction Value | Not used | Not used |
| Standard Deviation | 0 | 12.5 |

- Click “Save Criteria.”

- Click on “Test Value Ranges.”

- Accept or adjust thresholds for the various test criteria, including: MPH, Water Average, Lock Friction Value, Slip Friction Value, and Standard Deviation. These ranges differ from the “Test Criteria Settings” in that the software will eliminate data points identified in these ranges, as opposed to simply flagging them. The current values for “Test Value Ranges” are 90% of the minimum value and 110% of the maximum value in “Test Criteria Settings.” The current value for each threshold is shown in the table below.

| Criteria | Minimum Value | Maximum Value |
| --- | --- | --- |
| Speed | 19.8 mph | 46.2 mph |
| Water Average | 54% of speed | 88% of speed |
| Lock Friction Value | 9 | 82.5 |
| Slip Friction Value | Not used | Not used |
| Standard Deviation | 0 | 13.75 |

- Click “Save Parameters” and then “Save Ranges.”

- Make sure “Lock Test” option is selected.

- Select the appropriate county from the drop-down menu.

- Click on the “Get Save Data” and “Yes” buttons, and then close the pop-up window that appears.

- Click on the “Validate Files” and “Yes” buttons, and then close the pop-up window that appears.

- Click on the “Process Skid Data” and “Yes” Buttons.

- Click on the “Processed Data” tab. A list of processed files will appear. View plots of the various data runs by double-clicking on a route name. Flagged data points will be identified in both the process data table and the plots.

- Click “Show in Excel” and then “Save.”

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA, DWT with DAT, and EDW with PDW |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart and HLR Link. Added missing alt-text for images |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Data Migration

Click to go to Outlier Review

Click to go to Update Final Condition Detail Table

Click to go to Update Rut Detailed Table

Click to go to Update Cracking Detailed Table

Click to go to Update Concrete Detailed Table

Click to go to Migrating to Business Plan Tables

## Introduction

This section describes the set of standard operating procedures (SOP) conducted mainly by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to integrate the Base Tables to the Business Plan Tables. This set of SOPs includes six procedures.

The first step in the data migration process is to identify and review outliers in performance measure data (IRI, rut, structural crack density, and functional crack density). This helps the DAT to flag questionable data and investigate the possible causes in order to define remedial actions. Following the outlier review, four intermediate databases are created: the Final Condition Detail Table, the Rut Detailed Table, the Cracking Detailed Table, and the Concrete Detailed Table. These intermediate databases prepare the data for migration to the Business Plan Tables, which includes adjusting location information to address potential errors in GPS data, removing outliers, and formatting the data. The final step in the data migration process consists of the production of the Business Plan Tables through the use of Oracle SQL Developer packages. QC checks of the tables are performed throughout every step of data migration.

Figure 9 shows a flowchart of the Data Migration SOPs.

Figure 9: Data Migration SOPs Flowchart

## Outlier Review

General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Warehouse and Data Analysis Teams (DAT and DAT) to perform the outlier review with data collected by the Automatic Road Analyzer (ARAN) survey vans in the field by Field Explorations Division (FED) staff. This study is performed after the ARAN data have been uploaded from Vision to Oracle and the DAT has been notified by the Data Processing Team (DPT) (see Reporting and Uploading Data to Oracle). The outlier review process identifies data deviating from expected trends and determines possible causes for outlier performance measure values by evaluating historical ARAN data, including speed, construction history, flagged events, pavement imagery, Right-of-Way (ROW) imagery, and comments from the previous year’s study. The list of outlier codes is included as an appendix to this SOP (see Outlier Review - Outlier Codes). Outlier identification process identifies and reviews outliers for performance measures such as International Roughness Index (IRI), rut, structural crack density (SC), and functional crack density (FC). The initial study consisted of four phases to identify only IRI outliers and to determine the cause(s) of those anomalies, as illustrated in the figure below.

However, in 2018, a new study was conducted, and the procedures were revised to address the changes and improvements in data collection and processing technologies and to include other performance measures in the process. The new procedure looks at each permornance measure separately and uses unique outlier identification criteria for each measure. Each performance measure requires a unique and customized set of criteria due to the nature of each measure, and different data collection and processing methods and as a result different acceptable ranges of data variation for each measure. The new approach developed for identifying outliers in condition data is referred to as the Hybrid Approach because it combines different outlier identification approaches for parameters into a unified outlier flagging scheme. With the hybrid approach, a section is only flagged as an outlier if the deviation from the previous year is considered abnormal when previous year’s data is valid. The Hybrid approach makes use of parameter trends in the entire network to predict the expected current values and uses data variation in valid test loop sections to define acceptable boundaries for deviation in each parameter. The main assumption with this approach is that the rate of deterioration of a given parameter depends on the current performance level. Deterioration rate versus performance level graghs are developed based on network level data every year and are used for predicting the current year data based on previous year performance levels. Test loop data is carefully monitored and is considered as reliable data which can be used to assess pavement material behavior over time. Data from the test loop were utilized in defining an acceptable window of variation for each parameter. When no maintenance and rehabilitation activity is carried out on a section of test loop over a data collection season, the variation of the collected and processed data in that section can serve as a proxy for acceptable variation of measured pavement condition parameter values.

The following figure is a schematic representation of the Hybrid Approach for flagging outliers for each performance parameter. If a given observation does not fall within the acceptable prediction zone, it is flagged as an outlier. The prediction zone is simply “predicted value Â± acceptable range of variation/2”.

In the case of rut, it was observed that low rut values are extremely volatile and result in a high percentage of outliers. Thus, based on historic data, a threshold of 0.15” rut in the current year is used for previous year’s rut values below 0.07”.

Frequency

The outlier review is performed multiple times per year, after a set of ARAN data, typically by county or by district, have been uploaded to Oracle and the DAT has been notified by the Data Processing Team (DPT).

Purpose

The purpose of this SOP is to identify performance measure outliers and to determine the cause(s) of those anomalies.

Resource Requirements

The outlier review involves two or three people: (1) one or two database management experts from the DAT and DAT to perform the outlier identification and review and quality control (QC) checks, and (2) a supervisor who, as required, provides guidance and decision-making and performs quality assurance (QA) checks. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process every time a new set of data is uploaded to Oracle. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 2 | 24.0 |
| DAT TL | Supervisor and QA | 1 | AR1 |

Procedure

The procedure to perform the outlier review is comprised of two tasks:

- performing the outlier review and QC,

- performing QA on the outlier review results.

1- Performing Outlier Review – performed by database engineers

Summary: This task identifies causes for outlier performance measure values using historical ARAN data.

- Before the start of the outlier review, a new procedure named by current collection year needs to be created and updated in the “EVENTS_DEFAULT” package. For instance, for collection year 2018, the procedure “PHASE4_DEFAULTS_2018” needs to be created by copying the codes from the procedure “PHASE4_DEFAULTS_2017” and updating the table names used in the codes from “EDW17_BASE_4” to “EDW18_BASE_4”. The purpose of this procedure is to generate the default outlier codes based on the events identified during DPT’s VISION drive through efforts as well as the construction history records entered by the DPT team. Run the procedure to insert the default codes into the “Phase4_Outlier_Detail_Default” table.

- Open the DCS RoadCare software from the computer desktop and enter the username and password provided by the RoadCare administrator.

- In the RoadCare Explorer window, to the left of the screen, click on “Attributes” and then double-click on the intended performance measure QC attribute (“Performance_Measure_QC”). The procedures should be performed on following attributes: “AVG_IRI_QC”, “AVG_RUT_QC”, “FC_DENSITY_QC”, and “SC_DENSITY_QC”.

- Check the box next to “Allow Attribute Edit” at the top of the data window (see below image).

- Click the “Advanced Search” button on the far right side of the data window. Next, in the data window shown below, enter the following query: “EXTRACT(YEAR FROM DATE_) = QC YEAR” and then click “OK” (see below image). This step is intended to filter the data that will be replaced with imported data.

- Click on the red icon at the bottom of the data window back on the main RoadCare screen, and then click “Yes” on the dialogue box that appears (see below image). This step deletes the filtered data.

- After the particular year data is deleted, run the specific attribute query in Oracle SQL Developer from file named: “4Phase Data import queries-QC Year.sql ,” which is saved in: \\shahanpmdata1\pmdataprg\Shayan\4 Phase Study. This step is intended for checking the query and query results before importing the data.

- Double click on the “Import from data source…” button to open the “Data Import” window. Enter the database connection parameters in the left hand top portion of the window and click “Connect” to verify the connection. Paste the SQL statement for the specific QC attribute in the text box and click the “Import” button to import the data. This is the preferred method of data import for large datasets. Alternatively, for smaller datasets, copy and paste the results from the above query by pressing “CTRL+A” in Oracle SQL Developer and “CTRL+C” in RoadCare. Double check that the data is copied by clicking on the year drop-down menu in the main window and selecting that year (see below images).

- Following importing of the new data, close the “Performance_Measure_QC” window and perform steps 2-7 for the remaining “Performance_Measure_QC” attributes. Once data import is completed double-click on the “Networks” option in the left hand window followed by “MDSHA_‘YEAR’_QC Network” followed by “Dynamic Segmentation” followed by “Rollup” (see below image).

- Click the “Rollup Network” button.

- Open Engineering Data Warehouse (EDW) application.

- Click on “Analysis” and then select “DAT QC/QA”, “ARAN”, and “Outlier Review” from the drop-down menus.

- Click on “Get List from RoadCare” (see below image). The data will be imported from RoadCare and stored in the “phase4_list_storage” table in Oracle.

- In Oracle SQL Developer, go to the production database and run the following query: “SELECT * FROM PHASE4_LIST_STORAGE”. Export the data from “PHASE4_LIST_STORAGE” table to an excel file and save the file as a backup file with date in the corresponding folder. To export the table simply right click on the data, click on the Export option and save the file as a “.xlsx” file format. Then, copy all the data from “phase4_list_storage” table in Oracle SQL Developer using “CTRL+A.”

- Open the “Outlier_Identifier-Template.xlsm” in Microsoft Excel and paste the copied data in the corresponding columns of the “Data” sheet. The spreadsheet is saved at the following location: \\shahanpmdata1\ pmdataprg\ Shayan\4 Phase Study. Make sure the columns from the Excel output match the Excel template file. After pasting the data make sure the number of records is accurate and “COLLECTION_YEAR” column is populated for all the records.

- Select the “RUN OUTLIER IDENTIFICATION” button at the top of “Data” sheet and make sure number of copied rows matches the number of outlier calculation rows at the far right of the spreadsheet. If number of rows do not match, then extend or remove the calculation rows to obtain matching number of rows. Then save the file as “Outlier_Identification_RunX_MM-DD-YY.xlsm” in the corresponding year’s outlier review folder.

- Copy the values in columns A-AD from “Data” sheet of the latest “Outlier_Identification_RunX_MM-DD-YY.xlsm” file and paste them in the corresponding columns of the “Load_to_Oracle.xls” file saved in \\shahanpmdata1\ pmdataprg\ Shayan\4 Phase Study. Then, save the file as “Load_to_Oracle_MM-DD-YY.xls” in the corresponding year’s outlier review folder.

  - If the “Phase4_outcome” is 0, then the section is not an outlier and therefore not selected for further investigation.

  - If the “Phase4_outcome” is 1, then the section is an outlier and therefore selected for further investigation.

- In Oracle SQL Developer, right-click on the table titled “Phase4_ Outlier_Summary.” Remove previously imported data using one of the following methods.

  - In Oracle SQL Developer, use the following command: “DELETE PHASE4_ OUTLIER_SUMMARY WHERE COLLECTION_YEAR = [CURRENT YEAR].” The existing year current data will be deleted and replaced with the updated and current data.

  - Click on “Table” followed by “Truncate.” This will delete all the data from the table without deleting its properties (see below image). Use of this method should be limited to those cases where the table contains current year data only. This method should be avoided if the table contains data from previous years as it will remove all data.

- Click “Import Data” in the same table. Select the latest “Load_to_Oracle_MM-DD-YY” file. In the step 4 of the data import select the “Position” option in the “Match by” drop-down menu. Since data import is done based on column positions, check the column positions of both tables before data import.

- In the EDW application “Outlier Review” window (see Step 12 if window has closed), click on the year drop-down menu and select the revew year, then click on the View Outlier Summary Button to import the data from Oracle and populate relevant tables and data fields. Afterward, click on the “Filter” drop-down menu and select “Outlier Sections Checks Not Completed” (see below image).

- All the outlier sections where the “Outlier_outcome” has been marked as “1” previously will be shown in the window. Click on the “Fit” button to expand the columns.

- Investigate each section and enter additional data where necessary. Double-click on a section and an additional “Add/Edit Outlier Details” window will appear (see following image). Open the current and previous year Vision database to supplement the investigation. In Vision, check for recent or active construction activities, data collection lane changes, vehicle wander, new distresses or other possible causes for data discrepancy for the section in question. Review the previous year Vision database and historical Vision databases and compare with current year Vision for the same section to determine the reasons for data discrepancy. Use the “Detail_Default” and “Year” drop-down menu to explore data populated from existing data tables including construction history data and pavement event data flagged by DPT technicians as a complementary source of data for the review (Note: the current performance parameters are: IRI, structural crack density, functional crack density, rut, and friction. The section will be an outlier if any of these parameters are flagged as outliers).

  - If there is a difference in the Vision data affecting any of the performance parameters, specify the reason for change of affected parameters (IRI, structural crack density, functional crack density, rut, and friction) by highlighting the affected performance parameters and entering the corresponding outlier code in the “Outlier Code” text box in the EDW window. Outlier codes are included as an appendix (see [Outlier Review - Outlier Codes]).

  - If no reasons for the discrepancy can be identified after comparing Vision data, then an investigation will need to be conducted comparing current and historic years’ condition databases. This investigation should look for any possible reason including errors in data collection, processing, transfer, storage, and analysis. If no reason can be found, outlier code “25 - No Reason” will be selected and an explanation will be added.

  - If no explanation for the discrepancy is identified after comparing databases, and data discrepancy is considerable, then the section will go to a recollection list to validate the true value. Currently, outlier review is done gradually along the season on a county by county basis. Once data processing and reporting is complete for a county, outlier identification and review will be performed on the data. If the difference in parameters is not significant and data is marginally outside of the acceptable range, then usually nothing is done about it. However, if the difference is remarkable and such difference is observed in meny sections without any explanation, it is concluded that collected data has issues then source of the issue is investigated and if necessary, recollection is recommended.

- Investigate all outlier sections until either explanation has been provided for the outliers or the sections have been moved to the recollection list. Once done, the “Outlier Sections Checks Not Completed” list in the EDW should be empty. Once the “Add/Edit Outlier Details” window is completed and closed for a section, the section automatically moves to the “Outlier Sections Checks Completed” list.

- All the outlier codes identified during the outlier review will be populated into the “Phase4_Outlier_Detail” table. All the events identified earlier by ARAN operators, construction history, video review, and available data such as speed less than 15 mph with corresponding codes will be stored in “Phase4_Outlier_ Detail_Default” table. The data from those two tables should be exported into the “Condition” table using the procedure: “Imp_OutlierCode_To_cond” followed by “Imp_Default_Events_To_Cond.”

- After all the outlier codes are populated from the above referenced two tables, run the following SQL query to insert comma at the end of each outlier code: “Update condition set note=note||',' where note is not null and year = YEAR OF QC;”

2- Performing QA on Outlier Review Results – performed by DAT TL

Summary: This task performs QA checks on the output from the outlier review in order to identify possible causes of and explanations for the outlier data and to enable discussions with appropriate SHA teams in order to pursue possible remedial actions.

- Perform detailed investigation of outlier review output tables “Phase4_Outlier_ Detail” and “Phase4_Outlier_Detail_Default.” Using SQL Developer, run queries to review location of outliers and investigate possible causes or explanations for higher values (e.g., a group of outliers in the same area could be traced back to a problem with the Distance Measuring Instrument (DMI) calibration). This step specifically intends to identify systematic errors or errors that affect a group of data. If an issue is discovered, discuss remedial actions with the appropriate SHA team.

## Update Final Condition Detail Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to populate, update and quality control (QC) the annual Final Condition Detail Table. This procedure is performed after the update of the Base 4 Table (see Reporting and Uploading Data to Oracle) and after the completion of the Outlier Review (see Outlier Review). The Final Condition Detail Table is populated and updated with the processed International Roughness Index (IRI) and rut data. The population and updating of this table is one of several intermediate steps performed by the DAT prior to the migration of all intermediate tables to the Business Plan Tables.

### Frequency

The Final Condition Detail Table is populated and updated once per year.

### Purpose

The purpose of this SOP is to populate, to update and to QC the Final Condition Detail Table for migration of the processed IRI and rut depth data to the Business Plan Tables. This procedure performs location adjustments to the IRI and rut data to address inconsistencies in GPS data, removes outliers, and organizes the data in preparation for the Business Plan Table migration.

### Resource Requirements

The updating of the Final Condition Detail table involves two people: (1) a database management expert from the DAT to populate and update the table and perform QC, and (2) a supervisor, typically the DAT Team Leader (TL), who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 1 | 40.0 |
| DAT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to update the Final Condition Detail table is comprised of the following three tasks:

- manually populating initial table,

- applying location adjustments, and

- performing QC of updated table.

1- Manually Populating Initial Table – performed by database engineer

Summary: This task manually prepares the initial table using the Base 4 table. SQL Developer is utilized to add rows and to update field names in the table for the current year.

- Using Oracle SQL Developer, initialize the Final Condition Detail Table by adding rows of data for the current year. Rename fields that may have changed from the previous year.

2- Applying Location Adjustments – performed by database engineer

Summary: This task creates new fields in the table for location adjustments through the running of a package in Oracle SQL Developer.

- Run the IRI_CLEAN_UP_PKG package to correct inconsistencies in the GPS data. This package compares ARAN data GPS data to historical DSD GPS data and makes corrections where differences may occur. The package compares location data at an interval of every 4 mmi (milli-miles). Two sets of location fields are maintained. The original locators refer to the data collection year and the adjusted locators refer to the inventory year.

  - If the ARAN location data matches the historical location data at a data point, no adjustments are made.

  - If the ARAN location data deviates from the inventory year location data at a data point, the ARAN location data is adjusted to match the inventory year location data. All subsequent points along the route are adjusted by this difference.

3- Performing QC of Updated Table – performed by database engineer

Summary: This task checks that the updates to the Final Condition Detail Table have been performed correctly.

- Using Oracle SQL Developer, check that the new rows have been populated and updated in the Final Condition Detail Table.

  - If the table has been updated, proceed to Step 4.

  - If the table has not been updated, confirm that the process has been completed correctly by returning to Step 1.

- Using Oracle SQL Developer, check that the location adjustment fields have been created and populated.

  - If the new fields have been created and populated, process is complete.

  - If the new fields have not been created and/or populated, confirm that the process has been completed correctly by returning to Step 2.

## Update Rut Detailed Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to populate, update and quality control (QC) the annual Rut Detailed Table. This process is performed after the update of the Base 4 Table (see Reporting and Uploading Data to Oracle), after the completion of the Outlier Review (see Outlier Review), and after the update of the Final Condition Detail Table (see Update Final Condition Detail Table). The Rut Detailed Table is populated and updated with the processed rut data. The populating and updating of this table is one of several intermediate steps performed by the DAT prior to the migration of all intermediate tables to the Business Plan Tables.

### Frequency

The Rut Detailed Table is populated and updated once per year.

### Purpose

The purpose of this SOP is to populate, to update and to QC the Rut Detailed Table for migration of the processed rut data to the Business Plan Tables. This process performs location adjustments to the rut data to address inconsistencies in GPS data, removes outliers, and organizes the data in preparation for the table migration.

### Resource Requirements

The updating of the Rut Detailed table involves two people: (1) a database management expert from the DAT to populate and update the table and perform QC, and (2) a supervisor, typically the DAT Team Leader (TL), who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 1 | 16.0 |
| DAT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to populate, to update and to QC the Rut Detailed table is comprised of the following three tasks:

- manually populating initial table,

- running rut package, and

- performing QC of updated table.

1- Manually Populating Initial Table – performed by database engineer

Summary: This task manually prepares the initial table using the Base 4 table and the results from the location adjustments made in the Final Condition Detailed Table. SQL Developer is utilized to add rows and update field names in the table for the current year.

- Using Oracle SQL Developer, initialize the Rut Detailed Table by adding rows of data for the current year. Rename fields that may have changed from the previous year.

- Using Oracle SQL Developer, extract location adjustments from the processed rut data in the Final Condition Detail Table.

2- Running Rut Package – performed by database engineer

Summary: This task organizes the processed rut data through the running of a package in Oracle SQL Developer.

- Run the RUT_DETAILED_PKG package to update the Rut Detailed Table.

3- Performing QC of Updated Table – performed by database engineer

Summary: This task checks that the updates to the Rut Detailed Table have been performed correctly.

- Using Oracle SQL Developer, check that the new rows have been populated and updated in the Rut Detailed Table.

  - If the table has been updated, proceed to Step 4.

  - If the Table has not been updated, confirm that the process has been completed correctly by returning to Step 1.

- Using Oracle SQL Developer, check that the location adjustment fields have been created and populated.

  - If the new fields have been created and populated, process is complete.

  - If the new fields have not been created and/or populated, confirm that the process has been completed correctly by returning to Step 2.

## Update Cracking Detailed Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to populate, to update and to quality control (QC) the annual Cracking Detailed Table. This process is performed after the update of the Base 4 Table (see Reporting and Uploading Data to Oracle), after the completion of the Outlier Review (see Outlier Review), and after the updates of the Final Condition Detail Table (see Update Final Condition Detail Table) and Rut Detailed Table (see Update Rut Detailed Table). The Cracking Detailed Table is populated with the processed Laser Crack Measurement System (LCMS) data. The populating and updating of this table is one of several intermediate steps performed by the DAT prior to the migration of all intermediate tables to the Business Plan Tables.

### Frequency

The Cracking Detailed Table is populated and updated once per year.

### Purpose

The purpose of this SOP is to populate, to update and to QC the Cracking Detailed Table for migration of the processed cracking data to the Business Plan Tables. This process performs location and density adjustments to the cracking data to address inconsistencies in GPS data and cracking quantities, removes outliers and special event locations, and organizes the data in preparation for table migration.

### Resource Requirements

The updating of the Cracking Detailed table involves two people: (1) a database management expert from the DAT to populate and update the table and perform QC, and (2) a supervisor, typically the DAT Team Leader (TL), who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 1 | 16.0 |
| DAT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to populate, to update and to QC the Cracking Detailed Table is comprised of the following three tasks:

- manually populating initial table,

- applying cracking density adjustments, and

- performing QC of updated table.

1- Manually Populating Initial Table – performed by database engineer

Summary: This task manually prepares the initial table using the Base 4 table and the results from the location adjustments made in the Final Condition Detailed Table. SQL Developer is utilized to add rows and update field names in the table for the current year and to remove special event locations in the data.

- Using Oracle SQL Developer, initialize the Cracking Detailed Table by adding rows of data for the current year. Rename fields that may have changed from the previous year.

- Run the CRACKING_TABLES_MP_CORRECTION package to extract location adjustments from the processed data in the Final Condition Detail Table and apply those adjustments to the processed cracking data.

- Run the CRACKING_EVENT_LOCATION package to filter out rows marked with a special event. Event markers are placed on images that contain appurtenances, which should be removed from the cracking data, including bridges, railroad crossings, manhole covers, concrete pavement, etc. The placing of these markers is performed as a part of Performing 100% Drive Through.

2- Applying Cracking Density Adjustments – performed by database engineer

Summary: This task applies a correction factor for cracking density to the processed cracking data based on manual evaluations performed during data processing.

- Run the CRACKING_DENSITY_CORRECTION package to apply the required correction factor to the cracking quantities in the Cracking Detailed Table. As a part of the cracking data processing QC, 1% of the network is manually evaluated and the resulting quantities are compared to the automated quantities collected by the ARAN. This comparison results in the development of a correction factor that is applied to the processed data. For sections that did not receive a manual evaluation, an average correction factor is applied based on the county and functional class associated with the specific section. A separate table called CRK_AUTODENSITY_CORR_BINS is created prior to the running of the CRACKING_DENSITY_CORRECTION package, which contains the route-specific correction factors in bins that will be looked up by the density package.

3- Performing QC of Updated Table – performed by database engineer

Summary: This task checks that the updates to the Cracking Detailed Table have been performed correctly.

- Using Oracle SQL Developer, check that the new rows have been populated and updated in the Rut Detailed Table.

  - If the Table has been updated, proceed to Step 4.

  - If the Table has not been updated, confirm that the process has been completed correctly by returning to Step 1.

- Using Oracle SQL Developer, check that the location adjustment fields have been created and populated.

  - If the new fields have been created and populated, proceed to Step 7.

  - If the new fields have not been created and/or populated, confirm that the process has been completed correctly by returning to Step 2.

- Using Oracle SQL Developer, check that the cracking density adjustment fields have been created and populated.

  - If the new fields have been created and populated, process is complete.

  - If the new fields have not been created and/or populated, confirm that the process has been completed correctly by returning to Step 4.

## Update Concrete Detailed Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to update and quality control (QC) the annual Concrete Detailed Table. This process is performed after the update of the Base 4 Table (see Reporting and Uploading Data to Oracle), after the completion of the 4 Phase Study (see Outlier Review), and after the updates of the Final Condition Detail Table (see Update Final Condition Detail Table), Rut Detailed Table (see Update Rut Detailed Table), and Cracking Detailed Table (see Update Cracking Detailed Table). The Concrete Detailed Table is populated with all processed data for concrete pavements. The updating of this table is one of several intermediate steps performed by the DAT prior to the migration of all intermediate tables to the Business Plan Tables.

### Frequency

The Concrete Detailed Table is updated once per year.

### Purpose

The purpose of this SOP is to populate and update the Concrete Detailed Table for migration to the Business Plan Tables. This process organizes the data for concrete sections only in preparation for the table migration.

### Resource Requirements

The updating of the Concrete Detailed table involves two people: (1) a database management expert from the DAT to populate and update the table and perform QC, and (2) a supervisor, typically the DAT Team Leader (TL), who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 1 | 4.0 |
| DAT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to populate, to update and to QC the Concrete Detailed Table is comprised of the following three tasks:

- manually populating initial table,

- running concrete package, and

- performing QC of updated table.

1- Manually Populating Initial Table – performed by database engineer

Summary: This task manually prepares the initial table using the Base 4 table. SQL Developer is utilized to add rows and update field names in the table for the current year and to remove special event locations in the data.

- Using Oracle SQL Developer, initialize the Concrete Detailed Table by adding rows of data for the current year. Rename fields that may have changed from the previous year.

2- Running Concrete Package – performed by database engineer

Summary: This task takes the previously processed tables (Final Condition Detail Table, Rut Detailed Table, and Cracking Detailed Table) and extracts the concrete sections into a separate table.

- Run the CONCRETE_DETAILED_PKG package to populate the Concrete Detailed Table with data from the concrete sections only.

3- Performing QC of Updated Table – performed by database engineer

Summary: This task checks that the updates to the Concrete Detailed Table have been performed correctly.

- Using Oracle SQL Developer, check that the new rows have been populated and updated in the Concrete Detailed Table. If the Table has not been updated, confirm the process has been completed correctly by returning to Step 1.

## Migrating to Business Plan Tables

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to reformat and to migrate intermediate tables - Final Condition Detail Table (Update Final Condition Detail Table), Rut Detailed Table (Update Rut Detailed Table), Cracking Detailed Table (Update Cracking Detailed Table), and Concrete Detailed Table (Update Concrete Detailed Table) - to the Business Plan Tables and to perform quality control (QC) on the migrated tables. This process is performed after the completion of the intermediate tables.

### Frequency

Reformatting and migration of the referenced intermediate tables to the Business Plan Tables and QC of the migrated tables is performed once per year.

### Purpose

The purpose of this SOP is to reformat and to migrate the referenced intermediate tables to the Business Plan Tables and to perform QC checks on the migrated tables.

### Resource Requirements

Migration to Business Plan Tables involves two people: (1) a database management expert from the DAT to populate and update the tables and perform QC, and (2) a supervisor, typically the DAT Team Leader (TL), who, as required, provides guidance and decision-making. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Database Management and QC | 1 | 8.0 |
| DAT TL | Supervisor | 1 | AR06/12/2019 |

### Procedure

The procedure to reformat and to migrate all intermediate tables to the Business Plan Tables and to perform QC on the migrated tables is comprised of two tasks:

- reformatting and migrating intermediate tables to Business Plan Tables, and

- performing QC of table migration.

1- Reformatting and Migrating Intermediate Tables to Business Plan Tables – performed by database engineer

Summary: This task reformats and migrates all intermediate tables to Business Plan Tables.

- Discuss possible changes to the Business Plan Tables with TL. Changes could be related to the data structure (new fields added, existing field names changed) or the business plan itself (segmentation changed).

  - If changes have occurred from the previous year, modify the packages accordingly before proceeding to Step 2.

  - If no changes have occurred, proceed to Step 2.

- Using Oracle SQL Developer or the link provided in the Engineering Data Warehouse (EDW) program, run BUSINESS_PLAN_PKG package. The package will create the BUSINESS_PLAN_1MILE table using condition data from the intermediate tables. To use the EDW program, click “Analysis”, then “Business Plan Update”, then “Business Plan 1Mile.”

- Using Oracle SQL Developer or the link provided in the EDW program, run BUSINESS_PLAN_PKG_100MMI package. The package will create the BUSINESS_PLAN_100MMI table using condition data from the intermediate tables. To use the EDW program, click “Analysis”, then “Business Plan Update”, then “Business Plan 100mmi.”

- Using Oracle SQL Developer or the link provided in the EDW program, run BUSINESS_PLAN_LMY_PKG package in the Production Database (note: this is an exception as all other Oracle SQL packages are run in development, not production database). This package will create the BUSINESS_PLAN_LMY table, the BUSINESS_PLAN_CONHIST table, and the BUSINESS_PLAN_TREATMENTS table. To use the EDW program, click “Analysis”, then “Business Plan Update”, then “Business Plan LMY.”

- Using Oracle SQL Developer or the link provided in the EDW program, run BUSINESS_PLAN_TREND_PKG package. The package will create the BUSINESS_PLAN_TREND table. The trend table is produced by merging the BUSINESS_PLAN_LMY and BUSINESS_PLAN_ 100MMI tables. To use the EDW program, click “Analysis”, then “Business Plan Update”, then “Business Plan Trend.”

2- Performing QC of Table Migration – performed by database engineer

Summary: This task performs QC checks of the migrated tables to identify missing or unreasonable data.

- Perform QC of BUSINESS_PLAN_1MILE table and BUSINESS_PLAN_100MMI table. Compare each table with the same table from the previous year and investigate data that meet one of the following criteria:

  - Difference in rating category percentage of more than 2 points.

  - Condition element total average greater than 1%.

  - Total lane mileage difference of more than 50 miles.

Using SQL Developer, investigate all instances where one or more of the above criteria are met. Look for missing data and double check that issues are not the result of data processing. Check if year-to-year differences could be caused by other external factors, such as increased or decreased construction funding. Discuss with ADC and decide if reprocessing must take place or if the packages should be rerun before continuing.

- Perform QC of BUSINESS_PLAN_LMY table (intermediate tables Treatments and Con history are not checked). Compare total lane miles of treated sections with the same table from the previous year.

  - If there is an unreasonable difference (no specific tolerance since differences are expected, but only within reason), investigate and resolve data processing issues and rerun package.

  - Otherwise, proceed to Step 8.

- Perform QC of BUSINESS_PLAN_TREND table. Compare total lane mileage of treated sections in table with the total lane mileage of treated sections in the LMY table. The total lane mileage should be exactly the same.

  - If there is a difference between lane mileage totals, investigate difference, debug code if necessary, and rerun package.

  - Otherwise, proceed to Step 9.

- Notify TL that the Business Plan Tables are ready for QA.

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and DWT with DAT. |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Friction Management

## Introduction

This section describes the set of standard operating procedures (SOP) to document a systematic approach to addressing pavement friction requirements across the SHA pavement network. These procedures are conducted mainly by staff from the Data Analysis Team (DAT) to determine site and friction demand categories and corresponding threshold pavement surface friction levels (investigatory and intervention levels), to identify pavement locations that have inadequate friction levels and a higher possibility of wet surface accidents throughout the state. This section is divided into seven SOPs: development/review of friction site and demand categories, network definition and segmentation, site and demand category analysis, data aggregation, section prioritization (under development), report generation, and future improvements to the friction management program. Figure 10 shows the steps of Pavement Friction Management (PFM) program at SHA for generating annual friction management reports.

Figure 10 shows the steps of Pavement Friction Management (PFM) program at SHA.

Figure 10: SHA Pavement Friction Management (PFM) program Flowchart

## Development/Review of Friction Site and Demand Categories

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to identify the road features and attributes that play a role in occurrence of wet accidents and to quantify their effect on the road sections’ friction demand and investigatory and intervention friction levels. Investigatory and intervention levels are defined, respectively, as levels that prompt the need for a detailed site investigation or the application of a friction restoration treatment. The exact nature of the relationship between pavement friction and wet crashes is site specific, as it is defined not only by pavement friction but by many other factors such as highway alignment, highway features/environment, and highway traffic characteristics. Thus, pavement friction and wet crashes relationships must be developed for the site categories that are typically present in the pavement network.

This section describes the steps in development of site and friction demand category criteria that are used to classify network sections based on the effect of various factors on section’s friction demand. The goal of this procedure is to manage network friction more efficiently by determining the appropriate level of pavement surface friction for pavement sections within the highway network, based on each section’s friction demand. This procedure is intended to ensure the provision of adequate friction levels for a variety of roadway (intersections, approaches to traffic signals, tight curves) and traffic conditions. Pavement sections with measured friction values at or below an assigned investigatory level are subject to a detailed site investigation to determine the need for warning or remedial action, such as erecting warning signs, performing more frequent testing and analysis of friction data and crash data, or applying a short-term restoration treatment. For pavement sections with friction values at or below the intervention level, remedial action may consist of either immediately applying a restoration treatment or programming a treatment into the maintenance or construction work plan and erecting temporary warning signs at the site of interest.

Currently, the site categories presented in the following table are being used for assessing the friction adequacy of each section. However, this table might be revised in the future as site category criteria are assessed every year following the steps provided in this SOP.

| Site Category | Site Description | Investigatory Skid Number (SN) | Intervention Skid Number (SN) | Demand Category |
| --- | --- | --- | --- | --- |
| 1 | - 0.1 mile within approach to Railroad Crossings, Roundabouts, Traffic Lights, Pedestrian Crossings, Stop and Give Way Controlled Intersections, and the prevailing speed is ≥ 55 mph, OR - Curves with radius ≤ 750 ft, all speeds, OR - Downhill gradients > 10% and > 0.03 miles long, all speeds, OR - Freeway/highway off-ramps, all speeds. | 55 | 45 | High |
| 2 | - 0.1 mile within approach to Railroad Crossings, Roundabouts, Traffic Lights, Pedestrian Crossings, Stop and Give Way Controlled Intersections, and the prevailing speed is ≥ 35 mph and < 55 mph, OR - 0.1 mile within approach to all other intersections ≥ 35 mph, OR - Downhill gradients 5 to 10% and > 0.03 miles long and speed limit is ≥ 35 mph, OR - Undivided Highways without other geometric constraints which influence frictional demand, the traveling speed is at least 55 mph and average rutting > ¼”, OR - Curves with radius > 750 ft and ≤ 1,500 ft and speed limit ≥ 35 mph. | 45 | 35 | Medium |
| 3 | All other situations. | 40 | 30 | Low |

### Frequency

Development (first year) and review of friction site and demand categories is performed every year, before the end of the data collection season.

### Purpose

The purpose of this SOP is to identify the effect of factors that contribute to wet weather accidents and to develop and/or review criteria for using the section attributes to identify site and demand categories and investigatory and intervention friction levels for each category. The goal is to manage network friction more efficiently by determining the appropriate level of pavement friction for pavement sections within the highway network, based on each section’s friction demand.

### Resource Requirements

Development (first year) and review of friction site and demand categories involves two persons: a DAT staff member knowledgeable of friction performance of pavements, available road and pavement attribute data, and Oracle data tables and DAT team leader.

The estimated effort level in the table below represents the total time, in man-hours, to complete the development and review of friction demand categories. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 40.0 (first year) |
| DAT Staff | Analyst | 1 | 16.0 (following years) |
| DAT TL | Supervisor | 1 | 2 |

### Procedure

The procedure required to perform the development/update of friction site and demand categories process is comprised of the following three components:

- Identifying influential parameters and their data availability,

- defining site and demand category criteria based on the identified parameters,

- determining the investigatory and intervention friction levels for each site category.

1- Identifying Influential Parameters with Available Reliable Data – performed by DAT staff

Summary: Under this task, all the road features and pavement attributes that can affect the occurrence of wet accidents are listed and those with available reliable data are identified.

- Prepare a list of factors, road features, and pavement attributes that can affect the occurrence of wet accidents using credible pavement friction and safety documents.

- Investigate availability and reliability of data identified in step 1.

  - Identify data that are available in data tables and have established quality control procedures.

  - Identify data that are available in data tables or can be obtained internally by means of modifying the data processing procedures but need to be validated before they can be used and plan to establish data validation procedures.

  - Identify data that cannot be obtained through internal data collection and processing procedures but can be obtained from reliable outside sources and plan to acquire the data in a timely manner.

- Prepare a list of available factors, road features, and pavement attributes and their data sources. The following table presents the list of factors and their associated data source currently used in friction management program:

| Attribute | Source |
| --- | --- |
| ROAD_NAME | HMIS_MASTER_ALL_YEARS |
| DISTRICT | HMIS_MASTER_ALL_YEARS, EDW_BASE_1 |
| COUNTY | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| ROUTE | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| DIRECTION | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| BEGINROWPATH | CURVE_H_ALL_YRS |
| ENDROWPATH | CURVE_H_ALL_YRS |
| UNIQUERUN | CURVE_H_ALL_YRS |
| CURVE_YEAR | CURVE_H_ALL_YRS |
| CLASS | CURVE_H_ALL_YRS |
| GLOBAL_ROUTE_ID | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| SUB_ROUTE_ID | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| ROUTEID | CURVE_H_ALL_YRS |
| INV_BMP | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| INV_EMP | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| BEGINCHAINAGE | CURVE_H_ALL_YRS, EDW_BASE_1 |
| ENDCHAINAGE | CURVE_H_ALL_YRS, EDW_BASE_1 |
| BEGINDISTANCESTAMP | CURVE_H_ALL_YRS, EDW_BASE_1 |
| ENDDISTANCESTAMP | CURVE_H_ALL_YRS, EDW_BASE_1 |
| DIR_BMP | CURVE_H_ALL_YRS, EDW_BASE_1 |
| DIR_EMP | CURVE_H_ALL_YRS, EDW_BASE_1 |
| BEGIN_LAT | \\shahqhisdfs1\GIS_DATA\TRANSPORTATION\MILE_POINTS\CURRENT\1000TH\SHA_MILEPOINTS_1000TH_MD.gdb |
| BEGIN_LONG | \\shahqhisdfs1\GIS_DATA\TRANSPORTATION\MILE_POINTS\CURRENT\1000TH\SHA_MILEPOINTS_1000TH_MD.gdb |
| END_LAT | \\shahqhisdfs1\GIS_DATA\TRANSPORTATION\MILE_POINTS\CURRENT\1000TH\SHA_MILEPOINTS_1000TH_MD.gdb |
| END_LONG | \\shahqhisdfs1\GIS_DATA\TRANSPORTATION\MILE_POINTS\CURRENT\1000TH\SHA_MILEPOINTS_1000TH_MD.gdb |
| LENGTH | HMIS_MASTER_ALL_YEARS, CURVE_H_ALL_YRS, EDW_BASE_1 |
| BEGINELEVATION | CURVE_H_ALL_YRS |
| ENDELEVATION | CURVE_H_ALL_YRS |
| TYPE | CURVE_H_ALL_YRS |
| RADIUS | CURVE_H_ALL_YRS |
| ARCANGLE | CURVE_H_ALL_YRS |
| DEFLECTIONANGLE | CURVE_H_ALL_YRS |
| CURVATUREDEGREE | CURVE_H_ALL_YRS |
| BEGINHEAD | CURVE_H_ALL_YRS |
| ENDHEAD | CURVE_H_ALL_YRS |
| MEANERR | CURVE_H_ALL_YRS |
| MAXERR | CURVE_H_ALL_YRS |
| BEGINERR | CURVE_H_ALL_YRS |
| ENDERR | CURVE_H_ALL_YRS |
| BEGINX | CURVE_H_ALL_YRS |
| BEGINY | CURVE_H_ALL_YRS |
| ENDX | CURVE_H_ALL_YRS |
| ENDY | CURVE_H_ALL_YRS |
| CTRX | CURVE_H_ALL_YRS |
| CTRY | CURVE_H_ALL_YRS |
| FEATURETYPES | CURVE_H_ALL_YRS |
| MAXISUPERELEVATION | CURVE_H_ALL_YRS |
| ANGLEPOINT | CURVE_H_ALL_YRS |
| STARTSUPERELEVATION | CURVE_H_ALL_YRS |
| MIDDLESUPERELEVATION | CURVE_H_ALL_YRS |
| ENDSUPERELEVATION | CURVE_H_ALL_YRS |
| AVERAGESUPERELEVATION | CURVE_H_ALL_YRS |
| AVG_LANEWIDTH_FEET | RUT_DETAILED |
| AVG_PONDING_AREA | RUT_DETAILED |
| AVG_RUT_DEPTH | RUT_DETAILED |
| AVG_CROSSFALL_PERCENT | RUT_DETAILED |
| MIN_CROSSFALL_PERCENT | RUT_DETAILED |
| MAX_CROSSFALL_PERCENT | RUT_DETAILED |
| FIRST_CROSSFALLINPERCENT | EDW_BASE_1 |
| LAST_CROSSFALLINPERCENT | EDW_BASE_1 |
| FN_CURRENT | FRICTION |
| FN_MINUS1 | FRICTION |
| FN_MINUS2 | FRICTION |
| FN_MINUS3 | FRICTION |
| LAST_TREATMENT_YEAR | BUSINESS_PLAN_LMY & LU_TREATMENT |
| FN_NUMBER | FRICTION, BUSINESS_PLAN_LMY, & LU_TREATMENT |
| SC_DENSITY | CRACKING_DETAILED |
| FC_DENSITY | CRACKING_DETAILED |
| AVG_IRI | EDW18_BASE_4 |
| SURFACE_AGE | BUSINESS_PLAN_LMY & LU_TREATMENT |
| SURFACE_MATERIAL | BUSINESS_PLAN_TREATMENTS & LU_TREATMENT |
| SURFACE_TYPE | EDW18_BASE_4 and CONHIST_SURF_TYPE |
| AVG_GRADE | FINAL_CONDITION_DETAIL |
| MIN_GRADE | FINAL_CONDITION_DETAIL |
| MAX_GRADE | FINAL_CONDITION_DETAIL |
| MPD_WP_AVG | FRICTION |
| FUNC_CL | SECTION_TABLE |
| SPEED_LIMIT | SECTION_TABLE, HMIS_MASTER_ALL_YEARS |
| AADT | SECTION_TABLE |
| ADT | SECTION_TABLE |
| TRUCK_PERCENTAGE | Traffic Data (SHA’s Traffic Engineering Division)1 |
| MEDIAN_TYPE | SECTION_TABLE |
| WET_CRASH_RATE | Obtained from Office of Traffic and Safety annually |
| 500’_FROM_CROSS | HMIS_MASTER_ALL_YEARS, EDW18_BASE_4, OOT DATABASE |
| SITE CATEGORY | SITE_CATEGORY_TABLE |
| INVESTIGATORY_SN | SITE_CATEGORY_TABLE |
| INTERVENTION_SN | SITE_CATEGORY_TABLE |
| DEMAND_CATEGORY | SITE_CATEGORY_TABLE |
| SN_RSL | BUSINESS_PLAN_100MMI |
| OVERALL_RSL | BUSINESS_PLAN_100MMI |
| SN_DOMINANT | BUSINESS_PLAN_100MMI |

1: Source: https://www.roads.maryland.gov/Index.aspx?PageId=251

2- Defining Site and Demand Categories Based on the Identified Parameters – performed by DAT staff

Summary: Under this task, the effect of identified parameters in step 1 on required friction level of the sections for safe operation of all vehicles will be investigated, and site and demand category criteria (mentioned in Section 7.01.01) will be developed/reviewed accordingly.

- Use the most recent friction guidance documents published or recommended by transportation agencies and organizations to identify and establish relationships between the identified parameters and required friction levels. Some of the transportation agencies and organizations that have publications related to pavement friction are:

  - Federal Highway Administration (FHWA) - Office of Safety, Technical Advisories on skid crash reduction, pavement friction courses, and pavement texturing.

  - National Cooperative Highway Research Program (NCHRP) - Syntheses and Reports covering, among other things, safety, friction testing, and surface drainage.

  - American Association of State Highway and Transportation Officials (AASHTO) - Guides, Manuals, and Guide Specifications for highway geometric design, construction, maintenance, and pavement management.

    - State Departments of Transportation (DOT) - Friction management guides and plans.

    - Pavement industry groups - Bulletins and Manuals on surface mixture selection and texturing.

    - International Agencies (e.g., United Kingdom, Australia, Japan) - Various guides, manuals, and reports on friction testing, design, and safety management.

- Update site and demand category criteria based on the identified relationships using the best available guides and practices.

  - Identify attributes from step 1 (highway alignment, highway features/environment, and highway traffic characteristics factors) that are available in SHA data tables and have established quality control procedures.

  - Assess the effect of available attributes on friction demand of network sections using the documents published by organizations listed in step 1 and develop/modify friction site categories accordingly.

  - Check the site category criteria against criteria developed by other agencies with successful friction management programs and investigate any major discrepancy.

  - Correlate the latest wet-weather crash data with the road features and pavement attributes data to validate and quantify the effect of selected attributes and modify the demand category criteria if needed.

3- Determining the Investigatory and Intervention Friction Levels for Each Site Category – performed by DAT staff

Summary: Under this task, the two distinct threshold levels, investigatory and intervention friction levels, are defined/reviewed for assessment of friction adequacy of pavement sections within each site category.

- Assess the adequacy of current investigatory and intervention levels by using historical pavement friction data and crash data.

  - Plot friction versus pavement age and friction versus wet crash rates for each friction site category as illustrated in the figure below.

  - Identify investigatory level as the point corresponding to a large change in friction loss rate.

  - Set the friction number corresponding to a significant increase in wet crashes as intervention level.

  - Compare the existing and new investigatory and intervention levels and update the thresholds if necessary.

## Network Definition and Segmentation

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to define the PFM program network and then divide the network into segments based on factors that influence friction demand. In general, an array of factors comprising the following four broad categories affect the pavement friction demand: highway alignment, highway features/environment, highway traffic characteristics, and driver/vehicle characteristics. The last category, driver/vehicle characteristics, is difficult to assess in terms of friction demand but factors such as location, road geometrics, pavement type, construction history, road and median type, traffic level, road features (intersections, crosswalks, …), and so forth can be assessed. This section describes network segmentation procedure that is vital to preparation of annual friction management reports.

### Frequency

Network definition and segmentation is performed annually, after friction site and demand categories and network data tables are finalized. Additionally, selected attribute data are deleted and re-imported whenever changes to the source tables occur. Network definition and segmentation should be repeated every time an updated inventory is provided.

### Purpose

The purpose of this SOP is to define pavement sections with similar friction demand levels within the network using factors known to affect pavement friction demand.

### Resource Requirements

The network definition and segmentation involve three persons: a DAT staff member knowledgeable of pavement friction performance and pavement management data tables, a DAT staff member knowledgeable of Oracle and source data tables, and and a DAT team leader. The estimated effort level in the table below represents the total time, in man-hours, to complete the network segmentation and data aggregation process. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 4.0 |
| DAT Staff | Database Management | 1 | 24.0 |
| DAT TL | Supervisor | 1 | 2 |

### Procedure

The procedure required to perform the network definition/segmentation process is comprised of the following four components:

- Updating the network,

- Identifying segmentation factors,

- develop/review segmentation logic, and

- develop/update segmentation SQL packages based on segmentation logic.

1- Updating the Network – performed by DAT staff

Summary: Under this task, the network used for the PFM program will be defined/updated to match the current network.

- Use the most recent Highway Location Reference (HLR) table as main source to define the network.

- Perform manual quality control review of the inventory to identify and address missing data or wrong data. This review will include investigating the completeness of network and the accuracy and consistency of data such as direction, road name, and so on.

- Update the PFM network where needed to embrace recent changes in the network.

2- Identifying Segmentation Factors – performed by DAT staff

Summary: Under this task, factors that influence friction demand are identified.

- Identify the factors that affect friction demand and are accessible within the pavement management databases.

- Identify the best source table or tables with the most reliable and updated data for each factor.

3- Develop/Review Segmentation Logic – performed by DAT staff and Team Leader

Summary: Under this task, the network is divided into sections with similar friction demands along each section using a segmentation logic that takes the factors identified in step 1 into account (demand category table in section 7.01.01). Segmentation logic will be reviewed and, if necessary, updated every year. Any change in the segmentation logic should be reviewed and confirmed by a DAT Team Leader (TL).

- Apply Category 1 criteria as per the demand category table in section 7.01.01 of this guide. Section length should be at least 0.1 mile for all category 1 sections.

- Apply site category extension criteria as follows:

- Site Category Extension Criteria:

  - If any section with a site category is less than 0.1 mile,

    - For stop events such as traffic signal, stop sign, and so on, extend the section backward starting from end milepoint (EMP),

    - For qualifying curves and downhill gradients, compute the mid-point (mid_mp) of the section, mid_mp = (BMP+EMP)/2 and extend on both sides of the mid-point to achieve 0.1 mile.

  - If encountering a section with equal or higher site category (lower number represents higher site category), terminate BMP and/or EMP at the edge of that higher site category section and proceed to the next steps (consolidation and merge).

  - If Step 2 does not apply, break the mile point at 0.1 mile boundaries and apply the site category to the created 0.1 mile long section (0.1 mile approach to the stop event or between mid_mp - 0.05 and mid_mp + 0.05 in qualifying curves and downhill gradients).

  - Go back to Step 1 and repeat until no more sections less than 0.1 mile encountered.

- Apply site category consolidation criteria as follows:

- Site Category Consolidation Criteria:

- Consolidate adjacent sections with the same site category into one section.

- Apply site category merge criteria as follows:

- Site Category Merge Criteria:

  - Check the gap between any two sections with a site category. If the gap length is less than 0.1 mile:

    - If the gap is between two sections with similar site categories apply the site category to the section/sections in the gap.

    - If the gap is between two sections with different site categories apply the higher of the two site category numbers (e.g. between site categories 1 and 3, assign 3 to the section/sections in the gap)

  - If section is <0.1 mile, and the section is the beginning or ending section of a route, then assign the Site Category from the adjacent section to the begin/end section.

- At this point, the network is segmented as Category 1 or “Not Categorized”. Check whether all Category 1 segments are greater or equal to 0.1 mile in length and proceed to steps 6-9 to determine site category 2 sections.

- Apply Category 2 criteria based on the demand category table in section 7.01.01 of this guide for the “Not Categorized” sections of the network without any section length limit consideration.

- Apply site category extension criteria (step 2).

- Apply site category consolidation criteria (step 3).

- Apply site category merge criteria (step 4).

- At this point, the “Not Categorized” network is further segmented to Category 2 or “Not Categorized” sections and All Category 1 and Category 2 segments are at least 0.1 mile long. Proceed with Steps 11-14 using the “Not Categorized” sections of the network to assign site category 3.

- Apply Category 3 criteria based on the demand category table in section 7.01.01 of this guide for the “Not Categorized” sections of the network without any section length limit consideration.

- Apply site category extension criteria (step 2).

- Apply site category consolidation criteria (step 3).

- Apply site category merge criteria (step 4).

- At this point, the entire network should be categorized, and every segment should be at least 0.1 mile long.

- No given section should be downgraded from its assigned site category at any point throughout the segmentation steps. Perform a check on the entire segment to verify the network has been categorized correctly.

- Add GPS lat and long for each of the segmentation mile markers.

4- Develop/update network segmentation SQL packages – performed by DAT staff

Summary: Under this task, the site and demand category criteria and segmentation logics are translated into SQL procedures and packages. Segmentation SQL packages will be reviewed and, if necessary, updated every year.

- Review the segmentation logic, and

- Develop/update SQL procedures and packages for segmenting the network based on the segmentation logic.

All the SQL procedures and packages currently used for network segmentation are included in Appendix 11.14.

## Section Site and Demand Category Analysis

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to check the performance of the developed network segmentation SQL packages and to create a segmented network table with friction site and demand category and investigatory and intervention level information for each section in accordance with the established criteria and logics.

### Frequency

Demand category analysis is performed annually, after friction demand categories are reviewed, network data tables are finalized, network segmentation logic is established, and network segmentation SQL packages are developed or updated or confirmed. Additionally, if any relevant attribute data is re-imported due to a change to the source tables, or if SQL packages are updated due to a change in the segmentation logic, section site and demand category analysis should be repeated.

### Purpose

The purpose of this SOP is to segment the network using the established criteria and logic and to identify the friction site and demand category and investigatory and intervention friction levels for each section of the network.

### Resource Requirements

The section friction site and demand category analysis involves two persons: a DAT staff member knowledgeable of PFM program segmentation demand category criteria and segmentation logics and a DAT staff member knowledgeable of Oracle SQL Developer. The estimated effort level in the table below represents the total time, in man-hours, to complete the site and demand category analysis. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 12.0 |
| DAT Staff | Database Management | 1 | 4.0 |

### Procedure

The procedure required to perform the site and demand category analysis is comprised of the following two components:

- check the performance of the developed SQL packages.

- execute the segmentation and demand category SQL packages.

1- Check the performance of the developed SQL packages – performed by DAT staff

Summary: Under this task, the performance of the developed segmentation SQL packages will be examined. The performance/accuracy of the package needs to be examined after each package update.

- Select several sample routes with a variety of features and event affecting the friction demand.

- Segment the table manually and using the SQL package and compare the results.

- If manual and automated (SQL package) segmentation results match for all the sample routes then proceed to the next step (package execution), otherwise report the discrepancy to the DAT staff for troubleshooting.

2- Execute the segmentation and demand category SQL package – performed by DAT staff

Summary: Under this task, the network is segmented and site and demand category for each section of the network is determined by executing the segmentation and demand category SQL packages.

- Execute the segmentation and demand category SQL packages annually for the network.

- Run the segmentation and demand category SQL packages to segment the network and populate site category, demand category, and investigatory and intervention friction levels for each section of the network. The packages need to be executed at least once every year to account for inventory changes or construction related geometry changes. Moreover, the packages should be executed after every package update.

## Data Aggregation

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to aggregate data needed for performing friction analysis for each section of the network and prioritization of sections with friction deficiency. This section describes data aggregation logics and procedures to prepare the annual friction management table for friction analysis.

### Frequency

Network data aggregation for friction management table is performed annually, after friction demand categories are reviewed, network data tables are finalized, network segmentation logic is established, and network segmentation SQL packages are developed, examined, and executed. Additionally, selected attribute data are deleted and re-imported whenever changes to the source tables or segmentation table occur.

### Purpose

The purpose of this SOP is to prepare the friction management table for performing friction analysis for each section of the network and section prioritization.

### Resource Requirements

The data aggregation involves two persons: a DAT staff knowledgeable of friction table attributes and friction demand categories and a DAT staff member knowledgeable of Oracle and source data tables. The estimated effort level in the table below represents the total time, in man-hours, to complete the data aggregation process. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 3.0 |
| DAT Staff | Database Management | 1 | 6.0 |

### Procedure

The procedure required to perform the data aggregation process is comprised of the following two components:

- identify data aggregation method for each attribute, and

- perform data aggregation.

1- Identify Data Aggregation Method – performed by DAT staff

Summary: Under this task, the proper data aggregation method for each data field is identified.

- Identify/review data fields in the PFM table and update, add, or remove data fields if necessary.

- Identify the use and purpose of each data field in the pavement friction program.

- Based on the identified purpose of each data field, determine the right aggregation method/methods (minimum, maximum, average, dominant, …).

The following table presents some of the data fields in the pavement friction management program table with their purpose and identified aggregation method:

| Attribute | Effect/Purpose | Aggregation Method |
| --- | --- | --- |
| CURVE_YEAR | NA | Latest Year Available |
| TYPE | Curves Have Higher Demand | Dominant |
| RADIUS | Increases Demand | Maximum |
| CLASS | A: smooth curve (R>506 m), F: sharp curve (R<61 m) | Minimum |
| FEATURETYPES | Curve Type | Dominant |
| MAXISUPERELEVATION | Can Decrease Demand | Maximum |
| MINISUPERELEVATION | Can Decrease Demand | Minimum |
| STARTSUPERELEVATION | Can Decrease Demand | Begin |
| MIDDLESUPERELEVATION | Can Decrease Demand | Middle |
| ENDSUPERELEVATION | Can Decrease Demand | End |
| AVERAGESUPERELEVATION | Can Decrease Demand | Weighted Average |
| AVG_LANEWIDTH_FEET | NA | Weighted Average |
| PONDING_AREA | Increases Demand | Maximum |
| AVG_PONDING_AREA | Increases Demand | Weighted Average |
| RUT_DEPTH | Increases Demand | Maximum |
| AVG_RUT_DEPTH | Increases Demand | Weighted Average |
| AVG_CROSSFALL_PERCENT | Can Decrease Demand | Weighted Average |
| MIN_CROSSFALL_PERCENT | Can Decrease Demand | Minimum |
| MAX_CROSSFALL_PERCENT | Can Decrease Demand | Maximum |
| FIRST_CROSSFALLINPERCENT | Can Decrease Demand | First |
| LAST_CROSSFALLINPERCENT | Can Decrease Demand | Last |
| SN_CURRENT | SN_NUMBER | Stepped average |
| SN_MINUS1 | SN_NUMBER | NA |
| SN_MINUS2 | SN_NUMBER | NA |
| SN_MINUS3 | SN_NUMBER | NA |
| LAST_TREATMENT_YEAR | Years Used for Determining | Last treatment year |
| SN_NUMBER | Prioritization and Treatment Decision | Minimum |
| SC_DENSITY | Treatment Decision | Weighted Average |
| FC_DENSITY | Treatment Decision | Weighted Average |
| AVG_IRI | Treatment Decision | Weighted Average |
| SURFACE_AGE | Treatment Decision | Dominant |
| SURFACE_MATERIAL | Treatment Decision | Dominant |
| AVG_GRADE | Downhill gradient increases demand | Weighted Average |
| MIN_GRADE | Downhill gradient increases demand | Minimum |
| MAX_GRADE | Downhill gradient increases demand | Maximum |
| MPD_WP_AVG | Higher Mean Profile Depth Accelerates Surface Water Drainage | Weighted Average |
| FUNC_CL | Treatment Decision | Dominant |
| AADT | Higher Traffic Increases Demand | Maximum |
| SPEED | Higher Speed Limits Increase Demand | Maximum |
| ADT | Higher Traffic Increases Demand | Maximum |
| Truck Percentage | Higher Truck Percentage Increases Demand | Maximum |
| MEDIAN_TYPE | Undivided Highways have Higher Demand | Dominant |
| Wet Crash Rate | Demand Category Verification | Maximum |
| 528’_FROM_CROSS | Road Crossing Features Increase Demand | 0 or 1 |
| LATITUDE | Mapping | Closest |
| LONGITUDE | Mapping | Closest |

2- Perform Data Aggregation – performed by DAT staff

Summary: Under this task, data aggregation for pavement friction management program table data fields is performed.

- Compile SQL queries or SQL packages for aggregating data into the segmented PFM table using the determined data aggregation methods for each field.

- Execute the queries or packages to populate data into the segmented PFM table.

The SQL package is currently under development by the DAT staff.

## Section Prioritization

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to prioritize sections with friction deficiency based on their friction demand, road class, traffic characteristics, and friction level. The current version of PFM only uses site category and friction deficiency for prioritization, but a more comprehensive and robust prioritization procedure will be implemented into the PFM in the future.

### Frequency

Section prioritization is performed annually, once section site and demand category analysis, and data aggregation are completed. Additionally, if any attribute data is re-imported or network segmentation is updated due to a change to the source tables or a change in the inventory, section prioritization should be repeated.

### Purpose

The purpose of this SOP is to give higher priority to the sections with higher potential for wet weather accidents within each site category.

### Resource Requirements

The section prioritization involves one person: a DAT staff member knowledgeable of pavement friction performance and PFM. The estimated effort level in the table below represents the total time, in man-hours, to complete the demand category analysis. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 4.0 |

### Procedure

The procedure required to prioritize the sections is comprised of the following steps:

- Give the sections priorities based on site category and friction deficiency,

- Compare the results with the most recent wet crash data, and

- Update the skid data collection list if needed.

1- Give the Sections Priorities Based on Site Category and Friction Deficiency – performed by DAT staff

Summary: Under this task, sections are prioritized based on their site category and friction deficiency.

- Sort the sections based on site category from smallest to largest. Site category 1 has the highest and site category 3 has the lowest friction demand.

- Within each site category, order the sections based on friction deficiency by simply sorting them based on friction value from smallest to largest.

- If multiple sections have similar friction values, sort them based on traffic volume and speed limit from largest to smallest.

2- Compare the results with the most recent wet crash data – performed by DAT staff

Summary: Under this task, the most recent wet crash data are obtained and sections with high wet accident rates are further investigated.

- Obtain the most recent state wet crash list in cooperation with Office of Traffic and Safety (OOTS).

- Identify sections with high numbers of wet crashes.

- Investigate the identified sections and update their priority if needed.

3- Update the skid data collection list – performed by DAT staff

Summary: Under this task, the skid collection list is updated using the updated site categories.

- Populate the inventory mileage table based on updated site categories.

- Work with Field Explorations Division (FED) staff to determine available skid data collection resources and skid data collection limitations.

- Optimize the “% of miles collected per year” and “data collection interval” for each site category with a high emphasis on collecting more mileage and data points from site categories with a higher demand. The goal would be collecting 100% of roads in site category 1 using minimum possible data collection interval and adjusting the “% of miles collected per year” and “data collection interval” for the rest of the network (site categories 2 and 3) based on available data collection resources and feasible skid data points.

The following table is an example skid data collection list. Note that the actual annual list can change based on feedback from FED and data collection limitations.

| Site Category | Investigatory SN | Intervention SN | Demand Category | Total Mileage - SHA Only | >=1 Mile - Mileage | >=1 Mile - % of miles collected per year | >=1 Mile - Collection Interval (mile) | >=1 Mile - # of tests collected | <1 Mile - Mileage | <1 Mile - % of miles collected per year | <1 Mile - Collection Interval (mile) | <1 Mile - # of tests collected | # Miles Tested | # of tests collected |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 55 | 45 | High | 1,730 | 1,428 | 100% | 0.05 | 28,560 | 302 | 100% | 0.05 | 6,040 | 1,730 | 34,600 |
| 2 | 45 | 35 | Medium | 3,092 | 2,847 | 50% | 0.20 | 7,118 | 245 | 50% | 0.20 | 613 | 1,546 | 7,730 |
| 3 | 40 | 30 | Low | 6,192 | 5,870 | 33% | 0.50 | 3,909 | 322 | 33% | 0.50 | 536 | 2,062 | 4,446 |
| Total |  |  |  | 11,014 | 10,145 |  |  |  |  |  |  |  | 5,338 | 46,776 |

Overall statistics:

- Total SHA-only mileage: 11,014 miles

- Mileage >= 1 mile: 10,145 miles

- Total miles tested: 5,338 miles

- Total tests collected: 46,776 tests

- % Collected (overall): 48%

- Avg collection interval (overall): 0.114 miles

Linear (screen-reader friendly) view

- Site Category 1

  - Investigatory SN: 55

  - Intervention SN: 45

  - Demand Category: High

  - Total Mileage - SHA Only: 1,730

  - Mileage >=1 Mile: 1,428; % collected/year: 100%; Interval: 0.05 mile; # tests: 28,560

  - Mileage <1 Mile: 302; % collected/year: 100%; Interval: 0.05 mile; # tests: 6,040

  - Miles Tested: 1,730; # tests collected: 34,600

- Site Category 2

  - Investigatory SN: 45

  - Intervention SN: 35

  - Demand Category: Medium

  - Total Mileage - SHA Only: 3,092

  - Mileage >=1 Mile: 2,847; % collected/year: 50%; Interval: 0.20 mile; # tests: 7,118

  - Mileage <1 Mile: 245; % collected/year: 50%; Interval: 0.20 mile; # tests: 613

  - Miles Tested: 1,546; # tests collected: 7,730

- Site Category 3

  - Investigatory SN: 40

  - Intervention SN: 30

  - Demand Category: Low

  - Total Mileage - SHA Only: 6,192

  - Mileage >=1 Mile: 5,870; % collected/year: 33%; Interval: 0.50 mile; # tests: 3,909

  - Mileage <1 Mile: 322; % collected/year: 33%; Interval: 0.50 mile; # tests: 536

  - Miles Tested: 2,062; # tests collected: 4,446

CSV (copy/paste)

Site Category,Investigatory SN,Intervention SN,Demand Category,Total Mileage - SHA Only,>=1 Mile - Mileage,>=1 Mile - % collected per year,>=1 Mile - Collection Interval (mile),>=1 Mile - # of tests collected,<1 Mile - Mileage,<1 Mile - % collected per year,<1 Mile - Collection Interval (mile),<1 Mile - # of tests collected,# Miles Tested,# of tests collected
1,55,45,High,1730,1428,100%,0.05,28560,302,100%,0.05,6040,1730,34600
2,45,35,Medium,3092,2847,50%,0.2,7118,245,50%,0.2,613,1546,7730
3,40,30,Low,6192,5870,33%,0.5,3909,322,33%,0.5,536,2062,4446
Total,,, ,11014,10145,,,,,,,,,5338,46776

Footnotes:
1) This table is only a suggested skid data collection approach and can change based on feedback from FED and data collection limitations.
2) For site category descriptions, refer to the site category table in section 7.01.01.

## Generating Reports

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to generate annual reports and maps for the “Pavement Friction Management Plan”. This SOP is performed once the site and demand categories are populated and sections are sorted based on priority.

### Frequency

The generation of skid reports and maps is performed annually, after the end of the data collection season, and once all data have been processed and migrated into the appropriate tables, network is segmented, data is aggregated into the friction management table, and sections are prioritized. The reports may need to be re-generated if any of the source tables or segmentation change.

### Purpose

The purpose of this SOP is to generate friction demand category reports and maps to identify sections with friction values below intervention and investigatory levels in each category demand. These reports are complemented with guidance for selecting techniques and treatments that will provide adequate wet pavement friction or lower the chance of wet crash accidents.

### Resource Requirements

Generating reports and maps involves one person: a DAT staff member familiar with the friction management plan. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the report generation. This time estimate assumes no issues are encountered during the report generation process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 4.0 |

### Procedure

The procedure to generate reports is comprised of the following three tasks:

- generate statewide report,

- generate district and county reports, and

- generate color coded district and couty maps.

1- Generate Statewide Reports – performed by DAT Staff

Summary: Under this task, state-wide reports from the site and demand category analysis will be generated for the whole network.

- Publish state-wide PFM table and friction deficiency table.

  - Open the friction management table

  - List the sections based on site category and priority level.

  - Create another list by filtering out the sections that have friction values above investigatory level.

  - Publish both lists as state-wide pavement friction management (PFM) program table and state-wide sections with friction deficiency.

  - Attach the following statement to the report:

- Design engineers should use the latest Maryland State Highway Administration (SHA) Pavement & Geotechnical Design Guide to fulfill the requirements of the identified sections with friction deficiency. Section 2.08 of the Design Guide contains guide tables for identifying proper treatments based on pavement type, daily traffic, and pavement performance measures.

- Section 2.09 of the Design Guide provides detailed information for each treatment and shows which treatments are intended to improve skid resistance.

- Section 6.06.03.02.03.1 of Design Guide describes the pavement preservation and rehabilitation friction design concepts used by SHA. This section contains guide tables and equations for determining the initial skid numbers and deterioration rates for treatments such as Dense-Graded HMA, Gap-Graded HMA, Micro-Surfacing, High Friction Surface, Surface Abrasion, and Diamond Grinding.”

2- Generate District and County Reports – performed by DAT Staff

Summary: Under this task, district- and county-based reports from the site and demand category analysis will be generated for the whole network.

- Open the friction management table.

- Use the state-wide table to create individual district tables.

  - Filter the state-wide table using the district field.

  - List the sections based on site category and priority level.

  - Create another list by filtering out the sections that have friction values above investigatory level.

  - Publish both lists as district friction management program (PFM) table and district sections with friction deficiency for each district.

  - Attach the following statement to each report:

- Design engineers should use the latest Maryland State Highway Administration (SHA) Pavement & Geotechnical Design Guide to fulfill the requirements of the identified sections with friction deficiency. Section 2.08 of the Design Guide contains guide tables for identifying proper treatments based on pavement type, daily traffic, and pavement performance measures.

- Section 2.09 of the Design Guide provides detailed information for each treatment and shows which treatments are intended to improve skid resistance.

- Section 6.06.03.02.03.1 of Design Guide describes the pavement preservation and rehabilitation friction design concepts used by SHA. This section contains guide tables and equations for determining the initial skid numbers and deterioration rates for treatments such as Dense-Graded HMA, Gap-Graded HMA, Micro-Surfacing, High Friction Surface, Surface Abrasion, and Diamond Grinding.”

- Use the state-wide table to create individual county tables.

  - Filter the state-wide table using the county field.

  - List the sections based on site category and priority level.

  - Create another list by filtering out the sections that have friction values above investigatory level.

  - Publish both lists as county friction management program (PFM) table and county sections with friction deficiency for each district.

  - Attach the following statement to each report:

- “Design engineers should use the latest Maryland State Highway Administration (SHA) Pavement & Geotechnical Design Guide to fulfill the requirements of the identified sections with friction deficiency. Section 2.08 of the Design Guide contains guide tables for identifying proper treatments based on pavement type, daily traffic, and pavement performance measures.

- Section 2.09 of the Design Guide provides detailed information for each treatment and shows which treatments are intended to improve skid resistance.

- Section 6.06.03.02.03.1 of Design Guide describes the pavement preservation and rehabilitation friction design concepts used by SHA. This section contains guide tables and equations for determining the initial skid numbers and deterioration rates for treatments such as Dense-Graded HMA, Gap-Graded HMA, Micro-Surfacing, High Friction Surface, Surface Abrasion, and Diamond Grinding.”

3- generate color coded district and county maps – performed by DAT Staff

Summary: Under this task, district and county reports are used to create color coded skid maps.

- Map the data for each district and county using the latest geospatial skid data tables.

- Use the friction number, investigatory and intervention level of each section to color code the map as follows:

  - Use red if Skid Number is worse than Intervention Level.

  - Use yellow if Skid Number is between Intervention Level and Investigatory Level.

  - Use green if Skid Number is higher than Investigatory Level.

## Future Improvements to the PFM Program

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) to improve the current procedures of the Pavement Friction Management Plan. This SOP is performed annually after the publication of PFM reports or once any new relevant type of data with potential for improvement of program becomes available. Implementation of International Friction Index (IFI) into the plan and development of a more robust prioritization procedure for correction of the hazardous skid-prone locations are two expected improvements that SHA will be undertaking in near future. As more reliable data become available, the PFM procedures may be modified to increase the efficiency of the program.

### Frequency

The improvement to the PFM is performed annually, after the publication of PFM reports or once new relevant type of data become available.

### Purpose

The purpose of this SOP is to use the available resources to make the PFM as efficient as possible.

### Resource Requirements

Improving the PFM involves two persons: a DAT staff member familiar with the friction management plan and DAT team leader. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the report generation. This time estimate assumes no issues are encountered during the report generation process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 8.0 |
| DAT TL | Supervision | 1 | 2.0 |

### Procedure

The procedure to improve the PFM is comprised of the following two tasks:

- Identify procedures that can be improved by modifying the steps, and

- Investigate if new data sources can be used to improve the procedures.

1- Identify Procedures That Can Be Improved by Modifying the Steps – performed by DAT Staff

Summary: Under this task, steps within each procedure are assessed and areas where improvements can be made are identified.

- Assess the steps within each procedure and look for ways to improve the procedure.

- Make a list of suggested improvements and share it with DAT team leader.

- Improve the procedures based on the approved list of suggestions.

2- Investigate if New Data Sources Can Be Used to Improve the Procedures – performed by DAT Staff

Summary: Under this task, availability of new data sources and data fields will be investigated, and procedures will be modified accordingly to allow for incorporation of new resources.

- Assess the availability of new resources such as new data fields.

- Assess the reliability and accuracy of new data.

- Make a list of suggested improvements and share it with DAT team leader.

- Improve the procedures based on the approved list of suggestions.

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2021-Oct-21 | SHA PM Team | Section 7.0.1 - Site Categories were reduced from 5 to 3 and, consequently, Site Category descriptions, Investigatory Skid Numbers, and Intervention Skid Numbers were revised. The Site Category table was updated accordingly. Section 7.02.05 - The procedures were updated to reflect the changes made to the Site Category table. Section 7.05.05 - The procedures, the update of skid data collection list steps and the example skid data collection list table were revised to reflect the changes made to the Site Category table. |
| 2.1.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Updated flowchart image to mermaid chart. Added alt text to all images. Updated skid data collection list table to provide accessible alternative text |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Updated flowcharts. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# HPMS Data Reporting

Click to go to Production of HPMS Full Extent Table

Click to go to Production of HPMS Sample Table

Click to go to Production of HPMS Pavement Metadata file

Click to go to Production of HPMS Submission Comments

Click to go to QA of HPMS Tables

Click to go to Process PM2 Dataset

Click to go to NHS Pavement Condition Dataset

## Introduction

This section describes the set of standard operating procedures (SOP) conducted mainly by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to update, format, and deliver the Highway Performance Monitoring System (HPMS) tables to the DSD. This section is divided into three SOPs: updating of the HPMS Sample table, updating of the HPMS Full-Extent table, and QA of the tables. The first two cover the production and QC of the HPMS deliverables whereas the third SOPs covers the QA performed to the HPMS deliverables before they are submitted to the DSD.

Figure 11 shows a flowchart of the HPMS Data Reporting SOPs.

Figure 11: HPMS Data Reporting SOPs Flowchart

## Production of HPMS Full-Extent Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to produce the Highway Performance Monitoring System (HPMS) Full-Extent Table as required by the Federal Highway Administration (FHWA). This process is performed after the completion of Business Plan Tables (BPT) migration, as outlined in Migrating to Business Plan Tables. The HPMS Full-Extent Table is produced for delivery to the Data Services Division (DSD).

### Frequency

The HPMS Full-Extent Table is produced once per year.

### Purpose

The purpose of this SOP is to produce the annual HPMS Full-Extent Table, which meets FHWA requirements, and once completed to deliver the table to the DSD.

### Resource Requirements

The production of the HPMS Full-Extent Table involves three people: (1) a DAT member who produces the table and performs quality control (QC), (2) the DAT Team Leader (TL), who provides support and performs quality assurance (QA), and (3) the Assistant Division Chief (ADC), who, as requested, performs further QA. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Data Processing and QC | 1 | 50.0 |
| DAT TL | Support and QA | 1 | 4.0 |
| ADC | QA | 1 | AR06/12/2019 |

### Procedure

The procedure required to produce the HPMS Full-Extent Table is comprised of the following five tasks:

- producing HPMS_NETWORK_CONDITION_100MMI Table,

- performing QC of HPMS_NETWORK_CONDITION_100MMI Table,

- creating view and notifying TL of completion,

- performing QA of HPMS_NETWORK_CONDITION_100MMI Table, and

- creating final dataset of HPMS Full-Extent Table and notifying ADC for further QA.

1- Producing HPMS_NETWORK_CONDITION_100MMI Table – performed by DAT member

Summary: Under this task, the four detailed tables (Final Condition Detail Table, Rut Detailed Table, Cracking Detailed Table, and Concrete Detailed Table) developed in accordance with Update Final Condition Detail Table, Update Rut Detailed Table, Update Cracking Detailed Table and Update Concrete Detailed Table are used to adjust the section extents of the pavements in the network to adapt to FHWA HPMS requirements. The output is the creation of an intermediate table named HPMS_NETWORK_CONDITION_ 100MMI that is used to produce the final Full-Extent Table.

- Check if there are any changes in HPMS requirements from the previous year.

  - If there are no changes, proceed to Step 2.

  - If there are changes, modify code accordingly based on the new requirements. Discuss the HPMS requirement changes with respect to the previous year with the Assistant Division Chief (ADC) to confirm the packages will run properly.

- Using the SQL developer, run HPMS_CONDITION_100MMI package. The input will be the four detailed tables (Final Condition Detail Table, Rut Detailed Table, Cracking Detailed Table, and Concrete Detailed Table) developed in accordance with Update Final Condition Detail Table, Update Rut Detailed Table, Update Cracking Detailed Table and Update Concrete Detailed Table and the output will be the intermediate HPMS_NETWORK_CONDITION_100MMI Table.

2- Performing QC of HPMS_NETWORK_CONDITION_100MMI Table – performed by DAT member

Summary: Under this task, checks are conducted to confirm that the intermediate HPMS_NETWORK_CONDITION_100MMI Table was correctly populated based on completeness and then compares the data to last year’s submission for quality purposes.

- Check for completeness.

  - Check number of records missing for each of the data elements. No missing records are allowed. Investigate any missing records and attempt to recover. If recovery is not possible, in some cases it is permitted to carry last year’s value forward.

- Compare to last year’s submission.

  - Check each rating group (% Poor, Fair, Good) for each condition metric. If the percent change is equal to or less than 1 point, do nothing. If the change is greater than 1 point, investigate the possible causes. Determine if the issue is related to data processing, or if it is a legitimate change in distress rating.

  - Check surface types. Compare lane miles for each surface type. If the percent change is equal to or less than 1 point, do nothing. If the change is greater than 1 point, investigate the possible causes. Determine if the issue is related to data processing, or if it is a legitimate change in surface type amount.

3- Creating View and Notifying TL of Completion – performed by DAT TL

Summary: Under this task, gaps in the intermediate HPMS_NETWORK_ CONDITION_100MMI Table are filled with values from last year’s table.

- Check if there are changes regarding the data structure of the view. If there are no changes, do nothing. If there are changes, first modify the HPMS[YY]_ FULL_EXTENT_COND code accordingly; i.e., to reflect the revised data structure. Then, create the view in the production server called HPMS[YY]_FULL_EXTENT_COND. The view will fill in holes in current year’s table using values from the previous year.

- Notify DAT Team Leader that the intermediate HPMS_NETWORK_ CONDITION_100MMI Table is ready for QA and that the view, if necessary, has been created.

4- QA of HPMS_NETWORK_CONDITION_100MMI Table – performed by DAT TL

Summary: Under this task, the updated intermediate HPMS_NETWORK_ CONDITION_100MMI Table resulting from Task 3 is first checked to confirm it has been correctly populated from a completeness viewpoint and then compares the data to table submissions from previous years for quality purposes.

- Check for completeness.

  - Compare total lane mileage from last year’s reported HPMS data to the current year’s report HPMS data (~ +/- 10 miles of last years). Check mileage by pavement type, last rehabilitation year, last construction date, and by other inventory data types.

    - If checks are acceptable, proceed to Step 8.

    - If checks result in differences greater than the defined tolerance, further investigation is required. For example, if aggregate difference in lane miles by county is positive, this may be to duplicated records or added miles, while if negative may be due to routes on which data were not collected. Use SQL Developer to locate where discrepancies are located. If the cause of the discrepancy is valid, accept as-is. As indicated in the HPMS Field Manual, last year’s condition data may be used for a specific missing section (but confirm with HPMS Field Manual that this is allowed for metric in question). Otherwise, report section as missing value. If not valid, ask DAT staff member to fix the issue.

- Check for data sanity (i.e., range, acceptable values):

  - Using SQL Developer, follow HPMS manual guidelines for doing checks (ad hoc, no code exists). For example, check for International Roughness Index (IRI) values less than 30 in/mi and greater than 400 in/mi. Investigate source of the unacceptable value. If the value is a valid measurement with correct processing, submit it as-is and include a note to the Data Services Division (DSD) justifying the value. Otherwise, contact the TL of the department where the issue originated and request they investigate the issue until it is resolved.

- Check for discrepancies in the current year’s HPMS data with respect to the last two years of HPMS data submitted.

Check overall mean values of condition metrics at State level to confirm they are within reasonable values. The mean IRI, cracking percent, rutting percent, and faulting06/12/2019 percent change per year should not be more than +-2%. If values are reasonable, do nothing. If one or more checks result in a condition measurement difference outside the listed tolerance, discuss with ADC before continuing.

- Check for validity of non-condition data elements (data item numbered 54 through 62 in the HPMS Field Manual) by comparing them with the numbers in last year’s HPMS submission.

  - Using Oracle SQL Developer, check that the construction year is consistent with last year’s submittal.

  - Using SQL Developer, compare surface types between this year and last year. If the surface type has changed, check for construction events. If there are discrepancies that cannot be resolved, discuss them with DPT staff before continuing.

5- Creating View of Full-Extent Table and Notifying ADC for further QA – performed by DAT TL

Summary: Under this task, a view that will extract data from another view (pav.conhist.HPMS17_FULL_EXTENT_COND) previously created by DAT staff will be created. This view will contain the finalized Full-Extent Table for QA review by the ADC and then submission to the DSD.

- Check if there are changes regarding the data structure of the view or the HPMS requirements. If there are no changes, do nothing. If there are changes, modify the HPMS_FULL_EXTENT code accordingly. Then, using Oracle SQL Developer, recreate the HPMS_FULL_EXTENT View within the “Pav_READ” user06/12/2019.

- Notify ADC of completion of final data set for final review (see QA of HPMS Tables). Send email with name of the view that will be accessed through “Pav_READ” user.

## Production of HPMS Sample Table

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA’s) Data Analysis Team (DAT) to produce the Highway Performance Monitoring System (HPMS) Sample Table as required by the Federal Highway Administration (FHWA). The table utilizes the Data Services Division’s (DSD) Sample Section table, which is received in Excel format and includes a list of sections for the current year’s Sample Table. This process is performed after the completion of Business Plan Tables (BPT) migration and the production of the HPMS Full Extent table.

### Frequency

The HPMS Sample Table is produced once per year, typically after receiving the updated sample sections list from DSD and after completing the HPMS Full Extent Table.

### Purpose

The purpose of this SOP is to detail the procedure required to process the DSD Sample Section Table dataset and to populate the HPMS Sample Table with data for the current year for submission to the FHWA. The sample table includes pavement condition metrics, surface type information, and construction history data for selected representative sections of Maryland’s highway network.

### Resource Requirements

The production of the HPMS Sample Table involves two primary roles: (1) the DAT Staff who produces the table and performs quality control (QC), and (2) DAT members who provide database management support. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Data Processing and QC | 1 | 40.0 |
| DAT Staff | Database Management Support | 1 | 10.0 |

### Procedure

The procedure to produce the HPMS Sample Table is comprised of five main tasks with specific dependencies that must be satisfied before beginning the process.

#### Dependencies

Before running the HPMS Sample Table updates, ensure the following tables are updated:

- HPMS_NETWORK_CONDITION_100MMI

- CURVE_H_ALL_YRS
- EDW{YEAR}_BASE_100 tables for the most recent year and previous two years

- BUSINESS_PLAN_TREND
- BUSINESS_PLAN_TREATMENTS

- PMBASE_CONHIST

Excel file requirements:

- Latest version of Office 365 with Power Query

- Oracle client installed

- Oracle Client for Microsoft Tools installed

- ASSET_NEW and ASSETDEV environment configured in the Oracle client with database access credentials - Macros enabled in the Excel file

#### Backup and Preparation

- Back up the existing HPMS_SAMPLE table by creating a copy named HPMS_SAMPLE_YYYY, where YYYY represents the most recent submission year:

CREATE TABLE HPMS_SAMPLE_YYYY AS SELECT * FROM HPMS_SAMPLE;

- Update any changes to sample sections received from DSD. Typically, these updates involve new samples being added or changes to the limits of existing samples.

For example, in 2025, the following sample sections were sent to PAGD to be updated in the HPMS_SAMPLE table. Note the sample numbers and use them to get the existing data from the HPMS_SAMPLE table. The sample numbers are in the SAMPLEID column. The Diff columns are the difference between the existing and new data. The sample number is a unique identifier for each sample section.

| ROUTEID | BMP | EMP | SAMPLEID | BMP_EXISTING | EMP_EXISTING | BMP_Diff | EMP_Diff |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 01000CO00068–1—– | 2.06 | 4.038 | 303730000485 | 2.06 | 4.19 | 0 | -0.152 |
| 04000CO00006–1—– | 0.118 | 0.281 | RQKIQAXLELHA | 0.11 | 0.281 | 0.008 | 0 |
| 04000MD00765Q-1—– | 3.782 | 5.25 | IEKKURHJQGAD | 3.23 | 5.25 | 0.552 | 0 |
| 06000CO00142–1—– | 1.843 | 1.89 | GEKMUXZSDXBY | 1.83 | 1.89 | 0.013 | 0 |
| 06000CO00466–1—– | 0.578 | 0.938 | PHLWBBMKIJJD | 0.578 | 0.95 | 0 | -0.012 |
| 06000CO00733–1—– | 1.741 | 1.859 | DJKWLEHDOBXX | 1.74 | 1.859 | 0.001 | 0 |
| 08000CO00494–1—– | 0 | 1.315 | 304940000000 | 0 | 1.57 | 0 | -0.255 |
| 10000CO00134–1—– | 0.317 | 0.93 | OWKSHHFVPODZ | 0.31 | 0.93 | 0.007 | 0 |
| 10000CO00457–1—– | 1.94 | 2.247 | 304570000194 | 1.94 | 2.99 | 0 | -0.743 |
| 10000CO00730–1—– | 1.4 | 1.93 | 307300000140 | 1.4 | 2.03 | 0 | -0.1 |
| 16000CO00228–1—– | 0.636 | 0.808 | DKKIUSBFTLPL | 0.63 | 0.808 | 0.006 | 0 |
| 18000CO00195–1—– | 0.051 | 0.48 | WYKRFDRUDGUL | 0.04 | 0.48 | 0.011 | 0 |
| 21000MD00062–1—– | 0.392 | 0.46 | QWKRHBMMCXCE | 0.39 | 0.46 | 0.002 | 0 |
| 21000MD00063–1—– | 6.321 | 6.368 | GQKWTHDNCFVR | 6.321 | 6.7 | 0 | -0.332 |
| 22000CO00153–1—– | 3.251 | 4.06 | OLKGAAOGYATH | 1.98 | 4.06 | 1.271 | 0 |

Note: Sample numbers in the SAMPLEID column serve as unique identifiers for each sample section.

- Modify sample sections using PL/SQL procedures. For example:

BEGIN
  UPDATE HPMS_SAMPLE SET BMP=1.938, EMP=2.06 WHERE SAMPLE_NUMBER='UHKDRMMLLXLE';
  COMMIT; END;

- Insert new sample sections when needed. First obtain the maximum ORDER_ID:

SELECT MAX(ORDER_ID) FROM HPMS_SAMPLE;

- Then insert the new record with ORDER_ID incremented by 1:

BEGIN
  INSERT INTO HPMS_SAMPLE (ORDER_ID, BMP, EMP, SAMPLE_NUMBER, ROUTEID, ROUTEID_RH, GLOBAL_ROUTE_ID, SUB_ROUTE_ID)
  VALUES (9999, 1.938, 2.06, 'UHKDRMMLLXLE', '02000US00050  01EE**************','02000US00050--1-----', 11021,1);
  COMMIT;
END;

Note: At a minimum, populate ORDER_ID, BMP, EMP, SAMPLE_NUMBER, ROUTEID, ROUTEID_RH, GLOBAL_ROUTE_ID, and SUB_ROUTE_ID fields, though it is highly recommended to populate as many fields as possible.

#### Update Curve and Grade Information

- Run the Oracle package HPMS_SAMPLE_CURVE_GRADE in the ASSET_NEW environment to update curve and grade information for sample sections. This package replaces the deprecated BING_HMPS_CON package and typically takes 120-150 minutes to complete.

Note: Refer to the package documentation at \\shahanpmdata1\pmdataprg\Aditya\HPMS reporting\HPMS_SAMPLE_PACKAGES\HPMS_SAMPLE_CURVE_GRADE_DOC.md for detailed usage examples.

#### Update Pavement Structure Information

- Execute the HPMS_SAMPLE_PAV package to obtain base type and thickness data for sample sections. This package replaces the deprecated BING_HMPS_PAV package and typically requires 60-90 minutes to run.

Note: Refer to the package documentation at \\shahanpmdata1\pmdataprg\Aditya\HPMS reporting\HPMS_SAMPLE_PACKAGES\HPMS_SAMPLE_PAV_documentation.md for detailed usage instructions.

#### Update Data for Sample Sections Using Excel

- Open the provided Excel template from Z:\Aditya\HPMS reporting\Templates\ and copy it to your local C: drive for better performance.

- Enable macros in Excel and enter the most recent condition data year in cell Main!C31.

- Configure Excel security settings:

  - Under Excel ribbon > Data > Get Data > Query Options > Global > Security > Uncheck “Require user approval for new native database queries”

  - This allows SQL statements to run without interruptions

- Refresh the data by clicking “Refresh All” or “Refresh Only Condition Data” in the Data tab:

  - Refresh All executes the VBA subroutine RefreshAllQueries to refresh all data tables (10-20 minutes)

  - Refresh Only Condition Data executes the VBA subroutine RefreshPavConditionQuery to refresh only condition data (5-10 minutes)

Note: If prompted, select Database, and enter the database username and password for ASSET_NEW. Check the box to ignore privacy levels for this file and click “Run” on any prompts to approve running of native database queries.

- Update the HPMS_SAMPLE table by clicking the appropriate buttons:

  - Update All - executes VBA subroutine MergeRecords_HPMS_Samples to update all data fields (10-15 minutes)

  - Update Only Condition Data - executes VBA subroutine MergeRecords_HPMS_Samples_Pav_condition for condition metrics only (5-10 minutes)

  - Update Only ConHist Data - executes VBA subroutine MergeRecords_HPMS_Samples_Pav_Conhist for construction history only (5-10 minutes)

#### Excel Data Model and Sheet Information

The Excel file contains several sheets that store and process data. The data model follows HPMS Field Manual Table 4.3 calculation methods:

| Sheet Name | Description of Data |
| --- | --- |
| HPMS_SAMPLE | Existing HPMS_SAMPLE table with new data (columns A-Y existing, Z-AQ new data highlighted in green) |
| HPMS_CONDITION_100MMI | Data from full extent table for sample sections |
| ASPHALT CONCRETE NET THICKNESS | Pavement thickness data from business_plan_trend for sample sections (uses predominant value) |
| HPMS_YEAR_CONSTRUCTION | Construction year from PMBase_conhist AND BUSINESS_PLAN_TREATMENTS for sample sections |
| HPMS_YEAR_IMPROV | Year improved from BUSINESS_PLAN_TREATMENTS for sample sections |
| SURFACE_TYPE | Predominant surface type data for sample sections |
| BASE_THICKNESS_NEW | Base thickness and base type data for sample sections |
| Validation_Checks | General validation checks (condition and surface type) |
| Curve_Grade_Validation_Checks | Validation checks for curve/grade data |

#### HPMS Field Manual Calculation Methods

The following data items use specific calculation methods per HPMS Field Manual Table 4.3:

| Item Number | Data Item | Calculation Method |
| --- | --- | --- |
| 43 | Curve Classification | No Calculation Required |
| 44 | Grade Classification | No Calculation Required |
| 49 | Surface Type | Predominance |
| 50 | Rutting | Weighted Averaging |
| 51 | Faulting | Weighted Averaging |
| 52 | Cracking Percent | Weighted Averaging |
| 54 | Year of Last Improvement | Predominance |
| 55 | Year of Last Construction | Predominance |
| 56 | Last Overlay Thickness | Predominance |
| 57 | Thickness Rigid | Predominance |
| 58 | Thickness Flexible | Predominance |
| 59 | Base Type | Predominance |
| 60 | Base Thickness | Predominance |
| 61 | Climate Zone | Predominance |

Note: Values for data items 43 and 44 shall be reported for the defined limits of the Table of Potential Samples (TOPS) sections.

#### Quality Control and Validation

- Review built-in validation checks that are automatically applied:

  - Rutting data and rutting value date columns are updated only if surface type is asphalt (2,6,7,8)

  - Faulting data and faulting value date columns are updated only if surface type is jointed concrete (3,4,9,10)

  - IRI and cracking data with corresponding value date columns are updated for all sections

  - Any validation checks applied in full extent table are automatically applied to sample sections

- Examine validation checks in the “Validation_Checks” tab (added May 27, 2025), which identify:

  - Samples with IRI values that are now missing from the updated HPMS_SAMPLE table but were present in the previous year

  - Samples with cracking percent values that are now missing from the updated HPMS_SAMPLE table but were present in the previous year

  - Samples with rutting values (for asphalt sections) that are now missing from the updated HPMS_SAMPLE table but were present in the previous year

  - Samples with faulting values (for concrete sections) that are now missing from the updated HPMS_SAMPLE table but were present in the previous year

  - Samples with surface type values that are now missing from the updated HPMS_SAMPLE table but were present in the previous year

  - Samples with different predominant surface type values compared to the previous year

Note: Check the filters in the m-code and update before running. Compare these lists with any existing list of routes with missing data and send validation results to the Data Warehouse team member for review and approval.

- Verify curve and grade data using the “Curve_Grade_Validation_Checks” tab (added May 28, 2025) to identify:

  - Sections where the sum of curve category lengths (curve_a through curve_f) doesn’t equal the sample section length, rounded to three decimal places

  - Sections where the sum of grade category lengths (grade_a through grade_f) doesn’t equal the sample section length, rounded to three decimal places

- Execute the ADJUST_FINAL_LENGTHS procedure in the HPMS_SAMPLE_CURVE_GRADE package to address length mismatches.

- Run the final update statements to ensure data consistency per HPMS field manual specifications: sql     UPDATE HPMS_SAMPLE SET RUTTING = null, RUTTING_VALUE_DATE = null WHERE SURFACE_TYPE IN (3,4,5,9,10);     UPDATE HPMS_SAMPLE SET FAULTING = null, FAULTING_VALUE_DATE = null WHERE SURFACE_TYPE IN (1,2,5,6,7,8,11);     COMMIT;

- Update the HPMS_SAMPLE view in the PAV_READ schema by replacing the collect year immediately after the SELECT keyword with the current year in YYYY format: sql     CREATE OR REPLACE FORCE EDITIONABLE VIEW "PAV_READ"."HPMS_SAMPLE" AS     SELECT         2024 as COLLECT_YEAR,  -- Update this year for current submission         COUNTY,         ID_PREFIX,         ID_RTE_NO,         CASE WHEN MP_SUFFIX = '  ' THEN NULL ELSE MP_SUFFIX END MP_SUFFIX,         BMP,         EMP,         SAMPLE_NUMBER,         ROUTEID,         BEGIN_INTERSECT_MP,         END_INTERSECT_MP,         BEGIN_INTERSECT_DESC,         END_INTERSECT_DESC,         MUN_SORT,         ROAD_NAME,         NEW_SAMPLE,         SURFACE_TYPE,         ROUND(IRI,0) AS IRI, -- code IRI to the nearest inch per mile         TRIM(SUBSTR(IRI_VALUE_DATE,1,2)         || SUBSTR(IRI_VALUE_DATE,LENGTH(IRI_VALUE_DATE) - 4,5) ) AS IRI_MM_YYYY,         ROUND(RUTTING,2) as RUTTING, -- code Rutting to the nearest 0.01 inch         TRIM(SUBSTR(RUTTING_VALUE_DATE,1,2)         || SUBSTR(RUTTING_VALUE_DATE,LENGTH(RUTTING_VALUE_DATE) - 4,5) ) AS RUTTNG_MM_YYYY,         ROUND(FAULTING,2) as FAULTING, -- code Faulting to the nearest 0.01 inch         TRIM(SUBSTR(FAULTING_VALUE_DATE,1,2)         || SUBSTR(FAULTING_VALUE_DATE,LENGTH(FAULTING_VALUE_DATE) - 4,5) ) AS FAULTING_MM_YYYY,         ROUND(CRACKING_PERCENT,0) as CRACKING_PERCENT, -- code Cracking to the nearest 1%         TRIM(SUBSTR(CRACKING_VALUE_DATE,1,2)         || SUBSTR(CRACKING_VALUE_DATE,LENGTH(CRACKING_VALUE_DATE) - 4,5) ) AS CRACKING_MM_YYYY,         YEAR_LAST_IMPROV,         YEAR_LAST_CONSTRUCTION,         ROUND(LAST_OVERLAY_THICKNESS/0.5,0)*0.5 as LAST_OVERLAY_THICKNESS, -- code last overlay thickness to nearest 0.5 inch         ROUND(THICKNESS_RIGID/0.5,0)*0.5 AS THICKNESS_RIGID, -- code rigid thickness to nearest 0.5 inch         ROUND(THICKNESS_FLEXIBLE/0.5,0)*0.5 AS THICKNESS_FLEXIBLE, -- code flexible thickness to nearest 0.5 inch         BASE_TYPE,         ROUND(BASE_THICKNESS,0) AS BASE_THICKNESS, -- code base thickness to nearest inch         ROUND(CURVE_A,3), -- Round all curve and grade fields to three decimal places         ROUND(CURVE_B,3),         ROUND(CURVE_C,3),         ROUND(CURVE_D,3),         ROUND(CURVE_E,3),         ROUND(CURVE_F,3),         ROUND(GRADE_A,3),         ROUND(GRADE_B,3),         ROUND(GRADE_C,3),         ROUND(GRADE_D,3),         ROUND(GRADE_E,3),         ROUND(GRADE_F,3),         1 as CLIMATE_ZONE, -- Maryland is in LTPP climate Zone 1 (Wet freeze)         ROUTEID_RH     FROM         PAV_CONHIST.HPMS_SAMPLE;

#### HPMS Data Transformations

The SQL view includes these data transformations to meet HPMS field manual requirements:

- IRI: Rounded to the nearest inch per mile

- Rutting: Rounded to the nearest 0.01 inch

- Faulting: Rounded to the nearest 0.01 inch

- Cracking Percent: Rounded to the nearest 1%

- Last Overlay Thickness: Rounded to the nearest 0.5 inch

- Thickness Rigid: Rounded to the nearest 0.5 inch

- Thickness Flexible: Rounded to the nearest 0.5 inch

- Base Thickness: Rounded to the nearest inch

- Curve and Grade Fields (A-F): Rounded to three decimal places

- Climate Zone: Set to 1 (Maryland is in LTPP climate Zone 1 - Wet freeze)

Note: If the HPMS full extent or any dependent tables are updated, repeat the above steps to ensure the HPMS_SAMPLE table is up-to-date and re-send validations to the Data Warehouse team for approval.

## Production of HPMS Pavement Metadata file

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA) Pavement and Geotechnical Division (PAGD) staff to produce the Highway Performance Monitoring System (HPMS) Pavement Metadata file as required by the Federal Highway Administration (FHWA). The metadata file is typically provided by SHA’s Data Services Division (DSD) and requires annual updates to reflect the current reporting year’s data collection methods and protocols.

### Frequency

The HPMS Pavement Metadata file is produced once per year, typically in April when DSD provides the template file for the Interstate Submission.

### Purpose

The purpose of this SOP is to update the annual HPMS Pavement Metadata file with current year information, perform quality checks to ensure data consistency, and deliver the updated file to DSD for submission to FHWA. The metadata file documents the data collection methods, protocols, and reporting standards used for pavement data items in the HPMS submission.

### Resource Requirements

The production of the HPMS Pavement Metadata file involves one primary role: PAGD staff who updates the file and performs quality control (QC). The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| PAGD Staff | File Update and QC | 1 | 2.0 |

### Procedure

The procedure to produce the HPMS Pavement Metadata file consists of the following tasks:

1- Receive and Review Metadata File – performed by PAGD Staff

Summary: Under this task, the metadata file provided by DSD is received and reviewed for any necessary changes beyond the standard year update.

- Receive the metadata file from DSD, typically delivered in April for the Interstate Submission. The file is provided as a pipe (|) delimited text file.

- Review the file structure and verify it contains the following columns:

  - {“BEGIN_DATE”, type date}

  - {“State_Code”, Int64.Type}

  - {“Metadata_Type”, type text}

  - {“F_System”, Int64.Type}

  - {“Is_Urban”, type text}

  - {“Is_State_Owned”, type text}

  - {“Value_Numeric”, type number}

2- Update Metadata File – performed by PAGD Staff

Summary: Under this task, the metadata file is updated with the current year information and any necessary changes to data collection methods or reporting protocols.

- Update the BEGIN_DATE column to reflect the current reporting year. This is typically the only change required unless there have been modifications to data collection methods, protocols, or reporting standards.

- Review any potential changes to other pavement data items that may be necessary if there are:

  - Changes to data collection methods or protocols

  - Changes to reporting methods for the reported data

  - Updates to pavement management system procedures

3- Quality Control and Validation – performed by PAGD Staff

Summary: Under this task, quality control checks are performed to ensure data consistency between the current and previous year’s metadata submissions.

- Access the reference materials located in Z:\Aditya\HPMS reporting\pavement metadata\YYYY\:

  - Current year pavement metadata file: Metadata_Pavement_MMDDYYYY_submitted.txt

  - Pavement metadata descriptions and comparisons: Metadata Descriptions and Comparisons YYYY.xlsx

  - HPMS Field Manual: FHWA Highway Performance Monitoring System Field Manual

- Review the Excel workbook containing metadata descriptions and comparisons:

  - Worksheet 2017 Metadata_Pav Descriptions: Contains descriptions of each pavement data item

  - Worksheet Current_Year: Contains the current year submitted metadata

  - Worksheet Previous_Year: Contains the previous year submitted metadata

- Refresh the source data file in the PowerQuery M-code for both the Current_Year and Previous_Year worksheets to ensure accurate comparisons.

- Validate the data using the comparison formula in the Current_Year worksheet:

=CONCAT(Metadata_Pavement_CurYr[@[State_Code]:[Value_Numeric]])=CONCAT(Metadata_Pavement_PrevYr[@[State_Code]:[Value_Numeric]])

- This formula compares the Value_Numeric column and returns TRUE or FALSE to identify any unexpected discrepancies.

- Investigate any discrepancies identified in the comparison:

  - Review changes to determine if they are intentional and justified

  - Verify that unexpected differences are corrected before final submission

  - Document any legitimate changes and their reasons

4- Final Submission and Documentation – performed by PAGD Staff

Summary: Under this task, the validated metadata file is submitted to DSD and properly archived with supporting documentation.

- Finalize the metadata file ensuring it remains in pipe (|) delimited text file format.

- Submit the completed metadata file to DSD via email with appropriate documentation of any changes made.

- Archive the following items in Z:\Aditya\HPMS reporting\pavement metadata\YYYY\:

  - Copy of the submitted metadata file (.txt format)

  - Email correspondence (.msg format)

  - Updated comparison Excel workbook

  - Any supporting documentation for changes made

### Troubleshooting

During the HPMS Pavement Metadata file production process, several issues may arise:

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| File format issues | Incorrect delimiter or encoding | Verify file is saved as pipe delimited text format; check encoding settings |
| Comparison validation failures | Data quality issues | Compare with previous year’s data and investigate discrepancies; consult with Data Warehouse team before proceeding with updates |
| Missing reference files | Moved or renamed files | Check network drive access and file locations; contact IT support if needed |
| PowerQuery refresh errors | Database connection or file path issues | Verify file paths in M-code; update connections as needed |

### References

- Metadata file location: Z:\Aditya\HPMS reporting\pavement metadata\YYYY\

- HPMS Field Manual: FHWA Highway Performance Monitoring System Field Manual

- Pavement metadata descriptions and historical comparisons: Metadata Descriptions and Comparisons YYYY.xlsx

## Production of HPMS Submission Comments

### General

This section describes the standard operating procedure (SOP) used by staff from the Maryland Department of Transportation State Highway Administration’s (SHA) Pavement and Geotechnical Division (PAGD) to generate submission comments for the Highway Performance Monitoring System (HPMS) validation results. These comments provide explanations for data issues identified during the FHWA validation process and are submitted along with the HPMS data deliverables.

### Frequency

The HPMS Submission Comments document is produced once per year, typically after receiving validation results from the Data Services Division (DSD) and prior to final HPMS submission.

### Purpose

The purpose of this SOP is to document the process of generating standardized comments to explain validation issues identified in the HPMS data submission. These comments provide justification and context for data anomalies, missing values, or other issues flagged during validation, which helps FHWA understand the circumstances behind data exceptions.

### Resource Requirements

The production of the HPMS Submission Comments document involves one primary role: PAGD staff who generates and reviews the comments. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| PAGD Staff | Comment Generation and QC | 1 | 4.0 |

### Procedure

The procedure to produce the HPMS Submission Comments document consists of the following tasks:

1- Receive Validation Results – performed by PAGD Staff

Summary: Under this task, validation results are received from DSD and prepared for review and comment generation.

- Receive the validation results file from DSD. This file is typically delivered as a pipe (|) delimited text file containing validation rules and affected route segments.

- Review the file structure and verify it contains the following columns:

  - ValidationRule

  - ValidationDescription

  - RouteId

  - BeginPoint

  - EndPoint

  - DataItemsImpacted

  - Various data value columns (IriVn, RuttingVn, etc.)

  - Segment information (SegmentLength, LaneMiles, etc.)

2- Generate Submission Comments – performed by PAGD Staff

Summary: Under this task, submission comments are generated for validation issues using either manual methods or an automated tool.

PAGD staff may choose one of the following approaches to generate comments:

Option A: Manual Comment Generation

- Analyze the validation results to identify patterns and common issues. Group similar validation issues together for consistent commenting.

- Generate comments for each validation rule where the record count is greater than 0. Comments should follow this standard format:

[**Validation Rule SJ-PM2-##**: # Records (#.### miles)]

**Validation Description:** *Validation Description text*

**Comments:** Collected data was invalid or data was not collected due to various reasons including the following:
- Reason 1.
- Reason 2.

- Provide specific explanations for data issues based on the validation rule and affected data elements. Common explanations include:

  - Construction or work zones during data collection period

  - Equipment calibration or failure issues

  - Access restrictions to certain roadway segments

  - Weather-related data collection limitations

  - Data processing errors that could not be resolved

Option B: Automated Comment Generation

- Access the Python notebook generate_submission_comments.ipynb saved in the Z:\Aditya\HPMS reporting\submission comments\HPMS_Submission_Comments_Generator location.

- Verify the following requirements are met:

  - Pandoc is installed on the system

  - The template markdown file (Maryland HPMS Submission Comments_template.md) is available

  - The validation results file (Final_Pavement_Validations.txt) is properly formatted

- Execute the code in the Python notebook to automatically:

  - Read the template markdown file

  - Process the validation results file

  - Calculate total record counts and affected miles for each validation rule

  - Generate a dated output markdown file: Maryland HPMS Submission Comments_{YYYYMMDD}.md

  - Review and update the markdown file as needed

  - Convert the markdown to a properly formatted Word document (.docx) with consistent styling

- Review the automatically generated document for accuracy and completeness.

3- Finalize and Submit Comments – performed by PAGD Staff

Summary: Under this task, the comments document is finalized and submitted to DSD.

- Review the completed comments document, checking for:

  - Accuracy of validation rule descriptions

  - Completeness of comments for all validation issues

  - Clarity and consistency of explanations

  - Proper formatting and document structure

- Finalize the document by:

  - Adding any additional context or explanations as needed

  - Ensuring all validation rules with issues have appropriate comments

  - Proofreading for grammar, spelling, and formatting issues

- Submit the completed comments document to DSD via email with appropriate subject line and context.

- Archive the following items in Z:\Aditya\HPMS reporting\submission comments\YYYY\:

  - Copy of the validation results file

  - Copy of the submitted comments document

  - Email correspondence

  - Any supporting documentation

### Troubleshooting

During the HPMS Submission Comments generation process, several issues may arise:

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| Validation file format issues | Incorrect delimiter or column structure | Request correctly formatted file from DSD; verify file formatting before processing |
| Missing validation descriptions | Incomplete validation results file | Contact DSD for complete information; consult HPMS Field Manual for validation rule descriptions |
| Python notebook execution errors | Missing dependencies or file path issues | Verify Pandoc installation; check file paths and permissions; consult IT support if needed |
| Unclear validation issues | Complex or unusual data anomalies | Consult with data collection teams or subject matter experts for proper explanation |

### References

- HPMS Field Manual: FHWA Highway Performance Monitoring System Field Manual

- Python notebook location: Z:\Aditya\HPMS reporting\submission comments\HPMS_Submission_Comments_Generator\generate_submission_comments.ipynb

- Template files location: Z:\Aditya\HPMS reporting\submission comments\HPMS_Submission_Comments_Generator\

- Archive location: Z:\Aditya\HPMS reporting\submission comments\YYYY\

- Pandoc: Pandoc installation instructions

## QA of HPMS Tables

### General

This section describes the standard operating procedure (SOP) used by the Assistant Division Chief (ADC) of the Pavement Management Division of the Maryland Department of Transportation State Highway Administration (SHA) to perform quality assurance (QA) of the Highway Performance Monitoring System (HPMS) Sample Table and Full-Extent Table before they are submitted to the Data Services Division (DSD). This SOP is performed once the annual HPMS Sample Table and the HPMS Full-Extent Table are produced. It is the last step for the reporting of data to the Federal Highway Administration (FHWA).

### Frequency

The QA of HPMS Sample Table and Full-Extent Table is performed once per year.

### Purpose

The purpose of this SOP is:

- check that the HPMS Sample Table and the HPMS Full-Extent Table (1) have been correctly populated, (2) are complete, and (3) present an accurate description of the pavement network condition before the tables are delivered to the DSD;

- investigate, if needed, reasons that explain changes in the pavement network condition with respect to the previous year condition; and

- notify the DSD of delivery of the referenced tables and to have the DSD address data issues identified from their review of the submitted tables.

### Resource Requirements

All the procedural steps for performing QA of the HPMS Sample Table and the HPMS Full-Extent Table are to be carried out by the ADC. However, this SOP may involve the Team Leaders (TL) from other pertinent groups, who as required would provide support and information to resolve data issues encountered during the QA process. The estimated effort levels in the table below represent the total time, in man-hours, to complete the process. These time estimates assume no issues are encountered.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| ADC | QA checks | 1 | 8.0 |
| FED TL | Issue resolution | 1 | AR06/12/2019 |
| DPT TL | Issue resolution | 1 | AR1 |
| DAT TL | Issue resolution | 1 | AR1 |
| DAT TL | Issue resolution | 1 | AR1 |

### Procedure

The procedure required to perform QA of the HPMS tables is comprised of the following two tasks:

- performing QA of HPMS Sample Table and Full-Extent Table, and

- delivering the above referenced HPMS Tables to DSD.

1- Performing QA of HPMS Sample Table and Full-Extent Table – performed by ADC

Summary: Under this task, the final quality checks on the HPMS Sample Table and HPMS Full-Extent Table will be performed before they are delivered to the DSD. The checks in this task serve as a means to confirm the quality of the processed data and, as appropriate, to further investigate the reasons behind the observed changes in the pavement network condition over a one year period of time.

- Using SQL Developer, check that the data in both the HPMS Sample Table and HPMS Full-Extent Table are complete.

  - Use data from the DSD’s annual inventory table for the corresponding collection year to identify sections with missing data from HPMA Full-Extent Table.

  - Use data from the DSD’s Sample Section table for the corresponding collection year to identify sections with missing data from HPMA Sample Table.

Review the identified missing data, if any, from each table and discuss with appropriate SHA TL whether those data can be recovered through reprocessing of the data. If feasible, then request SHA TL to reprocess potentially recoverable data and confirm that these data have been effectively included in the referenced HPMS tables.

- Using SQL Developer, check that the data in both the HPMS Sample Table and HPMS Full-Extent Table have been populated correctly. For example, check that the correct data elements are associated with the correct surface type – e.g., faulting data should only be present for jointed concrete pavements and rutting data should only be present for asphalt concrete pavements. Proceed to next step if all data have been correctly populated. Otherwise, communicate data issues identified to the DAT TL for him/her to review these data and to make the appropriate corrections. Review HPMS tables corrected by DAT TL to confirm that the data issues identified have been resolved.

- Using SQL Developer and Microsoft Excel, query the HPMS Sample Table and HPMS Full-Extent Table for trends and summary statistics to evaluate the changes in condition of the pavement network over a one year period of time. Example checks performed include:

  - Check that data related to pavement condition metrics and HPMS condition ratings (percentage of lane miles in Good, Fair, and Poor condition) in the HPMS Full-Extent Table are reasonable as compared to summary statistics obtained from data collected for the State’s Pavement Management System and to the values reported in the HPMS reports from the previous year. Also, check the reasonableness of the statistics relative to the trends over time; e.g., the total lanes miles reported for a given surface type is not expected to differ by more than 1% from the previous year.

  - Check that data related to construction information (e.g., total lanes miles treated) in the HPMS Sample Table is reasonable when compared to the values reported in HPMS reports from the previous year.

Discuss data identified as suspicious (e.g., large changes from one year to the next or large discrepancies with data from other reports for the same data collection year) with pertinent SHA TL to evaluate whether the issues are the result of improper or erroneous data processing or if they are due to changing conditions and circumstances (e.g., a significant reduction in treated miles from previous year may be explained by a decreased budget). Proceed to next step if all data have been correctly populated. Otherwise, request pertinent SHA TL to address the data issues that have been identified. Lastly, review HPMS tables corrected by pertinent SHA TL to confirm that all data issues have been resolved.

2- Reviewing and Finalizing HPMS tables with DSD – performed by ADC

Summary: Under this task, the HPMS Sample Table and the HPMS Full-Extent Table are finalized through an iterative process between the ADC and DSD in order to eliminate any remaining issues with the data in the referenced tables.

- Notify DSD that the HPMS Sample Table and HPMS Full-Extent Table are ready for their review06/12/2019 and that the tables can be accessed through the “Pav_READ” user in the server.

- Upon receipt of DSD’s review report with the identified HPMS data issues, if any, investigate whether these issues can be resolved through discussion with the pertinent SHA TL. If any data can be recovered, take the necessary steps to recover them and update the tables. Identify and note a reason for unrecoverable data. Notify DSD that the tables are ready for re-review, identifying those data issues that were not resolved along with the reason why they could not be resolved.

- Repeat Step 5 until all remaining data issues highlighted in the DSD report are considered unrecoverable (e.g., missing data on sections under construction during the data collection season). Communicate to the DSD that the tables have been fully updated and that the work has been finalized.

## Process PM2 Dataset

### General

This section describes the SOP used by DAT to process the Performance Measure 2 (PM2) dataset received from the DSD. The PM2 dataset contains pavement condition data that is critical for HPMS reporting and performance monitoring activities. This procedure ensures accurate processing, validation, and integration of PM2 data into the organizational database systems for subsequent analysis and reporting purposes.

The PM2 dataset processing involves downloading compressed data files from DSD, executing specialized Python code for data transformation and validation, performing quality control reviews against official HPMS report cards, and importing the processed data into Oracle database systems. This comprehensive workflow ensures data integrity and accuracy throughout the entire process from initial receipt through final database integration.

### Frequency

The PM2 dataset processing is performed as needed when new PM2 datasets are received from DSD, typically on an annual basis or when the official pavement condition data becomes available from the HPMS software after submission is completed.

### Purpose

The purpose of this SOP is to establish a standardized methodology for processing PM2 datasets received from DSD to ensure accurate data transformation, comprehensive quality control validation, and proper integration into Oracle database systems. This procedure enables the organization to maintain current and accurate pavement condition data for HPMS reporting, performance monitoring, and analytical activities while ensuring data consistency and reliability across all processing steps.

### Resource Requirements

The processing of PM2 datasets requires one data analyst with specialized knowledge of Python programming, Oracle database operations, and HPMS data structures. The estimated effort level represents the total time required to complete the entire process from initial download through final database integration, assuming no significant data quality issues are encountered during processing.

| Position | Function | Resources | Effort Level |
| --- | --- | --- | --- |
| Data Analyst | Data Processing, Analysis and Quality Control | 1 | 4.0 - 8.0 hours |

### Procedure

The procedure for processing PM2 datasets consists of eight sequential steps that must be completed in order to ensure proper data handling, validation, and integration. The process begins with data acquisition from DSD and concludes with successful integration into the Oracle database table.

- Download the PM2 dataset from DSD and save it to the local machine. If the downloaded dataset is provided in a compressed format such as zip or 7zip, extract all files to an accessible local directory. Ensure that all files are properly extracted and that no data corruption occurred during the download or extraction process.

- Execute the Python code contained in Z:\Aditya\HPMS reporting\Submission 2025\HPMS-Pm2nonspatialJoin\Process_PM2_Data_from_DSD.ipynb, also shown below to process the dataset. This Jupyter notebook contains specialized algorithms for data transformation, cleaning, and preparation that convert the raw PM2 data into the required format for database integration. Run all cells in the notebook sequentially to ensure proper data processing workflow completion.

# Step 1: Import Libraries, read data, calculate condition labels and functional groups
import pandas as pd
import os
from datetime import datetime
import numpy as np

# Get user input for the pipe-delimited file path
file_path = input("Enter the path to the pipe-delimited text file: ")

# Check if file exists
if not os.path.exists(file_path):
    print(f"File not found: {file_path}")
else:
    print(f"Reading file: {file_path}")

# Define column data types based on the specification
dtype_dict = {
    'DataYear': 'Int64',
    'StateId': 'Int64', 
    'NonInterstateNhs': 'Int64',
    'RouteId': 'str',
    'BeginPoint': 'float64',
    'EndPoint': 'float64',
    'FsystemVn': 'Int64',
    'FacilityTypeVn': 'Int64',
    'ThroughLanesVn': 'Int64',
    'DirThroughLanesVn': 'Int64',
    'LanesToComputeLaneMiles': 'Int64',
    'UrbanIdVn': 'Int64',
    'NhsVn': 'Int64',
    'StructureTypeVn': 'Int64',
    'SurfaceTypeVn': 'Int64',
    'IriVn': 'float64',
    'IriVt': 'str',
    'PsrVn': 'float64',
    'PsrVt': 'str',
    'RuttingVn': 'float64',
    'RuttingVt': 'str',
    'FaultingVn': 'float64',
    'FaultingVt': 'str',
    'CrackingPercentVn': 'float64',
    'CrackingPercentVt': 'str',
    'FsystemBeginPoint': 'float64',
    'FsystemEndPoint': 'float64',
    'FacilityTypeBeginPoint': 'float64',
    'FacilityTypeEndPoint': 'float64',
    'ThroughLanesBeginPoint': 'float64',
    'ThroughLanesEndPoint': 'float64',
    'DirThroughLanesBeginPoint': 'float64',
    'DirThroughLanesEndPoint': 'float64',
    'UrbanIdBeginPoint': 'float64',
    'UrbanIdEndPoint': 'float64',
    'StructureTypeBeginPoint': 'float64',
    'StructureTypeEndPoint': 'float64',
    'SurfaceTypeBeginPoint': 'float64',
    'SurfaceTypeEndPoint': 'float64',
    'IriBeginPoint': 'float64',
    'IriEndPoint': 'float64',
    'PsrBeginPoint': 'float64',
    'PsrEndPoint': 'float64',
    'RuttingBeginPoint': 'float64',
    'RuttingEndPoint': 'float64',
    'FaultingBeginPoint': 'float64',
    'FaultingEndPoint': 'float64',
    'CrackingPercentBeginPoint': 'float64',
    'CrackingPercentEndPoint': 'float64',
    'ProcessId': 'str',
    'SegmentLength': 'float64',
    'LaneMiles': 'float64',
    'Irisl': 'float64',
    'Psrsl': 'str',
    'RuttingSl': 'float64',
    'FaultingSl': 'float64',
    'CrackingPercentSl': 'float64',
    'RuttingSc': 'float64',
    'FaultingSc': 'float64',
    'CrackingPercentSc': 'float64',
    'Bridge': 'Int64',
    'UnpavedOrOther': 'str',
    'MissingInvalidOrUnresolved': 'Int64',
    'SegmentRate': 'Int64',
    'IriUnresolved': 'str',
    'NhsBeginPoint': 'float64',
    'NhsEndPoint': 'float64',
    'ShapeId': 'Int64'
}

# Define datetime columns
datetime_cols = ['IriVd', 'PsrVd', 'RuttingVd', 'FaultingVd', 'CrackingPercentVd', 'SurfaceTypeVd']

# Read the pipe-delimited file
try:
    df = pd.read_csv(file_path, sep='|', dtype=dtype_dict, parse_dates=datetime_cols, low_memory=False)
    print(f"Successfully loaded {len(df)} records from the file")
except Exception as e:
    print(f"Error reading file: {e}")
    df = pd.DataFrame()  # Create empty DataFrame if reading fails

if not df.empty:
    # Complete column rename map based on M-code reference
    column_rename_map = {
        'DataYear': 'YEAR_RECORD',
        'StateId': 'STATE_CODE', 
        'RouteId': 'ROUTE_ID',
        'BeginPoint': 'BEGIN_POINT',
        'EndPoint': 'END_POINT',
        'FsystemVn': 'F_SYSTEM',
        'FacilityTypeVn': 'FACILITY_TYPE',
        'ThroughLanesVn': 'THROUGH_LANES',
        'DirThroughLanesVn': 'DIR_THROUGH_LANES',
        'LanesToComputeLaneMiles': 'LANES_TO_COMPUTE_LANEMILES',
        'UrbanIdVn': 'URBAN_CODE',
        'NhsVn': 'NHS',
        'StructureTypeVn': 'STRUCTURE_TYPE',
        'SurfaceTypeVn': 'SURFACE_TYPE',
        'IriVn': 'IRI',
        'IriVd': 'IRI_D',
        'IriVt': 'IRI_T',
        'PsrVn': 'PSR',
        'PsrVd': 'PSR_D',
        'PsrVt': 'PSR_T',
        'RuttingVn': 'RUTTING',
        'RuttingVd': 'RUTTING_D',
        'RuttingVt': 'RUTTING_T',
        'FaultingVn': 'FAULTING',
        'FaultingVd': 'FAULTING_D',
        'FaultingVt': 'FAULTING_T',
        'CrackingPercentVn': 'CRACKING_PERCENT',
        'CrackingPercentVd': 'CRACKING_PERCENT_D',
        'CrackingPercentVt': 'CRACKING_PERCENT_T',
        'FsystemBeginPoint': 'F_SYSTEM_BEGIN_POINT',
        'FsystemEndPoint': 'F_SYSTEM_END_POINT',
        'FacilityTypeBeginPoint': 'FACILITY_TYPE_BEGIN_POINT',
        'FacilityTypeEndPoint': 'FACILITY_TYPE_END_POINT',
        'ThroughLanesBeginPoint': 'THROUGH_LANES_BEGIN_POINT',
        'ThroughLanesEndPoint': 'THROUGH_LANES_END_POINT',
        'DirThroughLanesBeginPoint': 'DIR_THROUGH_LANES_BEGIN_POINT',
        'DirThroughLanesEndPoint': 'DIR_THROUGH_LANES_END_POINT',
        'UrbanIdBeginPoint': 'URBAN_CODE_BEGIN_POINT',
        'UrbanIdEndPoint': 'URBAN_CODE_END_POINT',
        'StructureTypeBeginPoint': 'STRUCTURE_TYPE_BEGIN_POINT',
        'StructureTypeEndPoint': 'STRUCTURE_TYPE_END_POINT',
        'SurfaceTypeBeginPoint': 'SURFACE_TYPE_BEGIN_POINT',
        'SurfaceTypeEndPoint': 'SURFACE_TYPE_END_POINT',
        'IriBeginPoint': 'IRI_BEGIN_POINT',
        'IriEndPoint': 'IRI_END_POINT',
        'PsrBeginPoint': 'PSR_BEGIN_POINT',
        'PsrEndPoint': 'PSR_END_POINT',
        'RuttingBeginPoint': 'RUTTING_BEGIN_POINT',
        'RuttingEndPoint': 'RUTTING_END_POINT',
        'FaultingBeginPoint': 'FAULTING_BEGIN_POINT',
        'FaultingEndPoint': 'FAULTING_END_POINT',
        'CrackingPercentBeginPoint': 'CRACKING_PERCENT_BEGIN_POINT',
        'CrackingPercentEndPoint': 'CRACKING_PERCENT_END_POINT',
        'LaneMiles': 'LANE_MILES',
        'Irisl': 'IRI_SL',
        'Psrsl': 'PSR_SL',
        'RuttingSl': 'RUTTING_SL',
        'FaultingSl': 'FAULTING_SL',
        'CrackingPercentSl': 'CRACKING_PERCENT_SL',
        'RuttingSc': 'RUTTING_SC',
        'FaultingSc': 'FAULTING_SC',
        'CrackingPercentSc': 'CRACKING_PERCENT_SC',
        'NonInterstateNhs': 'NON_INTERSTATE_NHS',
        'Bridge': 'BRIDGE',
        'MissingInvalidOrUnresolved': 'MISSING_INVALID_OR_UNRESOLVED',
        'UnpavedOrOther': 'UNPAVED_OR_OTHER',
        'SegmentLength': 'SEGMENT_LENGTH',
        'SegmentRate': 'SEGMENT_RATE',
        'IriUnresolved': 'MIU_IRI'
    }
    
    # Rename columns
    df = df.rename(columns=column_rename_map)
    
    # Apply filters based on M-code reference
    df_filtered = df[
           (df['F_SYSTEM'].notna()) & 
           (df['NHS'].notna()) & 
           (df['FACILITY_TYPE'].isin([1, 2])) & 
           (df['STRUCTURE_TYPE'].isna()) 
    #       (df['SURFACE_TYPE'].notna())
    ].copy()
    
    print(f"After filtering: {len(df_filtered)} records remain")
    
    # Define the conditions for IRI_CONDITION
    def iri_condition(iri):
        if pd.isnull(iri):
            return None
        elif iri < 95:
            return "GOOD"
        elif iri <= 170:
            return "FAIR"
        elif iri > 170:
            return "POOR"

    # Define the conditions for CRACKING_CONDITION
    def cracking_condition(cracking_percent, surface_type):
        if pd.isnull(cracking_percent):
            return None
        if surface_type in [2, 6, 7, 8]:
            if cracking_percent < 5:
                return "GOOD"
            elif 5 <= cracking_percent <= 20:
                return "FAIR"
            elif cracking_percent > 20:
                return "POOR"
        elif surface_type in [3, 4, 9, 10]:
            if cracking_percent < 5:
                return "GOOD"
            elif 5 <= cracking_percent <= 15:
                return "FAIR"
            elif cracking_percent > 15:
                return "POOR"
        elif surface_type == 5:
            if cracking_percent < 5:
                return "GOOD"
            elif 5 <= cracking_percent <= 10:
                return "FAIR"
            elif cracking_percent > 10:
                return "POOR"

    # Define the conditions for FAULTING_CONDITION
    def faulting_condition(faulting):
        if pd.isnull(faulting):
            return None
        elif faulting < 0.1:
            return "GOOD"
        elif faulting <= 0.15:
            return "FAIR"
        elif faulting > 0.15:
            return "POOR"

    # Define the conditions for RUTTING_CONDITION
    def rutting_condition(rutting):
        if pd.isnull(rutting):
            return None
        elif rutting < 0.2:
            return "GOOD"
        elif rutting <= 0.4:
            return "FAIR"
        elif rutting > 0.4:
            return "POOR"
    
    # Define the conditions for SURFACE_MATERIAL
    def surface_material(surface_type):
        if pd.isnull(surface_type):
            return "OTHER"
        elif surface_type in [2, 6, 7, 8]:
            return "ASPHALT"
        elif surface_type in [3, 4, 9, 10]:
            return "RJ"
        elif surface_type == 5:
            return "RC"
        else:
            return "OTHER"

    # Define the conditions for FUNCTIONAL_GROUP
    def functional_group(f_system, nhs, urban_code):
        if pd.isnull(nhs):
            return "Other"
        elif pd.isnull(f_system):
            return "Other"
        elif pd.isnull(urban_code):
            return "Other"    
        elif f_system == 1 and nhs in (1,2,3,4,5,6,7,8,9) and urban_code > 0:
            return "Interstate"
        elif f_system in (2,3,4,5,6,7) and nhs in (1,2,3,4,5,6,7,8,9) and urban_code > 0:
            return "Non-Interstate NHS"    
        else:
            return "Other"
    
    # Apply the condition functions
    df_filtered['IRI_CONDITION'] = df_filtered['IRI'].apply(iri_condition)
    df_filtered['CRACKING_CONDITION'] = df_filtered.apply(lambda row: cracking_condition(row['CRACKING_PERCENT'], row['SURFACE_TYPE']), axis=1)
    df_filtered['FAULTING_CONDITION'] = df_filtered['FAULTING'].apply(faulting_condition)
    df_filtered['RUTTING_CONDITION'] = df_filtered['RUTTING'].apply(rutting_condition)
    df_filtered['SURFACE_MATERIAL'] = df_filtered['SURFACE_TYPE'].apply(surface_material)
    df_filtered['FUNCTIONAL_GROUP'] = df_filtered.apply(
        lambda row: functional_group(row['F_SYSTEM'], row['NHS'], row['URBAN_CODE']), 
        axis=1
    )
    
    # Define the conditions for OVERALL_CONDITION
    def overall_condition(row):
        if pd.notnull(row['STRUCTURE_TYPE']) and row['STRUCTURE_TYPE'] == 1:
            return "STRUCTURE"
        
        # Check for MissingInvalidOrUnresolved field
        elif (pd.notnull(row.get('MISSING_INVALID_OR_UNRESOLVED')) and 
              row['MISSING_INVALID_OR_UNRESOLVED'] != 0):
            return "MISSING_DATA"
        
        surface_type = row['SURFACE_TYPE']
        
        if pd.isnull(surface_type):
            return "MISSING_DATA"
        
        # Continuous Concrete (Surface Type 5)
        elif surface_type == 5:
            iri_cond = row['IRI_CONDITION']
            crack_cond = row['CRACKING_CONDITION']
            
            if pd.isnull(iri_cond) or pd.isnull(crack_cond):
                return "MISSING_DATA"
            elif iri_cond == "GOOD" and crack_cond == "GOOD":
                return "GOOD"
            elif iri_cond == "POOR" and crack_cond == "POOR":
                return "POOR"
            else:
                return "FAIR"
        
        # Jointed Concrete (Surface Types 3, 4, 9, 10)
        elif surface_type in [3, 4, 9, 10]:
            iri_cond = row['IRI_CONDITION']
            crack_cond = row['CRACKING_CONDITION']
            fault_cond = row['FAULTING_CONDITION']
            
            if pd.isnull(iri_cond) or pd.isnull(crack_cond) or pd.isnull(fault_cond):
                return "MISSING_DATA"
            
            conditions = [iri_cond, crack_cond, fault_cond]
            good_count = conditions.count("GOOD")
            poor_count = conditions.count("POOR")
            
            if good_count == 3:
                return "GOOD"
            elif poor_count >= 2:
                return "POOR"
            else:
                return "FAIR"
        
        # Asphalt (Surface Types 2, 6, 7, 8)
        elif surface_type in [2, 6, 7, 8]:
            iri_cond = row['IRI_CONDITION']
            crack_cond = row['CRACKING_CONDITION']
            rutt_cond = row['RUTTING_CONDITION']
            
            if pd.isnull(iri_cond) or pd.isnull(crack_cond) or pd.isnull(rutt_cond):
                return "MISSING_DATA"
            
            conditions = [iri_cond, crack_cond, rutt_cond]
            good_count = conditions.count("GOOD")
            poor_count = conditions.count("POOR")
            
            if good_count == 3:
                return "GOOD"
            elif poor_count >= 2:
                return "POOR"
            else:
                return "FAIR"
        
        else:
            return "MISSING_DATA"  # Unknown surface type
    
    # Apply OVERALL_CONDITION
    df_filtered['OVERALL_CONDITION'] = df_filtered.apply(overall_condition, axis=1)
    
    # Define the final columns to keep based on requirements
    final_columns = [
        'STATE_CODE', 'ROUTE_ID', 'BEGIN_POINT', 'END_POINT', 'YEAR_RECORD',
        'IRI', 'IRI_T', 'IRI_D', 'PSR', 'PSR_T', 'PSR_D', 'SURFACE_TYPE',
        'RUTTING', 'RUTTING_T', 'RUTTING_D', 'FAULTING', 'FAULTING_T', 'FAULTING_D',
        'CRACKING_PERCENT', 'CRACKING_PERCENT_T', 'CRACKING_PERCENT_D',
        'NHS', 'URBAN_CODE', 'FACILITY_TYPE', 'F_SYSTEM', 'STRUCTURE_TYPE',
        'THROUGH_LANES', 'DIR_THROUGH_LANES', 'NON_INTERSTATE_NHS', 'SEGMENT_RATE',
        'LANES_TO_COMPUTE_LANEMILES', 'BRIDGE', 'MISSING_INVALID_OR_UNRESOLVED',
        'UNPAVED_OR_OTHER', 'LANE_MILES', 'IRI_SL', 'PSR_SL', 'RUTTING_SL',
        'FAULTING_SL', 'CRACKING_PERCENT_SL', 'RUTTING_SC', 'FAULTING_SC',
        'CRACKING_PERCENT_SC', 'MIU_IRI', 'SEGMENT_LENGTH',
        'F_SYSTEM_BEGIN_POINT', 'F_SYSTEM_END_POINT', 'FACILITY_TYPE_BEGIN_POINT',
        'FACILITY_TYPE_END_POINT', 'THROUGH_LANES_BEGIN_POINT', 'THROUGH_LANES_END_POINT',
        'DIR_THROUGH_LANES_BEGIN_POINT', 'DIR_THROUGH_LANES_END_POINT',
        'URBAN_CODE_BEGIN_POINT', 'URBAN_CODE_END_POINT', 'STRUCTURE_TYPE_BEGIN_POINT',
        'STRUCTURE_TYPE_END_POINT', 'SURFACE_TYPE_BEGIN_POINT', 'SURFACE_TYPE_END_POINT',
        'IRI_BEGIN_POINT', 'IRI_END_POINT', 'PSR_BEGIN_POINT', 'PSR_END_POINT',
        'RUTTING_BEGIN_POINT', 'RUTTING_END_POINT', 'FAULTING_BEGIN_POINT',
        'FAULTING_END_POINT', 'CRACKING_PERCENT_BEGIN_POINT', 'CRACKING_PERCENT_END_POINT',
        'IRI_CONDITION', 'RUTTING_CONDITION', 'CRACKING_CONDITION', 'FAULTING_CONDITION',
        'OVERALL_CONDITION', 'SURFACE_MATERIAL', 'FUNCTIONAL_GROUP'
    ]
    
    # Select only the columns that exist in the dataframe
    available_columns = [col for col in final_columns if col in df_filtered.columns]
    missing_columns = [col for col in final_columns if col not in df_filtered.columns]
    
    if missing_columns:
        print(f"Warning: The following required columns are missing from the data: {missing_columns}")
    
    # Keep only the specified columns
    all_records_df = df_filtered[available_columns].copy()
    
    print(f"Data processing completed. Final dataframe has {len(all_records_df)} records with {len(available_columns)} columns.")
    print(f"Final columns: {list(all_records_df.columns)}")
    
    # Print some debugging information
    print(f"\nFunctional Group breakdown:")
    if 'FUNCTIONAL_GROUP' in all_records_df.columns:
        print(all_records_df['FUNCTIONAL_GROUP'].value_counts())
    
    print(f"\nOverall Condition breakdown:")
    if 'OVERALL_CONDITION' in all_records_df.columns:
        print(all_records_df['OVERALL_CONDITION'].value_counts())
        
else:
    all_records_df = pd.DataFrame()
    print("No data loaded - creating empty dataframe")

# Step 2: Data Visualization. Plot Overall Condition by Functional Group and verify with PM2 report card
import pandas as pd
import matplotlib.pyplot as plt
from IPython.display import display

# Ensure plots are displayed inline in Jupyter Notebook
%matplotlib inline

df = all_records_df  # Make sure all_records_df is defined earlier in your code

# Define a custom order for the 'OVERALL_CONDITION' column
condition_order = ['GOOD', 'FAIR', 'POOR']

# Function to prepare data for a given functional group
def prepare_data(df, functional_group):
    # Debug: Print the unique functional groups available
    print(f"Available functional groups: {df['FUNCTIONAL_GROUP'].unique()}")
    print(f"Looking for functional group: {functional_group}")
    print(f"Records with this functional group: {len(df[df['FUNCTIONAL_GROUP'] == functional_group])}")
    
    # Filter the DataFrame for the specified functional group - simplified filtering
    base_filter = (df['FUNCTIONAL_GROUP'] == functional_group) & (df['LANE_MILES'] > 0)
    
    filtered_df = df[base_filter & 
                     (df['OVERALL_CONDITION'].isin(['GOOD', 'FAIR', 'POOR']))].copy()
    
    # Filter the DataFrame for only MISSING_DATA 
    filtered_df1 = df[base_filter & 
                      (df['OVERALL_CONDITION'] == "MISSING_DATA")].copy()
    
    print(f"Records after filtering for {functional_group}: {len(filtered_df)} + {len(filtered_df1)} (missing)")
        
    # Set the category type with the defined order using .loc
    if not filtered_df.empty:
        filtered_df.loc[:, 'OVERALL_CONDITION'] = pd.Categorical(filtered_df['OVERALL_CONDITION'], categories=condition_order, ordered=True)

        # Group by 'OVERALL_CONDITION' and calculate sum and percentage
        summary_df = filtered_df.groupby('OVERALL_CONDITION', observed=False)['LANE_MILES'].sum().reset_index()
        summary_df = summary_df[summary_df['LANE_MILES'] > 0]  # Remove zero entries
        summary_df['PERCENTAGE'] = (summary_df['LANE_MILES'] / summary_df['LANE_MILES'].sum()) * 100

        # Calculate the subtotals
        subtotal_lane_miles = summary_df['LANE_MILES'].sum()
    else:
        summary_df = pd.DataFrame(columns=['OVERALL_CONDITION', 'LANE_MILES', 'PERCENTAGE'])
        subtotal_lane_miles = 0
    
    # Handle missing data
    if not filtered_df1.empty:
        lane_miles_m = filtered_df1['LANE_MILES'].sum()
        percentage_m = (lane_miles_m / (subtotal_lane_miles + lane_miles_m)) * 100 if (subtotal_lane_miles + lane_miles_m) > 0 else 0
    else:
        lane_miles_m = 0
        percentage_m = 0
    
    summary_df2 = subtotal_lane_miles + lane_miles_m
    
    # Append a totals row to the summary DataFrame
    subtotals_row = pd.DataFrame([['SUBTOTAL', subtotal_lane_miles, 100.0]], columns=['OVERALL_CONDITION', 'LANE_MILES', 'PERCENTAGE'])
    missing_row = pd.DataFrame([['MISSING_DATA', lane_miles_m, percentage_m]], columns=['OVERALL_CONDITION', 'LANE_MILES', 'PERCENTAGE'])
    grand_total = pd.DataFrame([['GRAND TOTAL', summary_df2, 100.0]], columns=['OVERALL_CONDITION', 'LANE_MILES', 'PERCENTAGE'])
    summary_with_missing = pd.concat([summary_df, subtotals_row, missing_row, grand_total], ignore_index=True)
    
    return summary_with_missing

# Check if we have data to process
if not df.empty and 'FUNCTIONAL_GROUP' in df.columns and 'OVERALL_CONDITION' in df.columns:
    # Print debugging information
    print(f"Total records in dataframe: {len(df)}")
    print(f"Records with valid FUNCTIONAL_GROUP: {len(df[df['FUNCTIONAL_GROUP'].notna()])}")
    
    # Prepare data for both functional groups
    interstate_data = prepare_data(df, "Interstate")
    non_interstate_nhs_data = prepare_data(df, "Non-Interstate NHS")

    # Combine the data for both functional groups
    combined_data = interstate_data.copy()
    combined_data['LANE_MILES'] += non_interstate_nhs_data['LANE_MILES']

    # Recalculate the percentages for the combined data
    subtotal_lane_miles = combined_data.loc[combined_data['OVERALL_CONDITION'].isin(['GOOD', 'FAIR', 'POOR']), 'LANE_MILES'].sum()
    if subtotal_lane_miles > 0:
        combined_data.loc[combined_data['OVERALL_CONDITION'].isin(['GOOD', 'FAIR', 'POOR']), 'PERCENTAGE'] = (combined_data['LANE_MILES'] / subtotal_lane_miles) * 100
        combined_data.loc[combined_data['OVERALL_CONDITION'] == 'SUBTOTAL', 'PERCENTAGE'] = 100.0
        grand_total_miles = combined_data.loc[combined_data['OVERALL_CONDITION'] == 'GRAND TOTAL', 'LANE_MILES'].values
        if len(grand_total_miles) > 0 and grand_total_miles[0] > 0:
            combined_data.loc[combined_data['OVERALL_CONDITION'] == 'MISSING_DATA', 'PERCENTAGE'] = (combined_data.loc[combined_data['OVERALL_CONDITION'] == 'MISSING_DATA', 'LANE_MILES'] / grand_total_miles[0]) * 100
        combined_data.loc[combined_data['OVERALL_CONDITION'] == 'GRAND TOTAL', 'PERCENTAGE'] = 100.0

    # Function to plot pie chart for a given functional group
    def plot_pie_chart(data, title):
        fig, ax = plt.subplots(figsize=(8, 8))
        pie_data = data[data['OVERALL_CONDITION'].isin(['GOOD', 'FAIR', 'POOR'])]
        if not pie_data.empty and pie_data['LANE_MILES'].sum() > 0:
            colors = {'GOOD': 'green', 'FAIR': 'yellow', 'POOR': 'red'}
            labels = pie_data['OVERALL_CONDITION']
            sizes = pie_data['LANE_MILES']
            color_list = [colors[condition] for condition in labels]
            
            wedges, texts, autotexts = ax.pie(sizes, labels=labels, colors=color_list, autopct='%1.1f%%', startangle=140)
            ax.axis('equal')  # Equal aspect ratio ensures that pie is drawn as a circle.
            ax.set_title(title, fontsize=16)  # Set a larger font size for the title
        else:
            ax.text(0.5, 0.5, 'No data available', transform=ax.transAxes, ha='center', va='center')
            ax.set_title(title, fontsize=16)
        plt.show()

    # Function to format the summary table for Jupyter Notebook display
    def format_summary_table_for_display(summary_df):
        if summary_df.empty:
            return summary_df
            
        # Format the 'LANE_MILES' column with comma as thousands separator and round to three decimal places
        # Format the 'PERCENTAGE' column to round to two decimal places
        formatted_summary = summary_df.style.format({
            'LANE_MILES': '{:,.3f}',
            'PERCENTAGE': '{:.1f}%'
        })

        # Apply bold font weight to subtotal and grand total rows
        formatted_summary = formatted_summary.apply(lambda x: ['font-weight: bold' if x.name in [len(summary_df) - 3, len(summary_df) - 1] else '' for i in x], axis=1)

        # Apply red font color to MISSING_DATA row
        formatted_summary = formatted_summary.apply(lambda x: ['color: red' if x.name == len(summary_df) - 2 else '' for i in x], axis=1)

        return formatted_summary

    # Plot pie chart and display table for Interstate
    plot_pie_chart(interstate_data, '2024 Full Extent Lane Miles Rating: Interstate')
    display(format_summary_table_for_display(interstate_data))

    # Plot pie chart and display table for Non-Interstate NHS
    print('\n')
    plot_pie_chart(non_interstate_nhs_data, '2024 Full Extent Lane Miles Rating: Non-Interstate NHS')
    display(format_summary_table_for_display(non_interstate_nhs_data))

    # Plot pie chart and display table for Combined Data
    print('\n')
    plot_pie_chart(combined_data, '2024 Full Extent Lane Miles Rating: Combined')
    display(format_summary_table_for_display(combined_data))
else:
    print("No data available for visualization or missing required columns.")
    if df.empty:
        print("- DataFrame is empty")
    if 'FUNCTIONAL_GROUP' not in df.columns:
        print("- FUNCTIONAL_GROUP column is missing")
    if 'OVERALL_CONDITION' not in df.columns:
        print("- OVERALL_CONDITION column is missing")

# Step 3: Export the processed dataframe to CSV
import os

if not all_records_df.empty:
    # Get the directory where the notebook is located
    notebook_dir = os.path.dirname(os.path.abspath("New Notebook.ipynb"))
    
    # Create the output CSV file path
    output_csv_path = os.path.join(notebook_dir, "processed_pavement_data.csv")
    
    # Create a copy without FUNCTIONAL_GROUP for export
    export_df = all_records_df.copy()
    if 'FUNCTIONAL_GROUP' in export_df.columns:
        export_df = export_df.drop('FUNCTIONAL_GROUP', axis=1)
        print(f"Removed FUNCTIONAL_GROUP column for export")
    
    # Format date columns ending in _D as MM/DD/YYYY
    date_columns = [col for col in export_df.columns if col.endswith('_D')]
    if date_columns:
        print(f"Formatting date columns: {date_columns}")
        for col in date_columns:
            if export_df[col].dtype == 'datetime64[ns]':
                export_df[col] = export_df[col].dt.strftime('%m/%d/%Y')
            elif pd.api.types.is_datetime64_any_dtype(export_df[col]):
                export_df[col] = pd.to_datetime(export_df[col], errors='coerce').dt.strftime('%m/%d/%Y')
    
    # Export the dataframe to CSV
    try:
        export_df.to_csv(output_csv_path, index=False)
        print(f"Successfully exported {len(export_df)} records to: {output_csv_path}")
        
        # Display basic statistics about the exported data
        print(f"\nData Summary:")
        print(f"Total records: {len(export_df):,}")
        print(f"Total columns: {len(export_df.columns)}")
        
        if 'LANE_MILES' in export_df.columns:
            print(f"Total lane miles: {export_df['LANE_MILES'].sum():,.3f}")
        
        # Show breakdown by functional group using original dataframe
        if 'FUNCTIONAL_GROUP' in all_records_df.columns and 'LANE_MILES' in all_records_df.columns:
            functional_group_summary = all_records_df.groupby('FUNCTIONAL_GROUP').agg({
                'LANE_MILES': ['count', 'sum']
            }).round(3)
            functional_group_summary.columns = ['Record_Count', 'Total_Lane_Miles']
            print(f"\nBreakdown by Functional Group:")
            print(functional_group_summary)
        
        # Show breakdown by overall condition
        if 'OVERALL_CONDITION' in export_df.columns and 'LANE_MILES' in export_df.columns:
            condition_summary = export_df.groupby('OVERALL_CONDITION').agg({
                'LANE_MILES': ['count', 'sum']
            }).round(3)
            condition_summary.columns = ['Record_Count', 'Total_Lane_Miles']
            print(f"\nBreakdown by Overall Condition:")
            print(condition_summary)
        
        # Display the final column list
        print(f"\nFinal exported columns ({len(export_df.columns)}):")
        for i, col in enumerate(export_df.columns, 1):
            print(f"{i:2d}. {col}")
            
    except Exception as e:
        print(f"Error exporting to CSV: {e}")
else:
    print("No data available to export. Please ensure the pipe-delimited file was loaded successfully.")

- Review the output tables and visualizations generated by the notebook and compare the output with the official PM2 HPMS report card to ensure that the results are accurate and match exactly. This critical quality control step involves detailed examination of statistical summaries, data distributions, and key performance metrics to verify data integrity and processing accuracy.

- Adjust any necessary modifications to the code or data based on the review findings and re-run the notebook as needed. If discrepancies are identified during the comparison with official report cards, investigate the root causes and implement appropriate corrections to ensure data accuracy and consistency.

- Import the file generated by the notebook called processed_pavement_data.csv into Oracle database. This file contains the cleaned and validated PM2 data in the proper format for database integration and subsequent analytical operations.

- Connect to Oracle ASSETDEV instance and run the following statement truncate table hpms_pm2_intersections_tmp to clear the existing data from the temporary table. This preparatory step ensures that the temporary table is empty and ready to receive the new processed data without conflicts or data duplication issues.

- Import the csv file into the temporary table in Oracle using appropriate data loading procedures. Map fields by name to ensure proper data alignment between the CSV file structure and the Oracle table schema. Verify that all data types and field mappings are correct before proceeding with the data loading operation.

- Execute the following SQL statement to insert the data from the temporary table into the main hpms_pm2_intersections table:

INSERT INTO hpms_pm2_intersections (
    year_record,
    state_code,
    non_interstate_nhs,
    route_id,
    begin_point,
    end_point,
    f_system,
    facility_type,
    through_lanes,
    dir_through_lanes,
    lanes_to_compute_lanemiles,
    urban_code,
    nhs,
    structure_type,
    surface_type,
    iri,
    iri_d,
    iri_t,
    psr,
    psr_d,
    psr_t,
    rutting,
    rutting_d,
    rutting_t,
    faulting,
    faulting_d,
    faulting_t,
    cracking_percent,
    cracking_percent_d,
    cracking_percent_t,
    f_system_begin_point,
    f_system_end_point,
    facility_type_begin_point,
    facility_type_end_point,
    through_lanes_begin_point,
    through_lanes_end_point,
    dir_through_lanes_begin_point,
    dir_through_lanes_end_point,
    urban_code_begin_point,
    urban_code_end_point,
    structure_type_begin_point,
    structure_type_end_point,
    surface_type_begin_point,
    surface_type_end_point,
    iri_begin_point,
    iri_end_point,
    psr_begin_point,
    psr_end_point,
    rutting_begin_point,
    rutting_end_point,
    faulting_begin_point,
    faulting_end_point,
    cracking_percent_begin_point,
    cracking_percent_end_point,
    last_modified_by,
    last_modified_on,
    segment_length,
    lane_miles,
    iri_sl,
    psr_sl,
    rutting_sl,
    faulting_sl,
    cracking_percent_sl,
    rutting_sc,
    faulting_sc,
    cracking_percent_sc,
    bridge,
    unpaved_or_other,
    missing_invalid_or_unresolved,
    segment_rate,
    iri_condition,
    rutting_condition,
    cracking_condition,
    faulting_condition,
    overall_condition,
    surface_material,
    miu_iri,
    segment_rate_iri
) select * from hpms_pm2_intersections_tmp;
commit;

After successful execution of the INSERT statement, commit the transaction to ensure that all data changes are permanently saved to the database and the PM2 dataset processing is fully completed.

## NHS Pavement Condition Dataset

### General

The NHS Pavement Condition Dataset is a comprehensive data processing and analysis procedure that extracts, filters, and prepares pavement condition data for Federal reporting requirements. This process involves extracting data from the final HPMS PM2 dataset, applying specific filtering criteria, and processing the data through ArcGIS Pro using Python scripts to create a spatially-enabled dataset with Metropolitan Planning Organization (MPO) assignments. The procedure ensures that all NHS pavement condition data is properly classified, validated, and formatted according to Federal standards for submission to regulatory agencies.

The process begins with HPMS condition data extraction and progresses through multiple data transformation steps including spatial analysis, field mapping, domain assignment, and metadata preparation. All intermediate processing steps are documented and archived to maintain data lineage and support quality assurance requirements. The final output includes both the processed spatial dataset and comprehensive data dictionary documentation.

### Frequency

This procedure is performed annually to support Federal reporting requirements and maintain current pavement condition assessments for National Highway System roadways.

### Purpose

The NHS Pavement Condition Dataset serves multiple critical purposes in transportation asset management and Federal compliance. The primary objective is to provide accurate, spatially-referenced pavement condition data for National Highway System roadways that meets Federal Highway Performance Monitoring System requirements. This data supports Federal Transportation Asset Management Plan (TAMP) reporting obligations and enables performance monitoring of NHS infrastructure investments.

The dataset facilitates compliance with Federal reporting mandates while providing essential data for strategic planning, performance measurement, and resource allocation decisions. The spatial integration with Metropolitan Planning Organization boundaries enables regional analysis and supports coordination between state and regional transportation planning efforts. Additionally, the standardized data format and comprehensive documentation ensure data quality, accessibility, and long-term usability for transportation professionals and decision-makers.

### Resource Requirements

The following personnel and resources are necessary to successfully generate the NHS Pavement Condition Dataset:

| Position | Function | Resources | Effort Level | Additional Details |
| --- | --- | --- | --- | --- |
| GIS Analyst | Data Processing and Analysis | ArcGIS Pro License, Oracle Database Access, Python Environment | 3-5 days | Requires expertise in spatial analysis, database management, and Python scripting |
| Database Administrator | Data Extraction and Validation | Oracle SQL Developer, HPMS Database Access | 1-2 days | Responsible for initial data extraction and database operations |
| Transportation Analyst | Quality Assurance and Review | Domain Knowledge of NHS Classifications | 1 day | Reviews data for accuracy and compliance with transportation standards |

Essential software and system requirements include ArcGIS Pro with Spatial Analyst extension, Oracle database connectivity, Python scripting environment, and access to the ASSET_NEW database system. Hardware requirements include sufficient processing power and memory to handle large spatial datasets, typically requiring 16GB RAM minimum and solid-state storage for optimal performance.

### Procedure

The NHS Pavement Condition Dataset generation process involves systematic data extraction, transformation, and spatial analysis to produce a comprehensive pavement condition dataset for National Highway System roadways. The procedure encompasses database operations, GIS processing, Python scripting, and quality assurance validation to ensure data accuracy and compliance with Federal reporting standards. All steps must be completed sequentially to maintain data integrity and produce reliable results suitable for Federal submission.

The process requires careful attention to spatial relationships, data validation, and quality control measures throughout each phase. Proper documentation and archival of intermediate products ensures traceability and supports future updates or revisions to the dataset.

#### Pre-Requisites

The latest NHS pavement condition condition data must be extracted and made available for processing in the Oracle table HPMS_PM2_INTERSECTIONS in the ASSETDEV schema. The steps to process the PM2 data are detailed in Process PM2 Dataset section above. The PM2 data must be processed and validated before proceeding with the NHS Pavement Condition Dataset generation.

#### ArcGIS Pro Project Setup and Data Processing

- Create a new ArcGIS Pro Map Project named HPMS_PM2_INTERSECTIONS, using the previous project as a template from the Z:\Aditya\GIS Data\ArcGIS Pro Projects\NHS Pavement Condition Data\HPMS_PM2_INTERSECTIONS (NEW) directory for consistency and efficiency.

- Execute the Python code sequence below, ensuring to replace 2024 with the most recent year of condition data and update the folder path C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW) to match your project directory before running each code cell:

# Cell 1
# Import necessary libraries.
import oracledb
import pandas as pd

# Connect to the Oracle database using host and port
connection = oracledb.connect(user='pav_conhist', 
                              password='conhist1pav', 
                              dsn='ASSET_NEW')
# Define the SQL query
sql_query = """SELECT distinct 
    LU_GOVT_CONTROL.desc1 as GOVT_CONTROL_DESC1,    
    county.county_alpha as CO_NAME, 
    LU_MUN.MUN_NAME as MUN_NAME,
    PM2.ROUTE_ID as ROUTEID,
    CASE
      WHEN PM2.F_SYSTEM is NULL 
      THEN NULL
      WHEN PM2.F_SYSTEM = 1
      THEN 'Interstate'
      ELSE 'Non-Interstate NHS'
    END AS FUNCTIONAL_GROUP,
    PM2.BEGIN_POINT as BMP,
    PM2.END_POINT as EMP,
    case when PM2.Missing_Invalid_Or_Unresolved = 1 
     THEN 'MISSING_INVALID_UNRESOLVED' 
     else PM2.OVERALL_CONDITION end OVERALL_PAVEMENT_COND_FHWA,
    PM2.SURFACE_TYPE,
    PM2.SURFACE_MATERIAL,
    PM2.LANE_MILES,
    PM2.IRI,
    PM2.RUTTING,
    PM2.CRACKING_PERCENT,
    PM2.FAULTING,
    PM2.IRI_CONDITION as IRI_CONDITION,
    PM2.RUTTING_CONDITION as RUTTING_CONDITION,
    PM2.CRACKING_CONDITION as CRACKING_CONDITION,
    PM2.FAULTING_CONDITION as FAULTING_CONDITION,
    UNIV.DISTRICT,
    UNIV.ROUTE_NAME
    from HPMS_PM2_INTERSECTIONS@pav_Conhist_assetdev PM2 left join 
    (SELECT to_number(COUNTY) as county,
          ROUTE_ID,
          from_measure as BMP_UNIV,
          to_measure EMP_UNIV,
          FUNCTIONAL_CLASS, RURAL_URBAN,
          to_number(nhs) as NHS_CODE,
          to_number(ownership) as GOVT_CONTROL,
          SECTION_LENGTH,
          FACILITY_TYPE,
          to_number(MUN_SORT) as MUN_SORT,
          to_number(municipal_loc) as municipal_loc,
          MAIN_LINE,
          DISTRICT,
          trim(prefix) || ' ' || to_number(rte_no) || case when suffix = '--' then null else ' ' || replace(suffix,'-','') end ROUTE_NAME
        FROM archive.univ2024@hmis_old.prod.hisd.sha
    ) UNIV
    on PM2.route_id = UNIV.ROUTE_ID 
    AND PM2.BEGIN_POINT                     >= UNIV.BMP_UNIV
    AND PM2.BEGIN_POINT                     < UNIV.EMP_UNIV 

    left join
    (SELECT govt_control,
          abbrev,
          desc1,
          hpms_sub_code,
          hmis_abbrev
          -- ,status
        FROM lu_govt_control
    ) lu_govt_control

    on UNIV.govt_control = lu_govt_control.GOVT_CONTROL

    left join

    (SELECT MUN_SORT,
      MUN_NAME,
      HPMS_SUB_CODE,
      ABBREV,
      COUNTY,
      CO_NAME
    FROM LU_MUN@hmis_old.prod.hisd.sha
    ) LU_MUN    

    on LU_MUN.COUNTY                = UNIV.COUNTY
    AND LU_MUN.MUN_SORT              = UNIV.municipal_loc

    left join county
    on county.code = to_number(substr(PM2.route_id,1,2))

    where PM2.YEAR_RECORD = 2024
    and (PM2.Structure_Type is null or pm2.structure_type = 2)
    order by 4,6"""

# Execute the SQL query and fetch the results into a DataFrame
df = pd.read_sql(sql_query, connection)

# Close the database connection
connection.close()

# Export the DataFrame to CSV
df.to_csv(r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\PM2_FULL_EXTENT_DATA_ORCL.csv', index=False)

# Display the DataFrame
df.head()

# Cell 2
# Import the PM2 full extent pavement condition csv file into the GDB as a table
import arcpy

in_table_path = r"C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\PM2_FULL_EXTENT_DATA_ORCL.csv"
out_table_path = r"C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb\PM2_FULL_EXTENT_DATA"

field_mapping = arcpy.FieldMappings()
field_mapping.addTable(in_table_path)

# Define field mappings with data types
field_mappings = [
    ("GOVT_CONTROL_DESC1", "Text", 8000),
    ("CO_NAME", "Text", 8000),
    ("MUN_NAME", "Text", 8000),
    ("ROUTEID", "Text", 8000),
    ("FUNCTIONAL_GROUP", "Text", 8000),
    ("BMP", "Double", 8),
    ("EMP", "Double", 8),
    ("OVERALL_PAVEMENT_COND_FHWA", "Text", 8000),
    ("SURFACE_TYPE", "Long", 8),
    ("SURFACE_MATERIAL", "Text", 8000),
    ("LANE_MILES", "Double", 8),
    ("IRI", "Long", 8),
    ("RUTTING", "Double", 8),
    ("CRACKING_PERCENT", "Long", 8),
    ("FAULTING", "Double", 8000),
    ("IRI_CONDITION", "Text", 8000),
    ("RUTTING_CONDITION", "Text", 8000),
    ("CRACKING_CONDITION", "Text", 8000),
    ("FAULTING_CONDITION", "Text", 8000),
    ("ROUTE_NAME", "Text", 8000),
    ("DISTRICT", "Long", 4)
]

# Update field mappings with data types
for field_name, field_type, field_length in field_mappings:
    field_index = field_mapping.findFieldMapIndex(field_name)
    field_map = field_mapping.getFieldMap(field_index)
    field = field_map.outputField
    field.type = field_type
    if field_type == "Text":
        field.length = field_length
    elif field_type == "Double" or field_type == "Long":
        field.precision = field_length
    field_map.outputField = field
    field_mapping.replaceFieldMap(field_index, field_map)

arcpy.conversion.ExportTable(
    in_table=in_table_path,
    out_table=out_table_path,
    where_clause="",
    use_field_alias_as_name="NOT_USE_ALIAS",
    field_mapping=field_mapping,
    sort_field=None
)

# Cell 3
# Create route events for the PM2 pavement condition data
arcpy.lr.MakeRouteEventLayer(
    in_routes="MDOT_SHA_Roadway_Routes",
    route_id_field="ROUTEID_RH",
    in_table="PM2_FULL_EXTENT_DATA",
    in_event_properties="ROUTEID; LINE; BMP; EMP",
    out_layer="PM2_FULL_EXTENT_DATA_EVENTS",
    offset_field=None,
    add_error_field="ERROR_FIELD",
    add_angle_field="NO_ANGLE_FIELD",
    angle_type="NORMAL",
    complement_angle="ANGLE",
    offset_direction="LEFT",
    point_event_type="POINT"
)

Note: The spatial join step below may take about 5-10 minutes to complete in ArcGIS Pro. Save the ArcGIS Pro project after successful completion of this step. This may take an additional 10-15 minutes. ArcGIS Pro may become unresponsive during this time.

# cell 4
# Spatial Join MPO layer with PM2 dataset to add MPO Name and Acronym fields to the PM2 dataset 
arcpy.analysis.SpatialJoin(
    target_features="PM2_FULL_EXTENT_DATA_EVENTS",
    join_features="BSEC_MetropolitanPlanningOrganizations_MDOT",
    out_feature_class=r"C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb\PM2_FULL_EXTENT_MPO_JOINED",
    join_operation="JOIN_ONE_TO_ONE",
    join_type="KEEP_ALL",
    field_mapping='''\
        GOVT_CONTROL_DESC1 "GOVT_CONTROL_DESC1" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,GOVT_CONTROL_DESC1,0,8000;
        CO_NAME "CO_NAME" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,CO_NAME,0,8000;
        MUN_NAME "MUN_NAME" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,MUN_NAME,0,8000;
        ROUTEID "ROUTEID" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,ROUTEID,0,8000;
        FUNCTIONAL_GROUP "FUNCTIONAL_GROUP" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,FUNCTIONAL_GROUP,0,8000;
        BMP "BMP" true true false 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,BMP,-1,-1;
        EMP "EMP" true true false 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,EMP,-1,-1;
        OVERALL_PAVEMENT_COND_FHWA "OVERALL_PAVEMENT_COND_FHWA" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,OVERALL_PAVEMENT_COND_FHWA,0,8000;
        SURFACE_TYPE "SURFACE_TYPE" true true false 4 Long 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,SURFACE_TYPE,-1,-1;
        SURFACE_MATERIAL "SURFACE_MATERIAL" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,SURFACE_MATERIAL,0,8000;
        LANE_MILES "LANE_MILES" true true false 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,LANE_MILES,-1,-1;
        IRI "IRI" true true false 4 Long 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,IRI,-1,-1;
        RUTTING "RUTTING" true true false 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,RUTTING,-1,-1;
        CRACKING_PERCENT "CRACKING_PERCENT" true true false 4 Long 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,CRACKING_PERCENT,-1,-1;
        FAULTING "FAULTING" true true false 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,FAULTING,-1,-1;
        IRI_CONDITION "IRI_CONDITION" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,IRI_CONDITION,0,8000;
        RUTTING_CONDITION "RUTTING_CONDITION" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,RUTTING_CONDITION,0,8000;
        CRACKING_CONDITION "CRACKING_CONDITION" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,CRACKING_CONDITION,0,8000;
        FAULTING_CONDITION "FAULTING_CONDITION" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,FAULTING_CONDITION,0,8000;
        ROUTE_NAME "ROUTE_NAME" true true false 8000 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,ROUTE_NAME,0,8000;
        DISTRICT "DISTRICT" true true false 4 Long 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,DISTRICT,-1,-1;
        LOC_ERROR "LOC_ERROR" true true false 50 Text 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,LOC_ERROR,0,50;
        Shape_Length "Shape_Length" false true true 8 Double 0 0,First,#,PM2_FULL_EXTENT_DATA_EVENTS,Shape_Length,-1,-1;
        MPO_NAME "MPO Name" true true false 80 Text 0 0,First,#,BSEC_MetropolitanPlanningOrganizations_MDOT,MPO_NAME,0,80;
        ACRONYM "Acronym" true true false 10 Text 0 0,First,#,BSEC_MetropolitanPlanningOrganizations_MDOT,ACRONYM,0,10''',
    match_option="CLOSEST",
    search_radius=None,
    distance_field_name=""
)

#### MPO Assignment and Data Validation

Execute the following cells sequentially to properly assign Metropolitan Planning Organization data:

Note: Configure ArcGIS Pro settings by unchecking Add output datasets to an open map in the Project Menu > Geoprocessing settings before proceeding with data processing operations. Restart ArcGIS Pro after making this change to ensure the settings take effect.

# Cell 5
# Update MPO_NAME and ACRONYM to null for Caroline, Dorchester, Garrett, Kent, Somerset, Talbot, and Worcester counties.
# Uncheck "Add output datasets to an open map" in the Project Menu --> Geoprocessing settings before proceeding
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME IN ('Caroline', 'Dorchester', 'Garrett', 'Kent', 'Somerset', 'Talbot', 'Worcester')"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = None  # Set MPO_NAME to null
            row[1] = None  # Set ACRONYM to null
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to null.")
else:
    print("No records found matching the SQL criteria.")

# Cell 6
# Update MPO_NAME and ACRONYM to Cumberland Area MPO and CAMPO for Allegany County.
# import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME = 'Allegany'"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = 'Cumberland Area MPO'  # Set MPO_NAME to Cumberland Area MPO
            row[1] = 'CAMPO'  # Set ACRONYM to CAMPO
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to Cumberland Area MPO and CAMPO for Allegany County.")
else:
    print(f"No records found matching the SQL criteria: {sql_expression}")

# Cell 7        
# Update MPO_NAME and ACRONYM to Hagerstown-Eastern Panhandle MPO and HEPMPO for Washington County.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME = 'Washington'"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = 'Hagerstown-Eastern Panhandle MPO'  # Set MPO_NAME to Hagerstown-Eastern Panhandle MPO
            row[1] = 'HEPMPO'  # Set ACRONYM to HEPMPO
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to Hagerstown-Eastern Panhandle MPO and HEPMPO for Washington County.")
else:
    print(f"No records found matching the SQL criteria: {sql_expression}")

# Cell 8
# Update MPO_NAME and ACRONYM to Wilmington Area Planning Council and WILMAPCO for Cecil County.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME = 'Cecil'"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = 'Wilmington Area Planning Council'  # Set MPO_NAME to Wilmington Area Planning Council
            row[1] = 'WILMAPCO'  # Set ACRONYM to WILMAPCO
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to Wilmington Area Planning Council and WILMAPCO for Cecil County.")
else:
    print("No records found matching the SQL criteria.")

# Cell 9
# Update MPO_NAME and ACRONYM to NCR Transp Planning Board and TPB for Charles, Frederick, Montgomery, and Prince Georges counties.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME in ('Charles', 'Frederick', 'Montgomery', 'Prince Georges')"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = 'National Capital Region Transportation Planning Board'  # Set MPO_NAME to NCR Transp Planning Board
            row[1] = 'TPB'  # Set ACRONYM to TPB
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to NCR Transp Planning Board and TPB for CH, FR, MO, and PG counties.")
else:
    print("No records found matching the SQL criteria.")

# Cell 10
# Update MPO_NAME and ACRONYM to Baltimore Regional Transportation Board and BRTB for Anne Arundel, Baltimore, Baltimore City, Carroll, Harford, and Howard counties.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME in ('Anne Arundel', 'Baltimore', 'Baltimore City', 'Carroll', 'Harford', 'Howard')"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = 'Baltimore Regional Transportation Board'  # Set MPO_NAME to Baltimore Regional Transportation Board
            row[1] = 'BRTB'  # Set ACRONYM to BRTB
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to Baltimore Regional Transportation Board and BRTB for AA, BA, BC, CL, HA, and HO counties.")
else:
    print("No records found matching the SQL criteria.")

#### Manual Quality Assurance and Validation

Execute the following validation cells sequentially, reviewing selected sections before running each code block:

# Cell 11
# Start manual checks in the document. 
# Check if Cumberland Area MPO has any records other than Allegany County. If so, update MPO_NAME and ACRONYM to null for those records.
# ****REVIEW SELECTED SECTIONS AND ONLY THEN RUN THIS CODE BLOCK! ****
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Define the feature layer
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Define the SQL criteria
sql_expression = "CO_NAME <> 'Allegany' And MPO_NAME = 'Cumberland Area MPO'"

# Select features based on the SQL criteria
arcpy.MakeFeatureLayer_management(feature_layer, "selected_features", sql_expression)

# Get the count of selected records
record_count = int(arcpy.GetCount_management("selected_features").getOutput(0))

if record_count > 0:
    # Update fields MPO_NAME and ACRONYM to null for the selected records
    with arcpy.da.UpdateCursor("selected_features", ["MPO_NAME", "ACRONYM"]) as cursor:
        for row in cursor:
            row[0] = None  # Set MPO_NAME to null
            row[1] = None  # Set ACRONYM to null
            cursor.updateRow(row)
    
    print(f"[{timestamp}] {record_count} records updated successfully. Updated MPO_NAME and ACRONYM to null.")
else:
    print(f"[{timestamp}] No records found matching the SQL criteria: {sql_expression}")

Continue with cells 12-24 for additional validation checks, including Hagerstown-Eastern Panhandle MPO, Wilmington Area Planning Council, National Capital Region Transportation Planning Board, Baltimore Regional Transportation Board, and various route-specific validations for Queen Anne’s County sections, bay bridge sections, and other boundary adjustments.

# Cell 25
# Final checks in the word document. All nine checks in this code block must return zero records.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set up your connection details
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

# Pair each query with a check description
queries = [
    ("CO_NAME IN ('Caroline', 'Dorchester', 'Garrett', 'Kent', 'Somerset', 'Talbot', 'Worcester') AND MPO_NAME IS NOT NULL", "Check 1"),
    ("MPO_NAME = 'Cumberland Area MPO' AND CO_NAME <> 'Allegany'", "Check 2"),
    ("MPO_NAME = 'Hagerstown-Eastern Panhandle MPO' AND CO_NAME <> 'Washington'", "Check 3"),
    ("MPO_NAME = 'Wilmington Area Planning Council' AND CO_NAME <> 'Cecil'", "Check 4"),
    ("MPO_NAME = 'National Capital Region Transportation Planning Board' AND CO_NAME NOT IN ('Charles', 'Frederick', 'Montgomery', 'Prince Georges')", "Check 5"),
    ("MPO_NAME = 'Baltimore Regional Transportation Board' AND CO_NAME NOT IN ('Queen Anne''s', 'Anne Arundel', 'Baltimore', 'Baltimore City', 'Carroll', 'Harford', 'Howard')", "Check 6"),
    ("MPO_NAME = 'Calvert - St. Mary''s Metropolitan Planning Organization' AND CO_NAME NOT IN ('Calvert', 'Saint Marys')", "Check 7"),
    ("MPO_NAME = 'Salisbury-Wicomico MPO' AND CO_NAME <> 'Wicomico'", "Check 8"),
    ("MPO_NAME is not null and CO_Name in ('Caroline','Dorchester','Garrett','Kent','Somerset','Talbot','Worcester')", "Check 9")
]

try:
    # Execute each query and print the results
    for query, check in queries:
        # Make a temporary table view for each query
        table_view = arcpy.management.MakeTableView(feature_layer, "tempView", query)
        result = arcpy.management.GetCount(table_view)
        record_count = int(result[0])
        print(f"[{timestamp}] {check}: {record_count} records found")
        # Delete the temporary view
        arcpy.management.Delete(table_view)

except Exception as e:
    print(f"[{timestamp}] Error: {str(e)}")

#### Final Data Processing and Documentation

# Cell 26
# Update field Aliases
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

fc = "PM2_FULL_EXTENT_MPO_JOINED"

fields_to_alter = [
    ("GOVT_CONTROL_DESC1", "Maintenance Responsibility"),
    ("CO_NAME", "County Name"),
    ("MUN_NAME", "Municipality Name"),
    ("ROUTEID", "Route ID"),
    ("FUNCTIONAL_GROUP", "Functional Group"),
    ("BMP", "Beginning Milepoint"),
    ("EMP", "Ending Milepoint"),
    ("OVERALL_PAVEMENT_COND_FHWA", "Overall Pavement Condition FHWA"),
    ("SURFACE_TYPE", "Surface Type"),
    ("SURFACE_MATERIAL", "Surface Material"),
    ("LANE_MILES", "Lane Miles"),
    ("IRI", "IRI (in/mi)"),
    ("RUTTING", "Rutting (in)"),
    ("CRACKING_PERCENT", "Cracking Percent"),
    ("FAULTING", "Faulting (in)"),
    ("IRI_CONDITION", "IRI Condition"),
    ("RUTTING_CONDITION", "Rutting Condition"),
    ("CRACKING_CONDITION", "Cracking Condition"),
    ("FAULTING_CONDITION", "Faulting Condition"),
    ("LOC_ERROR", "Location Error"),
    ("MPO_NAME", "Metropolitan Planning Organization Name"),
    ("ACRONYM", "MPO Acronym"),
    ("ROUTE_NAME", "Route Name"),
    ("DISTRICT", "SHA District Number")
]

for field, new_alias in fields_to_alter:
    arcpy.AlterField_management(fc, field, field, new_alias)

print(f"[{timestamp}] Field aliases updated successfully!")

# Cell 27
# Assign domains to fields SURFACE_TYPE and SURFACE_MATERIAL
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Define a dictionary of feature classes, fields, and their corresponding domains
fieldDomains = {
    "PM2_FULL_EXTENT_MPO_JOINED": {
        "SURFACE_TYPE": "Surface_Type_Codes",
        "SURFACE_MATERIAL": "Surface_Material_Codes"
    }
}

# Loop through each feature class and its associated fields
for fc, field_dict in fieldDomains.items():
    for field_name, domain_name in field_dict.items():
        try:
            arcpy.management.AssignDomainToField(
                in_table=fc,
                field_name=field_name,
                domain_name=domain_name,
                subtype_code=None
            )
            print(f"[{timestamp}] Assigned domain '{domain_name}' to field '{field_name}' successfully!")
        except arcpy.ExecuteError as e:
            print(f"[{timestamp}] Error assigning domain to field '{field_name}': {e}")

# Cell 28
# Delete columns Join_Count and TARGET_FID
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Feature layer name
feature_layer = "PM2_FULL_EXTENT_MPO_JOINED"

try:
    # Disable editor tracking to allow field deletion
    arcpy.DisableEditorTracking_management(feature_layer)

    # List all fields in the feature layer
    fields = arcpy.ListFields(feature_layer)

    # List of fields to delete
    fields_to_delete = ["Join_Count", "TARGET_FID"]

    # Loop through fields and delete the specified ones
    for field in fields:
        if field.name in fields_to_delete:
            arcpy.management.DeleteField(feature_layer, field.name)

    print(f"[{timestamp}] Columns deleted successfully.")

except arcpy.ExecuteError:
    print(arcpy.GetMessages())
except Exception as e:
    print(e)

# Cell 29
# Rename the feature layer to "Pavement_Condition_NHS" in the GDB and current map.
import arcpy
from datetime import datetime

timestamp = datetime.now().strftime("%m/%d/%Y %H:%M:%S")

# Set the workspace
arcpy.env.workspace = r'C:\Users\aramachandran\Documents\ArcGIS\Projects\HPMS_PM2_INTERSECTIONS (NEW)\HPMS_PM2_INTERSECTIONS.gdb'

# Current and new feature layer names
current_name = "PM2_FULL_EXTENT_MPO_JOINED"
new_name = "Pavement_Condition_NHS"

# Rename the feature layer in the geodatabase
arcpy.management.Rename(current_name, new_name)

# Update the name of the feature layer on the map
project = arcpy.mp.ArcGISProject("CURRENT")  # Assumes the project is currently open

# Find the map by name
map_name = "Map"  # Adjust if your map has a different name
map_obj = project.listMaps(map_name)[0]

# Find the layer by name
for layer in map_obj.listLayers():
    if layer.name == current_name:
        # Update the layer's name
        layer.name = new_name

# Save the project
project.save()

print(f"[{timestamp}] Feature layer renamed to 'Pavement_Condition_NHS' and updated on the map.")

#### Update Metadata and Final Deliverables

- Import metadata from the previous submission by selecting the current Pavement_Condition_NHS layer in Catalog view and clicking Import Metadata. Select the XML file, choose “ArcGIS Metadata” for metadata type, and click OK to complete the import process.

- Update metadata by selecting the current Pavement_Condition_NHS layer and clicking Edit Metadata. Update the year to the latest year in YYYY format and click Save to finalize the metadata updates.

- Create a copy of the geodatabase and rename it to Pavement_Condition_NHS.gdb in the same project folder. Remove all intermediate layers and tables except for the Pavement_Condition_NHS layer to create a clean final product.

- Archive the complete ArcGIS Pro project folder, including the final submission geodatabase and metadata XML file, under the previous submission folder as a new directory named YYYY where YYYY represents the submission year.

- Prepare the data dictionary documentation by including the excel file Data dictionary_wip_05282024.xlsx with the final submission geodatabase at \\shahqhisdfs1\OMT\NHSPavementConditionCurrent\. This file contains two critical worksheets:

  - Data_Dictionary: Field names, aliases, data types, and descriptions for the Pavement_Condition_NHS layer

  - Domains: Domain names and descriptions used in the Pavement_Condition_NHS layer

- The Data Analysis Team Leader will email the network folder link that contains the final submission package to the SHA Service Desk and GIS Team. Use the template below and replace the placeholders before sending.

- Subject: MDOT SHA Pavement Condition (NHS) Dataset Update
To: SHA Service Desk SDesk@mdot.maryland.gov
Cc: SHA GIS gis@mdot.maryland.gov; additional recipients (optional)

- Please route this service request to the SHA GIS Team.

- OMT has finished processing the YYYY NHS Pavement Condition dataset and copied the following files to \\shahqhisdfs1\gis_staging\OMT\NHSPavementConditionDataCurrent\ :

  - Pavement_Condition_NHS.gdb

  - Data_dictionary_{DATE}.xlsx

- This is an update to the AGOL hosted layer (latest data: CY YYYY):
https://maryland.maps.arcgis.com/home/item.html?id=ef39f7453467446cbf4b95add26db4a4

- Please let me know if you have any questions or need anything else.

- Thanks,
{Your Name}
{Your Role}
{Contact Info}

### Troubleshooting

| Issue | Possible Cause | Solution |
| --- | --- | --- |
| Oracle database connection failure | Database credentials or network connectivity | Verify credentials and network access to ASSET_NEW database |
| ArcGIS Pro memory errors during processing | Insufficient system resources | Close other applications, move project to C: drive, increase virtual memory |
| Python script execution errors | Missing libraries or incorrect paths | Verify oracledb and pandas libraries are installed, check file paths |
| Spatial join operation failures | Missing or corrupted spatial reference | Verify coordinate systems match between datasets |
| MPO assignment validation errors | Incorrect county boundaries or route specifications | Review county names and route identifiers for accuracy |
| Field mapping inconsistencies | Data type mismatches or field length issues | Verify field specifications match source data requirements |
| Metadata import failures | Incompatible metadata formats | Ensure metadata XML file is in ArcGIS format |

Note: Ensure all intermediate processing steps are documented and validated before proceeding to the next phase. The accuracy of MPO assignments depends on careful review of spatial relationships and manual validation of boundary conditions. Always verify that final validation checks return zero records before completing the process.

### References

- Highway Performance Monitoring System (HPMS) Data Access Portal

- ArcGIS Pro Documentation for Linear Referencing and Spatial Analysis

- MDOT SHA Asset Management Database (ASSET_NEW) Schema Documentation

- Federal Highway Administration NHS Reporting Requirements

- Metropolitan Planning Organization Boundary Specifications

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-May-30 | Aditya R. | Updated Production of HPMS Sample Table section with comprehensive procedures and validation steps |
| 2.1.0 | 2025-Jun-02 | Aditya R. | Added Production of HPMS Pavement Metadata file section with detailed procedures for annual metadata file updates |
| 2.2.0 | 2025-Jun-15 | Aditya R. | Added Production of HPMS Submission Comments section with procedures for generating standardized validation responses |
| 2.3.0 | 2025-Aug-18 | Aditya R. | Added Process PM2 Dataset and NHS Pavement Condition sections with comprehensive SOP |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and DWT with DAT |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Trend Analysis

Click to go to Updating IRI Performance Model

Click to go to Updating Cracking Performance Models

Click to go to Updating Rutting Performance Model

Click to go to Updating Faulting Performance Model

Click to go to Updating Friction Performance Model

## Introduction

This section describes the set of standard operating procedures (SOP) conducted mainly by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to perform the annual update the performance models for the pavement condition metrics. This section is divided into five SOPs: updating the IRI performance model, updating the cracking performance models, updating the rutting performance model, updating the faulting performance model, and updating the friction performance model.

Figure 12 shows a flowchart of the Trend Analysis and Modeling SOPs.

Figure 12: Trend Analysis and Modeling SOPs Flowchart

## Updating IRI Performance Model

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to update the International Roughness Index (IRI) performance model parameters as new IRI data are collected and processed. The IRI performance model follows the exponential function shown in the equation below, which was developed from the analysis of historical SHA IRI data.06/12/2019

IRIest=IRIin⋅μ1expμ2⋅AGE

Where:
IRIest: Estimated IRI value of pavement at age AGE, in in/mile
IRIin: Initial pavement IRI value, in in/mile
AGE: Age of pavement, in years
μ1: Parameter capturing the effect of the last treatment
μ2: Parameter capturing the effect of surface type, region, and functional class.

The equation parameters μ1 and μ2 are typically updated yearly to account for possible changes in IRI trends resulting from the most recent IRI data. The current values of μ1 and μ2 are listed in IRI Performance Model, while a summary of the most recent average yearly IRI percent changes as a function of pavement family is also provided.

### Frequency

The update of IRI model parameters is typically performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Buiness Plan Tables).

### Purpose

The purpose of this SOP is to update the IRI performance model parameters to incorporate the most recently collected and processed IRI data.

### Resource Requirements

The updating of the IRI performance models involves the following three SHA staff members: (1) a database management expert from the DAT to update model parameters, (2) a database management expert from the DAT to validate the estimated models, and (3) a supervisor – typically the Assistant Division Chief (ADC) – who, as required, provides guidance and decision-making. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the update of the IRI performance model parameters. The estimate assumes no issues are encountered during the update process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Model Estimation | 1 | 16.0 |
| DAT Staff | Model Validation | 1 | 40.0 or as needed |
| ADC | Supervisor | 1 | As required06/12/2019 |

### Procedure

The procedure required to update the IRI performance models is comprised of the following two tasks:

- estimation of new IRI model parameters, and

- QA new IRI model parameters.

1- Estimation of New IRI Model Parameters – performed by DAT staff

Summary: Under this task, the most recently collected and processed IRI data are used to estimate new model parameters.

- In Oracle SQL Developer, run package “IRI_MODELS” to estimate new model parameters for each pavement family. Part of this package includes the removal of outliers from the collected IRI data. See Outlier Review - Outlier Codes for details regarding the outlier review and methodology for identifying outliers in the IRI data.

- Once the IRI model parameters for all families have been updated, inform DAT that parameters are ready for QA.

2- QA of New IRI Model Parameters – performed by DAT Staff

Summary: Under this task, QA checks are used to confirm that the newly estimated IRI model parameters produce reasonable trends. The QA checks are performed using an existing MS Excel QA workbook template.

- Open the most recent MS Excel QA workbook template06/12/2019 (sort by date modified) in the IRI models working folder.

- Copy and paste the family slopes from table “trend_iri_family_slopes” (located in Oracle SQL Developer as an output from the previous task) into the “slope” column (E) in the worksheet “Final Groups for IRI”.

- Plot the IRI model function over 100 years for each pavement family in any program of your choice (Excel, Python, etc.) to confirm that the trend looks reasonable. Refer to IRI Performance Model for the most recent average yearly IRI percent change for each pavement family.

IRI values are expected to deteriorate (i.e., increase) at about 2.4% per year on average, with some families deteriorating faster and others slower. Most families should reach an IRI value of 170 in/mile – lowest SHA IRI trigger value (IRIRSL0) corresponding to the Interstates functional class – within 100 years assuming an initial IRI value (immediately after last treatment) of 40 in/mile. Exceptions include reinforced concrete pavements and composite pavements with continuously reinforced concrete below the surface layer, which appear to deteriorate at a slower rate. The table below shows the IRIRSL_0 trigger values for the functional classes.

| FUNC_CL | FC_DESCRIPTION | IRI~RSL_0, in/mile |
| --- | --- | --- |
| 1 | Rural Principal Arterial - Interstate | 170 |
| 11 | Urban Principal Arterial - Interstate | 170 |
| 12 | Urban Principal Arterial - Other Freeways | 180 |
| 14 | Urban Principal Arterial - Other | 250 |
| 16 | Urban Minor Arterial | 265 |
| 17 | Urban Major Collector | 280 |
| 18 | Urban Minor Collector | 280 |
| 19 | Urban Local | 325 |
| 2 | Rural Principal Arterial - Other Freeways | 180 |
| 3 | Rural Principal Arterial - Other | 180 |
| 6 | Rural Minor Arterial | 250 |
| 7 | Rural Major Collector | 265 |
| 8 | Rural Minor Collector | 280 |
| 9 | Rural Local | 325 |

It is important to note that ride quality historically controls the condition of the lowest percentage of lane-miles among all five performance measures ranging between only 8-12% of the total lane-miles from 2014-2016. Therefore, it is reasonable to assume that the effect of IRI deterioration on overall pavement condition is somewhat insignificant when compared to the other four performance measures - rutting, structural and functional cracking, and friction.

- If models pass the above referenced QA checks, proceed to Step 6.

- Otherwise, identify issues with the IRI model parameters and discuss remedial actions with appropriate personnel before continuing.

- As needed, write/update the IRI model parameters for each pavement family in C#. The existing IRI model parameters can be found in the worksheet IRI Function, of the MS Excel QA workbook template. The updated parameters, and hence the IRI models, will be used later in the program optimization process (see Defining Network and Importing Data into the RoadCare Optimization Process).

## Updating Cracking Performance Models

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to update the cracking performance model parameters as new cracking data are collected and processed. This SOP covers the cracking performance model parameters used for predicting cracking condition metrics, including those used by the SHA pavement management system (PMS) as well as those for submission to the Federal Highway Administration (FHWA) Highway Performance Monitoring System (HPMS). Those metrics are listed in the table below.

| Surface Type | SHA PMS metrics | FHWA HPMS metrics |
| --- | --- | --- |
| Asphalt Concrete (AC) | Structural Cracking Density | Cracking Percent - AC |
| Asphalt Concrete (AC) | Functional Cracking Density | Cracking Percent - AC |
| Jointed Concrete (JCP) | Structural Cracking Density | Cracking Percent - JCP |
| Continuously Reinforced Concrete (CRCP) | Structural Cracking Density | Cracking Percent - CRCP |
| Continuously Reinforced Concrete (CRCP) | Functional Cracking Density | Cracking Percent - CRCP |

The function selected to model Functional and Structural Cracking Density over time is the Gompertz model. This s-shape function is given by the following equation:

CD=Lexp-expll_int+ ll_slp AGE

Where:
CD = cracking density (functional or structural) of pavement at age “AGE”
AGE = age of pavement since last treatment, in years
L = model parameter (asymptotic cracking density value for large AGEs)
ll_int = first model parameter affecting model curvature/deterioration rate
ll_slp = second model parameter affecting model curvature/deterioration rate.

The equation parameters L, ll_int and ll_slp are typically updated yearly to account for possible changes in cracking trends resulting from the most recent cracking data. The current values of the model parameters are provided in Cracking Performance Models according to pavement family.

### Frequency

The update of SHA cracking modeling parameters is typically performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables).

### Purpose

The purpose of this SOP is to update the SHA cracking performance model parameters to incorporate the mostly recently collected and processed cracking data.

### Resource Requirements

The updating of the cracking performance models involves the following three SHA staff members: (1) a database management expert from the DAT to update model parameters, (2) an analyst from the DAT to validate the estimated models, and (3) a supervisor – typically the Data Analysis Team Leader (TL) orAssistant Division Chief (ADC) – who, as required, provides guidance and decision-making. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the update of the cracking performance model parameters. The estimate assumes no issues are encountered during the update process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Model Estimation | 1 | 8 |
| DAT Staff | Model Validation | 1 | 40 or as needed |
| DAT TL or ADC | Supervisor | 1 | As required06/12/2019 |

### Procedure

The procedure required to update the SHA cracking performance models is comprised of the following two tasks:

- estimation of new model parameters, and

- QA new model parameters.

1- Estimation of New Cracking Model Parameters – performed by DAT staff

Summary: Under this task, the most recently collected and processed cracking data are used to estimate new model parameters.

- In Oracle SQL Developer, run package “CRACKING_MODELS” to estimate new model parameters for each pavement family.

- Once the cracking model parameters have been updated, inform DAT that the model parameters are ready to for QA.

2- QA of New Model Parameters – performed by DAT Staff

Summary: Under this task, QA checks are used to confirm that the newly estimated cracking model parameters produce reasonable trends. The QA checks are performed using an existing MS Excel QA template spreadsheet.

- Open the most recent MS Excel QA template spreadsheet in the cracking models working folder06/12/2019 and sort by date modified. The QA template spreadsheet is called “Cracking_Density_Models_[MMDDYYYY].xlsx”.

- Check new estimate parameters for reasonableness and flag those models that exhibit unexpected trends. This is done by plotting the Gompertz curve for 100 years using the graph application in MS Excel to confirm that the trends look reasonable for all pavement families.

It is expected that the cracking density for most pavement families will deteriorate to the RSL0 value of the least important functional class in that family in 100 years, assuming an initial cracking density of 0. The tables below show the cracking density RSL0 values for the various pavement functional classes. This check should be performed before running the optimization and corrections to the pavement family slopes should be made as needed. Analysis of historical data shows that cracking deteriorates the fastest among all performance measures, controlling between 36%-41% of the total lane miles in the SHA pavement network from 2015-2017. Therefore, it is reasonable to assume that the effect of cracking deterioration on overall pavement condition is quite significant.

| FUNC_CL | FC_DESCRIPTION | FCD_RSL_0 |
| --- | --- | --- |
| 1 | Rural Principal Arterial - Interstate | 25 |
| 11 | Urban Principal Arterial - Interstate | 25 |
| 2 | Rural Principal Arterial - Other Freeways | 35 |
| 12 | Urban Principal Arterial - Other Freeways | 35 |
| 3 | Rural Principal Arterial - Other | 35 |
| 14 | Urban Principal Arterial - Other | 35 |
| 6 | Rural Minor Arterial | 35 |
| 16 | Urban Minor Arterial | 35 |
| 7 | Rural Major Collector | 45 |
| 17 | Urban Major Collector | 45 |
| 8 | Rural Minor Collector | 45 |
| 18 | Urban Minor Collector | 45 |
| 9 | Rural Local | 50 |
| 19 | Urban Local | 50 |

| FUNC_CL | FC_DESCRIPTION | SCD_RSL_0 |
| --- | --- | --- |
| 1 | Rural Principal Arterial - Interstate | 15 |
| 11 | Urban Principal Arterial - Interstate | 15 |
| 2 | Rural Principal Arterial - Other Freeways | 35 |
| 12 | Urban Principal Arterial - Other Freeways | 35 |
| 3 | Rural Principal Arterial - Other | 35 |
| 14 | Urban Principal Arterial - Other | 35 |
| 6 | Rural Minor Arterial | 35 |
| 16 | Urban Minor Arterial | 35 |
| 7 | Rural Major Collector | 35 |
| 17 | Urban Major Collector | 35 |
| 8 | Rural Minor Collector | 35 |
| 18 | Urban Minor Collector | 35 |
| 9 | Rural Local | 40 |
| 19 | Urban Local | 40 |

The time to “zero RSL” for all pavement families is expected to be between 10 and 50 years and the second model parameter (slope ll_slp) should always be negative. Flagged models are subject to further investigation - not immediate rejection.

- As needed, write/update the cracking model parameters for each pavement family in C#. The existing functions can be found in Cracking Performance Models. The updated parameters and hence cracking models will be used later in the program optimization process.

## Updating Rutting Performance Model

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to update the rutting performance model parameters as new rutting data are collected and processed. The rutting performance model follows the linear function shown in the equation below, which was developed from the analysis of historical SHA rutting data.

RDest=RDin+ ∝1AGE

Where:
RDest = estimated rutting value of pavement at age “AGE”, in inches
RDin = initial pavement rutting value at time of last treatment, in inches
AGE = age of pavement for which rutting is estimated, in years
∝i = parameter (equal to slope of linear regression) that captures effect of surface type and functional class.

The model, and more specifically the ∝i parameter, is typically updated yearly to incorporate possible changes in rutting trends resulting from the most recent rutting data. The current value of ∝i for the different pavement families are listed in Rutting Performance Model, along with the most recent rutting trend plots by pavement family and the specific C# functions.

### Frequency

The update of rutting model parameter is typically performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables).

### Purpose

The purpose of this SOP is to update the rutting performance model parameter to incorporate the most recently collected and processed rutting data.

### Resource Requirements

The updating of the rutting performance models involves the following three SHA staff members: (1) a database management expert from the DAT to update model parameters, (2) an analyst from the DAT to validate the estimated models, and (3) a supervisor – typically the Data Analysis Team Leader (TL) or Assistant Division Chief (ADC) – who, as required, provides guidance and decision-making. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the update of the rutting performance model parameter. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Model Estimation | 1 | 8.0 |
| DAT Staff | Model Validation | 1 | 40.0 or as needed |
| DAT TL or ADC | Supervisor | 1 | As required06/12/2019 |

### Procedure

The procedure required to update the rutting performance model is comprised of the following two tasks:

- estimation of rutting new model parameter, and

- QA new rutting model parameter.

1- Estimation of New Rutting Model Parameter – performed by DAT staff

Summary: Under this task, the most recently collected and processed rutting data are used to estimate new model parameter.

- The “business_plan_trend_pkg” must be run in the ASSETDEV environment in Oracle SQL Developer before estimating the new rutting model parameter for each pavement family. This package will update the “business_plan_trend table.” Ensure this has been completed prior to moving to â€ŽStep 3.

2- QA of New Rutting Model Parameter – performed by DAT Staff

Summary: Under this task, QA checks are used to confirm that the newly estimated rutting model parameter for each pavement family produces reasonable trends. The QA checks are performed using an existing MS Excel QA workbook template.

- Open the most recent MS Excel QA workbook template 06/12/2019 (sort by date modified) in the rutting models working folder.

- Connect to the ASSETDEV environment in Oracle SQL Developer and run the SQL statement stored in cell “F1” in the worksheet “Data.” Paste the output of the SQL statement in the worksheet “Data” from cell A1. The source table in the SQL statement is “business_plan_trend.”

- Create a pivot table from the pasted data by selecting “Insert” and then “Pivot Table” from the Excel ribbon. Select the table range of the data that was pasted in â€ŽStep 3. Select “New Worksheet” and click “OK.”

- On the new worksheet, filter the pivot table by surface type equal to “F” (Flexible) and organize the fields as shown in the below images.

- Once the pivot table is organized, copy and paste the data underneath the pivot table in the same worksheet as shown in the below image.

- Perform a visual review of the data and remove any data points believed to be outliers, as illustrated by the yellow highlights in the above image. Previous experience with analyzing rutting data is required to perform this review.

- For each pavement family (or group of families), plot the rutting versus time model and obtain the slope by performing a linear regression in Excel. Review the plots for reasonableness and make adjustments to the groupings if necessary. For example, families exhibiting similar trends may be grouped together if the individual plots do not seem reasonable or the amount of data is insufficient. See Rutting Performance Model for the most recent plots by pavement family.

- As needed, write/update the rutting model parameter for each pavement family (or group of families) in C#. See Rutting Performance Model for the most recent model functions. The updated parameter for each pavement family, and hence rutting model, will be used later in the program optimization process (see Defining Network and Importing Data into the RoadCare Optimization Process).

- Repeat Step 4 through Step 9 for surface types “FCJ” (Flexible over JCP) and “FCC” (Flexible over CRCP).

## Updating Faulting Performance Model

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to update the faulting performance model parameters as new faulting data are collected and processed. The faulting performance model follows the linear function shown in the equation below, which was developed from the analysis of historical SHA faulting data.

Faultingest=Faultingin+ γ1AGE

Where:
Faultingest = estimated faulting value at age “AGE,” in inches
Faultingin = initial faulting value at time of last treatment, in inches
AGE = age of pavement for which faulting is estimated, in years
γi = parameter (equal to slope of linear regression) that captures effect of pavement type.

The model, and more specifically the γi parameter, is typically updated yearly to incorporate changes in faulting trends resulting from the most recent faulting data. The current values of γi for the jointed plain concrete (RJP) and jointed reinforced concrete (RJ) pavement families are provided in the table below. The number of years of historical data available to develop an estimate of the faulting model parameter were limited. Accordingly, improvements in the parameter are expected as additional data are incorporated in the coming years into the model development process.

| Pavement Type | Slope (γi) |
| --- | --- |
| Jointed Plain Concrete (RJP) | 0.004 |
| Jointed Reinforced Concrete (RJ) | 0.008 |

### Frequency

The update of faulting model parameter is typically performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables).

### Purpose

The purpose of this SOP is to update the faulting performance model parameter to incorporate the most recently collected and processed faulting data.

### Resource Requirements

The updating of the faulting performance models involves the following three MDT SHA staff members: (1) a database management expert from the DAT to update model parameters, (2) an analyst from the DAT to validate the estimated models, and (3) a supervisor – typically the Assistant Division Chief (ADC) – who, as required, provides guidance and decision-making. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the update of the faulting performance model parameter. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Model Estimation | 1 | 8.0 |
| DAT Staff | Model Validation | 1 | 20.0 or as needed |
| ADC | Supervisor | 1 | As required06/12/2019 |

### Procedure

The procedure required to update the faulting performance model is comprised of the following two tasks:

- estimation of new faulting model parameters, and

- QA new faulting model parameters.

1- Estimation of New Faulting Model Parameter – performed by DAT staff

Summary: Under this task, the most recently collected and processed faulting data are used to estimate new model parameters.

- The “business_plan_trend_pkg” must be run in the ASSETDEV environment in Oracle SQL Developer before estimating new faulting model parameter for each pavement family. This package will update the “business_plan_trend table.” Ensure this has been completed prior to moving to Step 3.

2- QA of New Faulting Model Parameter – performed by DAT Staff

Summary: Under this task, QA checks are used to confirm that the newly estimated faulting model parameter for each pavement family produces reasonable trends. The QA checks are performed using an existing MS Excel QA workbook template.

- Open the most recent MS Excel QA workbook template06/12/2019 (sort by date modified) in the faulting models working folder.

- Connect to the ASSETDEV environment in Oracle SQL Developer and run the SQL statement stored in cell “M1” in the worksheet “Sheet1.” Paste the output of the SQL statement in the Excel sheet. The source table in the SQL statement is “business_plan_trend.”

- The model parameter for each pavement family is estimated using a linear regression in MS Excel and the data resulting from â€ŽStep 3, as illustrated in the following two plots.

- Examine the faulting versus time plot for each pavement family (jointed plain and jointed reinforced concrete) using the output from the SQL statement in the previous step and check that the equations are showing reasonable results. If the two models pass the QA checks noted above, proceed to â€ŽStep 6. Otherwise, identify issues and discuss remedial actions with appropriate personnel before continuing

  - The check is intended to confirm that faulting reaches a value of 0.15 inches or greater within 100 years from the time of the last treatment, which is assumed to have an initial faulting value of 0. This value of 0.15 inches corresponds to the “Poor” threshold for faulting specified in the final rule.

  - It is expected that faulting for reinforced jointed concrete pavements deteriorate at a slower rate than jointed plain concrete pavements.

- As needed, write/update the faulting model parameter for each pavement of the two pavement families in C#. The updated parameter for each pavement family, and hence faulting model, will be used later in the program optimization process (see Defining Network and Importing Data into the RoadCare Optimization Process).

## Updating Friction Performance Model

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to update the friction performance model parameter as new friction data are collected and processed. The friction performance model follows the linear function shown in the equation below, which was developed from the analysis of historical SHA friction data.

Fest=Fin+ ∝1AGE

Where:
Fest = estimated friction value of pavement at age “AGE”
Fin = initial pavement friction value at time of last treatment
AGE = age of pavement for which friction is estimated, in years
∝i = parameter (equal to slope of linear regression) that captures effect of county and pavement functional class.

The model, and more specifically the ∝i parameter, is typically updated yearly to incorporate possible changes in friction trend from the most recent friction data. The current values for the different pavement families are listed in Friction Performance Model, along with the most recent friction trends and models by pavement family.

### Frequency

The update of the SHA friction model parameter is typically performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables).

### Purpose

The purpose of this SOP is to update the SHA friction performance model parameter to incorporate the mostly recently collected and processed friction data.

### Resource Requirements

The updating of the friction performance models involves the following three SHA staff members: (1) a database management expert from the DAT to update model parameters, (2) an analyst from the DAT to validate the estimated models, and (3) a supervisor – typically the Assistant Division Chief (ADC) – who, as required, provides guidance and decision-making. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the update of the friction performance model parameter. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Model Estimation | 1 | 8.0 |
| DAT Staff | Model Validation | 1 | 40.0 or as needed |
| ADC | Supervisor | 1 | As required06/12/2019 |

### Procedure

The procedure required to update the SHA friction performance model is comprised of the following two tasks:

- estimation of new friction model parameter, and

- QA new friction model parameter.

1- Estimation of New Friction Model Parameter – performed by DAT staff

Summary: Under this task, the most recently collected and processed friction data are used to estimate new model parameters.

- Open the most recent MS Excel file06/12/2019 (sort by date modified) in the friction models working folder.

- Connect to the ASSETDEV environment in Oracle SQL Developer and run the SQL statement stored in cell F1 of the worksheet “Data.” The source table in the SQL statement is “friction.” This will drop the existing table PAV_CONHIST.TREND_FRICTION_12_14_2017 and create a new table PAV_CONHIST.TREND_FRICTION [MM_DD_YYYY] (change date) with the updated data.

- While still in Oracle SQL Developer, run the SQL statement stored in cell E1 in the worksheet “Data.” Paste the output of the SQL statement in the worksheet “Data” in cell A1.

2- QA of New Friction Model Parameter – performed by DAT Staff

Summary: Under this task, QA checks are used to confirm that the newly estimated friction model parameter for each pavement family produces reasonable trends. QA checks are performed using a MS Excel workbook.

- Create a pivot table from the pasted data by selecting “Insert” and then “Pivot Table” from the MS Excel ribbon. Select the table range of the data that was pasted in â€ŽStep 3. Select “Existing Worksheet” and click “OK.”

- Organize the fields in the pivot table as shown in the following images.

- Once the pivot table is organized, copy and paste the data into a new worksheet.

- Perform a visual review of the data and remove any data points believed to be outliers, as illustrated by the yellow highlights in the above image. Previous experience is required to perform this review.

- For each pavement family, plot friction versus time data and calculate the slope of the regression line using the graph application in MS Excel. Review plot for reasonableness and make adjustments to the groupings if necessary. See Friction Performance Model for the most recent plots by pavement family.

- As needed, write/update the friction model parameter for each pavement family in C#. See Friction Performance Model for the most recent model functions. The updated parameter for each pavement family will be used later in the program optimization process (see Defining Network and Importing Data in the RoadCare Optimization Process).

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and DWT with DAT |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Optimization

Click to go to Network Definition and Data Import

Click to go to Network Segmentation and Data Rollup

Click to go to Needs Analysis and Running Simulation Engine

Click to go to Generating Reports

## Introduction

This section describes the set of standard operating procedures (SOP) conducted mainly by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration’s (SHA) Office of Materials Technology (OMT) to perform the “Conditional RSL/Cost Optimization” in the asset management software, RoadCare, version 3.67. This section is divided into four SOPs: defining the network and importing new data, updating the network segmentation and performing the data rollup, performing a needs analysis and running the simulation engine, and generating reports. These SOPs may require multiple iterations to complete the optimization process.

Figure 13 shows a flowchart of the Fund 77 Optimization procedures.

Figure 13: Fund 77 Optimization Flowchart

## Defining Network and Importing Data into the RoadCare Optimization Process

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to define the network of assets and to import the raw data associated with this network for input into the optimization process contained in the asset management software, RoadCare, version 3.67. The SOP described in this document may not be applicable to other versions of RoadCare.

### Frequency

The network definition and data import for the RoadCare optimization process is performed once per year, after the end of the data collection season and once all data have been processed and migrated into the appropriate tables. Additionally, selected attribute data are deleted and re-imported whenever changes to the source tables occur, which typically happens several times per year.

### Purpose

The purpose of this SOP is to prepare the input data required by the RoadCare optimization process by first defining the network of assets and then importing the required network data contained in Oracle tables.

### Resource Requirements

The network definition and data import process involves one person: a DAT staff member knowledgeable of Oracle and the RoadCare software. The estimated effort level in the table below represents the total time, in man-hours, to complete the network definition and data import process. The time estimate assumes no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 20.0 |

### Procedure

The procedure required to perform the network definition and data import process is comprised of the following two components:

- define network, and

- import data.

1- Define Network – performed by DAT staff

Summary: Under this task, the RoadCare asset management software’s “Route/Section Definition” module and a script run in Oracle SQL Developer are used to define the network of assets from the inventory data contained in the Oracle tables.

- Open RoadCare software.

- In RoadCare’s “Explorer” panel, expand the “Route/Section Definition” folder and then double-click on “Linear Referenced Routes” to display the contents of the network. Each linear asset is defined by grid columns including name (ROUTES), stations (BEGIN_STATION and END_Station), and direction. Before entering data, select the “Allow Definition Edit” option located at the top of the window as shown in the below image.

- In Oracle SQL Developer, run the first SQL statement labeled “CREATE_INVENTORY” in the “RoadCare Data Import Queries latest.sql” file06/12/2019 to update the materialized view called “RC_SECTION_MVIEW.” Update the “HMIS_YEAR” field in the SQL statement to retrieve the latest inventory data.

- After the materialized view is updated in the SQL Developer, select “pavconhist_assetdev” in the drop-down box next to “Update using:” to replace existing inventory data in RoadCare with the most recent inventory data from the materialized view.

- Document changes to the inventory using the Access database and spreadsheet template06/12/2019.

2- Perform Data Import – performed by DAT Staff

Summary: Under this task, raw data from various Oracle tables are validated for input into the RoadCare asset management software. These data represent the attributes that define the pavement network, which was updated in the previous task.

- Open RoadCare software.

- Double-click on an attribute name from the “Attributes (Raw)” list to show its contents in the display window. Check the box next (to the left) to “Allow Attribute Edit” at the top of the raw attribute data window, as shown in the below image.

- Click the “Import from Data Source…” button, which is shown at the bottom of the raw attribute data window in the above image, to open the “Data Import” Window.

- Enter the database connection parameters in the left hand top portion of the “Data Import” window. The top left section of the window allows the user to select the provider name, server path, database name, to specify if the use of integrated security is active, and to enter login and password information. When placing the cursor on any of the referenced fields, a description of the item is provided at the bottom of the top-left pane, as shown in the below image. For example, when selecting “Database,” the area under the information input section reads “Name of the database in which the attribute data resides.” These descriptions are intended to aid the user in determining the appropriate input for each field.

- After introducing the required data into the “Data Import” window, click “Connect” to establish communication with the selected server and access the desired database, as shown in the above image.

- When a connection is established, the “Database Tables” section will be populated with the attributes found in selected database. Any value within the “Database Tables” section can be clicked to display the available fields under the “Fields” section. Likewise, clicking any field value displays the data available for that field in the “Field Data” section, as shown in the below image.

- After database connection has been established, navigate to the data import SQL statement titled “RoadCare Data Import Queries latest”06/12/2019. If a connection cannot be established, discuss the matter with the database administrator before continuing.

- The attribute names in the .sql file will match the attribute names in RoadCare. For example, to find the SQL statement for Average IRI, open the .sql file in notepad, press Ctrl+F and search for “begin AVG_IRI.” Paste the appropriate SQL statement in the SQL statement text box and click the “Import” button to process the query. The resulting data entries are displayed in the attribute display window, while rejected data entries are displayed in the output window. Clicking the “Update*” button will store the SQL statement and connection parameters in the RoadCare database for future data imports. This is the preferred method of import. For small datasets, the user can copy and paste the results from the SQL statement into the RoadCare attribute window data grid. It is important to note that the data import sql statement(s) must be updated to capture any changes to the source tables. For example, if the IRI data in business_plan_100mmi table is updated for the past five years, then the criteria filter for the “REPORTING_YEAR” field must be updated to include those five years of data. If one or more source tables are dropped or modified extensively, then the relevant data import SQL statements must be updated as well.

- After the data import is completed, right click anywhere in the output window and select “Create Log File” to save a copy of the data import log as shown in the below image.

## Network Segmentation and Data Rollup

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to prepare collected data for the “Conditional RSL/Cost Optimization” process in the asset management software, RoadCare, version 3.67. The data preparation requires the creation and segmentation of a new network and the rollup of newly imported data. The SOP described in this document may not be applicable to other versions of RoadCare.

### Frequency

Network segmentation, segmentation criteria definition and data rollup are performed by the SHA once per year, after the end of the data collection season, once the data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables). The procedure may need to be repeated multiple times during the iterative “Condition RSL/Cost Optimization” process.

### Purpose

The purpose of this SOP is to prepare SHA collected data for the “Conditional RSL/Cost Optimization” process in RoadCare version 3.67 software by creating a new network, defining the network segmentation criteria, and performing a data rollup.

### Resource Requirements

Network segmentation, segmentation criteria definition, and data rollup requires one SHA staff member: a DAT staff member knowledgeable in Oracle and RoadCare software. The estimated effort levels shown in the table below represents the total time, in man-hours, to complete the network definition, segmentation criteria definition and data rollup. These time estimates assume no issues are encountered during the process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | AN06/12/2019 |

### Procedure

The procedure to perform the network segmentation, segmentation criteria definition and data rollup is comprised of the following four tasks:

- create new network and define segmentation logic,

- review and update segmentation results,

- rollup data in network, and

- define network specific area.

1- Create New Network and Define Segmentation Logic – performed by DAT Staff

Summary: Under this task, the new network is created and the logic for segmentation of the network is determined using the Segmentation Logic module in version 3.67 of RoadCare.

- Open RoadCare software.

- Right-click the Networks module and select Add New... as shown in the figure below. Type the name of the network in the text box next to Network Name and click OK to create the new network.

- Expand the options inside the Networks module by clicking on the + icon. Then, expand the options inside the Dynamic Segmentation menu that appears by clicking on the + icon.

- Double-click the on Segmentation Logic listed under the Dynamic Segmentation menu to display the user-interface screen as shown in the below image.

- Check the box in the top-right corner next to Allow Network Segmentation as shown in the above image.

- To define the segment subset criteria, the user can type the numerical or logic expression in the Subset Criteria field or click the 3-dots button in the top-right corner as shown in the above image. The Edit Segmentation Criteria window will appear as shown in the below image.

- Add a name to the segmentation criteria by typing it under the Subset Name field as shown in the above image. As a rule of thumb, the name should be related or self-explain the criteria being used.

- Next, select the desired attribute from the pull-down menu under the title “Attribute” as shown in the above image. All raw attributes belonging to the current network are available for selection. Once an attribute is selected, the “Values” pane is populated with the corresponding values for the attribute based on the network’s data. In addition, the ”Fields” pane located under the “Attribute” list is populated with the information posted under each attribute table. For example, if the raw attribute IRI table contains data fields that include ROUTES, BEGIN_STATION, END_STATION, DIRECTION, DATE, and YEARS, they will all appear listed under Fields. The last entry in the “Fields*” window is always the selected attribute, which appears within square brackets.

- Double-click on the desired field, then single-click on the numerical or logic expression, and finally double-click on value from the available list under the “Values” pane. As the field, expression, and values are selected, the bottom section of the pane is populated with expression for the segmentation criteria. Changes can be made directly by typing or erasing the expression. Once the criteria is input, the user can click on Check* to verify how many sections the expression will yield, OK* to accept the segmentation logic expression, or Cancel* to return to the previous screen. If there is an error in the syntax of the segmentation expression, a message is displayed in the output window. The numerical and logic expressions available to construct the segmentation criteria are shown in the below image.

  - If the expression is correct when Check is selected, then click on OK to return to the previous screen.

  - If the expression is not correct or returns a syntax error, revise the expression before clicking OK.

- The Segmentation Logic screen will display the new segmentation expression under the Subset Criteria field, and the name of the newly created subset name is displayed under Available Subset.

  - If more subsets need to be added, click on the New button located to the right of the Available Subset pane as shown in the following image. Repeat Step 7 through Step 9 for each subset that needs to be created. Many segmentation expressions can be input, and every time one is added, it is listed under the Available Subset pane.

  - If all subsets have been created, proceed to Step 11.

- Create the network root for the network segmentation tree. The network segmentation tree is a hierarchical construction that allows for further refinement of the segments. There must be at least one root level entry in the network tree to run the segmentation. After creating the root, the user can add additional levels. To create the network root, select the desired root subset from the Available Subset list and click on Add Root* (make sure the Allow Network Segmentation option is checked at the top right corner of the screen as shown in the above image). This will list the selected subset under the right pane of the window. Additional root-level subsets can be added following the same process.

- To add additional segmentation tree levels, select a different subset from the Available Subset list, select the root subset from the right pane, and click on Add*. This will automatically add the selected subset as a stem or “child node” to the root subset and a + icon will appear next to the root subset name. To expand the network tree, click on the + icon.

  - If a child node is added by error or to a level in the network that it does not correspond to, it can be deleted by selecting the subset name on the right pane of the window and press the Remove button.

  - To delete a subset from the Available Subset list, click on the subset name and press the Delete button. Note that the segmentation criteria Anychange and Anyrecord buttons cannot be included as child nodes; when this is attempted, the following error message is displayed on the output window: “Error: Cannot have ANYCHANGE, ANYRECORD in a child node.”

Note: Also, note that the network segmentation should be as general as possible since further analysis will be based on the segmented network. To split the network and consider parts of it with certain characteristics, a subnetwork should be created first.

- After the network segmentation tree has been defined, it is time to segment the roadway network. To accomplish his, click on the Segment Network* button. The entire definition of the roadway is determined using the contents of the network tree. If for any reason the segmentation cannot be done as specified, the output window will display an error message. If the segmentation is successful, the output window will display the time the segmentation was started and the time when the segmentation was completed. In addition, a summary of the segmentation criteria will be displayed detailing the reason for break (subset name), the Shortest segment distance, the longest segment distance, the average segment length, and the number of segments (i.e., Count). If several root subsets are used, a summary for each one of them is displayed (see below image).

2- Review and Edit Segmentation Results – performed by DAT Staff

Summary: Under this task, the segmentation results are reviewed and manually updated using the Segmentation Results module in RoadCare.

- Double-click on the Segmentation Results options located under the “Dynamic Segmentation” menu.

- The segmentation results will show the following fields: ROUTES, BEGIN_STATION, END_STATION, DIRECTION, and REASON (the reason for segmenting, in case two root subsets are used, for example) as shown in the below image. Using the “Route” pull-down menu, the user can choose to display all routes or choose a specific one. This is especially convenient when a large number of records exist in the network and the user wants to verify the information for particular ones. The segments can be manually modified by the user using one or more of the tools described below.

The segment length can be modified by changing the BEGIN_STATION and END_STATION limits. The user can type new values under each field for all the affected sections, as shown in the below image.

The segments can be further manipulated by clicking on Join or Split button located next to the “Route” pull-down menu. Highlight the records needing to be merged from the results grid and click on the Join Button, the segments will be combined in a single record and under the column field “Reason”, the legend “Joined” will appear. To split a segment, highlight the records that are to be divided from the results grid, enter the number of sections in the “Number of Split Sections” field, and click the “Split” Button. RoadCare will divide each of the selected segments evenly into the number of selected sections (e.g. if 3 segments are divided into 4 sections the result will be 12 sub-segments). Like the Join option, the legend Split will appear under the “Reason” column field.

Additional options to view and manipulate the resulting segments are found in the bottom line toolbar located below the data grid. The toolbar is shown in the below image.

Takes user to the first record on the list (row 1).

Moves user to the previous record based on the current selected record. If the first record of the list is the one selected, the option is displayed as a gray icon, meaning that it is not available for selection.

Shows position of the current selected record out of the total number of records. A specific record number can be selected by typing it in the window and pressing enter.

Moves user to the next record based on the current selected record. If the last record of the list is the one selected, the option is displayed as a gray icon, meaning that it is not available for selection.

Takes user to the last record on the list (last row).

Defines minimum distance value for the sub-segments sections

Defines maximum distance value for the sub-segments sections

Defines sub-segments gap intervals based on the section length and the minimum and maximum sub-segment sections values

Limits application of sub-segmentation to only the route being displayed

Sub-segmentation applied to all segmentation result records

Once the manual segmentation is complete, proceed to the next task.

3- Perform Rollup – performed by DAT Staff

Summary: Under this task, the segmented database rollup rules are defined using the “Rollup” module in RoadCare.

- Rolled-up (or processed) data is the second level of data storage. It is obtained by assigning raw data to the segments created through dynamic segmentation, using appropriate data aggregating methods (average, minimum, maximum, first, last, predominant etc.) to arrive at a representative value for a section. The “Rollup” option allows the user to define the segmented database rollup rules. Two main factors contribute to the rollup rules: “Attribute” and “Method.” The attribute information is extracted from the database and cannot be altered. However, the user can specify the rollup method to aggregate and distribute the data for each attribute. To open the “Rollup” interface, double-click on the “Rollup” option under the “Dynamic Segmentation” menu. The “Segmentation Rollup” window interface will appear as shown in the below image.

- Every time an attribute is selected from the Attribute list, a list of values is displayed on the pane located to the right of the window. Properly select the Method for each Attribute by utilizing the table found in Optimization Data Rollup Methods.

- Once the appropriate method has been selected for each attribute, press the “Rollup Network” button located under the Attribute and Method list pane shown in the above image. The output window will show a message stating when the rollup began, the progress of the rollup for each attribute, and when the rollup finished. The user can copy the attribute rollup methods from another network and apply it to the current network by clicking the pull-down menu next to “Update using” and selecting the appropriate network name. See Optimization Data Rollup Methods for further details regarding specific implementation of network segmentation.

4- Define Network Specific Area – performed by DAT Staff

Summary: Under this task, the network specific area is defined in RoadCare. SHA uses an average of Vehicle Miles Travelled (VMT) and Lane Miles (LM) to calculate the network-specific section area for treatment selection, benefit and remaining life calculations

- Open the MS Excel spreadsheet with calculations for network specific area06/12/2019.

- Copy and paste these data into the attribute LM_VMT_AREA.

- Right-click on the desired network and select Define Network Specific Area. This opens a pop-up window where user can specify the attribute name that contains the area calculations in the equation box as shown in the below image.

- Click OK to save changes.

## Needs Analysis and Running Simulation Engine

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to prepare collected data for the “Conditional RSL/Cost Optimization” process in the RoadCare version 3.67 asset management software. This is accomplished by performing a needs analysis and running the RoadCare Simulation Engine. This SOP is performed after the data rollup is completed as described in Network Segmentation and Data Rollup. The RoadCare Simulation Engine requires the following five major inputs:

- Analysis: inputs used to manage project selection methodology (optimization & prioritization) and spending rules (i.e., spend within defined budgets or determine required budgets to meet goals);

- Investment: inputs used to manage budget types and annual amounts as well as inflation and discount rates;

- Performance: inputs used to define equations for predicting future attribute values based on current values;

- Treatment: inputs used to manage Maintenance and Rehabilitation (M&R) activities, requirements for feasibility, applicable budget type and unit costs, and consequences resulting from the activity; and

- Committed: inputs used to manage projects that are planned/selected by the user independent of RoadCare’s selection methodology and treatment definition constraints.

Note: The SOP described in this document may not be applicable to other versions of RoadCare.

### Frequency

Performing a needs analysis and running the Simulation Engine is performed once per year, after the end of the data collection season, once the data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables). The procedure may need to be repeated multiple times during the iterative “Condition RSL/Cost Optimization” process.

### Purpose

The purpose of this SOP is to establish a required budget for the SHA network by performing a needs analysis and then optimizing the budget allocation through the Simulation Engine in the RoadCare software.

### Resource Requirements

Performing needs analysis and running the RoadCare Simulation Engine requires the following three SHA staff members: (1) a DAT staff member knowledgeable in Oracle and RoadCare software to perform the needs analysis and run the simulation; (2) the Assistant Division Chief (ADC) to review the reports generated from the simulation and suggest revisions to the inputs where necessary; and (3) the Division Chief (DC) to provide guidance and decision-making during the ADC review. The estimated effort levels shown in the table below represent the total time, in man-hours, to complete the needs analysis and simulation process. These time estimates assume no issues are encountered during the procedure.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | AN06/12/2019 |
| ADC | Review | 1 | AN |
| DC | Review | 1 | AN |

### Procedure

The procedure to run the RoadCare Simulation Engine is comprised of the following eight tasks:

- perform a needs analysis

- create a new Simulation,

- input the Analysis parameters,

- input the Investment parameters,

- input the Performance parameters,

- input the Treatment parameters

- input the Committed parameters, and

- run the simulation.

1- Perform a Needs Analysis – performed by DAT analyst

Summary: Under this task, a needs analysis is performed to identify the budget requirements for the pavement network. This analysis is performed only as required; i.e., if the simulation budget is not already available. Typically, a needs analysis is performed by running several “Conditional RSL As-Budget Permits” simulations by iteratively increasing the budget in the Investment window (See Task 4 - Total Investment of this SOP) to meet some pre-determined criteria. Simulations are run in a similar manner as explained in the following sections using the same inputs and criteria, and summaries from all sections reports are generated until these criteria are met. Some of these criteria are as follows: - How much money is needed to maintain current conditions? - How much money is needed to attain SHA’s business plan goal of RSL=20 in 10 years? - How much money is needed to achieve a benefit target of X LMY in fiscal year Y?

In order to meet the SHA business plan goal of 20 RSL with consistent funding, it has been determined that Fund 77 needs $260M to $270M each year overall, with construction remaining steady at $220M each year. This assumes a system preservation approach with Rehabilitation and Preventive Maintenance. This conclusion can be determined by summarizing various “All Sections” reports. Refer to the files below06/12/2019 for more details.

- Pavement RSL Predictions 2010-2027 - Adjusted with 220M v3.xlsx (Summary sheet)

- All_Sections - SW_SIM_BY_TREATMENT - DoNothing.xlsm (Do nothing simulation)

- All_Sections - SW_SIM_BY_TREATMENT - Optimized 220M.xlsm (data for $220M)

- All_Sections - SW_SIM_BY_TREATMENT - Optimized.xlsm (data for current budget)

If the simulation budgets has been finalized, then this task can be skipped altogether.

- Determine if a needs analysis must be performed based on the criteria described in the summary of Task 1.

  - If a needs analysis is required, proceed to Step 2.

  - If a needs analysis is not required, skip to Step 3.

- Follow the steps described below for creating a simulation and adjusting the simulation input parameters. Iteratively increase the budget in the Investment window (See Task 4- Total Investment of this SOP) to meet some pre-determined criteria. The criteria for the needs analysis should be discussed with the ADC prior to completion of the needs analysis.

2- Create a New Simulation – performed by DAT analyst

Summary: Under this task, a Simulation is created using RoadCare for the desired network.

- Open RoadCare software.

- Expand the options inside the “Networks” module by clicking on the “+” icon. Then, expand the options inside the “2016_NETWORK_COMBAREA”06/12/2019 menu that appears in the list by clicking on the “+” icon.

- Right-click the on “Simulations” listed under the “2016_NETWORK_ COMBAREA” menu and select the “Add New” option, as shown in the below image.

- Type an appropriate simulation name in the text box next to “Simulation Name” and select the appropriate network from the drop-down list next to “Network,” as shown in the below image. After the simulation is created, a new folder with the simulation name is listed under the “Simulations” tab.

3- Analysis Input – performed by DAT analyst

Summary: Under this task, the parameters required for the Analysis module are input in RoadCare. The parameters are summarized below.

- Budget: spending rules that are used to limit the allocation of funding to feasible treatments.

- Weighting: input modifies the calculated benefit or remaining life by a user-selected parameter. The selected weighting parameter is directly multiplied by the calculated benefit to give a weighted benefit.

- Benefit: input determines which attribute is used in the Incremental benefit/cost calculations. It is recommended to select a benefit variable even when performing a remaining life analysis because the benefit can be calculated and reported, even if not used as a basis for treatment selection.

- Jurisdiction criteria: creates a subset of given network data. For example, if a network contains data for an entire state highway agency, the Jurisdiction input can be used to run a Simulation on only District 1.

- Priority: tab on the analysis window is used to set budget priorities and constraints. The priority column is an integer number greater than 0, with no numbers repeating. The criteria column contains the values that causes this priority to apply; e.g. [SHOP]='CAMBRIDGE' for all sections assigned to the Cambridge Shop. These criteria are applied for every year of the simulation and are based upon the predicted value of the included attributes. The funding levels for rehabilitation, maintenance, and/or construction or other user-defined investment criteria as defined by a positive number between 0 and 100 (i.e. 100% is input as 100). This number represents the maximum cumulative percent of the budget that may be spent on this priority or lower priority levels (priority 1 is the highest priority level). Any money not spent on a higher priority is available to be spent on lower priority treatments/sections. If the criteria field is left blank, the priority applies to all sections.

- Target: tab on the analysis window allows the user to input the network goals which the simulation engine will attempt to satisfy (not applicable for conditional RSL/cost optimization).

- Deficient: tab on the analysis window, allows the user to input the network deficiency targets which the simulation attempts to satisfy (not applicable for conditional RSL/cost optimization).

- Expand the options inside the newly created simulation in Step 6 by clicking on the “+” icon. Double-click on the first option, which is “Analysis,” to display the main “Analysis” window as shown in the below image.

- Enter the required input data. Most inputs required for the simulation engine are conditional in nature. This means that the equation, constraint, or action is entirely dependent upon the current state or condition of each pavement management section. Simulation engine conditional statements are input using standard SQL query syntax. The interface includes a query builder and checker sub-module. For the “Analysis” sub-module, the window contains the following fields:

  - Optimization,

  - Budget,

  - Weighting,

  - Benefit,

  - Description,

  - Benefit Limit, and

  - Jurisdiction Criteria.

In addition, tabs labeled Priority, Target, and Deficient allow entering additional input data. These inputs vary from one simulation to another. For more details on these inputs, please see RoadCare Help. Listed below are a set of possible inputs to use in a simulation.

Optimization – Conditional RSL/Cost.

Budget – As Budget Permits.

Weighting – None.

Benefit and Benefit Limit – Not applicable for Conditional RSL/Cost optimization.

Jurisdiction Criteria: Varies depending on the simulation; e.g. ~sql ISRAMP=‘0’ AND GOVT_CONTROL=‘1’ AND FUNC_CLASS IS NOT NULL AND SIMDIV=‘1’ AND (END_STATION - BEGIN_STATION) >= 0.5 AND TOTAL_LANE_MILES > ‘0’ AND (FC_DENSITY > 0 OR SC_DENSITY > 0 OR AVG_RUT > 0 OR AVG_IRI > 0 OR SPADJ_SKID_NUMBER > 0) AND AGE >= 0~

Priority – See Steps 10-16.

Target & Deficient – Not applicable for Conditional RSL/Cost Optimization.

- When all the data necessary for the simulation are correctly entered, select “Run Simulation” to start the simulation engine. Errors that occur during the compilation or run of the simulation will be displayed in the output window.

4- Total Investment – performed by DAT analyst

Summary: Under this task, the necessary parameters required for the Investment module are input in RoadCare.

- In the “Explorer” window, double-click on the second option, “Investment,” under the simulations module for the chosen network to display the main “Investment” window, as shown in the below image. Enter the following inputs:

  - Start year: field used to define the first year of the simulation. For example, if the RoadCare database contains attribute data from 1999-2017, and a start year of 2020 is entered, RoadCare will roll forward the attributes according to the input performance equations. Usually, the start year is entered as the most recent inventory year plus 1.

  - Analysis period: determines the number of years that a simulation will run. The analysis period should be at least 10 years for the State Optimization. The inflation rate (%) and discount rate (%) fields are used to determine the present value of treatments and committed projects.

  - Inflation rate: 4.2% (most recent).

  - Discount rate: 0%

  - Budget Order: determines the budget expenditure hierarchy for the “As Budget Permits” and “Unlimited Budget” analyses.

Contact the Chief Engineer’s office to get the statewide construction budget for the next several years from the most recent CTP allocation and enter in the “STATEWIDE” budget category in the investment window. Note that committed projects must be in a separate budget category. For the “COMMITTED” category, add the cost of all committed projects by fiscal year to determine the budget. To add budget categories, click the 3 dots (…) button in the Investment window next to Budget order to display the Edit Budget Categories window shown in the image below. Type the name of the budget categories and click “Add”. No budget categories names should contain spaces or single quotes. To change the order, select a given category and select “Up” or “Down” based on the desired order of the category in the list. Any category can also be deleted by highlighting the category name and selecting “Delete.” Once the budget categories list is complete, select OK.

- Open the “Budget Per Shop” Excel file06/12/2019. Update all data in the worksheet “2016_SHA_Inventory” by running the SQL statements in cells I1, I21, I42, I69, K97, and P124 in Oracle SQL Developer in the production environment06/12/2019. Calculate $ MIN (%) and $ MAX (%) of LM and VMT for each Shop. Use LM if it is the max. This is called “DraftBudgetAlloc“ in the Excel file.

- Run a “Conditional RSL/Cost - As Budget Permits” simulation with all committed projects and most recent construction budgets from OCE without priorities/budget allocation. The only criteria to use in this simulation is “[GOVT_CONTROL] = 1” as shown in the following image. Budget priorities and investment inputs are provided in the simulation “TEST_SIM_BUDGETPERSHOP_1” in RoadCare. All other inputs remain the same. To access this simulation, expand the options inside the “Networks” module by clicking on the “+” icon. Then, expand the options inside the “2016_NETWORK_COMBAREA” menu that appears in the list by clicking on the “+” icon. Then expand the options inside the “Simulations” module by clicking on the “+” icon. This will show the list of simulations under the selected network. Then expand the options inside the selected simulation by clicking on the “+” icon next to the simulation name. See Steps 3 through 7 for screenshots and more details.

- Once the simulation is complete, generate a “Targets/Suggested Projects Report” (see Generating Reports). Copy the budget allocation by shop from the “Targets” tab in the spreadsheet. These $ numbers will be called “OptimizResults.”

- Paste “OptimizResults” in “BudgetPerShop” tab, Column V.

  - IF (OptimizResultsDraftBudgetAlloc) then use DraftBudgetAlloc else use OptimizResults. This is labeled “DRAFT Budgetq” in the Excel file.

- Apply the following formula in column H: =IF(DRAFT Budgetq=$ MIN (%),"MIN",IF(DRAFT Budgetq=$ MAX (%),"MAX",""))

- In column K, adjust the budget per shop distributions that are not classified as MAX so that the total adds up to 100%. Note that cumulative budget distributions will be used in RoadCare in the priorities window. These numbers are available in tab “RoadCare BudgetperShop 02202018” in the Excel file. Copy and paste the data in this file (exclude headers) and paste into the “Priority” tab in RoadCare by selecting the first cell under the “Priority” column and pressing Ctrl+V on the keyboard to paste the data (see below image).

5- Performance Models – performed by DAT analyst

Summary: Under this task, the necessary parameters required for the performance module are input in RoadCare.

- In the “Explorer” window, double-click on the third option, “Performance,” under the simulations module for the chosen network to display the main “Performance” window as shown in the below image.

- Double-click on the “AVG_IRI” field in the display window to open the “Edit Simulation Equations” window. Make sure “Check this box to enter equation as a function is checked.” Paste the updated C# function for IRI from the Excel file into the input window as shown in the image below. Click “OK.”

- Repeat Step 18 for the other four performance measures (functional cracking density, structural cracking density, rutting, and friction) by copying in the correct C# functions, updated in the Excel file, into the input window.

- Run a “Conditional RSL/Cost - As Budget Permits” simulation for at least 10 years using the most recent OCE budgets and treatment inputs. Budget priorities and investment inputs are provided in the simulation SIM1_BUDGETCAP_TREATMENT in RoadCare. All other inputs remain the same. To access this simulation, expand the options inside the “Networks” module by clicking on the “+” icon. Then, expand the options inside the “2016_NETWORK_COMBAREA” menu that appears in the list by clicking on the “+” icon. Then expand the options inside the “Simulations” module by clicking on the “+” icon. This will show the list of simulations under the selected network. Then expand the options inside the selected simulation by clicking on the “+” icon next to the simulation name. See Steps 3 through 7 for screenshots and more details.

- Once the simulation is complete, generate a “Targets/Suggested Projects Report” (see Generating Reports) for the current optimization year to review the treatment life extension for suggested projects in the optimization year. Compare against the typical treatment life extension ranges specified in the supplemental treatment information section of the SHA Pavement Design Guide.

  - If the predicted life extension for a specific treatment is outside the recommended range specified in the Design Guide, then the corresponding family of models (usually in cracking or IRI) must be revised to produce more reasonable results.

6- Treatments – performed by DAT analyst

Summary: Under this task, the parameters required for the Treatments module are input in RoadCare.

- In the “Explorer” window, double-click on the fourth option, “Treatment,” under the simulations module for the chosen network to display the main “Treatment” window as shown in the below image.

- Open the “Feasibility” tab and ensure the feasibility criteria for each treatment are properly specified. Multiple feasibility conditions can be specified as shown in the below example for a treatment type called “Asphalt Patch Only.”

- Open the “Cost” tab and specify the dollar amount to attach to each treatment. The cost can be defined in terms of an equation or a C# function. Unit cost data for each treatment is obtained using data from past construction projects. Unit cost is averaged for pavements grouped based on District, Road Class, Functional Class Category, and IRI condition and updated annually. The source for these costs is the “Treatment Tree” Excel file06/12/2019. The user must copy and paste the cost functions for each treatment from the “Treatment Tree” Excel file into the text box as shown in the below image.

- Run the following SQL statement to determine TOTAL_BID, LANE_MILES_RESURFACED, and COST_PER_LANE_MILE by fiscal year:

select fiscal_year, sum(total_bid) total_bid, sum(inv_emp - inv_bmp) lane_ miles_resurfaced, sum(total_bid)/sum(inv_emp - inv_bmp) cost_per_lane_ mile from business_plan_lmy where fund = 77
and treatment in (select treatment from lu_treatment where is_resurfacing = 1) and total_bid >0 and fiscal_year <2100 and max_layer_fy = 1 group by fiscal_year order by fiscal_year;

- Calculate the inflation rate using the “Solver” tool in Excel as shown in the below image.

- Use the inflation rates calculated in Step 26 to determine the historical unit cost for various treatments by running the SQL statement in the “Treatment Tree” Excel file (see column AW in worksheet “2007-CurYear Fund77 Projects”).

- Copy and paste the results of the SQL statement from the previous step into the table “TBL_UNIT_COST_PRJ” in the same worksheet. Exclude outliers by calculating the nth% percentile lower and upper limit for unit cost by treatment (start with 5% for LL and 95% for UL, but these numbers will vary by treatment type). These calculations are stored in the same worksheet in a summary table. If cost data are not available for a specific treatment, then assume a cost based on judgment/experience. The cost data and functions will be automatically updated in the individual treatment worksheets as they are linked to the summary table.

- Review the unit cost data for all treatments before the unit cost functions are entered in RoadCare.

- Open the “Consequence” tab (see below image) and define what happens after a given treatment is selected. “Consequence” contains the “Attribute” that is affected, the “Change” that is applied, and the “Criteria”, which selects the appropriate consequence. An example of consequence definitions for treatment type “Asphalt Patch Only” is shown in the below example. Refer again to “Treatment Tree” Excel file for the most recent treatment consequences.

7- Committed Projects – performed by DAT analyst

Summary: Under this task, the parameters required for the “Committed” module are input in RoadCare.

- In the “Explorer” window, double-click on the fifth option, “Committed,” under the simulations module for the chosen network to display the main “Committed” window as shown in the below image. The available inputs include Facility, Start Year, Analysis Period, Advanced Search, as well as a grid with a list of facilities, sections, and years.

- To prescribe treatments, select a given “Facility” from the pull-down menu or use the”Advanced Search” option. Then, select a “Start Year” and the”Analysis Period;” usually, these two parameters match the investment input values. When the appropriate section is identified, click on the cell that corresponds to the year of interest.

- Subsequently, on the pane to the left labeled “Simulation Property,” select the treatment type, the budget category that should be used to cover the cost of the treatment, as well as the total cost. Alternatively, committed projects could be imported as a tab delimited text file with the required parameters06/12/2019. The treatments defined under committed are accounted as mandatory activities during the simulation. Click the “Import Committed Projects” button in the top right-hand corner of the Committed Projects window. This will open a dialog box that will allow the user to import the Committed projects file.

8- Run Simulation – performed by DAT analyst, ADC, and DC

Summary: Under this task, the simulation is run in RoadCare to optimize the budget allocation for the pavement network. The results of this task will then be reviewed, and if revisions are required, another iteration of the entire process will be completed with the discussed implemented changes.

- Click “Run Simulation” in the “Analysis” window.

- Following Generating Reports, generate the required reports to be reviewed.

- Send reports to ADC and DC for review.

  - If changes are required, discuss next actions. Make adjustments to the segmentation, data rollup, or simulation inputs as needed. When adjustments are complete, return to Step 34.

  - If no changes are required, the process is complete.

## Generating Reports

### General

This section describes the standard operating procedure (SOP) used by staff from the Data Analysis Team (DAT) of the Maryland Department of Transportation State Highway Administration (SHA) to generate reports for the “Conditional RSL/Cost Optimization” process in the RoadCare version 3.67 asset management software. This SOP is performed after the simulation is completed as described in Needs Analysis and Running Simulation Engine.

Note: The SOP described in this document may not be applicable to other versions of RoadCare.

### Frequency

The generation of reports is performed once per year, after the end of the data collection season, and once all data have been processed and migrated into the appropriate Business Plan Tables (see Migrating to Business Plan Tables). The reports may need to be generated multiple times during the iterative Condition RSL/Cost Optimization process.

### Purpose

The purpose of this SOP is to generate budget reports for the yearly RoadCare simulation, which allocates funding for the needs identified in the pavement network.

### Resource Requirements

Generating reports involves one person: a DAT staff member knowledgeable in RoadCare software. The estimated effort level shown in the table below represents the total time, in man-hours, to complete the report generation. This time estimate assumes no issues are encountered during the report generation process.

| Position | Function | Resources | Effort Level (man-hrs) |
| --- | --- | --- | --- |
| DAT Staff | Analyst | 1 | 1.0 |

### Procedure

The procedure to generate reports in RoadCare is comprised of the following two tasks:

- generate standard reports, and

- generate special reports.

1- Generate Standard Reports – performed by DAT Staff

Summary: Under this task, standard reports from a funding needs simulation can be generated using the RoadCare software.

- Open RoadCare software.

  - If the report that is to be generated is one of the standard reports, proceed to Step 8.

  - If the report that is to be generated is a special report, proceed to Step 8 in Task 2.

- Expand the options inside the “Networks” module by clicking on the “+” icon. Then, expand the options inside the “MDSHA_NETWORK” menu that appears in the list by clicking on the “+” icon.

- Expand the options inside the “Simulations” module by clicking on the “+” icon.

- Right-click on the name of the desired simulation and then click on “Simulation Reports.” A menu of all the possible reports will appear. Click on the desired report. All reports will be outputted into a Microsoft Excel 2007 spreadsheet. The types of simulation reports that can be generated are shown in the below image.

2- Generate Special Reports – performed by DAT Staff

Summary: Under this task, special reports from a funding needs simulation can be generated using the RoadCare software. For the purposes of the Condition RSL/Cost Optimization process, only the following three report types are relevant:

- “FY Targets and Suggested Projects Report”,

- “All Sections Shop DM Year & VMT Report”, and

- “Attribute View Report.”

- Select “Tools” from the RoadCare top menu and click on “Special Reports” from the drop-down menu, as shown in the below image.

- On the “Special Report Generation” window, select the appropriate “Network,” “Simulation,” and “Report Type” from the drop-down menus, as shown in the below image.

  - If “FY Target & Suggested Projects Report” is selected, proceed to Step 10.

  - If “All Sections Shop DM Year & VMT” is selected, skip to Step 15.

  - If the “Attribute View Report” is required, exit the “Special Report Generation” window and skip to Step 20.

- Click “Generate Report.”

- In the subsequent window, select the appropriate “Fiscal Year” and “Destination Folder” and then click “Finish,” as shown in the below image. The user has the option of choosing a custom template file that can be edited if there is a need to modify existing summaries. The user must generate a statewide report for the optimization year and a preliminary report for the subsequent year.

- This “FY Target & Suggested Projects Report” generates a list of suggested projects and a list of pre-defined attribute values for each project in a selected simulation for a given fiscal year. This report also includes a comprehensive summary of budget and treatment distributions, benefit, suggested lane-miles and average life extension categorized by District, Maintenance Shop, Functional Class, Last Treatment, Treatment Type, and RSL Category. Open the Microsoft Excel file generated by the RoadCare software and review both the “Targets” and “Suggested Projects” reports as shown in the below images06/12/2019.

- One of the optimization goals is to create realistic overall benefit targets that Districts must meet every year. Towards this end, the following actions must be reviewed06/12/2019:

  - Average unit cost for each treatment is compared with historical unit cost data and contractor bid tables for each District.

  - Suggested lane miles (LM) per treatment should be high enough to be contract worthy. The exact number of LM varies by District and treatment.

  - Average life extension should be within the range of expected values for the treatment.

  - Overall benefit (LMY) and $/LMY Target by District.

- If no other reports are required, the procedure is complete. Otherwise, return to Step 9.

- Click “Generate Report” to open the “All Sections Shop DM Year & VMT Report” window, which is shown in the below image.

- Load a stored profile if available by selecting the appropriate profile name under “Stored Profiles” and then clicking “Load.”

  - If a stored profile is loaded, skip to Step 18.

  - Otherwise, proceed to Step 17.

- If a profile does not already exist for a simulation, select the appropriate “Networkwide,” “Simulation,” and “Performance” measures in the three tables to the left of the above image. Check “As New,” then enter a profile name in the text box under “Current Profile” and click “Save.”

- Click “Choose Folder” to select a destination folder and then click “Finish” to start the report generation.

- The report will be saved as a Microsoft Excel 2007 macro-enabled workbook (.xlsm) in the folder selected by the user. This report shows performance data for selected sections from the current year, forecasted condition data, feasible treatments with their cost, benefit (LMY) and other relevant information across the entire network for data discovery and reference purposes.

Additionally, the report supports aliases and query expressions in standard Oracle SQL query syntax for selected measures. A couple of examples are shown below06/12/2019:

CI_FUNCTIONAL AS FCI
CASE WHEN NHS_CODE > 0 THEN 'Yes' ELSE 'No' END NHS

The report also includes a new interface, shown in the below image, which allows the user to identify potential project candidates by filtering the report based on specific criteria. To use this feature, the user must enable macros in the Microsoft Excel spreadsheet report.

- If no other reports are required, the procedure is complete. Otherwise, return to Step 9.

- Return to the “RoadCare Explorer” window on the left hand side of the screen. Expand the options inside the “Viewers” module for the appropriate network by clicking on the “+” icon. Then, expand the options inside the “Attribute View” menu that appears in the list by clicking on the “+” icon.

- Double-click on an attribute to see its contents on the display window, as illustrated in the below image.

- The data grid lists “FACILITY,” “SECTION,” and the selected attribute values. In addition, the “BEGIN_STATION,” “END_STATION,” and “DIRECTION” fields can be displayed by checking “Show Begin/End/Direction” at the top of the viewer window. Clicking on any data grid column title sorts all the displayed data in ascending/descending order based on that attribute. Some cells in the data grid may appear empty. This is because the raw database did not have an attribute value for that specific section. For those sections, the default value used when the raw attribute was created is assigned during the analysis/simulation process. At the bottom of the data grid, the line toolbar allows the user to move within records and to display the attribute values that result from running a simulation.

- To display attribute values from a desired financial needs simulation, select the simulation name from the “Simulation” drop-down menu.

- Right-click on the results grid area and select “Update” to display the corresponding calculated attribute values, as illustrated in the below image.

- Right-click on the results grid area and select the “Edit Columns” option to display the “Select Attribute View Columns” window, which is shown in the below image.

- From the “Attributes” drop-down menu located at the top of the “Select Attribute View Columns” window, select an attribute to display (e.g. AADT in the above example). If the attribute was included in the simulation criteria, future years will be available from the “Years Available (multiple select)” list. By default, the years are listed in ascending order; to reverse the order, uncheck the option “Order years most recent first.” The “Attribute View Display Column (multiple select)” contains the list of attributes that will be displayed. To add attributes to the display, highlight the desired attributes from the “Years Available (multiple select)” list and click the right-facing arrows icon shown in the below image.

- The highlighted fields are now listed in the pane to the right labeled “Attribute View Display Column (multiple select).” These attribute values will be displayed in the “Attribute” view window. The order of the attributes can be sorted by choosing the “Up” or “Down buttons next to the”Attribute View Display Column (multiple select)” pane. The order in which the attributes are listed on the “Attribute View Display Column (multiple select)” pane is the actual order in which they will be displayed in the attribute view data grid. Attributes can be deleted from that list by highlighting them and pressing “Delete.” The selected attributes will disappear from the list and also from the attribute view data grid. When the order is set, click “OK” to accept the selected display columns or click “Cancel” to go back to the previous screen.

- The selected attributes can be exported as a report in Microsoft Excel format. To generate the report, right-click on “Attribute View” under the “Viewers” menu on the “RoadCare Explorer Window” and select “Create Attribute View Report,” as illustrated in the below image.

- The report will be displayed in Microsoft Excel format. An example of an attribute report is shown in the below image.

- If no other reports are required, the procedure is complete. Otherwise, return to Step 9.

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Added alt text to all images |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Updated flowchart image to mermaid chart |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

# Appendices

## Skid Collection List SQL Statement

Use the following statement in Oracle SQL Developer to create the Skid Collection List. Any occurrence of “collection_2016” should be replaced with the current year’s date (e.g. for 2019, update to read “collection_2019”.)

--RUN IN PAV_CONHIST DEVELOPMENT
SELECT
    RSECTION,
    DISTRICT,
    CO,
    ID_PREFIX,
    ID_RTE_NO,
    MUNICIPALITY,
    MP_SUFFIX,
    EXIT_NUMBER,
    RAMP_NUMBER,
    D,
    IS_INV_DIR,
    BEGIN_DIR_MP,
    END_DIR_MP,
    ROAD_NAME_START,
    BEGIN_DESCRIPTION,
    END_DESCRIPTION,
    SECTION_LENGTH,
    COLL_CATEGORY,
    CASE
        WHEN COLL_CATEGORY IN (4, 8, 9, 10)
        AND DISTRICT IN (1, 2, 3, 4)
        AND SECTION_LENGTH >= 0.1
        THEN 'D1, D2, D3, D4 SHORT_ROADS_AND_RAMPS'
        WHEN COLL_CATEGORY IN (4, 8, 9, 10)
        AND DISTRICT IN (5, 6, 7, 8)
        AND SECTION_LENGTH >= 0.1
        THEN 'D5, D6, D7, BC SHORT ROADS AND RAMPS'
        WHEN COLL_CATEGORY IN (3)
        THEN 'NON-INVENTORY DIRECTION >= 1 MILE '
        WHEN COLL_CATEGORY IN (5, 7)
        THEN 'INVENTORY DIRECTION >= 1 MILE '
        WHEN ID_PREFIX IN ('DM', 'TL')
        THEN 'QC_TESTING'
        ELSE NULL
    END AS FRICTION_GROUP,
    -- PRIORITY ARAN_PRIORITY,
    GOVT_CONTROL,
    NHS_CODE,
    INV_BMP,
    INV_EMP,
    BEG_LAT,
    BEG_LONG,
    END_LAT,
    END_LONG,
    GLOBAL_ROUTE_ID,
    SUB_ROUTE_ID,
    ROUTE,
    TRIMROUTE,
    ROUTEID,
    MULTIPLE_BEG_INT,
    MULTIPLE_END_INT,
    RVISIKEY,
    ASSOCIATED_ID_PREFIX,
    FDIR,
    BEGIN_SECTION_DESC,
    BEGIN_SECTION_MILE_POINT,
    END_SECTION_DESC,
    END_SECTION_MILE_POINT,
    COLLECTION_YEAR
FROM
    (
        ----Includes SHA and MDTA Routes in the Inventory Direction >=1 mile
        SELECT
            *
        FROM
            collection_list
        WHERE
            collection_year = 2020
            AND GOVT_CONTROL IN (1, 31)
            -- AND IS_INV_DIR <> 1 --2019 Collection
            AND IS_INV_DIR = 1 --2020 Collection
            AND RAMP_NUMBER IS NULL
            --AND coll_category IN (3) --2019 Collection Non Inventory Direction
            AND coll_category IN (5, 7) --2020 Collection - Inventory Direction
        UNION
        -- Includes Test Loops and Dummy Files
        SELECT
            *
        FROM
            collection_list C
        WHERE
            id_prefix IN ('DM', 'TL')
            AND collection_year = 2020
        UNION
        --Includes Half of SHA and MDTA ramps and short roads >=0.10
        --(alternating years D1-D4, then D5-D8)
        SELECT
            *
        FROM
            collection_list C
        WHERE
            RAMP_NUMBER IS NOT NULL
            -- AND DISTRICT IN (1,2,3,4) --2019 Collection
            AND DISTRICT IN (5, 6, 7, 8) --2020 Collection
            AND collection_year = 2020
            AND GOVT_CONTROL IN (1, 31)
            AND COLL_CATEGORY IN (4, 8, 9, 10)
            AND SECTION_LENGTH >= .1
            AND (GLOBAL_ROUTE_ID, SUB_ROUTE_ID) IN (
                SELECT
                    GLOBAL_ROUTE_ID,
                    SUB_ROUTE_ID
                FROM
                    (
                        SELECT
                            GLOBAL_ROUTE_ID,
                            SUB_ROUTE_ID,
                            SUM(DIR_EMP - DIR_BMP) MILES
                        FROM
                            SECTION_TABLE
                        WHERE
                            HMIS_YEAR = 2016 --Most recent inventory year
                        GROUP BY
                            GLOBAL_ROUTE_ID,
                            SUB_ROUTE_ID
                    )
                WHERE
                    MILES >= 0.1
            )
    )
ORDER BY
    district,
    routeid,
    begin_dir_mp;

## Test Loop Data Analysis Details

Data QC

Data QC checks are performed for data such as the ARAN vehicle’s DMI, pavement imagery, IRI, rut, cracking (FCD and SCD), and CrossFall. The SHA test loop data collection (and therefore data analysis) is divided into the following two types:

- Initial data collection program (pre-collection); and

- Three week SHA test loop data collection program (throughout the FED collection and delivery season).

Pre-Collection Data Quality Acceptance - Initial Data Collection (10 SHA test loop runs before data collection)

For each ARAN vehicle and within 30 days of the beginning of FED’s ARAN collection season, 10 consecutive SHA test loops runs are performed and the resulting data QC’d and QA’d by the FED Pavement Testing Team (PTT) ARAN staff, and the deliverables are provided to the Pavement and Geotechnical Division (PAGD) of the DPT for network uploading and SQL database importing.

DMI values should be within the range of 1,000 ± 1 millimiles for the 1,000 millimiles long DMI section.

For the remaining measures, which include IRI, cross slope, rut, cracking (FCD, SCD, percent cracking for concrete), and friction06/12/2019, the following acceptance criteria is defined as follows:

- Results from each section are compared to the values predicted using the past five years of data for that section.

- Prediction is done using a linear trend line (linear regression) over the past five years of data (average section performance measure values for each year).

If the value for the current year is within the range defined by the equation06/12/2019 below, then the data are accepted, and if other QC checks pass then routine SHA data collection can start. Otherwise, the accuracy of the data should be investigated and the source of the issue should be addressed.

Current year’s predicted value ±

(last year’s 95th percentile – last year’s 5th percentile)/2

Analysis and acceptance of the SHA test loop data is conducted within 5 days of data collection. All parties involved are notified of the results.

Three Week Data Collection Program (3 SHA test loop runs every 3 weeks during data collection season)

For each SHA operated ARAN vehicle, 3 consecutive data collection runs are performed on 45 test loop sample sections, which provides the pavement management engineers with the opportunity to continuously control the accuracy and consistency of the ARAN data throughout the data collection season. The sample sections vary in length, pavement structure, pavement type, traffic level, number of lanes, distress level, etc. Once the test loop data have been collected and QC’d and QA’d by the FED PTT ARAN staff, the deliverables are then provided to the PAGD DPT for network uploading and SQL database importing.

If the average section performance measure value is within the range of the equation06/12/2019 below, then the data are accepted, and if the remaining QC checks pass, then the routine data collection can start for the ARAN vehicle in question. Otherwise, the accuracy of the data should be investigated and the source(s) of issue should be addressed.

Current year’s average value from initial collection Â±

(last year’s 95th percentile – last year’s 5th percentile)/2

Analysis and acceptance of the SHA test loop data is conducted within 5 days of data collection. All parties involved are notified of the results.

Error Resolution (Corrective Actions)

- Identifying issue sources:

  - Inquiry from FED for recent change of settings, maintenance activity, or suspicious observations.

  - Inquiry from DPT and DAT to make sure used protocols used are valid and the processing steps have been correctly followed.

  - Exploring other issues not considered above, such as but not limited to data collection equipment calibration, equipment configuration, data processing protocols, and data processing steps.

- Evaluating the data collected after the last acceptable SHA test loop runs to determine the affected files.

- Decision-making for addressing issue:

  - Evaluating affected data and all possible solutions for correcting the data.

  - Establishing a course of action for replacing the affected data with acceptable data and, correcting the data if possible or sending a recollection list to FED.

- If the source of error is an ARAN-related change (settings or physical) then:

  - Addressing issue by FED

  - New SHA test loop runs

  - SHA test loop QC check

  - If QC check, passed then start routine ARAN data collection (and recollection, if needed).

Test Loop Section Classifications

Three defined groups of SHA test loop sections are used for data QC purposes. The first group are Ground Truth (GT) sections. These sections have had manual inspection/detection performed along the section (entire or a part of section) by trained raters for one or more condition measures and/or will have manual detections performed in the future when needed. At present, the SHA test loop has DMI, IRI, rut, and crack GT sections, which are listed in the below table. However, based on the need for GT studies, additional GT sections can be added to the list in the future.

SHA Test Loop Ground Truth sections with DMI, IRI, rut, and crack GT sections

These GT sections are used as a basis for verification and modification of pavement data collection and data processing techniques, protocols, and procedures for the year in which the GT study is conducted. GT studies are typically performed when a change takes place in the data collection and/or processing procedures and data quality control is needed for validating the data.

DMI GT section: The DMI section is exactly one mile long and it is used for controlling the accuracy of the ARAN vehicle’s DMI.

IRI GT sections: The IRI GT testing was performed in the right wheel path of the IRI GT sections in 2015 using a SurPRO 3500. The results were compared with the ARAN IRI results for validation purposes.

Rut GT sections: The rut GT study was performed in 2013. Data were collected using the following four reference approaches:

- a 6-ft straightedge used to collect data in each half of the lane,

- a stringline stretched across the full width of the lane,

- a stringline stretched across 10-ft of the lane, and

- a stringline stretched across 8-ft of the lane.

The manual rut measurement methods were compared with each other and with the automated ARAN rut measurements. The study was performed to establish reference rut values for the rut GT sections and to modify the ARAN rut processing protocol to obtain the most precise and accurate data from ARAN measurements.

Crack GT sections: The crack GT sections are four short asphalt pavement sections with different distress levels and crack types. Unsealed and sealed cracks were manually traced on the pavement images along these sections for test loop run 8390S3H4, which was performed on March 9, 2018. These sections will be used as the basis for checking the accuracy and consistency of the automated crack detection by the ARAN data collected vehicle and Vision software.

Cracks on concrete pavements are detected manually. At present there are no concrete crack GT sections in the SHA test loop, but since it is a manual process, all the concrete sections of the first 10 ARAN test loop data collections (pre-collection test loops) can be used as GT sections for controlling the consistency of detectors throughout the collection season.

GT sections for other measures: The second group of SHA test loop sections are the sections that have 100% reliable data and are ideal for use in run comparisons or for research purposes. These sections are used for the 3-week SHA test loop data QC. These sections are relevant to the performance measure being evaluated in terms of pavement type. Since the condition data are affected by ARAN transitioning from one lane to another lane, sections including lane transition zones are not a part of this group of test loop sections. For example, roughness, rutting, and other measures are only compared if the section does not include a lane transition zone or do not have any other event such as debris, damp pavement, or milled pavement that can affect the data quality. These sections are used for performing data QC checks throughout the collection season.

These QC sections are presented in the following table (assuming that the lane transition zones are the only existing event and no other event such as lane change, debris, wet pavement, and so forth exists):

Table showing QC sections for 3-week SHA test loop data quality control with lane transition zone assumptions

The third group of SHA test loop QC sections includes those relevant sections for each performance measure excluding the zones with unreliable data such as data from transition zones, milled sections, damp pavements, and so forth. That is, only data from the flagged milepoints are excluded from calculations.

Standard Deviation

In the outlier review process, as outlined in Outlier Review, the outliers for each section are determined by means of predicting the current year performance measure using the past 5 years of data and using the standard deviation for that measure to compare the actual current performance measure with the predicted value. The standard deviation used in the outlier review process for each measure is determined using the current year test loop data. Since the sections studied in the outlier review are of different lengths, a single standard deviation for reviewing all the sections cannot be used. The test loop data are used to determine length dependent standard deviations for each measure. That is, for each performance measure, data from different lengths of test loop sections are compared to determine the section length-dependent equation of the standard deviation. The shorter sections are expected to have wider ranges of variability and as a result in higher standard deviations. Matching sections with an initial length of 0.1 mile and with incremental length additions of 0.1 mile are compared to build the standard deviation equation based on the section length.

## Fund Type

Each Contract has a fund type and it is standard that the last two digits of the seven digit FMIS number indicate the Fund Type. For Example: “PG6395177,” the “77” indicates a resurfacing and rehabilitation project. This Appendix is a complete list of the Capital Program Fund Categories.

### Major Projects Funding Categories

- 60 - Inter County Connector (ICC)

- 70 - Primary

- 71 - Secondary

- 72 - Interstate

- 73 - Woodrow Wilson Bridge

### System Preservation Funding Categories

- 11 - Federal/County

- 12 - Highway and Safety Operations

- 13 - Operations - A & G Projects

- 14 - Maintenance

- 15 - General Construction

- 18 - High Priority Local Projects

- 23 - Truck Weigh

- 24 - Environmental Preservation - Landscaping, wildflower seeding, reforestation and rest areas

- 25 - Transportation Enhancements - Urban greenways, rail-trail conversions, preservations of certain historic sites, landscaping and pedestrian and bicycle improvements

- 26 - Noise Barriers - Retrofit noise barriers along existing highways, barrier rehabilitation and noise berms

- 27 - Rest Areas

- 28 - Access Controls

- 29 - Facilities and Equipment

- 30 - Crash Prevention

- 31 - Pilot Program (Discontinued)

- 32 - Guardrail End Treatment

- 33 - ADA (Americans with Disabilities Act) Retrofit

- 38 - Communications

- 39 - Capital Equipment

- 46 - Statewide Planning & Research

- 49 - Environmental Compliance

- 68 - Stimulus Package

- 74 - Drainage Improvements - Areas of recurring flood damage or road closures

- 75 - Emergencies - Major storm damage, slope failures, sinkholes or other unforeseen roadway or bridge emergency (must be approved by MDOT)

- 76 - Safety and Spot Improvements - Safety improvements at high accident locations, intersection capacity improvements, slide repairs, roundabouts, ramp modifications and R/R crossings

- 77 - Resurfacing and Rehabilitation - Resurfacing, including concrete patching/joint sealing and pavement markings

- 78 - Pedestrian Access to Transit Stops

- 79 - Sidewalks - Construction of retrofit sidewalks along State highways and reconstruction of replacement of existing sidewalks if a part of a revitalization effort in an officially designated urban revitalization area

- 80 - Bridge Replacement & Rehabilitation - Bridge replacement, deck replacements, major rehabilitations, deck overlays, parapet modifications, repainting and spot painting and all structure condition inspections

- 81 - Park and Ride - Park and Ride Lots, lot expansions and lighting

- 82 - TMDL Compliance - Total Maximum Daily Load for stormwater management

- 83 - Urban Street Reconstruction - Rehabilitation through urban areas that include pavement and drainage reconstruction. Projects may include local participation for sidewalks, street furniture, landscaping and other urban amenity improvements

- 84 - Community Safety & Enhancements - Improvements where the emphasis is on enhancing the existing infrastructure to promote economic revitalization such as resurfacing, reconstructing drainage, curb and gutter, landscaping, signing, parking bays and lighting (formerly Neighborhood Conservation)

- 85 - Traffic Management - New signals, signal system construction, signal reconstruction, raised pavement markers, lighting and signing

- 86 - CHART (Coordinated Highways Action Response Team) - Advanced traffic management systems

- 87 - Intersection Capacity

- 88 - Bicycle Retrofit

- 89 - Not in use

- 99 - Billwork

## POSPac List Files Macro

Copy and paste as directed in Step 3 of Post-Processing of ARAN GPS Data.

Sub MainList()
    'Updateby20150706
    Set folder = Application.FileDialog(msoFileDialogFolderPicker)
    If folder.Show <> -1 Then Exit Sub
    xDir = folder.SelectedItems(1)
    Call ListFilesInFolder(xDir, True)
End Sub

Sub ListFilesInFolder(ByVal xFolderName As String, ByVal xIsSubfolders As Boolean)
    Dim xFileSystemObject As Object
    Dim xFolder As Object
    Dim xSubFolder As Object
    Dim xFile As Object
    Dim rowIndex As Long

    Set xFileSystemObject = CreateObject("Scripting.FileSystemObject")
    Set xFolder = xFileSystemObject.GetFolder(xFolderName)
    rowIndex = Application.ActiveSheet.Range("A65536").End(xlUp).Row + 1

    For Each xFile In xFolder.Files
        Application.ActiveSheet.Cells(rowIndex, 1).Formula = xFile.Name
        rowIndex = rowIndex + 1
    Next xFile

    If xIsSubfolders Then
        For Each xSubFolder In xFolder.SubFolders
            ListFilesInFolder xSubFolder.Path, True
        Next xSubFolder
    End If

    Set xFile = Nothing
    Set xFolder = Nothing
    Set xFileSystemObject = Nothing
End Sub

Function GetFileOwner(ByVal xPath As String, ByVal xName As String)
    Dim xFolder As Object
    Dim xFolderItem As Object
    Dim xShell As Object

    xName = StrConv(xName, vbUnicode)
    xPath = StrConv(xPath, vbUnicode)

    Set xShell = CreateObject("Shell.Application")
    Set xFolder = xShell.Namespace(StrConv(xPath, vbFromUnicode))

    If Not xFolder Is Nothing Then
        Set xFolderItem = xFolder.ParseName(StrConv(xName, vbFromUnicode))
    End If

    If Not xFolderItem Is Nothing Then
        GetFileOwner = xFolder.GetDetailsOf(xFolderItem, 8)
    Else
        GetFileOwner = ""
    End If

    Set xShell = Nothing
    Set xFolder = Nothing
    Set xFolderItem = Nothing
End Function

## Base Report

Below are the parameters required to generate the base report for collected ARAN data.

Full Output in Stacked Format:

Column: IDLocator

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: IDSession

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: BeginDistanceStamp

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: EndDistanceStamp

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: BeginChainage

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: EndChainage

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: Status

- Aggregation: -

- Conditional Split: -

- Transformation: -

- Description: -

- Type: -

Column: REPORTING_YEAR

- Aggregation: FIRST

- Conditional Split:

- Transformation: ‘2017’

- Description:

- Type: Text

Column: COLLECT_DATE

- Aggregation: FIRST

- Conditional Split:

- Transformation: [DCSessions].[DCSTimeStamp]

- Description:

- Type: Text

Column: COUNTY

- Aggregation: FIRST

- Conditional Split:

- Transformation: [Locators].[L_County]

- Description:

- Type: Text

Column: MUNICIPALITY

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[MUNICIPALITY]

- Description:

- Type: Text

Column: ID_PREFIX

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[ID_PREFIX]

- Description:

- Type: Text

Column: ID_ROUTE_NO

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[ID_RTE_NO]

- Description:

- Type: Text

Column: MP_SUFFIX

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[MP_SUFFIX]

- Description:

- Type: Text

Column: DIRECTION

- Aggregation: FIRST

- Conditional Split:

- Transformation: [Locators].[L_Dir]

- Description:

- Type: Text

Column: EXIT_NUMBER

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[EXIT_NUMBER]

- Description:

- Type: Text

Column: RAMP_NUMBER

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[RAMP_NUMBER]

- Description:

- Type: Text

Column: ASSOCIATED_ID_PREFIX

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[ASSOCIATED_ID_PREFIX]

- Description:

- Type: Text

Column: GLOBAL_ROUTE_ID

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[GLOBAL_ROUTE_ID]

- Description:

- Type: NUMBER

Column: SUB_ROUTE_ID

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RoutedSegments].[SUB_ROUTE_ID]

- Description:

- Type: NUMBER

Column: ROUTEID

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {RoutedSegments, FIRST([ROUTEID]), , 0, 0}

- Description: HMIS ROUTEID, AS INPUT IN THE EFS

- Type: Text

Column: DIR_BMP

- Aggregation: FIRST

- Conditional Split:

- Transformation:

- Description: Will be populated in Oracle. Beginchainage/1000 for “matched” records

- Type: NUMBER

Column: DIR_EMP

- Aggregation: FIRST

- Conditional Split:

- Transformation:

- Description: Will be populated in Oracle. Enddistancestamp/1000 for “matched” records

- Type: NUMBER

Column: INV_BMP

- Aggregation: FIRST

- Conditional Split:

- Transformation:

- Description: Will be populated in Oracle.

- Type: NUMBER

Column: INV_EMP

- Aggregation: FIRST

- Conditional Split:

- Transformation:

- Description: Will be populated in Oracle

- Type: NUMBER

Column: UNIQUERUN

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {DCSessions, FIRST([UniqueRun]), , 0, 0}

- Description:

- Type: Text

Column: COLLECTION_VEHICLE

- Aggregation: FIRST

- Conditional Split:

- Transformation: [DCSessions].[CollectionVehicle]

- Description:

- Type: NUMBER

Column: VERSION

- Aggregation: FIRST

- Conditional Split:

- Transformation: [DCSessions].[ARANVersion]

- Description:

- Type: Text

Column: SAT_NUMBER

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, SUM([Satellites]), , 0, 0}

- Description:

- Type: NUMBER

Column: STATION_TIME

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {StationData, AVG([StationTime]), , 0, 0}

- Description:

- Type: NUMBER

Column: ELAPSED_TIME

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {StationData, AVG([ElapsedTime]), , 0, 0}

- Description:

- Type: NUMBER

Column: SPEED

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {StationData, AVG([Speed]), , 0, 0}*2.23694

- Description: M/H

- Type: NUMBER

Column: SPEED_FIRSTRAW

- Aggregation: FIRST

- Conditional Split:

- Transformation: [StationData].[Speed]*2.23694

- Description: M/H

- Type: NUMBER

Column: GPS_LAT_BEGIN

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, FIRST([Latitude]), , 0, 0}*57.2958

- Description:

- Type: NUMBER

Column: GPS_LONG_BEGIN

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, FIRST([Longitude]), , 0, 0}*57.2958

- Description:

- Type: NUMBER

Column: GPS_LAT_END

- Aggregation: LAST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, LAST([Latitude]), , 0, 0}*57.2958

- Description:

- Type: NUMBER

Column: GPS_LONG_END

- Aggregation: LAST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, LAST([Longitude]), , 0, 0}*57.2958

- Description:

- Type: NUMBER

Column: ELEV_AVG

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, AVG([Elevation]), , 0, 0}*3.2808399

- Description: FT

- Type: NUMBER

Column: GRADE_AVG

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {VehicleOrientations, AVG([Grade]), , 0, 0}*100

- Description:

- Type: NUMBER

Column: L_FAULTING_COUNT

- Aggregation: COUNT

- Conditional Split:

- Transformation: COMPUTE {ConcreteFaults, AVG([Height]), [location]=‘LeftFault’, 0, COUNT 0}

- Description:

- Type: NUMBER

Column: L_FAULTING_HEIGHT

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {ConcreteFaults, FIRST([Height]),[location]=‘LeftFault’ , 0, 0}*39.3701

- Description: IN

- Type: NUMBER

Column: R_FAULTING_COUNT

- Aggregation: COUNT

- Conditional Split:

- Transformation: COMPUTE {ConcreteFaults, AVG([Height]),[location]=‘RightFault’ , 0, 0}

- Description:

- Type: NUMBER

Column: R_FAULTING_HEIGHT

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {ConcreteFaults, FIRST([Height]),[location]=‘RightFault’ , 0, 0}*39.3701

- Description: IN

- Type: NUMBER

Column: FAULTING_LCMS_ABS_AVG_IN

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Faults, AVG([ABS_AvgFault]), , 0, 0}

- Description: Absolute Value Faulting, IN

- Type: NUMBER

Column: FAULTING_LCMS_MIN_IN

- Aggregation: MIN

- Conditional Split:

- Transformation: COMPUTE {FaultsReport, MIN([FaultMeasurement*1000 / 25.4]), , 0, 0}

- Description: Minimum Fault Value. Not the absolute value, just the minimum raw value, which can be negative., IN

- Type: NUMBER

Column: FAULTING_LCMS_MAX_IN

- Aggregation: MAX

- Conditional Split:

- Transformation: COMPUTE {FaultsReport, MAX([FaultMeasurement*1000 / 25.4]), , 0, 0}

- Description: Maximum faulting value, IN

- Type: NUMBER

Column: ROLL

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {TransverseData, AVG([Roll]), , 0, 0}*100

- Description:

- Type: NUMBER

Column: PITCH

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {AttitudeData, AVG([Pitch]), , 0, 0}*100

- Description:

- Type: NUMBER

Column: ELEV_BEGIN

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, FIRST([Elevation]), , 0, 0}*3.2808399

- Description: FT

- Type: NUMBER

Column: ELEV_END

- Aggregation: LAST

- Conditional Split:

- Transformation: COMPUTE {VehiclePositions, LAST([Elevation]), , 0, 0}*3.2808399

- Description: FT

- Type: NUMBER

Column: L_IRI_FIRSTRAW

- Aggregation: FIRST

- Conditional Split:

- Transformation: [StationData].[LeftIRI]*63.36

- Description: IN/MILE

- Type: NUMBER

Column: R_IRI_FIRSTRAW

- Aggregation: FIRST

- Conditional Split:

- Transformation: [StationData].[RightIRI]*63.36

- Description: IN/MILE

- Type: NUMBER

Column: L_RUT_FIRSTRAW

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RutData].[LeftRut]*1000/25.4

- Description: IN

- Type: NUMBER

Column: R_RUT_FIRSTRAW

- Aggregation: FIRST

- Conditional Split:

- Transformation: [RutData].[RightRut]*1000/25.4

- Description: IN

- Type: NUMBER

Column: EVENT_DEBRIS_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Debris’, 0, 0}>0;

- Transformation: 1; 0

- Description:

- Type: NUMBER

Column: EVENT_STOPBAR_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Stop_Bar’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_LANE_DEVIATION_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Lane_Deviation’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_MILLED_SURFACE_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_BRIDGE_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_CRACK_SEAL_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Sealed_Cracking_Exists’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_CROSSWALK_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Crosswalk’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_RAILROAD_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_DISREGARD_CRACK_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_BRICK_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_BAD_IMAGE_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_CRCP_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_JCP_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_WET_SURFACE_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Wet_Surface’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_PAVE_CHANGE_OFFICE

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Pave_Change’, 0, 0}>=0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: JCP_CRACKED_SLAB

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {ManualDistressesValues, COUNT([DistressTypeName]),[DistressTypeName]=‘JCP_Cracked_Slab’, 0, 0}

- Description: COUNT

- Type: NUMBER

Column: JCP_INTACT_SLAB

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {ManualDistressesValues, COUNT([DistressTypeName]),[DistressTypeName]=‘Intact_JCP_Slab’, 0, 0}

- Description: COUNT

- Type: NUMBER

Column: CRCP_PATCH_CONCRETE

- Aggregation: SUM

- Conditional Split: COMPUTE {ManualDistressesValues, SUM([Area*10.7639]), [DistressTypeName]=‘CRCP_Patch_Concrete’, 0, 0}>0;

- Transformation: COMPUTE {ManualDistressesValues, SUM([Area*10.7639]), [DistressTypeName]=‘CRCP_Patch_Concrete’, 0, 0};0

- Description: SQFT

- Type: NUMBER

Column: CRCP_PATCH_ASPHALT

- Aggregation: SUM

- Conditional Split: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName]=‘CRCP_Patch_Asphalt’, 0, 0}>0;

- Transformation: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName]=‘CRCP_Patch_Asphalt’, 0, 0};0

- Description: SQFT

- Type: NUMBER

Column: CRCP_PUNCHOUT

- Aggregation: SUM

- Conditional Split: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName] = ‘CRCP_Punchout’, 0, 0}>0;

- Transformation: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName] = ‘CRCP_Punchout’, 0, 0};0

- Description: SQFT

- Type: NUMBER

Column: CRCP_LONG

- Aggregation: SUM

- Conditional Split: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName] = ‘CRCP_Long’, 0, 0}>0;

- Transformation: COMPUTE {ManualDistressesValues, SUM([Area * 10.7639]), [DistressTypeName] = ‘CRCP_Long’, 0, 0};0

- Description: SQFT

- Type: NUMBER

Column: EVENT_LANE_DEVIATION_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Lane’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_CONSTRUCTION_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Construction’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_NEW_PAVEMENT_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘New’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_MILLED_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Milled’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_ROUNDABOUT_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Roundabout’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_WET_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Wet’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_DEBRIS_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedSpanEvents, FIRST([Length]),[EventName]=‘Desbris’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_ROAD_CLOSED_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedPointEvents, FIRST([MaxSeparation]),[EventName]=‘Closed’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_RUMBLE_STRIP_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedPointEvents, FIRST([MaxSeparation]),[EventName]=‘Strips’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: EVENT_TRAFFIC_FIELD

- Aggregation: MAX

- Conditional Split: COMPUTE {CollectedPointEvents, FIRST([MaxSeparation]),[EventName]=‘Traffic’, 0, 0}>0;

- Transformation: 1;0

- Description:

- Type: NUMBER

Column: ImageROW

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([JPEGNAME]), [Name] = ‘ROW’, 1, 1}

- Description:

- Type: TEXT

Column: ImageLeft

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([JPEGNAME]), [Name] = ‘Left’, 1, 1}

- Description:

- Type: TEXT

Column: ImagePavement_Optical

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([JPEGNAME]), [Name] = ‘Pavement’, 0, 0}

- Description:

- Type: TEXT

Column: ImagePavement_Depth

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([JPEGNAME]), [Name] = ‘LCMSRange’, 0, 0}

- Description:

- Type: TEXT

Column: ImagePavement_3D

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([JPEGNAME]), [Name] = ‘LCMS3D’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGEPATH_ROW

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagepath]), [Name] = ‘ROW’ , 0, 0}

- Description:

- Type: TEXT

Column: IMAGEPATH_LEFT

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagepath]), [Name] = ‘LEFT’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGEPATH_PAVEMENT_OPTICAL

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagepath]), [Name] = ‘Pavement’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGEPATH_PAVEMENT_DEPTH

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagepath]), [Name] = ‘LCMSRange’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGEPATH_PAVEMENT_3D

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagepath]), [Name] = ‘LCMS3D’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGENAME_ROW

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([IMAGENAME]), [Name] = ‘ROW’ , 0, 0}

- Description:

- Type: TEXT

Column: IMAGENAME_LEFT

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagename]), [Name] = ‘LEFT’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGENAME_PAVEMENT_OPTICAL

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagename]), [Name] = ‘PAVEMENT’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGENAME_PAVEMENT_DEPTH

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagename]), [Name] = ‘LCMSRange’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGENAME_PAVEMENT_3D

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, LASTKNOWN([imagename]), [Name] = ‘LCMS3D’, 0, 0}

- Description:

- Type: TEXT

Column: IMAGEFRAME

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Image, FIRST([imageframe]), , 0, 0}

- Description:

- Type: TEXT

Column: LANEWIDTH_AVG

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Lanewidth, AVG([Lanewidth]), [lanewidth]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LANEWIDTH_FIRST

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {Lanewidth, FIRST([Lanewidth]), [lanewidth]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LeftOffset

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([LeftOffset]),[IsValid] = 1 and [RutCalculationMethod] = ‘Wire’, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: RightOffset

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([RightOffset]),[IsValid] = 1 and [RutCalculationMethod] = ‘Wire’, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: CrossFallinPercent

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([CrossFallInPercent]),[IsValid] =1 and [RutCalculationMethod] = ‘Wire’, 0, 0}

- Description:

- Type: NUMBER

Column: CrossFallinRadians

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([CrossfallInRadians]),[IsValid] =1 and [RutCalculationMethod] = ‘Wire’ , 0, 0}

- Description:

- Type: NUMBER

Column: LeftPondDepth

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, FIRST([LeftPondDepth]), [LeftPondIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’ , 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: RightPondDepth

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([RightPondArea]), [RightPondIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’ , 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: LeftPondArea

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([LeftPondArea]),[LeftPondIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’, 0, 0}(1000/25.4)(1000/25.4)

- Description: SQIN

- Type: NUMBER

Column: RightPondArea

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([RightPondArea]),[RightPondIsValid]=1 and [IsValid] = 1 and [RutCalculationMethod] = ‘Wire’, 0, 0}(1000/25.4)(1000/25.4)

- Description: SQIN

- Type: NUMBER

Column: LaneOffset

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([LaneOffset]), [IsValid] = 1 and [RutCalculationMethod] = ‘Wire’, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: RutCalculationMethod

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, FIRST([RutCalculationMethod]), , 0, 0}

- Description:

- Type: TEXT

Column: LeftEdgeType

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, FIRST([LeftEdgeType]),[IsValid] = 1 and [LeftEdgeIsValid] =1 and [RutCalculationMethod] = ‘Wire’ , 0, 0}

- Description:

- Type: TEXT

Column: RightEdgeType

- Aggregation: FIRST

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, FIRST([RightEdgeType]),[IsValid] = 1 and [RightEdgeIsValid] = 1 and [RutCalculationMethod] = ‘Wire’ , 0, 0}

- Description:

- Type: TEXT

Column: L_RUT_WIRE

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([LeftRut]), [LeftRutIsValid] =1 and [IsValid] = 1 and [RutCalculationMethod] = ‘Wire’, 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: R_RUT_WIRE

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {CalculatedRutValues, AVG([RightRut]), [RightRutIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’, 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: AVG_WIRE_RUT

- Aggregation: AVG

- Conditional Split:

- Transformation: ((COMPUTE {CalculatedRutValues, AVG([LeftRut]), [LeftRutIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’, 0, 0}1000/25.4)+(COMPUTE {CalculatedRutValues, AVG([RightRut]), [RightRutIsValid]=1 and [IsValid]=1 and [RutCalculationMethod] = ‘Wire’, 0, 0}1000/25.4))/2

- Description: IN

- Type: NUMBER

Column: L_RUT_RAW

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RutData, AVG([LeftRut]), , 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: R_RUT_RAW

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RutData, AVG([RightRut]), , 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: AVG_RUT_RAW

- Aggregation: AVG

- Conditional Split:

- Transformation: (COMPUTE {RutData, AVG([LeftRut]), , 0, 0}1000/25.4 + COMPUTE {RutData, AVG([RightRut]), , 0, 0}1000/25.4)/2

- Description: IN

- Type: NUMBER

Column: L_IRI_ADJ_SP_CH

- Aggregation: AVG

- Conditional Split:

- Transformation:

- Description: This field is intentionally left blank when reported out of Vision. It will be populated with adjusted left IRI values to consider the change in travel speed of the vehicle during collection., IN/MILE

- Type: NUMBER

Column: R_IRI_ADJ_SP_CH

- Aggregation: AVG

- Conditional Split:

- Transformation:

- Description: This field is intentionally left blank when reported out of Vision. It will be populated with adjusted right IRI values to consider the change in travel speed of the vehicle during collection., IN/MILE

- Type: NUMBER

Column: PERC_IRI_FROM_SP_CH

- Aggregation: AVG

- Conditional Split:

- Transformation:

- Description: This field is intentionally left blank when reported out of Vision. It will be populated with a code indicating the extent and type of vehicle acceleration / deceleration during collection., IN/MILE

- Type: NUMBER

Column: L_IRI

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {PProcRoughness, AVG([LeftIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: R_IRI

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {PProcRoughness, AVG([RightIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: L_IRI_STATIONDATA

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {StationData, AVG([LeftIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: R_IRI_STATIONDATA

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {StationData, AVG([RightIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: L_IRI_PROCESSED

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {PProcRoughness, AVG([LeftIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: R_IRI_PROCESSED

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {PProcRoughness, AVG([RightIRI]), , 0, 0}*63.36

- Description: IN/MILE

- Type: NUMBER

Column: RAVELING_AFFECTED_PERC

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RavelingReport, AVG([AffectedPercentage]), , 0, 0}

- Description:

- Type: NUMBER

Column: RAVELING_MEAN_RI_CM3M2

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RavelingReport, AVG([MeanRI]), , 0, 0}

- Description:

- Type: NUMBER

Column: RAVELING_MEAN_EXIST_RI_CM3M2

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RavelingReport, AVG([MeanExistingRI]), , 0, 0}

- Description:

- Type: NUMBER

Column: RAVELING_MEAN_RPI_CM3M2

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RavelingReport, AVG([MeanRPI]), , 0, 0}

- Description:

- Type: NUMBER

Column: RAVELING_MEAN_AVC_CM3M2

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {RavelingReport, AVG([MeanAVC]), , 0, 0}

- Description:

- Type: NUMBER

Column: JOINT_LENGTH_TRANS_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {JointsReport, SUM([Length]),[Transverse]=1 , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: JOINT_HEIGHT_TRANS_IN

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {JointsReport, AVG([Height]), [Transverse]=1 , 0, 0}*39.37008

- Description: IN

- Type: NUMBER

Column: JOINT_LENGTH_LONG_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {JointsReport, SUM([Length]),[Transverse]=0 , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: JOINT_HEIGHT_LONG_IN

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {JointsReport, AVG([Height]), [Transverse]=0 , 0, 0}*39.37008

- Description: IN

- Type: NUMBER

Column: POTHOLE_AREA_L_SQIN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, SUM([AREA]), [SEVERITY] = ‘Low’, 0, 0}1000/25.41000/25.4

- Description: SQIN

- Type: NUMBER

Column: POTHOLE_MAX_DEPTH_L_IN

- Aggregation: MAX

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, MAX([MAXIMUMDEPTH]), [SEVERITY] = ‘Low’, 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: POTHOLE_AREA_M_SQIN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, SUM([AREA]), [SEVERITY] = ‘Moderate’, 0, 0}1000/25.41000/25.4

- Description: SQIN

- Type: NUMBER

Column: POTHOLE_MAX_DEPTH_M_IN

- Aggregation: MAX

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, MAX([MAXIMUMDEPTH]), [SEVERITY] = ‘Moderate’, 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: POTHOLE_AREA_H_SQIN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, SUM([AREA]), [SEVERITY] = ‘High’, 0, 0}1000/25.41000/25.4

- Description: SQIN

- Type: NUMBER

Column: POTHOLE_MAX_DEPTH_H_IN

- Aggregation: MAX

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, MAX([MAXIMUMDEPTH]), [SEVERITY] = ‘High’, 0, 0}*1000/25.4

- Description: IN

- Type: NUMBER

Column: POTHOLE_DEPTH_AREA_INSQIN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {POTHOLESREPORT, SUM([MAXIMUMDEPTH*AREA]), , 0, 0}1000/25.41000/25.4*1000/25.4

- Description: CUBIC IN

- Type: NUMBER

Column: MPD_LE_IN

- Aggregation: AVG

- Conditional Split: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 1, 0, 0}

- Transformation: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 1, 0, 0}

- Description: IN

- Type: NUMBER

Column: MPD_LWP_IN

- Aggregation: AVG

- Conditional Split: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 2, 0, 0}>0

- Transformation: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 2, 0, 0}

- Description: IN

- Type: NUMBER

Column: MPD_CTR_IN

- Aggregation: AVG

- Conditional Split: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 3, 0, 0} >0

- Transformation: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 3, 0, 0}

- Description: IN

- Type: NUMBER

Column: MPD_RE_IN

- Aggregation: AVG

- Conditional Split: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 5, 0, 0} >

- Transformation: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 5, 0, 0}

- Description: IN

- Type: NUMBER

Column: MPD_RWP_IN

- Aggregation: AVG

- Conditional Split: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 4, 0, 0} >

- Transformation: COMPUTE {PProcTexture, AVG([MPD ^(Inch)]), [wheelpath] = 4, 0, 0}

- Description: IN

- Type: NUMBER

Column: PVT_TY_ASPHALT_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, SUM([Length]),[eventtypename] = ‘LCMS’ and [Eventscorename] =‘Asphalt’ , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: PVT_TY_CONCRETE_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, SUM([Length]),[eventtypename] = ‘LCMS’ and [Eventscorename] =‘Concrete’ , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: PVT_TY_GROOVED_CONC_LONG_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, SUM([Length]),[eventtypename] = ‘LCMS’ and [Eventscorename] =‘Grooved Concrete - Longitudinal’ , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: PVT_TY_GROOVED_CONC_TRANS_FT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, SUM([Length]),[eventtypename] = ‘LCMS’ and [Eventscorename] =‘Grooved Concrete - Transversal’ , 0, 0}*3.28084

- Description: FT

- Type: NUMBER

Column: PVT_TY_LEFT_JOINT_CT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, COUNT([Eventscorename]),[EventTypeName]=‘LCMS’ and [Eventscorename] =‘Left Joint’, 0, 0}

- Description: COUNT

- Type: NUMBER

Column: PVT_TY_RIGHT_JOINT_CT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedEvents, COUNT([Eventscorename]),[EventTypeName]=‘LCMS’ and [Eventscorename] =‘Right Joint’, 0, 0}

- Description: COUNT

- Type: NUMBER

Column: EXCLUDED_FROM_CRACK_DATA

- Aggregation: MAX

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0

- Transformation: 1;1;1;1;1;1;1;1;0

- Description:

- Type: NUMBER

Column: LONGITUDINAL_LWP_MAP21EXTENT

- Aggregation: SUM

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0

- Transformation: NULL:NULL;NULL:NULL;NULL:NULL:NULL:NULL;COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] in (‘Longitudinal Crack (LWP)’,‘Longitudinal Sealed Crack (LWP)’) and [MetricName] = ‘CrackExtent’ and [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONGITUDINAL_RWP_MAP21EXTENT

- Aggregation: SUM

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0

- Transformation: NULL:NULL;NULL:NULL;NULL:NULL:NULL:NULL;COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] in (‘Longitudinal Crack (RWP)’,‘Longitudinal Sealed Crack (RWP)’) and [MetricName] = ‘CrackExtent’ and [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: SC_CRACK_AREA

- Aggregation: SUM

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0

- Transformation: NULL:NULL;NULL:NULL;NULL:NULL:NULL:NULL;COMPUTE {PIVOT_Distress, SUM([CrackLength * Width]), [DistressTypeName] in (‘Longitudinal Crack (LWP)’, ‘Longitudinal Crack (RWP)’) and [Length]>0,0, 0}3.2808398953.280839895

- Description: SQFT

- Type: NUMBER

Column: FC_CRACK_AREA

- Aggregation: SUM

- Conditional Split: COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Milled_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bridge’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Railroad’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Disregard_Cracking’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Brick_Pavement’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_Bad_Image’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_CRCP_Exists’, 0, 0}>0;COMPUTE {manualdistressesvalues, FIRST([Length]),[distresstypename]=‘Event_JCP_Exists’, 0, 0}>0

- Transformation: NULL:NULL;NULL:NULL;NULL:NULL:NULL:NULL;COMPUTE {PIVOT_Distress, SUM([CrackLength * Width]), [DistressTypeName] in (‘Longitudinal Crack (CTR)’, ‘Longitudinal Crack (LE)’, ‘Longitudinal Crack (RE)’,‘Transverse Crack’) and [Length]>0,0, 0}3.2808398953.280839895

- Description: SQFT

- Type: NUMBER

Column: LONGITUDINAL_LE_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Sealed Crack (LE)’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONGITUDINAL_LWP_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Sealed Crack (LWP)’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONGITUDINAL_CTR_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Sealed Crack (CTR)’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONGITUDINAL_RWP_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Sealed Crack (RWP)’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONGITUDINAL_RE_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Sealed Crack (RE)’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANSVERSE_ALL_SEALED

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Sealed Crack’ and [SeverityName] = ‘SEALED’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LE_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LE)’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_LWP_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (LWP)’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_CTR_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (CTR)’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RWP_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RWP)’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: LONG_RE_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Longitudinal Crack (RE)’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_16

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_16’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_3_8

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_3_8’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_3_4

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_3_4’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_1_1_2

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_1_1_2’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_2_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_2_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_3_IN

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_3_in’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: TRANS_CW_OVER3

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {RatedDistressesValues, SUM([MetricValue]), [DistressTypeName] = ‘Transverse Crack’ and [SeverityName] = ‘crackwidth_Over3’ and [MetricName] = ‘CrackLength’ AND [Length]>0, 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: BLEEDING_PERCENT_LWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([Percentage]), [WheelPath]=‘Left’, 0, 0}

- Description:

- Type: NUMBER

Column: BLEEDING_INDEX_LWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([INDEX]), [WheelPath]=‘Left’, 0, 0}

- Description:

- Type: NUMBER

Column: BLEEDING_SEVERITY_LWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([SEVERITY]), [WheelPath]=‘Left’, 0, 0}

- Description:

- Type: NUMBER

Column: BLEEDING_PERCENT_RWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([Percentage]), [WheelPath]=‘Right’, 0, 0}

- Description:

- Type: NUMBER

Column: BLEEDING_INDEX_RWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([INDEX]), [WheelPath]=‘Right’, 0, 0}

- Description:

- Type: NUMBER

Column: BLEEDING_SEVERITY_RWP

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {Bleedings, AVG([SEVERITY]), [WheelPath]=‘Right’, 0, 0}

- Description:

- Type: NUMBER

Column: PICKOUT_AREA_SQFT

- Aggregation: SUM

- Conditional Split:

- Transformation: COMPUTE {PickoutReport, SUM([Area]), , 0, 0}3.2808398953.280839895

- Description: SQFT

- Type: NUMBER

Column: PICKOUT_RADIUS_FT

- Aggregation: AVG

- Conditional Split:

- Transformation: COMPUTE {PickoutReport, AVG([Radius]), , 0, 0}*3.280839895

- Description: FT

- Type: NUMBER

Column: PICKOUT_COUNT

- Aggregation: COUNT

- Conditional Split:

- Transformation: COMPUTE {PickoutReport, COUNT([Distancestamp]), , 0, 0}

- Description: COUNT

- Type: NUMBER

## IRI Speed Adjustment

Below are the steps followed to establish the equations for the IRI adjustment based on change in speed (updated January 26, 2018).

- Calculate the change in speed (current record speed – previous record speed) between consecutive data stations and average IRI values (only valid if less than 9999) for a specific number of previous and next records depending on the data collection year (see table below for historical data).

| YEAR | Data Collection Interval Miles (Feet) | Previous Records (from current record) for Analysis (Distance in feet) | Next Records (from current record) for Analysis (Distance in feet) |
| --- | --- | --- | --- |
| 1995 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 1996 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 1997 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 1998 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 1999 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 2000 | 0.02 (105.6) | 6 (633.6) | 6 (633.6) |
| 2001 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2002 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2003 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2004 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2005 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2006 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2007 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2008 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2009 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2010 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2011 | 0.01 (52.8) | 12 (633.6) | 12 (633.6) |
| 2012 | 0.005 (26.4) | 24 (633.6) | 24 (633.6) |
| 2013 | 0.004 (21.1) | 30 (633) | 30 (633) |
| 2014 | 0.004 (21.1) | 30 (633) | 30 (633) |
| 2015 | 0.004 (21.1) | 30 (633) | 30 (633) |

- Plot the graphs for each year with previous and next record numbers (-5 to +5) in the x-axis and IRI values at each speed change (acceleration and deceleration) on the y-axis. Speed change occurs at the current station and only speed change values between -5 and 5 are selected for analysis since more than 95% of the data have the speed change values within this range (see sample graph shown below).

- Graph showing IRI values plotted against previous and next record numbers for speed change analysis, displaying acceleration and deceleration patterns

- As seen from the above graph, for deceleration, some of the “next records” are significantly higher than the “previous records”. In the case of acceleration, some of the “previous records” are higher than the “next records”. Based on the shape of graphs a decision will be made on number of previous and next records that are affected by speed change and need to be adjusted using the curve’s baseline. Since station intervals are different between years, the number of affected stations are different.

- In order to adjust the IRI values, the adjustment baseline should be determined. In the case of deceleration, the flat part of the curve right before the jump in IRI graph, and in the case of acceleration, the flat part of the graph right after the IRI curve flattens after the hump were chosen for calculating the baseline. The points selected for calculating the baseline (average IRI of selected stations) and adjustment for different years are shown in the table below.

| Year (s) | Baseline | Adjust |
| --- | --- | --- |
|  | Deceleration | Acceleration |
| 1995-2000 | Prev. 3 to prev. 6 | Next 2 to next 6 |
| 2001-2005 | Prev. 6 to prev. 10 | Next 3 to next 8 |
| 2006-2011 | Prev. 6 to prev. 10 | Next 3 to next 8 |
| 2012 | Prev. 9 to prev. 18 | Next 5 to next 14 |
| 2013-2015 | Prev. 9 to Prev. 14 | Next 8 to next 13 |

- Calculate the percent change in IRI (compared to the baseline) of each speed change (separately for deceleration and acceleration) at each station included in the study.

- Establish third degree polynomial equations for deceleration and acceleration at each affected station (previous, current, and next stations) based on the percent change IRI values for the affected station and speed changes at the current location. The third-degree polynomial equations are developed after several iterations (i.e. the percent change between baseline IRI value and to be adjusted IRI value should not be significantly high) to make sure the equation is only used for the adjustable ranges of speed change. The threshold speed changes are established beyond which either the IRI values are excluded from reporting or the original IRI value retained without any adjustment. The records with speed changes below the threshold value will be adjusted. The equation used for fitting is: y = (C3 * X3) + (C2 * X2) + (C1 * X1) + b.

- Evaluate the shape of the curve after each iteration. A typical curve should be in the shape of a semi-circle i.e., it should start at zero (no change in speed and so no change in IRI), increase, then decrease, and at some point, cross the zero and become negative (percent change in IRI value is negative). The curve not only should have the right shape but also should follow the percent IRI change curve closely before crossing the zero line. For each station, percent IRI changes follow a trend up to a certain speed change (threshold) and beyond that speed change become scattered and difficult to adjust using an equation. Thus, the IRI values are adjusted where the equation matches the percent IRI changes closely and beyond the threshold the IRI values are either excluded from reports or the original IRI values are retained (see below graph for illustration).

- Graph illustrating typical curve shape for IRI adjustment showing semi-circle pattern with zero crossing point and threshold values

- If the curve deviates from the “Typical” curve, then the equation should be adjusted by changing the data range. Performed iterations until the best fit is reached and the curve looks like a described typical shape. If even after several iterations, the curve keeps on going upward instead of going downward and crossing the zero after some point, then the engineer should determine the threshold value of speed change/percent change in IRI value beyond which the IRI value will be either “excluded/not reported” or retained as original.

- For the curves that go downward and cross zero, the threshold speed change/ percent change in IRI value will be the zero crossing point of the curve and IRI values beyond this point are retained and reported as they are (original values). However, if beyond the threshold, the percent IRI change curve continues to have an upward or downward trend (not scattered around the line y=0), then the IRI values are excluded from reporting for speed changes beyond the threshold.

- Once the coefficients (C3, C2, C1 and b) are determined for deceleration and acceleration for each affected station (previous or next from current record), these numbers are exported to Oracle to be used for calculating the adjusted IRI values. If a station has multiple adjustment factors (the current station moves along the road and thus each station can have different adjustment factors), the maximum adjustment will be considered.

- If the percent adjustment is ≤8% then the original IRI value will be reported as is and no adjustment will be made. If the percent adjustment is >8% then the original IRI value will be adjusted and the adjusted value will be reported.

- If the speed is <15 mph, the IRI value will be excluded and will not be reported. If the speed is ≥15 mph, the IRI value will be either adjusted or the original value be reported in accordance with the requirements stated above.

The final results of the “effect of speed change on IRI” study for years 1995 to 2015 are presented in the following table:

| Year(s) | Station | Name | m3 | m2 | m1 | b | Beyond Adj Threshold | Adj between 0 and |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1995-2000DECEL | -1 | PERC_AVG_PREV_IRI_Change | -0.00116 | -0.02198 | -0.11711 | -0.08318 | O | -14 |
|  | 0 | PERC_AVG_CUR_IRI_CHANGE | -0.00217 | -0.03130 | -0.20593 | -0.12910 | E | -14 |
|  | 1 | PERC_AVG_NEXT_IRI_CHANGE | -0.00002 | 0.00216 | -0.10718 | -0.05477 | E | -14 |
|  | 2 | PERC_AVG_NEXT_2_IRI_CHANGE | -0.00154 | -0.03338 | -0.22732 | -0.13379 | E | -14 |
|  | 3 | PERC_AVG_NEXT_3_IRI_CHANGE | -0.00013 | -0.00686 | -0.07670 | -0.00289 | O | -14 |
|  | 4 | PERC_AVG_NEXT_4_IRI_CHANGE | -0.00061 | -0.01301 | -0.08209 | -0.00458 | O | -14 |
| 1995-2000ACCEL | -4 | PERC_AVG_PREV_4_IRI_Change | 0.00075 | -0.01877 | 0.12645 | -0.03976 | O | 10 |
|  | -3 | PERC_AVG_PREV_3_IRI_CHANGE | -0.00017 | -0.00351 | 0.09993 | -0.03005 | O | 10 |
|  | -2 | PERC_AVG_PREV_2_IRI_CHANGE | -0.00268 | 0.03324 | 0.01137 | 0.00422 | E | 5 |
|  | -1 | PERC_AVG_PREV_IRI_CHANGE | -0.00163 | 0.02206 | 0.01614 | -0.00156 | E | 10 |
|  | 0 | PERC_AVG_CUR_IRI_CHANGE | 0.00002 | -0.00216 | 0.03358 | -0.01246 | O | 10 |
|  | 1 | PERC_AVG_NEXT_IRI_CHANGE | 0.00008 | -0.00079 | 0.01173 | -0.00815 | O | 10 |
| 2001-2005DECEL | -2 | PERC_PREVIOUS_2_IRI_CHANGE | 0.00072 | 0.00721 | -0.02428 | -0.01603 | O | -12 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE | 0.01869 | 0.08010 | 0.01197 | -0.00069 | O | -11 |
|  | 0 | PERC_CUR_IRI_CHANGE | 0.00053 | -0.01307 | -0.34568 | -0.31056 | E | -8 |
|  | 1 | PERC_NEXT_IRI_CHANGE | 0.00214 | 0.03017 | -0.12731 | -0.02076 | E | -7.3 |
|  | 2 | PERC_NEXT_2_IRI_CHANGE | 0.00356 | 0.01917 | -0.17969 | -0.04994 | O | -11 |
|  | 3 | PERC_NEXT_3_IRI_CHANGE | -0.00038 | -0.02186 | -0.22645 | -0.05445 | O | -12 |
|  | 4 | PERC_NEXT_4_IRI_CHANGE | 0.02454 | 0.09335 | -0.04579 | 0.00320 | O | -8.2 |
|  | 5 | PERC_NEXT_5_IRI_CHANGE | 0.01512 | 0.04367 | -0.07957 | -0.00087 | O | -7 |
|  | 6 | PERC_NEXT_6_IRI_CHANGE | 0.00273 | -0.01361 | -0.12185 | -0.00197 | O | -5 |
| 2001-2005ACCEL | -6 | PERC_PREVIOUS_6_IRI_CHANGE | 0.00317 | -0.04867 | 0.18700 | -0.02473 | O | 7 |
|  | -5 | PERC_PREVIOUS_5_IRI_CHANGE | 0.00305 | -0.04734 | 0.20904 | -0.02855 | O | 11 |
|  | -4 | PERC_PREVIOUS_4_IRI_CHANGE | 0.00126 | -0.03899 | 0.27189 | -0.08652 | O | 11 |
|  | -3 | PERC_PREVIOUS_3_IRI_CHANGE | 0.00112 | -0.04209 | 0.35558 | -0.14980 | O | 11 |
|  | -2 | PERC_PREVIOUS_2_IRI_CHANGE | -0.00611 | 0.05319 | 0.11003 | -0.04261 | E | 10.2 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE | -0.00276 | 0.02538 | 0.08636 | -0.01779 | E | 7.5 |
|  | 0 | PERC_CUR_IRI_CHANGE | -0.00204 | 0.00761 | 0.05302 | -0.00244 | O | 7 |
|  | 1 | PERC_NEXT_IRI_CHANGE | 0.00091 | -0.01042 | 0.04497 | -0.00673 | O | 7 |
| 2006-2011DECEL | -1 | PERC_PREVIOUS_IRI_CHANGE | -0.01714 | -0.06903 | -0.12196 | -0.00228 | O | -2 |
|  | 0 | PERC_CUR_IRI_CHANGE | 0.00102 | 0.00119 | -0.09409 | -0.00276 | E | -6.5 |
|  | 1 | PERC_NEXT_IRI_CHANGE | 0.00183 | 0.00535 | -0.14952 | -0.04700 | E | -6.5 |
|  | 2 | PERC_NEXT_2_IRI_CHANGE | 0.00511 | 0.02864 | -0.08809 | -0.00936 | O | -8 |
|  | 3 | PERC_NEXT_3_IRI_CHANGE | 0.00007 | -0.01187 | -0.13590 | -0.01302 | O | -11 |
|  | 4 | PERC_NEXT_4_IRI_CHANGE | 0.00580 | 0.01847 | -0.07835 | -0.00080 | O | -5.6 |
|  | 5 | PERC_NEXT_5_IRI_CHANGE | 0.00978 | 0.02161 | -0.07577 | -0.00002 | O | -6 |
| 2006-2011ACCEL | -5 | PERC_PREVIOUS_5_IRI_CHANGE | 0.00499 | -0.08172 | 0.27239 | -0.05097 | O | 4.4 |
|  | -4 | PERC_PREVIOUS_4_IRI_CHANGE | -0.00603 | 0.01800 | 0.10471 | 0.00239 | O | 6.2 |
|  | -3 | PERC_PREVIOUS_3_IRI_CHANGE | 0.00376 | -0.08346 | 0.40320 | -0.15034 | O | 6.5 |
|  | -2 | PERC_PREVIOUS_2_IRI_CHANGE | 0.00117 | -0.03477 | 0.22926 | -0.04137 | E | 9.4 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE | -0.01225 | 0.05972 | 0.06044 | -0.04615 | E | 5.7 |
|  | 0 | PERC_CUR_IRI_CHANGE | -0.01112 | 0.05732 | -0.02522 | -0.00104 | O | 4.7 |
|  | 1 | PERC_NEXT_IRI_CHANGE | -0.00483 | 0.02905 | -0.01443 | -0.00262 | O | 5.6 |
| 2012DECEL | -1 | PERC_PREVIOUS_IRI_CHANGE | 0.01729 | -0.05790 | -0.20439 | -0.00951 | O | -2.1 |
|  | 0 | PERC_CUR_IRI_CHANGE | 0.13573 | 0.26871 | -0.01729 | -0.00034 | O | -2.1 |
|  | 1 | PERC_NEXT_IRI_CHANGE | 0.10029 | 0.22843 | -0.03596 | -0.00114 | E | -2.5 |
|  | 2 | PERC_NEXT_2_IRI_CHANGE | 0.12142 | 0.29474 | -0.00907 | -0.00287 | E | -2.5 |
|  | 3 | PERC_NEXT_3_IRI_CHANGE | 0.08226 | 0.28588 | 0.03517 | -0.00212 | E | -3.4 |
|  | 4 | PERC_NEXT_4_IRI_CHANGE | 0.06541 | 0.18791 | -0.06089 | -0.00140 | E | -3 |
|  | 5 | PERC_NEXT_5_IRI_CHANGE | 0.06089 | 0.16298 | -0.07247 | -0.00195 | E | -3.2 |
|  | 6 | PERC_NEXT_6_IRI_CHANGE | 0.07148 | 0.16349 | -0.07148 | -0.00166 | O | -2.7 |
|  | 7 | PERC_NEXT_7_IRI_CHANGE | 0.03793 | 0.06310 | -0.12112 | -0.00143 | O | -2.9 |
|  | 8 | PERC_NEXT_8_IRI_CHANGE | -0.01378 | -0.05821 | -0.16589 | -0.00251 | O | -2.2 |
|  | 9 | PERC_NEXT_9_IRI_CHANGE | 0.04063 | 0.08397 | -0.07354 | -0.00065 | O | -2.8 |
|  | 10 | PERC_NEXT_10_IRI_CHANGE | 0.01092 | -0.03087 | -0.15878 | -0.00691 | O | -2.7 |
| 2012ACCEL | -8 | PERC_PREVIOUS_8_IRI_CHANGE | -0.19276 | 0.34384 | 0.04407 | -0.00199 | O | 2 |
|  | -7 | PERC_PREVIOUS_7_IRI_CHANGE | -0.33409 | 0.75262 | -0.18066 | -0.00018 | O | 2 |
|  | -6 | PERC_PREVIOUS_6_IRI_CHANGE | -0.36747 | 0.94653 | -0.31198 | -0.00294 | O | 2.2 |
|  | -5 | PERC_PREVIOUS_5_IRI_CHANGE | -0.49045 | 1.23441 | -0.44638 | -0.00193 | O | 2.1 |
|  | -4 | PERC_PREVIOUS_4_IRI_CHANGE | -0.40732 | 1.07071 | -0.38984 | -0.00176 | O | 2.2 |
|  | -3 | PERC_PREVIOUS_3_IRI_CHANGE | -0.35207 | 0.96729 | -0.38855 | -0.00139 | O | 2.3 |
|  | -2 | PERC_PREVIOUS_2_IRI_CHANGE | -0.27263 | 0.68402 | -0.23191 | -0.00373 | O | 2.2 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE | -0.24323 | 0.59040 | -0.20703 | -0.00389 | O | 2.1 |
|  | 0 | PERC_CUR_IRI_CHANGE | -0.19633 | 0.45747 | -0.15961 | -0.00428 | O | 2 |
|  | 1 | PERC_NEXT_IRI_CHANGE | -0.14449 | 0.33082 | -0.13090 | -0.00129 | O | 1.8 |
| 2013-2015DECEL | -5 | PERC_PREVIOUS_5_IRI_CHANGE_Formula | 0.00229 | -0.03412 | -0.07052 | -0.00462 | O | -1.5 |
|  | -4 | PERC_PREVIOUS_4_IRI_CHANGE_Formula | -0.00594 | -0.04018 | -0.06889 | 0.00321 | O | -2.4 |
|  | -3 | PERC_PREVIOUS_3_IRI_CHANGE_Formula | -0.01200 | -0.11101 | -0.15129 | -0.00442 | O | -1.6 |
|  | -2 | PERC_PREVIOUS_2_IRI_CHANGE_Formula | -0.06664 | -0.28180 | -0.30174 | -0.02083 | O | -1.8 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE_Formula | -0.07625 | -0.29471 | -0.33336 | -0.01863 | E | -2.4 |
|  | 0 | PERC_CUR_IRI_CHANGE_Formula | -0.04436 | -0.18910 | -0.27884 | -0.01440 | E | -2.5 |
|  | 1 | PERC_NEXT_IRI_CHANGE_Formula | -0.01298 | -0.04557 | -0.17321 | 0.00481 | E | -2.5 |
|  | 2 | PERC_NEXT_2_IRI_CHANGE_Formula | -0.01246 | -0.04168 | -0.20861 | -0.00495 | E | -3 |
|  | 3 | PERC_NEXT_3_IRI_CHANGE_Formula | 0.00972 | 0.01461 | -0.18929 | -0.00051 | O | -5.2 |
|  | 4 | PERC_NEXT_4_IRI_CHANGE_Formula | 0.01914 | 0.02410 | -0.21317 | -0.01063 | O | -4 |
|  | 5 | PERC_NEXT_5_IRI_CHANGE_Formula | 0.00351 | -0.05990 | -0.30682 | -0.02718 | O | -4 |
|  | 6 | PERC_NEXT_6_IRI_CHANGE_Formula | -0.00333 | -0.09431 | -0.33125 | -0.02870 | O | -3.8 |
|  | 7 | PERC_NEXT_7_IRI_CHANGE_Formula | 0.05887 | 0.09816 | -0.16763 | -0.00385 | O | -2.7 |
|  | 8 | PERC_NEXT_8_IRI_CHANGE_Formula | 0.03904 | 0.02926 | -0.21036 | -0.00841 | O | -2.7 |
|  | 9 | PERC_NEXT_9_IRI_CHANGE_Formula | -0.00520 | -0.11336 | -0.30998 | -0.02030 | O | -3.1 |
|  | 10 | PERC_NEXT_10_IRI_CHANGE_Formula | 0.00178 | -0.09249 | -0.27703 | -0.01406 | O | -2.3 |
|  | 11 | PERC_NEXT_11_IRI_CHANGE_Formula | 0.02047 | -0.04249 | -0.22880 | -0.00690 | O | -2.4 |
|  | 12 | PERC_NEXT_12_IRI_CHANGE_Formula | -0.00293 | -0.11079 | -0.26417 | -0.00984 | O | -2.4 |
| 2013-2015ACCEL | -12 | PERC_PREVIOUS_12_IRI_CHANGE_Formula | 0.02778 | -0.26908 | 0.40049 | -0.01831 | O | 1.7 |
|  | -11 | PERC_PREVIOUS_11_IRI_CHANGE_Formula | 0.05413 | -0.32881 | 0.44995 | -0.02393 | O | 2 |
|  | -10 | PERC_PREVIOUS_10_IRI_CHANGE_Formula | 0.05342 | -0.33693 | 0.47913 | -0.02945 | O | 2.1 |
|  | -9 | PERC_PREVIOUS_9_IRI_CHANGE_Formula | 0.03102 | -0.28365 | 0.46888 | -0.03069 | O | 2 |
|  | -8 | PERC_PREVIOUS_8_IRI_CHANGE_Formula | 0.00534 | -0.22313 | 0.45779 | -0.03330 | O | 2.1 |
|  | -7 | PERC_PREVIOUS_7_IRI_CHANGE_Formula | -0.03595 | -0.12353 | 0.42416 | -0.03423 | O | 2 |
|  | -6 | PERC_PREVIOUS_6_IRI_CHANGE_Formula | -0.09425 | 0.02266 | 0.36036 | -0.03329 | O | 2 |
|  | -5 | PERC_PREVIOUS_5_IRI_CHANGE_Formula | -0.19830 | 0.31719 | 0.18291 | -0.01922 | O | 2.1 |
|  | -4 | PERC_PREVIOUS_4_IRI_CHANGE_Formula | -0.24017 | 0.49000 | 0.05472 | -0.00837 | O | 2.1 |
|  | -3 | PERC_PREVIOUS_3_IRI_CHANGE_Formula | -0.19014 | 0.42620 | 0.06539 | -0.01249 | O | 2.3 |
|  | -2 | PERC_PREVIOUS_2_IRI_CHANGE_Formula | -0.13620 | 0.41526 | -0.03116 | 0.00587 | E | 2.9 |
|  | -1 | PERC_PREVIOUS_IRI_CHANGE_Formula | -0.01119 | 0.14443 | 0.09402 | -0.00972 | E | 2 |
|  | 0 | PERC_CUR_IRI_CHANGE_Formula | -0.04055 | 0.17229 | 0.03630 | -0.00232 | E | 2 |
|  | 1 | PERC_NEXT_IRI_CHANGE_Formula | -0.00325 | 0.07170 | 0.03694 | -0.00100 | E | 2.5 |
|  | 2 | PERC_NEXT_2_IRI_CHANGE_Formula | 0.02206 | -0.04167 | 0.10236 | -0.01325 | E | 2.4 |
|  | 3 | PERC_NEXT_3_IRI_CHANGE_Formula | 0.03593 | -0.09168 | 0.11552 | -0.01510 | E | 2.4 |
|  | 4 | PERC_NEXT_4_IRI_CHANGE_Formula | -0.09615 | 0.22503 | -0.09393 | 0.00748 | O | 1.8 |
|  | 5 | PERC_NEXT_5_IRI_CHANGE_Formula | -0.07561 | 0.18395 | -0.08516 | 0.00662 | O | 1.8 |

## Examples for Changing Latitude/Longitude

Below is an example of a latitude and longitude that needs to be changed. The green balloon should be at the gore (the place where the two solid white lines meet). Since the balloon is more than 4.2 millimiles from the gore, a change is required.

Example of balloon more than 4.2 millimiles from the gore

Below is another example of a balloon that is not at the gore but is within the 4.2 millimiles tolerance distance. Therefore, the latitude and longitude do not need to be changed.

Example of balloon within 4.2 millimiles tolerance distance

Below is an example of the collection stopping before the balloon (displayed in Vision in this case as a pink triangle). Since the ending location is correct, and the problem is that the collection did not continue to the end of the route, a gap should be placed instead of changing the latitude and longitude (see RouteMatching).

Example of collection stopping before the balloon

## Outlier Review - Outlier Codes

The table below is a list of active outlier codes (as of 2017).

The ID field is displayed as “OUTLIER_ID” in Oracle and represents the corresponding outlier code number of a specific outlier explanation.

The DESCRIPTION field is displayed as “OUTLIER_DESCRIP” in Oracle and provides a brief description of the type of outlier code.

The YEAR field is displayed as “CUR_OR_PREV_YEAR” in Oracle and describes what year’s data the outlier code applies to. For example, the “CUR” codes in this table are applied to the current year’s collection data (in this case, 2017), while the “PREV” codes in the table are applied to previous years’ collection data (in this case, 2013 through 2016).

The COMMENTS field provides more context regarding the nature and definition of the outlier code.

| ID | DESCRIP | YEAR | COMMENTS |
| --- | --- | --- | --- |
| 1 | Lane Change | CUR | Lane change observed, during current season’s data collection. If 2017 data are in question, the current season refers to 2017 data collection. |
| 3 | New Pave | CUR | New pavement surface observed. |
| 5 | Construction | CUR | Construction activities observed. |
| 6 | Road Widened | CUR | Road widening observed. |
| 11 | Lane Change | PREV | Lane change observed, during previous seasons’ data collections. If current season is 2017, the previous seasons refer to 2013 to 2016 data collection. |
| 13 | New Pave | PREV | New pavement surface observed, during previous seasons’ data collection. |
| 15 | Construction | PREV | Construction activities observed, during previous seasons’ data collection. |
| 16 | Road Widened | PREV | Road widening observed during previous seasons’ data collection. |
| 18 | Start/end mile point are different | PREV | Location differences in starting or ending mile points being greater than 100 ft between different data collection. |
| 21 | Right IRI > 640 | CUR | Right IRI greater than 640 in/mile. |
| 22 | Speed < 15 | CUR | Speed less than 15 miles/hour. |
| 24 | Distress | PREV | Distresses observed, during previous season’s data collection. |
| 25 | No Reason | CUR | No clear explanations on why outliers occur. |
| 27 | Left IRI > 640 | CUR | Left IRI greater than 640 in/mile. |
| 33 | Bridge/Structure | CUR | Bridge/structure observed. |
| 34 | Railroad Crossing | CUR | Railroad crossing observed. |
| 35 | Roundabout | CUR | Roundabout observed. |
| 36 | Concrete | CUR | Concrete pavement observed. |
| 37 | Rumble_Strip | CUR | Rumble strip observed. |
| 38 | OFFICE_STOPBAR | CUR | Code automatically created for stop-bar flagged by data processing team. |
| 39 | OFFICE_CROSSWALK | CUR | Code automatically created for crosswalk flagged by data processing team. |
| 40 | OFFICE_CRCP | CUR | Code automatically created for CRCP flagged by data processing team. |
| 41 | OFFICE_JCP | CUR | Code automatically created for JCP flagged by data processing team. |
| 42 | OFFICE_BRICK | CUR | Code automatically created for brick flagged by data processing team. |
| 43 | OFFICE_DEBRIS | CUR | Code automatically created for debris flagged by data processing team. |
| 44 | OFFICE_CRACK_SEAL | CUR | Code automatically created for crack seal flagged by data processing team. |
| 45 | OFFICE_LANE_DEVIATION | CUR | Code automatically created for lane deviation flagged by data processing team. |
| 46 | OFFICE_MILLED_SURFACE | CUR | Code automatically created for milled surface flagged by data processing team. |
| 47 | OFFICE_BRIDGE | CUR | Code automatically created for bridge flagged by data processing team. |
| 48 | OFFICE_RAILROAD | CUR | Code automatically created for railroad flagged by data processing team. |
| 49 | OFFICE_DISREGARD_CRACK | CUR | Code automatically created for disregarded crack flagged by data processing team. |
| 50 | OFFICE_BAD_IMAGE | CUR | Code automatically created for bad image flagged by data processing team. |
| 51 | ARAN_Lane Deviation | CUR | Code automatically created for lane deviation flagged by ARAN operator. |
| 52 | ARAN_New Pavement | CUR | Code automatically created for new pavement flagged by ARAN operator. |
| 53 | ARAN_Bridge | CUR | Code automatically created for bridge flagged by ARAN operator. |
| 54 | ARAN_Railroad | CUR | Code automatically created for railroad crossing flagged by ARAN operator. |
| 55 | ARAN_Crack Seal | CUR | Code automatically created for crack seal flagged by ARAN operator. |
| 56 | ARAN_Concrete | CUR | Code automatically created for concrete flagged by ARAN operator. |
| 57 | ARAN_Rumble_Strip | CUR | Code automatically created for rumble strip flagged by ARAN operator. |
| 58 | ARAN_Construction | CUR | Code automatically created for construction flagged by ARAN operator. |
| 59 | ARAN_Debris | CUR | Code automatically created for debris flagged by ARAN operator. |
| 60 | Raised_Crosswalk_ Historic_Data | PREV | Raised crosswalk observed, during previous seasons’ data collection. |
| 61 | ARAN_Damp_Pavement | CUR | Code automatically created for damp or wet pavement flagged by ARAN operator. |
| 62 | NEW_PAVEMENT_CONST_HISTORY | CUR | New pavement based on construction history records. |
| 63 | Minor Location Difference | CUR | Minor (shorter than 100 ft) location differences observed from one year to another, most at the beginning or ending mileposts. |
| 64 | Lane Configuration Change | CUR | Lane configuration change observed. |
| 99 | OTHER | CUR | Other reasons. |

## IRI Performance Model

The following tables provide current information for IRI model parameters.

Current IRI μ1 Values by Last Treatment

| Last Treatment | μ1 |
| --- | --- |
| Mill and Overlay with >1.5” Grade Increase – Gap | 0.800 |
| Mill >4” Overlay with Asphalt | 0.810 |
| Cold-in-place Recycling Overlay | 0.810 |
| Break Crack Seat Overlay | 0.810 |
| Rubbilization Overlay | 0.810 |
| Reconstruction | 0.810 |
| Mill and Overlay with <1.5” Grade Increase – Gap | 0.820 |
| Overlay with >1.5” Grade Increase – Gap | 0.820 |
| Bonded Portland Cement Concrete Overlay | 0.830 |
| Mill and Overlay with >1.5” Grade Increase | 0.850 |
| Micro Surfacing | 0.900 |
| High Friction Surface | 0.900 |
| Mill and Overlay with <1.5” Grade Increase | 0.978 |
| Mill Ultrathin Bonded Wearing Course | 0.978 |
| Other | 1.000 |
| Overlay <1.5” Asphalt | 1.023 |
| Thin Overlay <1” Asphalt | 1.065 |

Current IRI μ2 Values by Family

| Pavement Type | Region | Functional Class | Family | μ2 |
| --- | --- | --- | --- | --- |
| Flexible | Mountain | 2, 3 | 1 | 0.0322 |
| Flexible | Mountain | 7, 14, 17 | 2 | 0.0255 |
| Flexible | Mountain | 16, 18, 19 | 3 | 0.0255 |
| Flexible | Central | 2, 3, 7, 17 | 4 | 0.0219 |
| Flexible | Central | 12, 14, 16 | 5 | 0.0181 |
| Flexible | Central | 18, 19 | 6 | 0.0212 |
| Flexible | Coastal | 2, 7, 14, 17, 18 | 7 | 0.0219 |
| Flexible | Coastal | 3, 12, 16 | 8 | 0.0180 |
| Flexible | Coastal | 19 | 9 | 0.0214 |
| Flexible | All | 1, 11 | 10 | 0.0178 |
| Flexible | Mountain and Central | 6, 8 | 11 | 0.0239 |
| Flexible | Mountain and Central | 9 | 12 | 0.0242 |
| Flexible | Coastal | 6, 8 | 13 | 0.0219 |
| Flexible | Coastal | 9 | 14 | 0.0191 |
| Flexible over JCP | Mountain | 8, 9, 18, 19 | 15 | 0.0171 |
| Flexible over JCP | Mountain | 1, 2, 11, 12, 14, 17 | 16 | 0.0267 |
| Flexible over JCP | Central | 8, 9, 17 | 17 | 0.0269 |
| Flexible over JCP | Central | 14 | 18 | 0.0279 |
| Flexible over JCP | Central | 2, 12, 19 | 19 | 0.0241 |
| Flexible over JCP | Central | 18 | 20 | 0.0516 |
| Flexible over JCP | Central | 1, 11 | 21 | 0.0184 |
| Flexible over JCP | Coastal | 6, 8, 9, 12, 14, 17, 18, 19 | 22 | 0.0306 |
| Flexible over JCP | Coastal | 1, 2, 11 | 23 | 0.0377 |
| Flexible over JCP | Mountain and Central | 6 | 24 | 0.0260 |
| Flexible over JCP | Mountain | 7 | 25 | 4 |
| Flexible over JCP | Mountain | 16 | 26 | 4 |
| Flexible over JCP | Central | 7 | 27 | 0.0321 |
| Flexible over JCP | Central | 16 | 28 | 0.0261 |
| Flexible over JCP | Coastal | 7 | 29 | 0.0328 |
| Flexible over JCP | Coastal | 16 | 30 | 0.0176 |
| Flexible over JCP | All | 3 | 31 | 0.0265 |
| Flexible over CRCP | All | All | 32 | 0.0095 |
| Rigid Jointed | All | All | 33 | 0.0120 |
| Rigid Jointed Plain | All | All | 34 | 0.0220 |
| Rigid Continuous | All | All | 35 | 0.0114 |
| Other | - | - | - | 0.0239 |

IRI Table “trend_iri_family_slopes” with Most Recent Percent Change and Mileage by Family

| IRI_FAMILY06/12/2019 | WT_AVG_PCT_IRI_CHANGE, % | MILES_INCLUDED, miles |
| --- | --- | --- |
| 1 | 0.03215 | 603.6 |
| 2 | 0.02553 | 1551.8 |
| 3 | 0.02545 | 588.3 |
| 4 | 0.02192 | 4198.4 |
| 5 | 0.01806 | 9521.5 |
| 6 | 0.01254 | 327 |
| 7 | 0.02193 | 10051.7 |
| 8 | 0.01798 | 1752.5 |
| 9 | 0.02143 | 149.1 |
| 10 | 0.01784 | 3933.8 |
| 11 | 0.02391 | 6647.6 |
| 12 | 0.02422 | 562.1 |
| 13 | 0.02188 | 5155.7 |
| 14 | 0.01907 | 762 |
| 15 | 0.01713 | 357.1 |
| 16 | 0.02665 | 677.1 |
| 17 | 0.02687 | 675.1 |
| 18 | 0.02793 | 2648 |
| 19 | 0.02412 | 1160.3 |
| 20 | 0.05163 | 105.4 |
| 21 | 0.01835 | 1466.5 |
| 22 | 0.03061 | 6566.2 |
| 23 | 0.03768 | 1309.1 |
| 24 | 0.026 | 1918.5 |
| 25 | N/A | 0 |
| 26 | N/A | 0 |
| 27 | 0.03213 | 2123 |
| 28 | 0.02605 | 2406.3 |
| 29 | 0.03283 | 4935.3 |
| 30 | 0.01762 | 594.7 |
| 31 | 0.02647 | 123.6 |
| 32 | 0.00945 | 510 |
| 33 | 0.01195 | 91.4 |
| 34 | 0.02199 | 63.7 |
| 35 | 0.01135 | 322.1 |

## Cracking Performance Models

The following tables and scripts provide current information for cracking model parameters.

| Treatment Group | Pavement Type | Region | Func. Class | L_tilde (Ld) | ll_int (li) | ll_slp (ls) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | All | All | 1 | 90637719.44 | 2.8083 | -0.0046 |
| C | All | 1 | 14 | 266.3311 | 1.4455 | -0.0255 |
| 2 | Composite | 1 | 14 | 1787.4858 | 1.948 | -0.0111 |
| 2 | Flexible | 1 | 14 | 6.03267E+20 | 3.8714 | -0.0029 |
| 3 | All | 2 | 14 | 1.06942E+39 | 4.4886 | -0.0009 |
| C | All | 2 | 14 | 86.8394 | 1.6175 | -0.0618 |
| 2 | Composite | 2 | 14 | 150.0416 | 1.3954 | -0.05 |
| 2 | Flexible | 2 | 14 | 275.4735 | 1.6585 | -0.0442 |
| 3 | All | 3 | 14 | 4103.2955 | 2.2241 | -0.0275 |
| 4 | All | 3 | 14 | 55.5406 | 1.0076 | -0.1419 |
| C | All | 3 | 14 | 41969.4996 | 2.3521 | -0.0217 |
| 2 | Composite | 3 | 14 | 117.5573 | 1.4525 | -0.0749 |
| 2 | Flexible | 3 | 14 | 125.2176 | 1.5761 | -0.0606 |
| 4 | All | 1 or 2 | 14 | 76.2935 | 1.0401 | -0.0643 |
| 1 | All | All | 14 | 1.00288E+18 | 3.6796 | -0.0019 |
| A | All | All | 14 | 2.15601E+18 | 3.7308 | -0.0031 |
| 1 | Composite | All | 2-3-6 | 584.7963 | 1.464 | -0.0102 |
| 1 | Flexible | All | 2-3-6 | 138.1563 | 1.1843 | -0.0417 |
| C | All | 1 | 7-8-9 | 305.5609 | 1.7251 | -0.031 |
| 3 | All | 2 | 7-8-9 | 1956.3351 | 2.0539 | -0.0189 |
| C | All | 2 | 7-8-9 | 599.4863 | 1.8873 | -0.0317 |
| C | All | 3 | 7-8-9 | 21079742451 | 3.1608 | -0.0081 |
| 3 | Composite | 3 | 7-8-9 | 6.21395E+15 | 3.5844 | -0.0067 |
| 3 | Flexible | 3 | 7-8-9 | 1361.205 | 1.941 | -0.0285 |
| 1 | Composite | All | 7-8-9 | 215.1996 | 1.3605 | -0.0575 |
| 2 | Composite | All | 7-8-9 | 341.6614 | 1.7043 | -0.0338 |
| 4 | Composite | All | 7-8-9 | 6.04754E+22 | 3.9402 | -0.0045 |
| 6 | Composite | All | 7-8-9 | 50.5424 | 1.0538 | -0.0821 |
| 10 | Composite | All | 7-8-9 | Ld1=50.5424;li1=1.0538;ls1=-0.0821;Ld2=573.153;li2=1.5699;ls2=-0.0145; |  |  |
| 1 | Flexible | All | 7-8-9 | 109.2627 | 1.248 | -0.0402 |
| 2 | Flexible | All | 7-8-9 | 6966.2944 | 2.1961 | -0.0176 |
| 4 | Flexible | All | 7-8-9 | 145528821.1 | 2.8511 | -0.0076 |
| 6 | Flexible | All | 7-8-9 | 60.3093 | 1.4039 | -0.1222 |
| 7 | Flexible | All | 7-8-9 | 10471052.58 | 2.7114 | -0.0162 |
| 10 | Flexible | All | 7-8-9 | Ld1=60.3093;li1=1.4039;ls1=-0.1222;Ld2=10471052.5813;li2=2.7114;ls2=-0.0162; |  |  |
| 6 | Composite | All | 11-12-14 | 76.9602 | 1.2325 | -0.073 |
| 10 | Composite | All | 11-12-14 | Ld1=76.9602;li1=1.2325;ls1=-0.073;Ld2=573.153;li2=1.5699;ls2=-0.0145; |  |  |
| 6 | Flexible | All | 11-12-14 | 9.89737E+30 | 4.2652 | -0.0023 |
| 7 | Flexible | All | 11-12-14 | 1882.4205 | 1.9344 | -0.0382 |
| 10 | Flexible | All | 11-12-14 | Ld1=98973660234.9642*Math.Pow(10,20);li1=4.2652;ls1=-0.0023;Ld2=1882.4205;li2=1.9344;ls2=-0.0382; |  |  |
| 3 | All | 2 | 11-12 | 89359137.61 | 2.9464 | -0.015 |
| 5 | All | 2 | 11-12 | 1662.1596 | 1.9936 | -0.0288 |
| 3 | All | 3 | 11-12 | 3794.5556 | 1.951 | -0.0064 |
| 5 | All | 1 or 3 | 11-12 | 50.4719 | 1.1323 | -0.0834 |
| 1 | All | All | 11-12 | 2.27747E+16 | 3.5918 | -0.0025 |
| 4 | All | All | 11-12 | 50.3632 | 0.8305 | -0.0947 |
| A | All | All | 11-12 | 2683.6544 | 2.0435 | -0.0171 |
| C | All | All | 11-12 | 2492522027 | 3.0629 | -0.009 |
| 2 | Composite | All | 11-12 | 111.6238 | 1.2991 | -0.053 |
| 2 | Flexible | All | 11-12 | 2516.3924 | 2.013 | -0.0291 |
| B | All | 2 | 11-12-14-16-17-18-19 | 600 | 1.9327 | -0.0345 |
| C | All | 1 | 1-2-3-6 | 106.2486 | 1.5597 | -0.0593 |
| 2 | Composite | 1 | 1-2-3-6 | 97.2449 | 1.3891 | -0.0457 |
| 3 | All | 2 | 1-2-3-6 | 431.2878 | 1.7957 | -0.0347 |
| C | All | 2 | 1-2-3-6 | 7.16067E+17 | 3.7176 | -0.0039 |
| 2 | Composite | 2 | 1-2-3-6 | 4709385.613 | 2.7561 | -0.0194 |
| 3 | All | 3 | 1-2-3-6 | 1789.58 | 1.9201 | -0.0212 |
| 4 | All | 3 | 1-2-3-6 | 78.314 | 1.1183 | -0.1146 |
| 2 | Composite | 3 | 1-2-3-6 | 28894462.16 | 2.8285 | -0.0165 |
| C | Composite | 3 | 1-2-3-6 | 170.3627 | 1.499 | -0.0455 |
| C | Flexible | 3 | 1-2-3-6 | 397.2458 | 1.7853 | -0.0385 |
| 4 | All | 1 or 2 | 1-2-3-6 | 13659.7542 | 2.1062 | -0.0159 |
| 6 | Composite | All | 1-2-3-6 | 6073.429 | 2.1367 | -0.0255 |
| 10 | Composite | All | 1-2-3-6 | Ld1=6073.429;li1=2.1367;ls1=-0.0255;Ld2=573.153;li2=1.5699;ls2=-0.0145; |  |  |
| 2 | Flexible | All | 1-2-3-6 | 367611295.6 | 2.9757 | -0.0088 |
| 6 | Flexible | All | 1-2-3-6 | 951.8987 | 1.9191 | -0.0352 |
| 7 | Flexible | All | 1-2-3-6 | 201234894.5 | 2.9441 | -0.0166 |
| 10 | Flexible | All | 1-2-3-6 | Ld1=951.8987;li1=1.9191;ls1=-0.0352;Ld2=201234894.491;li2=2.9441;ls2=-0.0166; |  |  |
| B | All | 2 | 1-2-3-6-7-8-9 | 9.38799E+14 | 3.5453 | -0.005 |
| 2 | Composite | 1 | 16-17-18-19 | 1032.0372 | 1.8994 | -0.025 |
| 2 | Flexible | 1 | 16-17-18-19 | 5437.0356 | 2.1001 | -0.0225 |
| 3 | All | 2 | 16-17-18-19 | 860.3014 | 1.938 | -0.0366 |
| 2 | Composite | 2 | 16-17-18-19 | 437.9597 | 1.735 | -0.0345 |
| 2 | Flexible | 2 | 16-17-18-19 | 720.4722 | 1.8432 | -0.0266 |
| 3 | All | 3 | 16-17-18-19 | 573.903 | 1.7447 | -0.0175 |
| 4 | All | 3 | 16-17-18-19 | 1382900.394 | 2.5519 | -0.0148 |
| 2 | Composite | 3 | 16-17-18-19 | 404.1978 | 1.6935 | -0.0352 |
| 2 | Flexible | 3 | 16-17-18-19 | 130.2758 | 1.7928 | -0.0733 |
| 4 | Composite | 1 or 2 | 16-17-18-19 | 1.43697E+27 | 4.1141 | -0.0035 |
| 4 | Flexible | 1 or 2 | 16-17-18-19 | 58.1315 | 1.0264 | -0.0862 |
| A | All | All | 16-17-18-19 | 19646019564 | 3.1712 | -0.009 |
| 1 | Composite | All | 16-17-18-19 | 511.7243 | 1.5606 | -0.029 |
| 6 | Composite | All | 16-17-18-19 | 1.04014E+36 | 4.402 | -0.0008 |
| 10 | Composite | All | 16-17-18-19 | Ld1=10401445555430400*Math.Pow(10,20);li1=4.402;ls1=-0.0008;Ld2=573.153;li2=1.5699;ls2=-0.0145; |  |  |
| C | Composite | All | 16-17-18-19 | 2704.5306 | 2.0572 | -0.0246 |
| 1 | Flexible | All | 16-17-18-19 | 1.27334E+24 | 3.9863 | -0.0011 |
| 6 | Flexible | All | 16-17-18-19 | 2.21779E+35 | 4.384 | -0.0012 |
| 7 | Flexible | All | 16-17-18-19 | 73.0759 | 1.4134 | -0.151 |
| 10 | Flexible | All | 16-17-18-19 | Ld1=221778.766348933*Math.Pow(10,30);li1=4.384;ls1=-0.0012;Ld2=73.0759;li2=1.4134;ls2=-0.151; |  |  |
| C | Flexible | All | 16-17-18-19 | 44859729.16 | 2.8931 | -0.0125 |
| 5 | All | All | 16-17-18-19-14 | 398.49 | 1.7958 | -0.0324 |
| 5 | All | All | 1-7-8-9-2-3-6 | 2701.4546 | 2.077 | -0.0293 |
| A | All | All | 1-7-8-9-2-3-6 | 62120662.97 | 2.8845 | -0.0138 |
| 3 | All | 1 | All | 122962.0574 | 2.4522 | -0.0095 |
| B | All | 1 | All | 2013.4766 | 2.079 | -0.0346 |
| B | All | 3 | All | 1709.5229 | 1.9365 | -0.0165 |
| 8 | All | All | All | 1.71116E+11 | 3.2614 | -0.0093 |
| 7 | Composite | All | All | 573.153 | 1.5699 | -0.0145 |
| N/A | N/A | 1 | 1 | 3.99E+22 | 3.93 | -0.01 |
| N/A | N/A | 2 | 1 | 280 | 1.75 | -0.04 |
| N/A | N/A | 3 | 1 | 280 | 1.75 | -0.04 |
| N/A | N/A | 1 | 2-3-6 | 763 | 1.72 | -0.01 |
| N/A | N/A | 2 | 2-3-6 | 108 | 1.34 | -0.04 |
| N/A | N/A | 3 | 2-3-6 | 29300 | 2.28 | -0.02 |
| N/A | N/A | 1 | 7-8-9 | 220 | 1.58 | -0.03 |
| N/A | N/A | 2 | 7-8-9 | 1170 | 1.88 | -0.02 |
| N/A | N/A | 3 | 7-8-9 | 582 | 1.77 | -0.04 |
| N/A | N/A | 1 | 11-12 | 593 | 1.61 | -0.02 |
| N/A | N/A | 2 | 11-12 | 718 | 1.82 | -0.03 |
| N/A | N/A | 3 | 11-12 | 378 | 1.67 | -0.03 |
| N/A | N/A | 1 | 14 | 104 | 1.42 | -0.03 |
| N/A | N/A | 2 | 14 | 238 | 1.52 | -0.04 |
| N/A | N/A | 3 | 14 | 631 | 1.82 | -0.04 |
| N/A | N/A | 1 | 16-17-18-19 | 325 | 1.63 | -0.02 |
| N/A | N/A | 2 | 16-17-18-19 | 104 | 1.35 | -0.04 |
| N/A | N/A | 3 | 16-17-18-19 | 153 | 1.5 | -0.04 |

Table 1: FC Density Model parameters

| Treatment Group | Pavement Type | Region | Func. Class | L_tilde (Ld) | ll_int (li) | ll_slp (ls) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | All | All | 1 | 75.4278 | 1.8473 | -0.1143 |
| C | All | 1 | 14 | 4.75248E+25 | 4.0944 | -0.0029 |
| 2 | Composite | 1 | 14 | 3.47062E+22 | 3.9847 | -0.0035 |
| 2 | Flexible | 1 | 14 | 298.7579 | 2.0722 | -0.0526 |
| 3 | All | 2 | 14 | 3.81056E+35 | 4.4145 | -0.002 |
| C | All | 2 | 14 | 93.7206 | 1.9854 | -0.0693 |
| 2 | Composite | 2 | 14 | 232006421 | 3.024 | -0.0152 |
| 2 | Flexible | 2 | 14 | 4.89637E+16 | 3.6892 | -0.0084 |
| 3 | All | 3 | 14 | 3.35974E+33 | 4.384 | -0.0036 |
| 4 | All | 3 | 14 | 262.8322 | 1.4992 | -0.0224 |
| C | All | 3 | 14 | 6.52682E+22 | 3.9954 | -0.0047 |
| 2 | Composite | 3 | 14 | 18265317.62 | 2.9518 | -0.0297 |
| 2 | Flexible | 3 | 14 | 5.14982E+22 | 3.9923 | -0.0055 |
| 4 | All | 1 or 2 | 14 | 289.443 | 1.5248 | -0.0211 |
| 1 | All | All | 14 | 792.3727 | 1.8366 | -0.0405 |
| A | All | All | 14 | 4.18279E+13 | 3.5095 | -0.0099 |
| 1 | Composite | All | 2-3-6 | 112.2041 | 1.3241 | -0.042 |
| 1 | Flexible | All | 2-3-6 | 510.6388 | 1.729 | -0.0364 |
| C | All | 1 | 7-8-9 | 8.50373E+13 | 3.534 | -0.0097 |
| 3 | All | 2 | 7-8-9 | 6218479.458 | 2.8513 | -0.0153 |
| C | All | 2 | 7-8-9 | 1.26984E+14 | 3.537 | -0.0092 |
| C | All | 3 | 7-8-9 | 80144088.86 | 2.9847 | -0.0161 |
| 3 | Composite | 3 | 7-8-9 | 5.64932E+22 | 3.9899 | -0.008 |
| 3 | Flexible | 3 | 7-8-9 | 7.27463E+16 | 3.6944 | -0.0097 |
| 1 | Composite | All | 7-8-9 | 213.0146 | 1.5618 | -0.0778 |
| 2 | Composite | All | 7-8-9 | 7762591.691 | 2.8461 | -0.0167 |
| 4 | Composite | All | 7-8-9 | 620288.2706 | 2.4844 | -0.0038 |
| 6 | Composite | All | 7-8-9 | 76.1753 | 1.5778 | -0.0364 |
| 10 | Composite | All | 7-8-9 | Ld1=76.1753;li1=1.5778;ls1=-0.0364;Ld2=467.7251;li2=1.9154;ls2=-0.0684; |  |  |
| 1 | Flexible | All | 7-8-9 | 4657209.606 | 2.7342 | -0.022 |
| 2 | Flexible | All | 7-8-9 | 9.59826E+13 | 3.5271 | -0.0089 |
| 4 | Flexible | All | 7-8-9 | 1532.5528 | 1.7333 | -0.0123 |
| 6 | Flexible | All | 7-8-9 | 4.00975E+11 | 3.3658 | -0.0228 |
| 7 | Flexible | All | 7-8-9 | 645.599 | 1.9055 | -0.073 |
| 10 | Flexible | All | 7-8-9 | Ld1=400975145472.392;li1=3.3658;ls1=-0.0228;Ld2=645.599;li2=1.9055;ls2=-0.073; |  |  |
| 6 | Composite | All | 11-12-14 | 56.1012 | 1.6191 | -0.0758 |
| 10 | Composite | All | 11-12-14 | Ld1=56.1012;li1=1.6191;ls1=-0.0758;Ld2=467.7251;li2=1.9154;ls2=-0.0684; |  |  |
| 6 | Flexible | All | 11-12-14 | 252636546.4 | 3.0582 | -0.0175 |
| 7 | Flexible | All | 11-12-14 | 109.6255 | 1.7694 | -0.0889 |
| 10 | Flexible | All | 11-12-14 | Ld1=252636546.4457;li1=3.0582;ls1=-0.0175;Ld2=109.6255;li2=1.7694;ls2=-0.0889; |  |  |
| 3 | All | 2 | 11-12 | 4.5551E+39 | 4.5402 | -0.0019 |
| 5 | All | 2 | 11-12 | 3.12277E+28 | 4.2252 | -0.0032 |
| 3 | All | 3 | 11-12 | 37964958115 | 3.2774 | -0.0123 |
| 5 | All | 1 or 3 | 11-12 | 76695580.37 | 3.0379 | -0.0123 |
| 1 | All | All | 11-12 | 7.28212E+25 | 4.1089 | -0.0019 |
| 4 | All | All | 11-12 | 1.19669E+11 | 3.2652 | -0.0123 |
| A | All | All | 11-12 | 257.2113 | 2.2446 | -0.0478 |
| C | All | All | 11-12 | 1.08028E+19 | 3.8394 | -0.0081 |
| 2 | Composite | All | 11-12 | 2.84956E+12 | 3.415 | -0.0068 |
| 2 | Flexible | All | 11-12 | 5.08808E+20 | 3.9143 | -0.0074 |
| B | All | 2 | 11-12-14-16-17-18-19 | 1.26807E+14 | 3.576 | -0.0063 |
| C | All | 1 | 1-2-3-6 | 4.18041E+21 | 3.9646 | -0.0064 |
| 2 | Composite | 1 | 1-2-3-6 | 1052792.426 | 2.7573 | -0.0252 |
| 3 | All | 2 | 1-2-3-6 | 1.83458E+28 | 4.2083 | -0.005 |
| C | All | 2 | 1-2-3-6 | 228005.7859 | 2.6686 | -0.0206 |
| 2 | Composite | 2 | 1-2-3-6 | 1.06513E+11 | 3.3238 | -0.0154 |
| 3 | All | 3 | 1-2-3-6 | 442.6717 | 2.0893 | -0.052 |
| 4 | All | 3 | 1-2-3-6 | 84.7338 | 1.3176 | -0.0884 |
| 2 | Composite | 3 | 1-2-3-6 | 2599368.844 | 2.8142 | -0.018 |
| C | Composite | 3 | 1-2-3-6 | 14868.0287 | 2.4005 | -0.0265 |
| C | Flexible | 3 | 1-2-3-6 | 1.29154E+16 | 3.6623 | -0.0064 |
| 4 | All | 1 or 2 | 1-2-3-6 | 2716.8385 | 1.9974 | -0.0138 |
| 6 | Composite | All | 1-2-3-6 | 22015.125 | 2.5034 | -0.0316 |
| 10 | Composite | All | 1-2-3-6 | Ld1=22015.125;li1=2.5034;ls1=-0.0316;Ld2=467.7251;li2=1.9154;ls2=-0.0684; |  |  |
| 2 | Flexible | All | 1-2-3-6 | 1.47011E+17 | 3.7223 | -0.0052 |
| 6 | Flexible | All | 1-2-3-6 | 1.21619E+22 | 3.9576 | -0.0054 |
| 7 | Flexible | All | 1-2-3-6 | 12702.3337 | 2.4296 | -0.0567 |
| 10 | Flexible | All | 1-2-3-6 | Ld1=121.619407645123*Math.Pow(10,20);li1=3.9576;ls1=-0.0054;Ld2=12702.3337;li2=2.4296;ls2=-0.0567; |  |  |
| B | All | 2 | 1-2-3-6-7-8-9 | 582.5478 | 2.2089 | -0.0197 |
| 2 | Composite | 1 | 16-17-18-19 | 1.94491E+17 | 3.7262 | -0.0082 |
| 2 | Flexible | 1 | 16-17-18-19 | 7809.8883 | 2.3498 | -0.0355 |
| 3 | All | 2 | 16-17-18-19 | 207.9771 | 1.9912 | -0.0568 |
| 2 | Composite | 2 | 16-17-18-19 | 1721605.631 | 2.7579 | -0.0209 |
| 2 | Flexible | 2 | 16-17-18-19 | 4.72506E+21 | 3.9406 | -0.0059 |
| 3 | All | 3 | 16-17-18-19 | 2.61569E+17 | 3.7161 | -0.0061 |
| 4 | All | 3 | 16-17-18-19 | 560.9015 | 1.555 | -0.0141 |
| 2 | Composite | 3 | 16-17-18-19 | 69.4426 | 1.7037 | -0.0774 |
| 2 | Flexible | 3 | 16-17-18-19 | 671.6257 | 2.2066 | -0.0565 |
| 4 | Composite | 1 or 2 | 16-17-18-19 | 372.3782 | 1.5649 | -0.0217 |
| 4 | Flexible | 1 or 2 | 16-17-18-19 | 2067.2575 | 1.7989 | -0.0119 |
| A | All | All | 16-17-18-19 | 739.2838 | 2.0345 | -0.0385 |
| 1 | Composite | All | 16-17-18-19 | 2.38079E+17 | 3.6591 | -0.0027 |
| 6 | Composite | All | 16-17-18-19 | 1958.3861 | 2.0325 | -0.0116 |
| 10 | Composite | All | 16-17-18-19 | Ld1=1958.3861;li1=2.0325;ls1=-0.0116;Ld2=467.7251;li2=1.9154;ls2=-0.0684; |  |  |
| C | Composite | All | 16-17-18-19 | 726334947.4 | 3.0971 | -0.0132 |
| 1 | Flexible | All | 16-17-18-19 | 3.72605E+11 | 3.2569 | -0.0065 |
| 6 | Flexible | All | 16-17-18-19 | 128076.7042 | 2.5111 | -0.0236 |
| 7 | Flexible | All | 16-17-18-19 | 51.1047 | 1.7873 | -0.2267 |
| 10 | Flexible | All | 16-17-18-19 | Ld1=128076.7042;li1=2.5111;ls1=-0.0236;Ld2=51.1047;li2=1.7873;ls2=-0.2267; |  |  |
| C | Flexible | All | 16-17-18-19 | 4730810253 | 3.1961 | -0.0152 |
| 5 | All | All | 16-17-18-19-14 | 45.6078 | 1.8813 | -0.1221 |
| 5 | All | All | 1-7-8-9-2-3-6 | 3.24984E+33 | 4.3845 | -0.0021 |
| A | All | All | 1-7-8-9-2-3-6 | 54.3291 | 1.9695 | -0.1039 |
| 3 | All | 1 | All | 32864163.85 | 2.9756 | -0.0152 |
| B | All | 1 | All | 32864163.85 | 2.9756 | -0.0152 |
| B | All | 3 | All | 32864163.85 | 2.9756 | -0.0152 |
| 8 | All | All | All | 32864163.85 | 2.9756 | -0.0152 |
| 7 | Composite | All | All | 467.7251 | 1.9154 | -0.0684 |
| N/A | N/A | 1 | 1 | 1920 | 2.32 | -0.02 |
| N/A | N/A | 2 | 1 | 1500 | 2.27 | -0.01 |
| N/A | N/A | 3 | 1 | 1500 | 2.27 | -0.01 |
| N/A | N/A | 1 | 2-3-6 | 8.08E+11 | 3.35 | -0.01 |
| N/A | N/A | 2 | 2-3-6 | 31300 | 2.4 | -0.02 |
| N/A | N/A | 3 | 2-3-6 | 8.38E+17 | 3.76 | -0.01 |
| N/A | N/A | 1 | 7-8-9 | 658 | 2.01 | -0.05 |
| N/A | N/A | 2 | 7-8-9 | 746 | 1.97 | -0.03 |
| N/A | N/A | 3 | 7-8-9 | 420 | 1.99 | -0.06 |
| N/A | N/A | 1 | 11-12 | 1.12E+24 | 4.04 | -0.01 |
| N/A | N/A | 2 | 11-12 | 6.65E+19 | 3.87 | -0.01 |
| N/A | N/A | 3 | 11-12 | 3.05E+12 | 3.42 | -0.01 |
| N/A | N/A | 1 | 14 | 4.06E+17 | 3.74 | -0.01 |
| N/A | N/A | 2 | 14 | 586 | 1.96 | -0.04 |
| N/A | N/A | 3 | 14 | 8.92E+12 | 3.44 | -0.01 |
| N/A | N/A | 1 | 16-17-18-19 | 727 | 2.03 | -0.04 |
| N/A | N/A | 2 | 16-17-18-19 | 654 | 1.93 | -0.03 |
| N/A | N/A | 3 | 16-17-18-19 | 656 | 1.98 | -0.04 |

Table 2: SC Density Model parameters

| # | Last applied treatment |
| --- | --- |
| 1 | ASPHALT PATCH ONLY, POTHOLE PATCHING |
| 2 | MILL-OVERLAY <=1.5IN GRADE INCREASE |
| 3 | OVERLAY <=1.5IN ASPHALT, MILL-ULTRATHIN BONDED WEARING COURSE |
| 4 | CRACK SEAL, JOINT RESEALING |
| 5 | MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP |
| 6 | THIN OVERLAY <=1IN ASPHALT, HOT-IN-PLACE RECYCLING |
| 7 | MICRO SURFACING, CAPE SEAL, HIGH FRICTION SURFACE |
| 8 | OVERLAY <=1.5IN ASPHALT - GAP, ULTRATHIN BONDED WEARING COURSE |
| 9 | SURFACE ABRASION |
| 10 | FOG SEAL, REJUVANATOR, CHIP SEAL, SAND SEAL |
| A | RECONSTRUCTION, MILL >=4IN-OVERLAY WITH ASPHALT, |
|  | RUBBILIZATION-OVERLAY, COLD-IN-PLACE RECYCLING-OVERLAY, |
|  | BREAK-CRACK-SEAT-OVERLAY, FULL-DEPTH RECLAMATION-OVERLAY |
| B | OVERLAY >1.5IN ASPHALT - GAP, MILL-OVERLAY >1.5IN GRADE INCREASE - GAP |
| C | OVERLAY >1.5IN ASPHALT, MILL-OVERLAY >1.5IN GRADE INCREASE |

Table 3: Treatment Group definitions

| Functional Class | Description |
| --- | --- |
| 1 | Rural Principal Arterial - Interstate |
| 2 | Rural Principal Arterial - Other Freeways |
| 3 | Rural Principal Arterial - Other |
| 6 | Rural Minor Arterial |
| 7 | Rural Major Collector |
| 8 | Rural Minor Collector |
| 9 | Rural Local |
| 11 | Urban Principal Arterial - Interstate |
| 12 | Urban Principal Arterial - Other Freeways |
| 14 | Urban Principal Arterial - Other |
| 16 | Urban Minor Arterial |
| 17 | Urban Major Collector |
| 18 | Urban Minor Collector |
| 19 | Urban Local |

Table 4: Highway Functional Classes

| 1 = Mountain | 2 = Central | 3 = Coastal |
| --- | --- | --- |

Table 5: Region Definitions

FC Density Function

// FC Density Function
double FC0, FCage, FCDMR = [FC_DENSITY_MR], age = [AGE], Ld = 0, li = 0, ls = 0, gt = 0, Ld1 = 0, li1 = 0, ls1 = 0, Ld2 = 0, li2 = 0, ls2 = 0;
string PT = [PAVEMENT_TYPE], RG = [REGION], LT = [LAST_TREATMENT], FC = [FUNC_CLASS];

if ((FC == "1") && (gt == 0)) { if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 90637719.4409; li = 2.8083; ls = -0.0046; gt = 1; } }
if ((FC == "14") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 266.3311; li = 1.4455; ls = -0.0255; gt = 1; }
    else if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 1787.4858; li = 1.948; ls = -0.0111; gt = 1; }
    else if (RG == "1" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 6.03267467669406 * Math.Pow(10, 20); li = 3.8714; ls = -0.0029; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 10694218632770800000 * Math.Pow(10, 20); li = 4.4886; ls = -0.0009; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 86.8394; li = 1.6175; ls = -0.0618; gt = 1; }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 150.0416; li = 1.3954; ls = -0.05; gt = 1; }
    else if (RG == "2" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 275.4735; li = 1.6585; ls = -0.0442; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 4103.2955; li = 2.2241; ls = -0.0275; gt = 1; }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 55.5406; li = 1.0076; ls = -0.1419; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 41969.4996; li = 2.3521; ls = -0.0217; gt = 1; }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 117.5573; li = 1.4525; ls = -0.0749; gt = 1; }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 125.2176; li = 1.5761; ls = -0.0606; gt = 1; }
    else if ((RG == "1" || RG == "2") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 76.2935; li = 1.0401; ls = -0.0643; gt = 1; }
    else if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 1002.87617756533 * Math.Pow(10, 15); li = 3.6796; ls = -0.0019; gt = 1; }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY")) { Ld = 215600901.711267 * Math.Pow(10, 10); li = 3.7308; ls = -0.0031; gt = 1; }
}
if ((FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 584.7963; li = 1.464; ls = -0.0102; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 138.1563; li = 1.1843; ls = -0.0417; gt = 1; }
}
if ((FC == "7" || FC == "8" || FC == "9") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 305.5609; li = 1.7251; ls = -0.031; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 1956.3351; li = 2.0539; ls = -0.0189; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 599.4863; li = 1.8873; ls = -0.0317; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 2.10797424508439 * Math.Pow(10, 10); li = 3.1608; ls = -0.0081; gt = 1; }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 621395.355961961 * Math.Pow(10, 10); li = 3.5844; ls = -0.0067; gt = 1; }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 1361.205; li = 1.941; ls = -0.0285; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 215.1996; li = 1.3605; ls = -0.0575; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 341.6614; li = 1.7043; ls = -0.0338; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 604.754119009586 * Math.Pow(10, 20); li = 3.9402; ls = -0.0045; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 50.5424; li = 1.0538; ls = -0.0821; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 50.5424; li1 = 1.0538; ls1 = -0.0821; Ld2 = 573.153; li2 = 1.5699; ls2 = -0.0145; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 109.2627; li = 1.248; ls = -0.0402; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 6966.2944; li = 2.1961; ls = -0.0176; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 145528821.0712; li = 2.8511; ls = -0.0076; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 60.3093; li = 1.4039; ls = -0.1222; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE")) { Ld = 10471052.5813; li = 2.7114; ls = -0.0162; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 60.3093; li1 = 1.4039; ls1 = -0.1222; Ld2 = 10471052.5813; li2 = 2.7114; ls2 = -0.0162; gt = 1; }
}
if ((FC == "11" || FC == "12" || FC == "14") && (gt == 0))
{
    if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 76.9602; li = 1.2325; ls = -0.073; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 76.9602; li1 = 1.2325; ls1 = -0.073; Ld2 = 573.153; li2 = 1.5699; ls2 = -0.0145; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 98973660234.9642 * Math.Pow(10, 20); li = 4.2652; ls = -0.0023; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE")) { Ld = 1882.4205; li = 1.9344; ls = -0.0382; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 98973660234.9642 * Math.Pow(10, 20); li1 = 4.2652; ls1 = -0.0023; Ld2 = 1882.4205; li2 = 1.9344; ls2 = -0.0382; gt = 1; }
}
if ((FC == "11" || FC == "12") && (gt == 0))
{
    if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 89359137.6056; li = 2.9464; ls = -0.015; gt = 1; }
    else if (RG == "2" && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP")) { Ld = 1662.1596; li = 1.9936; ls = -0.0288; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 3794.5556; li = 1.951; ls = -0.0064; gt = 1; }
    else if ((RG == "1" || RG == "3") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP")) { Ld = 50.4719; li = 1.1323; ls = -0.0834; gt = 1; }
    else if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 22.7746814420507 * Math.Pow(10, 15); li = 3.5918; ls = -0.0025; gt = 1; }
    else if ((LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 50.3632; li = 0.8305; ls = -0.0947; gt = 1; }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY")) { Ld = 2683.6544; li = 2.0435; ls = -0.0171; gt = 1; }
    else if ((LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 2492522026.8003; li = 3.0629; ls = -0.009; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 111.6238; li = 1.2991; ls = -0.053; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 2516.3924; li = 2.013; ls = -0.0291; gt = 1; }
}
if ((FC == "11" || FC == "12" || FC == "14" || FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0)) { if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP")) { Ld = 600; li = 1.9327; ls = -0.0345; gt = 1; } }
if ((FC == "1" || FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 106.2486; li = 1.5597; ls = -0.0593; gt = 1; }
    else if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 97.2449; li = 1.3891; ls = -0.0457; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 431.2878; li = 1.7957; ls = -0.0347; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 71606660.6601704 * Math.Pow(10, 10); li = 3.7176; ls = -0.0039; gt = 1; }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 4709385.6133; li = 2.7561; ls = -0.0194; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 1789.58; li = 1.9201; ls = -0.0212; gt = 1; }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 78.314; li = 1.1183; ls = -0.1146; gt = 1; }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 28894462.1571; li = 2.8285; ls = -0.0165; gt = 1; }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 170.3627; li = 1.499; ls = -0.0455; gt = 1; }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 397.2458; li = 1.7853; ls = -0.0385; gt = 1; }
    else if ((RG == "1" || RG == "2") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 13659.7542; li = 2.1062; ls = -0.0159; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 6073.429; li = 2.1367; ls = -0.0255; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 6073.429; li1 = 2.1367; ls1 = -0.0255; Ld2 = 573.153; li2 = 1.5699; ls2 = -0.0145; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 367611295.5788; li = 2.9757; ls = -0.0088; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 951.8987; li = 1.9191; ls = -0.0352; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE")) { Ld = 201234894.491; li = 2.9441; ls = -0.0166; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 951.8987; li1 = 1.9191; ls1 = -0.0352; Ld2 = 201234894.491; li2 = 2.9441; ls2 = -0.0166; gt = 1; }
}
if ((FC == "1" || FC == "2" || FC == "3" || FC == "6" || FC == "7" || FC == "8" || FC == "9") && (gt == 0)) { if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP")) { Ld = 93879.8842929807 * Math.Pow(10, 10); li = 3.5453; ls = -0.005; gt = 1; } }
if ((FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0))
{
    if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 1032.0372; li = 1.8994; ls = -0.025; gt = 1; }
    else if (RG == "1" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 5437.0356; li = 2.1001; ls = -0.0225; gt = 1; }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 860.3014; li = 1.938; ls = -0.0366; gt = 1; }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 437.9597; li = 1.735; ls = -0.0345; gt = 1; }
    else if (RG == "2" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 720.4722; li = 1.8432; ls = -0.0266; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 573.903; li = 1.7447; ls = -0.0175; gt = 1; }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 1382900.3943; li = 2.5519; ls = -0.0148; gt = 1; }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 404.1978; li = 1.6935; ls = -0.0352; gt = 1; }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE")) { Ld = 130.2758; li = 1.7928; ls = -0.0733; gt = 1; }
    else if ((RG == "1" || RG == "2") && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 14369723.9977432 * Math.Pow(10, 20); li = 4.1141; ls = -0.0035; gt = 1; }
    else if ((RG == "1" || RG == "2") && (PT == "F" || PT == "FLEXIBLE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING")) { Ld = 58.1315; li = 1.0264; ls = -0.0862; gt = 1; }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY")) { Ld = 19646019564.2089; li = 3.1712; ls = -0.009; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 511.7243; li = 1.5606; ls = -0.029; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 10401445555430400 * Math.Pow(10, 20); li = 4.402; ls = -0.0008; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 10401445555430400 * Math.Pow(10, 20); li1 = 4.402; ls1 = -0.0008; Ld2 = 573.153; li2 = 1.5699; ls2 = -0.0145; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 2704.5306; li = 2.0572; ls = -0.0246; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING")) { Ld = 12733.4191692852 * Math.Pow(10, 20); li = 3.9863; ls = -0.0011; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING")) { Ld = 221778.766348933 * Math.Pow(10, 30); li = 4.384; ls = -0.0012; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE")) { Ld = 73.0759; li = 1.4134; ls = -0.151; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")) { Ld1 = 221778.766348933 * Math.Pow(10, 30); li1 = 4.384; ls1 = -0.0012; Ld2 = 73.0759; li2 = 1.4134; ls2 = -0.151; gt = 1; }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE")) { Ld = 44859729.159; li = 2.8931; ls = -0.0125; gt = 1; }
}
if ((FC == "16" || FC == "17" || FC == "18" || FC == "19" || FC == "14") && (gt == 0)) { if ((LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP")) { Ld = 398.49; li = 1.7958; ls = -0.0324; gt = 1; } }
if ((FC == "1" || FC == "7" || FC == "8" || FC == "9" || FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if ((LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP")) { Ld = 2701.4546; li = 2.077; ls = -0.0293; gt = 1; }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY")) { Ld = 62120662.968; li = 2.8845; ls = -0.0138; gt = 1; }
}
if (gt == 0)
{
    if (RG == "1" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE")) { Ld = 122962.0574; li = 2.4522; ls = -0.0095; gt = 1; }
    else if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP")) { Ld = 2013.4766; li = 2.079; ls = -0.0346; gt = 1; }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP")) { Ld = 1709.5229; li = 1.9365; ls = -0.0165; gt = 1; }
    else if ((LT == "OVERLAY <=1.5IN ASPHALT - GAP" || LT == "ULTRATHIN BONDED WEARING COURSE")) { Ld = 171115888500.86; li = 3.2614; ls = -0.0093; gt = 1; }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE")) { Ld = 573.153; li = 1.5699; ls = -0.0145; gt = 1; }
}
if ((FC == "1") && (gt == 0))
{
    if (RG == "1") { Ld = 399 * Math.Pow(10, 20); li = 3.93; ls = -0.01; gt = 1; }
    else if (RG == "2") { Ld = 280; li = 1.75; ls = -0.04; gt = 1; }
    else if (RG == "3") { Ld = 280; li = 1.75; ls = -0.04; gt = 1; }
}
if ((FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if (RG == "1") { Ld = 763; li = 1.72; ls = -0.01; gt = 1; }
    else if (RG == "2") { Ld = 108; li = 1.34; ls = -0.04; gt = 1; }
    else if (RG == "3") { Ld = 29300; li = 2.28; ls = -0.02; gt = 1; }
}
if ((FC == "7" || FC == "8" || FC == "9") && (gt == 0))
{
    if (RG == "1") { Ld = 220; li = 1.58; ls = -0.03; gt = 1; }
    else if (RG == "2") { Ld = 1170; li = 1.88; ls = -0.02; gt = 1; }
    else if (RG == "3") { Ld = 582; li = 1.77; ls = -0.04; gt = 1; }
}
if ((FC == "11" || FC == "12") && (gt == 0))
{
    if (RG == "1") { Ld = 593; li = 1.61; ls = -0.02; gt = 1; }
    else if (RG == "2") { Ld = 718; li = 1.82; ls = -0.03; gt = 1; }
    else if (RG == "3") { Ld = 378; li = 1.67; ls = -0.03; gt = 1; }
}
if ((FC == "14") && (gt == 0))
{
    if (RG == "1") { Ld = 104; li = 1.42; ls = -0.03; gt = 1; }
    else if (RG == "2") { Ld = 238; li = 1.52; ls = -0.04; gt = 1; }
    else if (RG == "3") { Ld = 631; li = 1.82; ls = -0.04; gt = 1; }
}
if ((FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0))
{
    if (RG == "1") { Ld = 325; li = 1.63; ls = -0.02; gt = 1; }
    else if (RG == "2") { Ld = 104; li = 1.35; ls = -0.04; gt = 1; }
    else if (RG == "3") { Ld = 153; li = 1.5; ls = -0.04; gt = 1; }
}

if (FCDMR == 0) { FCDMR = 0.01; }
if (LT == "MILL-ULTRATHIN BONDED WEARING COURSE" || LT == "ULTRATHIN BONDED WEARING COURSE")
{
    FCage = Ld * Math.Exp(-Math.Exp(li + ls * age)) * 4;
    FC0 = Ld * Math.Exp(-Math.Exp(li + ls * 0)) * 4;
}
else if (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")
{
    FCage = Ld1 * Math.Exp(-Math.Exp(li1 + ls1 * age)) * 0.4 + Ld2 * Math.Exp(-Math.Exp(li2 + ls2 * age)) * 0.6;
    FC0 = Ld1 * Math.Exp(-Math.Exp(li1 + ls1 * 0)) * 0.4 + Ld2 * Math.Exp(-Math.Exp(li2 + ls2 * 0)) * 0.6;
}
else
{
    FCage = Ld * Math.Exp(-Math.Exp(li + ls * age));
    FC0 = Ld * Math.Exp(-Math.Exp(li + ls * 0));
}
if (FCDMR <= FC0) { Answer = FCage - FC0; }
else { Answer = FCage; }
return Answer;

SC Density Function

```csharp
double SC0, SCage, SCDMR = [SC_DENSITY_MR], age = [AGE], Ld = 0, li = 0, ls = 0, gt = 0, Ld1 = 0, li1 = 0, ls1 = 0, Ld2 = 0, li2 = 0, ls2 = 0;
string PT = [PAVEMENT_TYPE], RG = [REGION], LT = [LAST_TREATMENT], FC = [FUNC_CLASS];

if ((FC == "1") && (gt == 0))
{
    if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 75.4278;
        li = 1.8473;
        ls = -0.1143;
        gt = 1;
    }
}

if ((FC == "14") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 475248.294659273 * Math.Pow(10, 20);
        li = 4.0944;
        ls = -0.0029;
        gt = 1;
    }
    else if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 347.061977009049 * Math.Pow(10, 20);
        li = 3.9847;
        ls = -0.0035;
        gt = 1;
    }
    else if (RG == "1" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 298.7579;
        li = 2.0722;
        ls = -0.0526;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 381056.048639361 * Math.Pow(10, 30);
        li = 4.4145;
        ls = -0.002;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 93.7206;
        li = 1.9854;
        ls = -0.0693;
        gt = 1;
    }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 232006421.0355;
        li = 3.024;
        ls = -0.0152;
        gt = 1;
    }
    else if (RG == "2" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 4896371.86214469 * Math.Pow(10, 10);
        li = 3.6892;
        ls = -0.0084;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 335974272.41171 * Math.Pow(10, 25);
        li = 4.384;
        ls = -0.0036;
        gt = 1;
    }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 262.8322;
        li = 1.4992;
        ls = -0.0224;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 652.682102347311 * Math.Pow(10, 20);
        li = 3.9954;
        ls = -0.0047;
        gt = 1;
    }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 18265317.6231;
        li = 2.9518;
        ls = -0.0297;
        gt = 1;
    }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 514.982069631293 * Math.Pow(10, 20);
        li = 3.9923;
        ls = -0.0055;
        gt = 1;
    }
    else if ((RG == "1" || RG == "2") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 289.443;
        li = 1.5248;
        ls = -0.0211;
        gt = 1;
    }
    else if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 792.3727;
        li = 1.8366;
        ls = -0.0405;
        gt = 1;
    }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY"))
    {
        Ld = 4182.78688673652 * Math.Pow(10, 10);
        li = 3.5095;
        ls = -0.0099;
        gt = 1;
    }
}

if ((FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 112.2041;
        li = 1.3241;
        ls = -0.042;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 510.6388;
        li = 1.729;
        ls = -0.0364;
        gt = 1;
    }
}

if ((FC == "7" || FC == "8" || FC == "9") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 85037347834966;
        li = 3.534;
        ls = -0.0097;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 6218479.4581;
        li = 2.8513;
        ls = -0.0153;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 126984274512314;
        li = 3.537;
        ls = -0.0092;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 80144088.8551;
        li = 2.9847;
        ls = -0.0161;
        gt = 1;
    }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 564.932399692712 * Math.Pow(10, 20);
        li = 3.9899;
        ls = -0.008;
        gt = 1;
    }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 7274634.69497847 * Math.Pow(10, 10);
        li = 3.6944;
        ls = -0.0097;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 213.0146;
        li = 1.5618;
        ls = -0.0778;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 7762591.6914;
        li = 2.8461;
        ls = -0.0167;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 620288.2706;
        li = 2.4844;
        ls = -0.0038;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 76.1753;
        li = 1.5778;
        ls = -0.0364;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 76.1753;
        li1 = 1.5778;
        ls1 = -0.0364;
        Ld2 = 467.7251;
        li2 = 1.9154;
        ls2 = -0.0684;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 4657209.6055;
        li = 2.7342;
        ls = -0.022;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 95982573657203;
        li = 3.5271;
        ls = -0.0089;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 1532.5528;
        li = 1.7333;
        ls = -0.0123;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 400975145472.392;
        li = 3.3658;
        ls = -0.0228;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE"))
    {
        Ld = 645.599;
        li = 1.9055;
        ls = -0.073;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 400975145472.392;
        li1 = 3.3658;
        ls1 = -0.0228;
        Ld2 = 645.599;
        li2 = 1.9055;
        ls2 = -0.073;
        gt = 1;
    }
}

if ((FC == "11" || FC == "12" || FC == "14") && (gt == 0))
{
    if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 56.1012;
        li = 1.6191;
        ls = -0.0758;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 56.1012;
        li1 = 1.6191;
        ls1 = -0.0758;
        Ld2 = 467.7251;
        li2 = 1.9154;
        ls2 = -0.0684;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 252636546.4457;
        li = 3.0582;
        ls = -0.0175;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE"))
    {
        Ld = 109.6255;
        li = 1.7694;
        ls = -0.0889;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 252636546.4457;
        li1 = 3.0582;
        ls1 = -0.0175;
        Ld2 = 109.6255;
        li2 = 1.7694;
        ls2 = -0.0889;
        gt = 1;
    }
}

if ((FC == "11" || FC == "12") && (gt == 0))
{
    if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 4555096024.28126 * Math.Pow(10, 30);
        li = 4.5402;
        ls = -0.0019;
        gt = 1;
    }
    else if (RG == "2" && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 312277401.799835 * Math.Pow(10, 20);
        li = 4.2252;
        ls = -0.0032;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 37964958115.2038;
        li = 3.2774;
        ls = -0.0123;
        gt = 1;
    }
    else if ((RG == "1" || RG == "3") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 76695580.3749;
        li = 3.0379;
        ls = -0.0123;
        gt = 1;
    }
    else if ((LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 728211.849279491 * Math.Pow(10, 20);
        li = 4.1089;
        ls = -0.0019;
        gt = 1;
    }
    else if ((LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 119668831116.782;
        li = 3.2652;
        ls = -0.0123;
        gt = 1;
    }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY"))
    {
        Ld = 257.2113;
        li = 2.2446;
        ls = -0.0478;
        gt = 1;
    }
    else if ((LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 1080282746.3041 * Math.Pow(10, 10);
        li = 3.8394;
        ls = -0.0081;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 2849556354718;
        li = 3.415;
        ls = -0.0068;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 5.08808444115597 * Math.Pow(10, 20);
        li = 3.9143;
        ls = -0.0074;
        gt = 1;
    }
}

if ((FC == "11" || FC == "12" || FC == "14" || FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0))
{
    if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 126806671084707;
        li = 3.576;
        ls = -0.0063;
        gt = 1;
    }
}

if ((FC == "1" || FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 41.8040558403263 * Math.Pow(10, 20);
        li = 3.9646;
        ls = -0.0064;
        gt = 1;
    }
    else if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 1052792.4264;
        li = 2.7573;
        ls = -0.0252;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 183457661.715306 * Math.Pow(10, 20);
        li = 4.2083;
        ls = -0.005;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 228005.7859;
        li = 2.6686;
        ls = -0.0206;
        gt = 1;
    }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 106513001677.249;
        li = 3.3238;
        ls = -0.0154;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 442.6717;
        li = 2.0893;
        ls = -0.052;
        gt = 1;
    }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 84.7338;
        li = 1.3176;
        ls = -0.0884;
        gt = 1;
    }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 2599368.8437;
        li = 2.8142;
        ls = -0.018;
        gt = 1;
    }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 14868.0287;
        li = 2.4005;
        ls = -0.0265;
        gt = 1;
    }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 12915430597549900;
        li = 3.6623;
        ls = -0.0064;
        gt = 1;
    }
    else if ((RG == "1" || RG == "2") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 2716.8385;
        li = 1.9974;
        ls = -0.0138;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 22015.125;
        li = 2.5034;
        ls = -0.0316;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 22015.125;
        li1 = 2.5034;
        ls1 = -0.0316;
        Ld2 = 467.7251;
        li2 = 1.9154;
        ls2 = -0.0684;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 14701100.3697198 * Math.Pow(10, 10);
        li = 3.7223;
        ls = -0.0052;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 121.619407645123 * Math.Pow(10, 20);
        li = 3.9576;
        ls = -0.0054;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE"))
    {
        Ld = 12702.3337;
        li = 2.4296;
        ls = -0.0567;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 121.619407645123 * Math.Pow(10, 20);
        li1 = 3.9576;
        ls1 = -0.0054;
        Ld2 = 12702.3337;
        li2 = 2.4296;
        ls2 = -0.0567;
        gt = 1;
    }
}

if ((FC == "1" || FC == "2" || FC == "3" || FC == "6" || FC == "7" || FC == "8" || FC == "9") && (gt == 0))
{
    if (RG == "2" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 582.5478;
        li = 2.2089;
        ls = -0.0197;
        gt = 1;
    }
}

if ((FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0))
{
    if (RG == "1" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 19449091.0508668 * Math.Pow(10, 10);
        li = 3.7262;
        ls = -0.0082;
        gt = 1;
    }
    else if (RG == "1" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 7809.8883;
        li = 2.3498;
        ls = -0.0355;
        gt = 1;
    }
    else if (RG == "2" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 207.9771;
        li = 1.9912;
        ls = -0.0568;
        gt = 1;
    }
    else if (RG == "2" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 1721605.6314;
        li = 2.7579;
        ls = -0.0209;
        gt = 1;
    }
    else if (RG == "2" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 47.2506007250682 * Math.Pow(10, 20);
        li = 3.9406;
        ls = -0.0059;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 26156886.9283831 * Math.Pow(10, 10);
        li = 3.7161;
        ls = -0.0061;
        gt = 1;
    }
    else if (RG == "3" && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 560.9015;
        li = 1.555;
        ls = -0.0141;
        gt = 1;
    }
    else if (RG == "3" && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 69.4426;
        li = 1.7037;
        ls = -0.0774;
        gt = 1;
    }
    else if (RG == "3" && (PT == "F" || PT == "FLEXIBLE") && (LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE"))
    {
        Ld = 671.6257;
        li = 2.2066;
        ls = -0.0565;
        gt = 1;
    }
    else if ((RG == "1" || RG == "2") && (PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 372.3782;
        li = 1.5649;
        ls = -0.0217;
        gt = 1;
    }
    else if ((RG == "1" || RG == "2") && (PT == "F" || PT == "FLEXIBLE") && (LT == "CRACK SEAL" || LT == "JOINT RESEALING"))
    {
        Ld = 2067.2575;
        li = 1.7989;
        ls = -0.0119;
        gt = 1;
    }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY"))
    {
        Ld = 739.2838;
        li = 2.0345;
        ls = -0.0385;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 238.079002878799 * Math.Pow(10, 15);
        li = 3.6591;
        ls = -0.0027;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 1958.3861;
        li = 2.0325;
        ls = -0.0116;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 1958.3861;
        li1 = 2.0325;
        ls1 = -0.0116;
        Ld2 = 467.7251;
        li2 = 1.9154;
        ls2 = -0.0684;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 726334947.4193;
        li = 3.0971;
        ls = -0.0132;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "ASPHALT PATCH ONLY" || LT == "POTHOLE PATCHING"))
    {
        Ld = 372604640857.231;
        li = 3.2569;
        ls = -0.0065;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "THIN OVERLAY <=1IN ASPHALT" || LT == "HOT-IN-PLACE RECYCLING"))
    {
        Ld = 128076.7042;
        li = 2.5111;
        ls = -0.0236;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE"))
    {
        Ld = 51.1047;
        li = 1.7873;
        ls = -0.2267;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL"))
    {
        Ld1 = 128076.7042;
        li1 = 2.5111;
        ls1 = -0.0236;
        Ld2 = 51.1047;
        li2 = 1.7873;
        ls2 = -0.2267;
        gt = 1;
    }
    else if ((PT == "F" || PT == "FLEXIBLE") && (LT == "OVERLAY >1.5IN ASPHALT" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE"))
    {
        Ld = 4730810253.4084;
        li = 3.1961;
        ls = -0.0152;
        gt = 1;
    }
}

if ((FC == "16" || FC == "17" || FC == "18" || FC == "19" || FC == "14") && (gt == 0))
{
    if ((LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 45.6078;
        li = 1.8813;
        ls = -0.1221;
        gt = 1;
    }
}

if ((FC == "1" || FC == "7" || FC == "8" || FC == "9" || FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if ((LT == "MILL-OVERLAY <=1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 324983517.00409 * Math.Pow(10, 25);
        li = 4.3845;
        ls = -0.0021;
        gt = 1;
    }
    else if ((LT == "RECONSTRUCTION" || LT == "MILL >=4IN-OVERLAY WITH ASPHALT" || LT == "RUBBILIZATION-OVERLAY" || LT == "COLD-IN-PLACE RECYCLING-OVERLAY" || LT == "BREAK-CRACK-SEAT-OVERLAY" || LT == "FULL-DEPTH RECLAMATION-OVERLAY"))
    {
        Ld = 54.3291;
        li = 1.9695;
        ls = -0.1039;
        gt = 1;
    }
}

if (gt == 0)
{
    if (RG == "1" && (LT == "OVERLAY <=1.5IN ASPHALT" || LT == "MILL-ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 32864163.8456;
        li = 2.9756;
        ls = -0.0152;
        gt = 1;
    }
    else if (RG == "1" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 32864163.8456;
        li = 2.9756;
        ls = -0.0152;
        gt = 1;
    }
    else if (RG == "3" && (LT == "OVERLAY >1.5IN ASPHALT - GAP" || LT == "MILL-OVERLAY >1.5IN GRADE INCREASE - GAP"))
    {
        Ld = 32864163.8456;
        li = 2.9756;
        ls = -0.0152;
        gt = 1;
    }
    else if ((LT == "OVERLAY <=1.5IN ASPHALT - GAP" || LT == "ULTRATHIN BONDED WEARING COURSE"))
    {
        Ld = 32864163.8456;
        li = 2.9756;
        ls = -0.0152;
        gt = 1;
    }
    else if ((PT == "FCC" || PT == "FCJ" || PT == "COMPOSITE") && (LT == "MICRO SURFACING" || LT == "CAPE SEAL" || LT == "HIGH FRICTION SURFACE"))
    {
        Ld = 467.7251;
        li = 1.9154;
        ls = -0.0684;
        gt = 1;
    }
}

if ((FC == "1") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 1920;
        li = 2.32;
        ls = -0.02;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 1500;
        li = 2.27;
        ls = -0.01;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 1500;
        li = 2.27;
        ls = -0.01;
        gt = 1;
    }
}

if ((FC == "2" || FC == "3" || FC == "6") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 808000000000;
        li = 3.35;
        ls = -0.01;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 31300;
        li = 2.4;
        ls = -0.02;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 83800000 * Math.Pow(10, 10);
        li = 3.76;
        ls = -0.01;
        gt = 1;
    }
}

if ((FC == "7" || FC == "8" || FC == "9") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 658;
        li = 2.01;
        ls = -0.05;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 746;
        li = 1.97;
        ls = -0.03;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 420;
        li = 1.99;
        ls = -0.06;
        gt = 1;
    }
}

if ((FC == "11" || FC == "12") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 11200 * Math.Pow(10, 20);
        li = 4.04;
        ls = -0.01;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 6650000000 * Math.Pow(10, 10);
        li = 3.87;
        ls = -0.01;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 305 * Math.Pow(10, 10);
        li = 3.42;
        ls = -0.01;
        gt = 1;
    }
}

if ((FC == "14") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 40600000 * Math.Pow(10, 10);
        li = 3.74;
        ls = -0.01;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 586;
        li = 1.96;
        ls = -0.04;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 892 * Math.Pow(10, 10);
        li = 3.44;
        ls = -0.01;
        gt = 1;
    }
}

if ((FC == "16" || FC == "17" || FC == "18" || FC == "19") && (gt == 0))
{
    if (RG == "1")
    {
        Ld = 727;
        li = 2.03;
        ls = -0.04;
        gt = 1;
    }
    else if (RG == "2")
    {
        Ld = 654;
        li = 1.93;
        ls = -0.03;
        gt = 1;
    }
    else if (RG == "3")
    {
        Ld = 656;
        li = 1.98;
        ls = -0.04;
        gt = 1;
    }
}

if (SCDMR == 0)
{
    SCDMR = 0.01;
}

if (LT == "MILL-ULTRATHIN BONDED WEARING COURSE" || LT == "ULTRATHIN BONDED WEARING COURSE")
{
    SCage = Ld * Math.Exp(-Math.Exp(li + ls * age)) * 4;
    SC0 = Ld * Math.Exp(-Math.Exp(li + ls * 0)) * 4;
}
else if (LT == "FOG SEAL" || LT == "REJUVANATOR" || LT == "CHIP SEAL" || LT == "SAND SEAL")
{
    SCage = Ld1 * Math.Exp(-Math.Exp(li1 + ls1 * age)) * 0.4 + Ld2 * Math.Exp(-Math.Exp(li2 + ls2 * age)) * 0.6;
    SC0 = Ld1 * Math.Exp(-Math.Exp(li1 + ls1 * 0)) * 0.4 + Ld2 * Math.Exp(-Math.Exp(li2 + ls2 * 0)) * 0.6;
}
else
{
    SCage = Ld * Math.Exp(-Math.Exp(li + ls * age));
    SC0 = Ld * Math.Exp(-Math.Exp(li + ls * 0));
}

if (SCDMR <= SC0)
{
    Answer = SCage - SC0;
}
else
{
    Answer = SCage;
}

return Answer;

## Rutting Performance Model

The following tables and scripts provide current information for rutting model parameters.

Rutting Families by Functional Class and Surface Type

| Pavement Type | Functional Class | Family | Slope |
| --- | --- | --- | --- |
| Flexible | 1 | 1 | 0.009734 |
| Flexible | 2, 3 | 2 | 0.006751 |
| Flexible | 6 | 3 | 0.007501 |
| Flexible | 7 | 4 | 0.006067 |
| Flexible | 8 | 5 | 0.005461 |
| Flexible | 9 | 6 | 0.005617 |
| Flexible | 11 | 7 | 0.006402 |
| Flexible | 12 | 8 | 0.007211 |
| Flexible | 14 | 9 | 0.009364 |
| Flexible | 16 | 10 | 0.008818 |
| Flexible | 17 | 11 | 0.006681 |
| Flexible | 18, 19 | 12 | 0.012963 |
| Flexible over JCP | 1 | 13 | 0.005547 |
| Flexible over JCP | 2, 3 | 14 | 0.007607 |
| Flexible over JCP | 6 | 15 | 0.003558 |
| Flexible over JCP | 7 | 16 | 0.005257 |
| Flexible over JCP | 8 | 17 | 0.003640 |
| Flexible over JCP | 9 | 18 | 0.007894 |
| Flexible over JCP | 11 | 19 | 0.010929 |
| Flexible over JCP | 12 | 20 | 0.004071 |
| Flexible over JCP | 14 | 21 | 0.005700 |
| Flexible over JCP | 16 | 22 | 0.007545 |
| Flexible over JCP | 17 | 23 | 0.006164 |
| Flexible over JCP | 18, 19 | 24 | 0.005736 |
| Flexible over CRCP | 1 | 25 | 0.003421 |
| Flexible over CRCP | 11 | 26 | 0.017038 |

Most Recent Plots by Family - “F” (Flexible) - “FCJ” (Flexible over JCP) - “FCC” (Flexible over CRCP)

Python code used to generate rutting performance curves.

# Cell 1
# Import necessary libraries
import pandas as pd
import numpy as np
import matplotlib.pyplot as plt
from scipy.stats import linregress
import json

def read_excel_to_dataset(file_path, sheet_name):
    # Read the Excel file into a pandas DataFrame
    df = pd.read_excel(file_path, sheet_name=sheet_name)

    # Filter the dataset for PAVEMENT_TYPE="F"
    df_filtered = df[df['PAVEMENT_TYPE'] == 'F'].copy()

    # Combine functional classes 18 and 19 into one group called 1819
    df_filtered.loc[df_filtered['FUNC_CLASS'].isin([18, 19]), 'FUNC_CLASS'] = 1819

    # Calculate mean and standard deviation for AVG_RUT
    mean_avg_rut = df_filtered['AVG_RUT'].mean()
    std_avg_rut = df_filtered['AVG_RUT'].std()

    # Exclude outliers (values outside mean plus or minus two standard deviations)
    df_filtered = df_filtered[(df_filtered['AVG_RUT'] >= mean_avg_rut - 2 * std_avg_rut) & 
                              (df_filtered['AVG_RUT'] <= mean_avg_rut + 2 * std_avg_rut)].copy()

    return df_filtered

def create_pivot_table(df):
    # Create a pivot table with average(AVG_RUT) and sum(MILES)
    pivot_table = pd.pivot_table(df, 
                                 values=['AVG_RUT', 'MILES'], 
                                 index=['FUNC_CLASS'], 
                                 columns=['AGE'], 
                                 aggfunc={'AVG_RUT': 'mean', 'MILES': 'sum'},
                                 margins=True,  # Add margins to include total for each row and column
                                 margins_name='Total')

    return pivot_table

# Example usage
file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_data_export.xlsx'
sheet_name = 'rutting_data'
df_filtered = read_excel_to_dataset(file_path, sheet_name)
pivot_table = create_pivot_table(df_filtered)
print(pivot_table)

# Replace df_pivot with your actual DataFrame
# Assuming df_pivot contains the provided pivot table
df_pivot = pivot_table

# Exclude 'Total' row
df_pivot = df_pivot.loc[df_pivot.index != 'Total']

# List to store slopes for each FUNC_CLASS
slopes = []

# Loop through each row (FUNC_CLASS) in the pivot table
for func_class in df_pivot.index:
    # Extract data for the FUNC_CLASS
    data = df_pivot.loc[func_class, 'AVG_RUT']

    # Remove NaN values
    data = data.dropna()

    # Exclude last three data points in FUNC_CLASS 12
    if func_class == 12:
        data = data.iloc[:-3]

    # Exclude last two data points in FUNC_CLASS 14
    elif func_class == 14:
        data = data.iloc[:-2]

    # Exclude last three data points in FUNC_CLASS 16
    elif func_class == 16:
        data = data.iloc[:-3]

    # Exclude the obvious outlier in FUNC_CLASS 9
    elif func_class == 9:
        if len(data) >= 6:
            # drop the label that is the 5th-from-last in the series' index
            data = data.drop(data.index[-6])       

    # Exclude 'Total' from the index before converting to integers
    age_values = data.index[data.index != 'Total'].astype(int)
    avg_rut_values = data.values[data.index != 'Total']

    # Fit a linear regression model using linregress from scipy.stats
    slope, intercept, r_value, _, _ = linregress(age_values, avg_rut_values)

    # Calculate R2 value
    r2 = r_value**2
    slopes.append(slope)  # Store the slope for later use

    # Set up the matplotlib plot
    plt.figure(figsize=(8, 5))

    # Plot scatter plot
    plt.scatter(age_values, avg_rut_values, label=f'FUNC_CLASS {func_class}', s=50)

    # Plot the trendline
    trendline = slope * age_values + intercept
    plt.plot(age_values, trendline, label=f'Trendline {func_class}', linestyle='dotted')

    # Set y-axis minimum value to 0
    plt.ylim(bottom=0)

    # Set x-axis increment by 1
    plt.xticks(np.arange(min(age_values), max(age_values)+1, 1))

    # Set plot labels and title
    plt.xlabel('AGE')
    plt.ylabel('AVG_RUT')
    plt.title(f'Scatter Plot for Avg. Rut vs. Age (Functional Class {func_class}) (Pavement Type F)')

    # Display the trendline equation and R2 value
    equation = f'y = {slope:.4f}x + {intercept:.4f}'
    r2_text = f'R2 = {r2:.4f}'
    plt.text(0.5, 0.9, equation, transform=plt.gca().transAxes, fontsize=10, ha='center')
    plt.text(0.5, 0.85, r2_text, transform=plt.gca().transAxes, fontsize=10, ha='center')

    # Show legend
    plt.legend()

    # Show the plot
    plt.show()

    # Print the slope for the FUNC_CLASS
    #print(f'Slope for FUNC_CLASS {func_class}: {slope}')

# Print the slopes for each FUNC_CLASS
for func_class, slope in zip(df_pivot.index, slopes):
    print(f'Slope for FUNC_CLASS {func_class}: {slope}')
    
# Create a dictionary to store the data
output_data = {}

# Add pavement type "F" to the dictionary
output_data['F'] = {}

# Add functional class and slopes under pavement type "F"
for func_class, slope in zip(df_pivot.index, slopes):
    output_data['F'][f'FUNC_CLASS_{func_class}'] = slope

# Specify the output JSON file path
json_file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_family_slopes.json'

# Write the dictionary to the JSON file
with open(json_file_path, 'w') as json_file:
    json.dump(output_data, json_file)

# Print a message indicating successful writing
print(f'Data has been written to {json_file_path}')    
# End of Cell 1


# Cell 2
# Pavement type FCJ - review and update filters in each FC group plot (WIP)

def read_excel_to_dataset(file_path, sheet_name):
    # Read the Excel file into a pandas DataFrame
    df = pd.read_excel(file_path, sheet_name=sheet_name)

    # Filter the dataset for PAVEMENT_TYPE="FCJ"
    df_filtered = df[df['PAVEMENT_TYPE'] == 'FCJ'].copy()

    # Combine functional classes 18 and 19 into one group called 1819
    df_filtered.loc[df_filtered['FUNC_CLASS'].isin([2, 3]), 'FUNC_CLASS'] = 23    
    
    # Combine functional classes 18 and 19 into one group called 1819
    df_filtered.loc[df_filtered['FUNC_CLASS'].isin([18, 19]), 'FUNC_CLASS'] = 1819

    # Calculate mean and standard deviation for AVG_RUT
    mean_avg_rut = df_filtered['AVG_RUT'].mean()
    std_avg_rut = df_filtered['AVG_RUT'].std()

    # Exclude outliers (values outside mean plus or minus two standard deviations)
    df_filtered = df_filtered[(df_filtered['AVG_RUT'] >= mean_avg_rut - 2 * std_avg_rut) & 
                              (df_filtered['AVG_RUT'] <= mean_avg_rut + 2 * std_avg_rut)].copy()

    return df_filtered

def create_pivot_table(df):
    # Create a pivot table with average(AVG_RUT) and sum(MILES)
    pivot_table = pd.pivot_table(df, 
                                 values=['AVG_RUT', 'MILES'], 
                                 index=['FUNC_CLASS'], 
                                 columns=['AGE'], 
                                 aggfunc={'AVG_RUT': 'mean', 'MILES': 'sum'},
                                 margins=True,  # Add margins to include total for each row and column
                                 margins_name='Total')

    return pivot_table

# Example usage
file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_data_export.xlsx'
sheet_name = 'rutting_data'  # replace with the actual sheet name
df_filtered = read_excel_to_dataset(file_path, sheet_name)
pivot_table = create_pivot_table(df_filtered)
print(pivot_table)

# Replace df_pivot with your actual DataFrame
# Assuming df_pivot contains the provided pivot table
df_pivot = pivot_table

# Exclude 'Total' row
df_pivot = df_pivot.loc[df_pivot.index != 'Total']

# List to store slopes for each FUNC_CLASS
slopes = []

# Loop through each row (FUNC_CLASS) in the pivot table
for func_class in df_pivot.index:
    # Extract data for the FUNC_CLASS
    data = df_pivot.loc[func_class, 'AVG_RUT']

    # Remove NaN values
    data = data.dropna()

    # Exclude last two data points in FUNC_CLASS 12
    if func_class == 12:
        data = data.iloc[:-2]

    # Exclude last two data points in FUNC_CLASS 14
    elif func_class == 14:
        data = data.iloc[:-2]

    # Exclude 'Total' from the index before converting to integers
    age_values = data.index[data.index != 'Total'].astype(int)
    avg_rut_values = data.values[data.index != 'Total']

    # Fit a linear regression model using linregress from scipy.stats
    slope, intercept, r_value, _, _ = linregress(age_values, avg_rut_values)

    # Calculate R2 value
    r2 = r_value**2
    slopes.append(slope)  # Store the slope for later use

    # Set up the matplotlib plot
    plt.figure(figsize=(8, 5))

    # Plot scatter plot
    plt.scatter(age_values, avg_rut_values, label=f'FUNC_CLASS {func_class}', s=50)

    # Plot the trendline
    trendline = slope * age_values + intercept
    plt.plot(age_values, trendline, label=f'Trendline {func_class}', linestyle='dotted')

    # Set y-axis minimum value to 0
    plt.ylim(bottom=0)

    # Set x-axis increment by 1
    plt.xticks(np.arange(min(age_values), max(age_values)+1, 1))

    # Set plot labels and title
    plt.xlabel('AGE')
    plt.ylabel('AVG_RUT')
    plt.title(f'Scatter Plot for Avg. Rut vs. Age (Functional Class {func_class}) (Pavement Type FCJ)')

    # Display the trendline equation and R2 value
    equation = f'y = {slope:.4f}x + {intercept:.4f}'
    r2_text = f'R2 = {r2:.4f}'
    plt.text(0.5, 0.9, equation, transform=plt.gca().transAxes, fontsize=10, ha='center')
    plt.text(0.5, 0.85, r2_text, transform=plt.gca().transAxes, fontsize=10, ha='center')

    # Show legend
    plt.legend()

    # Show the plot
    plt.show()

    # Print the slope for the FUNC_CLASS
    #print(f'Slope for FUNC_CLASS {func_class}: {slope}')

# Print the slopes for each FUNC_CLASS
for func_class, slope in zip(df_pivot.index, slopes):
    print(f'Slope for FUNC_CLASS {func_class}: {slope}')
    
# Function to append data to existing JSON file
def append_to_json(file_path, pavement_type, func_class_slopes):
    # Load existing data from the JSON file
    try:
        with open(file_path, 'r') as json_file:
            existing_data = json.load(json_file)
    except FileNotFoundError:
        existing_data = {}

    # Add or update data for the specified pavement type
    existing_data[pavement_type] = func_class_slopes

    # Write the updated data to the JSON file
    with open(file_path, 'w') as json_file:
        json.dump(existing_data, json_file)

# Define pavement type "FCJ" and its functional class and slope values
pavement_type_fcj = 'FCJ'
func_class_slopes_fcj = {}

# Loop through each row (FUNC_CLASS) in the pivot table
for func_class, slope in zip(df_pivot.index, slopes):
    func_class_slopes_fcj[f'FUNC_CLASS_{func_class}'] = slope

# Specify the output JSON file path
json_file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_family_slopes.json'

# Append data for pavement type "FCJ" to the existing JSON file
append_to_json(json_file_path, pavement_type_fcj, func_class_slopes_fcj)

# Print a message indicating successful appending
print(f'Data for pavement type {pavement_type_fcj} has been appended to {json_file_path}') 
# End of Cell 2


# Cell 3
# Pavement type FCC - review and update filters in each FC group plot (WIP)

def read_excel_to_dataset(file_path, sheet_name):
    # Read the Excel file into a pandas DataFrame
    df = pd.read_excel(file_path, sheet_name=sheet_name)

    # Filter the dataset for PAVEMENT_TYPE="FCC"
    df_filtered = df[df['PAVEMENT_TYPE'] == 'FCC'].copy()

    # Calculate mean and standard deviation for AVG_RUT
    mean_avg_rut = df_filtered['AVG_RUT'].mean()
    std_avg_rut = df_filtered['AVG_RUT'].std()

    # Exclude outliers (values outside mean plus or minus two standard deviations)
    df_filtered = df_filtered[(df_filtered['AVG_RUT'] >= mean_avg_rut - 2 * std_avg_rut) & 
                              (df_filtered['AVG_RUT'] <= mean_avg_rut + 2 * std_avg_rut)].copy()

    return df_filtered

def create_pivot_table(df):
    # Create a pivot table with average(AVG_RUT) and sum(MILES)
    pivot_table = pd.pivot_table(df, 
                                 values=['AVG_RUT', 'MILES'], 
                                 index=['FUNC_CLASS'], 
                                 columns=['AGE'], 
                                 aggfunc={'AVG_RUT': 'mean', 'MILES': 'sum'},
                                 margins=True,  # Add margins to include total for each row and column
                                 margins_name='Total')

    return pivot_table

# Example usage
file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_data_export.xlsx'
sheet_name = 'rutting_data'  # replace with the actual sheet name
df_filtered = read_excel_to_dataset(file_path, sheet_name)
pivot_table = create_pivot_table(df_filtered)
print(pivot_table)

# Replace df_pivot with your actual DataFrame
# Assuming df_pivot contains the provided pivot table
df_pivot = pivot_table

# Exclude 'Total' row
df_pivot = df_pivot.loc[df_pivot.index != 'Total']

# List to store slopes for each FUNC_CLASS
slopes = []

# Loop through each row (FUNC_CLASS) in the pivot table
for func_class in df_pivot.index:
    # Extract data for the FUNC_CLASS
    data = df_pivot.loc[func_class, 'AVG_RUT']

    # Remove NaN values
    data = data.dropna()

    # Exclude last seven data points for FUNC_CLASS 1
    #if func_class == 1:
    #    data = data.iloc[:-8]

    # Exclude last five data points for FUNC_CLASS 11
    #elif func_class == 14:
    #    data = data.iloc[:-6]

    # Exclude 'Total' from the index before converting to integers
    age_values = data.index[data.index != 'Total'].astype(int)
    avg_rut_values = data.values[data.index != 'Total']

    # Fit a linear regression model using linregress from scipy.stats
    slope, intercept, r_value, _, _ = linregress(age_values, avg_rut_values)

    # Calculate R2 value
    r2 = r_value**2
    slopes.append(slope)  # Store the slope for later use

    # Set up the matplotlib plot
    plt.figure(figsize=(8, 5))

    # Plot scatter plot
    plt.scatter(age_values, avg_rut_values, label=f'FUNC_CLASS {func_class}', s=50)

    # Plot the trendline
    trendline = slope * age_values + intercept
    plt.plot(age_values, trendline, label=f'Trendline {func_class}', linestyle='dotted')

    # Set x and y-axis minimum value to 0
    plt.ylim(bottom=0)
    plt.xlim(left=0)    

    # Set x-axis increment by 1
    plt.xticks(np.arange(min(age_values), max(age_values)+1, 1))

    # Set plot labels and title
    plt.xlabel('AGE')
    plt.ylabel('AVG_RUT')
    plt.title(f'Scatter Plot for Avg. Rut vs. Age (Functional Class {func_class}) (Pavement Type FCC)')

    # Display the trendline equation and R2 value
    equation = f'y = {slope:.4f}x + {intercept:.4f}'
    r2_text = f'R2 = {r2:.4f}'
    plt.text(0.5, 0.9, equation, transform=plt.gca().transAxes, fontsize=10, ha='center')
    plt.text(0.5, 0.85, r2_text, transform=plt.gca().transAxes, fontsize=10, ha='center')

    # Show legend
    plt.legend()

    # Show the plot
    plt.show()

    # Print the slope for the FUNC_CLASS
    #print(f'Slope for FUNC_CLASS {func_class}: {slope}')

# Print the slopes for each FUNC_CLASS
for func_class, slope in zip(df_pivot.index, slopes):
    print(f'Slope for FUNC_CLASS {func_class}: {slope}')
    
# Function to append data to existing JSON file
def append_to_json(file_path, pavement_type, func_class_slopes):
    # Load existing data from the JSON file
    try:
        with open(file_path, 'r') as json_file:
            existing_data = json.load(json_file)
    except FileNotFoundError:
        existing_data = {}

    # Add or update data for the specified pavement type
    existing_data[pavement_type] = func_class_slopes

    # Write the updated data to the JSON file
    with open(file_path, 'w') as json_file:
        json.dump(existing_data, json_file)

# Define pavement type "FCC" and its functional class and slope values
pavement_type_fcj = 'FCC'
func_class_slopes_fcj = {}

# Loop through each row (FUNC_CLASS) in the pivot table
for func_class, slope in zip(df_pivot.index, slopes):
    func_class_slopes_fcj[f'FUNC_CLASS_{func_class}'] = slope

# Specify the output JSON file path
json_file_path = r'Z:\Aditya\Optimization Process Updates 2024\Rutting Models\rutting_family_slopes.json'

# Append data for pavement type "FCC" to the existing JSON file
append_to_json(json_file_path, pavement_type_fcj, func_class_slopes_fcj)

# Print a message indicating successful appending
print(f'Data for pavement type {pavement_type_fcj} has been appended to {json_file_path}')     
# End of Cell 3

Most Recent Model Functions

double age = [AGE], SL, avg_rut = [AVG_RUT];

string FC = [FUNC_CLASS], PT = [PAVEMENT_TYPE];

if (PT == "F" || PT == "FLEXIBLE")
{
    if (FC == "1") { SL = 0.009734; }
    else if (FC == "6") { SL = 0.007501; }
    else if (FC == "7") { SL = 0.006067; }
    else if (FC == "8") { SL = 0.005461; }
    else if (FC == "9") { SL = 0.005617; }
    else if (FC == "11") { SL = 0.006402; }
    else if (FC == "12") { SL = 0.007211; }
    else if (FC == "14") { SL = 0.009364; }
    else if (FC == "16") { SL = 0.008818; }
    else if (FC == "17") { SL = 0.006681; }
    else if (FC == "2" || FC == "3") { SL = 0.006751; }
    else if (FC == "18" || FC == "19") { SL = 0.012963; }
}
else if (PT == "FCJ" || PT == "COMPOSITE")
{
    if (FC == "1") { SL = 0.005547; }
    else if (FC == "6") { SL = 0.003558; }
    else if (FC == "7") { SL = 0.005257; }
    else if (FC == "8") { SL = 0.00364; }
    else if (FC == "9") { SL = 0.007894; }
    else if (FC == "11") { SL = 0.010929; }
    else if (FC == "12") { SL = 0.004071; }
    else if (FC == "14") { SL = 0.0057; }
    else if (FC == "16") { SL = 0.007545; }
    else if (FC == "17") { SL = 0.006164; }
    else if (FC == "2" || FC == "3") { SL = 0.007607; }
    else if (FC == "18" || FC == "19") { SL = 0.005736; }
}
else if (PT == "FCC")
{
    if (FC == "1") { SL = 0.003421; }
    else if (FC == "11") { SL = 0.017038; }
}

double Answer = SL * age + avg_rut;

return Answer;

## Friction Performance Model

The following tables and scripts provide current information for friction model parameters.

Friction Families by County and Functional Class

| County | Functional Class | Family | Slope |
| --- | --- | --- | --- |
| CA, QA | 1, 2, 11 | 1A | -0.6550 |
| CA, QA | 9, 18, 19 | 1B | -0.5036 |
| CA, QA | 7, 12, 17 | 1C | -0.3022 |
| CA, QA | 3, 6, 8, 14, 16 | 1D | -0.3906 |
| CH, HO, WA | 1, 2, 11 | 2A | -0.6635 |
| CH, HO, WA | 9, 18, 19 | 2B | -0.4890 |
| CH, HO, WA | 7, 12, 17 | 2C | -0.2492 |
| CH, HO, WA | 3, 6, 8, 14, 16 | 2D | -0.2564 |
| BA | 1, 2, 11 | 3A | -0.4548 |
| BA | 7, 12, 17 | 3C | -0.3622 |
| BA | 3, 6, 8, 14, 16 | 3D | -0.2978 |
| MO, AA | 1, 2, 11 | 4A | -0.5653 |
| MO, AA | 9, 18, 19 | 4B | -0.5048 |
| MO, AA | 7, 12, 17 | 4C | -0.2890 |
| MO, AA | 3, 6, 8, 14, 16 | 4D | -0.3314 |
| SM, FR, CL | 1, 2, 11 | 5A | -0.5805 |
| SM, FR, CL | 9, 18, 19 | 5B | -0.2526 |
| SM, FR, CL | 7, 12, 17 | 5C | -0.2141 |
| SM, FR, CL | 3, 6, 8, 14, 16 | 5D | -0.2038 |
| PG, AL | 1, 2, 11 | 6A | -0.3108 |
| PG, AL | 9, 18, 19 | 6B | -0.6394 |
| PG, AL | 7, 12, 17 | 6C | -0.5430 |
| PG, AL | 3, 6, 8, 14, 16 | 6D | -0.2319 |
| CE, KE, TA, CO, WI | 1, 2, 11 | 7A | -0.2021 |
| CE, KE, TA, CO, WI | 9, 18, 19 | 7B | -0.2807 |
| CE, KE, TA, CO, WI | 7, 12, 17 | 7C | -0.3189 |
| CE, KE, TA, CO, WI | 3, 6, 8, 14, 16 | 7D | -0.3444 |
| SO, DO | 1, 2, 11 | 8A | -0.3635 |
| SO, DO | 9, 18, 19 | 8B | -0.2526 |
| SO, DO | 7, 12, 17 | 8C | -0.2457 |
| SO, DO | 3, 6, 8, 14, 16 | 8D | -0.1868 |
| HA, WO | 1, 2, 11 | 9A | -0.3064 |
| HA, WO | 9, 18, 19 | 9B | -0.2730 |
| HA, WO | 7, 12, 17 | 9C | -0.3906 |
| HA, WO | 3, 6, 8, 14, 16 | 9D | -0.2420 |
| GA | 1, 2, 11 | 10A | -0.4593 |
| GA | 7, 12, 17 | 10C | -0.7543 |
| GA | 3, 6, 8, 14, 16 | 10D | -0.4686 |

Most Recent Plots by Family

Titles are Family Names

X-axis is age in years

Y-axis is friction value

Most Recent Model Functions

// C# Function for friction models updated 12-15-2017

string CTY = [COUNTY], FC = [FUNC_CLASS];
double SL = 0, SKID = [SPADJ_SKID_NUMBER], age = [AGE];

if ((CTY == "CA" || CTY == "QA"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.655;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.5036;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.3022;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.3906;
}
else if ((CTY == "CH" || CTY == "HO" || CTY == "WA"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.6635;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.489;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.2492;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.2564;
}
else if (CTY == "BA")
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.4548;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.3622;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.2978;
}
else if ((CTY == "MO" || CTY == "AA"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.5653;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.5048;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.289;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.3314;
}
else if ((CTY == "SM" || CTY == "FR" || CTY == "CL"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.5805;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.2526;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.2141;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.2038;
}
else if ((CTY == "PG" || CTY == "AL"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.3108;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.6394;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.543;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.2319;
}
else if ((CTY == "CE" || CTY == "KE" || CTY == "TA" || CTY == "CO" || CTY == "WI"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.2021;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.2807;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.3189;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.3444;
}
else if ((CTY == "SO" || CTY == "DO"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.3635;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.2526;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.2457;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.1868;
}
else if ((CTY == "HA" || CTY == "WO"))
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.3064;
    else if (FC == "9" || FC == "18" || FC == "19") SL = -0.273;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.3906;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.242;
}
else if (CTY == "GA")
{
    if (FC == "1" || FC == "2" || FC == "11") SL = -0.4593;
    else if (FC == "7" || FC == "12" || FC == "17") SL = -0.7543;
    else if (FC == "3" || FC == "6" || FC == "8" || FC == "16" || FC == "14") SL = -0.4686;
}
else
{
    SL = -0.3906;
}

Answer = SL * age + SKID;

return Answer;

## Optimization Data Rollup Methods

The following table and supporting information relate to the data rollup process in RoadCare.

Rollup Methods by Attribute

To accomplish the stated objective, use the following data rollup methods for each specified attribute using RoadCare:

| Attribute | Avg | First | Last | Max | None | Predominant | Std. Dev | Sum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AADT | * |  |  |  |  |  |  |  |
| AGE |  |  |  |  |  | * |  |  |
| AGE_GROUP_HALFMILE |  |  |  |  |  | * |  |  |
| AGE_GROUPS |  |  |  |  |  | * |  |  |
| AVG_IRI | * |  |  |  |  |  |  |  |
| AVG_IRI_QC | * |  |  |  |  |  |  |  |
| AVG_REHAB_CYCLE |  |  |  |  | * |  |  |  |
| AVG_RUT | * |  |  |  |  |  |  |  |
| AVG_RUT_QC | * |  |  |  |  |  |  |  |
| AVG_RUT_SDV |  |  |  |  |  |  | * |  |
| BEGIN_LAT_LONG |  | * |  |  |  |  |  |  |
| BEGIN_LIMIT |  | * |  |  |  |  |  |  |
| BEGIN_MM |  | * |  |  |  |  |  |  |
| BRIDGE |  |  |  |  |  | * |  |  |
| BRIDGE_LONG |  |  |  |  |  | * |  |  |
| CI | * |  |  |  |  |  |  |  |
| CI_FAMILY_CODE |  |  |  |  | * |  |  |  |
| CI_FC_CATEGORY |  |  |  |  |  | * |  |  |
| CI_FUNCTIONAL | * |  |  |  |  |  |  |  |
| CI_MATERIAL_TYPE |  |  |  |  |  | * |  |  |
| CI_STRUCTURAL | * |  |  |  |  |  |  |  |
| CODE |  |  |  |  |  | * |  |  |
| CONDITION_IRI |  |  |  |  |  | * |  |  |
| COUNTY |  |  |  |  |  | * |  |  |
| CRACK_SEAL_EXISTS |  |  |  |  |  | * |  |  |
| CRACKING_SEG_GROUP |  |  |  |  |  | * |  |  |
| CURB |  |  |  |  |  | * |  |  |
| CY17_PMT_PROJECTS |  |  |  |  | * |  |  |  |
| D3_CS_CANDIDATES |  |  |  |  | * |  |  |  |
| D3_EXCLUSIONS |  |  |  |  |  | * |  |  |
| D3_MICRO_FY18 |  |  |  |  | * |  |  |  |
| DENSITY_AVG |  |  |  |  | * |  |  |  |
| DIR_BMP |  | * |  |  |  |  |  |  |
| DIR_EMP |  |  | * |  |  |  |  |  |
| DIR_LANES | * |  |  |  |  |  |  |  |
| DIR_MILES |  |  |  |  |  |  |  | * |
| DISTRICT |  |  |  |  |  | * |  |  |
| DIVIDED |  |  |  |  |  | * |  |  |
| END_LAT_LONG |  |  | * |  |  |  |  |  |
| END_LIMIT |  |  | * |  |  |  |  |  |
| END_MM |  |  | * |  |  |  |  |  |
| EVENT_PAVE_CHANGE |  |  |  |  |  | * |  |  |
| EXIT_NUMBER |  |  |  |  |  | * |  |  |
| FC_CATEGORY |  |  |  |  |  | * |  |  |
| FC_DENSITY | * |  |  |  |  |  |  |  |
| FC_DENSITY_MR | * |  |  |  |  |  |  |  |
| FC_DENSITY_QC | * |  |  |  |  |  |  |  |
| FC_DENSITY_SDV |  |  |  |  |  |  | * |  |
| FRICTION_RUT_13 |  |  |  |  | * |  |  |  |
| FRICTION_RUT_14 |  |  |  |  | * |  |  |  |
| FUNC_CLASS |  |  |  |  |  | * |  |  |
| FUND_RTE_LIST_LMR |  |  |  |  |  | * |  |  |
| FY16_17_SUG_PRJ |  |  |  |  |  | * |  |  |
| GLOBAL_ROUTE_ID |  |  |  |  |  | * |  |  |
| GOVT_CONTROL |  |  |  |  |  | * |  |  |
| HMIS_ROUTEID |  |  |  |  |  | * |  |  |
| HPMS_AVG_IRI |  |  |  |  | * |  |  |  |
| HPMS_AVG_RUT |  |  |  |  | * |  |  |  |
| HPMS_CRK_PCT_ASP |  |  |  |  | * |  |  |  |
| HPMS_CRK_PCT_CRC |  |  |  |  | * |  |  |  |
| HPMS_CRK_PCT_JCP |  |  |  |  | * |  |  |  |
| HPMS_FAULTING |  |  |  |  | * |  |  |  |
| HPMS_SEGMENTATION |  |  |  |  |  | * |  |  |
| HPMS_SURFACE_TYPE |  |  |  |  | * |  |  |  |
| INITIAL_SKID_NUMBER |  |  |  |  | * |  |  |  |
| INV_CHANGE |  |  |  |  | * |  |  |  |
| IRI_PER_SCALE |  |  |  |  | * |  |  |  |
| IRI_PHASE4_SEG_GAPS |  |  |  |  | * |  |  |  |
| ISCONHIST |  |  |  |  |  | * |  |  |
| ISRAMP |  |  |  |  |  | * |  |  |
| LANE_WIDTH |  |  |  |  | * |  |  |  |
| LAST_MAINT_TREATMENT |  |  |  |  |  | * |  |  |
| LAST_MAINT_YEAR |  |  |  |  |  | * |  |  |
| LAST_PMAINT_TREATMENT |  |  |  |  |  | * |  |  |
| LAST_PMAINT_YEAR |  |  |  |  |  | * |  |  |
| LAST_REHAB_TREATMENT |  |  |  |  |  | * |  |  |
| LAST_REHAB_YEAR |  |  |  |  |  | * |  |  |
| LAST_TREATMENT |  |  |  |  |  | * |  |  |
| LAST_YEAR |  |  |  |  |  | * |  |  |
| LM_DM_VMT_AREA_16 |  |  |  |  | * |  |  |  |
| LM_DM_VMT_AREA_17 |  |  |  |  | * |  |  |  |
| LM_DM_VMT_AREA_PM15 |  |  |  |  | * |  |  |  |
| LM_VMT_AREA | * |  |  |  |  |  |  |  |
| LM_VMT_AREA_15 |  |  |  |  | * |  |  |  |
| MAIN_LINE |  |  |  |  |  | * |  |  |
| MAPPING_AGE |  |  |  |  | * |  |  |  |
| MAPPING_LAST_YEAR |  |  |  |  | * |  |  |  |
| MDTA_FACILITY |  |  |  |  | * |  |  |  |
| MDTA_SEG |  |  |  |  | * |  |  |  |
| NHS_CODE |  |  |  |  |  | * |  |  |
| ONE_MILE_OR_MORE |  |  |  |  |  | * |  |  |
| PAVED_AREA_SY |  |  |  |  |  |  |  | * |
| PAVEMENT_TYPE |  |  |  |  |  | * |  |  |
| PCC_SEGMENTS |  |  |  |  |  | * |  |  |
| PRIMARY_DIRECTION |  |  |  |  |  | * |  |  |
| PROGRAM |  |  |  |  | * |  |  |  |
| PROGRAM_COST |  |  |  |  | * |  |  |  |
| PROGRAM_COST_FY12 |  |  |  |  | * |  |  |  |
| PROGRAM_FY12 |  |  |  |  | * |  |  |  |
| PROGRAM_FY13 |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR_FY12 |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR_FY13 |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR_FY14 |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR_FY15 |  |  |  |  | * |  |  |  |
| PROGRAM_YEAR_FY16 |  |  |  |  |  | * |  |  |
| PROGRAM_YEAR_FY17 |  |  |  |  |  | * |  |  |
| PROGRAM_YEAR_MDTA |  |  |  |  |  | * |  |  |
| PROGYEAR13_EXTRA |  |  |  |  | * |  |  |  |
| PROPOSED |  |  |  |  |  | * |  |  |
| PUBLIC_MAP_15 |  |  |  |  |  | * |  |  |
| PUBLIC_MAP_15_BEGLMT |  |  |  |  |  | * |  |  |
| PUBLIC_MAP_15_ENDLMT |  |  |  |  |  | * |  |  |
| R_IRI | * |  |  |  |  |  |  |  |
| RAMP_NUMBER |  |  |  |  |  | * |  |  |
| REGION |  |  |  |  |  | * |  |  |
| RESECTION |  |  |  |  | * |  |  |  |
| RESECTION_2013 |  |  |  |  | * |  |  |  |
| RESECTION_2014 |  |  |  |  | * |  |  |  |
| RESECTION_2015 |  |  |  |  | * |  |  |  |
| RESECTION_FY16 |  |  |  |  | * |  |  |  |
| RESECTION_FY17 |  |  |  |  |  | * |  |  |
| RESECTION_OCT2014 |  |  |  |  | * |  |  |  |
| RESECTION_OPT2015 |  |  |  |  | * |  |  |  |
| RESECTION_PUBLIC_MAP_15 |  |  |  |  |  | * |  |  |
| RIGHTIRI_QC | * |  |  |  |  |  |  |  |
| RNUM |  |  |  |  |  | * |  |  |
| ROAD_CLASS |  |  |  |  |  | * |  |  |
| ROAD_NAME |  |  |  |  |  | * |  |  |
| ROUTE |  |  |  |  |  | * |  |  |
| ROUTEID |  |  |  |  |  | * |  |  |
| RSL_WORST_SUM | * |  |  |  |  |  |  |  |
| RSUFF |  |  |  |  |  | * |  |  |
| RUT_FY17 |  |  |  |  | * |  |  |  |
| RUT_SEVERITY |  |  | * |  |  |  |  |  |
| SC_DENSITY | * |  |  |  |  |  |  |  |
| SC_DENSITY_MR | * |  |  |  |  |  |  |  |
| SC_DENSITY_QC | * |  |  |  |  |  |  |  |
| SC_DENSITY_SDV |  |  |  |  |  |  | * |  |
| SECTIONS_WO_AGE |  |  |  |  | * |  |  |  |
| SHOP |  |  |  |  |  | * |  |  |
| SHOULDER_WIDTH |  |  |  |  | * |  |  |  |
| SIMDIV |  |  |  |  |  | * |  |  |
| SKID_CATEGORY |  |  |  |  | * |  |  |  |
| SKID_CATEGORY_WAL |  |  |  |  | * |  |  |  |
| SKID_FY17 |  |  |  |  | * |  |  |  |
| SKID_RESECTION_WAL |  |  |  |  | * |  |  |  |
| SKID_RUT_FY16 |  |  |  |  | * |  |  |  |
| SKID_RUT_FY17 |  |  |  |  | * |  |  |  |
| SKID_RUT_LIST |  |  |  |  | * |  |  |  |
| SPADJ_SKID_CATEGORY |  |  |  |  |  | * |  |  |
| SPADJ_SKID_NUMBER | * |  |  |  |  |  |  |  |
| SPADJ_SKID_NUMBER_QC | * |  |  |  |  |  |  |  |
| SPEED_ARAN | * |  |  |  |  |  |  |  |
| SPEED_LIMIT |  |  |  |  |  | * |  |  |
| SPEED_SKID | * |  |  |  |  |  |  |  |
| SUB_ROUTE_ID | * |  |  |  |  |  |  |  |
| SURFACE_TYPE |  |  |  |  |  | * |  |  |
| SURVEY_SPEED |  |  |  |  | * |  |  |  |
| TEST_ATTRIBUTE |  |  |  |  | * |  |  |  |
| TOLL_PLAZA |  |  |  |  | * |  |  |  |
| TOTAL_LANES | * |  |  |  |  |  |  |  |
| TREATMENT_LEVEL |  |  |  |  |  | * |  |  |
| UNIT_COST_PROJECTS_15 |  |  |  |  | * |  |  |  |
| UNIT_COST_PROJECTS_16 |  |  |  |  | * |  |  |  |
| USE_AS_PRIMARY |  |  |  |  |  | * |  |  |
| WET_ACCIDENT_LIST |  |  |  |  | * |  |  |  |
| WET_ACCIDENT_LIST_15 |  |  |  |  | * |  |  |  |
| WET_ACCIDENT_LIST_16 |  |  |  |  | * |  |  |  |

Network Segmentation Implementation Details

To accomplish the stated objective, perform segmentation on the newly created network using the following segmentation logic using RoadCare:

- split: any change GOVT_CONTROL

- split: any record PROGRAM_YEAR_FY18 - committed projects from PMT, non Fund77 projects, and construction history.

- split: any record PCC (concrete)

- split: any record long bridge (bridge >= 0.25 mile)

- split: any record EVENT_PAVE_CHANGE (source vision base report)

In “Segmentation Results,” set minimum = 0.5, set maximum = 6, increments = Exact. Click on “Apply to all sections” button. This will set the minimum and maximum section length to 0.5 miles and 6 miles, respectively, unless they are split due to the five reasons listed above.

Once done, rollup network and then create an “Attribute View Report” by selecting the following attributes: BRIDGE_LONG, EVENT_PAVE_CHANGE, CRACKING_SEG_GROUP, FC_DENSITY, SC_DENSITY, GOVT_CONTROL, PCC_SEGMENTS, PROGRAM_YEAR_FY17, AGE_GROUPS, SIMDIV, USE_AS_PRIMARY, and BEGIN_LIMIT.

Next, create a spreadsheet template for segmentation using the resulting “attribute view report.’

CRACKING_SEG_GROUP is a formula field in the spreadsheet. It is defined as

CASE
 WHEN SC_DENSITY<5 and FC_DENSITY<4 THEN 1
 WHEN SC_DENSITY<5 and FC_DENSITY>=4 AND FC_DENSITY<10 THEN 2
 WHEN SC_DENSITY<5 and FC_DENSITY>=10 THEN 3
 WHEN SC_DENSITY>=5 AND SC_DENSITY<25 and FC_DENSITY<10 THEN 4
 WHEN SC_DENSITY>=5 AND SC_DENSITY<25 and FC_DENSITY>=10 THEN 5
 WHEN SC_DENSITY>=25 THEN 6
 END CRACKING_SEG_GROUP

If there is a pavement change recorded for any part of the route (i.e. count EVENT_PAVE_CHANGE >0), then make the following changes in the spreadsheet:

- join adjacent sections that belong to the same cracking group, and

- if there is no cracking data available, then just use the “event_pave_change” field for segmentation.

If there is no recorded pavement change for any part of the route (i.e. count EVENT_PAVE_CHANGE =0) then make the following change in the spreadsheet:

- join adjacent sections that belong to the same cracking group, or

- else split the section where the difference in pavement age is greater than five years.

Next, update committed project limits so that there are no small sections adjacent to the committed project, as illustrated in the example below.

| ROUTE | BMP_ORG | EMP_ORG | BMP_NEW | EMP_NEW | DIR | YEAR | TREATMENT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WO-MD 707E | 0.07 | 0.82 | 0.07 | 0.827 | W | 2017 | CRACK SEAL |

| ROUTE | SECTION | Original segmentation | Updated segmentation |
| --- | --- | --- | --- |
| WO-MD 707E | 0.07-0.1(W) | Route Begin - committed project | Route Begin - committed project |
| WO-MD 707E | 0.1-0.82(W) | join: with previous section - committed project | join: with previous section - committed project |

Once done, review the remaining long/short sections and make changes as needed. Next, copy and paste these sections into a new attribute called “RESECTION_FY18” (see Data Import). Lastly, segment the network using the segmentation logic “Anyrecord” RESECTION FY18 and rollup the network.

## Pavement Friction Management Program SQL Packages

The following SQL package is used to break the network at every road feature or event that effects friction demand:

--- PACKAGE SPEC
CREATE OR REPLACE PACKAGE FRICTION_GUIDE_PKG AS
--EXECUTE FRICTION_GUIDE_PKG.FMG_MAIN(2016,2019,NULL);
PROCEDURE P_SKID_INVENTORY (INV_YEAR NUMBER);
PROCEDURE FMG_MAIN (INV_YEAR NUMBER, COND_YEAR NUMBER,COUNTY_IN VARCHAR2);
PROCEDURE P_DH_GRADE (P_YEAR NUMBER);
PROCEDURE P_CURVE_RUT(P_YEAR NUMBER);
PROCEDURE P_FMG_BASE_SEGMENTATION(INV_YEAR NUMBER, COND_YEAR NUMBER,COUNTY_IN VARCHAR2 );
PROCEDURE P_FMG_INTERMED_SEGMENTATION;
PROCEDURE P_FMG_FINAL_SEGMENTATION;
PROCEDURE P_FMG_ATTRIBUTE_UPDATE;
END FRICTION_GUIDE_PKG;
/

---PACKAGE BODY
CREATE OR REPLACE PACKAGE BODY FRICTION_GUIDE_PKG AS
  PROCEDURE P_SKID_INVENTORY(INV_YEAR NUMBER)
  AS
    CURSOR C1 IS
      SELECT HMIS_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, ROUTEID, HMIS_ROUTEID, DIRECTION, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, GOVT_CONTROL
      FROM SECTION_TABLE
      WHERE 1=0;
  TYPE TYP1 IS TABLE OF C1%ROWTYPE;
  TAB1 TYP1;
  V_DIRBMP  NUMBER;
  V_DIREMP  NUMBER;
  V_INVBMP  NUMBER;
  V_INVEMP  NUMBER;
  V_INV_DIR NUMBER;
BEGIN
  EXECUTE IMMEDIATE 'TRUNCATE TABLE SKID_INVENTORY';
  FOR SEC_TABLE IN (SELECT DISTINCT HMIS_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL, ROUTEID, HMIS_ROUTEID, DIRECTION
  FROM SECTION_TABLE
  WHERE HMIS_YEAR = INV_YEAR
  AND GOVT_CONTROL IN (1,31)
  ORDER BY GLOBAL_ROUTE_ID, SUB_ROUTE_ID)
  LOOP
    V_INV_DIR := NULL;
    SELECT HMIS_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, ROUTEID, HMIS_ROUTEID, DIRECTION, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, GOVT_CONTROL BULK COLLECT
    INTO TAB1
    FROM SECTION_TABLE
    WHERE HMIS_YEAR     = SEC_TABLE.HMIS_YEAR
    AND GLOBAL_ROUTE_ID =SEC_TABLE.GLOBAL_ROUTE_ID
    AND DIRECTION       = SEC_TABLE.DIRECTION
    AND HMIS_ROUTEID    =SEC_TABLE.HMIS_ROUTEID
    AND GOVT_CONTROL    = SEC_TABLE.GOVT_CONTROL
    ORDER BY HMIS_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, DIR_BMP, DIR_EMP;
    BEGIN
      SELECT DISTINCT USE_AS_PRIMARY INTO V_INV_DIR
      FROM SECTION_TABLE
      WHERE GLOBAL_ROUTE_ID =SEC_TABLE.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID      = SEC_TABLE.SUB_ROUTE_ID
      AND HMIS_YEAR         =INV_YEAR;
    EXCEPTION
    WHEN NO_DATA_FOUND THEN
      DBMS_OUTPUT.PUT_LINE('NO RECORDS FOUND IN SECTION TABLE FOR '||SEC_TABLE.GLOBAL_ROUTE_ID||' - '||SEC_TABLE.SUB_ROUTE_ID||' - '||SEC_TABLE.DIRECTION);
    END;
    V_DIRBMP     := NULL;
    V_DIREMP     := NULL;
    V_INVBMP     := NULL;
    V_INVEMP     := NULL;
    IF V_INV_DIR IS NOT NULL THEN
      FOR I IN 1..TAB1.COUNT
      LOOP
        IF TAB1.COUNT = 1 THEN
          INSERT INTO SKID_INVENTORY(INV_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, ROUTEID, HMIS_ROUTEID, DIRECTION, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, GOVT_CONTROL)
          VALUES(TAB1(I).HMIS_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, TAB1(I).ROUTEID, TAB1(I).HMIS_ROUTEID, TAB1(I).DIRECTION, TAB1(I).DIR_BMP, TAB1(I).DIR_EMP, TAB1(I).INV_BMP, TAB1(I).INV_EMP, TAB1(I).GOVT_CONTROL);
        ELSIF TAB1.COUNT > 1 THEN
          IF I           = 1 THEN
            V_DIRBMP    := TAB1(I).DIR_BMP;
            V_DIREMP    := TAB1(I).DIR_EMP;
            V_INVBMP    := TAB1(I).INV_BMP;
            V_INVEMP    := TAB1(I).INV_EMP;
          ELSIF I               >1 THEN
            IF TAB1(I).DIR_BMP <= TAB1(I-1).DIR_EMP THEN
              V_DIREMP         := TAB1(I).DIR_EMP;
              IF V_INV_DIR      =1 THEN
                V_INVEMP       := TAB1(I).INV_EMP;
              ELSE
                V_INVBMP := TAB1(I).INV_BMP;
              END IF;
            ELSIF TAB1(I).DIR_BMP > TAB1(I-1).DIR_EMP THEN
              INSERT INTO SKID_INVENTORY(INV_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, ROUTEID, HMIS_ROUTEID, DIRECTION, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, GOVT_CONTROL)
              VALUES(TAB1(I).HMIS_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, TAB1(I).ROUTEID, TAB1(I).HMIS_ROUTEID, TAB1(I).DIRECTION, V_DIRBMP, V_DIREMP, V_INVBMP, V_INVEMP, TAB1(I).GOVT_CONTROL);
              V_DIRBMP := TAB1(I).DIR_BMP;
              V_DIREMP := TAB1(I).DIR_EMP;
              V_INVEMP := TAB1(I).INV_EMP;
              V_INVBMP := TAB1(I).INV_BMP;
            END IF;
          END IF;
          IF I =TAB1.COUNT THEN
            INSERT INTO SKID_INVENTORY(INV_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, ROUTEID, HMIS_ROUTEID, DIRECTION, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, GOVT_CONTROL)
            VALUES(TAB1(I).HMIS_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, TAB1(I).ROUTEID, TAB1(I).HMIS_ROUTEID, TAB1(I).DIRECTION, V_DIRBMP, V_DIREMP, V_INVBMP, V_INVEMP, TAB1(I).GOVT_CONTROL);
          END IF;
        END IF;
      END LOOP;
      COMMIT;
    END IF;
  END LOOP;
MERGE INTO SKID_INVENTORY A USING
( SELECT DISTINCT LU.DISTRICT, SEC.COUNTY, SEC.MUNICIPALITY, SEC.ROUTE, SEC.RNUM, SEC.RSUFF, SEC.ASSOCIATED_ID_PREFIX, SEC.EXIT_NUMBER, SEC.RAMP_NUMBER, SEC.GLOBAL_ROUTE_ID, SEC.SUB_ROUTE_ID
  FROM SECTION_TABLE SEC, SKID_INVENTORY BUS, PAVCORE.LU_COUNTY LU
  WHERE SEC.HMIS_YEAR     = INV_YEAR
  AND SEC.GLOBAL_ROUTE_ID = BUS.GLOBAL_ROUTE_ID
  AND SEC.SUB_ROUTE_ID    = BUS.SUB_ROUTE_ID
  AND LU.COUNTY_CODE      = SEC.COUNTY
)
B ON (A.GLOBAL_ROUTE_ID = B.GLOBAL_ROUTE_ID AND A.SUB_ROUTE_ID = B.SUB_ROUTE_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.DISTRICT          = B.DISTRICT,
    A.COUNTY              = B.COUNTY,
    A.MUNICIPALITY        = B.MUNICIPALITY,
    A.ROUTE               = B.ROUTE,
    A.RNUM                = B.RNUM,
    A.RSUFF               = B.RSUFF,
    A.ASSOCIATED_ID_PREFIX= B.ASSOCIATED_ID_PREFIX,
    A.EXIT_NUMBER         = B.EXIT_NUMBER,
    A.RAMP_NUMBER         = B.RAMP_NUMBER;
  MERGE INTO SKID_INVENTORY A USING
  (SELECT DISTINCT ROUTEID
  FROM HMIS_UNIVERSE_ALL_YEARS
  WHERE MAIN_LINE                =9
  AND YEAR                       =INV_YEAR
  ) B ON (SUBSTR(A.ROUTEID,1,14) = SUBSTR(B.ROUTEID,1,14))
WHEN MATCHED THEN
  UPDATE SET A.HMIS_ROUTEID = B.ROUTEID WHERE A.SUB_ROUTE_ID =2;
  COMMIT;
  UPDATE SKID_INVENTORY
  SET DIR_BMP=ROUND(DIR_BMP,2),
    INV_BMP  = ROUND(INV_BMP,2);
END P_SKID_INVENTORY;

PROCEDURE FMG_MAIN(INV_YEAR  NUMBER, COND_YEAR NUMBER, COUNTY_IN VARCHAR2)
AS
  INDEX_CHECK NUMBER;
BEGIN
  DBMS_PROFILER.START_PROFILER(SYSDATE);
  SHA_OPTIMIZE2;
  SELECT COUNT(*) INTO INDEX_CHECK
  FROM USER_INDEXES
  WHERE INDEX_NAME= 'FMG_BASE_IDX1';
  IF INDEX_CHECK  =1 THEN
    EXECUTE IMMEDIATE 'DROP INDEX FMG_BASE_IDX1';
  END IF;
  SELECT COUNT(*) INTO INDEX_CHECK
  FROM USER_INDEXES
  WHERE INDEX_NAME= 'FMG_BASE_IDX2';
  IF INDEX_CHECK  =1 THEN
    EXECUTE IMMEDIATE 'DROP INDEX FMG_BASE_IDX2';
  END IF;
  INSERT INTO PROC_PKG_RUN_LOG VALUES('BEGIN P_FMG_BASE_SEGMENTATION', SYSTIMESTAMP);
  COMMIT;
  P_FMG_BASE_SEGMENTATION(INV_YEAR , COND_YEAR, COUNTY_IN );
  SELECT COUNT(*) INTO INDEX_CHECK
  FROM USER_INDEXES
  WHERE INDEX_NAME= 'FMG_BASE_IDX1';
  IF INDEX_CHECK  =0 THEN
    EXECUTE IMMEDIATE 'CREATE INDEX FMG_BASE_IDX1 ON FMG_BASE_SEGMENTS(COLLECT_YEAR,GLOBAL_ROUTE_ID,SUB_ROUTE_ID,BMP,EMP)';
  END IF;
  SELECT COUNT(*) INTO INDEX_CHECK
  FROM USER_INDEXES
  WHERE INDEX_NAME= 'FMG_BASE_IDX2';
  IF INDEX_CHECK  =0 THEN
    EXECUTE IMMEDIATE 'CREATE INDEX FMG_BASE_IDX2 ON FMG_BASE_SEGMENTS(COLLECT_YEAR,HMIS_ROUTEID,EMP,GLOBAL_ROUTE_ID,SUB_ROUTE_ID)';
  END IF;
  INSERT INTO PROC_PKG_RUN_LOG VALUES('BEGIN P_FMG_INTERMED_SEGMENTATION', SYSTIMESTAMP);
  COMMIT;
  P_FMG_INTERMED_SEGMENTATION;
  INSERT INTO PROC_PKG_RUN_LOG VALUES('BEGIN P_FMG_FINAL_SEGMENTATION', SYSTIMESTAMP);
  COMMIT;
  P_FMG_FINAL_SEGMENTATION;
  INSERT INTO PROC_PKG_RUN_LOG VALUES('BEGIN P_FMG_ATTRIBUTE_UPDATE', SYSTIMESTAMP);
  COMMIT;
  P_FMG_ATTRIBUTE_UPDATE;
  INSERT INTO PROC_PKG_RUN_LOG VALUES('END P_FMG_ATTRIBUTE_UPDATE', SYSTIMESTAMP);
  COMMIT;
  DBMS_PROFILER.STOP_PROFILER;
END FMG_MAIN ;

PROCEDURE P_DH_GRADE(P_YEAR NUMBER)
AS
  CURSOR C0 IS
    SELECT DISTINCT S.GLOBAL_ROUTE_ID, S.SUB_ROUTE_ID
    FROM FINAL_CONDITION_DETAIL_SKID S
    WHERE COLLECT_YEAR = P_YEAR
    ORDER BY 1,2 ;
TYPE TYP0 IS TABLE OF C0%ROWTYPE;
  TAB0 TYP0;
  CURSOR C1 IS SELECT * FROM DH_GRADE_SKID ;
TYPE TYP1 IS TABLE OF C1%ROWTYPE;
  TAB1 TYP1;
  V_BMP NUMBER;
  V_EMP NUMBER;
BEGIN
  EXECUTE IMMEDIATE ' TRUNCATE TABLE DH_GRADE_SKID';
  OPEN C0;
  LOOP
    FETCH C0 BULK COLLECT INTO TAB0 LIMIT 100;
    EXIT WHEN TAB0.COUNT =0;
    FOR N IN 1..TAB0.COUNT
    LOOP
      SELECT * BULK COLLECT INTO TAB1
      FROM
        (SELECT P_YEAR, TAB0(N).GLOBAL_ROUTE_ID, TAB0(N).SUB_ROUTE_ID, BMP DIR_BMP, EMP DIR_EMP, EMP-BMP SEC_LENGTH, SEC_ATTRIBUTE
        FROM
          (SELECT MIN(DIR_BMP) BMP, MAX(DIR_EMP) EMP, SEC_ATTRIBUTE, SEC_RN
          FROM
            (SELECT DIR_BMP, DIR_EMP, SEC_ATTRIBUTE, RN_, ROW_NUMBER() OVER(ORDER BY RN_) -ROW_NUMBER() OVER(PARTITION BY SEC_ATTRIBUTE ORDER BY RN_) AS SEC_RN
            FROM
              (SELECT TT.*, ROWNUM RN_
              FROM
                ( SELECT DISTINCT DIR_BMP, DIR_EMP,
                  CASE
                    WHEN GRADE>-5 THEN 'NO_DH_GRADE'
                    WHEN GRADE >= -10 AND GRADE  <= -5 THEN 'DH_GRADE_5_TO_10'
                    WHEN GRADE < -10 THEN 'DH_GRADE_10'
                  END SEC_ATTRIBUTE
                FROM FINAL_CONDITION_DETAIL_SKID
                WHERE COLLECT_YEAR  = P_YEAR
                AND GLOBAL_ROUTE_ID = TAB0(N).GLOBAL_ROUTE_ID
                AND SUB_ROUTE_ID    = TAB0(N).SUB_ROUTE_ID
                ORDER BY 1
                ) TT
              )
            )
          GROUP BY SEC_ATTRIBUTE, SEC_RN
          )
        )
      WHERE SEC_ATTRIBUTE ='DH_GRADE_5_TO_10'
      ORDER BY 1,2,3,4 ;
      V_BMP := NULL;
      V_EMP := NULL;
      FOR I IN 1..TAB1.COUNT
      LOOP
        IF I                    = 1 THEN
          V_BMP                := TAB1(I).DIR_BMP;
          V_EMP                := TAB1(I).DIR_EMP;
        ELSIF I                 > 1 THEN
          IF TAB1(I).DIR_BMP   <= TAB1(I - 1).DIR_EMP+.01 THEN
            V_EMP              := TAB1(I).DIR_EMP;
          ELSIF TAB1(I).DIR_BMP > TAB1(I - 1).DIR_EMP+.01 THEN
            INSERT INTO DH_GRADE_SKID VALUES(TAB1(I).COLLECT_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, V_BMP, V_EMP, NULL, TAB1(I).SEC_ATTRIBUTE);
            V_BMP := TAB1(I).DIR_BMP;
            V_EMP := TAB1(I).DIR_EMP;
          END IF;
        END IF;
        IF I = TAB1.COUNT THEN
          INSERT INTO DH_GRADE_SKID VALUES(TAB1(I).COLLECT_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, V_BMP, V_EMP, NULL, TAB1(I).SEC_ATTRIBUTE);
        END IF;
      END LOOP;
    END LOOP;
  END LOOP;
  CLOSE C0;
  OPEN C0;
  LOOP
    FETCH C0 BULK COLLECT INTO TAB0 LIMIT 100;
    EXIT WHEN TAB0.COUNT =0;
    FOR N IN 1..TAB0.COUNT
    LOOP
      SELECT * BULK COLLECT INTO TAB1
      FROM
        (SELECT P_YEAR, TAB0(N).GLOBAL_ROUTE_ID, TAB0(N).SUB_ROUTE_ID, BMP DIR_BMP, EMP DIR_EMP, EMP-BMP SEC_LENGTH, SEC_ATTRIBUTE
        FROM
          (SELECT MIN(DIR_BMP) BMP, MAX(DIR_EMP) EMP, SEC_ATTRIBUTE, SEC_RN
          FROM
            (SELECT DIR_BMP, DIR_EMP, SEC_ATTRIBUTE, RN_, ROW_NUMBER() OVER(ORDER BY RN_) -ROW_NUMBER() OVER(PARTITION BY SEC_ATTRIBUTE ORDER BY RN_) AS SEC_RN
            FROM
              (SELECT TT.*, ROWNUM RN_
              FROM
                ( SELECT DISTINCT DIR_BMP, DIR_EMP,
                  CASE
                    WHEN GRADE>-5 THEN 'NO_DH_GRADE'
                    WHEN GRADE >= -10 AND GRADE  <= -5 THEN 'DH_GRADE_5_TO_10'
                    WHEN GRADE < -10 THEN 'DH_GRADE_10'
                  END SEC_ATTRIBUTE
                FROM FINAL_CONDITION_DETAIL_SKID
                WHERE COLLECT_YEAR  = P_YEAR
                AND GLOBAL_ROUTE_ID = TAB0(N).GLOBAL_ROUTE_ID
                AND SUB_ROUTE_ID    = TAB0(N).SUB_ROUTE_ID
                ORDER BY 1
                ) TT
              )
            )
          GROUP BY SEC_ATTRIBUTE, SEC_RN
          )
        )
      WHERE SEC_ATTRIBUTE ='DH_GRADE_10'
      ORDER BY 1,2,3,4 ;
      V_BMP := NULL;
      V_EMP := NULL;
      FOR I IN 1..TAB1.COUNT
      LOOP
        IF I                    = 1 THEN
          V_BMP                := TAB1(I).DIR_BMP;
          V_EMP                := TAB1(I).DIR_EMP;
        ELSIF I                 > 1 THEN
          IF TAB1(I).DIR_BMP   <= TAB1(I - 1).DIR_EMP+.01 THEN
            V_EMP              := TAB1(I).DIR_EMP;
          ELSIF TAB1(I).DIR_BMP > TAB1(I - 1).DIR_EMP+.01 THEN
            INSERT INTO DH_GRADE_SKID VALUES(TAB1(I).COLLECT_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, V_BMP, V_EMP, NULL, TAB1(I).SEC_ATTRIBUTE);
            V_BMP := TAB1(I).DIR_BMP;
            V_EMP := TAB1(I).DIR_EMP;
          END IF;
        END IF;
        IF I = TAB1.COUNT THEN
          INSERT INTO DH_GRADE_SKID VALUES(TAB1(I).COLLECT_YEAR, TAB1(I).GLOBAL_ROUTE_ID, TAB1(I).SUB_ROUTE_ID, V_BMP, V_EMP, NULL, TAB1(I).SEC_ATTRIBUTE);
        END IF;
      END LOOP;
    END LOOP;
  END LOOP;
  CLOSE C0;
  UPDATE DH_GRADE_SKID SET SEC_LENGTH = DIR_EMP- DIR_BMP;
END P_DH_GRADE;

PROCEDURE P_CURVE_RUT(P_YEAR NUMBER)
AS
  CNT       NUMBER;
  YEAR_LOOP NUMBER ;
BEGIN
  EXECUTE IMMEDIATE 'TRUNCATE TABLE FINAL_CONDITION_DETAIL_SKID';
  FOR I IN (SELECT DISTINCT GLOBAL_ROUTE_ID, SUB_ROUTE_ID
  FROM SKID_INVENTORY
  ORDER BY 1,2)
  LOOP
    YEAR_LOOP      := P_YEAR;
    WHILE YEAR_LOOP>=P_YEAR-10
    LOOP
      SELECT COUNT(*) INTO CNT
      FROM FINAL_CONDITION_DETAIL
      WHERE GLOBAL_ROUTE_ID = I.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID      = I.SUB_ROUTE_ID
      AND COLLECT_YEAR      = YEAR_LOOP;
      IF CNT >0 THEN
        INSERT INTO FINAL_CONDITION_DETAIL_SKID
        SELECT P_YEAR COLLECT_YEAR, COLLECT_YEAR SOURCE_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, DIR_BMP, DIR_EMP, INV_BMP, INV_EMP, L_RUT, R_RUT, GRADE
        FROM FINAL_CONDITION_DETAIL
        WHERE GLOBAL_ROUTE_ID = I.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID      = I.SUB_ROUTE_ID
        AND COLLECT_YEAR      = YEAR_LOOP
        ORDER BY DIR_BMP;
        EXIT;
      ELSE
        SELECT COUNT(*) INTO CNT
        FROM FINAL_CONDITION_DETAIL
        WHERE GLOBAL_ROUTE_ID_ORG = I.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID_ORG      = I.SUB_ROUTE_ID
        AND COLLECT_YEAR          = YEAR_LOOP;
        IF CNT                    >0 THEN
          INSERT INTO FINAL_CONDITION_DETAIL_SKID
          SELECT P_YEAR COLLECT_YEAR, COLLECT_YEAR SOURCE_YEAR, GLOBAL_ROUTE_ID_ORG, SUB_ROUTE_ID_ORG, DIR_BMP_ORG, DIR_EMP_ORG, INV_BMP_ORG, INV_EMP_ORG, L_RUT, R_RUT, GRADE
          FROM FINAL_CONDITION_DETAIL
          WHERE GLOBAL_ROUTE_ID_ORG = I.GLOBAL_ROUTE_ID
          AND SUB_ROUTE_ID_ORG      = I.SUB_ROUTE_ID
          AND COLLECT_YEAR          = YEAR_LOOP
          ORDER BY DIR_BMP;
          EXIT;
        ELSE
          YEAR_LOOP:=YEAR_LOOP-1;
        END IF;
      END IF;
    END LOOP;
  END LOOP;
END P_CURVE_RUT;

PROCEDURE P_FMG_BASE_SEGMENTATION(INV_YEAR  NUMBER, COND_YEAR NUMBER, COUNTY_IN VARCHAR2)
AS
  CURSOR C1 IS
    SELECT BMP, EMP
    FROM FMG_BASE_SEGMENTS;
TYPE TYP1 IS TABLE OF C1%ROWTYPE;
  TAB1 TYP1;
  MAX_MP     NUMBER;
  MIN_MP     NUMBER;
  ROUTE_CNT  NUMBER;
  DIR_MAX_MP NUMBER;
  DIR_MIN_MP NUMBER;
BEGIN
  EXECUTE IMMEDIATE 'TRUNCATE TABLE FMG_BASE_SEGMENTS';
  FOR ROUTE_LIST IN
  (SELECT DISTINCT S.GLOBAL_ROUTE_ID, S.SUB_ROUTE_ID, S.GOVT_CONTROL, S.HMIS_ROUTEID, S.DIR_BMP, S.DIR_EMP, S.INV_BMP, S.INV_EMP
  FROM SKID_INVENTORY S
  WHERE COUNTY =
    CASE
      WHEN COUNTY_IN IS NOT NULL
      THEN COUNTY_IN
      ELSE COUNTY
    END
  ORDER BY 1,2,5)
  LOOP
    IF ROUTE_LIST.SUB_ROUTE_ID =1 OR (ROUTE_LIST.SUB_ROUTE_ID =2 AND SUBSTR(ROUTE_LIST.HMIS_ROUTEID,16,1)=9 ) THEN
      INSERT INTO FMG_BASE_SEGMENTS(COLLECT_YEAR, GOVT_CONTROL, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, BMP, EMP)
    WITH SPEED_LIMIT AS
      ( SELECT DISTINCT ROUTEID, ID_MP BMP, ID_MP+SECTION_LENGTH EMP,
        CASE
          WHEN H.SPEED_LIMIT>=55 THEN 'ABOVE_55'
          WHEN H.SPEED_LIMIT>=35 AND H.SPEED_LIMIT  <55 THEN 'BELOW_55'
          ELSE 'BELOW_35'
        END SEC_ATTRIBUTE
      FROM HMIS_UNIVERSE_ALL_YEARS H
      WHERE YEAR  = INV_YEAR
      AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
      ORDER BY ID_MP
      ),
      DIVIDED AS
      ( SELECT DISTINCT ROUTEID, ID_MP BMP, ID_MP+SECTION_LENGTH EMP,
        CASE
          WHEN MEDIAN_TY IN (4,5) THEN 'UNDIVIDED'
          ELSE 'DIVIDED'
        END SEC_ATTRIBUTE
      FROM HMIS_UNIVERSE_ALL_YEARS
      WHERE YEAR  = INV_YEAR
      AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
      ORDER BY ID_MP
      ),
      RUT AS
      (SELECT *
      FROM
        ( SELECT DISTINCT DIR_BMP BMP, DIR_EMP EMP,
          CASE
            WHEN (L_RUT+R_RUT)/2 >0.25 THEN 'HIGH_RUT'
            ELSE 'LOW_RUT'
          END SEC_ATTRIBUTE
        FROM FINAL_CONDITION_DETAIL_SKID
        WHERE COLLECT_YEAR  = COND_YEAR
        AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
        ORDER BY 1
        ) TT
      WHERE SEC_ATTRIBUTE ='HIGH_RUT'
      ) ,
      CURVE AS
      (SELECT BMP DIR_BMP, EMP DIR_EMP, SEC_ATTRIBUTE
      FROM
        (SELECT DIR_BMP BMP, DIR_EMP EMP, DIR_EMP-DIR_BMP SEC_LENGTH,
          CASE
            WHEN RADIUS <=250*3 THEN 'CURVE_750'
            WHEN RADIUS <=250 *6 AND RADIUS   > 250*3 THEN 'CURVE_1500'
            ELSE 'CURVE_NOT750'
          END SEC_ATTRIBUTE
        FROM CURVE_H_SHA_NETWORK@PAV_CONHIST_ASSET
        WHERE YEAR          = COND_YEAR
        AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
        )
      WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
      ),
      INTERSECTIONS AS
      (SELECT
        CASE
          WHEN ID_MP-.1>=0 THEN ID_MP-.1
          ELSE 0
        END BMP, ID_MP EMP, MP_INT_RTE_NAME SEC_ATTRIBUTE
      FROM HMIS_MASTER_ALL_YEARS
      WHERE YEAR            = INV_YEAR
      AND ROUTEID           = ROUTE_LIST.HMIS_ROUTEID
      AND MP_LOCATION_TYPE <> 2
      AND (MP_INT_TYPE      > 0 AND MP_INT_TYPE       < 100 AND MP_INT_TYPE NOT  IN (7,10,14))
      ),
      DH_GRADE AS
      (SELECT *
      FROM DH_GRADE_SKID
      WHERE SEC_LENGTH   >=.03
      AND COLLECT_YEAR    = COND_YEAR
      AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
      ),
      RAMP AS
      (SELECT
        CASE
          WHEN ID_MP-.1>=0 THEN ID_MP-.1
          ELSE 0
        END BMP, ID_MP EMP, 'RAMP' SEC_ATTRIBUTE
      FROM HMIS_MASTER_ALL_YEARS
      WHERE YEAR           = INV_YEAR
      AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
      AND MP_LOCATION_TYPE = 2
      AND ID_PREFIX       <>'RP'
      AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%')
      UNION
      SELECT
        CASE
          WHEN ID_MP-.1>=0 THEN ID_MP-.1
          ELSE 0
        END BMP, ID_MP EMP, 'RAMP' SEC_ATTRIBUTE
      FROM HMIS_MASTER_ALL_YEARS
      WHERE YEAR           = INV_YEAR
      AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
      AND MP_LOCATION_TYPE = 2
      AND ID_PREFIX       <>'RP'
      AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%')
      UNION
      SELECT
        CASE
          WHEN ID_MP-.1>=0 THEN ID_MP-.1
          ELSE 0
        END BMP, ID_MP EMP, 'RAMP' SEC_ATTRIBUTE
      FROM HMIS_MASTER_ALL_YEARS
      WHERE YEAR           = INV_YEAR
      AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
      AND MP_LOCATION_TYPE = 2
      AND ID_PREFIX        ='RP'
      AND (MP_INT_RTE_NAME LIKE '% '|| TRIM(SUBSTR(ROAD_NAME,1,7))|| '%')
      ORDER BY BMP
      ),
      SITE_CAT_1 AS
      (SELECT
        CASE
          WHEN ID_MP-.1>=0 THEN ID_MP-.1
          ELSE 0
        END BMP, ID_MP EMP,
        CASE
          WHEN MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' THEN 'TRAFFIC_SIGNAL'
          WHEN MP_INT_RTE_NAME LIKE '%STOP SIGN%' THEN 'STOP_SIGN'
          WHEN (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') THEN 'ROUND_ABOUT'
          WHEN MP_INT_RTE_NAME LIKE '%SPUR TO%' THEN 'GIVE_WAY'
          WHEN MP_INT_RTE_NAME LIKE '%RAILROAD CROSSING%' THEN 'RAIL_ROAD'
        END SEC_ATTRIBUTE
      FROM HMIS_MASTER_ALL_YEARS
      WHERE YEAR  = INV_YEAR
      AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
      AND (MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' OR MP_INT_RTE_NAME LIKE '%STOP SIGN%' OR (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') OR MP_INT_RTE_NAME LIKE '%SPUR TO%' OR MP_INT_RTE_NAME LIKE '%RAILROADC ROSSING%')
      UNION
      SELECT
        CASE
          WHEN DIR_EMP-.1>=0 THEN DIR_EMP-.1
          ELSE 0
        END BMP, DIR_EMP EMP, 'PEDESTRIAN_CROSSING' PEDESTRIAN_CROSSING
      FROM EDW18_BASE_4@PAV_CONHIST_ASSET
      WHERE GLOBAL_ROUTE_ID      = ROUTE_LIST.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID           = ROUTE_LIST.SUB_ROUTE_ID
      AND STATUS                 = 'MATCHED'
      AND EVENT_CROSSWALK_OFFICE = 1
      ),
      FINAL_SEG AS
      ( SELECT BMP FROM SPEED_LIMIT UNION SELECT EMP FROM SPEED_LIMIT UNION SELECT BMP FROM DIVIDED UNION SELECT EMP FROM DIVIDED UNION SELECT BMP FROM RUT UNION SELECT EMP FROM RUT UNION SELECT BMP FROM INTERSECTIONS UNION SELECT EMP FROM INTERSECTIONS UNION SELECT DIR_BMP FROM CURVE UNION SELECT DIR_EMP FROM CURVE UNION SELECT DIR_BMP FROM DH_GRADE UNION SELECT DIR_EMP FROM DH_GRADE UNION SELECT BMP FROM RAMP UNION SELECT EMP FROM RAMP UNION SELECT BMP FROM SITE_CAT_1 UNION SELECT EMP FROM SITE_CAT_1 ORDER BY 1
      )
    SELECT COND_YEAR, ROUTE_LIST.GOVT_CONTROL, ROUTE_LIST.HMIS_ROUTEID, ROUTE_LIST.GLOBAL_ROUTE_ID, ROUTE_LIST.SUB_ROUTE_ID, BMP, EMP
    FROM
      ( SELECT BMP, LEAD (BMP) OVER ( ORDER BY BMP) EMP FROM FINAL_SEG
      )
    WHERE EMP IS NOT NULL
    AND BMP   >=0
    AND BMP   <> EMP
    AND BMP   >=ROUTE_LIST.DIR_BMP
    AND EMP   <=ROUTE_LIST.DIR_EMP
    ORDER BY BMP;
    UPDATE FMG_BASE_SEGMENTS SET INV_BMP         = BMP, INV_EMP           = EMP WHERE SUB_ROUTE_ID  =1 AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT F.ROWID ROW_ID, G.REVERSE_MILEPOINT INV_BMP
    FROM FMG_BASE_SEGMENTS F, GLOBAL_SEGMENT_PAV G
    WHERE F.GLOBAL_ROUTE_ID= G.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID     = G.SUB_ROUTE_ID AND F.EMP              = G.BEGINNING_MILEPOINT AND F.SUB_ROUTE_ID     =2 AND G.GLOBAL_ROUTE_ID  = ROUTE_LIST.GLOBAL_ROUTE_ID AND G.RETIRE_DATE     IS NULL
    ) B ON ( A.ROWID       = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.INV_BMP = B.INV_BMP;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT F.ROWID ROW_ID, G.REVERSE_MILEPOINT INV_EMP
    FROM FMG_BASE_SEGMENTS F, GLOBAL_SEGMENT_PAV G
    WHERE F.GLOBAL_ROUTE_ID= G.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID     = G.SUB_ROUTE_ID AND F.BMP              = G.BEGINNING_MILEPOINT AND F.SUB_ROUTE_ID     =2 AND G.GLOBAL_ROUTE_ID  = ROUTE_LIST.GLOBAL_ROUTE_ID AND G.RETIRE_DATE     IS NULL
    ) B ON ( A.ROWID       = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.INV_EMP = B.INV_EMP;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP,
      CASE
        WHEN MEDIAN_TY IN (4,5) THEN 'UNDIVIDED'
        ELSE 'DIVIDED'
      END SEC_ATTRIBUTE
    FROM HMIS_UNIVERSE_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND ( (ID_MP        >= F.BMP AND ID_MP            <F.EMP) OR (F.BMP           >= ID_MP AND F.EMP           <= ID_MP+SECTION_LENGTH) )
    ) B ON (A.ROWID      = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.DIVIDED = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP,
      CASE
        WHEN H.SPEED_LIMIT>=55 THEN 'ABOVE_55'
        WHEN H.SPEED_LIMIT>=35 AND H.SPEED_LIMIT  <55 THEN 'BELOW_55'
        ELSE 'BELOW_35'
      END SEC_ATTRIBUTE
    FROM HMIS_UNIVERSE_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND ( (ID_MP        >= F.BMP AND ID_MP            <F.EMP) OR (F.BMP           >= ID_MP AND F.EMP           <= ID_MP+SECTION_LENGTH) )
    ) B ON (A.ROWID      = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.SPEED_LIMIT = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, 'HIGH_RUT' SEC_ATTRIBUTE
    FROM FINAL_CONDITION_DETAIL_SKID H , FMG_BASE_SEGMENTS F
    WHERE H.COLLECT_YEAR = COND_YEAR AND F.COLLECT_YEAR   = H.COLLECT_YEAR
    AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= ROUTE_LIST.DIR_BMP AND F.EMP            <= ROUTE_LIST.DIR_EMP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND (L_RUT+R_RUT)/2   >0.25 AND (H.DIR_BMP <=F.BMP AND H.DIR_EMP  >=F.EMP)
    ) B ON (A.ROWID = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.RUT = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, SEC_ATTRIBUTE
      FROM FMG_BASE_SEGMENTS F,
        (SELECT *
        FROM
          (SELECT DIR_BMP , DIR_EMP , DIR_EMP-DIR_BMP SEC_LENGTH,
            CASE
              WHEN RADIUS <=250*3 THEN 'CURVE_750'
              WHEN RADIUS <=250 *6 AND RADIUS   > 250*3 THEN 'CURVE_1500'
              ELSE 'CURVE_NOT750'
            END SEC_ATTRIBUTE
          FROM CURVE_H_SHA_NETWORK@PAV_CONHIST_ASSET
          WHERE YEAR          = COND_YEAR
          AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
          AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
          )
        WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
        ) H
      WHERE F.COLLECT_YEAR  = COND_YEAR AND F.BMP            >= ROUTE_LIST.DIR_BMP AND F.EMP            <= ROUTE_LIST.DIR_EMP AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND (H.DIR_BMP <=F.BMP AND H.DIR_EMP  >=F.EMP)
      ORDER BY 2
      )
    WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
    ) B ON (A.ROWID      = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.CURVE = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
      FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
      WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
      AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.BMP          >= ID_MP -.1 AND F.EMP           <= ID_MP) AND (MP_INT_TYPE     > 0 AND MP_INT_TYPE      < 100 AND MP_INT_TYPE NOT IN (7,10,14))
      )
    WHERE RN        =1
    ) B ON (A.ROWID = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.INTERSECTIONS = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, H.SEC_ATTRIBUTE
    FROM FMG_BASE_SEGMENTS F,
      (SELECT *
      FROM DH_GRADE_SKID
      WHERE SEC_LENGTH  >=.03 AND SEC_ATTRIBUTE IN ('DH_GRADE_5_TO_10')
      )H
    WHERE F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= ROUTE_LIST.DIR_BMP AND F.EMP            <= ROUTE_LIST.DIR_EMP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND F.COLLECT_YEAR    = H.COLLECT_YEAR AND ( (H.DIR_BMP     >=F.BMP AND H.DIR_EMP        <=F.EMP) OR (H.DIR_BMP         <F.BMP AND H.DIR_EMP         >F.EMP) OR (H.DIR_BMP        <=F.BMP AND H.DIR_EMP         >F.BMP) )
    )B ON (A.ROWID        = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.DOWNHILL_GRADE = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, H.SEC_ATTRIBUTE
    FROM FMG_BASE_SEGMENTS F,
      (SELECT *
      FROM DH_GRADE_SKID
      WHERE SEC_LENGTH>=.03 AND SEC_ATTRIBUTE= ('DH_GRADE_10')
      )H
    WHERE F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= ROUTE_LIST.DIR_BMP AND F.EMP            <= ROUTE_LIST.DIR_EMP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND F.COLLECT_YEAR    = H.COLLECT_YEAR AND ( (H.DIR_BMP     >=F.BMP AND H.DIR_EMP        <=F.EMP) OR (H.DIR_BMP         <F.BMP AND H.DIR_EMP         >F.EMP) OR (H.DIR_BMP        <=F.BMP AND H.DIR_EMP         >F.BMP) )
    )B ON (A.ROWID        = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.DOWNHILL_GRADE = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
      FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
      WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
      AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.BMP          >= ID_MP -.1 AND F.EMP           <= ID_MP) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX       <>'RP' AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%')
      )
    WHERE RN        =1
    ) B ON (A.ROWID = B.ROWID)
  WHEN MATCHED THEN
    UPDATE
    SET A.RAMP = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
      FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
      WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
      AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.BMP          >= ID_MP -.1 AND F.EMP           <= ID_MP) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX       <>'RP' AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%')
      )
    WHERE RN        =1
    ) B ON (A.ROWID = B.ROWID)
  WHEN MATCHED THEN
    UPDATE
    SET A.RAMP = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
      FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
      WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
      AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.BMP          >= ID_MP -.1 AND F.EMP           <= ID_MP) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX        ='RP' AND (MP_INT_RTE_NAME LIKE '% '|| TRIM(SUBSTR(ROAD_NAME,1,7))|| '%')
      )
    WHERE RN        =1
    ) B ON (A.ROWID = B.ROWID)
  WHEN MATCHED THEN
    UPDATE
    SET A.RAMP = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    (SELECT *
    FROM
      (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, H.ID_MP,
        CASE
          WHEN MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' THEN 'TRAFFIC_SIGNAL'
          WHEN MP_INT_RTE_NAME LIKE '%STOP SIGN%' THEN 'STOP_SIGN'
          WHEN (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') THEN 'ROUND_ABOUT'
          WHEN MP_INT_RTE_NAME LIKE '%SPUR TO%' THEN 'GIVE_WAY'
          WHEN MP_INT_RTE_NAME LIKE '%RAILROAD CROSSING%' THEN 'RAIL_ROAD'
        END SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
      FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
      WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
      AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP           >= ROUTE_LIST.DIR_BMP AND F.EMP           <= ROUTE_LIST.DIR_EMP AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.BMP          >= ID_MP -.1 AND F.EMP           <= ID_MP) AND (MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' OR MP_INT_RTE_NAME LIKE '%STOP SIGN%' OR (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') OR MP_INT_RTE_NAME LIKE '%SPUR TO%' OR MP_INT_RTE_NAME LIKE '%RAILROADC ROSSING%' )
      )
    WHERE RN        =1
    ) B ON (A.ROWID = B.ROWID)
  WHEN MATCHED THEN
    UPDATE
    SET A.CAT_ONE_FEATURES = B.SEC_ATTRIBUTE;
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, 'PEDESTRIAN_CROSSING' SEC_ATTRIBUTE
    FROM EDW18_BASE_4@PAV_CONHIST_ASSET H , FMG_BASE_SEGMENTS F
    WHERE F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.BMP                 >= ROUTE_LIST.DIR_BMP AND F.EMP                 <= ROUTE_LIST.DIR_EMP AND F.GLOBAL_ROUTE_ID      = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID         = H.SUB_ROUTE_ID AND STATUS                 = 'MATCHED' AND EVENT_CROSSWALK_OFFICE = 1 AND ( (H.DIR_EMP-.1       >=F.BMP AND H.DIR_EMP             <=F.EMP) OR (H.DIR_EMP-.1           <F.BMP AND H.DIR_EMP              >F.EMP) OR (H.DIR_EMP-.1          <=F.BMP AND H.DIR_EMP              >F.EMP) )
    ORDER BY 2
    ) B ON (A.ROWID = B.ROWID)
  WHEN MATCHED THEN
    UPDATE SET A.CAT_ONE_FEATURES = B.SEC_ATTRIBUTE;
    COMMIT;
  ELSE
    SELECT MAX(DIR_EMP), MIN(DIR_BMP) INTO MAX_MP, MIN_MP
    FROM SKID_INVENTORY
    WHERE GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND SUB_ROUTE_ID     = 1;
    SELECT MAX(DIR_EMP), MIN(DIR_BMP) INTO DIR_MAX_MP, DIR_MIN_MP
    FROM SKID_INVENTORY
    WHERE GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND SUB_ROUTE_ID     = 2;
    SELECT COUNT(*) INTO ROUTE_CNT
    FROM SKID_INVENTORY
    WHERE GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND SUB_ROUTE_ID     = ROUTE_LIST.SUB_ROUTE_ID;
    INSERT INTO FMG_BASE_SEGMENTS(COLLECT_YEAR, GOVT_CONTROL, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, BMP, EMP)
  WITH SPEED_LIMIT AS
    ( SELECT DISTINCT ROUTEID, ID_MP BMP, ID_MP+SECTION_LENGTH EMP,
      CASE
        WHEN H.SPEED_LIMIT>=55 THEN 'ABOVE_55'
        WHEN H.SPEED_LIMIT>=35 AND H.SPEED_LIMIT  <55 THEN 'BELOW_55'
        ELSE 'BELOW_35'
      END SEC_ATTRIBUTE
    FROM HMIS_UNIVERSE_ALL_YEARS H
    WHERE YEAR  = INV_YEAR
    AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
    ORDER BY ID_MP
    ),
    DIVIDED AS
    ( SELECT DISTINCT ROUTEID, ID_MP BMP, ID_MP+SECTION_LENGTH EMP,
      CASE
        WHEN MEDIAN_TY IN (4,5) THEN 'UNDIVIDED'
        ELSE 'DIVIDED'
      END SEC_ATTRIBUTE
    FROM HMIS_UNIVERSE_ALL_YEARS
    WHERE YEAR  = INV_YEAR
    AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
    ORDER BY ID_MP
    ) ,
    RUT AS
    (SELECT *
    FROM
      ( SELECT DISTINCT DIR_BMP BMP, DIR_EMP EMP,
        CASE
          WHEN (L_RUT+R_RUT)/2 >0.25 THEN 'HIGH_RUT'
          ELSE 'LOW_RUT'
        END SEC_ATTRIBUTE
      FROM FINAL_CONDITION_DETAIL_SKID
      WHERE COLLECT_YEAR  = COND_YEAR
      AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
      ORDER BY 1
      ) TT
    WHERE SEC_ATTRIBUTE ='HIGH_RUT'
    ),
    CURVE AS
    (SELECT BMP DIR_BMP, EMP DIR_EMP, SEC_ATTRIBUTE
    FROM
      (SELECT DIR_BMP BMP, DIR_EMP EMP, DIR_EMP-DIR_BMP SEC_LENGTH,
        CASE
          WHEN RADIUS <=250*3 THEN 'CURVE_750'
          WHEN RADIUS <=250 *6 AND RADIUS   > 250*3 THEN 'CURVE_1500'
          ELSE 'CURVE_NOT750'
        END SEC_ATTRIBUTE
      FROM CURVE_H_SHA_NETWORK@PAV_CONHIST_ASSET
      WHERE YEAR          = COND_YEAR
      AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
      )
    WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
    ),
    INTERSECTIONS AS
    (SELECT
      CASE
        WHEN ID_MP+.1>=MAX_MP THEN MAX_MP
        ELSE ID_MP+.1
      END EMP, ID_MP BMP, MP_INT_RTE_NAME SEC_ATTRIBUTE
    FROM HMIS_MASTER_ALL_YEARS
    WHERE YEAR            = INV_YEAR
    AND ROUTEID           = ROUTE_LIST.HMIS_ROUTEID
    AND MP_LOCATION_TYPE <> 2
    AND (MP_INT_TYPE      > 0 AND MP_INT_TYPE       < 100 AND MP_INT_TYPE NOT  IN (7,10,14))
    ),
    DH_GRADE AS
    (SELECT *
    FROM DH_GRADE_SKID
    WHERE SEC_LENGTH   >=.03
    AND COLLECT_YEAR    = COND_YEAR
    AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
    ),
    RAMP AS
    (SELECT
      CASE
        WHEN ID_MP+.1>=MAX_MP THEN MAX_MP
        ELSE ID_MP+.1
      END EMP, ID_MP BMP, 'RAMP' SEC_ATTRIBUTE
    FROM HMIS_MASTER_ALL_YEARS
    WHERE YEAR           = INV_YEAR
    AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
    AND MP_LOCATION_TYPE = 2
    AND ID_PREFIX       <>'RP'
    AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%')
    UNION
    SELECT
      CASE
        WHEN ID_MP+.1>=MAX_MP THEN MAX_MP
        ELSE ID_MP+.1
      END EMP, ID_MP BMP, 'RAMP' SEC_ATTRIBUTE
    FROM HMIS_MASTER_ALL_YEARS
    WHERE YEAR           = INV_YEAR
    AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
    AND MP_LOCATION_TYPE = 2
    AND ID_PREFIX       <>'RP'
    AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%')
    UNION
    SELECT
      CASE
        WHEN ID_MP+.1>=MAX_MP THEN MAX_MP
        ELSE ID_MP+.1
      END EMP, ID_MP BMP, 'RAMP' SEC_ATTRIBUTE
    FROM HMIS_MASTER_ALL_YEARS
    WHERE YEAR           = INV_YEAR
    AND ROUTEID          = ROUTE_LIST.HMIS_ROUTEID
    AND MP_LOCATION_TYPE = 2
    AND ID_PREFIX        ='RP'
    AND (MP_INT_RTE_NAME LIKE '% '|| TRIM(SUBSTR(ROAD_NAME,1,7))|| '%')
    ORDER BY BMP
    ),
    SITE_CAT_1 AS
    (SELECT
      CASE
        WHEN ID_MP+.1>=MAX_MP THEN MAX_MP
        ELSE ID_MP+.1
      END EMP, ID_MP BMP,
        CASE
          WHEN MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' THEN 'TRAFFIC_SIGNAL'
          WHEN MP_INT_RTE_NAME LIKE '%STOP SIGN%' THEN 'STOP_SIGN'
          WHEN (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') THEN 'ROUND_ABOUT'
          WHEN MP_INT_RTE_NAME LIKE '%SPUR TO%' THEN 'GIVE_WAY'
          WHEN MP_INT_RTE_NAME LIKE '%RAILROAD CROSSING%' THEN 'RAIL_ROAD'
        END SEC_ATTRIBUTE
    FROM HMIS_MASTER_ALL_YEARS
    WHERE YEAR  = INV_YEAR
    AND ROUTEID = ROUTE_LIST.HMIS_ROUTEID
    AND (MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' OR MP_INT_RTE_NAME LIKE '%STOP SIGN%' OR (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') OR MP_INT_RTE_NAME LIKE '%SPUR TO%' OR MP_INT_RTE_NAME LIKE '%RAILROADC ROSSING%')
    UNION
    SELECT
      CASE
        WHEN DIR_EMP-.1>=0 THEN DIR_EMP-.1
        ELSE 0
      END BMP, DIR_EMP EMP, 'PEDESTRIAN_CROSSING' PEDESTRIAN_CROSSING
    FROM EDW18_BASE_4@PAV_CONHIST_ASSET
    WHERE GLOBAL_ROUTE_ID      = ROUTE_LIST.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID           = ROUTE_LIST.SUB_ROUTE_ID
    AND STATUS                 = 'MATCHED'
    AND EVENT_CROSSWALK_OFFICE = 1
    ),
    FINAL_SEG AS
    (SELECT MAX_MP- EMP+DIR_MIN_MP BMP FROM SPEED_LIMIT S UNION SELECT MAX_MP- BMP+DIR_MIN_MP EMP FROM SPEED_LIMIT S UNION SELECT MAX_MP- EMP+DIR_MIN_MP BMP FROM DIVIDED S UNION SELECT MAX_MP- BMP+DIR_MIN_MP EMP FROM DIVIDED S UNION SELECT BMP FROM RUT UNION SELECT EMP FROM RUT UNION SELECT MAX_MP- EMP+DIR_MIN_MP BMP FROM INTERSECTIONS S UNION SELECT MAX_MP- BMP+DIR_MIN_MP EMP FROM INTERSECTIONS S UNION SELECT DIR_BMP FROM CURVE UNION SELECT DIR_EMP FROM CURVE UNION SELECT DIR_BMP FROM DH_GRADE UNION SELECT DIR_EMP FROM DH_GRADE UNION SELECT MAX_MP- EMP+DIR_MIN_MP BMP FROM RAMP S UNION SELECT MAX_MP- BMP+DIR_MIN_MP EMP FROM RAMP S UNION SELECT MAX_MP- EMP+DIR_MIN_MP BMP FROM SITE_CAT_1 S UNION SELECT MAX_MP- BMP+DIR_MIN_MP EMP FROM SITE_CAT_1 S ORDER BY 1
    )
  SELECT COND_YEAR, ROUTE_LIST.GOVT_CONTROL, ROUTE_LIST.HMIS_ROUTEID, ROUTE_LIST.GLOBAL_ROUTE_ID, ROUTE_LIST.SUB_ROUTE_ID, BMP, EMP
  FROM
    ( SELECT BMP, LEAD (BMP) OVER ( ORDER BY BMP) EMP FROM FINAL_SEG
    )
  WHERE EMP IS NOT NULL
  AND BMP   >=0
  AND BMP   <> EMP
  AND BMP   >= CASE WHEN ROUTE_CNT> 1 THEN ROUTE_LIST.DIR_BMP ELSE DIR_MIN_MP END
  AND EMP <= CASE WHEN ROUTE_CNT> 1 THEN ROUTE_LIST.DIR_EMP ELSE DIR_MAX_MP END
  ORDER BY BMP;
  UPDATE FMG_BASE_SEGMENTS F
  SET INV_BMP            = MAX_MP- EMP+DIR_MIN_MP, INV_EMP              = MAX_MP- BMP+DIR_MIN_MP
  WHERE F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID     = ROUTE_LIST.SUB_ROUTE_ID;
  DELETE
  FROM FMG_BASE_SEGMENTS F
  WHERE F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID     = ROUTE_LIST.SUB_ROUTE_ID AND (INV_BMP           <MIN_MP OR INV_EMP             >MAX_MP) ;
  COMMIT;
  BEGIN
    MERGE INTO FMG_BASE_SEGMENTS A USING
    ( SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP,
      CASE
        WHEN MEDIAN_TY IN (4,5) THEN 'UNDIVIDED'
        ELSE 'DIVIDED'
      END SEC_ATTRIBUTE
    FROM HMIS_UNIVERSE_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID = H.ROUTEID AND ( (ID_MP      >= F.INV_BMP AND ID_MP          <F.INV_EMP) OR (F.INV_BMP     >= ID_MP AND F.INV_EMP     <= ID_MP+SECTION_LENGTH) )
    ) B ON (A.ROWID    = B.ROW_ID)
  WHEN MATCHED THEN
    UPDATE
    SET A.DIVIDED = B.SEC_ATTRIBUTE;
  EXCEPTION WHEN OTHERS THEN DBMS_OUTPUT.PUT_LINE('LINE2043- '||ROUTE_LIST.HMIS_ROUTEID);
  END;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  ( SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP,
    CASE
      WHEN H.SPEED_LIMIT>=55 THEN 'ABOVE_55'
      WHEN H.SPEED_LIMIT>=35 AND H.SPEED_LIMIT  <55 THEN 'BELOW_55'
      ELSE 'BELOW_35'
    END SEC_ATTRIBUTE
  FROM HMIS_UNIVERSE_ALL_YEARS H , FMG_BASE_SEGMENTS F
  WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
  AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
  AND F.HMIS_ROUTEID = H.ROUTEID AND ( (ID_MP      >= F.INV_BMP AND ID_MP          <F.INV_EMP) OR (F.INV_BMP     >= ID_MP AND F.INV_EMP     <= ID_MP+SECTION_LENGTH) )
  ) B ON (A.ROWID    = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.SPEED_LIMIT = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  ( SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, 'HIGH_RUT' SEC_ATTRIBUTE
  FROM FINAL_CONDITION_DETAIL_SKID H , FMG_BASE_SEGMENTS F
  WHERE H.COLLECT_YEAR = COND_YEAR AND F.COLLECT_YEAR   = H.COLLECT_YEAR
  AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= MIN_MP AND F.EMP            <= MAX_MP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND (L_RUT+R_RUT)/2   >0.25 AND (H.DIR_BMP <=F.BMP AND H.DIR_EMP  >=F.EMP)
  ) B ON (A.ROWID = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.RUT = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.BMP, F.EMP, SEC_ATTRIBUTE
    FROM FMG_BASE_SEGMENTS F,
      (SELECT *
      FROM
        (SELECT DIR_BMP , DIR_EMP , DIR_EMP-DIR_BMP SEC_LENGTH,
          CASE
            WHEN RADIUS <=250*3 THEN 'CURVE_750'
            WHEN RADIUS <=250 *6 AND RADIUS   > 250*3 THEN 'CURVE_1500'
            ELSE 'CURVE_NOT750'
          END SEC_ATTRIBUTE
        FROM CURVE_H_SHA_NETWORK@PAV_CONHIST_ASSET
        WHERE YEAR          = COND_YEAR
        AND GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID
        )
      WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
      ) H
    WHERE F.COLLECT_YEAR  = COND_YEAR AND F.BMP            >= MIN_MP AND F.EMP            <= MAX_MP AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND (H.DIR_BMP       <=F.BMP AND H.DIR_EMP        >=F.EMP)
    ORDER BY 2
    )
  WHERE SEC_ATTRIBUTE <> 'CURVE_NOT750'
  ) B ON (A.ROWID      = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.CURVE = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
    FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.INV_BMP      >= ID_MP AND F.INV_EMP       <= ID_MP+.1) AND (MP_INT_TYPE     > 0 AND MP_INT_TYPE      < 100 AND MP_INT_TYPE NOT IN (7,10,14))
    )
  WHERE RN        =1
  ) B ON (A.ROWID = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.INTERSECTIONS = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  ( SELECT DISTINCT F.ROWID ROW_ID, H.SEC_ATTRIBUTE
  FROM FMG_BASE_SEGMENTS F,
    (SELECT *
    FROM DH_GRADE_SKID
    WHERE SEC_LENGTH  >=.03 AND SEC_ATTRIBUTE IN ('DH_GRADE_5_TO_10')
    )H
  WHERE F.COLLECT_YEAR = COND_YEAR
  AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= MIN_MP AND F.EMP            <= MAX_MP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND F.COLLECT_YEAR    = H.COLLECT_YEAR AND ( (H.DIR_BMP     >=F.BMP AND H.DIR_EMP        <=F.EMP) OR (H.DIR_BMP         <F.BMP AND H.DIR_EMP         >F.EMP) OR (H.DIR_BMP        <=F.BMP AND H.DIR_EMP         >F.BMP) )
  )B ON (A.ROWID        = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.DOWNHILL_GRADE = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  ( SELECT DISTINCT F.ROWID ROW_ID, H.SEC_ATTRIBUTE
  FROM FMG_BASE_SEGMENTS F,
    (SELECT *
    FROM DH_GRADE_SKID
    WHERE SEC_LENGTH>=.03 AND SEC_ATTRIBUTE= ('DH_GRADE_10')
    )H
  WHERE F.COLLECT_YEAR = COND_YEAR
  AND F.GLOBAL_ROUTE_ID = ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = ROUTE_LIST.SUB_ROUTE_ID AND F.BMP            >= MIN_MP AND F.EMP            <= MAX_MP AND F.GLOBAL_ROUTE_ID = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID    = H.SUB_ROUTE_ID AND F.COLLECT_YEAR    = H.COLLECT_YEAR AND ( (H.DIR_BMP     >=F.BMP AND H.DIR_EMP        <=F.EMP) OR (H.DIR_BMP         <F.BMP AND H.DIR_EMP         >F.EMP) OR (H.DIR_BMP        <=F.BMP AND H.DIR_EMP         >F.BMP) )
  )B ON (A.ROWID        = B.ROW_ID)
WHEN MATCHED THEN
  UPDATE
  SET A.DOWNHILL_GRADE = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
    FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.INV_BMP      >= ID_MP AND F.INV_EMP       <= ID_MP +.1) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX       <>'RP' AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '||MP_DIRECTION||'%')
    )
  WHERE RN        =1
  ) B ON (A.ROWID = B.ROWID)
WHEN MATCHED THEN
  UPDATE
  SET A.RAMP = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
    FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.INV_BMP      >= ID_MP AND F.INV_EMP       <= ID_MP +.1) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX       <>'RP' AND (MP_INT_RTE_NAME LIKE '%TO '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%' OR MP_INT_RTE_NAME LIKE '%FR '||ID_PREFIX||' '||ID_RTE_NO||TRIM(MP_SUFFIX)||' '|| CASE WHEN MP_DIRECTION ='E' THEN 'N' WHEN MP_DIRECTION ='W' THEN 'S' WHEN MP_DIRECTION ='N' THEN 'E' WHEN MP_DIRECTION ='S' THEN 'W' END ||'%')
    )
  WHERE RN        =1
  ) B ON (A.ROWID = B.ROWID)
WHEN MATCHED THEN
  UPDATE
  SET A.RAMP = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, H.ID_MP, MP_INT_RTE_NAME SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
    FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID   = H.ROUTEID AND (F.INV_BMP      >= ID_MP AND F.INV_EMP       <= ID_MP +.1) AND MP_LOCATION_TYPE = 2 AND ID_PREFIX        ='RP' AND (MP_INT_RTE_NAME LIKE '% '|| TRIM(SUBSTR(ROAD_NAME,1,7))|| '%')
    )
  WHERE RN        =1
  ) B ON (A.ROWID = B.ROWID)
WHEN MATCHED THEN
  UPDATE
  SET A.RAMP = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  (SELECT *
  FROM
    (SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, H.ID_MP,
      CASE
        WHEN MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' THEN 'TRAFFIC_SIGNAL'
        WHEN MP_INT_RTE_NAME LIKE '%STOP SIGN%' THEN 'STOP_SIGN'
        WHEN (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') THEN 'ROUND_ABOUT'
        WHEN MP_INT_RTE_NAME LIKE '%SPUR TO%' THEN 'GIVE_WAY'
        WHEN MP_INT_RTE_NAME LIKE '%RAILROAD CROSSING%' THEN 'RAIL_ROAD'
      END SEC_ATTRIBUTE, ROW_NUMBER() OVER ( PARTITION BY F.ROWID ORDER BY ID_MP,MP_INT_RTE_NAME) RN
    FROM HMIS_MASTER_ALL_YEARS H , FMG_BASE_SEGMENTS F
    WHERE H.YEAR       = INV_YEAR AND F.COLLECT_YEAR = COND_YEAR
    AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
    AND F.HMIS_ROUTEID = H.ROUTEID AND (F.INV_BMP    >= ID_MP AND F.INV_EMP     <= ID_MP +.1) AND (MP_INT_RTE_NAME LIKE '%TRAFFIC SIGNAL%' OR MP_INT_RTE_NAME LIKE '%STOP SIGN%' OR (MP_INT_RTE_NAME LIKE '%ROUNDABOUT%' AND MP_INT_RTE_NAME NOT LIKE '%END ROUNDABOUT%') OR MP_INT_RTE_NAME LIKE '%SPUR TO%' OR MP_INT_RTE_NAME LIKE '%RAILROADC ROSSING%' )
    )
  WHERE RN        =1
  ) B ON (A.ROWID = B.ROWID)
WHEN MATCHED THEN
  UPDATE
  SET A.CAT_ONE_FEATURES = B.SEC_ATTRIBUTE;
  MERGE INTO FMG_BASE_SEGMENTS A USING
  ( SELECT DISTINCT F.ROWID ROW_ID, F.INV_BMP, F.INV_EMP, 'PEDESTRIAN_CROSSING' SEC_ATTRIBUTE
  FROM EDW18_BASE_4@PAV_CONHIST_ASSET H , FMG_BASE_SEGMENTS F
  WHERE F.COLLECT_YEAR = COND_YEAR
  AND F.GLOBAL_ROUTE_ID= ROUTE_LIST.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID   = ROUTE_LIST.SUB_ROUTE_ID
  AND F.BMP                 >= MIN_MP AND F.EMP                 <= MAX_MP AND F.GLOBAL_ROUTE_ID      = H.GLOBAL_ROUTE_ID AND F.SUB_ROUTE_ID         = H.SUB_ROUTE_ID AND STATUS                 = 'MATCHED' AND EVENT_CROSSWALK_OFFICE = 1 AND ( (H.DIR_EMP-.1       >=F.INV_BMP AND H.DIR_EMP             <=F.INV_EMP) OR (H.DIR_EMP-.1           <F.INV_BMP AND H.DIR_EMP              >F.INV_EMP) OR (H.DIR_EMP-.1          <=F.INV_BMP AND H.DIR_EMP              >F.INV_EMP) )
  ORDER BY 2
  ) B ON (A.ROWID = B.ROWID)
WHEN MATCHED THEN
  UPDATE SET A.CAT_ONE_FEATURES = B.SEC_ATTRIBUTE;
END IF;
COMMIT;
END LOOP;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY        = 1
WHERE (CAT_ONE_FEATURES IS NOT NULL
AND SPEED_LIMIT          ='ABOVE_55')
AND SITE_CATEGORY       IS NULL ;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY  = 2
WHERE (CURVE       = 'CURVE_750'
OR DOWNHILL_GRADE  ='DH_GRADE_10'
OR RAMP           IS NOT NULL)
AND SITE_CATEGORY IS NULL ;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY          = 3
WHERE ( (CAT_ONE_FEATURES IS NOT NULL
AND SPEED_LIMIT            ='BELOW_55')
OR (INTERSECTIONS        IS NOT NULL
AND SPEED_LIMIT           IN ('ABOVE_55','BELOW_55') )
OR (DOWNHILL_GRADE          ='DH_GRADE_5_TO_10'
AND SPEED_LIMIT           IN ('ABOVE_55','BELOW_55') )
OR (DIVIDED                ='UNDIVIDED'
AND SPEED_LIMIT            ='ABOVE_55'
AND RUT                    ='HIGH_RUT')
OR (CURVE                  = 'CURVE_1500'
AND SPEED_LIMIT           IN ('ABOVE_55','BELOW_55') ))
AND SITE_CATEGORY         IS NULL ;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY          = 4
WHERE ( (CAT_ONE_FEATURES IS NOT NULL
AND SPEED_LIMIT            ='BELOW_35')
OR ( DIVIDED               ='UNDIVIDED'
AND (SPEED_LIMIT          IN ('BELOW_55','BELOW_35')
OR RUT                    IS NULL) )
OR ( INTERSECTIONS       IS NOT NULL
AND SPEED_LIMIT           IN ('BELOW_35') )
OR ( DOWNHILL_GRADE          ='DH_GRADE_5_TO_10'
AND SPEED_LIMIT           IN ('BELOW_35') )
OR (DIVIDED                ='DIVIDED'
AND SPEED_LIMIT            ='ABOVE_55'
AND RUT                    ='HIGH_RUT')
OR (CURVE                  = 'CURVE_1500'
AND SPEED_LIMIT           IN ('BELOW_35') )
)
AND SITE_CATEGORY         IS NULL ;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY  = 5
WHERE ( DIVIDED    ='DIVIDED'
AND (SPEED_LIMIT  IN ('BELOW_55','BELOW_35')
OR RUT            IS NULL)
)
AND SITE_CATEGORY IS NULL;
UPDATE FMG_BASE_SEGMENTS
SET SITE_CATEGORY =
  CASE
    WHEN SITE_CATEGORY IN (1,2) THEN 1
    WHEN SITE_CATEGORY IN (3) THEN 2
    WHEN SITE_CATEGORY IN (4,5) THEN 3
  END;
COMMIT;
END P_FMG_BASE_SEGMENTATION;

PROCEDURE P_FMG_INTERMED_SEGMENTATION
AS
  CURSOR C0 IS SELECT SITE_CATEGORY FROM FMG_BASE_SEGMENTS;
TYPE TYP0 IS TABLE OF C0% ROWTYPE;
  TAB0 TYP0;
  CURSOR C1 IS SELECT BMP, EMP FROM FMG_BASE_SEGMENTS;
TYPE TYP1 IS TABLE OF C1% ROWTYPE;
  TAB1 TYP1;
  V_BMP NUMBER;
  V_EMP NUMBER;
BEGIN
  EXECUTE IMMEDIATE' TRUNCATE TABLE FMG_FINAL_SEGMENTS';
  FOR SEG IN
  (SELECT DISTINCT COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL
  FROM FMG_BASE_SEGMENTS)
  LOOP
    SELECT DISTINCT SITE_CATEGORY BULK COLLECT INTO TAB0
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND GOVT_CONTROL    =SEG.GOVT_CONTROL
    ORDER BY SITE_CATEGORY ;
    FOR N IN 1..TAB0.COUNT
    LOOP
      SELECT BMP, EMP BULK COLLECT INTO TAB1
      FROM FMG_BASE_SEGMENTS
      WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
      AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
      AND GOVT_CONTROL    =SEG.GOVT_CONTROL
      AND SITE_CATEGORY   = TAB0(N).SITE_CATEGORY
      ORDER BY BMP;
      V_BMP := NULL;
      V_EMP := NULL;
      FOR I IN 1..TAB1.COUNT
      LOOP
        IF I                 = 1 THEN
          V_BMP             := TAB1(I).BMP;
          V_EMP             := TAB1(I).EMP;
        ELSIF I              > 1 THEN
          IF TAB1(I).BMP     = TAB1(I - 1).EMP THEN
            V_EMP           := TAB1(I).EMP;
          ELSIF TAB1(I).BMP <> TAB1(I - 1).EMP THEN
            INSERT INTO FMG_FINAL_SEGMENTS(COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL, BMP, EMP, SITE_CATEGORY_ORG)
            VALUES(SEG.COLLECT_YEAR, SEG.HMIS_ROUTEID, SEG.GLOBAL_ROUTE_ID, SEG.SUB_ROUTE_ID, SEG.GOVT_CONTROL, V_BMP, V_EMP, TAB0(N).SITE_CATEGORY);
            V_BMP := TAB1(I).BMP;
            V_EMP := TAB1(I).EMP;
          END IF;
        END IF;
        IF I = TAB1.COUNT THEN
          INSERT INTO FMG_FINAL_SEGMENTS(COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL, BMP, EMP, SITE_CATEGORY_ORG)
          VALUES(SEG.COLLECT_YEAR, SEG.HMIS_ROUTEID, SEG.GLOBAL_ROUTE_ID, SEG.SUB_ROUTE_ID, SEG.GOVT_CONTROL, V_BMP, V_EMP, TAB0(N).SITE_CATEGORY);
        END IF;
      END LOOP;
    END LOOP;
  END LOOP;
  UPDATE FMG_FINAL_SEGMENTS SET SEC_LENGTH = EMP-BMP;
  COMMIT;
END P_FMG_INTERMED_SEGMENTATION;

PROCEDURE P_FMG_FINAL_SEGMENTATION
AS
  CURSOR C0 IS SELECT SITE_CATEGORY FROM FMG_BASE_SEGMENTS;
TYPE TYP0 IS TABLE OF C0% ROWTYPE;
  TAB0 TYP0;
  CURSOR C1 IS SELECT BMP, EMP FROM FMG_BASE_SEGMENTS;
TYPE TYP1 IS TABLE OF C1% ROWTYPE;
  TAB1 TYP1;
  V_BMP NUMBER;
  V_EMP NUMBER;
BEGIN
  EXECUTE IMMEDIATE' TRUNCATE TABLE FMG_SITE_CATEGORY';
  FOR SEG IN
  (SELECT DISTINCT COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL
  FROM FMG_BASE_SEGMENTS)
  LOOP
    SELECT DISTINCT SITE_CATEGORY BULK COLLECT INTO TAB0
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND GOVT_CONTROL    = SEG.GOVT_CONTROL
    ORDER BY SITE_CATEGORY ;
    FOR N IN 1..TAB0.COUNT
    LOOP
      SELECT BMP, EMP BULK COLLECT INTO TAB1
      FROM FMG_BASE_SEGMENTS
      WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
      AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
      AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
      AND GOVT_CONTROL    = SEG.GOVT_CONTROL
      AND SITE_CATEGORY   = TAB0(N).SITE_CATEGORY
      ORDER BY BMP;
      V_BMP := NULL;
      V_EMP := NULL;
      FOR I IN 1..TAB1.COUNT
      LOOP
        IF I                 = 1 THEN
          V_BMP             := TAB1(I).BMP;
          V_EMP             := TAB1(I).EMP;
        ELSIF I              > 1 THEN
          IF TAB1(I).BMP     = TAB1(I - 1).EMP THEN
            V_EMP           := TAB1(I).EMP;
          ELSIF TAB1(I).BMP <> TAB1(I - 1).EMP THEN
            INSERT INTO FMG_SITE_CATEGORY(COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL, BMP, EMP, SITE_CATEGORY)
            VALUES(SEG.COLLECT_YEAR, SEG.HMIS_ROUTEID, SEG.GLOBAL_ROUTE_ID, SEG.SUB_ROUTE_ID, SEG.GOVT_CONTROL, V_BMP, V_EMP, TAB0(N).SITE_CATEGORY);
            V_BMP := TAB1(I).BMP;
            V_EMP := TAB1(I).EMP;
          END IF;
        END IF;
        IF I = TAB1.COUNT THEN
          INSERT INTO FMG_SITE_CATEGORY(COLLECT_YEAR, HMIS_ROUTEID, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, GOVT_CONTROL, BMP, EMP, SITE_CATEGORY)
          VALUES(SEG.COLLECT_YEAR, SEG.HMIS_ROUTEID, SEG.GLOBAL_ROUTE_ID, SEG.SUB_ROUTE_ID, SEG.GOVT_CONTROL, V_BMP, V_EMP, TAB0(N).SITE_CATEGORY);
        END IF;
      END LOOP;
    END LOOP;
  END LOOP;
  COMMIT;
END P_FMG_FINAL_SEGMENTATION;

PROCEDURE P_FMG_ATTRIBUTE_UPDATE
AS
TYPE TYPE1 IS TABLE OF VARCHAR2(4000);
  TAB1 TYPE1;
  SEC_ATTRIBUTE VARCHAR2(4000);
BEGIN
  FOR SEG IN
  (SELECT COLLECT_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, BMP, EMP
    FROM FMG_SITE_CATEGORY
    ORDER BY COLLECT_YEAR, GLOBAL_ROUTE_ID, SUB_ROUTE_ID, BMP)
  LOOP
    SELECT DISTINCT DIVIDED BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP;
    SEC_ATTRIBUTE :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET DIVIDED         = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT SPEED_LIMIT BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP;
    SEC_ATTRIBUTE :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET SPEED_LIMIT     = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT
      CASE
        WHEN RUT IS NULL THEN 'LOW_RUT'
        ELSE RUT
      END BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP;
    SEC_ATTRIBUTE :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET RUT             = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT INTERSECTIONS BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP
    AND INTERSECTIONS  IS NOT NULL ;
    SEC_ATTRIBUTE      :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET INTERSECTIONS   = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT
      CASE
        WHEN DOWNHILL_GRADE IS NULL THEN 'NO_GRADE'
        ELSE DOWNHILL_GRADE
      END BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP;
    SEC_ATTRIBUTE :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET DOWNHILL_GRADE  = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT RAMP BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP
    AND RAMP           IS NOT NULL ;
    SEC_ATTRIBUTE      :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET RAMP            = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT
      CASE
        WHEN CURVE IS NULL THEN 'NO_CURVE'
        ELSE CURVE
      END BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
    AND BMP            >= SEG.BMP
    AND EMP            <= SEG.EMP;
    SEC_ATTRIBUTE :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET CURVE           = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR  = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID    = SEG.SUB_ROUTE_ID
        AND BMP             = SEG.BMP
        AND EMP             = SEG.EMP;
      END IF;
    END LOOP;
    SELECT DISTINCT CAT_ONE_FEATURES BULK COLLECT INTO TAB1
    FROM FMG_BASE_SEGMENTS
    WHERE COLLECT_YEAR    = SEG.COLLECT_YEAR
    AND GLOBAL_ROUTE_ID   = SEG.GLOBAL_ROUTE_ID
    AND SUB_ROUTE_ID      = SEG.SUB_ROUTE_ID
    AND BMP              >= SEG.BMP
    AND EMP              <= SEG.EMP
    AND CAT_ONE_FEATURES IS NOT NULL ;
    SEC_ATTRIBUTE        :=NULL;
    FOR I IN 1..TAB1.COUNT
    LOOP
      IF I            =1 THEN SEC_ATTRIBUTE:= TAB1(I); END IF;
      IF I >1 THEN SEC_ATTRIBUTE:= SEC_ATTRIBUTE||','||TAB1(I); END IF;
      IF I =TAB1.COUNT THEN
        UPDATE FMG_SITE_CATEGORY
        SET CAT_ONE_FEATURES = SEC_ATTRIBUTE
        WHERE COLLECT_YEAR   = SEG.COLLECT_YEAR
        AND GLOBAL_ROUTE_ID  = SEG.GLOBAL_ROUTE_ID
        AND SUB_ROUTE_ID     = SEG.SUB_ROUTE_ID
        AND BMP              = SEG.BMP
        AND EMP              = SEG.EMP;
      END IF;
    END LOOP;
    COMMIT;
  END LOOP;
END P_FMG_ATTRIBUTE_UPDATE;
END FRICTION_GUIDE_PKG;

## Section Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0.0 | 2020-Jun-15 | SHA PM Team | Initial document creation |
| 2.0.0 | 2025-Feb-25 | Aditya R. | Document format converted to plain text. Converted some tables to accessible format |
| 3.0.0 | 2026-Feb-05 | Aditya R. | No changes made to text. First Git commit |
| 3.0.2 | 2026-Feb-10 | Aditya R. | Replaced MDOT-SHA with SHA and added Section Version History table |
| 3.0.3 | 2026-Feb-13 | Aditya R. | Minor formatting edits and updated document change log table |
| 3.0.4 | 2026-Feb-17 | Aditya R. | Added section break and minor formatting changes |

## Document Change Log

The following table lists the major changes made since the previous version of this guide was published online. See the Section Version History table in the relevant section for more details.

| Section | Change |
| --- | --- |
| 1.0 | Updated flowcharts, organization charts, and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 2.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 3.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 4.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and EDW with PDW. Updated KPA Reports section with comprehensive procedures for managing KPA tables and views. |
| 5.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA, DWT with DAT, and EDW with PDW. |
| 6.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 7.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 8.0 | Updated flowcharts and added alt text to all images. Updated Production of HPMS Sample Table section with comprehensive procedures and validation steps. Added Production of HPMS Pavement Metadata file,Production of HPMS Submission Comments, Process PM2 Dataset, and NHS Pavement Condition sections with comprehensive procedures. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 9.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 10.0 | Updated flowcharts and added alt text to all images. Replaced MDOT-SHA with SHA and DWT with DAT. |
| 11.0 | Updated flowcharts and added alt text to all images. Converted some tables to accessible format. Replaced MDOT-SHA with SHA and DWT with DAT. |