NOTICE TO ALL HOLDERS OF THIS CONTRACT DOCUMENT

NATIONAL COOPERATIVE HIGHWAY RESEARCH PROGRAM (NCHRP) REPORT 350 AND THE MANUAL FOR ASSESSING SAFETY HARDWARE (MASH) COMPLIANCE FOR DEVICES USED IN THE MAINTENANCE OF TRAFFIC AND TRAFFIC CONTROL

Except as otherwise specified in this section, all items for the temporary traffic control, including those listed under the following categories and permanent roadside hardware such as concrete barriers, W-beam barriers, cable barriers, end terminals, crash cushions, sign supports and all other breakaway hardware, shall be crashworthy in conformance with the appropriate test level as required by the safety crash testing and performance criteria published in the National Cooperative Highway Research Program (NCHRP) Report 350, “Recommended Procedures for the Safety Performance Evaluation of Highway Features” or the Manual for Assessing Safety Hardware (MASH).  When conformance with NCHRP Report 350 or MASH is required, provide the Engineer with the manufacturers’ certifications that the devices comply with the specified criteria.  All temporary and permanent roadside devices shall comply with MASH 2016 criteria by the relevant dates in accordance with the Joint Implementation Agreement memorandum dated January 7, 2016 between the American Association of State Highway and Transportation Officials (AASHTO) and Federal Highway Administration (FHWA).  This document may be downloaded from:

https://safety.fhwa.dot.gov/roadway_dept/policy_guide/road_hardware/docs/memo_joint_implementation_agmt.pdf

Unless specifically waived in the Contract Documents, only devices approved by the Administration may be used.

Category 1 Devices

These devices include cones, tubular markers, flexible delineator posts, and drums, none of which have any accessories or attachments, and are used for channelization and delineation.

Category 2 Devices

These devices include Type I, II, and III barricades, portable sign supports with signs, intrusion alarms, drums, vertical panels, and cones, any of which having accessories or attachments.

Category 3 Devices

(a)	Truck Mounted Attenuators (TMAs) and Trailer Truck Mounted Attenuators (TTMAs).

(b)	Temporary Barrier.

(1)	Concrete Barrier.

(2)	Traffic Barrier W Beam and Water Filled Barrier.

(3)	Steel/Aluminum Barrier.

(c)	Temporary End Treatments.

Category 4 Devices

These devices include area lighting supports, arrow panels, and portable variable message signs that may be portable or trailer-mounted.

Category 1, 2 and 3 devices shall always conform to NCHRP Report 350 or MASH Criteria.   The AASHTO/FHWA Joint Implementation Agreement for MASH states that temporary work zone devices including category 4 devices manufactured after 12/31/2019 must be successfully tested to the 2016 edition of MASH before 12/31/2019 and relevant devices manufactured on or before 12/31/2019 and successfully tested to NCHRP 350, the 2009 edition of MASH, or otherwise authorized may continue to be used throughout their normal service life.  Use of Category 4 devices shall comply with the provisions of Part 6 of the MD MUTCD.