Contact the District Traffic Engineer to determine if Drone Radar is required on the project.

CATEGORY 100

PRELIMINARY

SECTION 104 — MAINTENANCE OF TRAFFIC

104.25 DRONE RADAR.

104.25.01 DESCRIPTION.  Furnish, install, and relocate drone radar as specified in the Contract Documents or as directed by the Engineer.  A drone radar consists of an electronic device that activates all types of on-board radar detectors without causing interference to normal police radar operations.

104.25.02 MATERIALS.

Drone Radar	As approved by the Office of Traffic and Safety

General.  The electronic device shall be capable of being securely mounted to a vertical or horizontal surface.  The unit shall be of weatherproof and waterproof construction and operate efficiently from -20 to +160 F.

FCC Equipment Authorization.  The device shall bear an FCC Equipment Authorization for unlicensed use by the general public under FCC Title 47, Part 15.  All applicable FCC equipment regulations shall be met without any additional licensing required of the Administration or the Contractor.

Range.  The drone radar shall have an effective linear range of at least 2600 ft.

Power Source.  12 volts DC or 120 volts AC compatible/convertible.

Current Consumption.  1/2 amp maximum.

Frequency.  24.150 ± 0.100 GHz.

104.25.03 CONSTRUCTION.  The drone radar shall be furnished, positioned, repositioned, operated, maintained, and removed, as needed or as directed by the Engineer.  The unit may be truck or trailer mounted, fixed to a special lighting unit, portable changeable message sign, arrow panel, traffic sign, or traffic barrier W-beam, as directed by the Engineer.

104.25.04 MEASUREMENT AND PAYMENT.  Drone Radar will be measured and paid for at the Contract unit price per day.  A unit day shall consist of any approved usage within a 24 hour calendar day.  If a unit is used for part of a day, it will be measured as a unit day.

The payment will be full compensation for drone radar unit, installation, power supply, wiring, supports, relocating as required by the Traffic Control Plan or as directed by the Engineer, removal, and for all material, labor, equipment, tools, and incidentals necessary to complete the work.  Each Drone Radar device will be paid for only once per unit day, which will include any work necessary to maintain, re-align, or relocate the device; or replace damaged, missing or stolen devices.