CATEGORY 400

STRUCTURES

SECTION 421 — REINFORCING STEEL

421.02 MATERIALS

ADD:

| Stainless Steel Bars | 908.03 |
| --- | --- |
| Low Carbon Chromium Bars | 908.13 |

421.03 CONSTRUCTION

DELETE:  421.03.06 Splicing in its entirety.

INSERT:  The following.

421.03.06 Splicing.  Furnish bar lengths and splices as specified in the Contract Documents, and as detailed in the working drawings stamped as accepted by the Administration.  Do not perform additional splicing without approval.  Make lap splices with the bars in contact and wired together.  Do not weld reinforcing steel or weld attachments to reinforcing steel without approval. Perform welding according to AWS D1.4.

DELETE:  421.03.08 Substitution in its entirety.

INSERT:  The following.

421.03.08 Substitution.  Low carbon chromium or stainless steel reinforcing may be substituted on a one-for-one basis for the specified epoxy coated or non-epoxy coated deformed steel bar reinforcing, at the option of the Contractor, as approved by the Engineer.  Welding of low carbon chromium bars is prohibited. No adjustment shall be allowed in the size, number, spacing, laps and configuration of the reinforcing for any increase in strength or other properties provided by the substitute reinforcing bars.

There will be no additional compensation for bars of differing material composition in lieu of the bars specified.