# CATEGORY 500

# PAVING

SECTION 504  ASPHALT PAVEMENT

504.03 CONSTRUCTION

504.03.01 Equipment.

ADD: The following after the last paragraph of Subsection Pavers.

Safety Edge Magnetic Protractor.  Safety edge magnetic protractor shall have increments of 0

to 90 degrees in four quadrants, with a minimum 4 in. lens diameter and two level vials.

504.03.13 Thin Lifts and Wedge/Level Courses.

DELETE:  The last sentence in Subsection (g), “Incentives…courses.”

INSERT:  The following.

Density incentives are not applicable for Thin Lift or Wedge/Level Courses.  Apply mix pay factors as determined by the Engineer and the Contractor at the Pre-pave meeting.

# ADD:  The following after 504.03.18.

504.03.19 Safety Edge.  Construct a sloped edge on the pavement shoulder of open section roadways using a device attached to the paver that constrains the asphalt head to produce a compacted profile.  The device shall adjust to accommodate varying paving thicknesses.  The device shall be capable of being detached or raised when not in use.

Construct a safety edge when the individual thickness is at least 1.5 in. or greater.  The target range of the slope shall not be greater than 45 degrees when measured from the horizontal axis. Construct the sloped edge of multiple lifts on the top two pavement lifts when specified.  Construct the entire sloped edge on underlying pavement.

- Use the safety edge as a standard treatment for the outside edge of all open section roadways with shoulders up to 6 ft wide.

- Do not use the safety edge when the resulting travel lane is less than 11 ft wide.

- Using the safety edge magnetic protractor, record 10 angle readings per mile in the presence of the Engineer.  Readings will be witnessed by the Engineer and the results will be documented on Form OOC-91 by the Contractor for the project files.  The performance criteria target is for 90 percent of the readings to fall below 45 degrees.

- The safety edge does not replace the requirements in Section 609.

504.04 MEASUREMENT AND PAYMENT

ADD:  The following after the last paragraph.

All work associated with the construction and angle measurements of the safety edge will not be measured but the cost will be incidental to the pertinent asphalt pavement item. The labor, equipment, and recordation of the angel measurements of the Safety Edge will not be measured for payment and will be incidental to the pertinent asphalt pavement items.  All asphalt materials used in the creation of the pavement edge will be included in the measured tonnage related to the pertinent asphalt pavement item and will be eligible for all related adjustments, disincentives, and incentives.  Shoulder backup material will be measured and paid for as specified in Section 609.