CATEGORY 600

SHOULDERS

SECTION 603 — SIDEWALKS

603.02 MATERIALS

ADD:  The following.

Borrow Excavation  916.01

603.03 CONSTRUCTION

603.03.01 Concrete Sidewalks.

DELETE:  603.03.01(f) in its entirety.

INSERT:  The following.

(f) Cold Weather Construction and Curing. Refer to 520.03.02 and 520.03.12, except the requirement for an approved spraying machine with drive wheels is waived when using the liquid membrane forming compound method.  Do not allow pedestrian and vehicular traffic during the curing period.

DELETE:  603.03.03 Backfill. In its entirety.

ADD:  The following.

603.03.03 Backfill.

Use approved material to backfill sidewalk edges and complete final grading by placing topsoil in landscape areas according to the Contract Documents or as directed.  Perform Turfgrass Establishment as specified in Section 705 or perform Turfgrass Sod Establishment as specified in Section 708 over areas of topsoil according to the Contract Documents or as directed.

603.04 MEASUREMENT AND PAYMENT

DELETE:  The second paragraph.

INSERT:  The following.

The removal and disposal of unsuitable material will be measured and paid for at the Contract unit price for Class 2 Excavation, which price includes the cost of using suitable excavation as replacement material.  When Borrow, Topsoil, or other selected backfill is specified as replacement material, payment will be made at the Contract unit price for the respective items.

ADD:  The following after 603.04.02.

603.04.03.  Turfgrass Establishment and Turfgrass Sod Establishment will be measured and paid for separately.