NOTICE TO ALL HOLDERS OF THIS CONTRACT DOCUMENT

NATIONAL COOPERATIVE HIGHWAY RESEARCH PROGRAM (NCHRP) REPORT 350 AND THE MANUAL FOR ASSESSING SAFETY HARDWARE (MASH) COMPLIANCE FOR DEVICES USED IN THE MAINTENANCE OF TRAFFIC AND TRAFFIC CONTROL

Except as otherwise specified in this section, all temporary and permanent highway safety features, including longitudinal barriers, transitions, end terminals, crash cushions, breakaway/yielding supports, truck-mounted attenuators, and work zone traffic control devices, shall meet values recommended when applicable tests are performed for evaluation criteria for the respective evaluation factors, as defined in NCHRP Report 350, or the MASH 2016, as noted herein.  When conformance with NCHRP Report 350 or MASH 2016 is required, provide the Engineer with the manufacturers’ certifications that the devices comply with the specified criteria.  All temporary and permanent highway safety features shall comply with MASH 2016 criteria by the relevant dates noted below.

TEMPORARY INSTALLATIONS FOR MAINTENANCE OF TRAFFIC

The implementation dates below apply to temporary work zone roadside safety hardware and devices.

Temporary work zone devices, including Category 1, 2, 3 and 4 devices manufactured after 12/31/2019, when applicable, must be successfully tested to the 2016 edition of MASH. Relevant devices manufactured on or before 12/31/2019, and successfully tested to NCHRP 350, the 2009 edition of MASH, or otherwise authorized, may continue to be used.

Unless specifically waived in the Contract Documents, only devices approved on Qualified Product List by the Administration may be used.

Category 1 Devices

These devices include cones, tubular markers, flexible delineator posts, and drums, none of which have any accessories or attachments, and are used for channelization and/or delineation.

Category 2 Devices

These devices include Type I, II, and III barricades, portable sign supports with signs, intrusion alarms, and vertical panels.  Category 1 devices, such as drums or cones, that are modified with accessories or attachments shall be considered Category 2 devices.

Category 3 Devices

- Truck Mounted Attenuators (TMAs) and Trailer Truck Mounted Attenuators (TTMAs).

- Temporary Barrier.

- Concrete Barrier.

- Traffic Barrier W-Beam and Water Filled Barrier.

- Steel/Aluminum Barrier.

- Temporary End Treatments.

Category 4 Devices

These devices include area lighting supports, arrow panels, and portable variable message signs that may be portable or trailer-mounted.

Use of Category 4 devices shall comply with the provisions of Part 6 of the MD MUTCD.

PERMANENT ROADSIDE HARDWARE INSTALLATION

The implementation dates below apply to both new and replacement installations of roadside safety hardware on National Highway System (NHS) roadways except when a waiver is approved by FHWA

December 31, 2017:  Contracts with bid openings after this date shall meet MASH 2016 testing criteria for all installations and replacements of W-beam barriers and cast-in-place concrete barriers as specified in Contract Documents.

June 30, 2018:  Contracts with bid openings after this date shall meet MASH 2016 testing criteria for all installations and replacements of W-beam tangent terminals as specified in Contract Documents.

December 31, 2018:  Contracts with bid openings after this date shall meet MASH 2016 testing criteria for all installations and replacements of crash cushions.

December 31, 2019:  Contracts with bid openings after this date shall meet  testing criteria as defined in MASH 2016 guidelines for all new permanent installations and full replacements of bridge rail, transitions, all other longitudinal barrier (including portable barriers installed permanently), other W-beam terminals (such as double-sided or median terminals, flared terminals, and terminals installed on a flare), sign supports, cable barrier, cable barrier terminals, all other terminals, and all other breakaway hardware as specified in Contract Documents.