CATEGORY 600

SHOULDERS

DELETE:  SECTION 606 — PERMANENT TRAFFIC BARRIER END TREATMENTS in its entirety.

INSERT:  The following.

SECTION 606 — Permanent Traffic Barrier End Treatments

606.01 DESCRIPTION

Furnish and install permanent traffic barrier end treatments.

606.02 MATERIALS

Refer to 605.02, 701.02, 705.02, 708.02, 709.02, and the following:

| End Treatments | As specified by the manufacturer |
| --- | --- |
| Spare Parts Packages | As specified by the manufacturer |
| Antifreeze Agent | As approved |
| Reflectorization | 950.03 |
| Plastic Barrels (Yellow) | Standard MD 104.01-70 |
| Sand | 901.01 |
| Portland Cement Concrete | 902.10, Mix 2 and 6 |
| Concrete | 902.10, Mix 7 |

606.03 CONSTRUCTION

606.03.01 End Treatments

- Buried-in-Backslope End Treatment (Type A). Excavate the slope to install these components.  Bury the ends of the traffic barrier, the end treatment anchorage, and the bottom rail, when required, in a cut slope.  Upon installation, backfill and stabilize the area as specified in 606.03.02 to match the adjacent slope.

For single rail systems, use 6 ft posts throughout the entire end treatment.  For double rail systems, use 8 ft posts, except for the last three posts buried in the cut slope.

Construct the end treatment anchorage according to the Book of Standards.

Install traffic barrier W-beam as specified in 605.03.

- One-Sided Flared End Treatment (Type B). Install according to the manufacturer’s recommendations and the Book of Standards.

- One-Sided Parallel End Treatment (Type C). Install in a straight line according to the manufacturer’s recommendations and the Book of Standards.

- Two-Sided End Treatment and Crash Cushion (Types D, E, F, and J ). Install these systems as specified by the manufacturer.  Refer to the manufacturer’s recommendations for installation methods and procedures.

- One-Sided Downstream End Treatment (Type K). Install according to the Book of Standards.

- Radius End Treatment (Type L). Install according to the Book of Standards.

- All Other Traffic Barrier End Treatments. Install these systems as specified by the manufacturer.  Refer to the manufacturer’s recommendations for installation methods and procedures.

- Nose Section. Reflectorize as shown in the Contract Documents and as approved by the Office of Traffic and Safety.

- Permanent Crash Cushion Sand Filled Plastic Barrels (SFPB). Provide the components and assemble, place in the required configuration, and fill each barrel according to the manufacturer's recommendations or as specified in the Contract Documents.  Ensure that each SFPB is watertight and separated from other SFPB by a distance of 3 in.  Place the last row of SFPB 12 in. from the shielded object.

Reflectorize the first barrel of the SFPB configuration as specified.  Mix approved antifreeze agent into loose, dry sand according to the manufacturer’s recommendations and install sand mixture in barrels.

606.03.02Grading Adjustment. When grading adjustment is required for installation of traffic barrier end treatments and crash cushions, adjust the grading using topsoil, crusher run aggregate CR-6, asphalt millings or grindings, subsoil, common borrow, or other approved material, as follows:

(a) Non-Landscaped Areas. For grading adjustment, install common borrow or subsoil as specified in 204.03.04 or 701.03.04 to the grade that is 4 in. below the final grade.  Complete the grading adjustment with crusher run aggregate CR-6 or asphalt millings or grindings.

(b) Landscaped Areas. For grading adjustment up to 8 in. depth, complete the adjustment with topsoil as specified in Section 701, and perform sodding or seeding as described in (c) below.  For surface adjustment more than 8 in. depth, the use of aggregate fill is prohibited.  Install subsoil or approved common borrow as specified in Section 204 or 701  to the grade that is 4 in. below the final grade.  Complete the adjustment with topsoil as specified in Section 701, and perform sodding or seeding as described in (c) below.

- Sodding or Seeding. Immediately after placing topsoil, perform Turfgrass Sod Establishment as specified in Section 708.  Alternatively, perform Turfgrass Establishment as specified in Section 705 and install Type A or Type E Soil Stabilization Matting as specified in Section 709.

606.04 MEASUREMENT AND PAYMENT

The payment will be full compensation for all excavation, removal, and disposal of the existing end treatment to be replaced, fabrication of all component parts, transitions to barriers, reflectorization, backfill, compaction, topsoil, crusher run aggregate CR-6, asphalt milling or grindings, common borrow or subsoil, and for all material, labor, equipment, tools, disposal fees, and incidentals necessary to complete the work, unless pay items are included in the Contract Documents.

Restoration of landscape and non-landscape areas with specified materials will be incidental to the pertinent traffic barrier end treatment item and will not be separately measured and paid for unless the aggregate or landscaping materials are included in the Contract Documents.

606.04.01 Buried-in-Backslope End Treatment (Type A) will be measured and paid for at the Contract unit price per each.

606.04.02 One-Sided Flared End Treatment (Type B), One-Sided Parallel End Treatment (Type C), One-Sided Downstream End Treatment (Type K), Radius End Treatment (Type L), and Two-Sided End Treatment and Crash Cushion (Type D, E, F and J) will be measured and paid for at Contract unit price per each.

606.04.03 Grading Adjustment using topsoil and subsoil will be measured and paid for at the Contract unit price as specified in 701.04.

606.04.04 Turfgrass Sod Establishment or Turfgrass Establishment will be measured and paid for at the Contract unit price per square yard.

606.04.05 Type A Soil Stabilization Matting or Type E Soil Stabilization Matting will be measured and paid for at the Contract unit price per square yard.

606.04.06 Grading Adjustment using CR-6 crusher run aggregate, or asphalt millings or grindings, will be measured and paid for at the Contract unit price per square yard, per ton, or as specified for the pertinent items.  The payment will also include full compensation for furnishing, adjusting, and compacting embankment or aggregate material.

606.04.07 Permanent Crash Cushion Sand Filled Barrels will be measured and paid for at the Contract unit price per barrel.  The payment will also include full compensation for furnishing and placing sand and antifreeze agent.

606.04.08 Repairs

- Traffic Barrier End Treatment Spare Parts Package furnished and installed will be measured and paid for at the Contract unit price per each for the type specified.  The payment will also include full compensation for the clearing and removal of debris and damaged unsalvageable parts, and for restoring damaged topsoil, turfgrass, or aggregate.

- When spare parts packages are furnished by the Administration, Repairing Traffic Barrier End Treatments will be measured and paid for at the Contract unit price per each for the type specified.  The payment will also include full compensation for all transportation, reconnection to fixed objects where necessary, clearing and removal of debris and damaged unsalvageable parts, and for restoring damaged topsoil, turfgrass, or aggregate.

- Payment will not be made for replacement end treatments or spare parts packages  used for end treatments damaged due to the Contractor’s operations.

606.04.09 Grading Adjustment using common borrow will be measured and paid for at the Contract unit price per cubic yard as specified in 203.04.

606.04.10 Grading Adjustment using other approved materials will be measured and paid for as pertinent items.