##### CATEGORY 900

MATERIALS

SECTION 905 — Pipe

##### DELETE:  905.03 in its entirety.

INSERT:  The following.

905.03 CERTIFIED CORRUGATED POLYETHYLENE PIPE PLANTS

Polyethylene pipe conforming to specifications will be accepted on the manufacturer’s certification based on the requirements outlined below.  This includes the sampling, testing documentation, and certification of the product by the manufacturer in combination with an Administration Monitoring Program.

905.03.01 Responsibilities of the Corrugated Polyethylene Pipe Manufacturer.  All manufacturers of Corrugated Polyethylene Pipe must be enrolled in the National Transportation Product Evaluation Program (NTPEP) to provide Corrugated Polyethylene Pipe to Administration projects.  If a new manufacturer has applied to NTPEP and is awaiting the NTPEP process, the manufacturer may submit to OMT proof of application, a copy of the QCP, and results of an audit and comparison testing by an independent third-party firm which has been signed by a Professional Engineer.  Upon review, a pending letter of approval will be issued until such time as the NTPEP audit process is completed.

905.03.02 Test Facilities.  The NTPEP Certification Program requires all tests to be conducted at laboratories that are accredited by AASHTO or approved by the Administration.  Each source may establish and maintain its own laboratory for the performance of quality control testing or may request to utilize an approved independent laboratory.  The manufacturer shall make a written request and have written approval from the Administration prior to having material tested off site.  The equipment required for all approved laboratories shall be qualified to perform the required test procedures as required by the applicable specification and standards.

905.03.03 Certification.  The manufacturer shall certify as specified in TC-1.03, each shipment of material.  The certification shall be submitted to the Structural Materials Lab for review and stamping upon shipment of material.  The certification shall include the following:

- Plant name, address, and location.

- Lot or production identification.

- Date of manufacture and shipment.

- Number of units of each size pipe or total linear ft of each size pipe.

- Contract number.

- Statement of specification compliance.

- Signature of the quality control manager, or authorized company representative (name shall be designated in the QCP).

- Perforation dimensions.

- Identification Markings.

- Test data as applicable.

  - Density

  - Melt Index

  - NCLS

  - UCLS

  - Carbon Black Content

  - Water Inlet Data

  - Pipe Stiffness

  - Pipe Flattening

  - Brittleness

  - OIT

  - Elongation at Break

905.03.04 Records.  The manufacturer shall maintain quality control records according to their Quality Control Plan.  All testing and inspection documents shall be maintained at the manufacturing facility for at least seven years from the manufactured date and made available upon request.

905.03.05 Responsibilities of the Administration.  The Administration may independently verify the manufacturer’s certification results by taking random samples of the finished pipe for testing from plant inspections or from project site samples and review all testing results for conformance to specifications.

The Administration will perform a visual inspection on pipe received at the project and will collect the manufacturer’s certification at the time of delivery.