Category 100

Preliminary

SECTION 104.22 — Stop/Slow Automated Flagger Assistance Device (AFAD)

104.22.01 DESCRIPTION

An automated flagger assistance device (AFAD) system consists of two paired AFAD units of the same make and model that can be paired and operated by one or two certified flaggers.  AFADs shall only be used in situations where there is only one lane of approaching traffic in the direction to be controlled.

Furnish, install, and maintain a portable, self-contained, trailer mounted STOP/SLOW AFAD for temporary traffic control as specified in the Contract Documents or as directed.  Furnish certified flaggers to operate the AFADs.  Flaggers operating the AFADs shall meet the requirements specified in 104.15.01.  AFAD shall meet Sections 6E.04 and 6E.05 of the MdMUTCD.

104.22.02 MATERIALS

| STOP/SLOW AFAD | QPL |
| --- | --- |

The AFAD system shall have a conflict monitor system to prevent two units from simultaneously flashing the SLOW indication.  Each AFAD shall be capable of being operated from the controller cabinet mounted on the unit.  The unit shall have the ability for manual override in case of remote failure.

104.22.02.01 STOP/SLOW Sign.  Each AFAD unit shall include a STOP/SLOW sign that alternately displays the STOP (R1-1) face and the SLOW (W20-8) face of a STOP/SLOW paddle.  The STOP/SLOW sign shall have an octagonal shape and shall be fabricated of rigid material.  The size of the STOP/SLOW sign shall be at least 30 in. x 30 in. with letters at least 8 in. high.  The background of the STOP face shall be red with white letters and border.  The SLOW face shall be diamond shaped with fluorescent orange background and black letters and border.  Both faces of the STOP/SLOW sign shall be reflectorized as specified in 950.03.  The bottom of the SLOW/STOP sign shall be mounted a minimum of 7 ft above the pavement.  Maintain signs in a new or like new condition.  When directed, replace signs that become faded, illegible, or damaged.

104.22.02.02 Stationary Signs.  Each AFAD shall be supplemented by stationary sign(s) with one of the following options:

(a) Separate WAIT ON STOP (R1-7) and GO ON SLOW (R1-8) signs attached side by side below the STOP/SLOW sign.  Each sign shall be 24 in. x 30 in.  The background of the sign shall be white with black letters and border, and the sign face shall be reflectorized as specified in 950.03.

(b) One combined sign with combined message “WAIT ON STOP GO ON SLOW” attached below the STOP/SLOW sign.  The sign shall be 36 in. x 36 in.  The background of the sign shall be white with black letters and border, and the sign face shall be reflectorized as specified in 950.03.

The signs (‘WAIT ON STOP’, ‘GO ON SLOW’, or the combined ‘WAIT ON STOP GO ON SLOW’) shall be positioned on the same support structure as the AFAD such that they are in the same direct line of view of approaching traffic as the sign faces of the AFAD.

104.22.02.03 Conspicuity Devices.  The AFAD shall be supplemented with active conspicuity devices.  Each AFAD unit shall include a red Stop Beacon mounted a maximum of 24 in. above the STOP/SLOW face and yellow Warning Lights mounted to the side or above the STOP/SLOW face.  The Stop Beacon shall consist of a 12 in. red LED signal head with visor and shall steadily illuminate in circular red only during the STOP sign display.  The Warning Lights shall illuminate in flashing yellow during the SLOW sign display.

104.22.02.04 Gate Arm.  The AFAD shall include a gate arm that descends to a horizontal position when the STOP sign is displayed and rises to vertical position when the SLOW sign is displayed.  The length of the arm shall be at least 8 ft and the end of the arm shall reach at least to the center of the lane being controlled.  The gate arm shall not extend in the opposite lane.  The arm shall have alternating red and white retroreflective stripes on both sides sloping downward at 45 degrees toward the side on which traffic will pass.  The arm shall be made of lightweight rigid material and be able to deflect and return to a functioning position after a vehicle strike.  A retroreflective red or fluorescent retroreflective orange/red flag (minimum of 18 in. square) shall be attached on the end of the roadside end of the gate arm.

104.22.02.05 Trailer.  Each unit shall include the trailer, structural support system, controller and ancillary equipment.  The trailer shall be able to withstand wind velocities of 80 MPH and remain operational.  The trailer shall be painted orange.  The trailer shall comply with Maryland Motor Vehicle Law.

104.22.02.06 Remote Control.  The AFAD shall be equipped with weatherproof handheld remote control that is capable of displaying the status of each sign.  The Remote control shall be powered by rechargeable batteries.  The remote control shall be programmable to control either one unit or two units and be able to control units over one-mile range.

104.22.02.07 Power Supply.  The AFAD shall be able to operate from a solar powered electrical system and consist of battery power and solar array panels.  The battery life shall sufficiently maintain the operation for the duration of the lane closure or have the ability to be recharged without deactivating the device.

104.22.03 CONSTRUCTION

104.22.03.01 AFAD.  Place the AFAD trailer as shown in the Contract Documents or as directed, with all signs and traffic control devices visible to the driver of the initial approaching vehicle.  Install advance warning signs and channelizing devices along the road center line and around the AFAD according to the Book of Standards and the Contract Documents.

The AFAD shall be like new, corrosion resistant, and unaffected by water spray, salt, oil, gasoline, and all other contaminants in the quantities normally found along the edge of the traveled roadway.  Construction, materials, and operation shall be according to NFPA, ULI, and NEC.

When used at night, the AFAD location shall be adequately illuminated.

104.22.03.02 Transition between STOP and SLOW Conditions

(a) SLOW to STOP:  The red Stop Beacon shall enter a “ﬂashing mode” approximately two seconds before transitioning from the SLOW face to the STOP face.  When the AFAD unit displays the STOP face, the red Stop Beacon shall illuminate in steady red, and the gate arm shall begin descent to the down position two seconds after the beacon has displayed the solid red light.

(b) STOP TO SLOW:  The gate arm shall begin to ascend to the upright position as the face changes from STOP to SLOW and the red stop light shall cease to illuminate.  The yellow beacon(s) shall enter flashing mode.

104.22.03.03 Flaggers.  Provide certified flaggers on-site and use one of the following Methods to operate the AFAD(s):

(a) Method 1 – Place an AFAD at each end of the work area.

(b) Method 2 – Place an AFAD at one end and place a flagger at the opposite end.

The flagger operating the AFAD shall be able to view the device and the approaching traffic during operation.

104.22.03.04 Channelizing Devices.  Install channelizing devices according to the MdMUTCD, the Book of Standards, the Contract Documents, and the following:

(a) Install a minimum of 11 channelizing devices spaced at 20 ft maximum along the center line on the approach to the AFAD.

(b) If the AFAD is not behind a barrier, use a minimum of 7 channelizing devices to form the shoulder taper to delineate the AFAD.

104.22.04 MEASUREMENT AND PAYMENT

104.22.04.01 Automated Flagger Assistance Device will be measured and paid for at the Contract price per unit day.  A unit day will consist of any approved usage within a work shift, regardless of start time or whether the shift runs into another calendar day.  Each Automated Flagger Assistance Device will be paid for only once per unit day, regardless of how many times it may be relocated.  When an Automated Flagger Assistance Device is used for part of a day, it will be measured and paid for as a unit day.  The payment will be full compensation for the Automated Flagger Assistance Device and for all material, labor, equipment, tools, and incidentals necessary to setup and operate the Automated Flagger Assistance Device at the site as required and at any relocated site as specified or as directed.

104.22.04.02 Certified flaggers to operate Automated Flagger Assistance Device will not be measured but the cost will be incidental to the Contract unit price for Automated Flagger Assistance Device.

104.22.04.03 Channelizing devices to delineate the center line in the approach to the Automated Flagger Assistance Device will not be measured but the cost will be incidental to the Contract unit price for Maintenance of Traffic.

104.22.04.04 Temporary traffic signs will be measured and paid for as specified in 104.08.