Category 100

Preliminary

SECTION 104.30 — Portable Traffic Signal (PTS))

104.30.03 CONSTRUCTION

DELETE:  The second paragraph, “The Contractor…Office of Traffic & Safety.” in its entirety.

INSERT:  The following.

The Contractor is responsible for all setup procedures necessary for the PTS including setting up the trailers, setting up and testing the remote monitoring system notification, aiming signal heads, interconnecting the signal controllers, aiming, and adjusting vehicle detection units, and installing the timing provided by the Office of Traffic & Safety.

104.30.03.01 Equipment

Operational Requirements.

DELETE:  Paragraphs (m) through (t), “The operating system…return to full operational mode.”

INSERT:  The following after paragraph (l).

(m) The operating system shall have an integrated mechanism capable of recording system malfunctions including date and time of system failure. This information shall be available in a printable format.

(n) The operating system shall be capable of accommodating a pre-emption system with optical activation which provides a priority green phase in the direction of appropriately equipped approaching emergency vehicles.

(o) The operating system shall have the capability to allow the PTS to be connected to and controlled by a standard NEMA controller.

(p) The operating system shall have a manual control option that will allow the Traffic Manager to manually control the signal to coordinate the PTS system for special operations.

(q) The operating system shall be equipped with diagnostic capabilities in the event of a system default.

(r) The system shall have the capability of identifying the default in a manner that will expedite the return to full operational mode.

Monitoring System Requirement. Furnish and install PTS monitoring system capable of reporting signal location, battery voltage and system default.  The monitoring system shall have the ability to communicate system default via text or email to responsible individuals.  As specified in GP-2.04 verify available communications at the site, e.g., cellular or satellite, and select an appropriate PTS monitoring system that works with that communication method.  Coordinate the installation of monitoring system with the ADE-T to develop an acceptable plan for monitoring signal operations and reporting on system default.

Default Requirements.

DELETE:  The first sentence, “PTS system…upon system default”.

INSERT:  The following.

PTS systems shall have the capability of reverting to a solid red, red flash, or yellow flash mode upon system default.

Field Operations

DELETE:  The third paragraph, “Make provisions…in the event of a failure.” in its entirety.

ADD:  The following after the last paragraph, “Operate…when available.

Provide MDOT SHA with the name and telephone number of an emergency contact person to maintain and operate or repair the signals.  Make provisions to have certified flaggers who have completed the MDOT SHA approved flagger training course and make provisions to have appropriate signing in place to maintain traffic at the PTS location in case of a PTS failure.  Notify the Engineer, the Statewide Operations Center (SOC), and the ADE-T in the event of a failure.  Maintain daily log at the signal site and record equipment malfunction.

104.30.04 MEASUREMENT AND PAYMENT

DELETE:  The first sentence of the second paragraph, “The payment…complete the work.”

INSERT:  The following.

The payment will be full compensation for the provision, installation, operation, interconnection, monitoring system, maintenance, reconfiguration, adjustments, flaggers, signing, repair technicians, relocation as required by the Traffic Control Plan or as directed by the Engineer, and for all materials, labor, equipment, tools, training by the portable traffic signal manufacturer, and incidentals necessary to complete the work.