Category 400

Structures

SECTION 430 — Metal Structures

430.03 CONSTRUCTION

430.03.12 Holes

ADD:  The following after 430.03.12(c) Subpunching and Reaming.

430.03.12(d) Computer Numerically Controlled (CNC) Drilling.  Instead of reaming sub-sized holes or drilling full-sized holes while assembled, the contractor may use CNC drilling or punching equipment to drill or punch full-sized holes in unassembled pieces, connections, and templates for use with matching sub-sized and reamed holes.  The contractor may use CNC equipment to either drill or punch holes through individual pieces or drill through any combination of pieces held tightly together.  Full-size punched holes shall meet the requirements of 430.03.12(a).

430.03.13 Shop Assembly.

ADD:  The following after the last paragraph in 430.03.13, “Secure parts…inspection.”

When using Computer Numerically Controlled (CNC) Drilling, furnish a check assembly for each major structural type of each project unless otherwise indicated or specified in the special provisions.  Provide check assemblies consisting of at least three contiguous shop sections or, for a truss, all members in at least three contiguous panels but not less than the number of panels associated with three contiguous chord lengths (i.e. length between field splices).  Base check assemblies on the proposed order of erection, joints in bearings, special complex points and similar considerations.  Special complex points include the portals of skewed trusses.

Use the first sections of each major structural type to be fabricated as the check assemblies.

Obtain approval for each CNC drilled check assembly before reaming or dismantling the assembly.  If a check assembly fails to demonstrate that the required accuracy is being obtained for camber, alignment, accuracy of holes and fit of milled joints, the Administration or our shop inspector may require additional check assemblies.  Additional check assemblies will be at no additional cost to the Administration.